3GPP TSG RAN WG1 Meeting #110	R1-2206158

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	8.14

Source:	MediaTek Inc.

Title:	Maintenance on NB-IoT/eMTC to support NTN

Document for:	Discussion and Decision

- 1 Introduction

In RAN#109-e [1], there are some pending issues for IoT NTN. In this contribution, we provide our perspectives.

- 2 Inclusion of UE-gNB RTT

There are some agreements on the UE-eNB RTT in RAN1 108e:

Agreement (RAN1 108e) For IoT NTN, calculate UE-eNB RTT using the following equation: RTTUEeNB=TTATf+Kmac where Tf = subframe duration (1ms).

In the Pseudo CR to TS36.321 Section 5.1.4 [2], the following is included:

If the UE is a BL UE or a UE in enhanced coverage: - if the random access preamble was transmitted in a non-terrestrial network: - RA Response window starts at the subframe that contains the end of the last preamble repetition plus 3 + UE-eNB RTT subframes and has length ra-ResponseWindowSize for the corresponding enhanced coverage level; If the UE is an NB-IoT UE: - if the random access preamble was transmitted in a non-terrestrial network: - RA Response window starts at the subframe that contains the end of the last preamble repetition plus X + UE-eNB RTT subframes has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions;

It is necessary to capture UE-eNB RTT in RAN1’s CR with the UE-eNB RTT provided in an integer number of subframes to align with the agreement.

Proposal 1: Agree on the capturing RAN1#108e agreement on RTTUEeNB=TTATf+Kmac in TP #1 to TS 36.213 Sections 16.3.1.

---------------------------------------- Start of TP #1 for 3GPP TS 36.213 ---------------------------------------- 36.213 section 16.3.1 <Unchanged Text Omitted> - Detection of a NPDCCH with DCI scrambled by RA-RNTI is attempted during a window controlled by higher layers (see [8], Clause 5.1.4), where UE- eNB RTT is calculated as floor(TTATf)+Kmac subframes, where TTA is specified in [TS 36.211, Clause 8.1], Tf is the subframe duration (1ms), and Kmac is provided by the higher layer parameter K-Mac in unit of 1 ms or Kmac=0 if K-Mac is not provided. If detected, the corresponding DL-SCH transport block is passed to higher layers. The higher layers parse the transport block and indicate the Nr-bit uplink grant to the physical layer, which is processed according to Clause 16.3.3. <Unchanged Text Omitted> ---------------------------------------- End of TP #1 for 3GPP TS 36.213 -----------------------------------------

3 Conclusion

In this contribution, remaining issues for IoT NTN were addressed. The following proposal was made

Proposal 1: Agree on the capturing RAN1#108e agreement on RTTUEeNB=TTATf+Kmac in TP #1 to TS 36.213 Sections 16.3.1.

---------------------------------------- Start of TP #1 for 3GPP TS 36.213 ---------------------------------------- 36.213 section 16.3.1 <Unchanged Text Omitted> - Detection of a NPDCCH with DCI scrambled by RA-RNTI is attempted during a window controlled by higher layers (see [8], Clause 5.1.4), where UE-eNB RTT is calculated as floor(TTATf)+Kmac subframes, where TTA is specified in [TS 36.211, Clause 8.1], Tf is the subframe duration (1ms), and Kmac is provided by the higher layer parameter K-Mac in unit of 1 ms or Kmac=0 if K-Mac is not provided. If detected, the corresponding DL-SCH transport block is passed to higher layers. The higher layers parse the transport block and indicate the Nr-bit uplink grant to the physical layer, which is processed according to Clause 16.3.3. <Unchanged Text Omitted> ---------------------------------------- End of TP #1 for 3GPP TS 36.213 -----------------------------------------

4 References

- “RAN1 Chair’s Notes 109e.”

- R2-2206794, “Corrections for Supporting Non-Terrestrial Network in NB-IoT and eMTC.”