3GPP TSG RAN WG1 #110		R1-2206166

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.2.3.1

Source:	Fujitsu

Title:	Evaluation on AI/ML for beam management

Document for:	Discussion/Decision

Introduction

In RAN#109 e-meeting, the initial EVM has been agreed [1] for evaluation on AI/ML for beam management. This contribution presents our views on EVM and the evaluation results of spatial-domain DL beam prediction.

Evaluation on spatial-domain DL beam prediction

Beam prediction for sTRP

For sTRP scenario, TRP configures subset of RSs for beam measurement. And UE measures L1-RSRPs of subset of beam pairs and input them to the AI/ML model. The AI/ML model will predict the L1-RSRPs of all beam pairs. Then, the potential one or several best beam pairs are selected among the predicted L1-RSRPs of all beam pairs. The function of AI/ML model is shown as Figure 1.

Beams in transmitter

Beams in receiver

not measured beam

measured beam

AI/ML

Model

RSRPs for sub-set of beam pairs

RSRPs for all beam pairs

Figure 1: the function of AI/ML model for sTRP scenario

For the AI/ML model, fully connected neural network is applied for spatial-domain DL beam prediction as following figure shows.

Input layer

Hidden layer (256,384)

Output layers

（256,384））

Figure 2 the architecture of AI/ML model

In above figure, the neural network includes 4 fully connected layers. Two hidden layers have 256 and 384 nodes respectively. The number of nodes for input and output layers depends on the number of input beams and expected output beams.

KPIs

In RAN#109 e-meeting, KPIs of EVM for spatial-domain DL beam prediction have been discussed and agreed in [1].

For beam prediction accuracy, the definition is still FFS with two options.

- Option 1: The beam prediction accuracy (%) is the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

- Option 2: The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

Considering the beam sweeping with two steps, in the first step, the AI/ML model will predict one or several optimal beams (Top-1/K predicted beams). The Top-K predicted beams will be configured for beam sweeping in the second step. If such Top-K predicted beams include the Top-1 genie-aided beam, the Top-1 genie-aided beam will be selected by the conventional method. In this case, the definition of option 2 is more suitable for two-step beam sweeping.

Proposal 1: Considering beam sweeping with two steps, the option 2 is selected as the KPI of beam prediction accuracy.

- The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

In RAN#109 e-meeting, the AI/ML model complexity and computational complexity have been agreed as one of KPIs, but the details are still FFS. From our points of view, the complexity of AI/ML model needs to consider the storage complexity and computational complexity which may depend on the architecture of neural network.

To analyze the impacts of complexity on different architectures of neural network, another AI/ML model is compared with the model in Figure 2.

Conv1

Conv2

FC

Conv3

Muti-conv

Conv4

Conv-m1

Conv-m2

Conv-m3

(3x3,16)

(3x3,1)

(3x3,16)

(3x3,1)

(3x3,16)

(3x3,32)

(3x3,16)

Figure 2-A the architecture of AI/ML model

In above figure, the neural network includes four convolution networks, one residual network and one fully connected network. And the fully connected network is the same as figure 2. The complexity of the two AI/ML models is compared in inference phase with the simulation condition described in section 2.3.1.

Table 1: complexity comparison for the different AI/ML models

|  | Number of float point of operations | Number of parameters | Memory size |
| --- | --- | --- | --- |
| FC (figure 2) | 213.4K | 214K | 857KB |
| FC + CNN (figure 2-A) | 14.6M | 270K | 1.1MB |

For the complexity of storage, the memory size depends on the storage format of the AI/ML model, but it’s an implementation issue and beyond our discussion. According to results of complexity comparison for two AI/ML models, the memory cost highly depends on the number of parameters if the same storage format is used. On the other hand, the data type of parameters (e.g. double-float, single-float, etc.) is also one factor to evaluate the model’s complexity of computation and storage.

When the computational complexity of AI/ML model is analyzed, different implementation technologies (e.g. CPU/GPU, ASIC) may have different parallel processing capability which is a typical implementation issue and beyond our discussion. The general KPI about computational complexity is required without considering the details of implementation technologies. According to the results of complexity comparison for two AI/ML models, different architectures of neural network have big impacts on computational complexity even if they have the similar number of parameters. About 70 times computation difference between FC network and FC+CNN network is observed since the calculation of convolution costs more computation. However, it’s agreed in [1] that RAN1 does not attempt to define any common AI/ML model as a baseline. In this case, different companies may use the different architectures of neural network during the evaluation. To compare the computational complexity of the AI/ML model, the number of float point of operations will be KPI without considering the architecture of neural network.

Observation 1:

- The number and data type of parameters can indicate the cost of memory storage for the AI/ML model.

- The number of float point of operations can indicate the computational complexity for the different architectures of AI/ML model.

Proposal 2: For the AI/ML model complexity, the following KPIs are selected

- The number and data type of parameters on AI/ML model

- The number of float point of operations for AI/ML model inference

Beam prediction for mTRPs

In mTRPs scenario, UE has the capability to receive two DL beams from two TRPs. The two DL beams transmitted for different directions simultaneously will improve the robustness of DL transmission. Since supporting mTRPs scenario is one of R16/R17 features, it is intuitive to extend the spatial-domain DL beam prediction to mTRPs scenario as well. In this section, we take the Case B described in [2] for the initial discussion of the spatial-domain DL beam prediction in mTRPs scenario.

In mTRPs scenario, each TRP configures the subset of RSs for beam management, UE measures the L1-RSRPs for each TRP and input them to the AI/ML model. With one AI/ML model, the optimal beam pairs for both TRPs are predicted simultaneously. The following figure shows the function of AI/ML model for mTRPs scenario.

Non-Measured beams

TRP1

TRP2

Panel 1

Panel 2

TX beam

RX beam

TX beam

RX beam

Measured beams

AI/ML

Model

Subset RSRPs of TRP1

All RSRPs of TRP1

Subset RSRPs of TRP2

All RSRPs of TRP2

Figure 3: the function of AI/ML model for mTRPs scenario

For the AI/ML model, the similar fully connected neural network with 4 layers shown as figure 2 is applied to mTRPs scenario with different number of nodes for input and output layer.

KPIs

In RAN#109 e-meeting, KPIs of EVM for spatial-domain DL beam prediction have been discussed and agreed in [1]. The intermediate KPIs include

- Average L1-RSRP difference of Top-1 predicted beam

- Beam prediction accuracy (%) for Top-1 and/or Top-K beams

- CDF of L1-RSRP difference for Top-1 predicted beam

- RS overhead reduction

For mTRPs scenario, the L1-RSRPs of all beam pairs for both TRPs will be output. In this case, the above intermediate KPIs can be used to evaluate the model performance for each TRP respectively.

Proposal 3: For spatial-domain DL beam prediction in mTPRs scenario, the following intermediate KPIs are suggested for each TRP respectively.

- Average L1-RSRP difference of Top-1 predicted beam

- Beam prediction accuracy (%) for Top-1 and/or Top-K beams

- CDF of L1-RSRP difference for Top-1 predicted beam

- RS overhead reduction

Simulation results

In our simulation, gNBs are assumed to be configured with 64 antenna elements which support 32 transmitting beams (4 beams in vertical and 8 beams in horizonal). UEs are configured with 2 panels and total 16 antenna elements which support 8 receiving beams (1 beam in vertical and 4 beams in horizonal for each panel). The details about the simulation parameters are shown in Table 4.

The samples of dataset generated by SLS are about 60k measurement results for total 256 beam pairs. 80% samples are used for model training and 20% samples are used for model testing. The AI/ML model is configured as a 4-layer fully connected neural network. The loss function of the network is MAE which gauges the differences between the predicted L1-RSRPs and the ground truth. The parameters about AI/ML training are shown in Table 5.

To evaluate the performance of AI/ML model for sTRP and mTRPs scenarios, the following KPIs are used

- Average L1-RSRP difference of Top-1 predicted beam

- Beam prediction accuracy (%) for Top-1 and/or Top-K beams (option 2)

- CDF of L1-RSRP difference for Top-1 predicted beam

- RS overhead reduction

  - Exhaustive beam sweeping is used as a baseline

- AI/ML model complexity

    - Evaluation results for sTRP scenario

For sTRP scenario, 16 transmitting beams and 4 receiving beams (total 64 beam pairs) are used to predict the L1-RSRPs of all the 256 beam pairs.

Beams in transmitter

Beams in receiver

Not measured beam

Measured beam

Panel 2

Panel 1

Figure 4: subset of beam pairs for sTRP scenario

With above evaluation scenario, the evaluation results are shown as follows.

Table 2: simulation results for sTRP scenario

|  | TOP1 | TOP2 | TOP4 | TOP8 |
| --- | --- | --- | --- | --- |
| Beam prediction accuracy (%) | 63.2 | 78.2 | 89.6 | 96.4 |
| Average L1-RSRP difference(dB) | 1.75 |  |  |  |

The CDF of L1-RSRP difference for Top-1 predicted beam is shown as following figure

Figure 5 CDF of L1-RSRP difference for sTRP scenario

Comparing to the exhaustive beam sweeping, total 256 beams pairs (32 transmitting beams and 8 receiving beams) are measured while the AI/ML method only measures 64 beam pairs. In this case, the RS overhead for beam sweeping is reduced 75%.

From the simulation results, it’s observed that

- The beam prediction accuracy of Top-2 is about 80%.

- The average L1-RSRP difference is less than 2 dB.

- The probability is about 70% with the estimated L1-RSRP difference for Top-1 predicted beam less than 2dB.

For the complexity of AI/ML model,

- The AI/ML model has total 214K parameters with single-float data type.

- The computational complexity of AI/ML model is about 213.4K float point of operations.

Observation 2:

For sTRP scenario,

- Comparing to the exhaustive beam sweeping, the performance of AI/ML model is

  - The RS overhead is reduced 75%.

  - The beam prediction accuracy of Top-2 is about 80%.

  - The average L1-RSRP difference is less than 2 dB.

  - The probability is about 70% with the estimated L1-RSRP difference for Top-1 predicted beam less than 2dB.

- For the complexity of AI/ML model,

  - The AI/ML model has total 214K parameters with single-float data type.

  - The computational complexity of AI/ML model is about 213.4K float point of operations.

    - Evaluation results for mTRPs scenario

For the spatial-domain DL beam prediction in mTRPs scenario, the Case B described in [2] is used to as the initial evaluation condition.

UEs are assumed to receive signal of 2 DL beams from 2 TRPs with 2 panels simultaneously. For each TRP, 8 transmitting beams and 2 receiving beams (total 16 beam pairs) are used to predict L1-RSRPs of all the 128 beam pairs.

Beams in transmitter of TRP#1

Beams in receiver of panel#1

Not measured beam

Measured beam

Figure 6: subset of beam pairs of each TRP for mTRP scenario

With above evaluation scenario, the evaluation results are shown as follows.

Table 3: simulation results for mTRPs scenario

|  | TRP #1 |  |  |  | TRP #2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TOP1 | TOP2 | TOP4 | TOP8 | TOP1 | TOP2 | TOP4 | TOP8 |
| Beam prediction accuracy (%) | 49.9 | 65.1 | 79.8 | 90.6 | 49 | 64.6 | 79 | 90.5 |
| Average L1-RSRP difference (dB) | 3.1 |  |  |  | 2.8 |  |  |  |

The CDF of L1-RSRP difference for Top-1 predicted beam is shown as following figure

Figure 7 CDF of L1-RSRP difference for mTRP scenarios

Comparing to the exhaustive beam sweeping, total 128 beams pairs (32 transmitting beams and 4 receiving beams) are measured for each TRP while the AI/ML method only measures 16 beam pairs. In this case, the RS overhead for beam sweeping is reduced 87.5%.

From the simulation results, it’s observed that

- The beam prediction accuracy of Top-4 is about 80% for both TRPs.

- The average L1-RSRP difference is about 3 dB for both TPRs.

- The probability is about 70% with the estimated L1-RSRP difference for Top-1 predicted beam less than 4 dB for both TRPs.

For the complexity of AI/ML model,

- The AI/ML model has total 312K parameters with single-float data type.

- The computational complexity of AI/ML model is about 311.7K float point of operations.

Observation 3:

For mTRPs scenario,

- Comparing to the exhaustive beam sweeping, the performance of AI/ML model is

  - The RS overhead is reduced 87.5%.

  - The beam prediction accuracy of Top-4 is about 80% for both TRPs.

  - The average L1-RSRP difference is about 3 dB for both TPRs.

  - The probability is about 70% with the estimated L1-RSRP difference for Top-1 predicted beam less than 4 dB for both TRPs.

- For the complexity of AI/ML model,

  - The AI/ML model has total 312K parameters with single-float data type.

  - The computational complexity of AI/ML model is about 311.7K float point of operations.

- Conclusion

In this contribution, we present our views and evaluation results of beam prediction on AL/ML beam management. For the discussion, we have the following proposals and observation.

Proposal 1: Considering beam sweeping with two steps, the option 2 is selected as the KPI of beam prediction accuracy.

- The beam prediction accuracy (%) is the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

Observation 1:

- The number and data type of parameters can indicate the cost of memory storage for the AI/ML model.

- The number of float point of operations can indicate the computational complexity for the different architectures of AI/ML model.

Proposal 2: For the AI/ML model complexity, the following KPIs are selected

- The number and data type of parameters on AI/ML model

- The number of float point of operations for AI/ML model inference

Proposal 3: For spatial-domain DL beam prediction in mTPRs scenario, the following intermediate KPIs are suggested for each TRP respectively.

- Average L1-RSRP difference of Top-1 predicted beam

- Beam prediction accuracy (%) for Top-1 and/or Top-K beams

- CDF of L1-RSRP difference for Top-1 predicted beam

- RS overhead reduction

Observation 2:

For sTRP scenario,

- Comparing to the exhaustive beam sweeping, the performance of AI/ML model is

  - The RS overhead is reduced 75%.

  - The beam prediction accuracy of Top-2 is about 80%.

  - The average L1-RSRP difference is less than 2 dB.

  - The probability is about 70% with the estimated L1-RSRP difference for Top-1 predicted beam less than 2dB.

- For the complexity of AI/ML model,

  - The AI/ML model has total 214K parameters with single-float data type.

  - The computational complexity of AI/ML model is about 213.4K float point of operations.

Observation 3:

For mTRPs scenario,

- Comparing to the exhaustive beam sweeping, the performance of AI/ML model is

  - The RS overhead is reduced 87.5%.

  - The beam prediction accuracy of Top-4 is about 80% for both TRPs.

  - The average L1-RSRP difference is about 3 dB for both TPRs.

  - The probability is about 70% with the estimated L1-RSRP difference for Top-1 predicted beam less than 4 dB for both TRPs.

- For the complexity of AI/ML model,

  - The AI/ML model has total 312K parameters with single-float data type.

  - The computational complexity of AI/ML model is about 311.7K float point of operations.

- Appendix

Table 4:  parameters of SLS for spatial-domain DL beam prediction

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz - SCS: 120 kHz |
| Deployment | 200m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | - For spatial-domain beam prediction, 3km/h |
| UE distribution | - 10 UEs per sector/cell for evaluation. - 80% indoor ,20% outdoor as in TR 38.901 |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration |  One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ Azimuth angle (degree) = [-78.75, -56.25, -33.75, -11.25, 11.25, 33.75, 56.25,78.75] Zenith angle (degree) = [22.5, 67.5,112.5, 157.5] Total 32 beams = 8(H)*4(V), DFT beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Panel structure: (M, N, P) = (1,4,2)  2 panels (left, right) with (Mg, Ng) = (1, 2) Azimuth angle (degree) = [22.5, 67.5, -67.5, -22.5] Total 8 beams = 4(H)*1(V)*2(panels), DFT beams |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| BS Tx Power | 40 dBm |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

Table 5:  parameters of AI/ML model training

| Parameter | Value |
| --- | --- |
| Test samples | 12600 |
| Training samples | 50400 |
| Batch-size | 32 |
| Initial learning rate | 1.00E-03 |
| Epoch | 100 |
| Optimizer | Adam |
| Lr adjust schedule | Warm up + cosine annealing |
| Loss function | MAE |

- Reference

# [1] Draft report of 3GPP TSG RAN WG1 #109 v0.3.0, E-meeting, May 9th-20th, 2022.

# [2] R1-2206167, “Sub use cases and specification impact on AI/ML for beam management”, Fujitsu.