3GPP TSG RAN WG1 #110	R1-2206174

Toulouse, France, August 22nd – 26th, 2022

Agenda item:	9.8.1

Source: 	Fujitsu

Title:	Discussion on side control information for NR network-controlled repeaters

Document for:		Discussion and decision

- Introduction

In this contribution, we share some discussions on side control information for NCR based on the current agreements [1].

- Discussion

  - Modelling of NCR

In RAN1#109-e meeting, the following agreements were achieved for modelling and terminology of NCR.

Agreement Capture the following model of network-controlled repeater in TR 38.867. - The NCR-MT is defined as a function entity to communicate with a gNB via Control link (C-link) to enable the information exchanges (e.g. side control information). The C-link is based on NR Uu interface. - Note: Side control information is at least for the control of NCR-Fwd - The NCR-Fwd is defined as a function entity to perform the amplify-and-forwarding of UL/DL RF signal between gNB and UE via backhaul link and access link. The behavior of the NCR-Fwd will be controlled according to the received side control information from gNB. Agreement Capture the following assumption of network-controlled repeater in TR 38.867. - At least one of the NCR-MT’s carrier(s) should be within the set of carriers forwarded by the NCR-Fwd in same frequency range. - The NCR-MT and NCR-Fwd operating in the same carrier is prioritized for the study.

In the second agreement, according to the discussions during the meeting, the possibility of multiple carriers for NCR-MT is remained due to company’s interests in out-of-band NCR of which some NCR-MT’s carrier is out of the set of carriers forwarded by NCR-Fwd. However, in RAN#96 meeting, whether to support out-of-band NCR in Rel-18 was discussed, and the conclusion is: “RAN1 study will focus on in-band only”. Therefore, at least in Rel-18, the carrier of NCR-MT and the carrier of NCR-Fwd should be in a same band, and it does not need to support multiple carriers for NCR-MT for the case of out-of-band NCR. In addition, since there is no user-plane data exchanged between gNB and NCR-MT and the traffic volume for side control information is small, it is also unnecessary to support multiple carriers for NCR-MT from data exchange point of view.

Observation 1:

- In Rel-18, RAN1 study will focus on in-band only. It is unnecessary to support multiple carriers for NCR-MT to enable out-of-band operation.

- There is no user-plane data exchanged between gNB and NCR-MT and the traffic volume for side control information is small, it is also unnecessary to support multiple carriers for NCR-MT for data exchange between gNB and NCR-MT.

Based on the discussions above, we think a single carrier for NCR-MT is enough for in-band NCR. Following the spirit of the study item that cost efficiency a key consideration point for NCR, we make the following proposal.

Proposal 1: In Rel-18, NCR-MT only has a single carrier, and the carrier is within the set of carriers of NCR-Fwd in same frequency band.

  - Beam information

    - Beam information for backhaul link

In RAN1#109-e meeting, the following agreements were achieved for beam management for control link and backhaul link, with some FFS points.

Agreement Both fixed beam and adaptive beam can be considered at NCR for both C-link and backhaul-link. - FFS: the mechanism for indication and determination of beam. - Note: Fixed beam refers to the case that beam at NCR for both C-link and backhaul-link cannot be changed. Agreement Capture the following assumption of network-controlled repeater in TR 38.867. - As baseline, same large-scale properties of the channel, i.e., channel properties in Type-A and Type-D (if applicable), are expected to be experienced by C-link and backhaul link (at least when the NCR-MT and NCR-Fwd operating in same carrier). Agreement As baseline, the same TCI states as C-link are assumed for beam at NCR-Fwd for backhaul link if the NCR-MT’s carrier(s) is within the set of carriers forwarded by the NCR-Fwd. - FFS: additional indication from gNB to determine the beam at NCR-Fwd for backhaul link or implicit determination of the beam at NCR-Fwd for backhaul link Note: the same assumption of the beam correspondence is applied for DL/UL of the backhaul link at NCR-Fwd as the DL/UL of the C-link at NCR-MT.

As agreed in the first agreement, both fixed beam and adaptive beam can be considered for control link and backhaul link, and fixed beam refers to the case that beam at NCR for both C-link and backhaul-link cannot be changed. It is obvious that fixed beam would limit the deployment scenarios for NCR and adaptive beam has more flexibility.

In our view, there are some cases where beam switching for control link and backhaul link is necessary.

First, FR2 deployment is prioritized for NCR according to the SID [2]. Meanwhile, as discussed in 2.2, only in-band case is considered, which means control link and backhaul link of NCR are in a same frequency band in FR2. For the high frequency range, there is high risk of beam blockage between gNB and NCR. In order to guarantee stable connection between the gNB and NCR, it is necessary to support adaptive beam for control link and backhaul link NCR.

Second, although the SID [2] confines that the NCR is stationary, it is still open for discussion whether the NCR can change the serving cell for NCR-MT and/or the target cells for NCR-Fwd’ amplify-and-forwarding. For example, change of the serving cell for NCR-MT and/or the target cells for NCR-Fwd’ amplify-and-forwarding may be due to cell on-off for network energy saving or mismatching between the serving cell of NCR-MT and target cells for NCR-Fwd’ amplify-and-forwarding (e.g. the carrier of the serving cell of NCR-MT is not within the set of carriers of target cells for NCR-Fwd’ amplify-and-forwarding). If change of the serving cell for NCR-MT and/or the target cells for NCR-Fwd’ amplify-and-forwarding is supported, adaptive beam is necessary, because different beams may be used for NCR-MT’s communicating with different cells or NCR-Fwd’s amplify-and-forwarding for different target cells.

According to the discussions above, it is highly expected that NCR can support adaptive beam for control link and backhaul link. However, we also see that NCR with fixed beam for the links costs less and can work well in some certain scenarios. With that in mind, we suggest supporting both fixed beam and adaptive beam. It can be up to implementation to choose either one.

Proposal 2: Support both fixed beam and adaptive beam at NCR for both control link and backhaul link.

In the second agreement, the relation between control link and backhaul link is clarified. However, it is just a baseline and says at least for the case of same carrier between NCR-MT and NCR-Fwd. Since RAN#96 meeting has concluded in-band only for NCR study, we think RAN1 can conclude that same large-scale properties of the channel, i.e., channel properties in Type-A and Type-D (if applicable), are expected to be experienced by control link and backhaul link of Rel-18 NCR. Therefore, we suggest making the following clarification for Rel-18 NCR.

Proposal 3: Same large-scale properties of the channel, i.e., channel properties in Type-A and Type-D (if applicable), are expected to be experienced by control link and backhaul link of Rel-18 NCR.

- When NCR-MT is receiving/transmitting DL/UL signals, same beam as control link is assumed for backhaul link if NCR-Fwd is forwarding signals.

Another thing is about how to indicate/determine beam for control link and backhaul link for the case of adaptive beam. Regarding to the beam for control link, it is straightforward that the legacy methods for UEs can be used. Regarding to the beam for backhaul link, although the baseline is to use the same beam as control link, it is still FFS whether to support additional indication or implicit method.

In our understanding, further discussion on the FFS may need to consider how the beam for control link is indicated/determined.

For example, if the beam for control link is indicated/determined by the method in Rel-15/Rel-16, the following options can be considered.

- Option 1: When NCR-MT is receiving/transmitting DL/UL signals, same beam is assumed for backhaul link. When NCR-MT is not receiving/transmitting DL/UL signals, a default beam is assumed for backhaul link. For example, the default beam can be the beam for CORESET#0.

- Option 2: Support separate indication of beam for backhaul link. Regardless of whether NCR-MT is receiving/transmitting DL/UL signals, backhaul link uses the beam indicated by its corresponding indication. gNB can guarantee same beam for control link and backhaul link when NCR-MT is receiving/transmitting DL/UL signals.

If the beam for control link is indicated/determined by the method in Rel-17 with unified TCI framework, besides the options above, the following options can also be considered.

- Option 3: When NCR-MT is receiving/transmitting DL/UL signals, same beam is assumed for backhaul link. When NCR-MT is not receiving/transmitting DL/UL signals, same unified TCI state as indicated for control link is assumed for the beam for backhaul link.

Observation 2: The following options can be considered to indicate/determine beam for backhaul link.

- Option 1: When NCR-MT is receiving/transmitting DL/UL signals, same beam is assumed for backhaul link. When NCR-MT is not receiving/transmitting DL/UL signals, a default beam is assumed for backhaul link. For example, the default beam can be the beam for CORESET#0.

- Option 2: Support separate indication of beam for backhaul link. Regardless of whether NCR-MT is receiving/transmitting DL/UL signals, backhaul link uses the beam indicated by its corresponding indication. gNB can guarantee same beam for control link and backhaul link when NCR-MT is receiving/transmitting DL/UL signals.

- Option 3: When NCR-MT is receiving/transmitting DL/UL signals, same beam is assumed for backhaul link. When NCR-MT is not receiving/transmitting DL/UL signals, same unified TCI state as indicated for control link is assumed for the beam for backhaul link (assuming the beam for control link is indicated/determined by the method in Rel-17 with unified TCI framework).

    - Beam information for access link

In RAN1#109-e meeting, the following agreements were achieved for beam management for control access link, with some FFS points.

Agreement Both the dynamic indication and semi-static indication can be considered for the beam of access link for NCR-Fwd. - FFS: the details of each indication - FFS: the maximum number of beams configured for NCR-Fwd access link Agreement From the perspective of signaling design, following mechanisms can be considered for the access link beamforming of the NCR-Fwd. - Option #2-1: Dynamic beam indication only - Option #2-2: Semi-static beam indication only Option #2-3: Dynamic beam indication and semi-static beam indication Agreement In the access link beam indication, an access link beam can be indicated by: - Option 1: A beam index - FFS: How to indicate the corresponding time domain resource of the beam. - Option 2: An index of a source RS (e.g. a TCI-like indicator) - FFS: The definition of the source RS. - FFS: How to indicate the corresponding time domain resource of the beam. - FFS: The definition of the association between the source RS and the beam. - Note: The above does not imply that the NCR can or cannot generate and transmit reference signals to a UE or receive and process reference signals from a UE. RAN1 to select one of the two options, combine the two options, or select both options in RAN1#110 Agreement The beam correspondence is assumed for: - the DL/UL of the access link at NCR-Fwd Agreement As for the time-domain granularity of the access link beam indication, one or both of the following options can be considered: - Option 1: slot-level - Option 2: symbol-level - FFS: The details of indication signaling Agreement The time at which the NCR applies an access link beam indication should be considered.

According to the first agreement, both dynamic indication and semi-static indication can be considered for the beam of access link. In the legacy, dynamic scheduling is supported. For a specific slot/symbols, gNB may dynamically schedule a DL/UL transmission (e.g. PDSCH/PUSCH) and determine which DL/UL beam should be used for the transmission. Introduction of NCR should not limit gNB’s scheduling flexibility. From NCR’s perspective, it should support dynamically change of beam for access link, and at least dynamic indication for beam of access link should be supported.

However, dynamic indication only for all cases may result in large signalling overhead. Considering that, it is beneficial to support semi-static indication as well. For example, semi-static indication can be used to indicate the beam of access link for forwarding SSB, SIB1 and PRACH etc.

Proposal 4: For beam indication for access link, support both dynamic indication and semi-static indication.

According to the third agreement, RAN1 will select the options in RAN1#110 meeting. We would like to share our consideration on the options.

In our understanding, for both options, association between source RS and the beam of access link is necessary for beam management for the UEs served by the NCR. The difference between Option 1 and Option 2 is whether the source RS associated with a beam of access link is visible to the NCR.

By Option 1, the gNB simply indicates the beam index of beam for access link to the NCR, and the association between the source RS and the beam (identified by beam index) can be maintained/updated by gNB and totally transparent to NCR.

By Option 2, the gNB would indicate the index of source RS associated with the beam for access link, hence the source RS associated with the beam for access link should be visible to the NCR, i.e. NCR should know which source RS is associated with which beam of access link. And additional configuration information for the source RS associated with the beam may be needed.

From NCR’s point of view, if it can determine which beam of access link should be used at which time, it can operate properly. With that said, it is redundant for NCR’s proper operation to be informed of source RS associated with the beam of access link. Compared with Option 2, Option 1 only provides least necessary information without redundant information, so it can achieve lower signalling overhead.

In addition, as noted by the SID [2], cost efficiency is a key consideration point for NCR. According to the discussion above, compared with Option1, Option 2 will increase the complexity of NCR due to obtaining and maintaining configurations of source RS for forwarding and the association between the source RS and the beam of access link. Option 1 is preferred considering cost efficiency of NCR. Besides, as indicated by the first and third FFS points under Option 2, Option 2 may require more standardization effort than Option 1 and thus Option 1 is also preferred from time budget point of view.

Based on the discussions above, Option 1 is preferred from perspectives of signalling overhead, cost efficiency for NCR, and time budget of standardization for Rel-18 NCR.

Observation 3: The key difference between Option 1 and Option 2 is whether the source RS associated with a beam of access link is visible to the NCR.

Observation 4: NCR can operate properly if it can determine which beam of access link should be used at which time. The knowledge about source RS associated with the beam of access link is redundant for NCR.

Observation 5: Compared with Option 1, Option 2 may require additional configuration information for the source RS associated with the beam, increase the complexity of NCR due to obtaining and maintaining configurations of source RS for forwarding and the association between the source RS and the beam of access link, and cost more extra standardization efforts.

Observation 6：For beam indication for access link, Option 1 (i.e. by a beam index) is preferred from perspectives of signalling overhead, cost efficiency for NCR, and time budget of standardization for Rel-18 NCR.

Proposal 5: For beam indication for access link, an access link beam is indicated by a beam index (Option 1).

According to the fourth agreement, beam correspondence is assumed for the DL/UL of the access link. This assumption can simplify the beam refinement between the NCR and the UEs which the NCR serves. It also means that in the access link beam indications from the gNB to the NCR, DL beam (i.e. Tx beam) and UL beam (i.e. Rx beam) can share same indexes.

Proposal 6: Support beam indication with a common beam index for DL/Rx beam and UL/Tx beam of access link.

According to the fifth agreement, slot-level (Option1) and symbol-level (Option 2) of access link beam indication can be considered.

In the legacy, gNB may switch beam within a slot and UEs are also able to switch beam within a slot. For example, in a slot, gNB may schedule PDSCHs for different UEs using different beams or for a same UE but using different beams. Again, introduction of NCR should not limit the flexibility of gNB as in the legacy. From this perspective, at least symbol-level (Option2) indication should be supported.

Similar to dynamic indication, symbol-level (Option2) indication only for all cases may result in large signalling overhead. Considering that, it is beneficial to support slot-level (Option1) indication as well to reduce signalling overhead for some cases.

Proposal 7: For the time-domain granularity of access link beam indication, support both slot-level (Option 1) and symbol-level (Option 2).

  - TDD configuration

In RAN1#109-e meeting, the following agreements were achieved for TDD configuration, with some FFS points.

Agreement For the TDD UL/DL configuration of network controller repeater: - At least semi-static TDD UL/DL configuration is needed for network-controlled repeater for links including C-link, backhaul link and access link. - FFS: handling of flexible symbols - Note1: The same TDD UL/DL configuration is always assumed for backhaul link and access link - Note2: The same TDD UL/DL configuration is assumed for C-link and backhaul link and access link if NCR-MT and NCR-Fwd are in the same frequency band. Agreement For the signaling of information on UL-DL TDD configuration, if the NCR-MT can acquire the TDD configuration as legacy UEs or from the OAM, new signaling may not be necessary. - Note 1: The same TDD UL/DL configuration is assumed for C-link and backhaul link and access link if the NCR-MT and the NCR-Fwd are in the same frequency band. - FFS: Other cases where new signaling may be necessary.

According to the agreements, same TDD configuration is assumed for control link, backhaul link and access link if the NCR-MT and the NCR-Fwd are in the same frequency band. And as discussed in 2.1, according to conclusion of RAN#96 meeting, RAN1 study should focus on in-band only, which means NCR-MT and NCR-Fwd of Rel-18 NCR are in a same frequency band. Therefore, we can conclude that same TDD configuration is assumed for control link, backhaul link and access link in Rel-18 and we do not need to discuss other cases.

Observation 7: RAN#96 meeting has concluded that RAN1 study will focus on in-band only, so it is not needed to consider other cases (out-of-band) for discussion on signalling for TDD configuration.

Proposal 8: For TDD UL/UL configuration, remove the FFS in the agreement:

Agreement (RAN1#109-e) For the signaling of information on UL-DL TDD configuration, if the NCR-MT can acquire the TDD configuration as legacy UEs or from the OAM, new signaling may not be necessary. - Note 1: The same TDD UL/DL configuration is assumed for C-link and backhaul link and access link if the NCR-MT and the NCR-Fwd are in the same frequency band. - FFS: Other cases where new signaling may be necessary.

Regarding signalling for TDD configuration, we think signalling dedicated for NCR-Fwd (backhaul link and access link) is not needed. Although the agreement says, “if the NCR-MT can acquire the TDD configuration as legacy UEs or from the OAM”, according to the discussion in the previous meeting, we do not see that case where the NCR-MT cannot acquire TDD configuration by the ways. NCR-Fwd can always assume the same TDD configuration as NCR-MT.

Proposal 9: For TDD UL/DL configuration, new signalling dedicated for NCR-Fwd is not needed.

According to the agreements, it has been agreed that at least semi-static TDD UL/DL configuration is needed for NCR. And we think dynamic TDD configuration should be supported as well. The dynamic TDD configuration can be supported by the legacy method based on DCI format 2_0. It will cost a little or no standardization effort and can guarantee same scheduling flexibility as in legacy. For example, if gNB dynamically schedule PDSCH/PUSCH for UEs served by the NCR on the flexible symbols configured by semi-static TDD configuration for NCR-MT, the gNB can overwrite the flexible symbols to be DL/UL by DCI format 2_0, so that NCR-Fwd can properly perform DL/UL forwarding for the scheduled PDSCH/PUSCH according to the dynamically indicated direction. In this way, for any symbols/slots with DL/UL transmission between gNB and UEs, corresponding transmission direction can be indicated to NCR and NCR-Fwd can amplify and forward accordingly. For the remaining flexible symbols according to both semi-static TDD configuration and dynamic TDD configuration, NCR-Fwd can assume no DL/UL transmission between gNB and UEs and does not amplify and forward signals.

Proposal 10: Same TDD UL/DL configuration is assumed for NCR-MT and NCR-Fwd.

- NCR-MT acquires semi-static TDD UL/DL configuration by RRC signaling and dynamic TDD UL/DL configuration by DCI format 2_0 as in the legacy.

- For the remaining flexible symbols according to both semi-static TDD UL/DL configuration and dynamic TDD UL/DL configuration, NCR-MT transmits or receives on the flexible symbols as legacy UEs, and NCR-Fwd assumes no DL/UL transmission between gNB and UEs served by the NCR and does not amplify and forward signals.

  - ON-OFF information

Application of ON-OFF information is useful for improving NCR performance in terms of power consumption and interference control. In the RAN1#109-e meeting, the following agreements were made.

Agreement ON-OFF information is beneficial and recommended for network-controlled repeater to control the behaviour of NCR-Fwd. - FFS: Detailed mechanism of ON-OFF indication and determination - FFS: explicit indication or implicit indication of ON-OFF information Agreement The following options can be considered to indicate the ON-OFF information from gNB to NCR for controlling the behaviour of NCR-Fwd: - Option 1: Explicit indication with on-off state (e.g., via dynamic or semi-static signalling) or on-off pattern (e.g., periodic/semi-static ON-OFF pattern or new DRX-like pattern for ON-OFF) - Option 2: Implicit indication via the signalling for other information (e.g., beam, DL/UL configuration, or PC information) - Note: This example does not imply that PC information is necessary or not. - Other solutions (e.g., potential combination of explicit and implication solution) can be further discussed. Agreement For indication of NCR-Fwd ON-OFF for efficient interference management and improved energy efficiency, both dynamic and semi-static indication can be considered - FFS: RAN1 to consider whether/how to handle the forwarding of broadcast and cell-specific signals/channels.

According to the agreements, both explicit indication and implicit indication can be considered. And both dynamic indication and semi-static indication can be considered. According to the discussion in the previous meeting, there are so many candidate methods in details. It is expected to further narrow down the discussion scope in RAN1#110 for lighter workload in the future.

It is natural that the NCR should be ON if there is DL/UL transmission between the gNB and the UEs served by the NCR and should be OFF otherwise to save power and reduce interference as much as possible. With that in mind, if other information can perfectly match with actual DL/UL transmissions between the gNB and the UEs served by the NCR, NCR-Fwd could just turn ON/OFF according to the other information and explicit indication would be unnecessary. Therefore, we would like to analyze Option 2 first.

In Option 2, beam information, TDD UL/DL configuration, and PC information are considered to implicitly indicate ON/OFF. In our understanding, the beam information refers to the beam indication for access link. As discussed in 2.2.2, beam indication for access link can be semi-static and/or dynamic, and symbol-level and/or slot-level. If beam indication for access link is semi-static only and/or has coarse granularity, it may well not perfectly match the actual transmission between gNB and UEs. For example, for several symbols/slots semi-statically configured with beam for access link, there may not be actual transmissions between gNB and UEs and the NCR-Fwd is expected to be OFF. And if beam indication for access link can be dynamic and has fine granularity, it is probably fine to implicitly indicate ON-OFF. Similar things happen to TDD UL/DL configuration and power control information.

As discussed above, whether implicit indication by other information can work well depends on how the signaling for other information is designed. It may be difficult to down select the options before the more progress on other information. However, the discussions on the ON-OFF information and other information should be parallel due to limited time budget.

Considering the situation, it is preferred to support combination of Option 1 and Option 2. For example, the NCR-Fwd is ON/OFF according to explicit ON-OFF information, except that NCR-Fwd is always OFF for flexible symbols/slots according to TDD UL/DL configuration. Another example is, NCR-Fwd turns ON/OFF according to explicit ON-OFF information if it is provided, and according to other information otherwise.

Observation 8：For ON-OFF information:

- Implicit indication by other information can save signalling overhead. If implicit indication can totally match the actual transmission state between the gNB and UEs, it is unnecessary to support explicit ON-OFF information.

- Whether it is necessary to introduce explicit ON-OFF information depends on the signalling design of the other information. It may be difficult to down select the options before the more progress on other information. However, discussions on the ON-OFF information and other information should be parallel due to limited time budget.

Proposal 11: For ON-OFF information, support both explicit indication (Option 1) and implicit indication (Option 2).

Another thing is about forwarding of broadcast and cell-specific signals/channels. Following the spirit mentioned above, the NCR-Fwd should be ON for the corresponding symbols/slots to forward broadcast and cell-specific signals/channels between gNB and UEs. In our understanding, no matter how the NCR acquires ON-OFF information (explicit/implicit, semi-static/dynamic), the gNB could guarantee that all symbols/slots for all broadcast and cell-specific signals/channels are configured as ON for NCR-Fwd.

Proposal 12: For broadcast and cell-specific signals/channels, it is up to gNB to guarantee that all symbols/slots for all broadcast and cell-specific signals/channels are configured as ON for NCR-Fwd.

  - Timing information

The agreements about timing of NCR were achieved in RAN1#109-e meeting as below.

Agreement For the timing of NCR, the following assumption is considered as baseline: - The DL receiving timing of the NCR-Fwd is aligned with the DL receiving timing of the NCR-MT. - The UL transmitting timing of the NCR-Fwd is aligned with the UL transmitting timing of the NCR-MT. - FFS: the impact of internal delay on the following timing relationships: - The DL receiving timing and DL transmitting timing of the NCR-Fwd - The UL transmitting timing and UL receiving timing of the NCR-Fwd Conclusion Legacy UE mechanism is sufficient to achieve DL/UL timing for NCR-MT Agreement For the signaling of the side control information of timing to align transmission / reception boundaries, new signaling may be unnecessary. - FFS: the impact of internal delay

According to the agreements, the impact of internal delay needs to be further studied.

In our understanding, from NCR’s perspective, it may need to determine its DL transmitting timing and UL receiving timing of NCR-Fwd for beam switching and/or ON-OFF etc. However, it can determine the DL transmitting timing and UL receiving timing of NCR-Fwd according to its internal delay and corresponding DL receiving timing and UL transmitting timing. More specifically, as shown in Figure 1, the DL transmitting timing can be determined according to DL receiving timing and the internal delay, and the UL receiving timing can be determined according to UL transmitting timing and the internal delay. From gNB’s and UE’s perspective, the internal delay is a part of propagation delay. It has impact on TA value. However, similar to the case where legacy RF repeater is deployment, the delay caused by NCR can be handled by the legacy method of transmission timing (or TA) adjustments at UEs. The internal delay of NCR is transparent to gNB and UE and has no impact on signalling.

Figure 1. Timing relationships in case of NCR deployment

Based on the discussions above, we have the following proposal.

Proposal 13: The NCR can determine DL transmission timing and UL receiving timing of NCR-Fwd according to internal delay and respective corresponding DL receiving timing and UL transmitting timing.

- It is up to NCR’s implementation. No spec. impacts.

- Remove the FFS in the agreement as below.

Agreement (RAN1#109-e) For the signaling of the side control information of timing to align transmission / reception boundaries, new signaling may be unnecessary. - FFS: the impact of internal delay

  - Power control information

In the RAN1#109-e meeting, the following agreements were made for power control of NCR-Fwd.

Agreement Recommend to capture the following observation in TR 38.867: - The benefits of power information used to control the behavior of NCR-Fwd for the DL of access link and/or UL of backhaul link are observed by the following inputs: - [Source-1, Huawei] shows that for the uplink transmission via NCR, a fixed NCR amplifying gain may lead to interference to the gNB or NCR UL coverage loss. For the downlink transmission via NCR, a fixed NCR amplifying gain may lead to NCR RU saturation or NCR DL coverage loss. - [Source-2, vivo] shows that the optimal system performance can be achieved when repeater’s gain is set to a proper value. - [Source-3, ETRI] shows that dynamic repeater gain/power control can provide additional SINR gain over semi-static repeater gain/power configuration. - [Source-4, Ericsson] mentions that the gain control is needed for self-interference management due to repeater oscillation. - This agreement does not change the prioritization of PC Agreement The controlling of the amplifying gain of NCR-Fwd is considered to enable the power control of NCR-Fwd if PC is recommended as side control information for NCR in Rel-18 - FFS: Controlling of the transmission power of NCR-Fwd

The thermal noise and signal distortion due to existing repeater systems lead to interference for the receiver, which degrades the network performance. For NCR-Fwd, a dynamic power control can reduce the interference. In addition, operation at full power all the time is not an appropriate operation in terms of power consumption. Power control can improve the network performance in terms of reducing interference, avoiding saturation, and guaranteeing the NCR coverage.

Although legacy RF repeater may have some mechanisms for adaptively adjusting transmission power, it seems not mandatory and just up to implementation. Therefore, from gNB/network point of view, since it has no knowledge whether/how the RF repeater adjusting transmission power, it is difficult to estimate or manage the interference caused by the RF repeater. Now, with respect to NCR, if power control information is supported, it will be helpful for the gNB/network manage the interference.

According to the discussions above, we suggest to support power control information for controlling of transmission power of NCR-Fwd. In order to reduce the spec. impact and standardization effort, semi-static indication of amplification gain is preferred.

Proposal 14: Support power control information for controlling of transmission power of NCR-Fwd.

- At least semi-static indication of amplification gain is supported.

- Conclusion

In this contribution, we discussed on side control information for NCR based on the current agreements. The proposals are summarized as follows.

Modelling of NCR

Observation 1:

- In Rel-18, RAN1 study will focus on in-band only. It is unnecessary to support multiple carriers for NCR-MT to enable out-of-band operation.

- There is no user-plane data exchanged between gNB and NCR-MT and the traffic volume for side control information is small, it is also unnecessary to support multiple carriers for NCR-MT for data exchange between gNB and NCR-MT.

Proposal 1: In Rel-18, NCR-MT only has a single carrier, and the carrier is within the set of carriers of NCR-Fwd in same frequency band.

Beam information for backhaul link

Proposal 2: Support both fixed beam and adaptive beam at NCR for both control link and backhaul link.

Proposal 3: Same large-scale properties of the channel, i.e., channel properties in Type-A and Type-D (if applicable), are expected to be experienced by control link and backhaul link of Rel-18 NCR.

- When NCR-MT is receiving/transmitting DL/UL signals, same beam as control link is assumed for backhaul link if NCR-Fwd is forwarding signals.

Observation 2: The following options can be considered to indicate/determine beam for backhaul link.

- Option 1: When NCR-MT is receiving/transmitting DL/UL signals, same beam is assumed for backhaul link. When NCR-MT is not receiving/transmitting DL/UL signals, a default beam is assumed for backhaul link. For example, the default beam can be the beam for CORESET#0.

- Option 2: Support separate indication of beam for backhaul link. Regardless of whether NCR-MT is receiving/transmitting DL/UL signals, backhaul link uses the beam indicated by its corresponding indication. gNB can guarantee same beam for control link and backhaul link when NCR-MT is receiving/transmitting DL/UL signals.

- Option 3: When NCR-MT is receiving/transmitting DL/UL signals, same beam is assumed for backhaul link. When NCR-MT is not receiving/transmitting DL/UL signals, same unified TCI state as indicated for control link is assumed for the beam for backhaul link (assuming the beam for control link is indicated/determined by the method in Rel-17 with unified TCI framework).

Beam information for access link

Proposal 4: For beam indication for access link, support both dynamic indication and semi-static indication.

Observation 3: The key difference between Option 1 and Option 2 is whether the source RS associated with a beam of access link is visible to the NCR.

Observation 4: NCR can operate properly if it can determine which beam of access link should be used at which time. The knowledge about source RS associated with the beam of access link is redundant for NCR.

Observation 5: Compared with Option 1, Option 2 may require additional configuration information for the source RS associated with the beam, increase the complexity of NCR due to obtaining and maintaining configurations of source RS for forwarding and the association between the source RS and the beam of access link, and cost more extra standardization efforts.

Observation 6：For beam indication for access link, Option 1 (i.e. by a beam index) is preferred from perspectives of signalling overhead, cost efficiency for NCR, and time budget of standardization for Rel-18 NCR.

Proposal 5: For beam indication for access link, an access link beam is indicated by a beam index (Option 1).

Proposal 6: Support beam indication with a common beam index for DL/Rx beam and UL/Tx beam of access link.

Proposal 7: For the time-domain granularity of access link beam indication, support both slot-level (Option 1) and symbol-level (Option 2).

TDD configuration

Observation 7: RAN#96 meeting has concluded that RAN1 study will focus on in-band only, so it is not needed to consider other cases (out-of-band) for discussion on signalling for TDD configuration.

Proposal 8: For TDD UL/UL configuration, remove the FFS in the agreement:

Agreement (RAN1#109-e) For the signaling of information on UL-DL TDD configuration, if the NCR-MT can acquire the TDD configuration as legacy UEs or from the OAM, new signaling may not be necessary. - Note 1: The same TDD UL/DL configuration is assumed for C-link and backhaul link and access link if the NCR-MT and the NCR-Fwd are in the same frequency band. - FFS: Other cases where new signaling may be necessary.

Proposal 9: For TDD UL/DL configuration, new signalling dedicated for NCR-Fwd is not needed.

Proposal 10: Same TDD UL/DL configuration is assumed for NCR-MT and NCR-Fwd.

- NCR-MT acquires semi-static TDD UL/DL configuration by RRC signaling and dynamic TDD UL/DL configuration by DCI format 2_0 as in the legacy.

- For the remaining flexible symbols according to both semi-static TDD UL/DL configuration and dynamic TDD UL/DL configuration, NCR-MT transmits or receives on the flexible symbols as legacy UEs, and NCR-Fwd assumes no DL/UL transmission between gNB and UEs served by the NCR and does not amplify and forward signals.

ON-OFF information

Observation 8：For ON-OFF information:

- Implicit indication by other information can save signalling overhead. If implicit indication can totally match the actual transmission state between the gNB and UEs, it is unnecessary to support explicit ON-OFF information.

- Whether it is necessary to introduce explicit ON-OFF information depends on the signalling design of the other information. It may be difficult to down select the options before the more progress on other information. However, discussions on the ON-OFF information and other information should be parallel due to limited time budget.

Proposal 11: For ON-OFF information, support both explicit indication (Option 1) and implicit indication (Option 2).

Proposal 12: For broadcast and cell-specific signals/channels, it is up to gNB to guarantee that all symbols/slots for all broadcast and cell-specific signals/channels are configured as ON for NCR-Fwd.

Timing information

Proposal 13: The NCR can determine DL transmission timing and UL receiving timing of NCR-Fwd according to internal delay and respective corresponding DL receiving timing and UL transmitting timing.

- It is up to NCR’s implementation. No spec. impacts.

- Remove the FFS in the agreement as below.

Agreement (RAN1#109-e) For the signaling of the side control information of timing to align transmission / reception boundaries, new signaling may be unnecessary. - FFS: the impact of internal delay

Power control information

Proposal 14: Support power control information for controlling of transmission power of NCR-Fwd.

- At least semi-static indication of amplification gain is supported.

References

[1] Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0, Online meeting, 9th – 20th May 2022

[2] RP-221229, Revised SI: Study on NR Network-controlled Repeaters, ZTE Corporation, RAN meeting #96, Budapest, Hungary, June 6-9, 2022