3GPP TSG-RAN WG1 Meeting #110	R1-2206270

Toulouse, France, August 22nd – 26th, 2022

| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.214 | CR | xxxx | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on M-sample measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh |  |  |  |  |  | Date: |  |  | 2022-08-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Rel-17 specifies M-sample measurement for downlink positioning measurement. It was agreed that the configured M = 1 or 4 sample measurement is applicable to all the concurrent NR positioing measurement methods and all frequency layers. Rel-17 also specified first path RSRP measurement: DL PRS-RSRPP. But the text in current 38.214 states that M = 1 or 4 sample is only appliable to DL RSTD, DL PRS-RSRP and UE Rx-Tx time difference measurement, but not to DL PRS-RSRPP. That would cause misunderstanding that the configured M = 1or 4 sample measurement is not applicable to DL PRS-RSRPP measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.1.6.5 of TS 38.214, add text to clarify that M = 1 or 4 sample measurement is also applicable to DL PRS-RSRPP measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior on DL PRS-RSRPP measurement is ambiguious. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.1.6.5	PRS receiption procedure

<Unchanged parts are omitted>

The UE may be requested, subject to UE capability, to measure and report one or more of the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP or UE Rx-Tx time difference measurements with either 1 or 4 samples, as defined in [11, TS 38.133], via higher layer parameter [numOfSamples-perMeasurement] [17, TS 37.355] which applies for all DL PRS positioning frequency layers.

<Unchanged parts are omitted>