3GPP TSG RAN WG1 #110	R1-2206504

Toulouse, France, August 22nd – 26th, 2022

Source:	Sharp

Title:	Discussion on evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for:	Discussion and Decision

# Deployment scenarios for FR1

Several agreements have been achieved in RAN1#109 e-meeting [1] as follows. We discuss remaining issues for deployment scenarios.

Agreement For discussion purpose for evaluation, define the following deployment cases for SBFD: - Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration. - Deployment Case 2 (Non-coexistence case with multiple SBFD subband configurations): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation, but different cells may use different SBFD subband configurations. - Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy TDD operation (static TDD operation) while the others use SBFD operation with the same SBFD subband configuration. - Deployment Case 3-1: Only 1-layer is considered - Deployment Case 3-2: 2-layer is considered - Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation (static TDD operation) while the other operator uses SBFD operation with the same SBFD subband configuration. Note: This definition has no intention to preclude any potential solutions for SBFD in AI9.3.2 Note: SBFD subband configuration is from gNB perspective. Agreement For SBFD Deployment Case 1, at least consider the following scenarios for evaluation: - For FR1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Urban macro (use Urban macro defined in TR38.802/TR38.901 as starting point) - FFS: UE outdoor/indoor proportion, clustering, etc - Optional: Dense Urban with 1-layer or 2-layer (use Dense Urban defined in TR38.802/TR38.901 as starting point) - FFS: Rural - For FR2-1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Dense Urban Macro layer (use Dense Urban defined in TR38.802 as starting point) - FFS: UE outdoor/indoor proportion, clustering, etc - Optional: Dense Urban micro (use Dense Urban micro defined in TR38.802/TR38.901 as starting point) - FFS: Whether FR2-2 is considered or not in Rel-18. Note: For optional scenarios, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results. Updated proposal 1-4b: For SBFD Deployment Case 4, at least consider the following scenarios for evaluation from RAN1 perspective: - FR1: Urban Macro - FR2-1: Dense Urban Macro layer - FFS: UE outdoor/indoor proportion, clustering, etc - FFS: the grid shift between two networks, e.g., 0%, 100% - FFS: Indoor hotspot, Dense Urban Micro layer

# Indoor office

Detailed parameters for Indoor office (or Indoor hotspot) specified in RP-180524 (Calibration results for self-evaluation), TR38.802, and TR38.901 are summarized in Table 1.

The number of antenna elements per TRxP

The only difference among 3 documents is whether or not the number of antenna panels in horizontal domain is 1 or 2 whereas all 3 documents support {M,N,P} = {4,4,2}. In our view, {M,N,P} = {4,4,2} could be the baseline assumption and the number of antenna panels could be more than 1.

Proposal 1: For Indoor office scenario, {M,N,P} = {4,4,2} is the baseline assumption for TRxP antenna configuration. The number of antenna panels could be more than 1.

The number of UE antenna elements

The number of antenna elements should be not less than 4 because of NR mandatory feature. If RedCap UE should be included in the scope of the duplex study, we can consider 2 as well.

Proposal 2: For Indoor office scenario, the number of antenna elements could be 4 or larger. If RedCap UE should be included in the scope of the duplex study, we can consider 2 as well.

BS antenna element gain and pattern

There are two different configurations as follows:

- Configuration 1: θ3dB=φ3dB=90°, SLAv=Am=25, and the maximum gain = 5 dBi

- Configuration 2: θ3dB=φ3dB=65°, SLAv=Am=30, and the maximum gain = 8 dBi

In Indoor office scenario, ceil-mounted antennas with 180° mechanical tilt are assumed. Therefore, larger half width (i.e., Configuration 1) should be better, to cover wider area. Therefore, we propose as follows:

Proposal 3: For Indoor office scenario, BS antenna pattern is θ3dB=φ3dB=90°, SLAv=Am=25, and the maximum gain = 5 dBi.

Table 1: Summary of parameters for Indoor office (or Indoor hotspot) specified in RP-180524 (Calibration results for self-evaluation), TR38.802, and TR38.901

|  | Self-evaluation (RP-180524) | TR38.802 | TR38.901 |
| --- | --- | --- | --- |
| BS antenna height | 3 m | 3 m | 3 m |
| Total transmit power per TRxP | Baseline: 21 dBm for 10MHz bandwidth | Baseline: 24 dBm for 20 MHz bandwidth | 24 dBm |
| UE power class | 23 dBm | 23 dBm |  |
| Inter-site distance | 20m | 20 m | 20 m |
| Number of antenna elements per TRxP | 32Tx/Rx, (M,N,P,Mg,Ng) = (4,4,2,1,1), (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization | Baseline: (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), (dH, dV) = (0.5, 0.5)λ | Config 1: M = 4,N = 4,P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ Config 2: Mg = Ng = 1, M = N = 2, P = 1 |
| Number of TXRU per TRxP | 32TXRU, (Mp,Np,P,Mg,Ng) = (4,4,2,1,1) (1-to-1 mapping) | 8, 16, or 32 TXRU |  |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |  | M = N = 1, P = 2 |
| Number of TXRU per UE | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) (1-to-1 mapping) |  |  |
| BS noise figure | 5 dB | 5 dB |  |
| UE noise figure | 7 dB | 9 dB | 9 dB |
| BS antenna element gain | 5 dBi | 8 dBi | 8 dBi |
| BS antenna element pattern |  |  |  |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| UE antenna element pattern | Omni-directional | Omni-directional | Omni-directional |
| UE antenna height | 1.5m | 1.5m | 1m |

# Urban Macro

Detailed parameters for Urban macro specified in RP-180524 (Calibration results for self-evaluation), TR38.802, and TR38.901 are summarized in Table 1.

Penetration loss assumption

Penetration loss assumption for indoor UEs affects the interference level. Therefore, it is preferred to align the penetration loss assumption for SBFD and dynamic/flexible TDD.

Number of antenna elements per TRxP and Electric tilt

Electric tilt is available for mitigating gNB-to-gNB CLI. Therefore, detailed design on electric tilt as well as gNB antenna configuration can be up to companies to report.

Proposal 4: Detailed design on electric tilt as well as gNB antenna configuration can be up to companies to report.

Table 2: Summary of parameters for Urban macro specified in RP-180524 (Calibration results for self-evaluation), TR38.802, and TR38.901

|  | Self-evaluation (RP-180524) | TR38.802 | TR38.901 |
| --- | --- | --- | --- |
| BS antenna height | 25 m | 25 m | 25 m |
| Total transmit power per TRxP | 46 dBm for 10 MHz bandwidth | 49 dBm per 20 MHz bandwidth | 49 dBm for 20 MHz bandwidth |
| UE power class | 23 dBm | 23 dBm |  |
| Percentage of high loss and low loss building type | 100% low loss (applies to Channel model B) | Option1 (80% Low loss, 20% High loss) Option 2 (50% Low loss, 50% High loss) | 50% Low loss, 50% High loss |
| Inter-site distance | 500 m | 500 m | 500 m |
| Number of antenna elements per TRxP | 64 Tx/Rx, (M,N,P,Mg,Ng) = (8,4,2,1,1), (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization | Baseline: (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) Note that for Urban macro, companies are also encouraged optionally to investigate larger panels, e.g. (8, 16, 2, 1, 1) (dH,dV) = (0.5, 0.8)λ | Config 1: M = 4,N = 4,P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2 Config 2: Mg = Ng = 1, M = N = 2, P = 1 |
| Number of TXRU per TRxP | 8TXRU, (Mp,Np,P,Mg,Ng) = (1,4,2,1,1) | 8, 16, 32, 64 TXRU for (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) 8, 16, 32 TXRU for (M, N, P, Mg, Ng) = (8, 16, 2, 1, 1) |  |
| Number of UE antenna elements | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1), (dH,dV) = (0.5, N/A)λ 0°, 90° polarization |  | M = N = 1, P = 2 |
| Number of TXRU per UE | 4TXRU (1-to-1 mapping) |  |  |
| Device deployment | 80% outdoor, 20% indoor Randomly and uniformly distributed over the area | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h |  |
| BS noise figure | 5 dB | 5 dB |  |
| UE noise figure | 7 dB | 9 dB | 9 dB |
| BS antenna element gain | 8 dBi | 8 dBi | 8 dBi |
| BS antenna element pattern |  |  |  |
| UE antenna element gain | 0 dBi | 0 dBi | 0 dBi |
| UE antenna element pattern | Omni-directional | Omni-directional | Omni-directional |
| UE antenna height | 1.5 m | 1 m | 1 m |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |  |  |
| Electronic tilt | [99°] in LCS | 102° | 102° |
| Minimum distance of TRxP and UE | d2D_min=10m |  | 10 m |

# Remaining issues on evaluation assumptions

Several agreements have been achieved in RAN1#109 e-meeting [1] as follows. We discuss remaining issues for evaluation assumptions.

Agreement At least the following metrics are considered for SBFD and dynamic/flexible TDD evaluation. - DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%}) using SLS - Latency (CDF or {mean, 5%, 50%, 95%}) using SLS - Resource utilization using SLS - DL/UL received SINR using SLS - Coverage metric - FFS: MPL to achieve a certain bit rate in UL and DL - FFS: definitions of the above metrics - FFS: other metrics Agreement Regarding traffic model for SBFD and dynamic/flexible TDD evaluation, at least FTP3 is considered. Performance evaluation comparison between different duplex modes (e.g., legacy static TDD vs. SBFD) should be performed based on the same amount of input traffic. - FFS: other traffic models, e.g., XR, VoIP - FFS: Packet size, traffic load, ratio of DL/UL traffic - FFS: additionally consider different amount of input traffic at least for adjacent-channel/co-channel coexistence studies Agreement For SBFD evaluation, consider the following for SBFD subband configurations: - SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. - SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth. - Use the following parameters for description of SBFD subband configuration in evaluation assumptions: - ND: the number of RBs in one DL subband - NU: the number of RBs in one UL subband - NG: the number of RBs in one guard band between one UL subband and one DL subband Agreement For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation under SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration), consider the following alternatives: - Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. FFS: whether dynamic TDD can optionally be used for legacy TDD for comparison. Agreement For gNB-gNB co-channel/adjacent-channel channel model and UE-UE co-channel/adjacent-channel channel model in RAN1 SLS, - Large scale fading (e.g., path loss, penetration loss, shadowing) should be modelled, and companies report whether small scale fading (e.g., fast fading including antenna gain) is also modelled in their simulation. - Note: Antenna gain is calculated based on the gNB-gNB or UE-UE LOS direction instead on the multi-path directions if fast fading is not modelled. - FFS: how to model realistic LOS probability for gNB-gNB and UE-UE channel model. - FFS: How to set aligned channel model amongst companies for SLS calibration (if needed). Agreement For gNB-gNB channel model, reuse gNB-to-UE channel model in TR 38.901 with necessary modification - Replacing the UE’s antenna height with gNB’s antenna height, updating the angular spread - FFS: whether/how to update LOS probability. - FFS: Other details and necessary modifications Agreement For SBFD simulation, consider 4GHz for FR1 and 30GHz for FR2-1. Agreement For evaluation and comparison between SBFD and legacy TDD, assume the total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. Regarding antenna elements, both of the two options can be used. - Opt 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. - Opt 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. - Companies report which option is assumed in their simulation. Agreement For SBFD Deployment Case 4, at least consider the following scenarios for evaluation from RAN1 perspective: - FR1: Urban Macro - FR2-1: Dense Urban Macro layer - FFS: UE outdoor/indoor proportion, clustering, etc - FFS: the grid shift between two networks, e.g., 0%, 100% - FFS: Indoor hotspot, Dense Urban Micro layer

# LoS probability

To model channels for gNB-gNB/UE-UE links, LoS probability should be developed. For indoor hotspot deployment, major factors for NLoS condition could be walls, or large furniture. Therefore, LoS probability for gNB-gNB/UE-UE links has similarity with gNB-UE links. Therefore, we propose the following:

Proposal 5: For indoor hotspot, reuse LoS probability formula for gNB-UE links for gNB-gNB/UE-UE links.

For Urban Macro, the gNB mounted on large buildings (25 m height in typical evaluation assumption). Therefore, 100% LoS probability can be assumed for gNB-gNB links.

On the other hand, UE-UE links may be assumed as NLoS in most cases. Due to the high density of buildings, it is expected that LoS probability for UE-UE links may be equal to or smaller than that for gNB-UE links. For simplicity, LoS probability for gNB-UE links can be reused for UE-UE links.

Proposal 6: For Urban Macro, 100% LoS probability can be assumed for gNB-gNB links.

Proposal 7: For Urban Macro, LoS probability formula for gNB-UE links can be assumed for UE-UE links.

# Preliminary evaluation results

Observations should be made after RAN4 input on SI/CLI modeling has been received. Therefore, the results are only for informative.

Figure 1 shows UPT (User Perceived Throughput) performance comparison between semi-static TDD and semi-static SBFD with the simulation assumption described in Table A1 for high load case (i.e., resource utilization is around 60%). In the result, self-interference at gNB is not modeled, and gNB-to-gNB/UE-to-UE CLI are modeled. For ACIR values of those CLIs, values specified in TR38.828 has been reused. We can see that uplink UPT is improved in any user conditions (5%-tile, 50%-tile, 95%-tile). This suggests that, with the CLI assumption specified in TR38.828, impact on UE-to-UE CLI could be minor in InH with the specific simulation assumption as in Table A1.

After RAN4 input on SI/CLI has been received, impact on SI/CLI should be carefully evaluated and discussed in various scenarios.

| Figure 1A: 5%-tile user perceived throughput | Figure 1B: 50%-tile user perceived throughput |
| --- | --- |
| Figure 1C: 95%-tile user perceived throughput | Figure 1D: Mean user perceived throughput |

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: For Indoor office scenario, {M,N,P} = {4,4,2} is the baseline assumption for TRxP antenna configuration. The number of antenna panels could be more than 1.

Proposal 2: For Indoor office scenario, the number of antenna elements could be 4 or larger. If RedCap UE should be included in the scope of the duplex study, we can consider 2 as well.

Proposal 3: For Indoor office scenario, BS antenna pattern is θ3dB=φ3dB=90°, SLAv=Am=25, and the maximum gain = 5 dBi.

Proposal 4: Detailed design on electric tilt as well as gNB antenna configuration can be up to companies to report.

Proposal 5: For indoor hotspot, reuse LoS probability formula for gNB-UE links for gNB-gNB/UE-UE links.

Proposal 6: For Urban Macro, 100% LoS probability can be assumed for gNB-gNB links.

Proposal 7: For Urban Macro, LoS probability formula for gNB-UE links can be assumed for UE-UE links.

# Annex

Table A1 shows simulation assumptions for SLS results.

Table A1: SLS simulation assumptions

| Parameters | Values |
| --- | --- |
| Simulation Scenario | InH |
| The number of sectors per BS | 1 |
| Carrier frequency | 4 GHz |
| Channel bandwidth | 100 MHz |
| Transmission bandwidth configuration | 273 RBs |
| Subcarrier spacing | 30 kHz |
| FFT size | 4096 |
| Noise floor | -174 dBm |
| BS antenna configuration | [M,N,P] = [1,1,2] |
| BS antenna port configuration | 1 port |
| BS antenna pattern | Directional |
| BS Tx power | 24 dBm |
| BS Tx gain | 5 dB |
| BS noise figure | 5 dB |
| BS mechanical tilt | 180 degrees |
| BS electrical tilt | 90 degrees |
| BS height | 3 m |
| BS half value for vertical | 90 degrees |
| BS half value for horizontal | 90 degrees |
| SLAv | 25 dB |
| Am | 25 dB |
| UE antenna configuration | [M,N,P] = [1,1,2] |
| UE antenna port configuration | 1 port |
| UE antenna pattern | Omni-directional |
| UE maximum Tx power | 23 dB |
| UE Noise figure | 9 dB |
| UE speed | 3 km/h |
| UE height | 1.5 m |
| TDD pattern | DDDSU or DXXXU (X denotes SBFD slot) |
| Special slot configuration | 12D2F0U |
| Subband configuration | [D,U,D] = [106, 51, 106] RBs |
| RBG size for PDSCH | 4 RBs |
| DMRS mapping for PDSCH | 3rd, 7th, and 11th symbol |
| MMSE IRC for PDSCH | off |
| RBG size for PUSCH | Wideband allocation |
| DMRS mapping for PUSCH | 3rd, 7th, and 11th symbol |
| MMSE IRC for PUSCH | off |
| CQI reporting period | Once per 50 TTIs |
| CQI reporting delay | 2 TTIs |
| CQI reporting mode | Subband reporting |
| Scheduler | Subband PF |
| Open loop link adaptation | ON |

# References

- [1]	RAN1 chairman’s note at RAN1#109 e-meeting, May 2022