3GPP TSG RAN WG1 #110	R1-2206576

Toulouse, France, August 22nd – 26th, 2022

Source:	Intel Corporation

Title:	Discussion on enhancement for 8Tx UL transmission

Agenda item:	9.1.4.2

Document for:	Discussion and Decision

# Introduction

In RAN1 #109-e meeting, the following agreements were reached on SRI/TPMI enhancement for 8Tx UL.

Agreement Study fully-coherent, partially-coherent and non-coherent UEs for uplink transmission with 8TX UEs . Agreement Study full power transmission for 8TX UEs . - Details are FFS upon completion of codebook design Agreement Adopt the following Table as the reference EVM for LLS evaluation - Companies may provide additional evaluation results per their case of interest - LLS is optionally used for 8Tx UL evaluation, if needed Parameter; Value. Carrier Frequency; 3.5 GHz. Waveform; CP-OFDM. SCS; 30 KHz. System bandwidth; 20 MHz, 100 MHz. Scheduled PRBs; 5, 25, 50, 260 PRBs. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1. UE speed; 3 Km/h. Number of Layers; Adaptive, Fixed (reported by company). AMC; Adaptive, Fixed (reported by company). DMRS configuration; Type 1; 1 front loaded + 1 additional symbol. Channel estimation; Real. Channel Model; CDL-A (30ns), CDL-B (100ns), CDL-C (300ns). Agreement For 8TX UE uplink transmission, study codebook- and non-codebook-based transmission with maximal layer number of both 4 and 8 layers. Agreement Adopt the following Table as the reference EVM for SLS evaluation.- - Companies may provide additional evaluation results per their case of interest. Parameter; Value. Frequency range; 3.5 GHz. Multiple access; OFDMA. Numerology; 14 OFDM symbol slot SCS , 30 KHz. Scenario; Outdoor FWA (38.901): UMa (ISD = 500 m), 100% Outdoor, 3Km/h. Indoor FWA (38.901): UMi (ISD = 200 m), 100% Indoor, 3Km/h. Industrial (38.901): Indoor Office (Inh ), 3Km/h. Channel model; 38.901. System bandwidth; 20 MHz, 100 MHz. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); Outdoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Indoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Industrial: (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ. gNB antenna radiation pattern parameters; Outdoor/Indoor FWA : 38.901 Table 7.3-1, 8 dBi , 65° HPBW Industrial: IMT.2412 Table 10,5 dBi , 90° HPBW. gNB receiver noise figure; 5dB. gNB receiver; MMSE-IRC. gNB scheduler; Single user with proportional fair. Modulation; - Up to 64 QAM - Up to 256QAM. MIMO scheme; SU-MIMO with rank adaptation. UE speed; 3 Km/h. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1. Traffic model; - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional). Suggested benchmarking; R15 UL 4-Tx codebook , Eigen-based, companies report PRG assumption. Precoder granularity; Wideband. Power control; Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario. UE power rating; 23 dBm (UE, 38.101) 32 dBm (FWA, 38.101). Metric; UL mean-user throughput, 5%-ile and 95%-ile UPT. Agreement For 8TX UE, consider the following UE antenna layouts for codebook design, - For non-coherent UEs, consider linear array (1D/2D) of cross-polarized or single-polarized antenna configuration - For fully/partial-coherent UEs, consider linear array (1D/2D) - Where the array is either cross-polarized antenna configuration or single polarized antenna configuration - Ng>=1 antenna groups can be considered where each group comprises coherent antennas, and across groups, antennas can be non-coherent/coherent depending on device types - An example of an antenna group is a panel - Within an antenna group, antenna elements are uniformly spaced. Across different antenna groups, companies to provide details. Additional information for definition of antenna layout - Based on the number of coherent groups, following exemplary cases can be considered where, within each group, antenna elements are spaced by 0.5λ, and then dG-H, dG-V represent the horizontal and vertical spacings between the centers of adjacent antenna groups, respectively - Further down-selection can be done in the next meeting, if needed - The shown exemplary placing of antenna groups can be used for evaluation purpose, but the codebook design is not restricted to shown cases. - Other antenna layouts for other use cases are not precluded. - To start companies may report their results according to their preferred layout. Case; Ng; (M, N, P) per group; Antenna Layout; Antenna Pattern/Antenna Element Gain. 1; 1; (2, 2, 2), (1, 4, 2); Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA). 2; 2; (1, 2, 2); Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA). 3; 4; (1, 1, 2); Isotropic (Indoor/Outdoor FWA & Industrial) 4 dBi, 110° HPBW(Indoor FWA & Industrial. o Other UE antenna assumption for the purpose of evaluation Outdoor FWA; Indoor FWA; Industrial. UE antenna height; 6, 3 m (To start); According to 36.873; According to 38.901. Agreement For 8TX UE codebook-based uplink transmission, down-select one of - Alt1-a: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for non-coherent UEs - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully/partially-coherent UEs - Alt1-b: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for partially/non-coherent UEs - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully-coherent UEs - Alt2-a: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs - Alt2-b: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) in combination with those based on NR Rel-15 DL Type I codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs - Alt3: - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially/non-coherent UEs - Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

In this contribution, we provide our views on the enhancements for 8Tx UL in Rel-18.

# Discussion on enhancement for 8Tx UL

## 2.1 Codebook based transmission

### 2.1.1 Codebook design

Codebook design is important for uplink enhancement with 8Tx in Rel-18. In previous meeting, the following agreement was reached.

Agreement For 8TX UE codebook-based uplink transmission, down-select one of - Alt1-a: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for non-coherent UEs - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully/partially-coherent UEs - Alt1-b: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for partially/non-coherent UEs - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully-coherent UEs - Alt2-a: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs - Alt2-b: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) in combination with those based on NR Rel-15 DL Type I codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs - Alt3: - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially/non-coherent UEs - Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

It can be observed that the design principle mainly includes:

- Based on antenna selection vectors

- Based on Rel-15 UL 2Tx/4Tx codebooks

- Based on Rel-15 Type I codebooks

For Alt2-a and Alt3, the design is unified solution for different UE coherence. For Alt1-a and Alt1-b, the design is different for different coherence assumptions.

We think a unified solution is better. For Rel-15 DL Type I codebook, it’s more suitable for a fully coherent UE. Thus, Alt2-a is the preferred solution if the unified design for different UE coherence is applied, i.e., the full coherent/partial coherent/non-coherent codebook could be based on Rel-15 2Tx/4Tx codebooks.

If the unified solution is not used, then different codebook design scheme could be applied for different UE coherence type.

Non-coherent precoders

For non-coherent UE with 8Tx, it’s natural that the non-coherent precoders are based on antenna selection vectors, since the UE can’t maintain relative phase among any Tx chains. For Rank-x, the 8-port non-coherent precoder consists of x antenna selection column vectors.

For example, for rank-1, the precoders could be:

1210000000,1201000000,1200100000,1200010000,1200001000,1200000100,1200000010,1200000001

For rank-2, the precoders could be:

121001000000000000,121000010000000000,121000000100000000,121000000001000000,121000000000010000,121000000000000100,121000000000000001,120010010000000000,…

Similarly, the non-coherent precoders could be obtained for other ranks.

It can be observed that for Rank-x, the number of non-coherent precoders could be C8x if all the combinations of antenna selection vectors are included.

For non-coherent UE, another solution is that the codebook is based on Rel-15 2Tx/4Tx codebook. For example, the precoder could be generated by taking the Kronecker product of 2Tx non-coherent codebook and 4Tx non-coherent codebook.

Partial coherent precoders

For a partial coherent UE, in the agreement from previous meeting, the codebook could be based on Rel-15 2Tx/4Tx codebook or Rel-15 Type I codebook. However, we think Rel-15 Type I codebook is more suitable for a full coherent UE. Thus, for partial coherent UE with 8Tx, the partial coherent precoders could be based on Rel-15 2Tx/4Tx codebook.

For partial coherent precoders with 8 ports, the precoders could be generated according to the following equation, where W1 and W2 could be Rel-15 2Tx or 4Tx codebook, and ⊗ represents Kronecker product operation.

W8Tx=W1⊗W2		(Equation 1)

In the previous meeting, it was also agreed that multiple antenna groups could be considered, and within each group the antennas are assumed to be coherent. Therefore, for a partial coherent UE with 8Tx, there could be two cases, one is the UE has two antenna groups (four coherent ports per group), and the other one is the UE has four antenna groups (two coherent ports per group).

Case A: partial coherent UE with two antenna groups

For partial coherent UE with two antenna groups, each group has four coherent ports. One example of the partial coherent UE is as shown in Figure 1. The coherent ports are {#0, #2, #4, #6} and {#1, #3, #5, #7}.

Figure 1 Partial coherent UE with two antenna groups

Then, the partial coherent precoders could be generated according to Equation (1). W1 is a Rel-15 full coherent 2Tx precoder with Rank-1, and W2 is a Rel-15 partial coherent 4Tx precoder with Rank-x, where x is the rank of the 8-port precoder.

For example, the rank-1 partial coherent precoders would be:

12210101010,12210-1010-10,12201010101,122010-1010-1,…

Case B: partial coherent UE with four antenna groups

For partial coherent UE with four antenna groups, each group has two coherent ports. The coherent ports could be {#0, #2}, {#4, #6}, {#1, #3}, and {#5, #7}, as shown Figure 2.

Figure 2 partial coherent UE with four antenna groups

In this case, the partial coherent precoders could be generated according to Equation (1). W1 is a Rel-15 non-coherent 2Tx precoder with Rank-1, and W2 is a Rel-15 partial coherent 4Tx precoder with Rank-x, where x is the rank of the 8-port precoder.

For example, the rank-1 partial coherent precoders could be:

12210100000,12200001010,12201010000,12200000101,…

Full coherent precoders

For full coherent UE with 8Tx, the precoders could be based on:

Option A) Rel-15 2Tx/4Tx codebook, or

Option B) Rel-15 DL Type I codebook.

With Rel-15 2Tx/4Tx codebook, the full coherent precoders with 8 ports could be generated according to Equation (1), where W1 is a Rel-15 full coherent 2Tx precoder with Rank-1, and W2 is a Rel-15 full coherent 4Tx precoder with Rank-x, and x is the rank of the 8-port precoder.

In order to evaluate the performance of different schemes, system level simulation was performed, where the UE antenna layout of one antenna group and (M, N, P)=(1, 4, 2) was assumed. The detailed simulation assumption is shown in Appendix.

The simulation results are shown in Table 1.

Table 1 System level simulation results

| PUSCH, maxRank=1 | PUSCH, maxRank=4 |
| --- | --- |

It could be observed that for maxRank of 1, the performance of Option A and Option B are almost the same. For maxRank of 4, the Type I codebook (Option B) shows some gain over Option A, but the gain is not big.

Therefore, Rel-15 Type I codebook could be used for full coherent UE with 8 Tx, especially for higher rank. However, the overhead should be considered since the number of precoders based on Rel-15 Type I codebook could be large.

Overhead

For the design of codebook with 8 Tx, the overhead should be taken into account. The number of precoders with 8 ports might be large.

For example, for non-coherent precoders with 8 Tx, the number of non-coherent precoders could be C8x for rank-x. The detailed number is shown in Table 2.

Table 2 Number of non-coherent precoders for 8Tx

| Rank | Number of non-coherent precoders for 8Tx |
| --- | --- |
| 1 | C81=8 |
| 2 | C82=28 |
| 3 | C83=56 |
| 4 | C84=70 |

It can be seen that if maxRank=4 is configured, then the number of non-coherent precoders would be 8+28+56+70=162.

If the non-coherent codebook is generated according to Equation (1), i.e., W1 is a Rel-15 non-coherent 2Tx precoder with Rank-1, and W2 is a Rel-15 non-coherent 4Tx precoder with Rank-x, where x is the rank of the 8-port precoder, then the number of non-coherent precoders could be reduced a lot. In general, further simulations and study is needed to determine a reasonable codebook size.

Large number of precoders will require a lot of DCI overhead for TPMI indication but many precoders may have a very low probability of selection. Thus, the overhead issue should be taken into account for the 8Tx UL codebook design.

Therefore, we have the following proposal for codebook design with 8Tx.

Proposal 1:

- For 8Tx UL codebook design, if RAN1 strives for unified solution for different coherence, then Alt2-a is preferred, i.e., the full coherent/partial coherent/non-coherent precoders could be based on Rel-15 2Tx/4Tx codebook.

- Otherwise, the codebook design could be based on Alt1-b, i.e.,

  - The non-coherent precoders are antenna selection vectors or based on Rel-15 2Tx/4Tx codebook

  - The partial coherent precoders could be based on Rel-15 2Tx/4Tx codebook

  - The full coherent precoders could be based on Rel-15 Type I codebook

Proposal 2:

- For 8Tx UL codebook design, RAN1 to take the overhead into account.

### 2.1.2 Max number of layers

For uplink transmission with 8 Tx, for the same rank, addition gain can be provided compared with 4Tx, since the number of antennas is increased.

However, we don’t see strong motivation for more than 4 layers since the performance requirements for DL 8 MIMO layers is not defined in RAN4. Therefore, we suggest taking 4 layers as starting point for uplink enhancement with 8Tx in Rel-18.

Proposal 3:

- For 8Tx UL transmission, the maximum of 4 layers could be prioritized.

### 2.1.3 Number of codewords

For uplink transmission with 8Tx, if the number of layers is extended to 8, then multiple codewords could be used, e.g., two codewords.

However, introducing multiple codewords would require a lot of spec work. For example, the DL codeword to layers mapping may be used. Different MCS/RV may be applied for the two codewords. Different HARQ processing maybe applied for the two codeword. These aspects may require a lot of discussion.

Considering that maximum of 4 layers is preferred, we think one codeword could be used for 8Tx UL transmission.

Proposal 4:

- For 8Tx UL transmission, single codeword is preferred.

### 2.1.4 SRS configuration

In Rel-16, for codebook-based transmission, the SRS configuration could be different depending on the full power operation, as shown in Table 3.

Table 3 Summary of SRS configuration for codebook-based UL in Rel-16

|  |  | SRS configuration |
| --- | --- | --- |
| Full power disabled |  | One SRS resource set. Up to two SRS resources with the same number of ports. |
| Full power enabled | Full power Mode 0 | One SRS resource set. Up to two SRS resources with the same number of ports. |
|  | Full power Mode 1 | One SRS resource set. Up to two SRS resources with the same number of ports. |
|  | Full power Mode 2 | One SRS resource set. Up to two or four SRS resources. The SRS resource may have different number of ports. |

In Rel-18, to support 8Tx uplink transmission, one SRS resource set could be configured with usage set to ‘codebook’, similar as the configuration in Rel-16.

However, regarding the number of SRS resources, and number of ports for SRS resources, it could be discussed after together with full power operation since the configuration is depended on the full power mode.

Proposal 5:

- For codebook based transmission with 8Tx, one SRS resource set could be configured. The number of SRS resources and number of ports for SRS resources could be discussed together with full power operation.

### 2.1.5 Full power operation

In Rel-16, full power operation was introduced with different full power operation mode, i.e., Mode 0, Mode 1 and Mode 2. There were three kinds of UE PA architectures considered in Rel-16 full power operation, as shown in Figure 3.

- All the UE PAs can deliver full power

- None of the UE PAs can deliver full power

- A subset of the UE PAs can deliver full power

The maximum output power of the PA could be 23dBm, 20dBm, or 17dBm.

Figure 3 UE PA architecture for Rel-16 full power operation with 2Tx

In previous meeting, it has been agreed to study the full power operation for 8Tx UE and the details could be discussed after the completion of codebook design.

However, it is worthy of discussion on the PA architecture of the UE with 8Tx for full power operation, which is not dependent on the codebook design. For example, whether all the three kinds of PA architectures will be considered in Rel-18. And whether the PA with lower max output power should be considered, e.g., 14 dBm.

Proposal 6:

- RAN1 to discuss the UE PA architectures to be considered for full power operation with 8Tx in Rel-18.

## 2.2 Non-codebook based transmission

### 2.2.1 SRS configuration

In Rel-15, for non-codebook based transmission, one SRS resource set is configured with up to 4 SRS resources, and each SRS resource is single port. In the DCI scheduling PUSCH, the SRI filed is the joint encoding of SRI and Rank indicator, as shown in Figure 4. The SRI field length depends on the number of SRS resources (NSRS) and the maximum number of MIMO layers (Lmax).

Figure 4 SRI field for non-codebook based transmission with Lmax=4 in Rel-15

In Rel-18, in order to support 8Tx uplink transmission, the non-codebook based transmission should be enhanced to support 8Tx.

Considering that maximum layers of 4 is preferred as indicated in Section 2.1.2, one SRS resource set could be configured for non-codebook based transmission with 8 Tx. And up to 8 SRS resources with single port could be configured in the SRS resource set. In the DCI, the joint encoding of SRI and RI could be applied to reduce the overhead, similar with Rel-15 design.

Proposal 7:

- For non-codebook based transmission, one SRS resource set could be configured, and joint encoding of SRI and RI is preferred.

# Conclusion

In conclusion, we have the following proposals on 8Tx UL enhancements in Rel-18.

Proposal 1:

- For 8Tx UL codebook design, if RAN1 strives for unified solution for different coherence, then Alt2-a is preferred, i.e., the full coherent/partial coherent/non-coherent precoders could be based on Rel-15 2Tx/4Tx codebook.

- Otherwise, the codebook design could be based on Alt1-b, i.e.,

  - The non-coherent precoders are antenna selection vectors or based on Rel-15 2Tx/4Tx codebook

  - The partial coherent precoders could be based on Rel-15 2Tx/4Tx codebook

  - The full coherent precoders could be based on Rel-15 Type I codebook

Proposal 2:

- For 8Tx UL codebook design, RAN1 to take the overhead into account.

Proposal 3:

- For 8Tx UL transmission, the maximum of 4 layers could be prioritized.

Proposal 4:

- For 8Tx UL transmission, single codeword is preferred.

Proposal 5:

- For codebook based transmission with 8Tx, one SRS resource set could be configured. The number of SRS resources and number of ports for SRS resources could be discussed together with full power operation.

Proposal 6:

- RAN1 to discuss the UE PA architectures to be considered for full power operation with 8Tx in Rel-18.

Proposal 7:

- For non-codebook based transmission, one SRS resource set could be configured, and joint encoding of SRI and RI is preferred.

# References

- 3GPP TS 38.211 v17.2.0, Physical channels and modulation

- 3GPP TS 38.212 v17.2.0, Multiplexing and channel coding

- 3GPP TS 38.213 v17.2.0, Physical layer procedures for control

- 3GPP TS 38.214 v17.2.0, Physical layer procedures for data

- 3GPP TS 38.331 v17.1.0, Radio Resource Control (RRC) protocol specification

- RP-213598, 3GPP TSG RAN Meeting #94-e, Dec 6 – Dec 17, 2021

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #109-e, May 9th – 20th, 2022

# Appendix: Simulation assumptions

Table 4 System level simulation assumptions

| Parameter | Value |
| --- | --- |
| Scenario | Umi |
| ISD | 200 m |
| Carrier frequency | 3.5 GHz |
| Simulation bandwidth | 20 MHz |
| Subcarrier spacing | 30 kHz |
| Number of UEs | 210 |
| UE speed | 3km/h |
| Traffic model | Full buffer |
| gNB antenna configuration | (M,N,P,Mg,Ng)=(4,4,2,1,1) with (dH,dV)=(0.5,0.8)λ |
| UE antenna configuration | (M,N,P,Mg,Ng)=(1,4,2,1,1) with (dH,dV)=(0.5,0.5)λ |
| MIMO scheme | SU-MIMO |
| Precoding | Wideband |
| Scheduling | Proportional fair |
| UE power class | 23 dBm |
| Power control | Alpha=0.8, P0=-80dBm |