3GPP TSG RAN WG1 #110                                                                 R1-2206579

Toulouse, France, August 22nd – 26th, 2022

Source:	Intel Corporation

Title:	Use-cases and specification for CSI feedback

Agenda item:	9.2.2.2

Document for:	Discussion and Decision

- Introduction

The new study item on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface has been approved in [1]. One of the study objectives includes the analysis of solutions for CSI feedback enhancements:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project

In this contribution, we express our views on sub use cases for AI/ML CSI including potential specification impact.

- Discussion

At the last RAN1 meeting the following conclusion was made on potential sub use cases for AI/ML CSI.

Conclusion - Further discuss temporal-spatial-frequency domain CSI compression using two-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion. - Further discuss improving the CSI accuracy based on traditional codebook design using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion. - Further discuss CSI prediction using one-sided model as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion - Further discuss CSI-RS configuration and overhead reduction as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion - Further discuss resource allocation and scheduling as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion - Further discuss joint CSI prediction and compression as a possible sub-use case for CSI feedback enhancement after evaluation methodology discussion.

The sub use case corresponding to temporal-spatial-frequency domain CSI compression using two-sided model can be divided in to two sub use cases. For the first sub use case, correlation of information in multiple consecutive CSI reports can be used, i.e. initial CSI report(s) carry the information about the channel while subsequent CSI report(s) carry novel channel information corresponding to the channel variation comparing to the initial CSI. Thus, reduction of CSI overhead and increase of CSI reporting efficiency can be achieved. This sub use case can be implemented by using recurrent neural network (NN). For the second sub use case it can be assumed that one CSI report contains information about CSI in multiple time instances. Since the CSI across multiple time instances is correlated – additional compression can be achieved. This sub use case can be used together with CSI prediction.

The first sub use case described in the above paragraph is an enhancement on top of the two-sided AI/ML model for CSI reporting. It is expected to be more efficient while it may be a challenge to achieve generalisation in conjunction with a network structure and training procedure that is reasonable in complexity. Thus, it may be considered after sufficient progress is achieved on two-sided AI/ML model for CSI without recurrent NN.

The second sub use case described in the above paragraph corresponds to the similar CSI reporting approach which is considered in Rel-18 MIMO WI for CSI enhancements for high/medium UE mobility. Thus, it is preferred to wait until the corresponding design for Rel-18 MIMO WI is finalized before considering AI/ML solution for the same problem.

One of the sub use cases considered in the above conclusion corresponds to the utilization of one-sided AI models with traditional CSI codebook feedback. Improving the CSI accuracy based on traditional codebook design using one-sided model can be supported by gNB and/or UE implementation. However, the performance is not expected to be better than the two-sided model due to potential codebook design inefficiencies.

CSI prediction via one-sided neural network is also considered as one sub use case for AI/ML CSI. In our view it makes sense to consider CSI prediction with one-sided or two-sided AI/ML model together with temporal-domain compression. Also, in our view it is preferred to defer discussion on this sub use case until the design for Rel-18 MIMO WI CSI enhancements for high/medium UE mobility is finalized since CSI prediction is considered there.

Regarding sub use case related to CSI-RS configuration, in our view CSI reporting should be the main focus for the study since there is no significant issue with CSI-RS overhead in the network, at the same time, improved accuracy and decreased overhead for CSI reporting may lead to substation improvement for the network performance.

Sub use case corresponding to resource allocation and scheduling is too broad and require further study to identify particular feature related to the considered SI objective especially keeping specification impact in mind.

Spatial-frequency domain CSI compression using two-sided AI model sub use case was agreed at the last RAN1 meeting (please see the below agreement).

Agreement Spatial-frequency domain CSI compression using two-sided AI model is selected as one representative sub use case. - Note: Study of other sub use cases is not precluded. - Note: All pre-processing/post-processing, quantization/de-quantization are within the scope of the sub use case.

In our contribution [2] it was shown by simulation results that ML based Autoencoder can outperform Rel-16 eType II PMI codebook at least for Rank-1 case in almost all overhead regimes for InH and Dense Urban Macro deployments. Thus, considering the above discussion, in our view the sub use case corresponding to Spatial-frequency domain CSI compression using two-sided AI model should be prioritized for this SI.

Proposal 1:

- Spatial-frequency domain CSI compression using two-sided AI model should be prioritized

In order to enable use of AI/ML for CSI feedback with two-sided AI model it is necessary to update NR PHY specification. In particular, for the NR specification the PMI codebooks are defined so the UCI bits for PMI are interpreted by the UE and gNB in the right way. The exact implementation of PMI search is not defined in the specification. For NN it is hard to define a concept like codebook while it is easier to describe how to calculate the UCI bits for PMI based on NN structure and NN weights (encoder part of the autoencoder NN).

Also, the NR specification has clear assumption on the precoder matrix applied for CQI calculation which corresponds to the selected PMI and determined by the PMI codebook. For CSI with the two-sided AI model, knowledge of decoder part of the autoencoder is required at the UE side in order to calculate accurate CQI. If the decoder is not known at the UE, then CQI mismatch is expected due to different precoder matrix assumption for the actual PDSCH transmission and the CQI calculation.

Based on the discussion above, knowledge of both encoder and decoder parts of the NN may be required at the UE side including NN structure and values of NN weights. There are different ways how to specify NN including direct description of NN structure and specification of NN weights in the 5G NR specification, RRC configuration of NN structure and downloading of weights at the UE or combination of the above.

Since AI/ML CSI requires definition of a new CSI quantity, UCI design for the AI/ML CSI with two-sided AI model shall be updated especially considering that some features of the existing UCI (e.g. partial UCI omission) may be useless for the NN. Also, since separate hardware block may be used at the UE for AI/ML processing, update of the UE capability framework for CSI may be required including support of separate CSI processing unit (CPU) for AI/ML CSI.

Proposal 2:

- Specification impact for the CSI compression using two-sided AI model may include the following

  - Introduction of NN concept in specification for PMI bits calculation (encoder part)

  - Introduction of NN concept in specification for precoder reconstruction for CQI (decoder part)

  - RRC configuration (including NN parameters configuration and NN coefficients)

  - UCI design (including new CSI quantity)

  - UE capabilities (including CPU definition for the AI/ML CSI)

- Conclusion

In this contribution, our views on sub use cases for AI/ML CSI including potential specification impact was discussed. The following proposals were made.

Proposal 1:

- Spatial-frequency domain CSI compression using two-sided AI model should be prioritized

Proposal 2:

- Specification impact for the CSI compression using two-sided AI model may include the following

  - Introduction of NN concept in specification for PMI bits calculation (encoder part)

  - Introduction of NN concept in specification for precoder reconstruction for CQI (decoder part)

  - RRC configuration (including NN parameters configuration and NN coefficients)

  - UCI design (including new CSI quantity)

  - UE capabilities (including CPU definition for the AI/ML CSI)

- References

- RP-213599 “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” Qualcomm, December 6 – 17, 2021.

- R1-2206578 “Evaluation for CSI feedback enhancements,” Intel Corporation, August 22nd – 26th, 2022.