3GPP TSG RAN WG1 #110		R1-2206744

Toulouse, France, August 22nd – 26th, 2022

Source:	vivo

Title:	Discussion on accuracy improvements for NR positioning

Agenda Item:	8.5

Document for:	Discussion and Decision

Introduction

In RAN1#109-e meeting, some remaining issues on accuracy enhancements were discussed and some agreements were achieved in [1].

In this contribution, we present our views on remaining issues on accuracy enhancements.

Remaining issues on Rx/Tx timing delay mitigating

UE Tx TEG

In TS 38.214 [2], the following paragraphs are easily misunderstood. That is, both of them are association information and reporting information, but they are different.

The UE may be configured, subject to UE capability, to report UE Tx TEGs (Timing Error Group), where UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin. The UE may be configured to report, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter [ueTxTEG].

For us, the first paragraph is more of a definition of UE Tx TEGs than the information to be configured and reported. So, it can be modified based on the following UE Rx TEG definition in section 5.1.6.5.

Timing Error Group(s) (TEG(s)) at UE side are defined: - UE Rx TEG is associated with one or more DL measurements, which have the Rx timing error difference within a certain margin. - UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin.

Therefore, we propose

- Adopt the following modifications into TS 38.214 for UE Tx TEG.

< Unchanged parts are omitted > The UE may be configured, subject to UE capability, to report UE Tx TEGs (Timing Error Group), where Timing Error Group(s) (TEG(s)) at UE side are defined: - UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin. The UE may be configured to report, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter [ueTxTEG]. < Unchanged parts are omitted >

Remaining issues on DL-AOD enhancements

PRS RSRPP reporting

Agreement: The agreement from RAN1#106e on the number of DL PRS RSRP measurements per TRP is extended as follows: - For UE-A DL-AOD, support reporting more than 8 up to 16 N DL PRS RSRP measurements per TRP, where N is UE capability and candidate values include {16,24}. - For UE-A DL-AOD, support reporting more than 8 up to 16 M first path PRS RSRP measurements per TRP, where M is a UE capability - FFS: Values of M. Candidate values include {2,4,8,16,24}. - FFS: Whether M is always equal to N - Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for same or different timestamps. - Note: the maximum number of DL PRS RSRP associated with the same Rx beam index is up to the UE implementation Agreement For the reporting of first path DL-PRS RSRPP in DL AOD, - For the 1st measurement, the report includes DL PRS RSRP and optionally DL PRS RSRPP using absolute reporting - For additional measurement, at least one of the two following measurement is reported: - First path DL PRS RSRPP can be optionally reported using differential reporting with the first measurement of DL PRS RSRPP, - DL PRS RSRP can be optionally reported using differential reporting with the first measurement of DL PRS RSRP. - Send the agreement in an LS reply to RAN2 as a reply regarding the question “For RAN1 agreements “The requested PRS measurement can be DL PRS RSRP and/or path PRS RSRP. ”, is there a need to request and provide only the RSRPP measurements for the additional measurements (without legacy RSRP)?” Agreement For DL-AoD, the additional RSRPP measurement takes from -30 dB to 30 dB as the reporting range in reference to the first RSRPP measurement - Send an LS to RAN2 and RAN4 informing of the agreement.

Based above agreement, and following the RAN2 specification[3], we can find that the 24 DL PRS RSRP for a TRP should include one absolute RSRPP in nr-DL-PRS-FirstPathRSRP-Result, and up to 23 differential RSRPP in nr-DL-PRS-FirstPathRSRP-ResultDiff.

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126), nr-DL-PRS-RxBeamIndex-r16 INTEGER (1..8) OPTIONAL, nr-DL-AoD-AdditionalMeasurements-r16 NR-DL-AoD-AdditionalMeasurements-r16 OPTIONAL, ..., [[ nr-DL-PRS-FirstPathRSRP-Result-r17 INTEGER (0..126) OPTIONAL, nr-los-nlos-Indicator-r17 CHOICE { perTRP-r17 LOS-NLOS-Indicator-r17, perResource-r17 LOS-NLOS-Indicator-r17 } OPTIONAL, nr-DL-AoD-AdditionalMeasurementsExt-r17 NR-DL-AoD-AdditionalMeasurementsExt-r17 OPTIONAL ]] } NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF NR-DL-AoD-AdditionalMeasurementElement-r16 NR-DL-AoD-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasAoD-r17)) OF NR-DL-AoD-AdditionalMeasurementElement-r17 NR-DL-AoD-AdditionalMeasurementElement-r16 ::= SEQUENCE { nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-DL-PRS-RSRP-ResultDiff-r16 INTEGER (0..30), nr-DL-PRS-RxBeamIndex-r16 INTEGER (1..8) OPTIONAL, ... } NR-DL-AoD-AdditionalMeasurementElement-r17 ::= SEQUENCE { nr-DL-PRS-ResourceID-r17 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r17 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r17 NR-TimeStamp-r16, nr-DL-PRS-RSRP-ResultDiff-r17 INTEGER (0..30) OPTIONAL, -- Cond rsrp nr-DL-PRS-RxBeamIndex-r17 INTEGER (1..8) OPTIONAL, nr-DL-PRS-FirstPathRSRP-ResultDiff-r17 INTEGER (0..61) OPTIONAL, -- Cond rsrpp nr-los-nlos-IndicatorPerResource-r17 LOS-NLOS-Indicator-r17 OPTIONAL, ... }

However, in TS 38.214 [2]with yellow highlighted, it seems to be saying that all 24 path RSRPs are reported via nr-DL-PRS-FirstPathRSRP-Result.

The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP meas1`urements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. The UE may be configured to measure and optionally report via higher layer signaling nr-DL-PRS-FirstPathRSRP-Result, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on different DL PRS resources associated with the same dl-PRS-ID. The UE may be configured to optionally report a differential DL RSRPP for a PRS resource with reference to [nr-DL-PRS-FirstPathRSRP-Result] and/or a differential DL PRS RSRP with reference to [nr-DL-PRS-RSRP-Result] via higher layer parameter [NR-DL-AoD-AdditionalMeasurementElement].

So, we propose:

- Adopt the following modifications into TS 38.214 for PRS RSRPP reporting .

< Unchanged parts are omitted > The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. The UE may be configured to measure and optionally report via higher layer signaling nr-DL-PRS-FirstPathRSRP-Result, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on different DL PRS resources associated with the same dl-PRS-ID. The UE may be configured to optionally report a differential DL RSRPP for a PRS resource with reference to [nr-DL-PRS-FirstPathRSRP-Result] and/or a differential DL PRS RSRP with reference to [nr-DL-PRS-RSRP-Result] via higher layer parameter [NR-DL-AoD-AdditionalMeasurementElement]. < Unchanged parts are omitted >

Conclusion

In this contribution, we discuss on accuracy improvement with the following proposals.

- Adopt the following modifications into TS 38.214 for UE Tx TEG.

< Unchanged parts are omitted > The UE may be configured, subject to UE capability, to report UE Tx TEGs (Timing Error Group), where Timing Error Group(s) (TEG(s)) at UE side are defined: - UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing error difference within a certain margin. The UE may be configured to report, subject to UE capability, association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE Tx TEG(s) via higher layer parameter [ueTxTEG]. < Unchanged parts are omitted >

- Adopt the following modifications into TS 38.214 for PRS RSRPP reporting .

< Unchanged parts are omitted > The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. The UE may be configured to measure and optionally report via higher layer signaling nr-DL-PRS-FirstPathRSRP-Result, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on different DL PRS resources associated with the same dl-PRS-ID. The UE may be configured to optionally report a differential DL RSRPP for a PRS resource with reference to [nr-DL-PRS-FirstPathRSRP-Result] and/or a differential DL PRS RSRP with reference to [nr-DL-PRS-RSRP-Result] via higher layer parameter [NR-DL-AoD-AdditionalMeasurementElement]. < Unchanged parts are omitted >

- References

- Chairman’s notes for 3GPP TSG RAN WG1 Meeting 109 e-Meeting

- 3GPP TS 38214, “Physical layer procedures for data”, V17.2.0

- 3GPP TS 37.355, “LTE Positioning protocol (LPP)”, V17.1.0