3GPP TSG RAN WG1 #110	R1-2206801

# Toulouse, France, August 22nd – 26th, 2022

Agenda Item:		8.8

Source:				Samsung

Title:					Joint channel estimation for PUSCH and PUCCH

Document for:	Discussion

# Introduction

This contribution discusses UL transmit power for PUSCH and PUCCH DM-RS bundling.

# Discussion

To perform joint channel estimation across repetitions of a PUSCH transmission, a same power and phase should be preserved over the repetitions. RAN1 introduced a time domain window (TDW) during which a UE is expected to maintain power consistency and phase continuity.

In RAN1#107-e [1] it was agreed the following WA:

Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW.

The above WA states that, when DMRS bundling is enabled, PUSCH repetitions during a time domain window are transmitted with a same power, and the power of PUSCH repetitions can change when the time domain window changes. The UE accumulates group common TPC commands that are received within the time domain window as per Rel-15 procedure described in TS 38.213 clause 7 but the transmit power should not be updated during the time domain window.

Discussions continued in RAN1#107bis-e and RAN1#108-e on how to capture the WA in the UL TPC procedure in TS 38.213. RAN1 did not reach an agreement on that, and in RAN1#109-e, it was concluded the following [2]:

Conclusion - No consensus on confirming the following working assumption in R17.

The UE behavior for DM-RS bundling is described in TS 38.214 clause 6.1.7 as “The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition…”  This implies that the UE does not update the transmit power during the time domain window. At a minimum it should be clarified in the specifications that the UE does not apply UL TPC procedures specified in TS 38.213 clause 7 to update the transmit power during the time domain window. Instead of adding text in TS 38.213 where DM-RS bundling is not even defined, we propose to add in TS 38.214 that during the time window the UE transmits with same power and does not apply UL TPC procedures described in clause 7 of TS 38.213 to update the transmit power.

Thus, we propose to add the following text in clause 6.1.7 of TS 38.214. A corresponding draft CR is submitted in [3].

Proposal 1: Agree to include the following text in TS 38.214:

## 6.1.7 Resource allocation in time domain ## *** Unchanged text is omitted *** - When PUSCH-Window-Restart is enabled, the start of a new actual TDW is the first symbol of the PUSCH transmission after the event which causes power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, and the PUSCH transmission is in a slot for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots. ## The UE maintains same power and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the actual TDW, and does not apply UL TPC procedures described in clause 7 of TS 38.213 to update the transmit power after a first slot or a first repetition for PUSCH repetition type B within the actual TDW.

# Conclusion

This contribution discusses UL transmit power for PUSCH and PUCCH DM-RS bundling. It is proposed the following:

Proposal 1: Agree to include the following text in TS 38.214:

## 6.1.7 Resource allocation in time domain ## *** Unchanged text is omitted *** - When PUSCH-Window-Restart is enabled, the start of a new actual TDW is the first symbol of the PUSCH transmission after the event which causes power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, and the PUSCH transmission is in a slot for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots. ## The UE maintains same power and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the actual TDW, and does not apply UL TPC procedures described in clause 7 of TS 38.213 to update the transmit power after a first slot or a first repetition for PUSCH repetition type B within the actual TDW.

# References

- Chairman’s notes, RAN1#107-e, November 2021.

- Chairman’s notes, RAN1#109-e, May 2022.

- R1-2206802, “Draft CR on transmit power for DMRS bundling”, Samsung, RAN1#110, August 2022.