3GPP TSG RAN WG1 #110	R1-2206841

# Toulouse, France, August 22nd – 26th, 2022

Agenda item:	9.8.2

Source:	Samsung

Title:	L1/L2 signaling for side control information

Document for:	Discussion and Decision

# Introduction

In RAN#94-e, a new study item on NR network-controlled repeaters was endorsed with the following objectives [1].

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions: - Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios. - For only single hop stationary network-controlled repeaters - Network-controlled repeaters are transparent to UEs - Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters. Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1] - Beamforming information - Timing information to align transmission / reception boundaries of network-controlled repeater - Information on UL-DL TDD configuration - ON-OFF information for efficient interference management and improved energy efficiency - Power control information for efficient interference management (as the 2nd priority) Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1] Study the following aspects of network-controlled repeater management - Identification and authorization of network-controlled repeaters [RAN2, RAN3] NOTE2: Coordination with SA3 may be needed.

In this contribution, we provide our view on configuration aspects and signalling design for network-controlled repeaters, referred to as “NCR” in the following.

# Configuration aspects for NCR

Functional architecture of the NCR was agreed in the previous RAN1 meeting [1], as shown in Figure 1. In particular, NCR consists of two functional entities: one is NCR mobile termination (NCR-MT) and the other is NCR forward unit (NCR-Fwd). NCR-MT can communicate with a gNB via Control link (C-link) to enable the information exchanges (e.g., the NCR control information); NCR-Fwd can perform the amplify-and-forward of UL/DL RF signal between gNB and UE via backhaul link and access link, without any digital processing (e.g., unlike an IAB-DU that performs a decode-and-forward operation). In particular, the NCR-Fwd does not create a new cell, and operates in a transparent manner to the UE(s). Moreover, the NCR-Fwd is controlled by the NCR control information, also referred to “side control information”, such as beamforming information, ON-OFF information, and so on.

Figure 1. Functional architecture of a network-controlled repeater (NCR)

For configuration aspects of the NCR, the following were agreed in RAN1#109-e [2].

Agreement The NCR-MT can obtain the necessary configuration for receiving the L1/L2 signaling of the side control information. - Option 1: The necessary configuration is from RRC. - Option 2: The necessary configuration is from OAM or hard-coded. - Option 3: The necessary configuration is partially configured by RRC and partially configured by OAM or hard-coded. Agreement For an NCR-MT, the necessary configurations from RRC and/or OAM(or hard-coded) contain: - The configurations of PHY channels to carry the L1/L2 signaling: - The configurations for receiving PDCCH and PDSCH. - The configurations for transmitting PUCCH, if needed. - The configurations for transmitting PUSCH, if needed. - The configurations of L1/L2 signaling: - The configurations for DCI. - The configurations for UCI, if needed. - The configurations for MAC CE, if needed. Agreement For the parameters in the necessary configurations for L1/L2 signaling, the existing parameters for PDCCH, PDSCH, PUCCH, PUSCH, DCI, UCI and MAC CE in Rel-17 are the baseline for further discussion. - Note 1: This does not imply that all Rel-17 parameters will be supported for the NCR-MT. - Note 2: This does not imply that PUCCH, PUSCH, UCI and MAC CE are currently agreed to be supported. Further consideration is needed.

Some additional aspects need to be studied about the configuration of the NCR.

A first aspects is about configuration of UL channels. Although NCR control information (NCI) being discussed in this study item, such as the beamforming information, the ON/OFF indication and so on, are in the form of downlink NCI (DL-NCI) from the gNB to the NCR-MT, it is expected that the NCR-MT will need to transmit uplink NCR control information (UL-NCI) to the gNB. Such UL-NCI includes at least confirmation (e.g., in the form of HARQ-ACK feedback) of the DL NCI, otherwise the repeater would be simply a network-informed repeater, rather than a network-controlled repeater. The UL NCI can include other legacy UCI, such as CSI for the C-link or backhaul link, or possibly also assistance information from the NCR to the gNB for better configuration or scheduling of (candidate) UE(s) that are served through the NCR, as discussed in [5].

Observation 1: The NCR-MT transmits uplink NCR control information (UL NCI) to the gNB, in addition to receiving downlink NCR control information (DL NCI) from the gNB; otherwise, the repeater would be simply a network-informed repeater, rather than a network-controlled repeater.

Proposal 1: Support configuration of UL channels including PUSCH and PUCCH, as well as UL control information UCI and UL MAC CE, as necessary information for the NCR-MT.

In addition to PHY channels, configuration of PHY reference signals (RSs) appears to be necessary for NCR-MT. Operation of NCR-MT, such as reception of PDCCH/PDSCH or transmission of PUSCH/PUCCH, relies on NCR-MT procedures for link adaptation, beam management, and so on, similar to that for a ‘normal’ UE. Some aspects of these procedures can be based on SSB information that the NCR-MT can already receive from cell-specific system information, without any additional signaling. However, at least in some cases, such procedures are based on reception of CSI-RS or transmission of SRS, especially when the NCR operates in FR2 as the main use-case for NCR operation. For example, NCR-MT needs to establish the beam for C-link and accordingly the backhaul link, which can be adaptive as agreed in the previous RAN1 meeting [2]. Also, SRS transmission may be also necessary if the NCR-MT reports a capability for beam correspondence with UL beam sweeping. Therefore, the NCR-MT needs to be provided with configuration information of dedicated CSI-RS and SRS (not to be confused with the CSI-RS and SRS configuration of the UEs served by the NCR). Due to cost considerations, it may be sufficient for an NCR-MT to support only a subset of Rel-17 features/functionalities for CSI-RS reception and SRS configuration, such as only aperiodic CSI-RS and aperiodic SRS.

Observation 2: The NCR-MT requires CSI-RS and SRS configuration for some procedures, such as C-link and backhaul link beam management in FR2.

Proposal 2: Support configuration of CSI-RS and SRS as necessary information for the NCR-MT.

A third aspect is about initial/random access procedure for NCR-MT. Based on the protocol architecture design for the NCR, it is possible that NCR-MT is capable of performing initial/random access procedures, and therefore, the NCR-MT is also responsible for the identification and authorization procedures for the NCR in the network. For example, the NCR-MT can establish the connection for NCR with the gNB, based on the NCR identity, and also establish an initial timing and beam for the NCR operation on the gNB-to-NCR link. Random access procedure may be also used, for example, for re-establishing uplink timing alignment or for NCR-MT beam failure recovery procedure, when applicable. The NCR-MT can receive the PRACH configuration from the cell-specific system information, so additional configuration signaling may not be necessary.

Observation 3: The NCR-MT performs random access procedure for initial access, identification and authorization procedures, re-establishing uplink timing alignment, or NCR-MT beam failure recovery procedure.

Proposal 3: Support random access procedure for NCR-MT that can be based on the cell-specific system information, without the need for additional configuration signaling.

The PHY design for NCR needs to consider the operating frequency for NCR and the interactions of NCR-MT and NCR-Fwd, such as hardware interactions and procedural interactions, including resource multiplexing aspects. The following were agreed in RAN1#109-e about PHY design principles for the NCR.

Agreement Recommend to capture the following examples of the transmission/reception of C-link and backhaul link by NCR in TR 38.867. - The DL of C-link and DL of backhaul link can be performed simultaneously or in TDM way. - The UL of C-link and UL of backhaul link can be performed in TDM way - Note-1: Multiplexing is under the control of gNB with consideration for NCR capability - Note-2: Simultaneous transmission of the UL of C-link and UL of backhaul link is subject to NCR’s capability Agreement Capture the following assumption of network-controlled repeater in TR 38.867. - At least one of the NCR-MT’s carrier(s) should be within the set of carriers forwarded by the NCR-Fwd in same frequency range. - The NCR-MT and NCR-Fwd operating in the same carrier is prioritized for the study. Agreement Capture the following assumption of network-controlled repeater in TR 38.867. - As baseline, same large-scale properties of the channel, i.e., channel properties in Type-A and Type-D (if applicable), are expected to be experienced by C-link and backhaul link (at least when the NCR-MT and NCR-Fwd operating in same carrier).

Different designs for the NCR hardware implementation can be considered. A first type of NCR implementations considers a shared RF path/chain for both the NCR-Fwd and the NCR-MT, while a second type of NCR implementations consider two separate or independent RF paths/chains for the NCR-Fwd and NCR-MT. The first type is beneficial due to reduced cost (one less RF path), and also because such design would avoid the need for special handling for self-interference, such as inter-modulation issues, from NCR-MT to NCR-Fwd and vice versa, which is present for the second type of NCR implementation. On the other hand, the second type is beneficial, since the separate RF paths allow for a simpler and more flexible RF design. Figure 3 shows the first type of NCR implementation with shared RF chains. The NCR-MT receives the DL NCI by extracting and digital processing certain time/frequency resources from of the downlink RF path from the gNB, and transmits the UL NCI by inserting the corresponding information in the uplink RF path to the gNB.

Observation 4: An NCR hardware implementation can be based on a shared RF path/chain for both NCR-Fwd and NCR-MT, or two separate RF paths/chains for the NCR-Fwd and NCR-MT.

Figure 3: An NCR design based on shared RF path for NCR-MT and NCR-Fwd

In addition, per RAN4 work item on Rel-17 NR repeaters [3, 4], it is understood that an NCR-Fwd can operate in a single “pass band” or in multiple non-contiguous “pass bands”, which are in the same frequency band for FR2 operation, and can be in same or different frequency bands for FR1 operation. Such flexible frequency domain settings needs to be considered in the study for NCR configuration and L1/L2 signaling.

Observation 5: An NCR-Fwd can operate with single or multiple “pass bands”.

Proposal 4: Consider different hardware implementations (such as same or different RF chains for NCR-MT and NCR-Fwd) and different frequency-domain settings (such as single or multiple pass bands for NCR-Fwd) in the design of configuration and L1/L2 signaling for NCR.

# L1/L2 signaling design for NCR

The following general categories can be considered as candidate L1/L2 signaling mechanisms for the NCR:

- Signaling based on control channel only, such as PDCCH only;

- Signaling based on data channel only, such as PDSCH only (e.g., SPS PDSCH with higher layer activation);

- Signaling based on both control and data channel, such as PDCCH + PDSCH (e.g., SPS PDSCH with DCI activation or dynamically scheduled PDSCH).

In addition, when PDSCH is selected as (part of) the NCR signaling method, a simple method is to treat the DL NCR control information (DL NCI) as L2 control information which is included in the PDSCH in the form of a MAC CE. Corresponding methods can be used for signaling of UL NCR control information (UL NCI).

Selection of a method for L1/L2 signaling for NCR among the above candidates needs to consider a number of factors such as the following:

- a payload size of the NCI: smaller payload size for NCI better fits a PDCCH, while larger payload sizes are best communicated via a PDSCH;

- a periodicity or time-domain ‘density’ of the NCI: long-term and less frequent NCR signaling can be communicated on a PDCCH only or dynamically scheduled PDSCH, while short-term and more frequent NCR signaling is best exchanged via a SPS PDSCH with higher layer or DCI activation, for more efficient resource utilization;

- a complexity associated with NCI signaling: exchange of DL NCI with less need for PDCCH blind decoding, such as by SPS PDSCH, reduces the NCR implementation cost/complexity, while signaling of DL NCI based on frequent PDCCH blind decoding increases the NCR implementation cost/complexity;

- a supported protocol stack: an NCR-MT with a protocol stack supporting at least up to L2 can readily handle MAC-CEs, but an NCR-MT supporting only L1 needs to exchange ‘DL NCI format’ as L1 information on a PDCCH;

- specification efforts: specification of (new) MAC CEs for exchange of NCI is more straightforward (e.g., similar to MAC CEs for IAB), while specification of “DL NCI formats” may need additional efforts.

Observation 6: Selection of a signalling method for the NCR needs to consider at least: a payload size of the NCI, time-domain periodicity/density of the NCI, a complexity associated with NCI signaling, a protocol stack supported by the NCR-MT, and the required specification efforts.

The RAN1 study on NCR should study the various possible L1/L2 signaling methods considering impacts on signalling overhead, resource utilization, cost/complexity considerations, and specification efforts. Based on the discussion above, including DL NCI as MAC CEs on a SPS PDSCH appears to be an efficient approach.

Proposal 5: Study L1/L2 signalling methods for NCR-MT considering impacts on signalling overhead, resource utilization, cost/complexity considerations, and specification efforts.

- Candidate signalling methods for the NCR include: PDCCH only, PDSCH only; or PDCCH + PDSCH;

- When PDSCH is used as (part of) the NCR signalling method, the downlink NCR control information can be L2 control information in the form of a MAC CE.

Based on the discussions in our companion Tdoc [5], the two main types of NCR control information include beamforming information and ON-OFF information, for which the following were agreed in the RAN1#109-e [2].

Agreement Both the dynamic indication and semi-static indication can be considered for the beam of access link for NCR-Fwd. - FFS: the details of each indication - FFS: the maximum number of beams configured for NCR-Fwd access link Agreement The beam correspondence is assumed for the DL/UL of the access link at NCR-Fwd Agreement From the perspective of signaling design, following mechanisms can be considered for the access link beamforming of the NCR-Fwd. - Option #2-1: Dynamic beam indication only - Option #2-2: Semi-static beam indication only - Option #2-3: Dynamic beam indication and semi-static beam indication Agreement As for the time-domain granularity of the access link beam indication, one or both of the following options can be considered: - Option 1: slot-level - Option 2: symbol-level - FFS: The details of indication signalling

Agreement ON-OFF information is beneficial and recommended for network-controlled repeater to control the behaviour of NCR-Fwd. - FFS: Detailed mechanism of ON-OFF indication and determination - FFS: explicit indication or implicit indication of ON-OFF information Agreement The following options can be considered to indicate the ON-OFF information from gNB to NCR for controlling the behaviour of NCR-Fwd: - Option 1: Explicit indication with on-off state (e.g., via dynamic or semi-static signalling) or on-off pattern (e.g., periodic/semi-static ON-OFF pattern or new DRX-like pattern for ON-OFF) - Option 2: Implicit indication via the signalling for other information (e.g., beam, DL/UL configuration, or PC information) - Note: This example does not imply that PC information is necessary or not. - Other solutions (e.g., potential combination of explicit and implication solution) can be further discussed. Agreement For indication of NCR-Fwd ON-OFF for efficient interference management and improved energy efficiency, both dynamic and semi-static indication can be considered - FFS: RAN1 to consider whether/how to handle the forwarding of broadcast and cell-specific signals/channels.

A first issue is about the interaction of higher layer or semi-static signalling corresponding to different types of NCR control information and interaction of L1/L2 or dynamic signalling corresponding to different types of NCR control information. It is expected that the semi-static signalling for beamforming information and semi-static signalling for ON-OFF information are consistent. For example, the NCR-MT does not expect to be provided with semi-static signalling that indicates an OFF state in a symbol/slot for which the UE receives semi-static indication of a certain beam for the access link or the backhaul link. Same holds for dynamic interaction of dynamic beam indication and dynamic ON-OFF indication, and also for implicit beam indication or ON-OFF indication.

Proposal 6: The NCR-MT expects that:

- semi-static signallings corresponding to different types of NCR control information (e.g., beamforming, ON-OFF) are consistent;

- dynamic signallings corresponding to different types of NCR control information (e.g., beamforming, ON-OFF) are consistent.

Such consistency can be further considered for the case of implicit ON-OFF indication. For example, a default behaviour can be for the NCR-Fwd to be ON during symbols/slots corresponding to cell-specific transmission or receptions, such as SSB receptions, PRACH transmissions, monitoring occasions (MOs) for reception of PDCCH for system information, paging, and so on. For interference management purposes, it is expected that the NCR-Fwd is ON only for such cell-specific transmissions that correspond to a number of beams / spatial filters, such as a number of SSB indexes, that the NCR-Fwd is configured for the amplify-and-forward operation in the NCR coverage area. The NCR may be provided, for example by semi-static signalling, information of additional SSB indexes, so that the NCR-Fwd is ON also during symbols/slots corresponding to the additional SSB indexes. For example, the additional SSB indexes can be “neighbour” SSB indexes to determine UE activity in proximity of NCR coverage area. The NCR-Fwd is expected to be OFF during cell-specific transmissions and receptions corresponding to any other SSB indexes. As discussed next, if needed, the gNB can override such default NCR behavior using dynamic ON-OFF indication.

Proposal 7: The NCR-Fwd is, by default, ON during cell-specific transmissions and receptions that correspond to SSB indexes configured for the amplify-and-forward operation, and possibly additional SSB indexes configured by semi-static signalling for gNB information purposes. The NCR-Fwd is, by default, OFF during cell-specific transmissions and receptions that correspond to any other SSB indexes.

A second issue is about the interaction of higher layer / semi-static signalling with L1/L2 or dynamic signalling corresponding to the same or different types of NCR control information. Dynamic signalling of NCR control information reflects gNB intention for short-term or real-time control of the NCR, for example, to handle the impact of UE mobility, arrival of new UEs, or departure of existing UEs from the NCR coverage area. Therefore, it is possible that a dynamic signalling is not consistent with previous semi-static signalling. In such “collision” cases, the dynamic signalling overrides the semi-static signalling and the NCR operates based on the indication provided by the dynamic signalling. For example, when semi-static signalling indicates access beam #1 for a given slot, say slot #10, and a subsequent dynamic signalling indicates access beam #2 for slot#10, the NCR operates with the access beam #2. For example, when semi-static signalling indicates an OFF state for a given slot, say slot #10, and a later dynamic signalling provides an access/backhaul beam indication for slot#10, the NCR determines that NCR-Fwd is in ON state during slot#10. Same holds with respect to implicit beam indication or ON-OFF indication.

Observation 7: The gNB uses dynamic signalling of NCR control information for short-term or real-time handling of the NCR, such as for UE mobility, arrival/departure of UEs to/from the NCR coverage area.

Proposal 8: When a dynamic signalling for an NCR control information is not consistent with a semi-static signalling for the NCR for the same or a different type of NCR control information, the dynamic indication overrides the semi-static indication.

Frequent application of dynamic signalling to override semi-static signalling for the same or different types of NCR control information can degrade the usefulness of semi-static signalling and can impact the NCR implementation operation due to the NCR requirement for timely reaction to frequent changes in the NCR control information. Therefore, it can be beneficial to assign “resource status” labels, such as Hard or Soft, to different symbols/slots with associated semi-static signalling for beam indication or for ON-OFF indication, in order to limit the impact of dynamic signalling. For example, for symbols/slots indicated as “Hard”, the NCR-Fwd applies the semi-static beam/ON-OFF indication, and does not expect to receive a dynamic signalling for the symbols/slots (that provide inconsistent indication). For symbols/slots indicated as “Soft”, the NCR-Fwd may (or may not) receive dynamic signalling that can be inconsistent with (thereby overriding) the semi-static signalling for the symbols/slots. Alternatively, semi-static ON-OFF indication values can be extended to {ON, OFF, Standby}, so that for symbols/slots semi-statically indicated as ON or OFF, the NCR-Fwd applies the semi-static beam/ON-OFF indication, and does not expect to receive a dynamic signalling for the symbols/slots (that provide inconsistent indication). For symbols/slots semi-statically indicated as Standby, the NCR determines the ON or OFF state based on dynamic ON-OFF indication. When the NCR does not receive an indication for a Standby symbol/slot, the NCR-Fwd is set to OFF.

Proposal 9: Support mechanisms for determination of slots/symbols for which semi-static signalling for beamforming or ON-OFF indication may or may not be overridden by a subsequent dynamic signalling.

- Example mechanisms include configuration of “Hard” or “Soft” labels for symbols/slots, or configuration of a Standby state for NCR in addition to ON and OFF states.

A last issue is about the impact of NCR on the signalling that gNB provides to the NCR, such as NCR-initiated or NCR-assisted indication. The baseline operation for NCR is based on full knowledge of gNB about the UEs in the serving cell, including those served by the NCR. However, it is possible that the NCR can determine local knowledge that may not be available to the gNB or may be available to the gNB with some latency. For example, when a UE in the NCR coverage area attempts initial/random access while the NCR-Fwd is indicated to be OFF, it is possible that the PRACH transmission from the UE fails and the gNB does not recognize the presence of the UE. However, the NCR (with sufficient capability) may be able to detect the presence of the UE, by performing sequence detection or by power measurements (such as EPRE measurements) of the PRACH transmission from the UE. Similar can be considered for NCR measurement of UE SRS resources. In such case, it is beneficial for the NCR to be able to transmit a “forwarding request” indication (FRI) to the gNB, to request to switch from the OFF state to the ON state. Herein, “forwarding request” is similar to UE procedure for scheduling request (SR) that can be used, for example, for disrupting DRX or PDCCH skipping. It can be up to gNB how to handle such FRI, or certain rules can be considered in the specifications for handling FRI.

Proposal 10: Support NCR-initiated or NCR-assisted signalling, such as transmission of a “forwarding request” indication (FRI) by the NCR-MT to the gNB to request for NCR-Fwd to go from OFF state to ON state.

# Conclusion

This contribution considered architecture aspects and signalling design for network-controlled repeaters (NCR), and proposed the following.

Proposals

Proposal 1: Support configuration of UL channels including PUSCH and PUCCH, as well as UL control information UCI and UL MAC CE, as necessary information for the NCR-MT.

Proposal 2: Support configuration of CSI-RS and SRS as necessary information for the NCR-MT.

Proposal 3: Support random access procedure for NCR-MT that can be based on the cell-specific system information, without the need for additional configuration signaling.

Proposal 4: Consider different hardware implementations (such as same or different RF chains for NCR-MT and NCR-Fwd) and different frequency-domain settings (such as single or multiple pass bands for NCR-Fwd) in the design of configuration and L1/L2 signaling for NCR.

Proposal 5: Study L1/L2 signalling methods for NCR-MT considering impacts on signalling overhead, resource utilization, cost/complexity considerations, and specification efforts.

- Candidate signalling methods for the NCR include: PDCCH only, PDSCH only; or PDCCH + PDSCH;

- When PDSCH is used as (part of) the NCR signalling method, the downlink NCR control information can be L2 control information in the form of a MAC CE.

Proposal 6: The NCR-MT expects that:

- semi-static signallings corresponding to different types of NCR control information (e.g., beamforming, ON-OFF) are consistent;

- dynamic signallings corresponding to different types of NCR control information (e.g., beamforming, ON-OFF) are consistent.

Proposal 7: The NCR-Fwd is, by default, ON during cell-specific transmissions and receptions that correspond to SSB indexes configured for the amplify-and-forward operation, and possibly additional SSB indexes configured by semi-static signalling for gNB information purposes. The NCR-Fwd is, by default, OFF during cell-specific transmissions and receptions that correspond to any other SSB indexes.

Proposal 8: When a dynamic signalling for an NCR control information is not consistent with a semi-static signalling for the NCR for the same or a different type of NCR control information, the dynamic indication overrides the semi-static indication.

Proposal 9: Support mechanisms for determination of slots/symbols for which semi-static signalling for beamforming or ON-OFF indication may or may not be overridden by a subsequent dynamic signalling.

- Example mechanisms include configuration of “Hard” or “Soft” labels for symbols/slots, or configuration of a Standby state for NCR in addition to ON and OFF states.

Proposal 10: Support NCR-initiated or NCR-assisted signalling, such as transmission of a “forwarding request” indication (FRI) by the NCR-MT to the gNB to request for NCR-Fwd to go from OFF state to ON state.

In addition, the following observations were made.

Observations

Observation 1: The NCR-MT transmits uplink NCR control information (UL NCI) to the gNB, in addition to receiving downlink NCR control information (DL NCI) from the gNB; otherwise, the repeater would be simply a network-informed repeater, rather than a network-controlled repeater.

Observation 2: The NCR-MT requires CSI-RS and SRS configuration for some procedures, such as C-link and backhaul link beam management in FR2.

Observation 3: The NCR-MT performs random access procedure for initial access, identification and authorization procedures, re-establishing uplink timing alignment, or NCR-MT beam failure recovery procedure.

Observation 4: An NCR hardware implementation can be based on a shared RF path/chain for both NCR-Fwd and NCR-MT, or two separate RF paths/chains for the NCR-Fwd and NCR-MT.

Observation 5: An NCR-Fwd can potentially operate in multiple non-contiguous “pass bands”.

Observation 6: Selection of a signalling method for the NCR needs to consider at least: a payload size of the NCI, time-domain periodicity/density of the NCI, a complexity associated with NCI signaling, a protocol stack supported by the NCR-MT, and the required specification efforts.

Observation 7: The gNB uses dynamic signalling of NCR control information for short-term or real-time handling of the NCR, such as for UE mobility, arrival/departure of UEs to/from the NCR coverage area.

# References

- RP-213700, “New SI: Study on NR Network-controlled Repeaters”, ZTE Corporation.

- Chair notes, RAN1#109-e.

- RP-210818, “New WID on NR Repeaters”, Qualcomm.

- RP-220544, “WI summary for WI: NR Repeaters”, Qualcomm.

- R1-2206840, “Side control information to enable NR network-controlled repeaters”, Samsung.