3GPP TSG RAN WG1 #110			R1-2206922

Toulouse, France, August 22nd – 26th, 2022

Agenda item:	9.5.3

Source: 	CMCC

Title: 	Discussion on RedCap positioning

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new SID on study on expanded and improved NR positioning was approved [1]. The motivation to evaluate the performance and identify performance gaps for RedCap UEs supporting NR positioning functionalities is justified:

- Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In RAN1#109-e meeting, the following agreements on the evaluation assumptions on RedCap UE positioning were made [2]:

Agreement For evaluation of RedCap UE positioning performances, all RAT based positioning methods can be considered. Sources should detail the chosen method(s) when presenting performance evaluations. Agreement For evaluation of positioning performance of redcap UEs, adopt the general parameters are detailed in the table below - TBD parameters are discussed separately Table 6-1: Common scenario parameters applicable for all scenarios for Redcap UEs evaluations FR1 Specific Values; FR2 Specific Values. Carrier frequency, GHz; 3.5GHz, 700MHz (optional) Note 1; 28GHz Note 1. Bandwidth, MHz; TBD; TBD. Subcarrier spacing, kHz; 30KHz, 15KHz (for 700MHz carriers); 120kHz. gNB model parameters. gNB noise figure, dB; 5dB; 7dB. UE model parameters. UE noise figure, dB; 9dB – Note 1; 13dB – Note 1. UE max. TX power, dBm; 23dBm – Note 1; 23dBm – Note 1 EIRP should not exceed 43 dBm.. UE antenna radiation pattern; Omni, 0dBi; Antenna model according to Table 6.1.1-2 in TR 38.855. PHY/link level abstraction; Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters.. Network synchronization; The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional). UE/gNB RX and TX timing error; (Optional) The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1: - T1: X ns for gNB and Y ns for UE - X and Y are up to sources - Note: RX and TX timing errors are generated per panel independently Apply the timing errors as follows: - For each UE drop, - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution. - For each gNB - For each panel (in case of multiple panels) - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution. - Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report. - For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms.. Note 1: According to TR 38.802 Note 2: According to TR 38.901. Agreement For the evaluation of RedCap positioning, the following bandwidth can be evaluated: - FR1: 20MHz baseline, 5MHz optional - FR2: 100MHz Agreement Adopt the following table for the UE model parameters FR1 Specific Values; FR2 Specific Values. UE model parameters. UE antenna configuration; Panel model 1 – Note 1 dH = 0.5λ, for 1Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 1, 1, 1) for 2Rx UEs: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1); - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) as minimum antenna configuration (baseline) - (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1) as optional configuration.. UE antenna radiation pattern; Omni, 0dBi; Antenna model according to Table 6.1.1-2 in TR 38.855. Number of UE branches; Baseline: 1Rx 1Tx Optional: 2Rx 1 Tx; TBD. Note 1: According to 3GPP TR 38.802. Agreement The following scenarios are evaluated for positioning performance of Redcap - Baseline: (Case 1): Umi street canyon, as described in Table 6.1-1-4 of 38.855 - Optional outdoor: - (Case 2): Uma, as described in Table 6.1-1-6 of 38.855 - (Case 3): Rma (FFS details of the scenario) - Baseline: (Case 4): InF-SH as described in Table 6.1-1 of 38.857 - Optional indoor: (Case 5) Indoor Open Office, as described in Table 6.1-1-3 of 38.855 - Optional indoor: (Case 6) InF-DH as described in Table 6.1-1 of 38.857 Agreement The FR2 UE antenna configuration is as follow: - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) as minimum antenna configuration (baseline) - (M, N, P, Mg, Ng) = (2, 2, 2, 1, 1) as optional configuration. Agreement The evaluation methodology for RedCap UEs positioning performance uses DL PRS and/or UL SRS for positioning. - The methodology does not define any baseline reference signal configuration. Sources should detail the chosen configuration of reference signal(s) when presenting performance evaluations. Agreement For evaluation of positioning performance of redcap UEs in 700MHz band, the gNB antenna model is: - gNB antenna configuration from TR38.830, (M,N,P,Mg,Ng) = (4,2,2,1,1), (dH, dV) = (0.5, 0.8)λ Agreement Use 2Rx and 1Tx for baseline number of UE branches in FR2 in the UE antenna configuration table for RedCap UEs evaluation. - FFS: optional configurations for number of UE branches in FR2.

However, in the last RAN1 meeting, as companies expressed different views on use cases and target requirements for RedCap UE positioning, no agreements were reached. In this contribution, we continually discuss on the use cases and requirements to assess the necessity of the enhancements.

Discussion

- 2.1 Use cases and requirements

In the objectives of RedCap positioning, the positioning performance of RedCap UEs will be evaluated to assess the necessity of further enhancements. In our views, typical use cases and requirements of RedCap UE positioning should be first discussed to facilitate the evaluation and the performance gap identification.

Observation 1: Typical use cases and requirements should be defined to assess the necessity of further enhancements.

Typical use cases of RedCap positioning include commercial use cases, such as the wearables, and industrial use cases, such as the logistics and warehousing, plant asset management, positioning for personal safety undermine, and real-time tracking of vehicle locations on the assembly line, etc.

For wearable device applications, the following positioning requirements are provided in TS 22.872 [3]:

| Use cases |  | Potential requirements per use cases |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Environment of Use | Position Accuracy | Velocity | Avail. | Update rate or interval | TTFF | Latency | Other KPI |
| 5.2.3 | Wearables | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 90 % | 30s - 300s | 10s |  | Power saving mode |
|  |  | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 99 % | 1s - 30s | 10s | 1s | Normal mode |

For the industrial use cases, based on our preliminary investigations, the industry has urgent needs on high positioning performance (e.g., <1m), long battery life (e.g., up to 12 month), and low device cost (e.g., up to 10~20 dollars).

In our views, a reasonable requirement can be referred to that defined for Rel-17 commercial use cases [4]:

    - Horizontal position accuracy (< 1 m) for 90% of UEs

    - Vertical position accuracy (< 3 m) for 90% of UEs

Proposal 1: The requirements of the RedCap UE positioning is defined as:

    - Horizontal position accuracy (< 1 m) for 90% of UEs

    - Vertical position accuracy (< 3 m) for 90% of UEs

- 2.2 Potential enhancements

Based on the evaluation in Rel-16/17 SI, it seems that it may have risk of a RedCap UE with 20MHz bandwidth and low Tx/Rx capability to achieve meter level positioning accuracy. In the following, we discuss some potential enhancements on RedCap UE positioning.

The most straightforward solution is to support SRS frequency hopping, by which a larger bandwidth can be stitched at the NW side. Note that the phase continuity may be deteriorated by the frequency hopping, and solutions to estimate and calibrate the phase distortion should be further studied.

In addition, the carrier phase positioning is under investigation under AI 9.5.2.2, if the performance of carrier phase positioning is not bandwidth sensitive, it can be considered as another solution to improve the positioning performance of Redcap UEs.

Proposal 2: Further study enhancements of RedCap UE positioning:

    - SRS frequency hopping

    - Carrier phase positioning

Conclusions

In this contribution, we provide our views on the requirements of RedCap positioning, and the following observation and proposals are made:

Observation 1: Typical use cases and requirements should be defined to assess the necessity of further enhancements.

Proposal 1: The requirements of the RedCap UE positioning is defined as:

    - Horizontal position accuracy (< 1 m) for 90% of UEs

    - Vertical position accuracy (< 3 m) for 90% of UEs

Proposal 2: Further study enhancements of RedCap UE positioning:

    - SRS frequency hopping

    - Carrier phase positioning

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, 3GPP TSG RAN Meeting #94e.

- Chair’s Notes, RAN1#109-e 9.5 EOM2.

- TS 22.872, Study on positioning use cases (Stage 1), v16.1.0.

- TR 38.857, Study on NR Positioning Enhancements, v1.0.0.