3GPP TSG RAN WG1 Meeting #110 	 R1-2207005

Toulouse, France, August 22nd – 26th, 2022

Agenda Item: 7.2.10

Source: MediaTek Inc.

Title: 	On Type-1 HARQ-ACK codebook in PUCCH for SCell dormancy

Document for: Discussion and decision

Introduction

In this contribution, we discuss the Type-1 HARQ-ACK codebook determination in PUCCH for SCell dormancy. In 38.331 V16.9.0 [1], UE is not expected to be configured with PDCCH monitoring for an SCell dormant BWP. In 38.213 V16.10.0 [2] 9.1.2.1, when determining the provided K1 values for Type-1 HARQ-ACK codebook, it is assumed UE would monitor at least one of DCI 1_0/1_1/1_2. However, when an SCell enters a dormant BWP, UE does not monitor any of DCI 1_0/1_1/1_2, and it is not clear which set of K1 values UE should assume. The intention of this document is to fix this issue on Type-1 HARQ-ACK codebook determination in PUCCH for SCell dormancy in 38.213.

Discussion on Type-1 HARQ-ACK codebook in PUCCH for SCell dormancy

In 38.331 V16.9.0 [1], UE is not expected to be configured with PDCCH monitoring for an SCell dormant BWP as shown below.

In 38.213 V16.10.0 [2] 9.1.2.1, when determining the provided K1 values for Type-1 HARQ-ACK codebook, it is assumed UE would monitor at least one of DCI 1_0/1_1/1_2, as shown below in highlighted sentences:

- 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values K1 associated with the active UL BWP

a)	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 on serving cell c, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}

b)	If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK or dl-DataToUL-ACK-r16

c)	If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-DCI-1-2

d)	If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK or dl-DataToUL-ACK-r16 and dl-DataToUL-ACK-DCI-1-2

e)	If an inapplicable value in dl-DataToUL-ACK-r16 is provided, the value is excluded from K1

;

However, when an SCell enters a dormant BWP, UE does not monitor any of DCI 1_0/1_1/1_2, and it is not clear which set of K1 values UE should assume.

Observation 1: In 38.331 V16.9.0 [1], UE is not expected to be configured with PDCCH monitoring for an SCell dormant BWP. In 38.213 V16.10.0 [2] 9.1.2.1, when determining the provided K1 values for Type-1 HARQ-ACK codebook, it is assumed UE would monitor at least one of DCI 1_0/1_1/1_2. However, when an SCell enters a dormant BWP, UE does not monitor any of DCI 1_0/1_1/1_2, and it is not clear which set of K1 values UE should assume.

Hence, we have the following text proposal to fix this issue.

- 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values K1 associated with the active UL BWP

a)	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 on serving cell c, or the active DL BWP for serving cell c is dormant BWP, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}

b)	If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK or dl-DataToUL-ACK-r16

c)	If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-DCI-1-2

d)	If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK or dl-DataToUL-ACK-r16 and dl-DataToUL-ACK-DCI-1-2

e)	If an inapplicable value in dl-DataToUL-ACK-r16 is provided, the value is excluded from K1

Proposal 1: Adopt the CR below in 38.213 V16.10.0 [2] to assign a provide K1 values set when the active DL BWP for a serving cell c is dormant BWP:

Conclusion

In this contribution, we discuss the issue of “Type-1 HARQ-ACK codebook determination in PUCCH for SCell dormancy” and have the following observations and proposals:

Observation 1: In 38.331 V16.9.0 [1], UE is not expected to be configured with PDCCH monitoring for an SCell dormant BWP. In 38.213 V16.10.0 [2] 9.1.2.1, when determining the provided K1 values for Type-1 HARQ-ACK codebook, it is assumed UE would monitor at least one of DCI 1_0/1_1/1_2. However, when an SCell enters a dormant BWP, UE does not monitor any of DCI 1_0/1_1/1_2, and it is not clear which set of K1 values UE should assume.

- 9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based:

a)	on a set of slot timing values K1 associated with the active UL BWP

a)	If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for either DCI format 1_1 or DCI format 1_2 on serving cell c, or the active DL BWP for serving cell c is dormant BWP, K1 is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8}

b)	If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell c, K1 is provided by dl-DataToUL-ACK or dl-DataToUL-ACK-r16

c)	If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell c, K1 is provided by dl-DataToUL-ACK-DCI-1-2

d)	If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell c, K1 is provided by the union of dl-DataToUL-ACK or dl-DataToUL-ACK-r16 and dl-DataToUL-ACK-DCI-1-2

e)	If an inapplicable value in dl-DataToUL-ACK-r16 is provided, the value is excluded from K1

Proposal 1: Adopt the CR below in 38.213 V16.10.0 [2] to assign a provide K1 values set when the active DL BWP for a serving cell c is dormant BWP:

Reference

3GPP TS 38.331 V16.9.0, NR; Radio Resource Control (RRC); Protocol specification

3GPP TS 38.213 V16.10.0, NR; Physical layer procedures for control