3GPP TSG-RAN WG1 Meeting #110	R1-2207161

# Toulouse, France, August 22nd – 26th ,2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | DRAFT | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Frequency Hopping Offset and Interval Determination for DMRS Bundling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In section 6.3.1, regarding the Rel-17 inter-slot frequency hopping when PUSCH-DMRS-Bundling is enabled, - The determination of starting RB of a frequency hop is in square brackets, due to the concern of including system frame number in the equation. However, including SFN is beneficial. Frequency hopping intervals as long as 20 slots are supported, and so can be longer than a radio frame. Including system frame number in the equation can guarantee the balanced distribution of two hops, with a frequency hopping pattern across radio frame boundary, rather than being restarted at the boundary. On the other hand, some values of frequency hopping intervals don’t evenly divide the number of slots in a radio frame. This also causes unequal lengths of two hops in a radio frame if the slot index is limited to within a radio frame. - It was agreed in RAN1#107 that the default frequency hopping interval for the hopping pattern with DMRS bundling is determined from the time domain window length as follows; however, this is not yet captured in the specifications. - Support separate RRC configuration(s) for hopping interval and configured TDW length. - if hopping interval is not configured, the default hopping interval is the same as the configured TDW length - FFS: if both hopping interval and TDW length are not configured - Note: hopping interval is only determined by the configuration of hopping interval if hopping interval is configured |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The tentatively agreed hopping offset in the spec is confirmed. - The default value for the frequency hopping interval with DMRS bundling is set to be the length of the time domain window length, PUSCH-TimeDomainWindowLength. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Frequency hopping with DMRS bundling is not supported, since the frequency hopping offsets are not defined for this case. - The case where the UE determines a default value for the frequency hopping interval when hopping is used with DMRS bundling is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: . |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.3.1	Frequency hopping for PUSCH repetition Type A and for TB processing over multiple slots

---------------------------------------------------Unchanged Text Omitted---------------------------------------------------------

In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is enabled, and when a PUSCH is not scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot  is given by:

RBstartns,fμ=RBstart RBstart+RBoffsetmodNBWPsize   ns,fμ+nf∙Nslotframe,μNFH mod 2=0ns,fμ+nf∙Nslotframe,μNFH mod 2=1

where ns,fμ is the current slot number within a system radio frame, nf is the number of the system radio frame containing the current slot, Nslotframe,μ is the number of slots per frame for subcarrier spacing configuration μof the UL BWP that the PUSCH is transmitted on, NFH is the value of the higher layer parameter PUSCH-Frequencyhopping-Interval if configured and otherwise NFH is the value of the higher layer parameter PUSCH-TimeDomainWindowLength,  is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) andis the frequency offset in RBs between the two frequency hops.

---------------------------------------------------Unchanged Text Omitted---------------------------------------------------------