3GPP TSG-RAN WG1 Meeting #110	Tdoc R1-2207163

Toulouse, France, Aug 22nd–26th, 2022

Agenda Item:	9.1.4.2

Source:	Ericsson

Title:	SRI/TPMI Enhancements for Enabling 8 TX UL Transmission

Document for:	Discussion

# 1	Introduction

During RAN#94e, a new WID for Rel-18 MIMO evolution for DL and UL was agreed. [1]. The objective of the work item concerning 8 Tx UL transmission reads as follows:

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 TX UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/ vehicle/industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, we consider the design of mechanisms to fulfill this objective.  The benefits of increasing the maximum number of layers to 8 are evaluated, considering maximum modulation orders of 64 and 256 QAM.  Codebook design aspects such as complexity benefits and performance gains of non-coherent precoders are investigated. Suitable oversampling ratios in terms of performance and implementation complexity for codebook designs based on DL Type-1-based precoders are also studied.  Lastly, high-level aspects of non-codebook and multi-TPMI based approaches are further discussed.

# 2	Discussion

## 2.1	On the maximum number of layers for 8 TX UEs

During RAN1#109-e, the following agreement was reached:

Agreement
For 8 TX UE uplink transmission, study codebook- and non-codebook-based transmission with maximal layer number of both 4 and 8 layers.

In this section, we evaluate the possible throughput gains that can be achieved by increasing the maximum number of layers from 4 to 8 for an 8 TX UE. Specifically, we present non-full-buffer (NFB) SLS results for the case when inter-sector interference is considered at different system loads. The sets of SLS parameters used in this section, for different scenarios (i.e., “indoor FWA” and “outdoor FWA”), are collected in Appendix. It is worth pointing out that for the “outdoor FWA” scenario, all UEs are directed such that boresight of the antenna array is towards the nearest gNB. Furthermore, note that the modulation, in this section, is up to 256 QAM unless otherwise stated. This may help address comments made during RAN1#109-e that gains from 8 layers may not be present if 256 QAM is used and is discussed in detail later in this section.

We start by evaluating the potential SU-MIMO throughput gains for non-codebook (NCB) operation. In Figure 1 and Figure 2, we show mean-user UL throughput for an 8 TX UE for the “indoor FWA” and “outdoor FWA” scenario, respectively. Here, we compare two different UL schedulers: One that restricts the maximum transmission rank to 4 (such that the candidate rank is 1—4) and one that supports up to 8 layers (such that the candidate rank is 1—8). For reference, we also show the mean-user UL throughput for a legacy 4 TX UE. Based on these SLS evaluations, we make the following observation:

Increasing the maximum number of layers from 4 to 8 for NCB-based transmission yields significant throughput gains for 8 TX UEs regardless of the scenario studied, but gains are particularly high in the outdoor FWA scenario.

The throughput gains are especially pronounced in the “outdoor FWA” scenario. Indeed, 8 TX yields hardly any throughput gains compared to legacy 4 TX if the maximum transmission rank is not increased beyond 4. This is consistent with the high SNRs in this scenario: capacity gains at high SNR come from increases in layers or bandwidth rather than additional array gain. For “indoor FWA”, the additional array gain provided by the 8 TX array yields throughput gains irrespectively of whether the transmission rank is increased or not, as one would expect from the lower SNRs in this scenario. However, the gains are more pronounced, especially at high load, when the candidate rank is 1—8 (compared to 1—4).

Figure 1	Mean-user UL throughout versus served traffic, as a function of the maximum transmission rank, for NCB and “outdoor FWA” scenario. The corresponding SLS parameters are collected in Table 3 in Appendix.

Figure 2	Mean-user UL throughout versus served traffic, as a function of the maximum transmission rank, for NCB and “indoor FWA” scenario. The corresponding SLS parameters are collected in Table 4 in Appendix.

The above results are valid for NCB operation and will have an impact on the number of SRS resources, and the associated number of bits in the SRI field in DCI, that are needed. Next, we repeat the SLS evaluations for codebook (CB) operation, to determine whether there is sufficient reason to extend UL codebooks to beyond 4 layers. The results are shown in Figure 3 and Figure 4 for the “outdoor FWA” and “indoor FWA” scenario, respectively. As will be discussed in detail in Section 2.3.1, there are many ways to construct an 8 Tx UL codebook (codebook design is still FFS). Here, for simplicity, we consider the case for which the UE is fully coherent, and the 8 Tx CB is a wideband Type-1 codebook designed for a 1x4 cross-polarized ULA (i.e., N1, N2=[4,1]) with O1, O2=[2,1]. Based on these SLS evaluations, we make the following observation:

Increasing the maximum number of layers from 4 to 8 for CB-based transmission also yields significant throughput gains for 8 TX UEs regardless of the scenario studied, and is particularly the high outdoor FWA scenario.

Figure 3	Mean-user UL throughout versus served traffic, as a function of the maximum transmission rank, for FC CB and “outdoor FWA” scenario. The corresponding SLS parameters are collected in Table 3 in Appendix.

Figure 4	Mean-user UL throughout versus served traffic, as a function of the maximum transmission rank, for FC CB and “indoor FWA” scenario. The corresponding SLS parameters are collected in Table 4 in Appendix.

In Table 1 and Table 2, we summarize the UL mean-user throughput gains for NCB and CB, respectively. Here, the gains are with respect to legacy 4 Tx UL MIMO performance at low, medium, and high system resource utilization.

Table 1	UL mean-user throughput gains for NCB operation as a function of reference (legacy 4 Tx UL MIMO) resource utilization (i.e., at low, medium, and high load).

| Scheme | Scenario | 20% (low) | 50% (medium) | 70% (high) |
| --- | --- | --- | --- | --- |
| NCB, 8 TX, rank 1—4 | “Outdoor FWA” | 0% | 1% | 1% |
|  | “Indoor FWA” | 6% | 6% | 7% |
| NCB, 8 TX rank 1—8 | “Outdoor FWA” | 8% | 14% | 20% |
|  | “Indoor FWA” | 8% | 11% | 14% |

Table 2	UL mean-user throughput gains for CB operation as a function of reference (legacy 4 Tx UL MIMO) resource utilization (i.e., at low, medium, and high load).

| Scheme | Scenario | 20% (low) | 50% (medium) | 70% (high) |
| --- | --- | --- | --- | --- |
| FC CB, 8 TX, rank 1—4 | “Outdoor FWA” | 0% | 0% | 0% |
|  | “Indoor FWA” | 4% | 5% | 6% |
| FC CB, 8 TX rank 1—8 | “Outdoor FWA” | 5% | 11% | 15% |
|  | “Indoor FWA” | 6% | 9% | 11% |

As was discussed in RAN1#109-e, and introduced above, increasing the modulation order can naturally reduce the need for additional MIMO layers, at least to some degree.  Therefore, it is of interest to compare the performance of up to 8-layer transmission when 64 QAM vs. 256 QAM is used. Figure 5 below shows the performance in the “outdoor FWA” scenario, where either modulation state is used and with 4- or 8-layer transmission.  We observe that 8-layer transmission always has higher throughput than 4-layer transmission, even when 8 layers and 64 QAM are used compared to 4 layers and 256 QAM.

Figure 6 below shows the performance in the “indoor FWA” scenario.  Here, we see similar results, although the curves are not spread apart as much.  The lower SNR of the “indoor FWA” scenario allows precoding gains at lower ranks to increase the benefit of higher-order modulation, so the lower rank curves are closer to the higher rank curves. Similarly, 256 QAM is less likely to be beneficial at lower SNRs, and so rank-8 256 QAM is less likely to be beneficial compared to rank-8 64 QAM in this scenario.

4-layer 256 QAM does not outperform 8-layer 64 QAM (or 8-layer 256 QAM) in scenarios with varying SNRs.

Figure 5	Mean-user UL throughout versus served traffic, with 64 QAM and 256 QAM, for FC CB and “outdoor FWA” scenario. The corresponding SLS parameters are collected in Table 3 Error! Reference source not found.in Appendix.

Figure 6	Mean-user UL throughout versus served traffic, with 64 QAM and 256 QAM, for FC CB and “indoor FWA” scenario. The corresponding SLS parameters are collected in Table 4 in Appendix.

Based on the above results and observations, we make the following proposal.

Support up to 8 layers for both codebook- and non-codebook-based transmission.

Note that this implies that the number of DMRS ports that can be assigned to a single UE needs to be increased to 8.

## 2.2	On coherency for 8 TX UEs

Coherent transmission naturally provides the best performance in a codebook-based design, but support for coherence can be difficult in a UE.  This difficulty meant that the first commercial UL-MIMO UEs had only non-coherent UL MIMO support.  It is further demonstrated by the relatively loose RAN4 relative coherence requirements in TS 38.101-1 of ± 40° between Tx chains. Since coherence requirements strongly impact UE implementation, it is crucial to consider codebook designs that support varying degrees of transmitter coherence. Consequently, during RAN1#109-e, the following agreements was reached:

Agreement
Study fully-coherent, partially-coherent, and non-coherent UEs for uplink transmission with 8TX UEs.

Agreement
Study full power transmission for 8TX UEs.

- Details are FFS upon completion of codebook design.

Figure 7 below shows some initial simulation results of the performance of a non-coherent codebook in the “outdoor FWA” scenario using 4 Tx 1x2 cross-polarized ULA. In this high-SNR scenario, we see that 8-layer non-coherent transmission significantly outperforms 4-layer non-coherent transmission in terms of mean-user throughput, where the performance is similar to the fully-coherent transmission (cf. Figure 3). Furthermore, there is negligible gain by increasing the number of antenna ports to 8 from 4, while limiting the rank to 4. These reinforce the observation that at high SNR, the performance gain is derived from increasing the number of layers, rather than from maintaining coherence or increasing array gain.

Figure 7	Mean-user UL throughout versus served traffic, as a function of the maximum transmission rank, for NC CB and “outdoor FWA” scenario. Here, the bandwidth is set to 100 MHz. The corresponding SLS parameters are collected in Table 3 in the Appendix.

Figure 8 below shows some initial simulation results of the performance of a non-coherent codebook in the indoor FWA scenario using 4 directional cross-polarized antennas (pointing in different directions).  Even in this lower SNR scenario, we see that 8-layer non-coherent transmission significantly outperforms 4 Tx UL MIMO in terms of mean throughput.

Figure 8	Mean-user UL throughout versus served traffic, as a function of the maximum transmission rank, for NC CB and “indoor FWA” scenario. Here, the bandwidth is set to 20 MHz. The corresponding SLS parameters are collected in Table 4 in Appendix.

8-layer non-coherent UL MIMO operation can significantly outperform 4-layer operation even in the lower SNRs of the “indoor FWA” scenario

Given the greater degrees of freedom in an 8 Tx design, there are many more possible combinations of coherent and non-coherent antenna pairs than for 4 Tx.   Four such possibilities are illustrated below.  Furthermore, non-coherent precoders are relatively easily designed by enumerating the possible combinations of the antenna elements.  Non-coherent precoders are also well suited to non-homogenous arrays with different panels or with directional elements. Therefore, from both a UE implementation and a specification effort point of view, non-coherent precoders are a particularly beneficial part of a UL codebook design.

Figure 9	8 Tx Coherence Combinations.

Non-coherent precoders are relatively easily implemented in UEs, straightforward to design, and well suited to non-homogenous and multi-panel operation.

Support non-coherent precoders in 8 Tx codebook-based operation.

## 2.3	Transmission schemes for 8 TX UEs

There are three fundamental approaches that fall within the scope of the agreement above: Defining a codebook to be used over 8 SRS ports, defining non-codebook-based operation with 8 SRS ports, and indicating multiple TPMIs with no more than 4 ports each. We consider each of these in the following.

### 2.3.1 Codebook-based approach

During RAN1#109-e, the following agreement was reached regarding antenna layouts for codebook design, including where 1, 2, and 4 antenna groups are to be studied for partially coherent precoding:

Agreement
For 8TX UE, consider the following UE antenna layouts for codebook design,

- For non-coherent UEs, consider linear array (1D/2D) of cross-polarized or single-polarized antenna configuration

- For fully/partial-coherent UEs, consider linear array (1D/2D)

  - Where the array is either cross-polarized antenna configuration or single polarized antenna configuration

  - Ng>=1 antenna groups can be considered where each group comprises coherent antennas, and across groups, antennas can be non-coherent/coherent depending on device types

    - An example of an antenna group is a panel

  - Within an antenna group, antenna elements are uniformly spaced. Across different antenna groups, companies to provide details.

Additional information for definition of antenna layout

- Based on the number of coherent groups, following exemplary cases can be considered where, within each group, antenna elements are spaced by 0.5λ, and then dG-H, dG-V represent the horizontal and vertical spacings between the centers of adjacent antenna groups, respectively

  - Further down-selection can be done in the next meeting, if needed

  - The shown exemplary placing of antenna groups can be used for evaluation purpose, but the codebook design is not restricted to shown cases.

  - Other antenna layouts for other use cases are not precluded.

  - To start companies may report their results according to their preferred layout.

| Case | Ng | (M, N, P) per group | Antenna layout | Antenna pattern/antenna element gain |
| --- | --- | --- | --- | --- |
| 1 | 1 | (2, 2, 2), (1, 4, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA) |
| 2 | 2 | (1, 2, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA) |
| 3 | 4 | (1, 1, 2) |  | Isotropic (Indoor/Outdoor FWA & Industrial) 4 dBi, 110° HPBW(Indoor FWA & Industrial) |

Support for a larger codebook requires a significant design effort, as can be seen above. A new 8-port SRS will be needed as well as the codebook itself. The codebook design is likely to be a much more complex task, since the number of potential codewords can grow exponentially with the number of elements, depending on the UE antenna configurations assumed. Non-homogeneities such as partial coherence, power modes, and directionality require extra effort in codebook designs, and these are more complex to design for than in the 4-port case.

To restrict the number of possible codebook designs, the following agreement was reached:

Agreement
For 8TX UE codebook-based uplink transmission, down-select one of

- Alt1-a:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for non-coherent UEs

  - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully/partially-coherent UEs

- Alt1-b:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for partially/non-coherent UEs

  - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully-coherent UEs

- Alt2-a:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs

- Alt2-b:

  - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) in combination with those based on NR Rel-15 DL Type I codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs

- Alt3:

  - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially/non-coherent UEs

Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

For non-coherent UEs, according to alternatives above, there are three options for Rel-18 codebook design:

- 8x1 antenna-selection vector,

- NR Rel-15 UL 2TX/4TX codebook,

- NR Rel-15 DL Type I codebook.

First, considering the antenna-selection option: There are 255 possible antenna-selection vectors with 8 Tx. Note that all possible combinations of existing non-coherent subset of NR Rel-15 2TX/4TX codebook yield all 255 possible antenna-selection vectors. Hence, option 1 and 2 are the equivalent. The NR Rel-15 DL Type I codebook assumes a cross-polarized array for which, at least, 2 ports are coherent. Hence, this option is not relevant for non-coherent codebook design. Therefore, in what follows, irrespectively of which alternative that is evaluated, we use all 255 antenna-selection vectors for the non-coherent subset of the 8 Tx codebook.

For fully coherent UEs, according to alternatives above, there are two ways to design the codebook:

- Use NR Rel-15 UL 2TX/4TX codebook as the starting point,

- Use NR Rel-15 DL Type I codebook as the starting point.

Note that, if the number of antenna groups, Ng, is larger than one, an 8 Tx fully coherent codebook can be obtained by co-phasing antenna groups of Nt/Ng ports each.

An open question is the oversampling ratios (OSRs), i.e., the values of [O1,O2] that should be supported per [N1,N2] in a NR Rel-15 DL Type-I codebook design, and so we provide some initial simulation results in Figure 10 where compare the performance of OSRs of 1, 2, and 4 for an 8-port cross-polarized 1x4 ULA in an “outdoor FWA” scenario. Here, the OSR is in the horizontal domain (i.e., O1≥1 and O2=1). We see that there is almost no difference between the OSRs.  This is predictable, since while there is less loss at the beam crossover points, and hence increased average antenna gain of higher OSRs, this is not likely to improve system throughput in such a high SNR scenario, especially given the SU-MIMO scheduling used in the simulation. Figure 11 has the same setup, except that an “indoor FWA” scenario is used.  In this case, the relative performance of the OSR is a little more discernible, where the OSR=1 case can be seen to have slightly worse performance than the higher oversampling ratios.  We should point out that these simulations assume maximum rank of 8, and so the benefits of higher OSR under configurations with lower maximum rank (that may be of interest, e.g., to MU-MIMO operation) are not studied.  However, these initial results point to the potential benefit of simpler codebooks with OSR as low as 1 (for a UE equipped with a 1x4 cross polarized ULA).

Figure 10	Mean-user UL throughout versus served traffic, for different OSRs and for the “outdoor FWA” scenario. Here, the bandwidth is set to 100 MHz. The corresponding SLS parameters are collected in Table 3 in Appendix.

Figure 11	Mean-user UL throughout versus served traffic, for different OSRs and for the “indoor FWA” scenario. Here, the bandwidth is set to 100 MHz. The corresponding SLS parameters are collected in Table 4 in Appendix.

Rel-15 codebooks include elements that are powers of j, which can be maintained with sufficiently low OSRs in the 8 Tx codebook designs.  If designs using such low OSRs do perform well enough scenarios, this feature of Rel-15 codebooks could be retained for 8 Tx.

Consider restricting codebooks for 8 TX UEs such that elements of the precoding matrices are limited to the set {+1, +j, -1, -j}.

In what follows, for the fully coherent subsets of the 8 Tx codebook design, when evaluating alternatives that use NR Rel-15 DL Type I codebook as the starting point, we set the OSR such that elements of the precoding matrix is limited to the set {+1, +j, -1, -j}. This implies that O1=2 for Ng=2 and that O1=1 for Ng=1 (for the considered an 1x4 cross-polarized ULA). It is worth noting that the subset of fully coherent precoders in the NR Rel-15 UL 2TX/4TX codebook is based on NR Rel-15 DL Type-I codebook. Specifically, with N1=Nt/2, N2=1, O1=2, and O2=1 one obtains a superset of the fully coherent candidates the in NR Rel-15 UL 2TX/4TX codebook.

For partially coherent UEs, according to alternatives above, there are two ways to design the codebook:

- Use NR Rel-15 UL 2TX/4TX codebook as the starting point,

- Use NR Rel-15 DL Type I codebook as the starting point.

In what follows, we construct the partially coherent subset of the 8 Tx codebook by generating all possible combinations of NR Rel-15 UL 4TX codebook and NR Rel-15 DL 4TX Type I codebook, respectively. The OSR is again set such that elements of the precoding matrix are limited to the set {+1, +j, -1, -j}.

Figure 12 and Figure 13 show the performance different codebook design alternatives (labelled as Alt 1a, 1b, 2a, 2b, and 3) for the “outdoor FWA”, and “indoor FWA” scenario, respectively. The codebooks include all precoder candidates for Ng∈{1,2} (i.e., both single- and multi-panel (with co-phasing factor ∅∈{+1,+j.-1,-j} precoders).

No performance differences are seen in the “outdoor FWA” scenario, while one scheme, Alt 2a, performs slightly worse than the others in the indoor FWA case.  The difference in schemes is driven by the amount of precoding gain possible from each scheme, and in general the differences are small.  Given the high SNRs of the outdoor FWA scenario, the differences in precoding gain are not as likely to affect the relative performance of the schemes, while the indoor FWA scenarios lower SNRs can allow greater gains from precoding.  Alt 2a is different from the other schemes in that the UE transmits on at most 4 ports for rank 1 transmission since there is not an 8 Tx precoder available in the Rel-15 UL MIMO codebooks.  This reduces the precoding gain, but not the Tx power, since full power Mode 0 is used in the simulations.  Because the performance is scenario dependent, the relative performance of the schemes should be further studied before drawing hard conclusions, but at least in these initial simulations, the relative performance of schemes is close in important scenarios, and schemes that preclude 8 Tx coherent transmission may underperform in at least low SNR scenarios.

Figure 12	Mean-user UL throughout versus served traffic, for different codebook designs and for the “outdoor FWA” scenario. Here, the bandwidth is set to 100 MHz. The corresponding SLS parameters are collected in Table 3 in Appendix.

Figure 13	Mean-user UL throughout versus served traffic, for different codebook designs and for the “indoor FWA” scenario. Here, the bandwidth is set to 20 MHz. The corresponding SLS parameters are collected in Table 4 in Appendix.

The relative performance of the design alternatives agreed in RAN1#109 is close in important scenarios, but designs that preclude 8 Tx coherent transmission may underperform in at least low SNR scenarios.

The fact that all alternatives perform similarly suggest that there is a redundant amount of precoders in the codebook. In fact, the size of the codebook is substantial for all of the above alternatives. To emphasize this point, e.g., for Alt 1a, 2b, and 3, note that only the partially coherent subset of the 8 Tx codebook, constructed by brute combining 2 groups of NR Rel-15 DL 4TX Type I (with OSR=2) codebooks, results in 2400 precoder candidates for rank 1—8 transmission. Such large codebooks may be problematic for, e.g., DCI overhead, UE implementation complexity, and UL scheduler implementation (which has to evaluate all candidates to find the best precoder). On the other hand, we should design the codebook such that it is applicable to a wide range of UEs (e.g., with different coherency capabilities, directional/isotopic antennas, single/multi-panel, etc.), which will result in a large number of precoder candidates.  Hence, there is a trade-off between the size and generality of the codebook. Therefore, irrespectively of which of the above alternatives that is adapted, we propose:

Study techniques to reduce/restrict the number of precoder candidates for 8 Tx codebook designs.

### 2.3.2 Non-codebook-based approach

Extending NCB-based designs to 8 layers is a natural alternative to consider for an 8 Tx codebook design.  NCB-based operation allows quite flexible UE implementations with respect to precoder design, since the precoder is determined by the UE.  Antenna selection, analog beamforming, or digital precoding can all be used in a way that is transparent to the gNB scheduler.  This also avoids the need to signal precoding, allowing relatively low DCI overhead.

The specification effort to extend Rel-15 NCB-based precoding to 8 layers could be relatively small.  The number of supported SRS resources and layers would need to be increased, as would be the number of SRS resource combinations that can be signaled by SRI.  SRI currently indicates all possible combinations of SRS resources to be transmitted for a given number of layers, and number of (single port) SRS resources.  This method can be reused with up to 8 layers and 8 SRS resources, which is simpler than defining a second method for >4 layers and SRS resources, as well as allowing better DCI overhead.

Considerations for CB designs such as coherence among Tx chains and non-homogeneous configurations are part of UE implementation for NCB-based operation, and so will not need to be addressed.  On the other hand, there are no RAN4 tests for NCB-based operation in Rel-17, and so extra effort is needed from a RAN4 perspective compared to CB-based operation (which has been tested since Rel-15).

While NCB-based operation is relatively simple in specifications, UE implementation may not be simple as compared to (at least non-coherent) CB-based operation. In non-coherent CB-based operation, the UE transmits one layer per Tx chain, and need not maintain coherence among any of the Tx chains. The UE may further have PAs where, some, or all of which can transmit less than their full rated power. In contrast, NCB-based operation requires that UEs always transmit at their rated power. Furthermore, if the NCB-based UE supports determining precoding from CSI-RS measurements, it may need to transmit coherently, since the intention of the capability is in our understanding to support advanced precoding schemes such as SVD-based precoding to have a benefit over NCB capabilities without CSI-RS measurements.

By its nature, NCB-based operation has higher SRS overhead than CB-based operation.  Each single port NCB-based SRS resource must be transmitted in a different OFDM symbol from the other resources.  As the number of potential precoders grows with the number of Tx chains, the number of SRS resources can grow substantially to provide enough CSI to maintain performance, especially under fast fading conditions.

Observations 7-8:

- Extending Rel-15 non-codebook-based operation to 8 layers may be relatively straightforward to specify.

Rel-15 SRI indication can be used, where all combinations of SRS resources are indicated for a given maximum number of layers and number of configured SRS resources.

However, RAN4 specifications will need to be developed, as non-codebook is not tested as of Rel-17.

- Eight-layer NCB-based operation can have relatively complex UE implementations and high SRS overhead.

Full power operation is required for NCB-based operation.

Transparent precoding can require coherent Tx chains, e.g., for reciprocity-based operation.

Each SRS resource requires an OFDM symbol, and the number of needed SRS resources can grow substantially with the number of layers.

Focus the study of Rel-15 NCB-based operation with up to 8 layers on using Rel-15 principles, allowing any combination of SRS resources for a given maximum number of layers and SRS resources.

### 2.3.3 Multiple-TPMI-based approach

Extending CB-based operation to 8 Tx is also possible with multiple TPMIs.  A UE could be configured with multiple SRS resources, and a TPMI could be provided for each SRS resource.  The UE then precodes each set of layers its corresponding indicated precoding matrix.  For example, two 4 port SRS resources could have two respective rank 4 precoders, and the UE could then transmit 8 layers from a total of 8 ports.  This is like Rel-17 multi-TRP operation where UEs can be configured with multiple SRS resource sets, and they repeat a TB using different precoders corresponding to the different SRS resources.

The specification effort to 8-port operation with multiple TPMI and SRS resources could be limited.  Existing fully/partially/non-coherent codebooks, full power modes, and SRS designs could be reused.  Multiple TPMI indication would be needed, as would enhancements to SRI.  As with the other design alternatives, the number of DMRS ports will need to be increased to support 8 layers.  It is also possible that RAN4 tests could be simplified since requirements could be based on those of Rel-15 2 Tx or 4 Tx codebooks.

If each precoder is used only over the ports in one SRS resource, the precoding gain can be less than when a precoder is applied over all ports that the UE can transmit.  However, the net impact depends on the antennas used at the UE, coherence assumptions, as well as rank distributions throughout the cell.  Therefore, an 8 Tx codebook could possibly outperform multi-TPMI/SRS based designs that apply precoders within each SRS resource, but this requires further study.

In principle, it is also possible to use a precoder across SRS resources. Since all elements can combine coherently in this case, the performance with a fully coherent array can be the same as an 8 Tx codebook.  However, coherently combining across SRS resources would deviate significantly from Rel-15 principles and UE designs, since there is no requirement for UEs to transmit SRS resources coherently.  A step forward in down selection can be to preclude coherent combining across SRS resources in the study of transmission using multiple SRS resources.

Observations 9-12:

- 8-port/8-layer operation can be supported by indicating multiple TPMIs and SRS resources.

UEs transmit a portion of layers according to a Rel-15 TPMI that corresponds to an indicated SRS resource.

- Multi-TPMI/SRS based methods should require relatively limited specification effort.

Existing fully/partially/non-coherent codebooks, full power modes, and SRS designs could be reused.

RAN4 tests might be extended from 2 and 4 Tx CB-based operation.

- The performance compared to an 8 Tx codebook may be worse but needs study.

A larger codebook combines more ports allowing more gain, but this depends heavily on UE implementation, such as coherence and antenna designs, as well as on rank distributions over a cell.

- Coherent combining across SRS resources could be used to allow the same performance with an 8 Tx codebook, but this would deviate significantly from Rel-15 principles and UE designs.

Study indicating multiple Rel-15/16 TPMIs, each corresponding to an SRS resource, where precoders do not combine coherently across SRS resources.

# 3 	Conclusion

In this contribution, we have considered the design of various mechanisms to support 8 Tx UL MIMO operation.  The benefits of increasing the maximum number of layers to 8 were evaluated, considering maximum modulation orders of 64 and 256 QAM.  Codebook design aspects such as complexity benefits and performance gains of non-coherent precoders were investigated. Suitable oversampling ratios in terms of performance and implementation complexity for codebook designs based on DL Type 1 based coherent precoders were also studied.  Lastly, high level aspects of non-codebook and multi-TPMI based approaches were further discussed.

Based on the discussion, we made the following proposals:

Proposal 1	Support up to 8 layers for both codebook- and non-codebook-based transmission.

Proposal 2	Support non-coherent precoders in 8 Tx codebook-based operation.

Proposal 3	Consider restricting codebooks for 8 TX UEs such that elements of the precoding matrices are limited to the set {+1, +j, -1, -j}.

Proposal 4	Study techniques to reduce/restrict the number of precoder candidates for 8 Tx codebook designs.

Proposal 5	Focus the study of Rel-15 NCB-based operation with up to 8 layers on using Rel-15 principles, allowing any combination of SRS resources for a given maximum number of layers and SRS resources.

Proposal 6	Study indicating multiple Rel-15/16 TPMIs, each corresponding to an SRS resource, where precoders do not combine coherently across SRS resources.

# 4 	References

Samsung, “RP-213598, MIMO evolution for downlink and uplink,” 3GPP TSG RAN meeting #94-e, electronic meeting, Dec. 2021.

# Appendix

Below, we have collected simulation parameters for the “outdoor FWA” scenario. Unless otherwise explicitly stated, these are the parameters used for all above evaluations for the “outdoor FWA” scenario.

Table 3	Parameters for SLS simulations for “outdoor FWA” scenario

| System-level simulation parameters |  |
| --- | --- |
| Metric | UL mean-user throughput |
| Scheduler | Proportional Fair |
| Traffic model | FTP Model 1 |
| ISD | 500 m |
| Number of sites | 7 |
| Number of UEs | 10000 |
| UE distribution | 100% Outdoor |
| Handover margin | 3 dB |
| Carrier frequency | 3.5 GHz |
| Bandwidth | 100 MHz |
| Subcarrier spacing | 30 kHz |
| Channel model | UMa (according to TR 38.901) |
| Packet size | 500 kB |
| MIMO scheme | SU-MIMO |
| Power mode | Rel-16 mode 0 |
| Power control | P0= -50, α=0.8 |
| Modulation | Up to 256 QAM |
| Receiver | MMSE-IRC |
| BS antenna configuration (for outdoor BS) | (M,N,P,Mg,Ng,Mp,Np) = (8,8,2,1,1,4,8) with (dH, dV) = (0.5, 0.8)λ |
| BS antenna pattern | Directional (8 dBi, 65° BW) |
| BS antenna height | According to TR 38.901 |
| BS noise figure | 5 dB |
| UE antenna configuration | ULA, pointing towards nearest gNB |
| UE antenna pattern | Directional (8 dBi, 65° BW) |
| UE antenna height | 6 m |
| UE transmit power | 32 dBm |
| UE speed | 3 km/h |

Below, we have collected simulation parameters for the “indoor FWA” scenario. Unless otherwise explicitly stated, these are the parameters used for all above evaluations for the “indoor FWA” scenario.

Table 4	Parameters for SLS simulations for “indoor FWA” scenario

| System-level simulation parameters |  |
| --- | --- |
| Metric | UL mean-user throughput |
| Scheduler | Proportional Fair |
| Traffic model | FTP Model 1 |
| ISD | 200 m |
| Number of sites | 7 |
| Number of UEs | 10000 |
| UE distribution | 100% Indoor |
| Handover margin | 3 dB |
| Carrier frequency | 3.5 GHz |
| Bandwidth | 100 MHz |
| Subcarrier spacing | 30 kHz |
| Channel model | UMi (according to TR 38.901) |
| Packet size | 500 kB |
| MIMO scheme | SU-MIMO |
| Power mode | Rel-16 mode 0 |
| Power control | P0= -50, α=0.8 |
| Modulation | Up to 256 QAM |
| Receiver | MMSE-IRC |
| BS antenna configuration (for outdoor BS) | (M,N,P,Mg,Ng,Mp,Np) = (8,8,2,1,1,4,8) with (dH, dV) = (0.5, 0.8)λ |
| BS antenna pattern | Directional (8 dBi, 65° BW) |
| BS antenna height | According to TR 38.901 |
| BS noise figure | 5 dB |
| UE antenna configuration | ULA, randomly oriented |
| UE antenna pattern | Isotropic/Directional (8 dBi, 65° BW) |
| UE antenna height | According to 36.873 |
| UE coherency | Fully/Partial/Non coherent |
| UE transmit power | 23 dBm |
| UE speed | 3 km/h |