  - 3GPP TSG RAN WG1 Meeting #110     						          																	  R1-2207205

  - Toulouse , France, 22nd August – 26th August, 2022

Agenda Item:		8.10

Source:						Qualcomm Incorporated

Title:								Remaining issues on eIAB

Document for:		Discussion and decision

# Introduction

This contribution provides our views on remaining issues on eIAB.

# Interaction between time and frequency domain DU resource configuration – RAN1-106bis-e Working Assumption

The following working assumption and decisions were made in previous RAN1 meetings:

RAN1-106bise working assumption: If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected: Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied. RAN1-107e Agreement: Whether or not an IAB node can operate under a given non-TDM multiplexing mode (i.e. multiplexing info in 38.473) is left to IAB implementation in Rel-17 RAN1-109e Conclusion If both Rel-16 H/S/NA and Rel-17 H/S/NA are configured for a given resource and the child node is operating in TDM multiplexing mode, consider the following alternatives until RAN1#110: - Alt. 1: the child node follows the Rel-16 H/S/NA configuration for the resource - Alt. 2: the child node follows the Rel-17 H/S/NA configuration for the resource - Alt. 3: A resource configured with Rel-16 H or Rel-16 S with dynamic indication of availability overrides the Rel-17 H/S/NA configuration, otherwise the child node follows the Rel-17 H/S/NA configuration for the resource - Alt. 4 the child node follows the Rel-16 or Rel-17 H/S/NA based on implicit indication (e.g. Case 6 timing indication) between parent and child node.

The fact an IAB-node can decide autonomously to operate in TDM vs. non-TDM mode leads to some uncertainty at the parent node, but it is allowed by the aforementioned RAN1#107e agreement. Additional uncertainty at the parent node is created if the IAB-node is further allowed to autonomously select which DU H/S/NA resource configuration to use, as proposed in the aforementioned working assumption.

This uncertainty is undesirable because in general it can lead to resource conflicts at the IAB-node or inefficiencies in the upstream link (between the IAB-node and its parent). The former occurs if the parent node attempts to more aggressively use the link to the IAB-node (e.g. the parent adheres to its FDM configuration and schedules the IAB-node MT for some RB sets while the IAB-node decides to operate in TDM and operates its DU in all RB sets because the TDM configuration is H for the given symbol). The latter occurs when the parent node makes a more conservative assumption about what the IAB-node may do (e.g. assuming that the IAB-node will use a H resource with TDM, the parent does not schedule anything in that resource, even though the FDM configuration would allow the parent to do so for at least some RB sets).

The possible combinations are illustrated in Figure 2-1.

Figure 2-1 – Combinations of time domain and frequency domain DU resource configurations

The following observations are made:

- For all the *-TNA cases there is no advantage for the IAB-node to use the time domain configuration.

- For the FS-TH and FNA-TH cases the IAB-node has the benefit of using the full bandwidth in the downstream link but may experience some conflicts with the parent’s utilization of the upstream link, since the parent node is most likely going to use the frequency domain configuration. For the FH-TH case there is no advantage for the IAB-node to use the time domain configuration.

- For the *-TS cases the IAB-node could benefit, without penalty, from using the time domain configuration only if the parent indicates the time domain S resource available or if the IAB-node can implicitly determine it as available. Otherwise there is no advantage.

As a result, in the context of the alternatives listed in the RAN1-109e conclusion, in principle a modified Alt. 3. could be considered:

- Alt. 3b: A resource configured Rel-16 S with dynamic indication of availability overrides the Rel-17 H/S/NA configuration, otherwise the child node follows the Rel-17 H/S/NA configuration for the resource.

On the other hand, it is reasonable to assume that if the time domain configuration has a symbol marked as S, the frequency domain configuration for the same symbol would have some RB sets configured as S, in addition to some RB sets configured as H. The latter aspect (i.e. some RB sets configured as H) is an expected characteristic of a frequency domain configuration, which would typically enable the DU to operate in every symbol at least in some RB sets. In such a typical scenario, if it is assumed the parent node would release the time domain S resource, the parent could instead release the S resources in the frequency domain configuration. Therefore, in a typical scenario, there is no significant benefit to use the time domain configuration, since it is expected the DU can operate at all times, at least in a subset of the RB sets of a symbol.

Moreover, upon investigation of the current specifications there does not appear to be a simple way to reflect Alt. 3b without significant specification changes. As a result, for Rel-17 it is recommended to select Alt 2.

Proposal 2.1

Adopt the following TP for TS38.213:

Reason for change: Resolution of ambiguity in presence of both time domain and frequency domain H/S/NA DU resource configuration. Summary of change: The frequency domain H/S/NA DU resource configuration takes priority over the time domain H/S/NA DU resource configuration. Consequences if not approved: Specifications would continue to have ambiguity for the IAB-DU behavior, which may result in overall system performance degradation

# Reply to RAN3 LS R1-2205705

The R1-2205705 LS was received from RAN3. There are two questions to RAN1 related to the RB set configuration:

In regard to the first question, our view is that the Rel-17 frequency domain DU resource configuration should be provided to the IAB-donor DU as well, similarly to the Rel-16 time domain DU resource configuration, as described in TS 38.300 and TS 38.473.

In regard to the second question, our view is that the current F1AP signalling description about the RB set size is sufficiently clear.

Proposal 3.1

Provide the following responses in the reply LS to RAN3 in response to R1-2205705:

- Yes, the RB set needs to be configurable to the IAB-donor DU.

- Yes, the current F1AP signalling about RB set size is clear enough.

# Conclusion

This contribution provided our views on remaining issues on eIAB. The following proposals were made:

Proposal 2.1

Adopt the following TP for TS38.213:

Reason for change: Resolution of ambiguity in presence of both time domain and frequency domain H/S/NA DU resource configuration. Summary of change: The frequency domain H/S/NA DU resource configuration takes priority over the time domain H/S/NA DU resource configuration. Consequences if not approved: Specifications would continue to have ambiguity for the IAB-DU behavior, which may result in overall system performance degradation.

Proposal 3.1

Provide the following responses in the reply LS to RAN3 in response to R1-2205705:

- Yes, the RB set needs to be configurable to the IAB-donor DU.

- Yes, the current F1AP signalling about RB set size is clear enough.