3GPP TSG RAN WG1 #110			R1-2207314

Toulouse, France, August 22nd – 26th, 2022

Agenda item:	8.12

Source:	Apple Inc.

Title:	Remaining issues on  NR Multicast and Broadcast Services

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, new work item on NR Multicast and Broadcast Services [1] was agreed. Some of objectives of this study item are showing below,

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

    - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

  - Specify support for dynamic change of Broadcast/Multicast service delivery between multicast (PTM) and unicast (PTP) with service continuity for a given UE [RAN2, RAN3]

  - Specify support for basic mobility with service continuity [RAN2, RAN3]

  - Assuming that the necessary coordination function (like functions hosted by MCE, if any) resides in the gNB-CU, specify required changes on the RAN architecture and interfaces, considering the results of the SA2 SI on Broadcast/Multicast (SP-190625) [RAN3]

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

  - Study the support for dynamic control of the Broadcast/Multicast transmission area within one gNB-DU and specify what is needed to enable it, if anything [RAN2, RAN3]

In RAN1#109-e meeting, some agreements were made on MBS [2]. In this contribution, we discuss the MBS remaining issues, multiplexing NACK-only feedback with SR, NAKC-only feedback for more than one G-RNTI, etc.

# Discussion

After RAN1#109-e meeting, the following issues are still open.

Multiplexing NACK-only feedback with SR

For NACK-only based feedback multiplexing with SR, it was discussed in RAN1#108-e meeting, the followings were proposed from FL’s summary.

#### Proposal 3.2.1.4-1 (GTW) [3] When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following: - For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16. - Alt1-1: PUCCH resources configured for unicast will be used. - Note: either the PUCCH resource for SR or for ACK/NACK is used per the legacy procedure of Rel-15/16. - the PUCCH resource is determined by the PRI in the last DCI for multicast; - Alt1-2: the PUCCH resource is determined by RRC configuration; - Alt1-3: the PUCCH resource for NACK-only feedback transmission is SR resource. - For the case of more than two TBs of NACK-only, FFS which alternative from the following is supported: - Alt2-1: SR is dropped - Alt2-2: NACK-only feedback is transformed into ACK/NACK HARQ bits and PF 2/3/4 is used for the transmission per the legacy SR and HARQ-ACK multiplexing rule of Rel-15/16. - Alt2-3: NACK-only is dropped #### Proposal 3.2.1.4-2 (if proposal 3.2.1.4-1 is not agreeable) Multiplexing NACK-only based feedback for multicast with SR is not supported in Rel-17, - It is up to UE implementation when the PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR.

For up to two HARQ-ACK bits case, the two HARQ-ACK bits could come from NACK-only feedback or one bit is from NACK-only feedback, another bit is from unicast. The HARQ-ACK multiplexing with SR is UE specific, so the UE dedicated PUCCH resource should be used. The Alt-1 is straight forward and preferred.

For more than two HARQ-ACK bits case, it’s up to which NACK-only feedback mode is configured. If Alt 1 is configured, Alt 2-2 can be adopted. Alt4 only supports up to four bits, then Alt2-3 can be applied, as PUCCH resource for NACK-only feedback can be shared by group of users, the impact of dropping the NACK-only bits can be acceptable.

Proposal 1:  When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following:

  - For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16.

    - Alt1-1: PUCCH resources configured for unicast will be used.

      - Note: either the PUCCH resource for SR or for ACK/NACK is used per the legacy procedure of Rel-15/16.

      - the PUCCH resource is determined by the PRI in the last DCI for multicast;

  - For the case of more than two TBs of NACK-only, FFS which alternative from the following is supported:

    - If NACK-only feedback mode Alt 1 is configured. Alt2-2: NACK-only feedback is transformed into ACK/NACK HARQ bits and PF 2/3/4 is used for the transmission per the legacy SR and HARQ-ACK multiplexing rule of Rel-15/16.

    - If NACK-only feedback mode Alt 4 is configured. Alt2-3: SR is dropped

NACK-only based HARQ feedback

For NACK-only feedback mode Alt4, the mapping between the HARQ-ACK bit and PUCCH resource for single G-RNTI wase agreed. Supporting multiple G-RNTIs was discussed in case 2 and case 3 in FL summary in [4], but no conclusion could be made. Considering the limited PUCCH resources, i.e., 15 PUCCH resources, supporting more than two G-RNTIs would mean the scheduled TB for a G-RNTI is really limited and doesn’t make sense. If up to two G-RNTIs are supported for Alt4, the four TBs can split equally for each G-RNTI, the TB1 and TB2 are for G-RNTI1, TB3 and TB4 are for G-RNTI2. In this case, the agreed mapping table in last RAN1 meeting can be re-used, the standard and implementation efforts are minor.  For case 3, even the different UEs could be configured with different G-RNTIs, the PUCCH resources are per G-RNTI configured, so the UEs configured with the same G-RNTI(s) could share the same PUCCH resources still.  With the TB split between the two G-RNTIs, the C-DAI can still be used as counter DAI for HARQ-ACK bit countering and ordering.

Agreement For UE configured with NACK-only HARQ-ACK feedback, for Alt4 (from previous agreement), define the mapping between HARQ-ACK values and the PUCCH resources for the TBs configured with NACK-only as follows: - The mapping is applied to both a single G-RNTI - FFS: multiple configured G-RNTIs cases. - FFS: how to order the TBs, whether/how to account for missed or non-scheduled TBs - FFS whether/how PRI is used for indication. Agreement For multicast, for addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4: - Case 1: when the HARQ-ACK-to-PUCCH mapping table for Alt4 (from previous agreement) is applied to a single G-RNTI (from network and UE perspective) - Opt1-1: based on C-DAI included in DL DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. - FFS: whether Opt1-1 can also work when different G-RNTIs are used for different UEs - Note: other cases are not precluded #### Proposal 2.3.2.4 [4] For multicast, for addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4: - Case 2: for the case of all UEs configured with the same set of G-RNTIs - Opt2-1: support Alt4 for this case - Opt2-1-1: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits, i.e., the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), 15 PUCCH resources correspond to 4 TBs, and based on Opt2-1-2 or Opt2-1-3 for ordering the HARQ-ACK bits (CATT) - Opt2-1-2: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. (Apple, Nokia, NSB, DOCOMO, ZTE, Qualcomm) - Opt2-1-3: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs (Lenovo, Ericsson, Samsung) - Opt2-2: does not support Alt4 for this case. (Apple) - Case 3: for the case of different UEs configured with different G-RNTIs - Opt3-1: support Alt4 for this case - Opt3-1-1: based on the number of PUCCH resources in the PUCCH resource set configured for NACK-only to count the number of HARQ-ACK bits, i.e., the number of HARQ-ACK bits=log2(the number of PUCCH resource+1), 15 PUCCH resources correspond to 4 TBs, and based on Opt3-1-2 or Opt2-1-3 for ordering the HARQ-ACK bits (CATT) - Opt3-1-2: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. (Apple with 2 G-RNTIs, Nokia, NSB, DOCOMO) - Opt3-1-3: based on C-DAI* included in the last scheduling DCI for counting the number of HARQ-ACK bits and for ordering the HARQ-ACK bits. The C-DAI* is accumulating the scheduled TBs across different G-RNTIs. (Lenovo, , Ericsson, ZTE) - Opt3-2: does not support Alt4 for this case. (Samsung, Qualcomm)

- Table 1: HARQ-ACK bit mapping to PUCCH resource

| HARQ-ACK feedback |  |  |  | PUCCH resource |
| --- | --- | --- | --- | --- |
| G-RNTI1 |  | TB3 and TB4 for G-RNTI2 |  |  |
| TB1 | TB1,TB2 | TB1,TB2,TB3 | TB1,TB2,TB3,TB4 |  |
| N | N,N | N,N,N | N,N,N,N | 1st PUCCH resource |
|  | N,A | N,A,N | N,A,N,N | 2nd PUCCH resource |
|  | A,N | A,N,N | A,N,N,N | 3rd PUCCH resource |
|  |  | N,N,A | N,N,A,N | 4th PUCCH resource |
|  |  | N,A,A | N,A,A,N | 5th PUCCH resource |
|  |  | A,N,A | A,N,A,N | 6th PUCCH resource |
|  |  | A,A,N | A,A,N,N | 7th PUCCH resource |
|  |  |  | N,N,N,A | 8th PUCCH resource |
|  |  |  | N,N,A,A | 9th PUCCH resource |
|  |  |  | N,A,N,A | 10th PUCCH resource |
|  |  |  | A,N,N,A | 11th PUCCH resource |
|  |  |  | N,A,A,A | 12th PUCCH resource |
|  |  |  | A,N,A,A | 13th PUCCH resource |
|  |  |  | A,A,N,A | 14th PUCCH resource |
|  |  |  | A,A,A,N | 15th PUCCH resource |

Proposal 2: For case 2 and case3, adopt the above HARQ-ACK bit mapping to PUCCH resource table for NACK-only feedback mode Alt4 to support two G-RNTIs.

Proposal 3: C-DAI in DL DAI is used for countering the number of HARQ-ACK bits and for HARQ-ACK bit ordering.

# Summary

In this contribution, we discuss MBS remaining issues, and have the following proposals:

Proposal 1:  When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following:

  - For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16.

    - Alt1-1: PUCCH resources configured for unicast will be used.

      - Note: either the PUCCH resource for SR or for ACK/NACK is used per the legacy procedure of Rel-15/16.

      - the PUCCH resource is determined by the PRI in the last DCI for multicast;

  - For the case of more than two TBs of NACK-only, FFS which alternative from the following is supported:

    - If NACK-only feedback mode Alt 1 is configured. Alt2-2: NACK-only feedback is transformed into ACK/NACK HARQ bits and PF 2/3/4 is used for the transmission per the legacy SR and HARQ-ACK multiplexing rule of Rel-15/16.

    - If NACK-only feedback mode Alt 4 is configured. Alt2-3: SR is dropped

Proposal 2: For case 2 and case3, adopt the above HARQ-ACK bit mapping to PUCCH resource table for NACK-only feedback mode Alt4 to support two G-RNTIs.

Proposal 3: C-DAI in DL DAI is used for countering the number of HARQ-ACK bits and for HARQ-ACK bit ordering.

# References

- RP-193248, “New Work Item on NR Multicast and Broadcast Services”, Huawei, HiSilicon

- R1-2205701, “Report of RAN1#109-e meeting”, MCC

- R1-2202883, “FL summary#5 on improving reliability for MBS for RRC_CONNECTED UEs”, Moderator (Huawei)

- R1-2205157, “FL summary#3 on improving reliability for MBS for RRC_CONNECTED UEs”, Moderator (Huawei)