3GPP TSG RAN WG1 #110    			       	R1- 2207362

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.5.3

Source: 	LG Electronics

Title: 	Discussion on positioning support for RedCap UEs

Document for:   Discussion and decision

Introduction

In this contribution, we discuss potential enhancements that needs to be discussed/studied from the RAN1’s point of view.

Potential enhancements

The first thing to be discussed should be whether RedCap UE specific PRS/SRS resource configuration is supported or not by PRS/SRS resource configuration for legacy UEs. For example, the positioning frequency layer, bandwidth of set of PRS resources are configured per PFL, which means reusing current configuration structure of PFL directly to RedCap UE may not be desirable since the RedCap UE is not accounted for such configuration. It needs to be clarified whether reusing legacy PRS/SRS configuration for RedCap UEs is appropriate or not. One of the simplest method will be reusing PFL for legacy UEs to RedCap UE. However, the bandwidth of legacy UE and that of RedCap UE can be expected to be different. Since the PFL contains the information of bandwidth, reusing it directly might lead configuration of bandwidth larger than expected by RedCap UE. In such case, methodology or rule for handling larger bandwidth for RedCap UE would be required. Alternatively designing RedCap UE-specific PFL configuration could be considered. There will be no mismatch of bandwidth however it could lead resource overhead problem. From these perspectives, how to configure PRS resources to support RedCap UE positioning should be considered.

Proposal 1: Discuss whether and how to configure PRS resources for RedCap UEs.

Due to limited capabilities of RedCap UEs, it is very likely that positioning performance to be degraded compared to legacy UE. Therefore, it would be worth to study possible solutions for compensate the degraded performance. Schemes identified in eMTC/NB-IoT positioning can be a good starting point for the discussion in RedCap positioning. One of promising technique is obtaining diversity gain via frequency hopping and/or time domain repetition. For those techniques, requirements and necessary configurations can be discussed with consideration of legacy behavior. For example, frequency hopping of SRS based on BWP can be considered since the SRS transmission spans the configure BWP. Also burst transmission of SRS can be considered for time domain repetition which is no supported currently.

Proposal 2: Discuss time/frequency domain enhancement (e.g., BWP hopping/switching and burst transmission of SRS for positioning) to compensate performance degradation of RedCap UE)

Conclusion

In this contribution, we discussed positioning for RedCap UE, and our proposals are summarized below:

Proposal 1: Discuss whether and how to configure PRS resources for RedCap UEs.

Proposal 2: Discuss time/frequency domain enhancement (e.g., BWP hopping/switching and burst transmission of SRS for positioning) to compensate performance degradation of RedCap UE)