3GPP TSG RAN WG1 #110			R1-2207364

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.3.2

Source: 	LG Electronics

Title: 	Study on Subband non-overlapping full duplex

Document for:	Discussion and decision

Introduction

In RAN#94-e meeting, a study item on evolution of NR duplex operation is approved and the corresponding description is provided in [1].

According to the SID, the subband non-overlapping Full Duplex (subband FD) and potential enhancements on dynamic/flexible TDD are studied. Also, identification of deployment scenarios and developing evaluation methodology are also included in the scope as follows.

In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we discuss on the subband non-overlapping full duplex.

Discussion

For 5G, many of the new service types (e.g., XR service, AI based service, Self-driving Car) are characterized by dynamic variation of traffic in both DL and UL directions, requiring low latency in the packet delivery. For 5G services, demands on the traffic load will be exploding to support various emerging use cases. Existing semi-static or dynamic TDD UL/DL configuration has a limit of delay in time and inter-operator interference issue. SB-FD operation within a single cell can be expected to provide latency reduction and throughput enhancement. Also, from the initial result of performance evaluation in our companion documentation [2], it is observed that SB-FD shows better performance than HD TDD.

However, because SB-FD is not supported in current NR system, some modification is necessary and new functions seems to be introduced. In Rel-18 Duplex Evolution (DE) SI, followings should be studied for supporting SB-FD.

- Time Resource Utilization for SB-FD operation (TDD configuration for SBFD, Timing Boundary Alignment)

- Resource Allocation in Frequency Domain  (BWP, Subband partitioning)

- Interference Handling (Inter-subband CLI, Self-Interference)

- MIMO operation (Antenna configuration, Antenna group, TCI/SRI, CSI-RS)

Time Resource Utilization for SB-FD operation

TDD configuration for supporting SB-FD

For supporting SB-FD operation, it needs to be discussed which time resource among time resources configured as DL/Flexible/UL can be used for SB-FD operation. One of considerable point for decision is a transmission power and inter-cell interference. For example, if SB-FD is allowed in the time resource configured as UL and gNB is allowed to transmit with higher power in the time resource configured as UL, unexpected inter-subband gNB to gNB cross link interference (CLI) would be appeared and UL performance of other cell or inter-operator would be degraded due to higher power of inter-subband gNB to gNB CLI. On the other hand, when SB-FD is allowed in the time resource configured as DL, a part of subband is changed for UL transmission at UE side. When UE transmits UL channel/signal, inter-subband UE to UE CLI can be appeared, but this CLI may not be harmful to DL performance of UE in other cell. In this sense, it seems reasonable that SB-FD is allowed in the time resource configured as DL and/or Flexible.

Proposal 1: SB-FD operation can be allowed in the time resource configured as DL and/or Flexible.

One of potential enhancement of TDD configuration for supporting SB-FD operation is to inform time resource where SB-FD operation is available at gNB side. In RAN1#109-e meeting, followings were agreed.

Agreement Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation. Conclusion For discussion purpose only, SBFD symbol is defined as symbol with subbands that gNB would use for SBFD operation.

When it is considered that at least available bandwidth can be different in the time source depending on HD operation and SB-FD operation, it seems reasonable that the information of time resource for SB-FD operation is required to UE for determining UE behavior. For example, this indication of time resource for SB-FD operation could be provided on top of TDD configuration.

Proposal 2: Information of time resource for SB-FD operation is provided to UE for determining UE behavior. Study how to indicate the information of time resource for SB-FD operation.

Legacy TDD configuration is designed for HD TDD operation, where time resource (slot or symbol) for downlink only, time resource for uplink only, and flexible resource can be designated in time domain. Also, current TDD configuration is designed based on the assumption of a type of {DL Flexible UL} pattern. In Rel-18 DE SI, it was decided that gNB SB-FD and UE HD are assumed. That is, because UE operates in half duplex manner, TDD configuration should be informed to UE with DE capability.

But, when time resource for SB-FD operation is used, additional type of pattern would be required TDD configuration. For example, it can be assumed that when time resource for SB-FD is located in the front slots within TDD configuration period. For supporting this kind of combination link direction, new configuration seems to be necessary. In this sense, in Rel-18 DE SI, it needs to be discussed whether/how TDD configuration mechanism is enhanced for supporting SB-FD operation.

Proposal 3: Study whether TDD configuration mechanism should be updated for supporting SB-FD operation. If it is agreed that enhancement of TDD configuration for SB-FD operation is studied, following can be studied.

- Which type of TDD configuration needs to be enhanced for supporting SB-FD operation

In NR system, TDD UL/DL pattern can be configured by TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated and Dynamic Slot-Format Indicator (SFI). And, this TDD UL/DL pattern is applied to every BWPs. When SB-FD operation is adopted, multiple types of TDD UL/DL pattern can be indicated for UE with capability for supporting SB-FD operation. For example, it can be indicated which type of TDD UL/DL pattern is applied for each BWP. In this sense, it seems necessary to be studied whether multiple type of TDD UL/DL pattern can be allowed for BWPs when SB-FD operation is adopted.

Proposal 4: Study whether multiple type of TDD UL/DL pattern can be allowed for BWPs when SB-FD operation is adopted.

In NR system, DL/UL collision handling rule is defined. For supporting SB-FD operation, currently defined UL/DL collision handling rule needs to be modified. For example, UE does not transmit UL channel/signal if transmission would be overlapped with any symbol of set of symbols for SS/PBCH block. When SB-FD operation is adopted, set of symbols for SS/PBCH block can be involved within time resource for SB-FD operation, and symbols indicated for UL channel/signal transmission can be overlapped with any symbol of set of symbols for SS/PBCH block. In this case, UE may not transmit UL channel/signal if current DL/UL collusion handling rule is applied. That is, UL subbands within SB-FD symbol cannot be used when symbols for SS/PBCH block are located at the SB-FD symbols. On the other hand, if DL/UL collision rule is modified for allowing UL transmission at the SB-FD symbols, UE may not receive symbols for SS/PBCH block and UE may not operate L3 measurement, time/frequency tracking, Rx beam selection by using SS/PBCH block. Therefore, it seems DL/UL collision rule should be enhanced considering on efficient DL/UL operation at the SB-FD symbols.

Proposal 5: Study whether/how DL/UL collision rule is enhanced for efficient DL/UL operation at the SB-FD symbols

Even if SB-FD is adopted, HD TDD seems to be required for supporting legacy operation which could be suitable for transmitting DL or UL signal/cannel with wider bandwidth. Therefore, it needs to be studied how to operate well both legacy HD TDD and SB-FD within time duration.

Proposal 6: Study how to operate well both legacy HD TDD and SB-FD within time duration.

Time boundary alignment

For supporting SB-FD operation, time boundary alignment between UL and DL within a slot for SB-FD should be studied. Following two candidates of time boundary alignment (i.e., symbol level and slot level) can be considered for study.

Firstly, symbol boundary alignment between DL signal/channel and UL signal/channel could be assumed. One of challenging point of operating SB-FD is how to reduce self-interference. Symbol level alignment could be helpful to adopt simplified implementation of self-interference cancelation (SIC) receiver in digital domain. On the other hand, in this case, since UL symbol boundary should be aligned to DL symbol boundary, the time grid of UL symbol within slot for SB-FD is different from the time grid of UL symbol within slot for HD TDD. Hence, two different values of NTA,offset for UL transmission in UE side could be required depending on slots where HD TDD is operated or SB-FD is operated.

Secondly, slot boundary alignment between DL and UL could be assumed. In this case, symbol boundary alignment between DL and UL within slots for SB-FD is not required. Depending on implementation of SIC receiver, SIC can be operated even if symbol boundary between DL and UL is not aligned. One of advantage of slot level alignment without symbol level alignment is to keep the time grid of UL symbol between slot for SB-FD and slot for HD TDD.

Proposal 7: Study time boundary alignment between UL and DL within a slot for SB-FD. Following options can be studied.

- Option 1: Symbol boundary alignment between DL signal/channel and UL signal/channel

- Option 2: Slot boundary alignment without symbol boundary alignment between DL signal/channel and UL signal/channel

Resource Allocation in Frequency Domain

BWP

According to current specification for TDD unpaired spectrum, UE does not expect to receive a configuration where the center frequency of a DL BWP is different from the center frequency of an UL BWP since the center frequency is shared among DL/UL BWPs with same ID.

TS38.331 BWP information element BWP ::= SEQUENCE { locationAndBandwidth INTEGER (0..37949), subcarrierSpacing SubcarrierSpacing, cyclicPrefix ENUMERATED { extended } OPTIONAL -- Need R } BWP field descriptions locationAndBandwidth Frequency domain location and bandwidth of this bandwidth part. The value of the field shall be interpreted as resource indicator value (RIV) as defined TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting NsizeBWP=275. The first PRB is a PRB determined by subcarrierSpacing of this BWP and offsetToCarrier (configured in SCS-SpecificCarrier contained within FrequencyInfoDL / FrequencyInfoUL / FrequencyInfoUL-SIB / FrequencyInfoDL-SIB within ServingCellConfigCommon / ServingCellConfigCommonSIB) corresponding to this subcarrier spacing. In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency (see TS 38.213 [13], clause 12)

In Rel-18 DE SI, it is assumed that SB-FD is operated in TDD unpaired spectrum. But, in case of SB-FD in TDD unpaired spectrum, different group of subbands within a spectrum are used for DL and UL, respectively. When distinguished subbands are designated for DL BWP and UL BWP which have same BWP-ID, it can be assumes that the center frequency of a DL BWP is same as or different from that of a UL BWP. For example, when DL BWP and UL BWP are configured as case of {D U D} pattern in frequency domain, same center frequency can be assumed for each BWP. On the other hand, if DL BWP and UL BWP are configured as case of {D U} pattern in frequency domain, center frequency of DL BWP can be same as or different from that of the UL BWP. Therefore, in Rel-18 DE SI, it needs to be discussed whether center frequency of a DL BWP of SB-FD should be same as or can be different from that of a UL BWP of SB-FD which has same BWP-ID.

Proposal 8: Study whether center frequency of a DL BWP of SB-FD should be same as or can be different from that of a UL BWP of SB-FD which has same BWP-ID.

Subband Partitioning

For supporting SB-FD operation, separated subband within a spectrum should be assigned for downlink and uplink. Also, guard frequency is required between subbands for DL and subbands for UL in order to prevent subband emission from DL subband to UL subband. Therefore, subband partitioning for supporting SB-FD operation should be studied considering on these basic elements (i.e., DL subband, UL subband and guard subband).

The DL subband, UL subband and guard subband can be configured within a BWP. Also, configured bandwidth for DL/UL can be consecutive or not. In addition, the subband partitioning could be semi-statically configured. For supporting SB-FD operation, it needs to be discussed whether discontinuous subband can be assigned for a BWP. For SB-FD case, it can be assumed that subbands at both lower frequency and higher frequency within spectrum can be used for downlink and subbands in the middle of spectrum can be used for uplink. In this case, it can be assumed that two groups of subbands for DL are used for consisting DL BWP in order to provide more frequency resources and frequency diversity gain. But, in current specification for BWP configuration, this type of BWP configuration cannot be supported because BWP is consisted of contiguous RBs which is indicated by the parameter ‘locationAndBandwidth’ in current BWP configuration.

If it is allowed that a BWP is consisted of discontinuous subbands, it needs to be discussed how to configure BWP with discontinuous subbands. As a simple example, it can be considered that additional parameter for resource indicator value (RIV) is introduced for indicating a resource blocks which is not used for the BWP.

Proposal 9: Study whether/how a BWP is consisted of discontinuous subbands.

In case of legacy TDD, length of available BW of DL BWP (or UL BWP) is not changed. On the other hand, when SB-FD operation is allowed, length of available BW of DL BWP (or UL BWP) would be different depending on time resource. For example, wide BW can be used in time resource for DL only (i.e., Half Duplex operation), and narrower BW would be available in time resource for SB-FD. Therefore, it can be assumed if time resource for half duplex (HD) and time resource for SB-FD are timely multiplexed, length of available BW of DL BWP (or UL BWP) can be changed depending on time resource. In this sense, it needs to be discussed whether/how different available BW of BWP depending on time resource for HD or SB-FD is supported.

Proposal 10: Study whether/how different length of available BW of a BWP depending on time resource for HD or SB-FD is supported.

Interference handling

In SB-FD, performance degradation could be caused due to various type of interference (i.e., self-interference, UE2UE Cross Link Interference (CLI), BS2BS CLI, inter-carrier interference between operators).  In order to fully take advantage from SB-FD, interference handling method should be studied in Rel-18 DE SI.

Cross Link Interference Handling

When SB-FD is operated, UL performance would be degraded due to severe inter-cell BS2BS CLI. In our companion contribution [2], UL performance due to inter-subband emission from DL subband of aggressor gNB is observed. In addition, intra-subband CLI could be appeared if frequency position for subbands designated for DL transmission and UL reception is not same at each gNBs. In order for handling inter-subband emission and/or intra-subband CLI from aggressor gNB, interference management schemes (e.g., long-term CLI measurement, scheduling, link adaption, power control, etc) can be studied.

In case of SB-FD, both intra-cell UE2UE CLI and inter-cell UE2UE CLI can be appeared, which is different from dynamic TDD scenario. For inter-cell UE2UE CLI handling, L3 based CLI measurement and reporting (i.e., L3 CLI-RSSI and SRS-RSRP) can be reused. On the other hand, for intra-cell UE2UE CLI handling, L1/L2 based CLI measurement and reporting can be applied, which was proposed as one of candidates for UE2UE CLI handling in Rel-16 CLI/RIM WI.

In addition, for inter-subband CLI measurement, CLI-RSSI can be configured, basically. But, for inter-subband CLI measurement, SRS-RSRP measurement seems not feasible because UE monitors emission power from inter-subband. For operating SRS-RSRP of inter-subband CLI measurement, it seems some enhancement is required.

Proposal 11: Study Cross Link Interference handling method for SB-FD. Followings can be studied.

- Interference management schemes (e.g., long-term CLI measurement, scheduling, link adaption, power control, etc) for handling inter-subband emission and/or intra-subband CLI from aggressor gNB

- L1/L2 based CLI measurement and reporting for intra-cell UE2UE CLI management

- CLI handling frame work for inter-cell UE2UE CLI management

Self-Interference Cancellation/Mitigation

For Self-Inference (SI) cancellation/mitigation, information of transmitted signal/channel can be fully shared between transmitter and receiver via internal interface, which is different characteristic from CLI handling. For example, beam management for DL transmission and UL reception can be applied simultaneously. Also, DL power adjustment and UL power control can be applied simultaneously for reduction of self-interference and increasing of received power level of desired signal/channel which is transmitted with higher power from UE according to power control indication of gNB. In addition, various types of implementation (e.g., antenna separation, RF level SI cancellation, Baseband level SI cancellation, subband filtering, etc.) can be applied.

Proposal 12: Study Self-Interference cancellation/mitigation method for SB-FD. Followings can be studied.

- Simultaneous beam management for DL transmission and UL reception

- Simultaneous power control for DL transmission and UL reception

- gNB implementation for Self-Interference cancellation/mitigation (e.g., antenna separation, RF level SI cancellation, baseband level SI cancellation, subband filtering, etc.)

MIMO operation

Depending on duplex scheme operated in gNB side, antenna scheme and/or beam management scheme can be differently applied.

In SB-FD case, antenna separation based self-interference reduction scheme can be applied. For example, some panel(s) among total panels can be used for DL transmission, and the other panel(s) which is geographically separated from the panel for DL transmission can be used for UL reception. But, in HD TDD case, following options for using multiple panels can be considered.

- Option 1: All panels can be used for DL transmission or UL reception in HD TDD case.

- Option 2: Designated panel(s) for DL transmission (or UL reception) in SB-FD case is used in HD TDD case.

Also, beam management among DL and UL can be applied for self-interference reduction in SB-FD case. On the other hand, in HD TDD case, beam management for DL and UL can be operated independently. In this case, UE may be configured different TCI state according to duplex scheme in gNB side.

In addition, in SB-FD case, reduced DL Tx power can be applied for reduction of self-interference. On the other hand, in HD TDD case, higher DL Tx power can be applied for maintaining DL coverage. If different level of DL Tx power is applied, CSI measurement/report could be required according to Tx power level.

For UL transmission at the SB-FD symbols, UE may transmit UL channel/signal with higher power spectral density in order to overcome residual self-interference of gNB.

Furthermore, if it is allowed that non-contiguous subband can be designated within a BWP for SB-FD case, several features (e.g., FDRA, reference signal, etc.) need to be modified for supporting SB-FD.

In this sense, it needs to be studied whether MIMO related configuration (e.g., antenna configuration, beam management, power control, CSI measurement/report, reference signal, etc.) can be differently applied according to duplex schemes (i.e., HD TDD and SB-FD) operated in gNB side.

Proposal 13: Study whether MIMO related configuration (e.g., antenna configuration, beam management, power control, CSI measurement/report, reference signal, etc.) can be differently applied according to duplex schemes (i.e., HD TDD and SB-FD) operated in gNB side.

Summary

In this contribution, we discussed on the subband non-overlapping full duplex.

1. Time Resource Utilization for SB-FD operation

TDD configuration for supporting SB-FD

Proposal 1: SB-FD operation can be allowed in the time resource configured as DL and/or Flexible.

Proposal 2: Information of time resource for SB-FD operation is provided to UE for determining UE behavior. Study how to indicate the information of time resource for SB-FD operation.

Proposal 3: Study whether TDD configuration mechanism should be updated for supporting SB-FD operation. If it is agreed that enhancement of TDD configuration for SB-FD operation is studied, following can be studied.

- Which type of TDD configuration (e.g., TDD-UL-DL-ConfigurationCommon and TDD-UL-DL-ConfigDedicated and Dynamic Slot-Format Indicator (SFI)) needs to be enhanced for supporting SB-FD operation

Proposal 4: Study whether multiple type of TDD UL/DL pattern can be allowed for BWPs when SB-FD operation is adopted.

Proposal 5: Study whether/how DL/UL collision rule is enhanced for efficient DL/UL operation at the SB-FD symbols

Proposal 6: Study how to operate well both legacy HD TDD and SB-FD within time duration.

Time boundary alignment

Proposal 7: Study time boundary alignment between UL and DL within a slot for SB-FD. Following options can be studied.

- Option 1: Symbol boundary alignment between DL signal/channel and UL signal/channel

- Option 2: Slot boundary alignment without symbol boundary alignment between DL signal/channel and UL signal/channel

2. Resource Allocation in Frequency Domain

BWP

Proposal 8: Study whether center frequency of a DL BWP of SB-FD should be same as or can be different from that of a UL BWP of SB-FD which has same BWP-ID.

Subband Partitioning

Proposal 9: Study whether/how a BWP is consisted of discontinuous subbands.

Proposal 10: Study whether/how different length of available BW of a BWP depending on time resource for HD or SB-FD is supported.

3. Interference handling

Cross Link Interference Handling

Proposal 11: Study Cross Link Interference handling method for SB-FD. Followings can be studied.

- Interference management schemes (e.g., long-term CLI measurement, scheduling, link adaption, power control, etc) for handling inter-subband emission and/or intra-subband CLI from aggressor gNB

- L1/L2 based CLI measurement and reporting for intra-cell UE2UE CLI management

- CLI handling frame work for inter-cell UE2UE CLI management

Self-Interference Cancellation/Mitigation

Proposal 12: Study Self-Interference cancellation/mitigation method for SB-FD. Followings can be studied.

- Simultaneous beam management for DL transmission and UL reception

- Simultaneous power control for DL transmission and UL reception

- gNB implementation for Self-Interference cancellation/mitigation (e.g., antenna separation, RF level SI cancellation, baseband level SI cancellation, subband filtering, etc.)

4. MIMO operation

Proposal 13: Study whether MIMO related configuration (e.g., antenna configuration, beam management, power control, CSI measurement/report, reference signal, etc.) can be differently applied according to duplex schemes (i.e., HD TDD and SB-FD) operated in gNB side.

Reference

- CMCC, “New SI: Study on evolution of NR duplex operation,” RP-213591, e-Meeting, December 6th – 17th, 2021

- LG Electronics, “Study on Evaluation for NR duplex evolution,” R1-2204529, e-Meeting, May 9th – 20th, 2022

- 3GPP RAN1 Chair’s note in RAN1#109-e meeting

- Annex. Agreements in RAN1#109-e meeting [3]

Agreement

Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation.

Conclusion

For discussion purpose only, SBFD symbol is defined as symbol with subbands that gNB would use for SBFD operation.

Agreement

Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation.

Conclusion

For discussion purpose, for SBFD operation within a TDD carrier, a SBFD subband consists of 1 RB or a set of consecutive RBs for the same transmission direction.

Agreement

At least study SBFD operation within a TDD carrier

Agreement

The time and frequency location of subbands within a TDD carrier are not fixed in the specification.

- Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier.

- Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed.