3GPP TSG RAN WG1 #110			R1-2207397

Toulouse, France, May 9th – 20th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on SRS enhancement

Agenda Item:	9.1.3.2

Document for: 	Discussion and Decision

- Introduction

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are two objectives requiring SRS enhancement, M-TRP CJT and 8TX UL transmission.

In this contribution, we discuss SRS enhancement for 8TX UL transmission and M-TRP CJT in Rel-18 MIMO.

- SRS enhancement for 8 TX UL transmission

As discussed in our companion contribution [2] for 8TX UL transmission, 8 ports SRS resource(s) with usage of ‘codebook’ or ‘nonCodebook’ are needed to support up to 8 layers for CB/NCB PUSCH transmission. In addition, SRS enhancement for antenna switching for 8T8R should be also discussed. The agreement below is considered as a starting point for the subsequence clauses.

Agreement For SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices, study aspects include, for SRS for CB/NCB/AS, - Design parameters, including the maximum number of SRS resource sets, number of SRS resource sets, number of SRS resources, number of ports per resource, number of OFDM symbols, the allowed configurations for comb / comb shifts / cyclic shifts, number of simultaneous ports / resources / resource sets per OFDM symbol - For the next decision point, study - Whether to support 8 ports in one or multiple resources - Whether to support 8 ports in one or multiple OFDM symbols - The maximum number of SRS resource sets. - Note: For SRS for NCB, number of ports per SRS resource is still 1 (same as R15)

2.1 Enhancements for CB

In Rel-15/16/17, an SRS-Resource can be configured with up to 4 ports. How to extend this feature needs to be discussed for up to 8 UL layers. We think the three factors captured in the agreement above (i.e., the number of required SRS-Resources, the number of required OFDM symbols, and the maximum number of SRS-ResourceSets) would be good starting points. Considering them, we try to draw some alternatives to achieve SRS for CB transmission with up to 8 layers as follows:

Table 2.1-1: Alternatives for CB transmission with up to 8 layers

|  | # of ports per resource | # of ports per OFDM symbol | Pros | Cons |
| --- | --- | --- | --- | --- |
| Legacy (4-port) | 4 ports | 4 ports | N/A | N/A |
| Alt-1 | 8 ports | 8 ports | - No more OFDM symbols needed | - Tx power per port decreases - Extension of #SRS ports per SRS-Resource |
| Alt-2 | 4 ports | 8 ports | - No more OFDM symbols needed | - Tx power per port decreases - Relationship b/w resources may be needed |
| Alt-3 | 8 ports | 4 ports | - Tx power per port is kept | - 2 OFDM symbols are consumed - 2 OFDM symbols need to be identified by an SRS-Resource configuration |
| Alt-4 | 4 ports | 4 ports | - Tx power per port is kept | - Relationship b/w resources may be needed |

Alt-1 is 8-port SRS within 1 OFDM symbol by an SRS-Resource configuration. It implies the maximum number of SRS ports is extended. Alt-2 can also achieve 8-port SRS within 1 OFDM symbol, while not by an SRS-Resource configuration. One approach could be to configure multiple SRS-Resources within 1 OFDM symbol, where 8 ports can be achieved in total. Meanwhile, Alt-3 doesn’t support 8-port SRS within 1 OFDM symbol, while 8-port SRS can be configured by an SRS-Resource. It may be achieved by configuring (or indicating) multiple OFDM symbols for an SRS-Resource, where up to 4 ports can be sounded within each OFDM symbol. Alt-4 support neither 8-port SRS within 1 OFDM symbol nor configuring 8-port SRS by an SRS-Resource.

As per the categorization above, Alt-1 and Alt-2 have a clear advantage in terms of UL resource efficiency, but per-port transmit power will decrease. In addition, Alt-1 requires new design for SRS-Resource to support 8-port SRS transmission, while Alt-2 requires indication of the relationship between multiple SRS-Resources. On the other hand, Alt-3 or Alt-4 does not decrease per-SRS transmit power since maximum SRS ports within a OFDM symbol remains the same as in Rel-17, while they require 2 OFDM symbols to sound. Alt-3 also requires extending SRS-Resource to configure 2 OFDM symbols. Alt-4 requires to define relationship between multiple SRS-Resources.

Among the 4 alternatives above, we prefer Alt-1 the best. The main reason is UL resource efficiency considering that this alternative requires 1 OFDM symbol only, as it seems the most straightforward for a UE supporting up to 8 UL layers. We believe such a UE can easily support 8-port SRS transmission within 1 OFDM symbol, which deserves extending the number of SRS ports per SRS-Resource. Also, a disadvantage of Alt-1, i.e., decrease of per-port transmit power, wouldn’t be very serious as advanced UEs (e.g., FWA, CPE, etc) are the main target, for which higher transmit power can be allowed compared with handheld UEs.

If Alt-1 is not very preferable for companies, we would be open to consider some other alternative(s) at this very beginning stage. For example, from UL resource efficiency perspective, we see Alt-2 could be more beneficial as well since it also requires up to 1 OFDM symbol to sound 8 layers. However, we would like to point out that in Alt-2, there would be some required enhancements on the structure of relevant RRC parameter(s). For example, a relationship between multiple SRS-Resources would be required to sound 8 layers, which will be fundamentally different from the legacy RRC parameter.

Proposal 2-1

- For CB PUSCH transmission with more than 4 layers, support 8-port SRS transmission within 1 OFDM symbol

In NR, so far, up to four ports are available for SRS transmission per UE, which follows the following extraction from 38.211 [3]:

##### 6.4.1.4.2 Sequence generation The sounding reference signal sequence for an SRS resource shall be generated according to r(pi)n,l'=ru,v(αi,δ)n 0≤n≤Msc,bSRS-1 l'∈0,1,…,NsymbSRS-1 where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.2 with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb. The cyclic shift αi for antenna port pi is given as αi=2πnSRScs,inSRScs,max nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,maxif NapSRS=4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise, where nSRScs∈0,1,…,nSRScs,max-1 is contained in the higher layer parameter transmissionComb. The maximum number of cyclic shifts nSRScs,max are given by Table 6.4.1.4.2-1. […] Table 6.4.1.4.2-1: Maximum number of cyclic shifts nSRScs,max as a function of KTC. KTC; nSRScs,max. 2; 8. 4; 12. 8; 6. ##### 6.4.1.4.3 Mapping to physical resources […] The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+noffsetFH+noffsetRPFS where k0(pi)=nshiftNscRB+kTC(pi)+koffsetl' mod KTC kTCpi=kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6 kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise noffsetFH=b=0BSRSmSRS,bNscRBnb noffsetRPFS=NscRBmSRS,BSRSkF+khopmod PFPF and - kF∈0,1,…,PF-1 is given by the higher-layer parameter StartRBIndex if configured, otherwise kF=0; - khop is given by Table 6.4.1.4.3-3 with khop=nSRSb'=bhopBSRSNb' mod PF Nbhop=1 if the higher-layer parameter EnableStartRBHopping is configured, otherwise khop=0.

In summary:

- For 2-port SRS, the same comb index is applied for both ports. Port multiplexing is achieved by applying different cyclic shifts (with the most discrete phase rotation values).

- For 4-port SRS,

  - For comb index, when the maximum number of cyclic shifts is 6 (i.e., comb8 is configured), or the configured cyclic shift is larger than half of the maximum number of cyclic shifts, two different comb indexes are applied: one for port 1000 and 1002, and the other for port 1001 and 1003. Otherwise, the same comb index is applied across all the ports, similar to 2-port SRS.

  - For cyclic shift values, when the maximum number of cyclic shifts is 6 (i.e., comb8 is configured), two different cyclic shift values are applied: one for port 1000 and 1001, and the other for 1002 and 1003. Otherwise, different comb indexes are used for each port.

Observation 2-1

- Legacy SRS supports up to 4 ports by using multiple cyclic shifts and/or multiple comb indexes depending on the maximum number of cyclic shifts

We believe the above should be a starting point if we extend the number of SRS ports per SRS-Resource (i.e., Alt-1 or Alt-3 above). On top of that, we see several potential directions for the extension as described below:

- Use of more cyclic shift values

So far, up to four cyclic shifts are available for SRS transmissions, where different SRS port is associated with different cyclic shift values. Similarly, more than four cyclic shifts (e.g., eight cyclic shifts) can be considered to support 8-port SRS transmissions. In this case, the amount of resources required for the SRS transmissions can be minimized. However, orthogonality among ports can be degraded in case of large delay spread.

- Use of more frequency domain resources (e.g., more comb indexes)

As summarized above, except for some cases, a single comb index is shared by SRS ports transmitted by a UE. For 8-port SRS, more comb indexes (i.e., two indexes always, or four comb indexes) can be considered. In this case, the number of cyclic shifts used for multiplexing in a RE can be kept as in Rel-17, thus, the same orthogonality among ports would be achieved even in case of large delay spread. On the other hand, more comb indexes a UE uses, more frequency domain resources it consumes. It may degrade the system wise performance especially for UL. Also, per-symbol multiplexing capacity for different UEs may be degraded.

- Use of more time-domain resources

The third direction is to use more SRS symbols to express more than 4 SRS ports. Generally this direction has similar pros and cons to the use of more frequency-domain resources, i.e., orthogonality between different ports can be kept as in Rel-17, while more resources are needed for SRS transmission from UE perspective. Meanwhile, this direction could be a bit better than the use of more frequency-domain resources since the per-symbol multiplexing capacity may not be degraded, considering the amount of frequency-domain resources may not need to be increased.

Several approaches can be considered for this direction. One is, similar to DM-RS, TD-OCC across multiple SRS symbols. Another is, for example, to define the relationship between SRS port number(s) and the symbol number(s) configured for the SRS transmission. The former approach may have more significant specification/implementation impact since TD-OCC for SRS transmission will be a totally new functionality. The latter approach can be a bit more implementation-friendly, but may still require a certain amount of specification impact for e.g., specifying the relationship between SRS port and SRS symbol, which is also a new aspect for SRS. One note for the latter approach is that it may no longer be defined as 8-port SRS, rather a combination of the legacy SRS across symbols.

Table 2.1-1: Summary of the potential directions for supporting 8-port SRS

| Direction # | Pros | Cons |
| --- | --- | --- |
| - More cyclic shift values | The same amount of time/freq. resources as in Rel-17 can be achieved | Orthogonality between different SRS ports can be degraded in case of large delay spread |
| - More frequency-domain resources | Orthogonality between different SRS ports can be kept as in Rel-17 | More resources in frequency-domain are required |
| - More time-domain resources | Orthogonality between different SRS ports can be kept as in Rel-17 | More resources in time-domain are required |

From our perspective, the direction#1 or direction#2 above can achieve Alt-1, while the direction#3 achieves Alt-3. At this very beginning stage of the WI, we in general believe any direction can be studied further in RAN1. Meanwhile, we understand that the amount of workload for each agenda should be assessed and decided carefully. If the discussion needs to be narrowed down more, our suggestion is to focus on the direction#2 and/or #3, since cons of direction#1 (i.e., the orthogonality degradation) may be more significant issue. The other two directions, although it may consume more resources, can maintain the orthogonality, even in case of large delay spread.

Proposal 2-2

- To extend the number of SRS ports per SRS-Resource, the following can be considered:

  - Use of more cyclic shifts (lower priority)

  - Use of more frequency domain resources

    - E.g., use of more comb indexes

  - Use of more time-domain resources

    - E.g., use of more SRS symbols for multi-port SRS transmission

If we do not extend the number of SRS ports per symbol (i.e., Alt-2 or Alt-4), it could be required to define a relationship between multiple SRS-Resources in our view since it tries to achieve 8 ports by two SRS-Resources. For example, UE with more than 4 layers needs to be aware that each port configured by the multiple SRS-Resources (which may be associated with the same or different SRS-ResourceSet(s)) is associated with different antenna ports. How to express it in the specification should be discussed. Alt-2 can be achieved by configuring the same SRS symbol in the multiple SRS-Resources. It may be almost equivalent to Alt-1 technically. Alt-4 can be achieved by similar approach, while the multiple SRS-Resources must be configured with different SRS symbols. It is not very preferred by us as it consumes doubled UL resources.

Given above, we draw the following proposals.

Proposal 2-3

- To support 8-port SRS without extending the number of SRS ports per SRS-Resource, a relationship between multiple SRS-Resources (which may be associated with the same or different SRS-ResourceSet(s)) can be required.

  - E.g., different SRS ports across the SRS-Resources are associated with different antenna ports

2.2 Enhancements for NCB

For NCB PUSCH transmission, one or two SRS-ResourceSet(s) is configured, where at most 4 SRS-Resources with up to 1 port can be configured. When scheduling a PUSCH, gNB indicates one or multiple beams to be used via SRI, whose bit width considers the total number of SRS-Resource combinations.

When the number of PUSCH layers is extended to more than 4, some enhancements should be considered to cover NSB PUSCH transmission. Firstly, we need to decide how to configure the required number of SRS-Resources. As per the agreement above, the number of SRS ports per SRS-Resource is 1 even in Rel-18, so up to 8 SRS-Resources need to be configured for NCB. One approach is to extend the maximum number of SRS-Resources per SRS-ResourceSet to 8, which we believe is straightforward.

Also, how to indicate a certain subset of SRS-Resources for NCB PUSCH transmission should be considered. If we totally follow the same method as in Rel-17, any combination from up to 8 SRS-Resources shall be covered by DCI indication, which results in much heavier DCI overhead. For 8-layer NCB PUSCH transmission, we believe whether/how to solve this issue should be discussed well. Especially for partial-coherent UEs, considering the port coherency within each coherent group, the possible combinations from 8 SRS-Resources are limited. Even for fully-/non-coherent UEs, SRI indication from limited combinations can be also sufficient.

We would like to note that in the current agenda separation, one could say which agenda to discuss the issue of SRI indication for NCB might be unclear. We believe it would be straightforward to discuss it in this agenda as agreed in the last e-meeting. Meanwhile, we would be ok even if this issue is discussed in another agenda, e.g., 9.1.4.1 or 9.1.4.2.

Proposal 2-4

- For NCB PUSCH transmission with more than 4 layers, the following should be discussed

  - How to configure 8 SRS-Resources for NCB PUSCH transmission

  - How to indicate a subset of SRS-Resources for NCB PUSCH transmission

2.3 SRS for 8T8R antenna switching

In Rel-15, SRS antenna switching configuration for DL CSI acquisition was supported, where less than 4 Rx ports with 1/2/4 Tx port(s) are considered. For T=R (1T=1R, 2T=2R, or 4T=4R), up to two SRS resource sets each with one SRS resource can be configured with usage of ‘antennaSwitching’. In Rel-17, this functionality was extended to cover antenna architectures with 6/8 Rx ports, where the maximum number of Tx ports is still 4. With a new Rel-17 UE capability reporting, up to two SRS resources configured with ‘Semi-persistent’ and up to one SRS resource set configured with ‘periodic’ can be configured.

In Rel-18, since we consider more than 4 layers for UL transmission, additional antenna architecture(s), e.g., 6 or 8 Tx ports, can be considered as well. Therefore, depending on the exact approach(s) to support more than 4 layers for UL transmission, whether/how to support SRS antenna switching configuration for such antenna architectures may need to be discussed in Rel-18. We see this aspect much dependent on whether to support 8-port SRS, as described in section 2.1. If we support 8-port SRS for antenna switching, full use of 8 Tx ports can be considered for the purpose of SRS antenna switching. On the other hand, if we do not support 8-port SRS for antenna switching, assuming the number of Tx ports for SRS transmission is kept as in Rel-17, we do not see the need to discuss a specification impact on this functionality since all cases can be covered by Rel-17 enhanced antenna switching configurations.

Proposal 2-5

- Discuss SRS configuration with usage of ‘antennaSwitching’ for 8T8R.

  - Study whether to configure 8-port SRS for antenna switching.

- SRS enhancement for TDD CJT

As discussed in our companion contribution [4] for CJT, there are two deployment scenarios, intra-site M-TRP CJT and inter-site M-TRP CJT. For both scenarios, in TDD operation, UE can be configured with SRS transmission and multiple TRPs can measure the SRS to derive DL CSI.

Between the two scenarios (i.e., intra-site M-TRP or inter-site M-TRP), we believe intra-site M-TRP should be prioritized in Rel-18. Therefore, the discussion on SRS for TDD CJT should also consider potential enhancements beneficial for intra-site M-TRP scenario as higher priority.

Moreover, there can be multiple types of SRS usage for TDD CJT. For example, an SRS from a UE may be measured by more than a TRP simultaneously. It may be easily achieved in intra-site CJT scenario as the coherent TRPs will be located closely. It may be possible even in inter-site CJT scenario if some conditions are satisfied. With this approach, from a UE perspective, an SRS per cell (i.e., TRP-common SRS) can be sufficient. Comparing the other approach (e.g., per-TRP SRS), TRP-common SRS usage will decrease SRS overhead to be required for CJT scenario. Therefore, for CJT, TRP-common SRS usage is preferred in our view. Potential enhancements in Rel-18 should consider such scenario/usage.

Proposal 3-1

- For TDD CJT, TRP-common SRS usage (i.e., an SRS from a UE is measured by multiple coherent TRPs) should be considered as high-priority

As can be seen in the following agreement in the last e-meeting, there are a number of proposals for this issue:

Agreement Study the following for SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS interference randomization and/or capacity enhancement - Randomized frequency-domain resource mapping for SRS transmission - E.g., further enhancements to frequency hopping, comb hopping - Randomized code-domain resource mapping for SRS transmission - E.g., cyclic shift hopping/randomization, sequence hopping/randomization, per-hop sequence from a long SRS sequence - Randomized transmission of SRS - E.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS - Per-TRP power control and/or power control of one SRS towards to multiple TRPs - SRS TD OCC - Increasing the maximum number of cyclic shifts - E.g., multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts - Precoded SRS for DL CSI acquisition - Enhanced signaling for flexible SRS transmission - E.g., dynamic update of SRS parameters - Partial frequency sounding extensions - E.g., larger partial frequency sounding factor, starting RB location hopping enhancements, partial frequency hopping on other bandwidths corresponding to , besides the last bandwidth - Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment - E.g., configuration of v (sequence index within a group) per SRS resource - E.g., configuration of cyclic shift per SRS port per SRS resource. - Resource mapping for SRS transmission based on network-provided parameters or system parameters - E.g., SRS resource mapping based on network-provided parameters (e.g., configurable indexes) or system parameters (e.g., slot index) Note: PAPR performance and maintaining DFT waveform property should be considered when deciding the enhancement for Rel-18.

Above can be categorized according to issue(s) to be solved captured in the WID:

Table 3-1: Candidate solutions per issue

| Issues | Candidate solutions |
| --- | --- |
| Interference randomization | - Randomized frequency-domain resource mapping for SRS transmission - E.g., further enhancements to frequency hopping, comb hopping - Randomized code-domain resource mapping for SRS transmission - E.g., cyclic shift hopping/randomization, sequence hopping/randomization, per-hop sequence from a long SRS sequence - Randomized transmission of SRS - E.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS - Per-TRP power control and/or power control of one SRS towards to multiple TRPs - Precoded SRS for DL CSI acquisition - Enhanced signaling for flexible SRS transmission - E.g., dynamic update of SRS parameters - Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment - E.g., configuration of v (sequence index within a group) per SRS resource - E.g., configuration of cyclic shift per SRS port per SRS resource. - Resource mapping for SRS transmission based on network-provided parameters or system parameters - E.g., SRS resource mapping based on network-provided parameters (e.g., configurable indexes) or system parameters (e.g., slot index) |
| Capacity enhancement | - SRS TD OCC - Increasing the maximum number of cyclic shifts - E.g., multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts - Partial frequency sounding extensions - E.g., larger partial frequency sounding factor, starting RB location hopping enhancements, partial frequency hopping on other bandwidths corresponding to , besides the last bandwidth |

When TRP-common SRS usage is mainly considered, we believe interference randomization (or mitigation) would be more significant than capacity since the same per-cell SRS capacity as in legacy NR releases can be expected for the usage.

Proposal 3-2

- For SRS enhancement for TDD CJT, considering TRP-common SRS usage, interference randomization (or mitigation) should be prioritized

3.1 Interference randomization for SRS

If there are many CJT UEs from multiple TRPs configured with SRS transmission for DL CSI acquisition, the orthogonality of those SRS transmissions cannot be guaranteed. Then the collision of two SRS resources from different TRPs is un-avoided and inter-TRP cross-SRS interference exists. As shown in Fig. 3-1, if SRS resources from UEs in different TRPs collide, the SRS resource from a CJT UE will suffer strong interference from a cell center UE. In addition, due to semi-static configuration of SRS parameters, the strong inter-TRP interference is continuous and will largely impact the accuracy of DL CSI acquisition.

Fig 3-1: An example of inter-TRP cross-SRS interference

To avoid continuous serious inter-TRP interference on SRS measurement, how to achieve interference randomization for SRS transmission should be studied. In the last e-meeting, several alternatives were captured in the agreement above, we have tried to summarize/compare with the alternatives in the following table:

Table 3-2: Analysis of candidate solutions for interference randomization

| Solutions | Issue to be solved | Pros | Cons |
| --- | --- | --- | --- |
| Randomized f-domain resource | Inter-TRP interference in a f-domain resource | Less signaling overhead | Applicable to FH case only |
| Randomized c-domain resource | Inter-TRP interference in a c-domain resource | Less signaling overhead No/smaller restriction on sounding performance | Gain may be smaller? |
| Randomized t-domain resource | Inter-TRP interference in a t-domain resource | Less signaling overhead | Sounding may be canceled in a certain occasion |
| Power control (per TRP/multi-TRP) | Inter-TRP interference for inter-site scenario | Legacy design for f-/c-/t-domain can be reused | Benefit for intra-site scenario is unclear SRS overhead increases per TRP |
| Precoded SRS | Inter-TRP interference for inter-site scenario (or S-domain capacity) | Legacy design for f-/c-/t-domain can be reused | Benefit for intra-site scenario is unclear SRS overhead increases per TRP |
| Enhanced signal for flexible SRS transmission | Inter-TRP interference in a f-/c-/t-domain resource | More controllable by gNB | Signaling overhead |
| Enhanced sequence/CS configuration | Inter-TRP interference in a c-domain resource | More controllable by gNB No/smaller restriction on sounding performance | Gain may be smaller? |
| Resource mapping based on NW-provided parameter | Inter-TRP interference | More controllable by gNB | More different from the legacy design |

Amongst the alternatives above, one of our preferred ones is enhanced signaling for flexible SRS transmission. One of the benefits would be its controllability by gNB. It makes the NW operation easier compared with the solutions with randomization. Also, due to its flexibility, larger gain of interference mitigation can be achieved. We view its disadvantage captured above, i.e., signaling overhead, not very significant as such signaling will be required if and only if it is necessary depending on the situation.

If we consider a kind of randomization considering signaling overhead, another preference from our side would be sequence (group) hopping/randomization, which was exemplified in randomized code-domain resource. Comparing with frequency-/time-domain randomization, which needs to randomize sounding time-frequency resource, it would not need such restriction on sounded resources. One possibility is to take into account TRP index (or information per TRP) in addition to SRS sequence index and time-domain location of SRS symbol for SRS sequence determination. With such approaches, variety of SRS sequences would be extended as per the number of coherent TRPs within a cell. In addition, the TRP-related information can be time-varying to optimize SRS sequence per SRS symbol.

Proposal 3-3

- For interference randomization for TDD CJT, support the following

  - Enhanced signaling for dynamic update of SRS resource parameters, e.g., time/frequency resource allocation, hopping, sequence group number, sequency number, comb, CS, etc.

  - Randomized code-domain resources, based on TRP index.

3.2 SRS capacity enhancement

As we describe in the section 3, assuming TRP-common SRS usage can be mainly considered, we do not see so strong necessity to enhance SRS capacity on top of Rel.17 enhancement. On the other hand, if NW sees the need of per-TRP SRS, the required capacity would increase based on the number of coherent TRPs. In this case, it could be necessary to improve SRS capacity. Otherwise, NW may have difficulty to accommodate all the SRS resources within limited orthogonal resources, which results in inefficient usage of UL resources for SRS.

In Rel-17, partial sounding has improved the SRS multiplexing capacity in frequency domain. Considering following constraints on SRS enhancement for TDD CJT in Rel-18 MIMO WID: 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences, further consideration on time domain (e.g., TD-OCC), frequency domain (e.g., partial sounding with larger partial frequency sounding factor), or sequence domain (e.g., larger max. cyclic shift) enhancement can be discussed. With above methods, SRS transmissions from more UEs can be multiplexed on the same UL resources.

In our view, the extension of partial sounding can be considered as one of the straightforward solutions as extending some existing parameters can be sufficient. Cyclic shift extension might be related to the SRS enhancements for 8Tx UL as there might be the discussion on extending cyclic shifts as well. TD-OCC for SRS can also be considered by e.g., reusing DMRS design, however, it may not help improving per-symbol SRS capacity.

Proposal 3-4

- For SRS capacity enhancement, if needed, support extending partial sounding in Rel-17.

  - e.g., support smaller value for P_F

- Conclusion

In this contribution, we discussed SRS enhancement for 8TX UL transmission and M-TRP CJT in Rel-18 MIMO. Based on the discussion, we made following proposals.

SRS enhancement for 8TX UL transmission

Proposal 2-1

- For CB PUSCH transmission with more than 4 layers, support 8-port SRS transmission within 1 OFDM symbol

Observation 2-1

- Legacy SRS supports up to 4 ports by using multiple cyclic shifts and/or multiple comb indexes depending on the maximum number of cyclic shifts

Proposal 2-2

- To extend the number of SRS ports per SRS-Resource, the following can be considered:

  - Use of more cyclic shifts (lower priority)

  - Use of more frequency domain resources

    - E.g., use of more comb indexes

  - Use of more time-domain resources

    - E.g., use of more SRS symbols for multi-port SRS transmission

Proposal 2-3

- To support 8-port SRS without extending the number of SRS ports per SRS-Resource, a relationship between multiple SRS-Resources (which may be associated with the same or different SRS-ResourceSet(s)) can be required.

  - E.g., different SRS ports across the SRS-Resources are associated with different antenna ports

Proposal 2-4

- For NCB PUSCH transmission with more than 4 layers, the following should be discussed

  - How to configure 8 SRS-Resources for NCB PUSCH transmission

  - How to indicate a subset of SRS-Resources for NCB PUSCH transmission

Proposal 2-5

- Discuss SRS configuration with usage of ‘antennaSwitching’ for 8T8R.

  - Study whether to configure 8-port SRS for antenna switching.

SRS enhancement for TDD CJT

Proposal 3-1

- For TDD CJT, TRP-common SRS usage (i.e., an SRS from a UE is measured by multiple coherent TRPs) should be considered as high-priority

Proposal 3-2

- For SRS enhancement for TDD CJT, considering TRP-common SRS usage, interference randomization (or mitigation) should be prioritized

Proposal 3-3

- For interference randomization for TDD CJT, support the following

  - Enhanced signaling for dynamic update of SRS resource parameters, e.g., time/frequency resource allocation, hopping, sequence group number, sequency number, comb, CS, etc.

  - Randomized code-domain resources, based on TRP index.

Proposal 3-4

- For SRS capacity enhancement, if needed, support extending partial sounding in Rel-17.

  - e.g., support smaller value for P_F

References

- 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e, Dec. 2021.

- 3GPP R1-2207399, “Discussion on 8 TX UL transmission”, RAN1#110, August 2022.

- 3GPP TSG RAN 38.211 V17.2.0, “Physical channels and modulation”, June 2022

- 3GPP R1-2207395, “Discussion on CSI enhancement”, RAN1#110, August 2022.