3GPP TSG-RAN WG1 #110			R1-2207428

Toulouse, France, August 22nd – 26th, 2022

Agenda item:	9.12.1

Title:	Discussion on coverage enhancement for NR NTN

Source:	NTT DOCOMO, INC.

Document for:	Discussion/Decision

- Introduction

At the RAN#94-e meeting a new work item on NR NTN enhancements was endorsed [1][2]. One of the objectives is to evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN. In this contribution, we present our views on coverage enhancement for NTN.

- Discussion

  - Link budget calculation

At the RAN1#109-e meeting, the following agreements were made [4].

Agreement Coverage performance in NR NTN is evaluated according to the following steps. - Step 1: CNR is calculated as defined in 6.1.3.1 of TR38.821 - For polarization loss, - 3 dB polarization loss is assumed as baseline, and companies are encouraged to report the value and corresponding justification if other value is used - Step 2: Required SNR of target service is evaluated by LLS - Step 3: The CNR and the required SNR are compared Agreement Coverage performance in NR NTN is evaluated for GEO/LEO-1200/LEO-600 scenarios. - Note: Service type for each scenario is discussed separately - Note: Parameter set (Set-1/2) is discussed separately - Note: MEO can be evaluated optionally Agreement For link budget calculation, parameters in the following table is assumed. Parameters; Notes. Carrier frequency; 2 GHz for DL and UL (S-band). Channel bandwidth; FFS. Satellite altitude; 600 km, 1200 km, 10000 km, 35786 km. Target elevation angle; [30 (LEO), 12.5 (GEO-Set 1) , 20° (GEO –Set 2), 30° (MEO)]. Atmospheric loss; Equation (6.6-8) in [2]. Shadowing margin; 3 dB. Scintillation loss; Section 6.6.6 in [2] Ionospheric loss: = 2.2 dB (note 1) Tropospheric loss: Table 6.6.6.2.1-1 of [2]. Additional loss; 0 dB. Clear sky conditions; Yes. Satellite antenna polarization; Circular polarization. Terminal type; [S band: (M, N, P) = (1,1,2)]. Free space path loss; Equation (6.6-2) in [2]. Terminal RF parameters; FFS. Satellite RF parameters; FFS. Polarization loss; As agreed separately. Outcome; CNR. - NOTE 1: Based on P3 curve for 1% of time from Figure 6.6.6.1.4-1 of [2] after frequency scaling. - dB - NOTE 2: [2] in this table is 3GPP TR 38.811 v15.2.0: "Study on New Radio (NR) to support non-terrestrial networks (Release 15).

For the CNR calculations, we considered only GEO and LEO-1200 scenarios.

    - UE antenna gain

At RAN1#109-e the following agreement was made.

Agreement Evaluate coverage performance for the following UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration. Characteristics; Handheld. Frequency band; S band (i.e. 2 GHz). Antenna type and configuration; 1 TX, 2TX (optional) / 2 RX with omni-directional antenna element Note: companies should provide their assumption on polarization. Polarisation; Linear. Rx Antenna gain; [X] dBi per element. Antenna temperature; 290 K. Noise figure; 7 dB. Tx transmit power; 200 mW (23 dBm). Tx antenna gain; [X] dBi per element. - X = -5 as working assumption - Send an LS to RAN4 to ask whether above antenna gain is valid and if invalid, appropriate value.

CNR can be calculated by the following equation as defined in 6.1.3.1 of TR38.821. The major difference from Rel-16 NTN study is the -5 dBi UE Tx/Rx antenna gain. UE antenna gain affects G/T for DL CNR and EIRP for UL CNR.

CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ]

EIRP dBW=PT dBW-LC dB+GT [dBi]

GT dBi=GT,e dBi+10log10NT,a,  &array antenna10log10η∙π2∙D[m]2λ[m]2,  &parabolic antenna

GTdB=GR dBi-Nf [dB]-10log10T0 [K]+Ta K-T0 [K]10-0.1Nf [dB]

GR dBi=GR,edBi+10log10NR,a-Lp[dB],  &array antenna10log10η∙π2∙D[m]2λ[m]2,  &parabolic antenna

    - PFD limit

In ITU radio regulations articles [5], the following table can be found.

TABLE  21-4     (Rev.WRC-19)

| Frequency band | Service* | Limit in dB(W/m2) for angles of arrival (δ) above the horizontal plane |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- |
|  |  | 0°-5° | 5°-25° | 25°-90° |  |
| 1 525-1 530 MHz 7 (Region 1, Region 3) 1 670-1 690 MHz 12 1 690-1 700 MHz (Nos. 5.381 and 5.382) 1 700-1 710 MHz 2 025-2 110 MHz 2 200-2 300 MHz | Meteorological-satellite (space-to-Earth) Space research (space-to-Earth) (space-to-space) Space operation (space-to-Earth) (space-to-space) Earth exploration- satellite (space-to-Earth) (space-to-space) | 0-5° | 5°-25° | 25°-90° | 4 kHz |
|  |  | −154 | −154 + 0.5(δ − 5) | −144 |  |

Since there are no regulations that include a carrier frequency of 2 GHz (i.e., 2000 MHz), values for the closest frequency band were used to consider power flux density (PFD). The calculation results of PFD limits are as follows.

- GEO – set1 (elevation angle: 12.5°)

−154 + 0.5(12.5 − 5) = -150.3 dBW/m2/4kHz

    - -126.3 dBW/m2/MHz

- GEO – set2 (elevation angle: 20°)

-154 + 0.5(20 – 5) = -146.5 dBW/m2/4kHz

    - -122.5 dBW/m2/MHz

- LEO-1200 (elevation angle: 30°)

-144 dBW/m2/4kHz

    - -120 dBW/m2/MHz

    - DL EIRP density

PFD can be calculated by the following equation, where d is the distance between the satellite and the ground terminal.

PFD=EIRP density-10log4πd2

The results of the PFD calculation are as follows. The parameters for each satellite are listed in Table 6.1.1.1-1 and 6.1.1.1-2 in TR38.821.

- GEO – set1 (elevation angle: 12.5°)

  - EIRP density = 59 dBW/MHz, d = 40319 km

    - PFD = 59 – 163.1 = -104.1 dBW/m2/MHz

- GEO – set2 (elevation angle: 20°)

  - EIRP density = 53.5 dBW/MHz, d = 39551 km

    - PFD = 53.5 – 162.9 = -109.4 dBW/m2/MHz

- LEO-1200 – set1 (elevation angle: 30°)

  - EIRP density = 40 dBW/MHz, d = 1999 km

    - PFD = 40 – 137.0 = -97.0 dBW/m2/MHz

- LEO-1200 – set2 (elevation angle: 30°)

  - EIRP density = 34 dBW/MHz, d = 1999 km

    - PFD = 34 – 137.0 = -103.0 dBW/m2/MHz

Each is larger than the PFD limit calculated in the previous section. If PFD regulation is to be considered, the EIRP density would need to be reduced to the following values:

- GEO – set1: 36.8 dBW/MHz

- GEO – set2: 40.4 dBW/MHz

- LEO-1200 – set1: 17.0 dBW/MHz

- LEO-1200 – set2: 17.0 dBW/MHz

    - CNR calculation results

Table 1 shows the results of the CNR calculations based on the above considerations. The number of UE transmit antennas is set to one, and the number of UE receive antennas is set to two.

Table 1 CNR calculation results

| Satellite orbit | Satellite parameter set | Channel | Frequency [GHz] | TX: EIRP [dBm] | RX: G/T [dB/T] | Bandwidth [MHz] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GEO | 1 | DL channels w/o consideration of PFD | 2.0 | 103.8 | -36.6 | 20.0 | 190.6 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -5.0 |
|  |  | DL channels w/ consideration of PFD | 2.0 | 81.6 | -36.6 | 20.0 | 190.6 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -27.2 |
|  |  | PUCCH (1RB) | 2.0 | 18.0 | 19.0 | 0.18 | 190.6 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -16.0 |
|  |  | PUSCH (2RB) | 2.0 | 18.0 | 19.0 | 0.36 | 190.6 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -19.0 |
|  |  | PRACH format 0/2 | 2.0 | 18.0 | 19.0 | 1.08 | 190.6 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -23.7 |
|  |  | PRACH format B4 | 2.0 | 18.0 | 19.0 | 2.16 | 190.6 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -26.7 |
|  | 2 | DL channels w/o consideration of PFD | 2.0 | 98.3 | -36.6 | 20.0 | 190.4 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -10.3 |
|  |  | DL channels w/ consideration of PFD | 2.0 | 85.2 | -36.6 | 20.0 | 190.4 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -23.4 |
|  |  | PUCCH (1RB) | 2.0 | 18.0 | 14.0 | 0.18 | 190.4 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -20.8 |
|  |  | PUSCH (2RB) | 2.0 | 18.0 | 14.0 | 0.36 | 190.4 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -23.8 |
|  |  | PRACH format 0/2 | 2.0 | 18.0 | 14.0 | 1.08 | 190.4 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -28.5 |
|  |  | PRACH format B4 | 2.0 | 18.0 | 14.0 | 2.16 | 190.4 | 0.2 | 3.0 | 2.2 | 3.0 | 0.0 | -31.5 |
| LEO-1200 | 1 | DL channels w/o consideration of PFD | 2.0 | 84.8 | -36.6 | 20.0 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | 2.2 |
|  |  | DL channels w/ consideration of PFD | 2.0 | 61.8 | -36.6 | 20.0 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -20.8 |
|  |  | PUCCH (1RB) | 2.0 | 18.0 | 1.1 | 0.18 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -7.7 |
|  |  | PUSCH (2RB) | 2.0 | 18.0 | 1.1 | 0.36 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -10.7 |
|  |  | PRACH format 0/2 | 2.0 | 18.0 | 1.1 | 1.08 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -15.4 |
|  |  | PRACH format B4 | 2.0 | 18.0 | 1.1 | 2.16 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -18.4 |
|  | 2 | DL channels w/o consideration of PFD | 2.0 | 78.8 | -36.6 | 20.0 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -3.8 |
|  |  | DL channels w/ consideration of PFD | 2.0 | 61.8 | -36.6 | 20.0 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -20.8 |
|  |  | PUCCH (1RB) | 2.0 | 18.0 | -4.9 | 0.18 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -13.7 |
|  |  | PUSCH (2RB) | 2.0 | 18.0 | -4.9 | 0.36 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -16.7 |
|  |  | PRACH format 0/2 | 2.0 | 18.0 | -4.9 | 1.08 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -21.4 |
|  |  | PRACH format B4 | 2.0 | 18.0 | -4.9 | 2.16 | 164.5 | 0.1 | 3.0 | 2.2 | 3.0 | 0.0 | -24.4 |

  - Link level simulation

Link level simulations of PUSCH, PUCCH, PRACH and PDCCH were performed based on the agreed simulation assumptions. The simulation assumptions common to all channels are listed in Table 2.

Table 2 Common simulation assumptions for all channels

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| Channel model | NTN-TDL-C (Rural) |
| UE speed | 3 km/h |
| BWP bandwidth | 20 MHz |
| Number of UE Tx antennas | 1 |
| Number of UE Rx antennas | 2 ((M, N, P) = (1,1,2)) |
| Frequency offset | 0.1 ppm |

    - PUSCH

Simulation assumptions for PUSCH are shown in Table 3.

Table 3 Simulation assumptions for PUSCH

| Parameter | Value |
| --- | --- |
| Frequency hopping | w/ inter-slot hopping hopping offset: 1/2 BWP, interval: 4 slots |
| SCS | 15 kHz |
| Target BLER | Msg3: 10% iBLER. VoIP: 2% rBLER Low data rate service: 10% iBLER |
| DMRS configuration | Type1, single symbol, no multiplexing with data dmrs-AdditionalPosition: pos3 |
| Waveform | DFT-s-OFDM |
| Number of PRBs | 2 |
| PUSCH duration | 14 OS |
| Modulation | QPSK |
| TBS | Msg3: 56 bits VoIP: 184 bits Low data rate service: 128 bits |
| Number of repetitions | Msg3: 16 VoIP: 20 Low data rate service: 32 |
| HARQ configuration | No HARQ retransmission |
| TBoMS | Disabled |
| Joint channel estimation | Msg3, VoIP: disabled Low data rate service: enabled (window length: 4) |

PUSCH simulation results are shown in Figure 1, Figure 2 and Figure 3.

Figure 1 Simulation results of PUSCH (GEO – set1)

Figure 2 Simulation results of PUSCH (GEO – set2)

Figure 3 Simulation results of PUSCH (LEO-1200)

The gap between required SNR and CNR is shown in Table 4. In the table, scenarios with negative performance gap values are those with insufficient coverage. Coverage enhancement is needed for PUSCH except Msg.3 for LEO-1200 with parameter set 1.

Table 4 Gap between required SNR and CNR for PUSCH

| Channel | Scenario | Required SNR | CNR | Performance gap |
| --- | --- | --- | --- | --- |
| PUSCH (Msg.3) | GEO – set1 | -14.0 dB | -19.0 dB | -5.0 dB |
|  | GEO – set2 | -12.7 dB | -23.8 dB | -11.1 dB |
|  | LEO-1200 – set1 | -11.3 dB | -10.7 dB | 0.6 dB |
|  | LEO-1200 – set2 | -11.3 dB | -16.7 dB | -5.4 dB |
| PUSCH (low data rate) | GEO – set1 | -15.0 dB | -19.0 dB | -4.0 dB |
|  | GEO – set2 | -13.7 dB | -23.8 dB | -10.1 dB |
| PUSCH (VoIP) | LEO-1200 – set1 | -6.0 dB | -10.7 dB | -4.7 dB |
|  | LEO-1200 – set2 | -6.0 dB | -16.7 dB | -10.7 dB |

Observation 1: PUSCH coverage enhancement is needed for both GEO and LEO-1200.

Proposal 1: Specify some mechanism for PUSCH coverage enhancements.

    - PUCCH

Simulation assumptions for PUCCH are shown in Table 5.

Table 5 Simulation assumptions for PUCCH

| Parameter | Value |
| --- | --- |
| PUCCH format | Format 1 |
| SCS | 15 kHz |
| Number of UCI bits | Msg.4 HARQ-ACK: 1 bit 2 bits UCI: 2 bits |
| Frequency hopping | w/ inter-slot frequency hopping startingPRB: 0, secondHopPRB: 109 interval: Msg.4 HARQ-ACK: 1 slot, 2 bits UCI: 4 slots |
| BLER | DTX to ACK probability: 1%. NACK to ACK probability: 0.1%. ACK missed detection probability: 1%. |
| Number of repetitions | 8 |
| PUCCH duration | 14 OS |
| Joint channel estimation | Msg.4 HARQ-ACK: disabled 2 bits UCI: GEO: enabled, LEO: disabled |

PUCCH simulation results are shown in Figure 4, Figure 5 and Figure 6.

Figure 4 Simulation results of PUCCH (GEO – set1)

Figure 5 Simulation results of PUCCH (GEO – set2)

Figure 6 Simulation results of PUCCH (LEO-1200)

The gap between required SNR and CNR is shown in Table 6. Coverage enhancement for PUCCH format 1 is needed for GEO and LEO-1200 with parameter set 2.

Table 6 Gap between required SNR and CNR for PUCCH

| Channel | Scenario | Target | Required SNR | CNR | Performance gap |
| --- | --- | --- | --- | --- | --- |
| PUCCH format 1 (Msg.4 HARQ-ACK) | GEO – set1 | NACK-to-ACK 0.1% | -16.1 dB | -16.0 dB | 0.1 dB |
|  |  | ACK missed detection 1% | -16.6 dB | -16.0 dB | 0.6 dB |
|  | GEO – set2 | NACK-to-ACK 0.1% | -14.5 dB | -20.8 dB | -6.3 dB |
|  |  | ACK missed detection 1% | -14.8 dB | -20.8 dB | -6.0 dB |
|  | LEO-1200 – set1 | NACK-to-ACK 0.1% | -11.9 dB | -7.7 dB | 4.2 dB |
|  |  | ACK missed detection 1% | -11.9 dB | -7.7 dB | 4.2 dB |
|  | LEO-1200 – set2 | NACK-to-ACK 0.1% | -11.9 dB | -13.7 dB | -1.8 dB |
|  |  | ACK missed detection 1% | -11.9 dB | -13.7 dB | -1.8 dB |
| PUCCH format 1 (2 bits UCI) | GEO – set1 | NACK-to-ACK 0.1% | -12.9 dB | -16.0 dB | -3.1 dB |
|  |  | ACK missed detection 1% | -15.1 dB | -16.0 dB | -0.9 dB |
|  | GEO – set2 | NACK-to-ACK 0.1% | -11.5 dB | -20.8 dB | -9.3 dB |
|  |  | ACK missed detection 1% | -13.8 dB | -20.8 dB | -7.0 dB |
|  | LEO-1200 – set1 | NACK-to-ACK 0.1% | -8.3 dB | -7.7 dB | 0.6 dB |
|  |  | ACK missed detection 1% | -10.9 dB | -7.7 dB | 3.2 dB |
|  | LEO-1200 – set2 | NACK-to-ACK 0.1% | -8.3 dB | -13.7 dB | -5.4 dB |
|  |  | ACK missed detection 1% | -10.9 dB | -13.7 dB | -2.8 dB |

Observation 2: Coverage enhancement for PUCCH format 1 is needed for GEO and LEO-1200 with parameter set 2.

Proposal 2: Specify some mechanism for coverage enhancements of PUCCH format 1.

    - PRACH

Simulation assumptions for PRACH are shown in Table 7.

Table 7 Simulation assumptions for PRACH

| Parameter | Value |
| --- | --- |
| Format | Format 0, Format B4, Format 2 |
| SCS | Format 0, Format 2: 1.25 kHz Format B4: 15 kHz |
| Performance metric | 1% missed detection at 0.1% false alarm probability |

PRACH simulation results are shown in Figure 7.

Figure 7 Simulation results of PRACH

The gap between required SNR and CNR is shown in Table 8. Coverage enhancement for PRACH is needed at least for GEO and LEO-1200 with parameter set 2.

Table 8 Gap between required SNR and CNR for PRACH

| Channel | Scenario | Required SNR | CNR | Performance gap |
| --- | --- | --- | --- | --- |
| PRACH format 0 | GEO – set1 | -15.1 dB | -23.7 dB | -8.6 dB |
|  | GEO – set2 | -15.1 dB | -28.5 dB | -13.4 dB |
|  | LEO-1200 – set1 | -15.1 dB | -15.4 dB | -0.3 dB |
|  | LEO-1200 – set2 | -15.1 dB | -21.4 dB | -6.3 dB |
| PRACH format 2 | GEO – set1 | -17.6 dB | -23.7 dB | -6.1 dB |
|  | GEO – set2 | -17.8 dB | -28.5 dB | -10.7 dB |
|  | LEO-1200 – set1 | -17.6 dB | -15.4 dB | 2.2 dB |
|  | LEO-1200 – set2 | -17.6 dB | -21.4 dB | -3.8 dB |
| PRACH format B4 | GEO – set1 | -14.1 dB | -26.7 dB | -12.6 dB |
|  | GEO – set2 | -14.1 dB | -31.5 dB | -17.4 dB |
|  | LEO-1200 – set1 | -14.1 dB | -18.4 dB | -4.3 dB |
|  | LEO-1200 – set2 | -14.1 dB | -24.4 dB | -10.3 dB |

Observation 3: PRACH coverage enhancement is needed for both GEO and LEO-1200.

Proposal 3: Specify some mechanism for PRACH coverage enhancements.

    - PDCCH

Simulation assumptions for PDCCH are shown in Table 9.

Table 9 Simulation assumptions for PDCCH

| Parameter | Value |
| --- | --- |
| SCS | 15 kHz |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48 PRBs |
| Target BLER | 1% BLER |

PRACH simulation results are shown in Figure 8.

Figure 8 Simulation results of PDCCH

The gap between required SNR and CNR is shown in Table 10.

Table 10 Gap between required SNR and CNR for PDCCH

| Channel | Scenario | Required SNR | CNR | Performance gap |
| --- | --- | --- | --- | --- |
| PDCCH (w/o consideration of PFD regulation) | GEO – set1 | -10.8 dB | 2.2 dB | 13.0 dB |
|  | GEO – set2 | -10.7 dB | -10.3 dB | 0.4 dB |
|  | LEO-1200 – set1 | -10.7 dB | 2.2 dB | 12.9 dB |
|  | LEO-1200 – set2 | -10.7 dB | -3.8 dB | 6.9 dB |
| PDCCH (with consideration of PFD regulation) | GEO – set1 | -10.8 dB | -27.2 dB | -16.4 dB |
|  | GEO – set2 | -10.7 dB | -23.4 dB | -12.7 dB |
|  | LEO-1200 – set1 | -10.7 dB | -20.8 dB | -10.1 dB |
|  | LEO-1200 – set2 | -10.7 dB | -20.8 dB | -10.1 dB |

If PFD regulation are to be considered, coverage enhancements for PDCCH will be needed. However, the gap is so large that it is expected to be difficult to introduce coverage enhancements that can fill the gap. RAN1 should discuss further whether to consider PFD regulation.

Also, the gap that needs to be filled for UL channels depends largely on the parameter set. The gap in scenarios with parameter set 2 is large and may be difficult to fill. RAN1 should discuss whether to support satellites with parameter set 2.

Observation 4: PDCCH coverage enhancement is needed for both GEO and LEO-1200 only if PFD regulation are to be considered.

Proposal 4: RAN1 should discuss further whether to consider PFD regulation.

Proposal 5: RAN1 should discuss whether to support satellites with parameter set 2.

- Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: PUSCH coverage enhancement is needed for both GEO and LEO-1200.

Proposal 1: Specify some mechanism for PUSCH coverage enhancements.

Observation 2: Coverage enhancement for PUCCH format 1 is needed for GEO and LEO-1200 with parameter set 2.

Proposal 2: Specify some mechanism for coverage enhancements of PUCCH format 1.

Observation 3: PRACH coverage enhancement is needed for both GEO and LEO-1200.

Proposal 3: Specify some mechanism for PRACH coverage enhancements.

Observation 4: PDCCH coverage enhancement is needed for both GEO and LEO-1200 only if PFD regulation are to be considered.

Proposal 4: RAN1 should discuss further whether to consider PFD regulation.

Proposal 5: RAN1 should discuss whether to support satellites with parameter set 2.

- References

- RP-220842, Meeting Report for TSG RAN meeting #94e

- RP-213690, New WI: NR NTN (Non-Terrestrial Networks) enhancements

- TR 38.821 V16.1.0, Solutions for NR to support non-terrestrial networks (NTN)

- Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0

- ITU Radio Regulations Articles, Edition of 2020