3GPP TSG RAN WG1 #110	R1-2207440

Toulouse, Aug 22-26, 2022

Agenda Item:	9.9.2

Source:	Ericsson

Title:	UE support for overlapping CRS rate matching patterns

Document for:	Discussion and Decision

# Introduction

In this document we discuss remaining aspects to specify UE support for overlapping CRS rate matching patterns.

WID description for the feature is as below

- Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1]

  - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources.

- Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

# Discussion

Following agreements were made in RAN1 #109-e

Working Assumption

Introduce two new RRC parameters (i.e., list3 and list4). gNB cannot configure legacy list1/2 and new list3/4 simultaneously. If new list3/4 are configured, UE applies list3/4 regardless of whether CORESETPoolIndex= 1 is configured for any of DL BWPs of the serving cell.

Working Assumption

  - Introduce two new RRC parameters lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 in ServingCellConfig around which the UE shall do rate matching for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS.

    - The network does not configure lte-CRS-PatternList3-r18 and any of lte-CRS-ToMatchAround, lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 simultaneously. Lte-CRS-PatternList4-r18 is configured if lte-CRS-PatternList3-r18 is configured in ServingCellConfig.

    - For case when UE is not configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied

    - For case when UE is configured with two CORESET pools two different values of coresetPoolIndex in ControlResourceSet, and configured with lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18

      - If UE is configured with crs-RateMatch-PerCoresetPoolIndex, lte-CRS-PatternList3-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘0’, and lte-CRS-PatternList4-r18 is applied if the PDSCH is associated with coresetPoolIndex set to ‘1’

      - Otherwise, both lte-CRS-PatternList3-r18 and lte-CRS-PatternList4-r18 are applied.

    - The legacy configuration rule in TS 38.331 is applied in Rel-18 DSS, i.e.,

      - “The first LTE CRS pattern in lte-CRS-PatternList4 shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList3, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList31, and so on.”

Agreement

- Introduce new UE capability(ies) for support of two overlapping LTE CRS patterns in Rel-18 DSS if the UE is NOT configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet.

  - NW can configure two LTE-CRS overlapping rate matching patterns without coresetPoolIndex only if the UE indicates support of the new capability(ies).

- Clarify that the Rel-16 UE capability overlapRateMatchingEUTRA-CRS-r16 is subject to support of multiDCI-Multi-TRP-r16.

- Maximum number of LTE-CRS rate matching patterns for PDSCH supported by a UE (i.e., maxNumberPatterns-r16 and maxNumberNon-OverlapPatterns-r16) is kept unchanged.

We propose to agree to below TP to 38.214 (also discussed in RAN1 #109-e in [109-e-R18-DSS-02] email thread) to reflect the agreements/WA

#### Proposal 1

- Agree to below TP1 for 38.214 sub-clause 5.1.4.2 for consideration during drafting of Rel18 specifications

--------------------------------------- start TP1 to 38.214 sub-clause 5.1.4.2 ----------------------------------

5.1.4.2	 PDSCH resource mapping with RE level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList.

The procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_1 and the procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config-Multicast instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config.

A UE may be configured with any of the following higher layer parameters:

-	REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	For the UE for broadcast reception, REs indicated by 'RateMatchPatternLTE-CRS' in PDSCH-Config-MCCH or PDSCH-Config-MCCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe.

-	If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH:

-	if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to '1';

-	otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig.

-	If the UE is not configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet, and if the UE is configured by higher layer parameter <lte-CRS-PatternList3-r18> and <lte-CRS-PatternList4-r18> in ServingCellConfig, REs indicated by <lte-CRS-PatternList3-r18> and <lte-CRS-PatternList4-r18> are declared as not available for PDSCH.

-	If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter <lte-CRS-PatternList3-r18> and <lte-CRS-PatternList4-r18> in ServingCellConfig, the following REs are declared as not available for PDSCH:

-	if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in <lte-CRS-PatternList3-r18> if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in <lte-CRS-PatternList4-r18> if PDSCH is associated with coresetPoolIndex set to '1';

-	otherwise, REs indicated by <lte-CRS-PatternList3-r18> and <lte-CRS-PatternList4-r18>, in ServingCellConfig.

-	Within a BWP, the UE can be configured with one or more ZP CSI-RS resource set configuration(s) for aperiodic, semi-persistent and periodic time-domain behaviours (higher layer parameters aperiodic-ZP-CSI-RS-ResourceSetsToAddModList,  sp-ZP-CSI-RS-ResourceSetsToAddModList and p-ZP-CSI-RS-ResourceSet respectively comprised in PDSCH-Config), with each ZP CSI-RS resource set consisting of at most 16 ZP CSI-RS resources (higher layer parameter ZP-CSI-RS-Resource) in numerology of the BWP. The REs indicated by p-ZP-CSI-RS-ResourceSet are declared as not available for PDSCH. The REs indicated by sp-ZP-CSI-RS-ResourceSetsToAddModList and aperiodic-ZP-CSI-RS-ResourceSetsToAddModList are declared as not available for PDSCH when their triggering and activation are applied, respectively. The following parameters are configured via higher layer signaling for each ZP CSI-RS resource configuration:

-	zp-CSI-RS-ResourceId in ZP-CSI-RS-Resource determines ZP CSI-RS resource configuration identity.

-	nrofPorts in CSI-RS-ResourceMapping defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	cdm-Type in CSI-RS-ResourceMapping defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	resourceMapping in ZP-CSI-RS-Resource defines the OFDM symbol and subcarrier occupancy of the ZP CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211].

-	periodicityAndOffset in ZP-CSI-RS-Resource defines the ZP-CSI-RS periodicity and slot offset for periodic/semi-persistent ZP CSI-RS.

<unchanged text omitted>

------------------------------------ end TP1 to 38.214 sub-clause 5.1.4.2 ----------------------------

Updates are also needed to RAN2 specs (38.331 and 38.306) to reflect the RAN1 agreements/WA. We propose to send LS to RAN2 to inform them of the RAN1 agreements and request them to reflect them in the Rel18 specifications.

#### Proposal 2

- Send LS to RAN2 with details of the agreed new RRC parameters and request to reflect them in 38.331

# Conclusion

In this document, we discuss remaining aspects to specify UE support for overlapping CRS rate matching patterns and propose the following

Proposal 1

- Agree to below TP1 for 38.214 sub-clause 5.1.4.2 for consideration during drafting of Rel18 specifications

--------------------------------------- start TP1 to 38.214 sub-clause 5.1.4.2 ----------------------------------

5.1.4.2	 PDSCH resource mapping with RE level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList.

The procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_1 and the procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config-Multicast instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config.

A UE may be configured with any of the following higher layer parameters:

-	REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	For the UE for broadcast reception, REs indicated by 'RateMatchPatternLTE-CRS' in PDSCH-Config-MCCH or PDSCH-Config-MCCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe.

-	If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH:

-	if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to '1';

-	otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig.

-	If the UE is not configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet, and if the UE is configured by higher layer parameter <lte-CRS-PatternList3-r18> and <lte-CRS-PatternList4-r18> in ServingCellConfig, REs indicated by <lte-CRS-PatternList3-r18> and <lte-CRS-PatternList4-r18> are declared as not available for PDSCH.

-	If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter <lte-CRS-PatternList3-r18> and <lte-CRS-PatternList4-r18> in ServingCellConfig, the following REs are declared as not available for PDSCH:

-	if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in <lte-CRS-PatternList3-r18> if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in <lte-CRS-PatternList4-r18> if PDSCH is associated with coresetPoolIndex set to '1';

-	otherwise, REs indicated by <lte-CRS-PatternList3-r18> and <lte-CRS-PatternList4-r18>, in ServingCellConfig.

-	Within a BWP, the UE can be configured with one or more ZP CSI-RS resource set configuration(s) for aperiodic, semi-persistent and periodic time-domain behaviours (higher layer parameters aperiodic-ZP-CSI-RS-ResourceSetsToAddModList,  sp-ZP-CSI-RS-ResourceSetsToAddModList and p-ZP-CSI-RS-ResourceSet respectively comprised in PDSCH-Config), with each ZP CSI-RS resource set consisting of at most 16 ZP CSI-RS resources (higher layer parameter ZP-CSI-RS-Resource) in numerology of the BWP. The REs indicated by p-ZP-CSI-RS-ResourceSet are declared as not available for PDSCH. The REs indicated by sp-ZP-CSI-RS-ResourceSetsToAddModList and aperiodic-ZP-CSI-RS-ResourceSetsToAddModList are declared as not available for PDSCH when their triggering and activation are applied, respectively. The following parameters are configured via higher layer signaling for each ZP CSI-RS resource configuration:

-	zp-CSI-RS-ResourceId in ZP-CSI-RS-Resource determines ZP CSI-RS resource configuration identity.

-	nrofPorts in CSI-RS-ResourceMapping defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	cdm-Type in CSI-RS-ResourceMapping defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	resourceMapping in ZP-CSI-RS-Resource defines the OFDM symbol and subcarrier occupancy of the ZP CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211].

-	periodicityAndOffset in ZP-CSI-RS-Resource defines the ZP-CSI-RS periodicity and slot offset for periodic/semi-persistent ZP CSI-RS.

<unchanged text omitted>

------------------------------------ end TP1 to 38.214 sub-clause 5.1.4.2 ----------------------------

Proposal 2

- Send LS to RAN2 with details of the agreed new RRC parameters and request to reflect them in 38.331

# References

[1] RP-213575, “New WI: Enhancement of NR Dynamic spectrum sharing (DSS)”, Ericsson, RAN#94-e, December 2021.

[2] R1-2205500, “Feature lead summary #4 on UE support for two overlapping CRS rate matching patterns”, Moderator (ZTE Corporation), RAN1#109-e, May 2022.