3GPP TSG-RAN WG1 Meeting #110	R1-2207572

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	8.9

Source:	Ericsson

Title:	Clarification on the mapping to physical resources for 16-QAM in NB-IoT

Document for:	Discussion and Decision

# 1	Introduction

In TS 36.211 clause 10.1.3.6 [1], it has been identified that there is a need to add a clarification on “MrepNPUSCH” and “MrepNPDSCH” in particular because of NPUSCH with 16-QAM and NPDSCH with 16 QAM respectively.

# 2	Clarification on “MrepNPUSCH” for NPUSCH with 16-QAM

On the clarification on “MrepNPUSCH” for NPUSCH with 16-QAM we have the following observations:

In TS 36.211 clause 10.1.3.6, the following statement can be found: “Each NPUSCH codeword can be mapped to one or more than one resource units, , as given by clause 16.5.1.2 of TS 36.213 [4], each of which shall be transmitted  times”.

The statement under discussion (see “Observation 1”) cites TS 36.213 clause 16.5.1.2, however it says nothing about the number of repetitions for 16-QAM.

It may be argued that TS 36.213 clause 16.5.1.2 in turn cites 16.5.1.1 which mentions “For NPUSCH with 16QAM, NRep=1”. However, nowhere is mentioned that “NRep= ” which makes it unclear especially for NPUSCH with 16-QAM.

Based on the above observations, the clarification on “MrepNPUSCH” for NPUSCH with 16-QAM is proposed to be incorporated as follows:

Text Proposal 1

------------------------------------------------------- Text Start ------------------------------------------------------------ #### 10.1.3.6 Mapping to physical resources Each NPUSCH codeword can be mapped to one or more than one resource units, , as given by clause 16.5.1.2 of TS 36.213 [4], each of which shall be transmitted times, where is given by in clause 16.5.1.1 of TS 36.213 [4]. ------------------------------------------------------- Text Ends ------------------------------------------------------------

Clarify in TS 36.211 clause 10.1.3.6 the possible values that the variable “MrepNPUSCH” can take, through referring to “NRep” in TS 36.213 clause 16.5.1.1 (See Text Proposal 1 in section 2 of this contribution).

“Consequences if not approved”: The no established relationship between “NRep ” and “MrepNPUSCH” will prevail, causing confusion and a possible risk of misalignment. Moreover, the statement in TS 36.211 clause 10.1.3.6 touching upon “MrepNPUSCH” is written in plural, which results confusing especially for NPUSCH with 16-QAM which uses only one repetition.

# 3	Clarification on “MrepNPDSCH” for NPDSCH with 16-QAM

On the clarification on “MrepNPDSCH” for NPDSCH with 16-QAM we have the following observations:

In TS 36.211 clause 10.2.3.4, the following statement can be found: “Each NPDSCH codeword can be mapped to one or more than one subframes, , as given by clause 16.4.1.3 of TS 36.213 [4], each of which shall be transmitted MrepNPDSCH times”.

The statement under discussion (see “Observation 5”) cites TS 36.213 clause 16.4.1.3, which mentions “For NPDSCH with 16QAM, NRep=1”. However, nowhere is mentioned that “NRep= MrepNPDSCH” which makes it unclear especially for NPDSCH with 16-QAM.

Based on the above observations, the clarification on “MrepNPDSCH” for NPDSCH with 16-QAM is proposed to be incorporated as follows:

Text Proposal 2

------------------------------------------------------- Text Start ------------------------------------------------------------ #### 10.2.3.4 Mapping to resource elements Each NPDSCH codeword can be mapped to one or more than one subframes, , as given by clause 16.4.1.3 of TS 36.213 [4], each of which shall be transmitted MrepNPDSCH times, where MrepNPDSCH is given by in clause 16.4.1.3 of TS 36.213 [4]. ------------------------------------------------------- Text Ends ------------------------------------------------------------

Clarify in TS 36.211 clause 10.2.3.4 the possible values that the variable “MrepNPDSCH” can take, through referring to “NRep” in TS 36.213 clause 16.4.1.3 (See Text Proposal 2 in section 3 of this contribution).

“Consequences if not approved”: The no established relationship between “NRep ” and “MrepNPDSCH” will prevail, causing confusion and a possible risk of misalignment. Moreover, the statement in TS 36.211 clause 10.2.3.4 touching upon “MrepNPDSCH” is written in plural, which results confusing especially for NPDSCH with 16-QAM which uses only one repetition.

The need for clarifying “MrepNPDSCH/MrepNPUSCH” became more noticeable upon introducing 16-QAM which uses only one repetition, but overall, clarifying the relationship between “NRep ” and “MrepNPDSCH/MrepNPUSCH” is needed prior Rel-17. Thus, to simplify things a single Rel-17 CR can be prepared including in the “Other comments” field the following note: “The clarification in this CR also applies to previous releases”.

# 5	Conclusion

In the previous sections we made the following observations:

Observation 1	In TS 36.211 clause 10.1.3.6, the following statement can be found: “Each NPUSCH codeword can be mapped to one or more than one resource units, , as given by clause 16.5.1.2 of TS 36.213 [4], each of which shall be transmitted  times”.

Observation 2	The statement under discussion (see “Observation 1”) cites TS 36.213 clause 16.5.1.2, however it says nothing about the number of repetitions for 16-QAM.

Observation 3	It may be argued that TS 36.213 clause 16.5.1.2 in turn cites 16.5.1.1 which mentions “For NPUSCH with 16QAM, NRep=1”. However, nowhere is mentioned that “NRep= ” which makes it unclear especially for NPUSCH with 16-QAM.

Observation 4	“Consequences if not approved”: The no established relationship between “NRep ” and “MrepNPUSCH” will prevail, causing confusion and a possible risk of misalignment. Moreover, the statement in TS 36.211 clause 10.1.3.6 touching upon “MrepNPUSCH” is written in plural, which results confusing especially for NPUSCH with 16-QAM which uses only one repetition.

Observation 5	In TS 36.211 clause 10.2.3.4, the following statement can be found: “Each NPDSCH codeword can be mapped to one or more than one subframes, , as given by clause 16.4.1.3 of TS 36.213 [4], each of which shall be transmitted MrepNPDSCH times”.

Observation 6	The statement under discussion (see “Observation 5”) cites TS 36.213 clause 16.4.1.3, which mentions “For NPDSCH with 16QAM, NRep=1”. However, nowhere is mentioned that “NRep= MrepNPDSCH” which makes it unclear especially for NPDSCH with 16-QAM.

Observation 7	“Consequences if not approved”: The no established relationship between “NRep ” and “MrepNPDSCH” will prevail, causing confusion and a possible risk of misalignment. Moreover, the statement in TS 36.211 clause 10.2.3.4 touching upon “MrepNPDSCH” is written in plural, which results confusing especially for NPDSCH with 16-QAM which uses only one repetition.

Observation 8	The need for clarifying “MrepNPDSCH/MrepNPUSCH” became more noticeable upon introducing 16-QAM which uses only one repetition, but overall, clarifying the relationship between “NRep ” and “MrepNPDSCH/MrepNPUSCH” is needed prior Rel-17. Thus, to simplify things a single Rel-17 CR can be prepared including in the “Other comments” field the following note: “The clarification in this CR also applies to previous releases”.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Clarify in TS 36.211 clause 10.1.3.6 the possible values that the variable “MrepNPUSCH” can take, through referring to “NRep” in TS 36.213 clause 16.5.1.1 (See Text Proposal 1 in section 2 of this contribution).

Proposal 2	Clarify in TS 36.211 clause 10.2.3.4 the possible values that the variable “MrepNPDSCH” can take, through referring to “NRep” in TS 36.213 clause 16.4.1.3 (See Text Proposal 2 in section 3 of this contribution).

# 6	References

TS 36.211, Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation, v17.2.0.