3GPP TSG RAN WG1 #110			R1-2207579

Toulouse, France, August 22nd – 26th, 2022

Source: 	     Xiaomi

Title:	Discussion on sidelink positioning solutions

Agenda item: 	9.5.1.3

Document for:  Discussion

Introduction

In RAN#95-e meeting, a new WID on expanded and improved NR positioning has been approved [1], including the following objectives:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2]

- Scenario/requirements

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

- Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

- Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

  - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1]

  - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

  - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

  - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, the solutions of sidelink positioning are discussed.

SL positioning methods

In RAN1#109-e meeting, it has been agreed that both PC5-only-based positioning (scenario 1) and combination of Uu and PC5 based positioning (scenario 2) are considered. For PC5-only-based positioning, it can be further divided according to the number of anchor UEs participating in the SL positioning.

- One-to-one direct ranging: the solution can be utilized between two UEs no matter whether the UEs are in-coverage, partial-coverage, or out-of-coverage. No infrastructure support is necessary for this kind of solution;

- N-to-1 SL positioning: There will be multiple anchor UEs in the operation scenario. Assistance or coordination information exchange among anchor UEs or between anchor UE and the network would be needed. The positioning may need the support of infrastructure to achieve the synchronization among anchor UEs e.g. RSU deployment, etc.

From RAN1 perspective, one-to-one direct ranging can provide positioning service for more application scenarios, and can be the baseline for other positioning operation scenarios. Therefore, one-to-one direct ranging shall be focused in RAN1 study of SL positioning solutions.

Figure 1 PC-5 only based and combination of Uu and PC5 based solutions

Proposal 1: One-to-one direct ranging shall be focused in RAN1 study of SL positioning solutions.

In RAN1#109-e, multiple SL positioning methods have been proposed for further study, including RTT type solutions, SL-AoA, SL-TDOA, SL-AoD, etc. The applicability of these solutions shall be investigated.

One-way or Two-way RTT

RTT type solutions includes one-way and two-way RTT. The ranging distance between two UEs can be estimated by measuring the round-trip time between forward and backward sidelink transmissions. The relative/absolute position of the target UE can be estimated when it is jointly used with other SL positioning methods such as SL-AoA. The procedures of one-way and two-way RTT are as illustrated in figure 2.

Figure 2 illustration of one-way RTT and two-way RTT

For one-way RTT, UE B receives the SL PRS signal from UE A, and then transmits SL PRS signal back to UE A.  Each UE measures the time difference between the transmission and reception of two SL-PRS, and the propagation time is estimated as Tprop=Tround-Treply2. The anchor UE needs to send the measurement results to the target UE, so the target UE can calculate the ranging distance.

For two-way RTT, two round-trip time measurements are used and combined to get the propagation time estimation. The propagation time is estimated as:

Tprop=Tround1*Tround2-Treply1*Treply2Tround1+Tround2+Treply1+Treply2

The accuracy of one-way RTT and two-way RTT will not be impact by the timing error between UE A and UE B, as only relative time duration length is measured. However, the accuracy will be impacted by the clock error of UE A and UE B.

The estimation error of one-way RTT is as following:

Terror=Treply*Kb-Ka2

Where Ka and Kb denote that the clock on UE A and UE B run at Ka and Kb times the frequency of an ideal true clock. Assuming the clock error of UE A and UE B is ±0.1 ppm, the maximum clock error between UE A and UE B would be 0.2ppm. If  Treply is 1ms, the estimation error will be 0.1ns, causing about 0.03 meter ranging distance estimation error.

Compared with one-way RTT, the estimation error of two-way RTT can be further reduced as:

Terror=Tprop*1-Ka+Kb2

Assuming 100 meter ranging distance, and the clock error of UE A and UE B is ±0.1 ppm, the maximum estimation error would only cause 0.01 mm ranging distance estimation error.

Observation1: The impact of UE synchronization error on the accuracy of one-way and two-way RTT is marginal.

From the above analysis, it can be observed that both one-way RTT and two-way RTT are very suitable for SL positioning and ranging, especially considering that the synchronization accuracy among target UE and anchor UEs cannot be always guaranteed.

Proposal 2: One-way/two-way RTT is supported for SL positioning/ranging.

SL-AoA

For SL-AoA method, a UE can measure the azimuth/zenith angle of arrival of sidelink signals transmitted by other UE. The ranging direction between the pair of UEs can thus be estimated using the measurement. The absolute/relative position of the target UE can be estimated when SL-AoA is jointly used with other SL positioning method such as one-way/two-way RTT.

Proposal 3: SL-AoA is supported for SL positioning/ranging.

SL-AoD

To support SL-AoD method, SL beam management would be necessary. Since SL beam management is still not supported in SL communication, the study of SL-AoD method can be postponed.

SL-TDOA

For SL-TDOA method, the position of the target UE is estimated based on timing difference measurements of sidelink signals transmitted from multiple anchor UEs. However, the timing difference among anchor UEs will have great impact on the accuracy of SL-TDOA method. Different from NR Uu positioning where good synchronization can be assumed among gNBs, in NR sidelink the synchronization reference of a UE can be from GNSS, gNB, eNB, other sidelink UEs, or the UE itself, as shown in figure 3. Therefore, the condition of using SL-TDOA shall be studied, including the synchronization requirements among anchor UEs.

Figure 3 Synchronization scenarios for ranging UEs

Proposal 4: The condition of using SL-TDOA shall be studied including the synchronization among anchor UEs.

Sidelink reference signal design

In RAN1#109-e meeting, it has been agreed that the existing PRS/SRS design shall be used as the baseline for the design of new reference signal for SL positioning/ranging.

There are two options for the design of SL PRS sequence:

Option 1: ZC-based design, (e.g. similar to the sequences used for NR SRS)

Option 2: Pseudorandom sequence (e.g. Gold sequence signals, similar to DL-PRS, or SL-CSI-RS).

Our evaluation has shown that similar performance can be achieved by ZC-based SN and pseudorandom SN [2]. ZC-based SN has lower PAPR compared with pseudorandom SN, and thus is more beneficial when the requirement on ranging/positioning distance is relatively long. However, the current reference signal design of SL communications including DMRS, CSI-RS, etc. is based on pseudorandom sequence, and thus option 2 has more commonality with the current SL RS design.

Observation 2: Compared with pseudorandom SN, ZC based design has lower PARA but less commonality with the current SL RS design.

RAN1 should decide whether ranging/positioning distance is an issue for SL positioning, and accordingly decide which sequence design shall be selected. There seems no need to define two different SN options for SL PRS.

Proposal 5: RAN1 shall down-select between ZC-based and pseudorandom SN for SL PRS sequence design.

Following the design in Uu interface, fully staggered SL-PRS pattern shall be at least supported. It can be further investigated whether partial SL-PRS pattern can be additionally supported, based on the evaluation on the performance of different SL-PRS patterns.

Sidelink positioning resource allocation

In RAN1#109-e, two options have been proposed for further study with regard to the SL positioning resource allocation:

Option 1: Dedicated resource pool for SL-PRS

Option 2: Shared resource pool with sidelink communication.

Between the two options, option 1 shall be considered as the baseline for the design of SL positioning. SL positioning may have much larger bandwidth requirement compared with SL communications. It can be difficult to satisfy the requirements of both services in a same sidelink resource pool. In addition, semi-statically configured resource partition can be a simple way to avoid the interference between SL communication and SL positioning, and reduce the potential standardization complexity.

Proposal 6: Dedicated resource pool for SL PRS shall be supported.

For option 1, the SL PRS dedicated resource pool can be defined on a SL BWP or frequency layer different from the SL BWP used by SL communication resource pool. As the bandwidth requirement of SL positioning may be much larger than that of SL communication, keeping UE working in the maximum bandwidth would have negative impact on UE power consumption. If separate SL positioning BWP or frequency layer is defined, a UE can work in large bandwidth BWP or frequency layer only in limited time duration when it is performing SL positioning. The UE can fall back to smaller BWP for SL communication when it does need to transmit/receive SL PRS signal.

Figure 4 Separate SL positioning BWP or frequency layer

Setting a separate SL positioning BWP or frequency layer can also enable a higher SCS for SL PRS, which is beneficial to reduce the time domain resource utilization of a SL PRS transmission, and thus can accommodate more SL PRS transmissions in a given time duration. Therefore, we propose:

Proposal 7: A separate SL positioning BWP or frequency layer can be defined which is different from SL communication BWP.

An example of the slot structure of SL PRS resource pool can be seen as in Figure 5. A SL PRS transmission may occupy (pre)configured number of symbols in a slot. For each SL-PRS transmission, there will be an associated physical layer control information transmission in the resource pool, which can indicate or reserve the resource of the current or the future SL-PRS transmission(s). The current PSCCH/PSSCH design in SL communication can be the baseline for the design of channel to convey the information. There will be an AGC symbol before the transmission of control information. An AGC symbol is also needed if the control information and the associated SL PRS transmission occupies different bandwidth.

It is also possible to accommodate more than 1 SL PRS transmissions in a slot, as shown in the right figure of Figure 5. As the transmitter of SL PRS transmissions can be different, a guard symbol needs to be inserted between the adjacent SL PRS transmissions. The guard symbols is also necessary between the control information and the first SL PRS transmission symbol, since the associated SL PRS transmission of control information can be in a later SL PRS resource.

Alt 1: 1 SL PRS resource in a slot                          Alt 2: >1 SL PRS resource in a slot

Figure 5 Slot structure of SL PRS dedicated resource pool

Proposal 8: Physical layer sidelink control information is transmitted in the dedicated SL positioning pool to indicate or reserve the resource of current/future SL-PRS transmission(s)

- SL PRS transmission and the associated control information transmission are TDMed

Although option 1 can be the baseline for SL positioning design, option 2 can be still beneficial on resource utilization as SL PRS and communication can dynamically share the resource. For option 2, SL PRS can be transmitted together with SL PSCCH/PSSCH transmission, as shown in Figure 6. SL PRS has the same bandwidth as the PSCCH/PSSCH transmission of SL communication, and thus the guard symbol and AGC symbol are not needed. The existing resource reservation and selection mechanisms of SL communication can also be mostly reused. RAN1 can further investigate the benefit and necessity of this option if time allows.

Figure 6 Slot structure for shared resource pool

Proposal 9: For shared resource pool design, study SL PRS transmission together with SL PSCCH/PSSCH transmission in a slot.

In RAN1#109-e meeting, the following two resource allocation schemes are identified for further study:

Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution)

Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution)

For scheme 1, to support partial coverage scenario, the NW shall allocate the SL PRS resource only to the transmitter of SL PRS. Similar as SL communication, the receiver of SL PRS shall blindly detect the control information associated with the SL PRS transmission, and then receive SL PRS in the indicated time-frequency domain resource. For UE autonomous SL PRS resource selection, sensing based resource (re)selection similar as that in SL communication can be supported. A UE can keep sensing the control information associated with the SL PRS transmission, and try to select the SL PRS resource which are not yet occupied by other UEs.

Proposal 10: Sensing based SL PRS resource selection shall be supported for UE autonomous resource allocation.

It shall be further investigated whether a UE can allocate SL PRS resource for another UE. For example, when performing ranging between two UEs and one-way RTT is used, the UE initiates the ranging procedure can allocate resource for the peer UE to send back the SL PRS signal.

Proposal 11: Further study whether and how a UE allocate SL PRS resource for other UE transmission shall be supported.

Conclusion

In this contribution, the solution of sidelink positioning is discussed. Based on the discussion, the following observations are made and proposals are proposed:

Observation1: The impact of UE synchronization error on accuracy of one-way and two-way RTT is marginal.

Observation 2: Compared with pseudorandom SN, ZC based design has lower PARA but less commonality with the current SL RS design.

Proposal 1: One-to-one direct ranging shall be focused in RAN1 study of SL positioning solutions.

Proposal 2: One-way/two-way RTT is supported for SL positioning/ranging.

Proposal 3: SL-AoA is supported for SL positioning/ranging.

Proposal 4: The condition of using SL-TDOA shall be studied including the synchronization among anchor UEs.

Proposal 5: RAN1 shall down-select between ZC-based and pseudorandom SN for SL PRS sequence design.

Proposal 6: Dedicated resource pool for SL PRS shall be supported.

Proposal 7: A separate SL positioning BWP or frequency layer can be defined which is different from SL communication BWP.

Proposal 8: Physical layer sidelink control information is transmitted in the dedicated SL positioning pool to indicate or reserve the resource of current/future SL-PRS transmission(s)

- SL PRS transmission and the associated control information transmission are TDMed

Proposal 9: For shared resource pool design, study SL PRS transmission together with SL PSCCH/PSSCH transmission in a slot.

Proposal 10: Sensing based SL PRS resource selection shall be supported for UE autonomous resource allocation.

Proposal 11: Further study whether and how a UE allocate SL PRS resource for other UE transmission shall be supported.

# Reference

[1] RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, RAN#94-e, Dec. 2021.

[2] R1-2203823, Discussion on sidelink positioning solutions, xiaomi, RAN1#109-e, May, 2022