3GPP TSG RAN WG1 #110	R1-2207583

Toulouse, France, 22 – 26 August, 2022

Agenda item:		8.16.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for NR MBS

Document for:		Discussion and Decision

# Introduction

In RAN1#108-e the RAN1 UE features have been further developed [1, 2]. In this contribution we present our views on the UE features for NR MBS.

# Discussion

Detailed comments on UE features are listed below. For reference, the latest feature list from RAN1#109-e on the topics treated in this contribution is available in the Annex.

- 33-1-2:

  - Since RAN2 has decided on 33-1 as optional without capability signaling, it is not clear how the gNB could take into account the information about 33-1-2. Hence, we propose leaving the FG type  up to RAN2 for consistency.

- 33-2:

  - Add 33-1 as pre-requisite, it is unclear why a UE would support multicast but not broadcast.

- 33-2i:

  - Per band indication, as agreed already for 36-1 and 36-1a (unicast 1024QAM).

- 33-3-1:

  - Per UE

- 33-3-2:

  - Per UE

- 33-3-3:

  - Per UE

- 33-3-3a:

  - Per UE

- 33-3-3b:

  - Per UE

- 33-3-4:

  - Per UE

- 33-3-5:

  - Per UE, motivation for finer granularity is not clear. If limitations on level of support is needed, then it is preferrable to add those restrictions explicitly in the component description.

- 33-5-1:

  - Per UE

- 33-5-1e/f/g/i:

  - Per UE

- 33-5-2:

  - Per UE

- 33-6-1/1a:

  - Per UE

- 33-6-2:

  - Per UE

- 33-8-1:

  - Per UE

- 33-9:

  - Per UE

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for NR MBS, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#109-e, May 2022

- R1-2205608, ” Updated RAN1 UE features list for Rel-17 NR after RAN1 #109-e Week2 including remaining RAN1 issues”, Moderators (AT&T, NTT DOCOMO, INC.), May 2022

# Annex

(See next page)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | 1. Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. 2. Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. 3. Support of CFR configuration for broadcast. 4. Support of CORESET and common search space for broadcast. 5. Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. 6. Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. 7. Support MCCH change notification indication via DCI. 8. support of higher layer configured slot-level repetition up to 8 for MTCH |  | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  | It is up to RAN2 whether/how to introduce the capability for support of N > 1 G-RNTIs for broadcast for a UE | Up to RAN2 |
| 33. NR_MBS | 33-1-1 | DCI indicated slot-level repetition up to 16 for broadcast MTCH | Support up to 16 times dynamic slot-level repetition for broadcast MTCH. | 33-1 | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  |  | Up to RAN2 |
| 33. NR_MBS | 33-1-2 | FDM-ed unicast PDSCH and group-common PDSCH for broadcast | 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot. | 33-1 | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast for PCell | 1. Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. 2. Support of CFR configuration for multicast. 3. Support of CORESET and common search space configuration for multicast. 4. Support of DCI format 1_0 with CRC scrambled with G-RNTI for multicast. 5. Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. 6. Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast | FFS | Yes |  |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast | 33-2 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast | Support of PTP retransmission for multicast [on the cell same as multicast initial transmission] | 33-2a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2e | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast | [33-2] | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FG 33-2e is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |
| 33. NR_MBS | 33-2f | Dynamic multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast | 33-2 | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2g | MIMO layers for multicast PDSCH | Supported maximal number of MIMO layers for multicast PDSCH | 33-2 | Yes |  | UE supports 1 MIMO layer only for multicast PDSCH | Per FSPC | N/A | N/A |  | Candidate values: {2,4,8} Note: If UE supports up to 8 layers, the UE supports TB2 | Optional with capability signalling |
| 33. NR_MBS | 33-2h | Dynamic scheduling for multicast for SCell | Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for SCell. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2i | Supported maximal modulation order for multicast PDSCH | 1. For FR1, up to 1024QAM is supported, candidate values {256QAM, 1024QAM} 2. For FR2, up to 256QAM is supported, candidate values {64QAM, 256QAM} FFS additional components | FFS | Yes |  | The UE supports the same modulation order as unicast | FFS | FFS | FFS |  | Note: A UE shall support the corresponding mandatory maximum modulation for unicast. | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | 1. Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast. | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH for multicast | 1. Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in RRC CONNECTED mode in a slot. | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | 1. Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. 2. Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC 3. Support TDM among N (N>1) group-common PDSCHs in a slot per CC 4. Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC 5. The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | [33-1, 33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3a | FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | Support of FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast FFS value of X G-RNTIs | [TBD] | Yes |  |  | [Per UE] | [No] | [No] |  | Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. | Optional with capability signalling |
| 33. NR_MBS | 33-3-3b | Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | Support of Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast FFS value of X G-RNTIs | [TBD] | Yes |  |  | [Per UE] | [No] | [No] |  | Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Mode 1 for type1 codebook generation | Supports type1-Codebook-Generation-Mode configured as mode 1 | [TBD], 33-3-3b | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-5 | Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type | Support of multiplexing HARQ-ACK for unicast and multicast with the same priority and different HARQ-ACK codebook types in the same PUCCH slot | [33-2b] | Yes |  |  | [Per FSPC] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming | 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits [2. Support of shared PUCCH resource configurations with unicast] | 33-2a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4a | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission | 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) Multiple TB with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate PUCCH resource configurations from unicast | 33-4 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signalling | [33-4] | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | 1. Support one SPS group-common PDSCH configuration for multicast 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH | 33-2 | Yes |  |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH | 33-5-1 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1b | DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling | 33-5-1a | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d | PTP retransmission for SPS group-common PDSCH for multicast | Support of PTP retransmission for SPS multicast [on the cell same as multicast initial transmission] | 33-5-1a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1e | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast. | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1g | DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling | 33-5-1f | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast | 33-5-1 | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FGs 33-5-1h is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |
| 33. NR_MBS | 33-5-1i | Multicast SPS scheduling with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling FFS whether to include retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI | [33-5-1] | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | 1. Support up to 8 SPS group-common PDSCH configuration per CFR for multicast 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 [per cell], and activated SPS group-common PDSCH configurations is no larger than M. | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate value set for M is {1, 2, …, 8} | Optional with capability signalling |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. 2. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1a | DL priority configuration for SPS multicast | Support of priority configured for multicast HARQ-ACK feedback of SPS multicast | 33-6-1 | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-2a | Yes |  |  | [Per band or per FSPC] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | [33-5-1] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-10 | Support group-common PDSCH RE-level rate matching for multicast | 1) Supported SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns 2) Supported P ZP-CSI-RS for group-common PDSCH RE-mapping patterns 3) Support p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast same as or different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config Note 1: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 | 2-33a, 33-2 | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FG 33-10 is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |