3GPP TSG RAN WG1 Meeting #110															R1-2207667

Toulouse, France, August 22 - 26, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on dormancy indication on SCell |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DSS-Core |  |  |  |  |  |  |  | Date: |  | 2022-08-12 |
| Category: | F |  |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - SCell dormancy is used for power saving of UE. SCell dormancy indication can be only transmitted on PCell, including: - DCI format 2_6 outside the DRX active time - DCI format 0_1/1_1 within the DRX active time - The UE capabilities for supporting SCell dormancy indication in DCI format 2_6 and in DCI format 0_1/1_1 are independently reported by the UE. The UE may choose to only support SCell dormancy indication in DCI format 0_1/1_1 for power saving. - For R17 DSS Type A UE not indicating additional FG 34-1a, DCI formats 0_1,1_1 monitoring is not supported on PCell, therefore the benefit of UE power saving is unnecessarily restricted. |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | SCell dormancy indication can be transmitted on the SCell configured with cross-carrier scheduling to PCell. |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For a R17 DSS Type A UE not indicating additional FG 34-1a and only supporting SCell dormancy indication in DCI format 0_1/1_1, SCell dormancy indication can not be indicated for power saving. |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3 |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |
| Other specs |  |  | X |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |

## 10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

< Unchanged parts are omitted >

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if

-	the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if

-	a one-shot HARQ-ACK request field is not present or has a '0' value, and if

-	the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, or detects a DCI format 1_1 on the secondary cell that includes a carrier indicator field with value corresponding to a PCell and if

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1

< Unchanged parts are omitted >