3GPP TSG-RAN WG1 Meeting #110						R1-2207675

Toulouse, France, August 22nd – 26th 2022

Agenda Item:	8.10

Source:	Ericsson

Title:	Discussion on DL Tx power control

Document for:	Discussion, Decision

# Introduction

This contribution concerns the DL Tx power control and an inconsistency between the specification and the agreements that RAN1 has made in the matter. We provide an associated draft CR in [1].

# Discussion

RAN1 made the following agreements for DL Tx power control in RAN1 #107-e [2]:

Agreement The provided DL TX power adjustment is applied only to PDSCH and its associated DMRS and PTRS. Agreement The indicated desired/provided DL TX power adjustment is in terms of a relative offset to the PDSCH a CSI-RS TX power that is RRC configured.

This is reflected in the following specification text [3]:

For a serving cell of an IAB-MT, the IAB-MT can be provided a set of TCI states or a set of RS resource indexes corresponding to a SS/PBCH block or to a CSI-RS resource index for a slot where a PDSCH EPRE adjustment is indicated by DL Tx Power Adjustment MAC CE as described in [11, TS 38.321]. The PDSCH EPRE can be derived from a downlink CSI-RS EPRE as described in [6, TS 38.214] and a PDSCH power offset provided by powerControlOffsetIAB as described in [11, TS 38.321]. For a downlink DM-RS and/or PT-RS associated with a PDSCH, the IAB-MT may assume that the ratio of PDSCH EPRE to DM-RS EPRE, and/or PT-RS EPRE to PDSCH EPRE, is obtained as for a "UE" in [6, TS 38.214]. If no TCI state or RS resource index is provided to the IAB-MT, the IAB-MT may assume that a same PDSCH EPRE adjustment applies to all TCI states or RS resource indexes configured for the IAB-MT. A PDSCH EPRE adjustment provided by DL Tx Power Adjustment MAC CE may be associated with - multiplexing mode of the IAB-node, if provided, and/or - when simultaneous reception by the IAB-MT and transmission/reception by an IAB-DU cell occur in non-overlapping frequency resources, if provided, or when simultaneous reception by the IAB-MT and transmission/reception by an IAB-DU cell occur in overlapping frequency resources, if provided, and/or - slots indicated by slot index, if provided.

TS 38.321 [4] itself does not state to what offset the field DL Tx Power adjustment in the MAC CE DL Tx Power Adjustment refers to, see below.

- DL Tx Power adjustment: For the case of the Desired DL TX Power Adjustment MAC CE, this field contains desired DL TX power adjustment send by an IAB-MT to assist with the parent node's DL TX power allocation. For the case of the DL TX Power Adjustment MAC CE, this fields contains adjustment to a node's DL TX power sent by its parent node. The value of the power adjustment is in both cases contained in the 5 right-most bits of the field, while the remaining 3 bits are set to zero. The length of the field is 8 bits;

At the same time, a UE may be configured with up to 192 CSI-RS resources per cell and each one can have different CSI-RS TX EPRE power settings [5].

NZP-CSI-RS-Resource information element -- ASN1START -- TAG-NZP-CSI-RS-RESOURCE-START NZP-CSI-RS-Resource ::= SEQUENCE { nzp-CSI-RS-ResourceId NZP-CSI-RS-ResourceId, resourceMapping CSI-RS-ResourceMapping, powerControlOffset INTEGER (-8..15), powerControlOffsetSS ENUMERATED{db-3, db0, db3, db6} OPTIONAL, -- Need R scramblingID ScramblingId, periodicityAndOffset CSI-ResourcePeriodicityAndOffset OPTIONAL, -- Cond PeriodicOrSemiPersistent qcl-InfoPeriodicCSI-RS TCI-StateId OPTIONAL, -- Cond Periodic ... } -- TAG-NZP-CSI-RS-RESOURCE-STOP -- ASN1STOP NZP-CSI-RS-Resource field descriptions. periodicityAndOffset Periodicity and slot offset sl1 corresponds to a periodicity of 1 slot, sl2 to a periodicity of two slots, and so on. The corresponding offset is also given in number of slots (see TS 38.214 [19], clause 5.2.2.3.1). Network always configures the UE with a value for this field for periodic and semi-persistent NZP-CSI-RS-Resource (as indicated in CSI-ResourceConfig).. powerControlOffset Power offset of PDSCH RE to NZP CSI-RS RE. Value in dB (see TS 38.214 [19], clauses 5.2.2.3.1 and 4.1).. powerControlOffsetSS Power offset of NZP CSI-RS RE to SSS RE. Value in dB (see TS 38.214 [19], clause 5.2.2.3.1).. qcl-InfoPeriodicCSI-RS For a target periodic CSI-RS, contains a reference to one TCI-State in TCI-States for providing the QCL source and QCL type. For periodic CSI-RS, the source can be SSB or another periodic-CSI-RS. Refers to the TCI-State or DLorJoint-TCIState which has this value for tci-StateId and is defined in tci-StatesToAddModList or in dl-orJoint-TCI-State-ToAddModList in the PDSCH-Config included in the BWP-Downlink corresponding to the serving cell and to the DL BWP to which the resource belongs to (see TS 38.214 [19], clause 5.2.2.3.1).. resourceMapping OFDM symbol location(s) in a slot and subcarrier occupancy in a PRB of the CSI-RS resource.. scramblingID Scrambling ID (see TS 38.214 [19], clause 5.2.2.3.1)..

In our understanding, each CSI-RS EPRE is derived from the fixed reference SSB EPRE, using powerControlOffsetSS. For a certain CSI-RS resource (determined by higher layer configuration), the CSI-RS EPRE and powerControlOffset are used to derive the PDSCH EPRE. Therefore, “a CSI-RS” resource in the above RAN1 agreement is ambiguous, considering there can be many CSI-RS resources configured. On the other side, any CSI-RS resource configuration (i.e., the combination of CSI-RS EPRE and powerControlOffset) will result in the same PDSCH EPRE for a cell. RAN1 has agreed that DL power control is only applied to PDSCH and its associated DMRS and PTRS. We believe the RAN1’s intention is that MAC CE should indicate ONE relative power offset to the non-offset PDSCH EPRE, regardless CSI-RS TX EPRE, as described in the current TS 38.213 [3].

In order to avoid implementing new signaling, i.e., specifying a new CSI-RS index field in TS 38.321 [4], we propose RAN1 to amend the RAN1#107-e agreement as:

The following RAN1#107-e agreement is amended as: The indicated desired/provided DL Tx power adjustment is in terms of a relative offset to the non-offset PDSCH Tx power, as derived from a CSI-RS TX power that is RRC configured.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The following RAN1#107-e agreement is amended as: The indicated desired/provided DL Tx power adjustment is in terms of a relative offset to the non-offset PDSCH Tx power, as derived from a CSI-RS TX power that is RRC configured.

# References

R1-2207674, “Draft CR on DL Tx power control,” Ericsson, RAN1 #110, August 2022.

“Chairman’s Notes”, 3GPP TSG RAN WG1 Meeting #107-e, November 2021.

TS 38.213, “NR; Physical layer procedures for control (Release 17)”, 3GPP, V17.2.0, June 2022.

TS 38.321, “NR; Medium Access Control (MAC) protocol specification (Release 17)”, 3GPP, V17.1.0, June 2022.

TS 38.331, “NR; Radio Resource Control (RRC) protocol specification (Release 17)”, 3GPP, V17.1.0, June 2022.