3GPP TSG RAN WG1 #110			R1- 2207695

Toulouse, France, August 22nd – 26th, 2022

Agenda item:	9.10.1

Source: 	Spreadtrum Communications

Title: 	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:	Discussion and decision

Introduction

In RAN#94 meeting, a new Work Item [1] was agreed to support multi-cell PUSCH/PDSCH scheduling with a single DCI. There were remarkable agreements and conclusions [2] on various topics during RAN1#109e.

This contribution provides further considerations on multi-cell PUSCH/PDSCH scheduling with a single DCI.

Discussion

Scenarios and basic framework

Cell or PUCCH group

Regarding multi-cell PUSCH scheduled by DCI 0_X, the PUCCH group should also be applied, because the UCI multiplexing is done within the PUCCH group.

Agreement in RAN1#109e - All the co-scheduled cells by a DCI format 1_X and the scheduling cell are included in the same PUCCH group. - FFS: All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same [cell or PUCCH group].

For example, if DAI in DCI 0_X indicates to do HARQ-ACK multiplexing, there would be some problems to decide the HARQ-ACK bits to be multiplexed on PUSCH in the co-scheduled cell. Such as if there are two PUCCH groups within a cell group, there should be two T-DAI counters in the DCI. Especially, those two DAI fields would have more complex issues related with HARQ-ACK multiplexing and decision. Thus we recommend to limit multi-cell PUSCH scheduled by one DCI belonging to the same PUCCH group for easy HARQ-ACK on PUSCH multiplexing.

- All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same PUCCH group.

DCI format 0-X/1-X on sSCell

According to whether a DCI format 0-X/1-X can be transmitted on a SCell if the DCI format 0-X/1-X schedules PUSCH/PDSCH on PCell, the point is for whether this WID can include the enhancement of Rel-17 DSS a SCell cross carrier scheduling PCell.

Agreement in RAN1#109e - DCI format 0-X/1-X can be transmitted on PCell. - DCI format 0-X/1-X can be transmitted on a SCell at least when the DCI format 0-X/1-X does not schedule PUSCH/PDSCH on PCell. - FFS whether a DCI format 0-X/1-X can be transmitted on an SCell if the DCI format 0-X/1-X schedules PUSCH/PDSCH on PCell.

From our point of view, the enhancement of PCell scheduling would be better to reuse the framework of Rel-17 DSS, including PDCCH BD/CCE counting, DCI size budget, DCI field depend on PCell parameters, CIF function, etc. Thus even if a SCell which is configured to CCS PCell is allowed to monitoring DCI format 0-X/1-X, all the design related with the issues above are different with Rel-17 DSS. In conclusion, we are fine to support a DCI format 0-X/1-X can be transmitted on a SCell if the DCI format 0-X/1-X schedules PUSCH/PDSCH on PCell. And the framework still reuse multi-cell scheduling, but not do additional design for this case. So, we propose if multi-cell scheduling can be directly use for this case, it is fine to support. Otherwise, it cannot be supported.

- If multi-cell scheduling can be directly use for sSCell, it is fine to support a DCI format 0-X/1-X can be transmitted on the sSCell. Otherwise, it cannot be supported.

Same or different SCS and carrier type

We support to prioritize the cases 1-1 and 1-2 on SCS, cases 2-1 and 2-2 on carrier type, to simplify the design of this work item.

Proposal 1-7rev1: - At least below cases 1-1 and 1-2 on SCS are prioritized: - Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell. - Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell. - Case 1-3: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and different SCS is used among the co-scheduled cells including the scheduling cell. - Case 1-4: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and different SCS is used among the co-scheduled cells. - At least below cases 2-1 and 2-2 on carrier type are prioritized: - Case 2-1: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells including the scheduling cell and same carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among all the co-scheduled cells including the scheduling cell. - Case 2-2: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells not including the scheduling cell and same carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among all the co-scheduled cells which may be same or different carrier type to the scheduling cell. - Case 2-3: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells including the scheduling cell and different carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among the co-scheduled cells including the scheduling cell. - Case 2-4: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells not including the scheduling cell and different carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among the co-scheduled cells - Note: for Case 2-2, 2-3 and 2-4, using an unlicensed cell for scheduling a set of co-scheduled cells including licensed cell is not prioritized.

According to the difficulties of case 1-3/1-4,

- First, the processing timeline as mentioned in the previous meeting would be difficult to handle. Because the processing timeline of different numerologies can be widely varying. It might lead to complex scheduling restrictions such as the PUSCH scheduling and HARQ-ACK feedback.

- Second, BD/CCE limits are calculated per scheduling cell numerology in the previous release. There is a possibility that BD/CCE calculation still need to consider the numerology of scheduled cell based on the alternatives provided in the last meeting. In order to reduce the complexity of BD/CCE design for multi-cell scheduling, same SCS is a good choice.

According to the difficulties of case 2-3/2-4,

- The different carrier type may use significantly different configurations. Such as the cells in licensed or unlicensed bands, the frequency domain resource allocation might have totally different FDRA type, such as licensed cell can have Type-0 or Type-1 resource allocation. However, cell in unlicensed bands may be configured as interlaced structure. Those different configurations will make the scheduling less efficiency and increase the DCI size.

- The different carrier type may have entirely different scheduling strategy. Such as the cells in FR1 do not care QCL-D very much, nevertheless the QCL-D and beams are the basic decision in FR2, furthermore, FR2-2 use a lot of multi-PDSCH/PUSCH scheduling instead of single slot due to the short slot length. Therefore, it is easier to limit the same carrier type scheduling to make the scheduling decision more reasonable for the certain carrier type and the band/frequency range.

- Support Proposal 1-7rev1

- At least below cases 1-1 and 1-2 on SCS are prioritized:

- Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell.

- Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.

- Case 1-3: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and different SCS is used among the co-scheduled cells including the scheduling cell.

- Case 1-4: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and different SCS is used among the co-scheduled cells.

- At least below cases 2-1 and 2-2 on carrier type are prioritized:

- Case 2-1: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells including the scheduling cell and same carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among all the co-scheduled cells including the scheduling cell.

- Case 2-2: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells not including the scheduling cell and same carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among all the co-scheduled cells which may be same or different carrier type to the scheduling cell.

- Case 2-3: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells including the scheduling cell and different carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among the co-scheduled cells including the scheduling cell.

- Case 2-4: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells not including the scheduling cell and different carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among the co-scheduled cells

- Note: for Case 2-2, 2-3 and 2-4, using an unlicensed cell for scheduling a set of co-scheduled cells including licensed cell is not prioritized.

Same or different priority scheduled by one DCI

One open issue is a DCI format 0-X/1-X on a scheduling cell can schedules multiple PDSCH/PUSCH on multiple cells can only be same priority (low priority HARQ-ACK feedbacks/PUSCHs, high priority HARQ-ACK feedbacks/PUSCHs), or can be different priorities.

For different priority scheduling, such as one DCI scheduling both high priority PUSCH and low priority PUSCH on different cells, it will lead to complex configuration and DCI design.

- First, HP and LP have different reliable requirement for PDCCH, it does make sense to use the single DCI always targeting the high priority BLER even for low priority scheduling.

- Second, this HP and LP scheduling have different delay requirement. For example, HP HARQ-ACK and LP HARQ-ACK may be configured with different sub slot length. The PDSCH to HARQ-ACK feedback timing will be very complex and with many issue problems to be handled, like which PDSCH is used as the reference time for HARQ-ACK, the HP or LP or the latest ending symbol.

So we prefer to limited to same priority scheduling for this feature, at least in Rel-18.

- Only support same priority of PUSCHs on multi-cells scheduled by one DCI0_X, and same priority of HARQ-ACK of PDSCHs on multi-cells scheduled by one DCI 1_X in Rel-18.

DCI format design

Maximum number of cells that can be scheduled simultaneously

In Rel-17 DSS, up to two cells PDSCH were discussed, and found that it was too limited for the one DCI scheduling multiple cells. From this point, more number of cells such as more than 2 can be considered to be supported in this WI, for better feasibility and usable of this feature. Another understanding is when the supported cell number by one DCI is increased, the DCI size would be rapidly multiplied, which requires larger aggregation level. So the simultaneous scheduling cell number should not be too big to beyond the maximum DCI sizes which fit to AL16.

Agreement in RAN1#109e - For a UE, the maximum number of cells scheduled by a DCI format 0_X can be same or different to the maximum number of cells scheduled by a DCI format 1_X. Agreement in RAN1#109e - One value for the maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is selected from {3, 4, 8}. - For a UE, the maximum number of co-scheduled cells by a DCI format 0_X can be smaller than or equal to the maximum number supported in Rel-18. Agreement in RAN1#109e - One value for the maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is selected from {3, 4, 8}. - For a UE, the maximum number of co-scheduled cells by a DCI format 1_X can be smaller than or equal to the maximum number supported in Rel-18.

4 is a trade-off between 8 and 3. 3 is too small step comparing Rel-17 DSS study item. Regarding 8, it is too hard to satisfy the maximum 140 bits of DCI size excluding CRC. Because it means 17 bits per cell in DCI, which requires the most part of DCI should be shared by the cell. Until now, only five fields were intended to be agreed to be Type-1 fields. The other fields are Type-2 or 3, so they need more bits.

- The value for the maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4, and the value for the maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

Scheduling possibilities of multi-cell scheduling and possible single-cell scheduling

In the previous releases, only single-cell scheduling is supported, including self-carrier scheduling and cross-carrier scheduling. When self-carrier scheduling is used, the PDCCH and PDSCH are on the same cell. While cross-carrier scheduling is used for one serving cell, the PDCCH is on another serving cell and PDSCH is on this cell. Before Rel-17 DSS, either self-carrier scheduling or cross-carrier scheduling is configured for one cell. In Rel-17 DSS, P(S)Cell can be configured to both of self-carrier scheduling and cross-carrier scheduling.

For multi-cell scheduling, there was a proposal trying to address the scheduling possibilities of multi-cell scheduling and possible single-cell scheduling.

(merged)Proposal 2-4 & 2-5rev2: - At least following is supported: - For each scheduled cell, a UE monitors DCI format 0_X/1_X on at most one scheduling cell. - (working assumption)For a cell within a set of configured cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring DCI format 0_X/1_X and legacy DCI format(s) from a same scheduling cell. - FFS: whether DCI format 0_X/1_X and legacy DCI format(s) are monitored simultaneously - FFS: for which cell within the set of configured cells this is supported - FFS whether to support monitoring DCI format 0_X/1_X and legacy DCI format(s) from different scheduling cells for a cell within a set of configured cells which can be co-scheduled by a DCI format 0_X/1_X.

Number of multi-cell scheduling cell for a scheduled cell

At most one multi-cell scheduling cell for a scheduled cell was proposed during the last meeting. We support this proposal.

- First, it is a good balance between scheduling flexibility and complexity. One multi-cell scheduling cell for a scheduled cell can provide this multi-cell scheduling feature for any single cell. Thus it has ready support to reduce the control signalling overhead for all the configured cells. In addition, more than one scheduled cell for a scheduling cell would lead to the problem of the out of order, the overlapping of transmission or reception .etc.

- Second, it can simplify the BD/CCE handling. If multiple scheduling cell can do multi-cell scheduling for one scheduled cell, it would be very difficult to define its BD and CCE on every multi-cell scheduling configuration.

Multi-cell and single-cell schedule from the same/different scheduling cell(s) for a scheduled cell

Multi-cell and single-cell schedule from the same scheduling cell for a scheduled cell is a most reasonable configuration from our point of view. On the other side, multi-cell and single-cell schedule from different scheduling cells for a scheduled cell would lead to much more problems and increased unnecessary complexity.

- First, it continues the original policies that there is only one scheduling cell for a scheduled cell, and it only extended the scheduling cell to both multi-cell and single-cell schedule.

- Furthermore, it can simplify the BD/CCE handling. If different scheduling cells for multi-cell and single-cell scheduling for one scheduled cell, it would be very difficult to define its BD and CCE calculation which part should be based on each scheduling cell.

Figure 1: An example for multi-cell and single-cell schedule from the same scheduling cell(s) for a scheduled cell

The Table 1 gives all possible implementation of multi-cell scheduling. If the column and row with the same content setting, it means one cell only use one type of scheduling, self-carrier scheduling or cross-carrier scheduling. If they are with different content, it means this cell can use different type of scheduling combination, such as self-carrier scheduling and cross-carrier scheduling introduced in Rel-17 DSS.

Table 1: Relationship with the current self-carrier scheduling and cross-carrier scheduling

| Combinations | Self-carrier scheduling | Cross-carrier scheduling | Multi-cell scheduling |
| --- | --- | --- | --- |
| Self-carrier scheduling | Rel-15 |  |  |
| Cross-carrier scheduling | Rel-17 DSS (PSCell) | Rel-15 |  |
| Multi-cell scheduling | Type 1, One Cell is configured as self-carrier scheduling, meanwhile can be multi-cell scheduling by itself. | Type 2, One Cell is configured as cross-carrier scheduling, meanwhile can be multi-cell scheduling by the same cell. | Type 0 One Cell’s scheduling only from one cell’s multi-cell scheduling, not configured as self-carrier nor cross-carrier scheduling |

Thus, there can be three type of multi-carrier scheduling:

- Type 0 one cell’s scheduling only from multi-cell scheduling, not configured as self-carrier nor cross-carrier scheduling

- Type 1, one cell is configured as self-carrier scheduling, meanwhile can be multi-cell scheduling by itself.

- Type 2, one cell is configured as cross-carrier scheduling, meanwhile can be multi-cell scheduling by the same scheduled cell.

Figure 2: An example for Multi-cell scheduling Type 0

However, it should clarify whether allow multi-cell only scheduling for a scheduled cell, such as Type 0 multi-cell scheduling (CC2 and CC3 in Figure 1), and the cell does not configure the single-cell scheduling cell/search space/DCI format.

- It should clarify whether or not allow multi-cell scheduling only for a scheduled cell.

- Support (merged)Proposal 2-4 & 2-5rev2 with further steps (confirm the WA of the second bullet):

- For each scheduled cell, a UE monitors DCI format 0_X/1_X on at most one scheduling cell.

- For a cell within a set of configured cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring DCI format 0_X/1_X and legacy DCI format(s) from a same scheduling cell.

  - FFS: whether DCI format 0_X/1_X and legacy DCI format(s)  are monitored simultaneously

  - FFS: for which cell within the set of configured cells this is supported

- Not support monitoring DCI format 0_X/1_X and legacy DCI format(s) from different scheduling cells for a cell within a set of configured cells which can be co-scheduled by a DCI format 0_X/1_X.

New or existing DCI format for multi-cell scheduling

For new or existing DCI format, we support introduce new DCI format for multi-cell scheduling, rather than reuse the legacy DCI formats.

- Firstly, new DCI format can give a clear DCI design for multi-cell scheduling. There is no interaction problems with single-cell scheduling DCI fields.

- Secondly, if legacy DCI formats are extended to support multi-cell scheduling, it requires more effort to handle the different DCI format sizes in different search space. Such as SS1 is a single-cell scheduling search space, while SS2 is a multi-cell scheduling search space. The DCI formats associated with SS1 and SS2 should be same, i.e. DCI format 0_1/1_1. However, it is clearly that DCI format 0_1/1_1 in SS1 and SS2 are totally different, either DCI size or DCI fields design etc.

So we propose to confirm the following WA.

Agreement in RAN1#109e - (Working assumption) DCI format 0_X/1_X is a new DCI format for multi-cell scheduling - DCI format 0_X can be used for single cell PUSCH scheduling. - DCI format 1_X can be used for single cell PDSCH scheduling. - FFS: UE monitors one of or both multi-cell scheduling DCI and legacy single cell scheduling DCI for a scheduled cell.

- DCI format 0_X/1_X is a new DCI format for multi-cell scheduling

Single or two-stage DCI

According to the issue of single or two-stage DCI, it would be good to down select single-stage DCI in this meeting. It is the fundament for the DCI format design. It can save a lot of time to give a clear direction for the main structure of multi-cell DCI. Furthermore, there are a lot of DCI fields need to settle down, and it is easier to make decisions for the DCI field types. So we have the following proposal that only single-stage DCI is supported in Rel-18.

- Single-stage DCI format is supported for multi-cell PDSCH or PUSCH scheduling.

Indication of scheduled cells

Another important issue is how to indicate the co-scheduled cells. The agreement in RAN1#109 provided three Options.

Agreement in RAN1#109e For multi-cell scheduling, the co-scheduled cells are indicated by DCI format 0_X/1_X. At least the following options are considered: - Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells. - The table is configured by RRC signaling. - FFS: Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling. - Option 2: An indicator in the DCI is a bitmap corresponding to a set of configured cells that can be scheduled by the DCI 0_X/1_X - FFS: Separate sets of configured cells for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling. - Option 3: using existing field (e.g., CIF, FDRA) to indicate whether one or more cells are scheduled or not - Other options are not precluded. - Note: It does not preclude other DCI information fields (e.g., BWP) to be jointly indicated by the indicator of the co-scheduled cells.

For Indicator in Option 1, one example was given in Table 2. Clearly, when scheduling the different co-scheduled cell combinations, the DCI payload can be different.

Table 2: Option 1 of scheduled cells indication

| Option 1: Indicator | 1st Co-scheduled cell | 2nd Co-scheduled cell | 3rd Co-scheduled cell | 4th Co-scheduled cell |
| --- | --- | --- | --- | --- |
| 000 | CC0 | CC3 |  |  |
| 001 | CC1 | CC3 | CC3 |  |
| 010 | CC2 | CC3 |  |  |
| 011 | CC0 | CC1 | CC3 |  |
| 100 | CC0 | CC1 | CC2 |  |
| 101 | CC1 | CC2 |  |  |
| 110 | CC0 | CC1 | CC2 | CC3 |
| 111 | CC0 | CC2 | CC3 |  |

For the bitmap in Option 2, if bit field length depend on the maximum number of co-scheduled cells that the DCI can schedule. Every bit in the bitmap in DCI to indicate whether this cell is scheduled or not. But the disadvantage is the payload is high, because the bitmap length is fixed.

For Option 3, it reuse the legacy CIF to indicate whether one or more cells are scheduled or not. From our understanding, CIF in option 3 is similar as indicator in option 1. An example is shown in Table 3.

Table 3: Option 1 of scheduled cells indication

| CIF | 1st Co-scheduled cell | 2nd Co-scheduled cell | 3rd Co-scheduled cell | 4th Co-scheduled cell |
| --- | --- | --- | --- | --- |
| 000 | CC0 | CC3 |  |  |
| 001 | CC1 | CC3 | CC3 |  |
| 010 | CC2 | CC3 |  |  |
| 011 | CC0 | CC1 | CC3 |  |
| 100 | CC0 | CC1 | CC2 |  |
| 101 | CC1 | CC2 |  |  |
| 110 | CC0 | CC1 | CC2 | CC3 |
| 111 | CC0 | CC2 | CC3 |  |

We propose to further study CIF in option 3 and option 1. And from the co-scheduled cell indication point of view, CIF in option 3 and option 1 are identical. Since CIF in DCI 0_1/1_1/0_2/1_2 is used to indicate scheduled cell, it is natural to use it. Nevertheless, CIF has some relations with search space design. It would be good to discuss CIF as the indication together with the search space issues.

- Support Option 1 and CIF in Option 3 as the indication of co-scheduled cell combination

  - Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells

  - Option 3: using existing field CIF to indicate whether one or more cells are scheduled or not

Another important issue is relationship of DCI size and co-scheduled cell combinations. The size of the legacy DCI formats is fixed, depending on the configuration of the scheduled cell. So the size of multi-cell scheduling DCI format on one scheduling cell can be three directions:

- Alt1: there is only one size of DCI 0_X/1_X on one scheduling cell, which means when the DCI schedule different co-scheduled cell combinations, the DCI size should be aligned in the DCI generation procedure.

- Alt2: Different co-scheduled cell combinations can have the different DCI size that they do not need to add padding to align the DCI size.

- Alt 3: some of co-scheduled cell combinations (marked as one combination group) can have the same size, while there can be other co-scheduled cell combinations have a different DCI size. So there are combination groups, each combination group can have a unified DCI size, and the combinations in one group have to do DCI size alignment, while the combinations in different group do not need to do.

We prefer to use Alt1 as the starting point, to simplify the DCI size discussion.

- Only one size of DCI 0_X/1_X on one scheduling cell can be monitored.

Search space of DCI 0_X/1_X

For the search space associated with DCI 0_X/1_X, there are three issues need to be considered.

Association of search space among the scheduling and co-scheduled cells

Regarding cross carrier scheduling, the search space sets are also configured on the scheduled cell, and associated with the search space on the scheduling cell with the same search space ID, but only with separate PDCCH candidates and AL configurations. Thus, this issue is also valid for multi-cell scheduling, including which cell should be configured with search space of DCI 0_X/1_X, scheduling cell only (Alt 1) or scheduling cell and co-scheduled cells (Alt 2).

- Alt 1: configure the search space of DCI 0_X/1_X only on scheduling cell

- Alt 2: configure the search space of DCI 0_X/1_X on scheduling cell and co-scheduled cells

Figure 3: Alt 1, configure the search space of DCI 0_X/1_X only on scheduling cell

- For Alt 2, if the search space is configured on the co-scheduled cell, there can be three sub-alternatives:

- Alt 2-1: configure the search space of DCI 0_X/1_X on each co-scheduled cell, and associated with the search space on the scheduling cell with the same search space ID

- Alt 2-2: only configure the search space of DCI 0_X/1_X on one specified co-scheduled cell, and associated with the search space on the scheduling cell with the same search space ID

- Alt 2-3: configure the search space of DCI 0_X/1_X on a subset of co-scheduled cells, and associated with the search space on the scheduling cell with the same search space ID

| Alt 2-1 | Alt2-2 | Alt2-3 |
| --- | --- | --- |

Figure 4: Alt 2, configure the search space of DCI 0_X/1_X on scheduling cell and co-scheduled cells

We prefer Alt 2-1 more than other alternatives. It is a straight forward enhancement of search space configuration for cross carrier scheduling. Because only if the serving cell have the specific search space, the configuration of this cell can be applied to the DCI formats/fields associated with the search space. Such as the time domain resource allocation field, if multiple SLIV for each co-scheduled cell is indicated by one field, the SLIV indexes of each cell refer to its own TDRA table. If one scheduled cell do not configure the search space, it is impossible to found the configuration information of some DCI fields.

- The search space of DCI 0_X/1_X is configured on each co-scheduled cell.

Hash function: CCE indexes of PDCCH candidates

Generally, it is fine to reuse the current hash function of search space, except CIF parameter. In Rel-15, the CCE index of a PDCCH candidates depends on  if CCS is applied, and is also the carrier indicator field in the DCI to indicate which carrier is scheduled. And the CCE indexes of PDCCH candidates obtained by one CIF value, the DCI size is fixed to one value. So for multi-cell scheduling, there are two issues need to be studied for hash function.

- Issue 1: How to decide the CCE indexes of PDCCH candidates if the Option 1 or Option 2 is used for the indication of co-scheduled cells. CIF in Option 3 does not have this problem, it can reuse CIF value.

  - Method1: different combinations of co-scheduled cells can use the different CCE indexes, such as use the indicator in option 1 or bitmap in option 2 to replace . In this case, Option 1 and CIF in Option 3 are same, Option 2 has a finer granularity in co-scheduled cell indication but with higher payload.

  - Method2: different combinations of co-scheduled cells can share the same CCE indexes, such as several indicators in option 1 or bitmap in option 2 can related with a same . The Table 4 gives an example. Then, several co-scheduled cell combinations can share the same set of CCE indexes of PDCCH candidates.

- Issue 2: the DCI size should be same or can be different for the CCE indexes of PDCCH candidates obtained by one indicator/bitmap/CIF value. We prefer to have the same DCI size for one indicator/bitmap/CIF value, to simplify the DCI design and PDCCH monitoring.

Table 4: An example of method2 for issue 1,

| CIF | Option 1: Indicator | 1st Co-scheduled cell | 2nd Co-scheduled cell | 3rd Co-scheduled cell | 4th Co-scheduled cell |
| --- | --- | --- | --- | --- | --- |
| 0 | 000 | CC0 | CC3 |  |  |
|  | 001 | CC1 | CC3 |  |  |
| 1 | 010 | CC2 | CC3 |  |  |
|  | 011 | CC0 | CC1 | CC3 |  |
| 2 | 100 | CC0 | CC1 | CC2 |  |
|  | 101 | CC1 | CC2 |  |  |
| 3 | 110 | CC0 | CC1 | CC2 | CC3 |
|  | 111 | CC0 | CC2 | CC3 |  |

- Study the relationship between co-scheduled cell combination and the CCE indexes of PDCCH candidates, and down select between method 1 and method 2 below.

    - Method1: different combinations of co-scheduled cells can use the different CCE indexes

    - Method2: different combinations of co-scheduled cells can share the same CCE indexes

- The DCI size should be same for the CCE indexes of PDCCH candidates obtained by one indicator/bitmap/CIF value.

Search space configured with DCI format 0_X/1_X and legacy DCI format(s)

Another issue is whether or not the search space configured with DCI format 0_X/1_X can also be configured with legacy DCI formats, such as DCI format 0_X/1_X and DCI format 0_1/1_1. We propose to not support these mixed/simultaneous DCI formats configuration.

- First, if DCI format 0_X/1_X and legacy DCI format(s) are configured with one search space, it requires to always monitor single-cell and multi-cell scheduling simultaneously. But some UE may not support this type of monitor.

- Second, there is a possibility that a cross-carrier scheduled cell linked to this search space, but it does not configure multi-cell scheduling and vice versa. In this case, there would be some misunderstanding that how many DCI formats should be counted under the search space.

- Third, the DCI size counting and search space configuration have a lot issues need to decide. It would be good to make think easier, that separate search space for DCI format 0_X/1_X and legacy DCI format, not allowed the mixed configurations.

- The search space configured with DCI format 0_X/1_X cannot be configured with legacy DCI formats.

DCI size and BD/CCE budget

DCI size

The DCI size is calculated per serving cell in the previous release. For multi-cell scheduling, one DCI schedule multi-cell’s PDSCH/PUSCH, the DCI size counter and budget maintaining are to be decided. For DCI size budget, the agreements in RAN1#109e provided 2 Options 8 alternatives, and they are listed below.

Agreement in RAN1#109e Further study DCI size budget including below options for multi-cell scheduling DCI: - Option 1: Existing DCI size budget is maintained per scheduled cell. - Alt 1-1: DCI size budget is maintained via DCI size alignment and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells. - Alt 1-2: DCI size budget is maintained via configured size for multi-cell scheduling DCI and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells. - Alt 1-3: DCI size budget is maintained via DCI size alignment and DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell. - Option 2: Existing DCI size budget is not necessarily maintained per scheduled cell. - Alt 2-1: DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell. - Alt 2-2: DCI size budget of multi-cell scheduling DCI is not counted per serving cell and not considered in the related serving cell specific DCI size alignment procedure, e.g., for K co-scheduled cells, gNB guarantee the total budget of 3*K DCI sizes is not exceeded. - Alt 2-3: voiding the “3+1” limit for multi-cell scheduling - Alt 2-4: the DCI size budget for DCI size alignment can be separately configured for each cell - Alt 2-5: DCI size budget of the scheduling cell can be increased to account for the DCI format for multi-cell scheduling. Accordingly, the DCI size budget of a scheduled cell can be reduced. - Other options/alternatives could be considered.

For DCI size counting, it is the total number of different DCI formats configured for one serving cell, which is the sum up number of all DCI formats related with all search spaces monitored by one cell. Thus, if a scheduled cell configures a search space, the DCI format size should be counted for this cell. From this understanding, the search space configuration discussed in section 2.3.1 is the foundation for DCI size counting. Furthermore, DCI size counting and DCI size budget are two linked closely. And for acceptable UE implementation complexity, only Option 1(Existing DCI size budget is maintained per scheduled cell) is reasonable. The alternatives in Option 2 not only have huge impact on specification, but also for PDCCH monitoring. They are undesirable. For short, if DCI size is counted per each of the co-scheduled cell, and DCI size budget should be maintained per scheduled cell.

For easy tracking, we also list the search space configurations alternatives in the Table 5, and try to give the relationship between DCI size budget and search space.

Table 5:  DCI size counting/budget and search space configurations

| Association of search space among the scheduling and co-scheduled cells | DCI size counting | DCI size budget |
| --- | --- | --- |
| Alt 1: configure the search space of DCI 0_X/1_X only on scheduling cell | DCI size is counted only for the scheduling cell | DCI size budget is maintained for scheduling cell. Not in the scope of agreement |
| Alt 2-1: configure the search space of DCI 0_X/1_X on each co-scheduled cell, and associated with the search space on the scheduling cell with the same search space ID | DCI size is counted for each of the co-scheduled cell | Alt 1-1/2 |
| Alt 2-2: only configure the search space of DCI 0_X/1_X on one specified co-scheduled cell, and associated with the search space on the scheduling cell with the same search space ID | DCI size is counted for one specific co-scheduled cell | Alt 1-3 |
| Alt 2-3: configure the search space of DCI 0_X/1_X on a subset of co-scheduled cells, and associated with the search space on the scheduling cell with the same search space ID | DCI size is counted for each of the subset of the co-scheduled cell which configured with search space. | Not in the scope of agreement |

With same reason of our preference for search space configuration, the Alt1-1 in Option 1 is a good choice.

- Support Alt 1-1: DCI size budget is maintained via DCI size alignment and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

BD/CCE budget

There were six alternatives for BD/CCE counting for multi-cell scheduling DCI.

Agreement in RAN1#109e Further study BD/CCE counting for multi-cell scheduling DCI based on below options: - Alt 1: counted on each co-scheduled cell - Alt 2: counted only in one scheduled cell - Alt 3: scaled down to each of co-scheduled cell according to the number of co-scheduled cells - Alt 4: counted as part of the scheduling cell instead of each scheduled cell - Alt 5: scaled down to each of scheduled cells excluding scheduling cell - Alt 6: counted on each co-scheduled cell excluding scheduling cell - Other alternatives could be considered.

We think BD/CCE counting should use the same rule as DCI size counting/budget, which is counted on each co-scheduled cell. It is wired that DCI size budget is maintained by each of co-scheduled cell, but BD/CCE budget is by another rule. In order to align those two closely associated factors, Alt 1 should be used for BD/CCE counter.

- Support Alt 1: counted on each co-scheduled cell

DCI field design

Regarding the fields in the DCI for payload reduction, there are can be three types, which were agree in RAN1#109e.

Agreement in RAN1#109e For design of multi-cell scheduling DCI, companies are encouraged to consider following types of DCI fields: - Type-1 field: A single field indicating common information to all the co-scheduled cells or separate information to each of co-scheduled cells via joint indication or an information to only one of co-scheduled cells - Type-2 field: Separate field for each of the co-scheduled cells, or each sub-group comprising one or more co-scheduled cells where a single field is commonly applied to the co-scheduled cells belonging to a same sub-group - Type-3 field: Common or separate to each of the co-scheduled cells or to each sub-group. - FFS: whether it is dependent on explicit configuration or implicit condition (e.g., intra or inter band CA, FR1 or FR2). - Other types are not precluded.

Types field in DCI

The DCI payload size is the main concern, because with the increase of DCI size, the performance of PDCCH is exponential decreased, such as reliability of PDCCH and coding/decoding complexity. All the fields of the DCI can be divided into above three types of fields. Clearly, Type-3 field can configured into Type-1 or Type-2 field.

For Type-1 field, A single field indicating common information to all the co-scheduled cells (marked as Type-1A) or separate information to each of co-scheduled cells via joint indication (marked as Type 1B) or an information to only one of co-scheduled (marked as Type 1C). The Table 6/7/8 give separate examples for Type-1A/B/C field.

Table 6 Type-1A field (1bit): common information to all the co-scheduled cells

| DCI code point | Value for cell 1 | Value for cell 2 |
| --- | --- | --- |
| 0 | A | A |
| 1 | B | B |

Table 7 Type-1B field (2bits): separate information to each of co-scheduled cells via joint indication

| DCI code point | Value for cell 1 | Value for cell 2 |
| --- | --- | --- |
| 0 | A | X |
| 1 | B | Y |
| 2 | C | Z |
| 3 | D | W |

Table 8 Type-1C field (2bits): an information to only one of co-scheduled cells

| DCI code point | Value for cell 1 |
| --- | --- |
| 0 | A |
| 1 | B |
| 2 | C |
| 3 | D |

For Type-2 field, separate field for each of the co-scheduled cells (Type-2A), or each sub-group comprising one or more co-scheduled cells where a single field is commonly applied to the co-scheduled cells belonging to a same sub-group (Type-2B). The Table 9/10 give two examples for Type-2A/B field.

Table 9 Type-2A field (2bits*cell number): Separate field for each of the co-scheduled cells

| DCI code point A,B,C,D={00,01,10,11} | Value for cell 1 | Value for cell 2 | Value for cell 3 | Value for cell 4 |
| --- | --- | --- | --- | --- |
| ABCD | A | B | C | D |
| BCDA | B | C | D | A |
| CDBA | C | D | B | A |
| DABC | D | A | B | C |

Table 10 Type-2B field (2bits*sub-group number): Separate field for each of the co-scheduled cells

|  | Sub-group 0 |  | Sub-group 1 |  |
| --- | --- | --- | --- | --- |
| DCI codepoint A,B,C,D={00,01,10,11} | Value for cell 1 | Value for cell 2 | Value for cell 3 | Value for cell 4 |
| AB | A | A | B | B |
| BC | B | B | C | C |
| CD | C | C | D | D |
| DA | D | D | A | A |

For Type-3 field, either common or separate to each of the co-scheduled cells or to each sub-group can be configured or implicit decided. Then Type-3 field can be changed into the Table 6~10, depending on the filed design.

When the scheduled cells share more similar transmission condition or scheduling status, there can be more common fields, otherwise, more separate fields. As shown in the Figure 3 below, it is an example of single stage DCI design for multi-cell scheduling.

- For Type-1 field, such as Identifier for DCI formats field, it is still one bit applied for all the scheduled cells, only PDSCH scheduling or PUSCH scheduling.

- The Type-2 field in the DCI contains independent sub-fields for each scheduled cell. Such as Frequency domain resource assignment, if this DCI schedule two cells, it provides two sub-fields, the first sub-field is for the 1st scheduled cell, and the second sub-field is for the 2nd scheduled cell. The length of each sub-fields depend on its FDRA of corresponding cell.

- Regarding Type-3 field, it can be common field or separate field based on RRC configuration, then it is same as the above.

Figure 5: Three types of DCI fileds for multi-cell scheduling

DL/UL DCI fields

In this section, we provide our preference for all the DCI fields in DCI format 0_X and DCI format 1_X, based on the Proposal 3-2rev1. We support all the type-1 and Type-2 fields listed below, also give more detailed type for type-1 and Type-2, such as Type-1A/1B/1C. In addition, for the other FFS fields, their types are also proposed. Next, the configuration granularity is important for the fields, e.g. per BWP or per cell, or per cell group. Thus, we also gives some analysis on the granularity of the fields, and try to give more information on their bit length under multi-cell scheduling based on the legacy DCI formats.

Proposal 3-2rev1: - For DCI format 0_X/1_X which can schedule more than one cell, below type classification can be a starting point for further discussion: - Type-1 fields at least include below: - Identifier for DCI formats - Downlink assignment index - TPC for scheduled PUCCH - PUCCH resource indicator - PDSCH-to-HARQ timing indicator - FFS: Type-2 fields at least include below: - New data indicator - Redundancy version - FFS: - Indicator of co-scheduled cells - UL/SUL indicator - PRB bundling size indicator - Rate matching indicator - ZP CSI-RS trigger - Antenna port(s) - TCI - SRS request - DMRS sequence initialization - TPC for scheduled PUSCHs - Modulation and coding scheme - Bandwidth part indicator - Time domain resource assignment - Frequency domain resource assignment - VRB-to-PRB mapping - HARQ process number - One-shot HARQ-ACK request - ChannelAccess-Cpext - CSI request - SRI - beta offset indicator - Other fields

DCI fields in DCI format 1_X

Table 11: DCI fields in DCI 1_X

| Fields | DCI format 1_X |  | DCI format 1_1 |  | DCI format 1_2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Granularity | Type1/2/3 | Granularity | Bits length | Granularity | Bits length |
| Identifier for DCI formats | fixed | Type-1A | fixed | 1 bit | fixed | 1 bit |
| Carrier indicator o/Indicator/bitmap of co-scheduled cells | fixed | Type-1B | Per cell | 0 or 3 bit | Per cell | Configurable # of bits (0, 1, 2 or 3 bits) |
| Bandwidth part indicator | NA | NA | Per cell | 0, 1 or 2 bits | Per cell | Configurable # of bits (0, 1 or 2 bits) |
| Frequency domain resource assignment | Per Cell & Per BWP | Type2A/2B | Per BWP | RA type 0 and or RA type 1 | Per BWP | resourceAllocationDCI-1-2-r16 |
| Time domain resource assignment | Per cell & Per BWP | Type-1B | Per BWP | 0 or 1 or 2 or 3 or 4 bits | Per BWP | Configurable # of bits (0 or 1 or 2 or 3 or 4 bits) |
| VRB-to-PRB mapping | Per cell & Per BWP | Type2A/2B | Per BWP | 0 or 1 bit | Per BWP | Configurable # of bits (0 or 1 bit) |
| PRB bundling size indicator | Per cell &Per BWP | Type-3 | Per BWP | 0 or 1 bit | Per BWP | Configurable # of bits (0 or 1 bit) |
| Rate matching indicator | Per cell &Per BWP | Type-2A | Per BWP | 0 or 1 or 2 bits | Per BWP | Configurable # of bits (0 or 1 bit or 2 bits) |
| ZP CSI-RS trigger | Per cell &Per BWP | Type-3 | Per BWP | 0 or 1 or 2 bits | Per BWP | Configurable # of bits (0 or 1 bit or 2 bits) |
| Modulation and coding scheme | Per cell &Per BWP | Type-3 | Per BWP | 5 bits | Per BWP | 5bits |
| New data indicator | fixed | Type-2A | fixed | 1 bit | fixed | 1 bit |
| Redundancy version | fixed | Type-2A | fixed | 2 bits | Per BWP | Configurable # of bits (0 or 1 or 2 bits) |
| HARQ process number | fixed | Type-3 | fixed | 4 bits | Per BWP | Configurable # of bits (0 or 1 or 2 or 3 or 4 bits) |
| Downlink assignment index | Per cell group | Type-1A | Per cell group | 0 or 2 or 4 bits | Per cell group | Configurable # of bits (0 or 1 or 2 or 4 bits) |
| TPC command for scheduled PUCCH | fixed | Type-1A | fixed | 2 bits | Fixed | 2 bits as same as Rel-15 |
| PUCCH resource indicator | fixed | Type-1A | Per UL BWP and per PUCCH group | 3 bits | Per UL BWP and per PUCCH group | Configurable # of bits (0 or 1 or 2 or 3 bits) |
| PDSCH-to-HARQ_feedback timing indicator | fixed | Type-1A | Per UL BWP and per PUCCH group | 0 or 1 or 2 or 3 bits | Per UL BWP and per PUCCH group | Configurable # of bits (0 or 1 or 2 or 3 bits), log2(I) |
| One-shot HARQ-ACK request | Per cell group | Type-1A | Per cell group | 0 or 1 bit | Per cell group | 0 or 1 bit |
| Enhanced Type 3 codebook indicator | Per cell group | Type-1A | Per cell group | 0, 1, 2, or 3 bits | Per cell group | 0, 1, 2, or 3 bits |
| PDSCH group index | Per cell group | Type-1A | Per cell group | 0 or 1 bit | N/A | 0 |
| New feedback indicator | Per cell group | Type-1A | Per cell group | 0, 1 or 2 bits | N/A | 0 |
| Number of requested PDSCH group(s) | Per cell group | Type-1A | Per cell group | 0 or 1 bit | N/A | 0 |
| HARQ-ACK retransmission indicator | N/A | 0 | Per cell group | 0 or 1 bit | Per cell group | 0 or 1 bit |
| Antenna port(s) | Per cell &Per BWP | Type-3 | Per BWP | 4 or 5 or 6 bits | Per BWP | Configurable # of bits (0 or 4 or 5 or 6 bits) |
| Transmission configuration indication | Per cell &Per BWP | Type-3 | Per CORESET | 0 or 3 bits | Per CORESET | Configurable # of bits (0 or 1 or 2 or 3 bits) |
| SRS request | Per cell &Per BWP | Type-3 | Per cell | 2 bits or 3 bits w/o SUL | Per BWP & per cell | Configurable # of bits (0 or 1 or 2 or 3 bits) |
| SRS offset indicator | Per cell &Per BWP | Type-3 | Per cell | 0, 1 or 2 bits | Per cell | 0, 1 or 2 bits |
| DMRS sequence initialization | Per cell | Type-3 | fixed | 1 bit | Per BWP | Configurable # of bits (0 or 1 bit) |
| Priority indicator | Per cell &Per BWP | Type-1A | Per BWP | 0 or 1 bit | Per BWP | Configurable # of bits (0 or 1 bit) |
| ChannelAccess-CPext | Per cell &Per BWP | Type-1A | Per BWP | 0, 1, 2, 3 or 4 bits | Per BWP | 0, 1, 2, 3 or 4 bits |
| Minimum applicable scheduling offset indicator | NA | 0 | Per BWP | 0 or 1 bit | NA | 0 |
| SCell dormancy indication | NA | 0 | P(S)cell only | 0, 1, 2, 3, 4 or 5 bits | NA | 0 |
| PDCCH monitoring adaptation indication | Per scheduling cell | Type-1A | Per BWP | 0, 1 or 2 bits | Per BWP | 0, 1 or 2 bits |
| Modulation and coding scheme for TB 2 | N/A | 0 | Per BWP | 0 or 5 bits | N/A | 0 |
| New data indicator for TB 2 | N/A | 0 | Per BWP | 0 or 1 bit | N/A | 0 |
| Redundancy version for TB 2 | N/A | 0 | Per BWP | 0 or 2 bits | N/A | 0 |
| CBG transmission information | N/A | 0 | Per BWP | 0 or 2 or 4 or 6 or 8 bits | N/A | 0 |
| CBG flushing information | N/A | 0 | Per BWP | 0 or 1 bit | N/A | 0 |

- The fields in DCI format 1_X are with the below type classification:

- Type-1 fields include below:

  - Identifier for DCI formats (Type-1A)

  - Downlink assignment index (Type-1A)

  - TPC for scheduled PUCCH (Type-1A)

  - PUCCH resource indicator (Type-1A)

  - PDSCH-to-HARQ timing indicator (Type-1A)

  - Carrier indicator o/Indicator/bitmap of co-scheduled cells (Type-1B)

  - Time domain resource assignment (Type-1B)

  - One-shot HARQ-ACK request(Type-1A)

  - Priority indicator (Type-1A)

  - ChannelAccess-Cpext  (Type-1A)

  - Enhanced Type 3 codebook indicator (Type-1A) if eType-3 HARQ-ACK is supported

  - If eType-2 HARQ-ACK is supported for multi-cell scheduling

    - PDSCH group index  (Type-1A)

    - New feedback indicator  (Type-1A)

    - Number of requested PDSCH group(s) (Type-1A)

  - PDCCH monitoring adaptation indication (Type-1A)

- Type-2 fields include below:

  - New data indicator (Type-2A)

  - Redundancy version (Type-2A)

  - Frequency domain resource assignment (Type2A/2B)

  - VRB-to-PRB mapping (Type2A/2B)

  - Rate matching indicator (Type-2A)

- Type-3 fields include below:

  - MIMO related:

    - PRB bundling size indicator

    - ZP CSI-RS trigger

    - Antenna port(s)

    - TCI

    - SRS request

    - CSI request

    - DMRS sequence initialization

    - SRS offset indicator

  - Modulation and coding scheme

  - HARQ process number

- Fields not needed or supported for multi-carrier scheduling include below:

  - Bandwidth part indicator

  - Second TPC command for scheduled PUCCH

  - Transport block 2

    - Modulation and coding scheme

    - NDI

    - RV

  - CBG

    - CBG transmission information

    - CBG flushing information

  - HARQ-ACK retransmission indicator

  - Minimum applicable scheduling offset indicator

  - SCell dormancy indication

DCI fields in DCI format 0_X

Table 12: DCI fields in DCI 0_X

| Fields | DCI format 0_X |  | DCI format 0_1 |  | DCI format 0_2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Granularity | Type1/2/3 | Granularity | Bits length | Granularity | Bits length |
| Identifier for DCI formats | fixed | Type-1A | Fixed | 1 bit | fixed | 1 bit |
| Carrier indicator o/Indicator/bitmap of co-scheduled cells | fixed | Type-1B | Per cell | 0 or 3 bit | Per cell | Configurable # of bits (0, 1, 2 or 3 bits) |
| DFI flag | NA | 0 |  | Reserve | NA | NA |
| UL/SUL indicator | FFS |  | Per cell | 0 bit or 1 bit | Per cell | 0 bit or 1 bit |
| Bandwidth part indicator | NA | NA | Per cell | 0, 1 or 2 bits | Per cell | 0, 1 or 2 bits |
| Frequency domain resource assignment | Per Cell & Per BWP | Type2A/2B | Per BWP | RA type 0/1/2 | Per BWP | RA type 0 and/or RA type 1 |
| Time domain resource assignment | Per cell & Per BWP | Type-1B | Per BWP | 0 or 1 or 2 or 3 or 4 or 5, or 6 bits | Per BWP | 0 or 1 or 2 or 3 or 4 or 5, or 6 bits |
| Frequency hopping flag | Per cell & Per BWP | Type2A/2B | Per BWP | 0 or 1 bit | Per BWP | 0 or 1 bit |
| Modulation and coding scheme | Per cell &Per BWP | Type-3 | Fixed | 5 bits | Fixed | 5 bits |
| New data indicator | fixed | Type-2A | Fixed | 1 bit | Fixed | 1 bit |
| Redundancy version | fixed | Type-2A | Fixed | 2 bits | Per BWP | Configurable # of bits (0 or 1 or 2 bits) |
| HARQ process number | fixed | Type-3 | Fixed | 4 bits [Note]: 5bits is for NTN | Per BWP | Configurable # of bits (0 or 1 or 2 or 3 or 4 bits) |
| (1st) Downlink assignment index | Per cell group | Type-1A | Per cell group | 0 or 2 or 4 bits | Per BWP | Configurable # of bits (0 or 1 or 2 or 4 bits) |
| 2nd downlink assignment index | Per cell group | Type-1A | Per cell group | 0, 2 or 4 bits | NA | 0 |
| TPC command for scheduled PUSCH | Per cell | Type-2A/2B | fixed | 2 bits | fixed | 2 bits |
| SRS resource set indicator | Per cell &Per BWP | Type-3 | Per BWP | 0 or 2 bits | Per BWP | 0 or 2 bits |
| SRS resource indicator | Per cell &Per BWP | Type-3 | Per BWP | 0 or 1 or 2 or 3 or 4 bits | Per BWP | 0 or 1 or 2 or 3 or 4 bits |
| Second SRS resource indicator | NA | 0 | Per BWP | 0 or 1 or 2 or 3 or 4 bits | Per BWP | 0 or 1 or 2 or 3 or 4 bits |
| Precoding information and number of layers | Per cell &Per BWP | Type-3 | Per BWP | 0 or 1 or 2 or 3 or 4 or 5 or 6 bits | Per BWP | 0 or 1 or 2 or 3 or 4 or 5 or 6 bits |
| Second Precoding information | NA | 0 | Per BWP | 0 or 1 or 2 or 3 or 4 or 5 bits | Per BWP | 0 or 1 or 2 or 3 or 4 or 5 bits |
| Antenna port(s) | Per cell &Per BWP | Type-3 | Per BWP | 2 or 3 or 4 or 5 bits | Per BWP | (0 or 2 or 3 or 4 or 5 bits) |
| SRS request | Per cell &Per BWP | Type-3 | Per Cell | 2 or 3 bits | Per Cell | Configurable # of bits (0 or 1 or 2 or 3 bits) |
| SRS offset indicator. | Per cell &Per BWP | Type-3 | Per cell | 0, 1 or 2 bits | Per cell | 0, 1 or 2 bits |
| CSI request | Per cell &Per BWP | Type-3 | per PUCCH group | 0 or 1 or 2 or 3 or 4 or 5 or 6 bits | per PUCCH group | (0 or 1 or 2 or 3 or 4 or 5 or 6 bits) |
| CBG transmission information (CBGTI) | NA | 0 | Per BWP | 0, 2, 4, 6, or 8 bits | NA | NA |
| PTRS-DMRS association | Per cell &Per BWP | Type-3 | Per BWP | 0 or 2 bits | Per BWP | 0 or 2 bits |
| Second PTRS-DMRS association | NA | 0 | Per BWP | 0 or 2 bits | Per BWP | 0 or 2 bits |
| beta offset indicator | Per cell &Per BWP | Type-3 | Per BWP | 0 or 2 bits | Per BWP | (0 or 1 or 2 bits) |
| DMRS sequence initialization | Per cell &Per BWP | Type-3 | Per BWP | 0 or 1 bit | Per BWP | (0 or 1 bit) |
| UL-SCH indicator | NA | 0 | Fixed | 1 bit | Fixed | 1 bit |
| ChannelAccess-CPext-CAPC | Per cell &Per BWP | Type-1A | Per BWP | 0, 1, 2, 3, 4, 5 or 6 bits | Per BWP | 0, 1, 2, 3, 4, 5 or 6 bits |
| Open-loop power control parameter set indication | NA | 0 | Per BWP | 0 or 1 or 2 bits | Per BWP | 0 or 1 or 2 bits |
| Priority indicator | Per PUCCH group | Type-1A | Per BWP | 0 or 1 bit | Per BWP | 0 or 1 bit |
| Invalid symbol pattern indicator | NA | 0 | Per BWP | 0 or 1 bit | Per BWP | 0 or 1 bit |
| Minimum applicable scheduling offset indicator | NA | 0 | Per BWP | 0 or 1 bit | NA | 0 |
| SCell dormancy indication | NA | 0 | P(S)Cell only | 0, 1, 2, 3, 4 or 5 bits | NA | 0 |
| PDCCH monitoring adaptation indication | Per scheduling cell | Type-1A | Per BWP | 0, 1 or 2 bits | Per BWP | 0, 1 or 2 bits |

- The fields in DCI format 0_X are with the below type classification:

- Type-1 fields include below:

  - Identifier for DCI formats (Type-1A)

  - Carrier indicator o/Indicator/bitmap of co-scheduled cells (Type-1B)

  - Time domain resource assignment (Type-1B)

  - Downlink assignment index (Type-1A)

  - Priority indicator (Type-1A)

  - ChannelAccess-CPext-CAPC  (Type-1A)

  - PDCCH monitoring adaptation indication (Type-1A)

- Type-2 fields include below:

  - New data indicator (Type-2A)

  - Redundancy version (Type-2A)

  - Frequency domain resource assignment (Type2A/2B)

  - Frequency hopping flag (Type2A/2B)

  - TPC command for scheduled PUSCH (Type2A/2B)

- Type-3 fields include below:

  - MIMO related:

    - SRS resource indicator

    - Precoding information and number of layers

    - Antenna port(s)

    - SRS request

    - SRS offset indicator

    - CSI request

    - PTRS-DMRS association

    - DMRS sequence initialization

  - Modulation and coding scheme

  - HARQ process number

  - beta offset indicator

- Fields not needed or supported for multi-carrier scheduling include below:

  - DFI flag

  - Bandwidth part indicator

  - SRS resource set indicator

  - Second SRS resource indicator

  - Second Precoding information

  - Second PTRS-DMRS association

  - UL-SCH indicator

  - Transport block 2

    - Modulation and coding scheme

    - NDI

    - RV

  - CBG

    - CBG transmission information

  - Invalid symbol pattern indicator

  - Minimum applicable scheduling offset indicator

  - SCell dormancy indication

  - Open-loop power control parameter set indication

HARQ enhancements

According to HARQ enhancements, the working assumption proposed to apply all HARQ=ACK codebooks for multi-carrier PDSCH scheduling. The further HARQ-ACK codebook generation need further provided.

Working Assumption in RAN1#109e - All HARQ-ACK codebook types (Type-1/2/3) are applicable when multi-carrier PDSCH scheduling is configured.

Type-1 HARQ-ACK codebook

Type-1 codebook is related to the design of TDRA indication in the multi-cell PDSCH scheduling DCI. We prefer to use Type-1B for Time domain resource allocation field of DCI 1_X. A TDRA table is defined for the co-scheduled cells with each row indicating multiple SLIVs for the multiple scheduled cells. Then SLIV pruning and K1 set extension should be applied for Type-1 HARQ-ACK codebook.

- If Type-1B is used for Time domain resource allocation field of DCI 1_X, SLIV pruning and K1 set extension should be applied for Type-1 HARQ-ACK codebook

Type 2 HARQ-ACK codebook

Regarding the Type 2 HARQ-ACK codebook, we are fine with the Proposal 4-3rev2 and Proposal 4-4, and suggest to agree them before the further discussion.

- We support Proposal 4-3rev2 and Proposal 4-4.

Proposal 4-3rev2: - For Type-2 HARQ-ACK codebook, UE does not expect the multi-cell PDSCH scheduling and CBG-based PDSCH transmission are configured simultaneously on the same or different cell within a same PUCCH group. - FFS whether simultaneous configuration of multi-cell PDSCH scheduling and CBG-based PDSCH transmission on the same or different cell within a same PUCCH group is supported for Type-1 HARQ-ACK codebook. - FFS whether simultaneous configuration of multi-cell PUSCH scheduling and CBG-based PUSCH transmission on the same or different cell within a same PUCCH group is supported. - FFS whether simultaneous configuration of multi-cell PDSCH/PUSCH scheduling and multi-slotPDSCH/PUSCH scheduling with different DCIs on a same or different cell(s) within a same PUCCH group is supported. Proposal 4-4: - For Type-2 HARQ-ACK codebook, two sub-codebooks are generated with a first sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single cell and a second sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling more than one cell. - Separate DAI counting for DCI(s) with each scheduling a single cell and DCI(s) with each scheduling more than one cell - FFS whether the DCI scheduling a single cell and the DCI scheduling more than one cell are determined based on the number of cells indicated by DCI or the number of cells with actual PDSCH reception - Type-2 HARQ-ACK codebook is generated by concatenating the first sub-codebook and the second sub-codebook. - At least following is supported: Number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is determined based on the maximum number of cells co-scheduled by a DCI format 1_X in the PUCCH-group for the UE. - FFS for the case with 2-TB PDSCH scheduling without spatial bundling configuration - HARQ-ACK information bits for co-scheduled PDSCHs by a DCI format 1_X is ordered based on serving cell indices associated with co-scheduled PDSCHs.

One issue is it needs to clarify whether eType2 HARQ-ACK codebook also need to support for multi-cell scheduling.

Type 3 HARQ-ACK codebook

Regarding Type 3 HARQ-ACK codebook, we think the legacy mechanism can be applied directly for multi-cell scheduling. One issue is it needs to clarify whether eType3 HARQ-ACK codebook also need to support for multi-cell scheduling. If so, the following update for the WA should be made.

- Clarify whether eType2 and eType3 HARQ-ACK codebook are in the scope.

UE PDSCH/PUSCH reception preparation time with multi-cell scheduling

According to UE PDSCH reception preparation time with multi-cell scheduling, if the SCS of scheduled cell is different from the scheduling cell, the gap between PDCCH end symbol and the starting position of PDSCH should be applied, as same as cross carrier scheduling. The value of Npdsch PDCCH symbols defined in 38.214 clause 5.5 can be reused here.

- The gap between PDCCH end symbol and the starting position of PDSCH defined in 38.214 should be applied for multi-cell scheduling with a single DCI when the SCS of scheduled cell is different from the scheduling cell.

Conclusion

In this contribution, we made the following observation and proposals.

- All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same PUCCH group.

- If multi-cell scheduling can be directly use for sSCell, it is fine to support a DCI format 0-X/1-X can be transmitted on the sSCell. Otherwise, it cannot be supported.

- Support Proposal 1-7rev1

- At least below cases 1-1 and 1-2 on SCS are prioritized:

- Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell.

- Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.

- Case 1-3: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and different SCS is used among the co-scheduled cells including the scheduling cell.

- Case 1-4: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and different SCS is used among the co-scheduled cells.

- At least below cases 2-1 and 2-2 on carrier type are prioritized:

- Case 2-1: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells including the scheduling cell and same carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among all the co-scheduled cells including the scheduling cell.

- Case 2-2: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells not including the scheduling cell and same carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among all the co-scheduled cells which may be same or different carrier type to the scheduling cell.

- Case 2-3: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells including the scheduling cell and different carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among the co-scheduled cells including the scheduling cell.

- Case 2-4: A DCI format 0-X/1-X on a scheduling cell can schedules multiple cells not including the scheduling cell and different carrier type (FDD or TDD, licensed or unlicensed, FR1 or FR2-1 or FR2-2) is used among the co-scheduled cells

- Note: for Case 2-2, 2-3 and 2-4, using an unlicensed cell for scheduling a set of co-scheduled cells including licensed cell is not prioritized.

- Only support same priority of PUSCHs on multi-cells scheduled by one DCI0_X, and same priority of HARQ-ACK of PDSCHs on multi-cells scheduled by one DCI 1_X in Rel-18.

- The value for the maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4, and the value for the maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

- Support (merged)Proposal 2-4 & 2-5rev2 with further steps (confirm the WA of the second bullet):

- For each scheduled cell, a UE monitors DCI format 0_X/1_X on at most one scheduling cell.

- For a cell within a set of configured cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring DCI format 0_X/1_X and legacy DCI format(s) from a same scheduling cell.

  - FFS: whether DCI format 0_X/1_X and legacy DCI format(s)  are monitored simultaneously

  - FFS: for which cell within the set of configured cells this is supported

- Not support monitoring DCI format 0_X/1_X and legacy DCI format(s) from different scheduling cells for a cell within a set of configured cells which can be co-scheduled by a DCI format 0_X/1_X.

- DCI format 0_X/1_X is a new DCI format for multi-cell scheduling

- Single-stage DCI format is supported for multi-cell PDSCH or PUSCH scheduling.

- Support Option 1 and CIF in Option 3 as the indication of co-scheduled cell combination

  - Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells

  - Option 3: using existing field CIF to indicate whether one or more cells are scheduled or not

- Only one size of DCI 0_X/1_X on one scheduling cell can be monitored.

- The search space of DCI 0_X/1_X is configured on each co-scheduled cell.

- Study the relationship between co-scheduled cell combination and the CCE indexes of PDCCH candidates, and down select between method 1 and method 2 below.

    - Method1: different combinations of co-scheduled cells can use the different CCE indexes

    - Method2: different combinations of co-scheduled cells can share the same CCE indexes

- The DCI size should be same for the CCE indexes of PDCCH candidates obtained by one indicator/bitmap/CIF value.

- The search space configured with DCI format 0_X/1_X cannot be configured with legacy DCI formats.

- Support Alt 1-1: DCI size budget is maintained via DCI size alignment and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

- Support Alt 1: counted on each co-scheduled cell

- The fields in DCI format 1_X are with the below type classification:

- Type-1 fields include below:

  - Identifier for DCI formats (Type-1A)

  - Downlink assignment index (Type-1A)

  - TPC for scheduled PUCCH (Type-1A)

  - PUCCH resource indicator (Type-1A)

  - PDSCH-to-HARQ timing indicator (Type-1A)

  - Carrier indicator o/Indicator/bitmap of co-scheduled cells (Type-1B)

  - Time domain resource assignment (Type-1B)

  - One-shot HARQ-ACK request(Type-1A)

  - Priority indicator (Type-1A)

  - ChannelAccess-Cpext  (Type-1A)

  - Enhanced Type 3 codebook indicator (Type-1A) if eType-3 HARQ-ACK is supported

  - If eType-2 HARQ-ACK is supported for multi-cell scheduling

    - PDSCH group index  (Type-1A)

    - New feedback indicator  (Type-1A)

    - Number of requested PDSCH group(s) (Type-1A)

  - PDCCH monitoring adaptation indication (Type-1A)

- Type-2 fields include below:

  - New data indicator (Type-2A)

  - Redundancy version (Type-2A)

  - Frequency domain resource assignment (Type2A/2B)

  - VRB-to-PRB mapping (Type2A/2B)

  - Rate matching indicator (Type-2A)

- Type-3 fields include below:

  - MIMO related:

    - PRB bundling size indicator

    - ZP CSI-RS trigger

    - Antenna port(s)

    - TCI

    - SRS request

    - CSI request

    - DMRS sequence initialization

    - SRS offset indicator

  - Modulation and coding scheme

  - HARQ process number

- Fields not needed or supported for multi-carrier scheduling include below:

  - Bandwidth part indicator

  - Second TPC command for scheduled PUCCH

  - Transport block 2

    - Modulation and coding scheme

    - NDI

    - RV

  - CBG

    - CBG transmission information

    - CBG flushing information

  - HARQ-ACK retransmission indicator

  - Minimum applicable scheduling offset indicator

  - SCell dormancy indication

- The fields in DCI format 0_X are with the below type classification:

- Type-1 fields include below:

  - Identifier for DCI formats (Type-1A)

  - Carrier indicator o/Indicator/bitmap of co-scheduled cells (Type-1B)

  - Time domain resource assignment (Type-1B)

  - Downlink assignment index (Type-1A)

  - Priority indicator (Type-1A)

  - ChannelAccess-CPext-CAPC  (Type-1A)

  - PDCCH monitoring adaptation indication (Type-1A)

- Type-2 fields include below:

  - New data indicator (Type-2A)

  - Redundancy version (Type-2A)

  - Frequency domain resource assignment (Type2A/2B)

  - Frequency hopping flag (Type2A/2B)

  - TPC command for scheduled PUSCH (Type2A/2B)

- Type-3 fields include below:

  - MIMO related:

    - SRS resource indicator

    - Precoding information and number of layers

    - Antenna port(s)

    - SRS request

    - SRS offset indicator

    - CSI request

    - PTRS-DMRS association

    - DMRS sequence initialization

  - Modulation and coding scheme

  - HARQ process number

  - beta offset indicator

- Fields not needed or supported for multi-carrier scheduling include below:

  - DFI flag

  - Bandwidth part indicator

  - SRS resource set indicator

  - Second SRS resource indicator

  - Second Precoding information

  - Second PTRS-DMRS association

  - UL-SCH indicator

  - Transport block 2

    - Modulation and coding scheme

    - NDI

    - RV

  - CBG

    - CBG transmission information

  - Invalid symbol pattern indicator

  - Minimum applicable scheduling offset indicator

  - SCell dormancy indication

  - Open-loop power control parameter set indication

- If Type-1B is used for Time domain resource allocation field of DCI 1_X, SLIV pruning and K1 set extension should be applied for Type-1 HARQ-ACK codebook

- We support Proposal 4-3rev2 and Proposal 4-4.

- Clarify whether eType2 and eType3 HARQ-ACK codebook are in the scope.

- The gap between PDCCH end symbol and the starting position of PDSCH defined in 38.214 should be applied for multi-cell scheduling with a single DCI when the SCS of scheduled cell is different from the scheduling cell.

References

RP-220834 Revised WID on Multi-carrier enhancements

Draft_Minutes_report_RAN1#109-e_v030

R1-2205488	Feature lead final summary on multi-cell PUSCH/PDSCH scheduling with a single DCI	Moderator (Lenovo)