3GPP TSG-RAN WG1 Meeting #110	R1-2207729

Toulouse, France, 22nd – 26th August 2022

Agenda Item:	8.6

Title:	FL summary #3 for Rel-17 RedCap maintenance

Source:	Moderator (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This feature lead (FL) summary (FLS) concerns the Rel-17 work item (WI) for support of reduced capability (RedCap) NR devices [1, 2]. Earlier RAN1 agreements for this WI are summarized in [3], and the FLSs from the previous RAN1 meeting can be found in [4, 5, 6, 7, 8].

This document summarizes the contributions [9] – [45] submitted to agenda item 8.6 and captures this email discussion:

[110-R17-RedCap] To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, Tdoc number of the moderator summary for online session, etc. – Johan (Ericsson)

The issues that were in the focus of the previous rounds are tagged FL1 through FL5, and the FLSs for the previous rounds can be found in [54, 55]. The outcome of the Thursday 25th August online session is tagged FL6.

FL5 Question 1-1: Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| Spreadtrum | Huayu Zhou | huayu.zhou@unisoc.com |
| Qualcomm | Jing Lei | leijing@qti.qualcomm.com |
| CATT | Yongqiang FEI | feiyongqiang@catt.cn |
| ZTE, Sanechips | Youjun Hu | hu.youjun1@zte.com.cn |
| Samsung | Feifei Sun Seunghoon Choi | feifei.sun@samsung.com seunghoon.choi@samsung.com |
| FUTUREWEI | Vip Desai | vipul.desai@futurewei.com |
| CMCC | Lijie Hu | hulijie@chinamobile.com |
| Ericsson | Sandeep Narayanan Kadan Veedu | sandeep.narayanan.kadan.veedu@ericsson.com |
| NEC | Takahiro Sasaki | takahiro.sasaki@nec.com |
| OPPO | Zhisong Zuo | zuozhisong@oppo.com |
| vivo | Lihui Wang | wanglihui@vivo.com |
| Sharp | Liqing LIU | liu.liqing@sharp.co.jp |
| Ericsson | Sandeep Narayanan Kadan Veedu | sandeep.narayanan.kadan.veedu@ericsson.com |

# 2	BWP operation

## 2.1	SSB presence in 38.213

RAN1#109e discussed several text proposals (TPs) for 38.213 clause 17.1 that intended to better capture earlier RAN1 agreements. Contributions [9, 17, 18, 24] propose to adopt similar changes as TP#10 in the RAN1#109e FLS [5], which looked like this:

[The following paragraph captures presence of SSB in idle and inactive modes.] For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonRedCapSIB, if a UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to a Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, the UE assumes that does not expect the initial DL BWP does not to include SS/PBCH blocks or and the CORESET with index 0. If the UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type2-PDCCH CSS set, the UE assumes that the initial DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. if the UE used the SS/PBCH block to obtain SIB1 - includes a SS/PBCH block and does not include the CORESET with index 0 if the initial DL BWP does not include the SS/PBCH block the UE used to obtain SIB1 For an active DL BWP provided by BWP-DownlinkDedicated, a UE assumes that the active DL BWP includes a SS/PBCH block, unless the UE indicates a capability to operate in the DL BWP without receiving an SS/PBCH block, and does not include the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for separate initial DL BWP configured by BWP configuration option 1.] For an active DL BWP not provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for non-initial DL BWP configured by BWP configuration option 1 and initial/non-initial DL BWP configured by BWP configuration option 2.] For an active DL BWP provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 or the SS/PBCH blocks provided by NonCellDefiningSSB. If the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1, for SS/PBCH block and CORESET multiplexing pattern 1, the UE expects the active DL BWP to include the CORESET with index 0. If the active DL BWP includes the SS/PBCH blocks provided by NonCellDefiningSSB, these SS/PBCH blocks and the SS/PBCH blocks that the UE used to obtain SIB1 have the same quasi-colocation properties, if they have the same index.

Contributions [9, 10, 11, 14, 17, 18, 24, 30, 32 (section 2.2), 35, 36 (section 3)] aim to capture some of or all the agreements that TP#10 aimed to capture. Somewhat related, contribution [11] also proposes to remove the statement that “A UE with reduced capabilities (RedCap UE) supports all Layer-1 UE features that are mandatory without capability signalling” in 38.213 clause 17.

FL1 Question 2.1-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 3 | It is the left issue in the last meeting. The agreements of SSB presence for several meetings are pending to be captured in RAN1 spec. We also provide our TP in [12]. By the way, we have the following questions coming from checking companies’ TPs, which may reflect the different understandings among companies. - For paging reception in idle/inactive mode, should SSB presence be captured in RAN1 spec? It has been captured in RAN2 spec, and paging/SIB1/OSI should have the same behavior for SSB/CORESET#0 presence. Moreover, there is no explicit RAN1 agreement for paging reception other than BWP#0 configuration option 1 in connected mode - Our answer could be no - For paging reception for BWP#0 configuration option 1 in connected mode, should SSB presence be captured separately? It was agreed in RAN1#108e, but seem being overridden by RAN1#109e agreements - Our answer could be no - For RAR, how to understanding “SSB/CORESET#0” in the corresponding agreements? “SSB and CORESET#0” or “SSB or CORESET#0”? - Our answer could be “SSB and CORESET#0” - For CORESET#0, should CORESET#0 presence be captured in RAN1 spec? In RAN1 agreements, whether CORESET#0 is present is differentiated by FR1 and FR2, for now, it is differentiated by multiplexing pattern 1 and others in some companies’ view - Our answer could be yes |
| Nordic | 3 | RAN1 made the above mentioned agreements regarding presence of SSB/CORESET#0 in BWPs in IDLE and CONNECTED state, we believe those should be captured in RAN1 specs. When it comes to paging related, we would prefer to capture in RAN2 though. Regarding removal of “A UE with reduced capabilities (RedCap UE) supports all Layer-1 UE features that are mandatory without capability signalling”. We could keep the sentence and add an exception for FG 6-1? |
| vivo | 3 | About removing the statement that “A UE with reduced capabilities (RedCap UE) supports all Layer-1 UE features that are mandatory without capability signalling”, we understand there is a note in FG28-1 saying that a UE supporting FG28-1 is not required to support FG6-1. While there are other Rel-15 L1 UE capabilities mandatory without capability signalling that we did not update for RedCap. Given the corrections for SSB presence we will do in section 2.1, We prefer to keep it with following modifications: “A UE with reduced capabilities (RedCap UE) supports all Layer-1 UE features that are mandatory without capability signalling, unless stated otherwise.” |
| Intel | 3 | Also support the modification suggested by vivo on the handling of FG 6-1 for a UE supporting FG 28-1. |
| Qualcomm | 3 | Support the TP above for Clause 17.1 of TS 38.213. vivo’s suggestion looks good to us. |
| CATT | 3 | Also fine with vivo’s update. |
| ZTE, Sanechips | 3 | Paging in FR1 and FR2 should be differentiated if spec correction is needed for paging in RAN1. |
| Sharp | 3 | The above TP can be used as a baseline for discussion and some modifications need to be refined. For example, if a UE configured to monitor paging, the initial DL BWP should include CD-SSB regardless of the RRC state the UE is in. |
| Samsung | 3 | Fine with vivo’s update. |
| FUTUREWEI | 3 | Ok with vivo’s update |
| CMCC | 3 | This issue is about how to correctly capture the agreements about the SSB presence, and we think it is high priority. For the TP given above, it does not reflect the agreement as following, which means for an active DL BWP not provided by BWP-DownlinkDedicated in connected mode, if it is configured to monitor PDCCH according to Type2-PDCCH CSS set on this BWP, it will expect CD-SSB. Agreement:(RAN1#108e) - […] - For BWP#0 configuration option 1, - For FR1, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - For FR2, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - […] So we propose the following the TP, For an initial DL BWP provided by initialDownlinkBWP-RedCap-r17 in DownlinkConfigCommonSIB, the UE assumes that the initial DL BWP does not include SS/PBCH blocks or the CORESET with index 0 for the following cases, - if a UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, or - if the BWP is not provided by BWP-DownlinkDedicated, the UE in RRC_CONNECTED state indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block and it is not configured to monitor PDCCH according to Type2-PDCCH CSS set on this BWP. Otherwise, the UE assumes that the initial DL BWP includes SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. For the Layer-1 UE features part, fine with vivo’s update. |
| Nokia, NSB | 3 | Fine with Vivo’s update. |
| Ericsson | 3 | Fine with Vivo’s update. |
| NEC | 3 | Fine with vivo’s update. |
| OPPO | 3 | Ok with vivo’s update |
| Huawei, HiSilicon | 3 | Regarding vivo’s suggestion, we understand the intention and consider it is doable. However, we have a preference to remove the whole because what UE capability RedCap shall be able to support is already clear by 38306, thus this sentence in RAN1 is unnecessary/duplicate and is not RAN1 usual experience, now even conflict with RAN1 agreements. |
| FL2 | Based on received responses, the following proposal can be considered. High Priority Proposal 2.1-1a: - Agree the following TP for 38.213 clause 17 in principle: A UE with reduced capabilities (RedCap UE) supports all Layer-1 UE features that are mandatory without capability signalling, unless stated otherwise.. - Agree the following TP for 38.213 clause 17.1 in principle: [The following paragraph captures presence of SSB in idle and inactive modes.] For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonRedCapSIB, if a UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to a Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, the UE assumes that does not expect the initial DL BWP does not to include SS/PBCH blocks or and the CORESET with index 0. If the UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type2-PDCCH CSS set, the UE assumes that the initial DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. if the UE used the SS/PBCH block to obtain SIB1 - includes a SS/PBCH block and does not include the CORESET with index 0 if the initial DL BWP does not include the SS/PBCH block the UE used to obtain SIB1 For an active DL BWP provided by BWP-DownlinkDedicated, a UE assumes that the active DL BWP includes a SS/PBCH block, unless the UE indicates a capability to operate in the DL BWP without receiving an SS/PBCH block, and does not include the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for separate initial DL BWP configured by BWP configuration option 1.] For an active DL BWP not provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block or if a UE monitors PDCCH according to Type2-PDCCH CSS set, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for non-initial DL BWP configured by BWP configuration option 1 and initial/non-initial DL BWP configured by BWP configuration option 2.] For an active DL BWP provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 or the SS/PBCH blocks provided by NonCellDefiningSSB. If the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1, for SS/PBCH block and CORESET multiplexing pattern 1, the UE expects the active DL BWP to include the CORESET with index 0. If the active DL BWP includes the SS/PBCH blocks provided by NonCellDefiningSSB, these SS/PBCH blocks and the SS/PBCH blocks that the UE used to obtain SIB1 have the same quasi-colocation properties, if they have the same index.. |
| FL3 | RAN1 made the following agreement on Tuesday 23rd August: Agreement: - The following TP for 38.213 clause 17 is endorsed in principle: A UE with reduced capabilities (RedCap UE) supports all Layer-1 UE features that are mandatory without capability signalling, unless stated otherwise.. The second part of the proposal can be considered again, i.e.: High Priority Proposal 2.1-1b: - Agree the following TP for 38.213 clause 17.1 in principle: [The following paragraph captures presence of SSB in idle and inactive modes.] For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonRedCapSIB, if a UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to a Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, the UE assumes that does not expect the initial DL BWP does not to include SS/PBCH blocks or and the CORESET with index 0. If the UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type2-PDCCH CSS set, the UE assumes that the initial DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. if the UE used the SS/PBCH block to obtain SIB1 - includes a SS/PBCH block and does not include the CORESET with index 0 if the initial DL BWP does not include the SS/PBCH block the UE used to obtain SIB1 For an active DL BWP provided by BWP-DownlinkDedicated, a UE assumes that the active DL BWP includes a SS/PBCH block, unless the UE indicates a capability to operate in the DL BWP without receiving an SS/PBCH block, and does not include the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for separate initial DL BWP configured by BWP configuration option 1.] For an active DL BWP not provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block or if a UE monitors PDCCH according to Type2-PDCCH CSS set, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for non-initial DL BWP configured by BWP configuration option 1 and initial/non-initial DL BWP configured by BWP configuration option 2.] For an active DL BWP provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 or the SS/PBCH blocks provided by NonCellDefiningSSB. If the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1, for SS/PBCH block and CORESET multiplexing pattern 1, the UE expects the active DL BWP to include the CORESET with index 0. If the active DL BWP includes the SS/PBCH blocks provided by NonCellDefiningSSB, these SS/PBCH blocks and the SS/PBCH blocks that the UE used to obtain SIB1 have the same quasi-colocation properties, if they have the same index.. |
| Company | Y/N | Comments |
| Qualcomm | Y |  |
| Spreadtrum | N for paging part | We are fine for the most parts of CR except for paging part. Paging for idle/inactive state should be removed. If the UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type2-PDCCH CSS set, the UE assumes that the initial DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. 1) WA on paging reception made in RAN1#107e was NOT confirmed in RAN1#108e. And we changed to agree that paging reception can be specified in RAN1 spec only for BWP#0 configuration option 1 in connected state. Thus, in my memory there is no explicit agreement for paging reception in idle/inactive state in RAN1. 2) Current wording may not be aligned to RAN2 agreement. RAN2 confirms that if RedCap-specific initial DL BWP does not contain CD-SSB and CORESET#0, then this BWP will not be configured with a paging search space in any RRC state. In this case, the RedCap UE in RRC_CONNECTED state is not required to read paging.. Indeed RAN2 spec 38.331 has included the paging is monitored in the initial DL BWP containing CD-SSB in any RRC state. pagingSearchSpace ID of the Search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0.. Correct me if I’m wrong. Thanks. |
| vivo | N | We need to delete “in RRC_IDLE state or in RRC_INACTIVE state” based on RAN2’s agreements in following descriptions. If the UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type2-PDCCH CSS set, the UE assumes that the initial DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. if the UE used the SS/PBCH block to obtain SIB1 Note that following text only address type 2 PDCCH monitoring for initial BWP configuration option 1, it does not cover the initial BWP configuration option 2. For an active DL BWP not provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block or if a UE monitors PDCCH according to Type2-PDCCH CSS set, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. |
| Sharp | Y with some modifications | The initial DL BWP needs to include CD-SSB regardless of the UE’s RRC state as long as the UE is configured to monitor Paging. In addition, the blue text covers only BWP#0 configuration option 1 and cannot cover the BWP #0 configuration option 2. We therefore prefer to delete some texts as below. If the UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type2-PDCCH CSS set, the UE assumes that the initial DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. For an active DL BWP not provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block or if a UE monitors PDCCH according to Type2-PDCCH CSS set, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. |
| Spreadtrum2 | Compromise and questions raised for paging in idle/inactive state | Although I still think there is no agreement in RAN1 for paging reception in idle/inactive state, I’m fine for capturing it in RAN1 spec if it is the majority view. However, I still have a question whether current wording is aligned to RAN2’s agreement and spec (38.331). In RAN2’s agreement and spec, pagingSearchSpace ID of the Search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0.. It means UE should assume that the initial DL BWP includes SSB AND CORESET#0 regardless of SSB/CORESET#0 multiplexing pattern. However, the CR means UE should assume the initial DL BWP includes CD-SSB and CORESET#0 for SSB/CORESET#0 multiplexing pattern 1, CD-SSB only for SSB/CORESET#0 multiplexing pattern 2/3. They are different. To be honest, paging part in the CR is hard for me to read, and thus I still suggest removing it (without it Specs are not broken, and RAN2 spec is more accurate, and the reader may have to take RAN2 spec as reference). |
| Nordic | Agree with SPRD | We should remove from RAN1 spec all text related to TYPE2 SS (paging) We should capture in RAN1 only TYPE1 SS, and also that irrespective of Option 1 or Option 2, in RRC connected state UE expects SSB unless have capability. Thus the text can be simplified to For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonRedCapSIB, if a UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to a Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, the UE assumes that does not expect the initial DL BWP does not to include SS/PBCH blocks or and the CORESET with index 0. Unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 or the SS/PBCH blocks provided by NonCellDefiningSSB. for SS/PBCH block. If the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1, for SS/PBCH block and CORESET multiplexing pattern 1, the UE expects the active DL BWP to include the CORESET with index 0. If the active DL BWP includes the SS/PBCH blocks provided by NonCellDefiningSSB, these SS/PBCH blocks and the SS/PBCH blocks that the UE used to obtain SIB1 have the same quasi-colocation properties, if they have the same index. |
| ZTE, Sanechips | N | we do not know why the CORESET#0 should be included together with SSB for FR2, which is conflicting with the agreement actually. |
| CATT | Y in general | Agree to incorporate the modification from companies above to address the requirement of CD-SSB when paging is configured (regardless of any RRC state) |
| Intel | Y | For the sake of progress, we can also accept removing the paging related conditions and leaving it up to the RAN2 specs. |
| CMCC |  | The conflict agreements are copied in the following, RAN1#108e made the following agreements. Agreement: [38.213] - The following working assumptions from RAN1#107-e are NOT confirmed for idle/inactive mode and furthermore they are replaced by the agreements further down for connected mode. - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For BWP#0 configuration option 1, - For FR1, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - For FR2, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - Note: For BWP#0 configuration option 2, - For FR1, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB - For FR2, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE supporting FG 6-1a does not expect it to contain SSB/CORESET#0/SIB RAN2#118e made the following agreements 29. Clarify in the RRC field description that the paging search space is configured in an initial BWP only if that BWP includes the CD-SSB. 30. RAN2 confirms that if RedCap-specific initial DL BWP does not contain CD-SSB and CORESET#0, then this BWP will not be configured with a paging search space in any RRC state. In this case, the RedCap UE in RRC_CONNECTED state is not required to read paging. 31. Reply to RAN1 and explain there is no need to support paging connected RedCap UEs in a RedCap-specific initial DL BWP which does not contain CD-SSB and CORESET#0.. We think the RAN2 agreements also include BWP#0 configuration option 2, and require it to contain CD-SSB if configured for paging. However, above RAN1 agreements clearly express that BWP#0 configuration option#2 can be configured for a UE with FG28-1a to monitor paging even if it does not include SSB, or for UE with FG28-1 to monitor paging if it include NCD-SSB. So decision is needed for us to follow RAN1 or RAN2 agreements. If we follow RAN1 agreements, Proposal 2.1-1b can be adopted. If we follow RAN2 agreements, then the original proposal with “ in RRC_IDLE state or in RRC_INACTIVE state” deleted in the following sentence is enough. If the UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type2-PDCCH CSS set, the UE assumes that the initial DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. |
| Ericsson | Y |  |
| FL4 | Based on the responses from companies, Proposal 2.1-1c can be considered. The main change compared to previous round is that the following paging-related agreement from RAN2 has been incorporated in the last paragraph. RAN2 confirms that if RedCap-specific initial DL BWP does not contain CD-SSB and CORESET#0, then this BWP will not be configured with a paging search space in any RRC state. In this case, the RedCap UE in RRC_CONNECTED state is not required to read paging.. High Priority Proposal 2.1-1c: - Agree the following TP for 38.213 clause 17.1 in principle: [The following paragraph captures presence of SSB in idle and inactive modes.] For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonRedCapSIB, if a UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to a Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, the UE assumes that does not expect the initial DL BWP does not to include SS/PBCH blocks or and the CORESET with index 0. If the UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type2-PDCCH CSS set, the UE assumes that the initial DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. if the UE used the SS/PBCH block to obtain SIB1 - includes a SS/PBCH block and does not include the CORESET with index 0 if the initial DL BWP does not include the SS/PBCH block the UE used to obtain SIB1 For an active DL BWP provided by BWP-DownlinkDedicated, a UE assumes that the active DL BWP includes a SS/PBCH block, unless the UE indicates a capability to operate in the DL BWP without receiving an SS/PBCH block, and does not include the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for separate initial DL BWP configured by BWP configuration option 1.] For an active DL BWP not provided by BWP-DownlinkDedicated, unless if a UE does not indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block or if a UE monitors PDCCH according to Type2-PDCCH CSS set, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for non-initial DL BWP configured by BWP configuration option 1 and initial/non-initial DL BWP configured by BWP configuration option 2.] For an active DL BWP provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 or the SS/PBCH blocks provided by NonCellDefiningSSB. If the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1, for SS/PBCH block and CORESET multiplexing pattern 1, the UE expects the active DL BWP to include the CORESET with index 0. If the active DL BWP includes the SS/PBCH blocks provided by NonCellDefiningSSB, these SS/PBCH blocks and the SS/PBCH blocks that the UE used to obtain SIB1 have the same quasi-colocation properties, if they have the same index. [The following paragraph captures presence of SSB in all RRC states for both BWP configuration option 1 and BWP configuration option 2 when paging is monitored.] For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonSIB, a UE in RRC_IDLE state or in RRC_INACTIVE state or in RRC_CONNECTED state does not monitor PDCCH according to Type2-PDCCH CSS set if the initial DL BWP does not include the SS/PBCH blocks that the UE used to obtain SIB1 and the CORESET with index 0.. |
| Company | Y/N | Comments |
| Nordic |  | For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonRedCapSIB, if a UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to a Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, the UE assumes that does not expect the initial DL BWP does not to include SS/PBCH blocks or and the CORESET with index 0. This is OK If the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1, for SS/PBCH block and CORESET multiplexing pattern 1, the UE expects the active DL BWP to include the CORESET with index 0. If the active DL BWP includes the SS/PBCH blocks provided by NonCellDefiningSSB, these SS/PBCH blocks and the SS/PBCH blocks that the UE used to obtain SIB1 have the same quasi-colocation properties, if they have the same index. This paragraph should be standalone? For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonSIB, a UE in RRC_IDLE state or in RRC_INACTIVE state or in RRC_CONNECTED state does not monitor PDCCH according to Type2-PDCCH CSS set if the initial DL BWP does not include the SS/PBCH blocks that the UE used to obtain SIB1 and the CORESET with index 0. This paragraph is not needed, as will be captured in RAN2 spec? Nordic proposal: Unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 or the SS/PBCH blocks provided by NonCellDefiningSSB. for SS/PBCH block. This paragraph is generally true irrespective of BWP Option 1 or 2 or whether dedicated BWP is configured or not. |
| Spreadtrum | Generally Y | Thanks for FL’s update to make it clearer. We support to delete the original paging part. For the newly added paging part (in blue), it is intended to align to RAN2 agreement. However, it has been captured in RAN2 spec and paging/SIB1/OSI are in the same UE behaviors. pagingSearchSpace ID of the Search space for paging (see TS 38.213 [13], clause 10.1). If the field is absent, the UE does not receive paging in this BWP (see TS 38.213 [13], clause 10). This field is absent for the RedCap specific initial DL BWP, if it does not include CD-SSB and the entire CORESET#0.. I doubt that whether we need capture it in RAN1 spec which seems a totally duplication of RAN2 spec. For the misalignment mentioned by CMCC, the RAN1’s note in RAN1#108e shown paging can be configured if the separate initial DL BWP includes NCD-SSB. - Note: For BWP#0 configuration option 2, - For FR1, - For a separate initial DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), if it is configured for paging, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB. In my understanding, the RAN1’s note is just a clarification which has no spec impact, and the following RAN2’s agreements in RAN2#118e have overridden it. Anyway, I can live with this version, although I think the paging part is completely captured in RAN2 spec. |
| FL5 | High Priority Proposal 2.1-1d: - Agree the following TP for 38.213 clause 17.1 in principle: [The following paragraph captures presence of SSB in idle and inactive modes.] For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonRedCapSIB, if a UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to a Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, the UE assumes that does not expect the initial DL BWP does not to include SS/PBCH blocks or and the CORESET with index 0. If the UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type2-PDCCH CSS set, the UE assumes that the initial DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. if the UE used the SS/PBCH block to obtain SIB1 - includes a SS/PBCH block and does not include the CORESET with index 0 if the initial DL BWP does not include the SS/PBCH block the UE used to obtain SIB1 For an active DL BWP provided by BWP-DownlinkDedicated, a UE assumes that the active DL BWP includes a SS/PBCH block, unless the UE indicates a capability to operate in the DL BWP without receiving an SS/PBCH block, and does not include the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for separate initial DL BWP configured by BWP configuration option 1.] For an active DL BWP not provided by BWP-DownlinkDedicated, unless if a UE does not indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block or if a UE monitors PDCCH according to Type2-PDCCH CSS set, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for non-initial DL BWP configured by BWP configuration option 1 and initial/non-initial DL BWP configured by BWP configuration option 2.] For an active DL BWP provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 or the SS/PBCH blocks provided by NonCellDefiningSSB. If the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1, for SS/PBCH block and CORESET multiplexing pattern 1, the UE expects the active DL BWP to include the CORESET with index 0. If the active DL BWP includes the SS/PBCH blocks provided by NonCellDefiningSSB, these SS/PBCH blocks and the SS/PBCH blocks that the UE used to obtain SIB1 have the same quasi-colocation properties, if they have the same index. [The following paragraph captures presence of SSB in all RRC states for both BWP configuration option 1 and BWP configuration option 2 when paging is monitored.] For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonSIB, a UE in RRC_IDLE state or in RRC_INACTIVE state or in RRC_CONNECTED state does not monitor PDCCH according to Type2-PDCCH CSS set if the initial DL BWP does not include the SS/PBCH blocks that the UE used to obtain SIB1 and the CORESET with index 0.. |
| FL6 | RAN1 made the following agreement in an online session on Thursday 25th August. The agreed TP was incorporated in a 38.213 CR in R1-2208247. Agreement: - The following TP for 38.213 clause 17.1 in principle. [The following paragraph captures presence of SSB in idle and inactive modes.] For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonRedCapSIB, if a UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to a Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, the UE assumes that does not expect the initial DL BWP does not to include SS/PBCH blocks or and the CORESET with index 0. If the UE in RRC_IDLE state or in RRC_INACTIVE state monitors PDCCH according to Type2-PDCCH CSS set, the UE assumes that the initial DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. if the UE used the SS/PBCH block to obtain SIB1 - includes a SS/PBCH block and does not include the CORESET with index 0 if the initial DL BWP does not include the SS/PBCH block the UE used to obtain SIB1 For an active DL BWP provided by BWP-DownlinkDedicated, a UE assumes that the active DL BWP includes a SS/PBCH block, unless the UE indicates a capability to operate in the DL BWP without receiving an SS/PBCH block, and does not include the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for separate initial DL BWP configured by BWP configuration option 1.] For an active DL BWP not provided by BWP-DownlinkDedicated, unless if a UE does not indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block or if a UE monitors PDCCH according to Type2-PDCCH CSS set, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0. [The following paragraph captures presence of SSB in connected mode for non-initial DL BWP configured by BWP configuration option 1 and initial/non-initial DL BWP configured by BWP configuration option 2.] For an active DL BWP provided by BWP-DownlinkDedicated, unless a UE indicates a capability to operate in the active DL BWP without receiving an SS/PBCH block, the UE in RRC_CONNECTED state assumes that the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1 or the SS/PBCH blocks provided by NonCellDefiningSSB. If the active DL BWP includes the SS/PBCH blocks that the UE used to obtain SIB1, for SS/PBCH block and CORESET multiplexing pattern 1, the UE expects the active DL BWP to include the CORESET with index 0. If the active DL BWP includes the SS/PBCH blocks provided by NonCellDefiningSSB. [The following paragraph captures presence of SSB in all RRC states for both BWP configuration option 1 and BWP configuration option 2 when paging is monitored.] For an initial DL BWP provided by initialDownlinkBWP-RedCap in DownlinkConfigCommonSIB, a UE in RRC_IDLE state or in RRC_INACTIVE state or in RRC_CONNECTED state does not monitor PDCCH according to Type2-PDCCH CSS set if the initial DL BWP does not include the SS/PBCH blocks that the UE used to obtain SIB1 and the CORESET with index 0.. |

## 2.2	Center frequency alignment in 38.213

As already mentioned, RAN1#109e discussed several TPs for 38.213 clause 17.1 that intended to capture earlier RAN1 agreements. Contributions [16 (issue 1), 17, 18] propose to adopt similar changes as TP#9 in the RAN1#109e FLS [5], which looked like this:

A RedCap UE does not expect to receive a configuration where the center frequency for an initial DL BWP in which the UE is configured to monitor Type1-PDCCH CSS set (separate or shared with non-RedCap UEs) is different than the center frequency for an initial UL BWP (separate or shared with non-RedCap UEs) in which the RedCap UE may transmit Msg1/Msg3 or MsgA.

FL1 Question 2.2-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 3 | It is the left issue in the last meeting. It is important for UE implementation. We also provide our TP in [12]. |
| Nordic | 3 | We support the TP |
| vivo | 3 | Center frequency alignment is important for RedCap UE to reduce the complexity. We think it is a high priority issue. |
| Intel | 3 | Agree with the above comments and we also see a need to address this gap. |
| Qualcomm | 3 | Suggest to add “single carrier operation on unpaired spectrum” as a condition for the TP above. |
| CATT | 3 | And clarify that this is for operation in unpaired spectrum. |
| ZTE, Sanechips |  | We are OK to discuss this issue. However, if the initial DL/UL BWP has the same enter frequency, RF retuning between initial DL/UL BWP can be avoided and there is no need to further mandate the same center frequency for Type1-PDCCH CSS set and initial UL BWP. Moreover, if Type1-PDCCH CSS set and initial UL BWP has the same center frequency, what about the Type2-PDCCH CSS set? Shall we also need to mandate the center frequency with initial UL BWP? This kind of further restriction for Type1-PDCCH and/or Type2-PDCCH would cause scheduling complexity for gNB and UE receiving performance due to restricted position. |
| Sharp | 3 | Also agree with Qualcomm and CATT’s suggestion. |
| Samsung | 3 | Need to clarify this for TDD only (unpaired spectrum.). |
| FUTUREWEI | 3 | Need to clarify that this is for TDD |
| CMCC | 3 | Share similar view with CATT, Sharp, Samsung, and FUTUREWEI that this should be for TDD. |
| Nokia, NSB | 3 | Need to clarify that this is for TDD |
| Ericsson | 3 |  |
| NEC | 3 | Support the TP and agree with Qualcomm and CATT. |
| OPPO | 3 |  |
| Huawei, HiSilicon | 3 | We generally agree with the intention but unclear whether it is a good wording to write “shared with non-RedCap UEs” since a UE does not care about whether a BWP is shared with others or not, who just follow the configured BWP index. The description of “type 1 CSS” is also complicated. It would be simpler to just say the BWP provided in IE xxx. |
| FL2 | Based on received responses, the following proposal can be considered. High Priority Proposal 2.2-1a: Agree following TP for 38.213 clause 17.1 in principle. For unpaired spectrum operation, a RedCap UE does not expect to receive a configuration where the center frequency for an initial DL BWP in which the UE is configured to monitor Type1-PDCCH CSS set is different than the center frequency for an initial UL BWP in which the RedCap UE may transmit Msg1/Msg3 or MsgA.. |
| FL3 | RAN1 made the following agreement on Tuesday 23rd August: Agreement: The following TP for 38.213 clause 17.1 is endorsed in principle. For unpaired spectrum operation, a RedCap UE does not expect to receive a configuration where the center frequency for an initial DL BWP in which the UE is configured to monitor Type1-PDCCH CSS set is different than the center frequency for an initial UL BWP in which the RedCap UE may transmit Msg1/Msg3 or MsgA.. |

## 2.3	Maximum bandwidth in 38.213

Contributions [16 (issue 5), 45] propose some clarifications related to the maximum bandwidth in 38.213 clause 17.1.

FL1 Question 2.3-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 1 | It seems complete that the maximum bandwidth of any BWP for RedCap UEs is no wider than the maximum RedCap bandwidth. |
| Nordic | 1/2 | [16] We do not see issue with differentiating DL and UL BW, even if in R17 the limit is the same for both DL and UL. [45] We OK with clarification, but it is not of highest priority |
| vivo | 2 | The correction is the common understanding, should be quickly converged during the meeting. |
| Intel | 1/2 | Same view as Nordic. |
| Qualcomm | 1 |  |
| CATT | 1 |  |
| ZTE, Sanechips | 2 | Not a necessary correction but can make it clearer. We are OK to discuss with a relatively lower priority. |
| Sharp | 2 | Agree with the proposed clarifications. |
| Samsung | 1 |  |
| FUTUREWEI | 1 | For [45], ok with clarification. A reason to differentiate DL and UL BW is based on clause 5.3.6 “Asymmetric channel bandwidths” in 38.101-1 where a FDD band can have different sized UL and DL BWs. |
| CMCC | 1 | Both DL and UL share the same maximum RedCap bandwidth, so this issue seems not critical. We are also ok to correct this. |
| Nokia, NSB | 1 |  |
| Ericsson | 1 |  |
| NEC | 1 |  |
| OPPO | 1 |  |
| Huawei, HiSilicon | 3 | The corrections are straightforward. |

## 2.4	Common PUCCH resource set determination in 38.213

Contributions [31, 44] propose to clarify the common PUCCH resource set index determination in 38.213 clause 17.1 and to send an LS to ask RAN2 to clarify in 38.331 that RedCap-specific common PUCCH resource is always provided for a RedCap-specific initial UL BWP.

Contributions [36 (section 4), 41] propose a correction of the PUCCH PRB offset parameter name in 38.213 clause 17.1.

FL1 Question 2.4-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Nordic | 2 | While rather obvious that if configured with RedCap PUCCH UE will not use non-RedCap PUCCH, but could be specified |
| vivo | 2 | We think it is a high priority issue. |
| Intel | 2 | OK to discuss |
| Qualcomm | 3 | Considering the potential impacts on UE implementation, we think it should be treated as high priority at this meeting. |
| CATT | 2 | Fine to have a clear conclusion. |
| ZTE, Sanechips | 2 | We are OK to discuss, and also OK to wait for RAN2 further discussion. |
| Sharp | 3 | We support to figure out the common PUCCH resource set index determination in the initial UL BWP shared with non-RedCap UEs. |
| FUTUREWEI | 2 | Ok to discuss |
| CMCC | 2 | Ok to discuss. |
| Nokia, NSB | 2 |  |
| Ericsson | 2 |  |
| NEC | 2 |  |
| OPPO | 2 |  |
| Huawei, HiSilicon | 3 | It is natural to be clarified. |
| FL2 | Based on received responses, the following proposal can be considered. Medium Priority Proposal 2.4-1a: - Agree the draft CR for 38.213 clause 17.1 in R1-2207000 in principle. - Also update the the RRC parameter name pucch-ResourceConfig-RedCap to additionalPRBOffset in 38.213. - Discuss whether to send an LS to RAN2 to clarify that pucch-ResourceCommon-RedCap-r17 is always provided in a RedCap-specific initial UL BWP and to ask RAN2 to clarify this in 38.331 as proposed in R1-2207494. |  |
| FL3 | The proposal was discussed in an online session on Tuesday 23rd August. Medium Priority Question 2.4-1b: Companies are invited to comment further on the following proposal and propose potential resolutions in the Comments field. - Agree the draft CR for 38.213 clause 17.1 in R1-2207000 in principle. - Also update the the RRC parameter name pucch-ResourceConfig-RedCap to additionalPRBOffset in 38.213. - Discuss whether to send an LS to RAN2 to clarify that pucch-ResourceCommon-RedCap-r17 is always provided in a RedCap-specific initial UL BWP and to ask RAN2 to clarify this in 38.331 as proposed in R1-2207494. |  |
| Qualcomm | Y |  |
| vivo | We are fine with the proposal and agree to send the LS to RAN2. |  |
| ZTE, Sanechips | - We are OK to update the parameter name as additionalPRBOffset in 38.213. - For the CR R1-2207000, we do not need to determine whether the UE need to read pucch-ResourceCommon if pucch-ResourceCommon-RedCap is absent, which should be decided by RAN2. - For the LS regarding pucch-ResourceCommon-RedCap-r17 is always provided in a RedCap-specific initial UL BWP, we think in RAN1, we can make the decision like, e.g., PUCCH resource should be configured for RedCap in separate initial UL BWP or in shared initial UL BWP As for how to configure the PUCCH resource, e.g., based on pucch-ResourceCommon or pucch-ResourceCommon-RedCap, it depends on RAN2 discussion. So, our suggestion would be try to agree the following proposal and send the LS to RAN2 inform them the proposal Proposal: PUCCH resource should be configured for RedCap in separate initial UL BWP or in shared initial UL BWP |  |
| Sharp | Y |  |
| Nordic | Y |  |
| CATT | Y |  |
| Nokia, NSB | Y |  |
| Intel | Y |  |
| CMCC | Y |  |
| NEC | Y |  |
| Ericsson | Yes, we are fine with the draft CR, the parameter name correction, and sending an LS to RAN2. |  |
| FL4 FL5 | Based on the responses from companies, the following proposal can be considered: High Priority Proposal 2.4-1c: - Agree the draft CR for 38.213 clause 17.1 in R1-2207000 in principle. - Also update the the RRC parameter name pucch-ResourceConfig-RedCap to additionalPRBOffset in 38.213. - Send an LS to RAN2 to clarify that pucch-ResourceCommon-RedCap-r17 is always provided in a RedCap-specific initial UL BWP and to ask RAN2 to clarify this in 38.331 as proposed in R1-2207494. |  |
| FL6 | RAN1 made the following agreement in an online session on Thursday 25th August. The agreed TP was incorporated in a 38.213 CR in R1-2208247 and the LS was sent in R1-2208208. Agreement: - The draft CR for 38.213 clause 17.1 in R1-2207000 is endorsed in principle. - Also update the the RRC parameter name pucch-ResourceConfig-RedCap to additionalPRBOffset in 38.213. - Send an LS to RAN2 to clarify that pucch-ResourceCommon-RedCap-r17 is always provided in a RedCap-specific initial UL BWP and to ask RAN2 to clarify this in 38.331 as proposed in R1-2207494. |  |

## 2.5	Relation between PUSCH and NCD-SSB in 38.213/38.214

Contributions [21, 22, 32 (section 2.3), 34, 40] propose to clarify the relation between PUSCH and NCD-SSB in various subclauses to 38.214 clause 6.1, whereas contribution [39] proposes to clarify this in 38.213 clause 17.1.

FL1 Question 2.5-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 1 | The actually transmitted SSB pattern is not provided in IE NonCellDefiningSSB. Maybe it should follow ssb-PositionsInBurst for CD-SSB or have a separate ssb-PositionsInBurst. It needs the further discussion. |
| Nordic |  | Agree with Spreadtrum, the TPs are technically wrong. Moreover, ssb-PositionsInBurst should be the same for CD and NCD SSB. So there is no issue with using ssb-PositionsInBurst in SIB1 for NCD SSB as well |
| vivo | 3 | It is necessary to clarify that the SSB should include NCD-SSB. About correction proposed in [39], we are not sure the necessity of this statement given all collision handling between SSB and UL transmissions for RedCap HD-FDD are captured in clause 17.2. In addition, we are not sure such correct is accurate, since for TDD, there is no collision between valid PRACH and SSB; but for HD-FDD, the collision can happen, and UE behavior is left to UE implementation. |
| Intel | 3 | SSB provided by NonCellDefiningSSB is already referenced in current 213 specifications and that can be followed. A reference to NonCellDefiningSSB is necessary even if ssb-PositionsInBurst may be common between CD- and NCD-SSB due to potential different periodicities and offsets. |
| Qualcomm | 2 | If clarification is needed, we suggest to add a general description in 213 and/or 214 spec, instead of pursuing brute-force changes in various clauses of multiple specs. For example, the following description can be added to 213 spec: - If an active DL BWP of RedCap UE includes the SS/PBCH blocks configured by NonCellDefiningSSB of the serving cell, the UE assumes the SS/PBCH blocks transmitted within a NCD-SSB burst is indicated by ssb-PositionsInBurst in SIB1, and the SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1 and transmitted within the active DL BWP refer to the SS/PBCH blocks configured by NonCellDefiningSSB. |
| CATT | 2 | Agree with Spreadtrum and vivo. We are open to make it more clear, but just to remind that, RAN2 already made the following definition for nonCellDefiningSSB-r17 in 331: If configured, the RedCap UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the cell-defining SSB of the serving cell (e.g. obtaining sync, measurements, RLM,…). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the “SSB” configured in the QCL-Info IE; the “ssb-Index” configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitily to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.. |
| ZTE, Sanechips | 3 | NCD-SSB related issues need to be addressed together. If only ssb-PositionsInBurst is used in the spec, there is no need to differentiate the CD-SSB and NCD-SSB, since RRC specific SSB and cell specific SSB can all refer to the ssb-PositionsInBurst. However, if we need to differentiate the SSB types or symbols from different SSB type, the SIB indication or cell specific indication or RRC indication needs to be differentiated. |
| Sharp | 3 | Agree with Intel. The ssb-PositionsInBurst in SIB1 cannot cover the NCD-SSB considering different time offsets. |
| Samsung | 2 | Might need some clarification. It seems companies have different understanding. |
| CMCC | 2 | This issue can be clarified, and how to capture this can be discussed, e.g.by separate clarification in the TS or modify each related section. |
| Nokia, NSB | 2 | Agree with ZTE, all related NCD-SSB issues should be discussed together. |
| Ericsson | 2 |  |
| NEC | 2 | Agree with Spreadtrum. We prefer a TP into 17.1, if needed. Qualcomm’s TP seems OK. |
| OPPO | 2 | We also think NCD-SSB issue to be treated together. |
| Huawei, HiSilicon | 3 | The issue is straightforward but the change can be simpler if made in RAN2 RRC. |
| FL2 | Based on received responses, the following proposal can be considered. Medium Priority Proposal 2.5-1a: For the relation between PUSCH and NCD-SSB for RedCap UEs, agree the TP for 38.213 clause 17.1 in R1-2207274. |  |
| FL3 | The proposal was discussed in an online session on Tuesday 23rd August. Medium Priority Question 2.5-1b: Companies are invited to comment further on the following proposal and propose potential resolutions in the Comments field. - For the relation between PUSCH and NCD-SSB for RedCap UEs, agree the TP for 38.213 clause 17.1 in R1-2207274. |  |
| Qualcomm | We support this TP in principle, but UL transmission include not only PUSCH, but also PRACH, PUCCH and SRS. Therefore, DL/UL collision handling in TDD should cover UL slots/symbols indicated by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated. Another non-trivial issue related to collision handling procedure of RedCap UE is the RO validation in RRC connected state, when a RedCap UE is configured with NCD-SSB and PRACH resources in RRC-configured UL and DL BWP, respectively. It is important to clarify which SSB is to be used for RO validation. Basically, a RedCap UE should not consider both CD-SSB and NCD-SSB for RO validation, if a non-zero time offset exists. Therefore, we propose to revise the TP for Clause 17.1 of 38.213 as follows: - Procedures for a RedCap UE are same as described for a UE in all other clauses of this document unless stated otherwise. For a RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB in unpaired spectrum, collision handling between uplink transmissions and the SS/PBCH blocks are same as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses. - For unpaired spectrum, a RedCap UE does not expect the set of symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated to overlap with the set of symbols indicated presence of SS/PBCH blocks by NonCellDefiningSSB within an active DL BWP. - For unpaired spectrum, if a RedCap UE is configured with PRACH resources in an active UL BWP, which are associated with SS/PBCH blocks indicated by NonCellDefiningSSB in an active DL BWP, a valid PRACH occasion for RedCap UE does not precede a SS/PBCH block indicated by NonCellDefiningSSB in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block symbol indicated by NonCellDefiningSSB, where Ngap is provided in Clause 8.1 of TS 38.213. |  |
| vivo | About the correction of R1-2207274, we are not sure the necessity of this statement given all collision handling between SSB and UL transmissions for RedCap HD-FDD are captured in clause 17.2. In addition, we are not sure such correct is accurate, since for TDD, there is no collision between valid PRACH and SSB; but for HD-FDD, the collision can happen, and UE behavior is left to UE implementation. |  |
| ZTE, Sanechips | No need to have the general text here, since we already have the specific text proposal for the collision handling cases related to NCD-SSB. The general text here is redundant and may cause some forward compatibility issues. |  |
| Sharp | We support the TP and the additions proposed from Qualcomm. Regarding vivo’s comments, the correction of R1-2207274 is intended for collision handling in TDD case. The correction has nothing to do with HD-FDD, because the TP is intended for Clause 17.1. |  |
| Nordic | Would be covered by A RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB, handles SS/PBCH block as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses. |  |
| CATT | We have similar view as vivo and ZTE, and feel hesitant to support Qualcomm’s modification. The collision rule in single cell TDD for CD-SSB mainly concerns that CD-SSB is ‘always-on’ in cell level, so the CD-SSB symbol in TDD the gNB will of course be DL (even if the BWP does not contain CD-SSB, this rule still holds). But NCD-SSB seems more close to CSI-RS in this regard and not at the same level with CD-SSB. |  |
| Intel | We’d prefer to capture the handling consistent with the current specs to keep the references similar to current specs. However, we can also accept a compact statement in Section 17.1 of that’s the majority view. |  |
| CMCC | Generally fine with the TP, we are not sure whether the additional two cases proposed by Qualcomm can also be covered by the first paragraph. |  |
| vivo2 | Based on Sharp’s explanation. We are fine with TP and the additions proposed from Qualcomm. |  |
| Ericsson | We are fine with the TP in general. We propose a minor update: For a RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB in unpaired spectrum, collision handling between uplink transmissions and the SS/PBCH blocks are same as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses, unless otherwise stated. |  |
| FL4 FL5 | Based on the comments from the previous round, the following proposal can be considered. The updates proposed by Qualcomm related to PRACH, PUCCH and SRS is posed as a new question in Question 2.5-2a. High Priority Proposal 2.5-1c: - For the relation between PUSCH and NCD-SSB for RedCap UEs, agree the TP for 38.213 clause 17.1 in R1-2207274 with the following change in green: For a RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB in unpaired spectrum, collision handling between uplink transmissions and the SS/PBCH blocks are same as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses, unless otherwise stated. |  |
| Company | Y/N | Comments |
| Nordic | Y, but | A RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB, handles SS/PBCH block as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses, unless otherwise stated We strongly disagree to treat NCD SSB is like CSI-RS. This is why we prefer to capture a general text to clarify this. Moreover, NCD-SSB is used for initial access RACH procedure. |
| FL6 | RAN1 made the following agreement in an online session on Thursday 25th August. The agreed TP was incorporated in a 38.213 CR in R1-2208247. Agreement: - For the relation between PUSCH and NCD-SSB for RedCap UEs, agree the TP for 38.213 clause 17.1 in R1-2207274 with the following change in green: For a RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB in unpaired spectrum, collision handling between uplink transmissions and the SS/PBCH blocks are same as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses, unless otherwise stated. |  |

Qualcomm in their response to Question 2.5-1b has proposed to clarify that DL/UL collision handling in TDD covers UL slots/symbols indicated by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated and to clarify which SSB (CD or NCD) is to be used for RO validation. The corresponding TP from Qualcomm is available below.

FL4/FL5 High Priority Question 2.5-2a: Companies are invited to comment on the following proposal and propose potential resolutions in the Comments field.

- Agree the following TP for 38.213 clause 17.1

| For unpaired spectrum, a RedCap UE does not expect the set of symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated to overlap with the set of symbols indicated presence of SS/PBCH blocks by NonCellDefiningSSB within an active DL BWP. For unpaired spectrum, if a RedCap UE is configured with PRACH resources in an active UL BWP, which are associated with SS/PBCH blocks indicated by NonCellDefiningSSB in an active DL BWP, a valid PRACH occasion for RedCap UE does not precede a SS/PBCH block indicated by NonCellDefiningSSB in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block symbol indicated by NonCellDefiningSSB, where Ngap is provided in Clause 8.1 of TS 38.213. |  |
| --- | --- |
| Company | Comments |
| Nordic | This aspects would be covered by having a general sentence that NCS SSB is handled the same way as CD SSB |

## 2.6	PDSCH resource mapping around NCD-SSB in 38.214

Contributions [16 (issue 2), 25, 40] propose to clarify PDSCH resource mapping around NCD-SSB in 38.214 clause 5.1.4.

FL1 Question 2.6-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 1 | Similar to FL1 Question 2.5-1 |
| Nordic | 1 | Agree with SPRD |
| Vivo | 3 | We think it is a high priority issue to define RedCap UE behavior for PDSCH resource mapping around NCD-SSB. |
| Intel | 2 | While we are OK to discuss the issue, in our understanding current description in 214 only refers to “ssb-PositionsInBurst” without reference to SIB1 or ServingCellConfigCommon. So, this depends on how ssb-PositionsInBurst is defined for NCD-SSB. |
| Qualcomm | 2 | Same view as our comments on FL1 Question 2.5-1 |
| CATT | 1 | Same as 2.5-1, is the following definition in 331 already enough? If configured, the RedCap UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the cell-defining SSB of the serving cell (e.g. obtaining sync, measurements, RLM,...). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitily to this NCD-SSB.. |
| ZTE, Sanechips |  | It can be discussed together with Question 2.5-1. |
| Sharp | 3 | Share same view with vivo. |
| Samsung | 1 | Agree with CATT |
| CMCC | 2 | This issue can be discussed. |
| Nokia, NSB | 2 |  |
| Ericsson | 2 |  |
| NEC | 2 | Prefer to handle this together with FL1 question 2.5-1. |
| OPPO | 1 |  |
| Huawei, HiSilicon | 3 | The statement is natural while we think there may be no need to further clarify since we do not expect different UE behavior. |
| FL2 | Based on received responses, the following proposal can be considered, where the TP is from [16 (issue 2)]. Medium Priority Proposal 2.6-1a: For the PDSCH resource mapping around NCD-SSB for RedCap UEs, consider adopting the following TP either for 38.213 clause 17.1 (‘RedCap UE procedures’) or 38.214 clause 5.1.4 (‘PDSCH resource mapping’). For the case of reduced capability UE configured with NonCellDefiningSSB, when receiving the PDSCH, the UE assumes SS/PBCH block transmission according to NonCellDefiningSSB, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.. |
| FL3 | The proposal was discussed in an online session on Tuesday 23rd August. Medium Priority Question 2.6-1b: Companies are invited to comment further on the following proposal and propose potential resolutions in the Comments field. - For the PDSCH resource mapping around NCD-SSB for RedCap UEs, consider adopting the following TP either for 38.213 clause 17.1 (‘RedCap UE procedures’) or 38.214 clause 5.1.4 (‘PDSCH resource mapping’). For the case of reduced capability UE configured with NonCellDefiningSSB, when receiving the PDSCH, the UE assumes SS/PBCH block transmission according to NonCellDefiningSSB, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.. |
| Qualcomm | Y |  |
| vivo | Support |  |
| ZTE, Sanechips | We actually think ssb-PositionsInBurst can refer to all kinds of SSBs, since NCD-SSB also has to use the ssb-PositionsInBurst. So, maybe we do not need to separately describe that and the TP is not needed. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted. For the case of reduced capability UE configured with NonCellDefiningSSB, when receiving the PDSCH, the UE assumes SS/PBCH block transmission according to NonCellDefiningSSB, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.. |
| Sharp | Support |  |
| Nordic | Would be covered by A RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB, handles SS/PBCH block as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses. |  |
| CATT | OK. |  |
| Nokia, NSB | OK |  |
| Intel | We agree with ZTE that “SS/PBCH block transmission according to ssb-PositionsInBurst” can already cover NCD-SSB for this case. So, TP is not needed. |  |
| CMCC | Agree with ZTE, the original sentence “the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst” does not limit the SSB to CD-SSB. |  |
| vivo2 | We agree with ZTE, it may not be needed given understanding that the ssb-PositionsInBurst” can cover NCD-SSB. |  |
| Huawei, HiSilicon | It is not needed to complicated RAN1 spec by specifically differentiating NCD-SSB from CD-SSB when both are already covered. The UE behavior for NCD is same as current spec which covers CD-SSB. |  |
| NEC | Agree with ZTE. |  |
| Ericsson | We tend to think that the TP may be needed considering that the mapping rule would apply only to UEs configured with NCD-SSB. We could also make the proposal more compact as below: A reduced capability UE assumes SS/PBCH block transmission according to NonCellDefiningSSB if configured, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. |  |
| FL4 | Majority of the received responses don’t see a need for this TP. Based on the received responses, the TP is not considered further in RAN1#110. |  |

## 2.7	Relation between control channels and NCD-SSB in 38.213

Contributions [16 (issue 4), 20, 22, 26, 32 (section 2.3), 33] propose to clarify the relations between various control channels and NCD-SSB in one or more of clauses 8.1, 8.1A, 9.2.6, 10, 11.1, 11.1.1 and 19.1 in 38.213.

Contribution [36 (section 5)] concerns the definition and values of the recently introduced NCD-SSB time offset parameter.

FL1 Question 2.7-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 1 | Similar to FL1 Question 2.5-1 |
| Nordic | 1 | Agree with SPRD |
| vivo | 3 | We think it is a high priority issue to define the RedCap UE behavior at least for PDCCH colliding NCD-SSB. |
| Intel | 3 | Share same view with vivo. |
| Qualcomm | 2 | Same view as our comments on FL1 Question 2.5-1 |
| CATT | 1 | Same as 2.5-1. Let’s see whether the definition of nonCellDefiningSSB-r17 in 331 is already enough. |
| ZTE, Sanechips | 3 | NCD-SSB related correction for adding NonCellDefiningSSB can be discussed together. |
| Sharp | 3 | Share same view with vivo. |
| Samsung | 1 |  |
| CMCC | 2 | This issue needs to be discussed. |
| Nokia, NSB | 2 |  |
| Ericsson | 2 |  |
| NEC | 2 | Prefer to handle this together with FL1 question 2.5-1. |
| OPPO | 2 |  |
| FL2 | Based on received responses, the following proposal can be considered, where the TP is from [16 (issue 4)]. Medium Priority Proposal 2.7-1a: For the relation between PDCCH and NCD-SSB for RedCap UEs, consider adopting the following TP either for 38.213 clause 17.1 (‘RedCap UE procedures’) or 38.214 clause 10 (‘UE procedure for receiving control information’). For monitoring of a PDCCH candidate by a reduced capability UE configured with NonCellDefiningSSB, if the UE - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by NonCellDefiningSSB, the UE is not required to monitor the PDCCH candidate.. |
| FL3 | The proposal was discussed in an online session on Tuesday 23rd August. Medium Priority Question 2.7-1b: Companies are invited to comment further on the following proposal and propose potential resolutions in the Comments field. - For the relation between PDCCH and NCD-SSB for RedCap UEs, consider adopting the following TP either for 38.213 clause 17.1 (‘RedCap UE procedures’) or 38.214 clause 10 (‘UE procedure for receiving control information’). For monitoring of a PDCCH candidate by a reduced capability UE configured with NonCellDefiningSSB, if the UE - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by NonCellDefiningSSB, the UE is not required to monitor the PDCCH candidate.. |
| Qualcomm | We are fine with the TP for PDCCH validation. Besides, it is necessary to clarify that a RedCap UE will NOT perform extra validation for NCD-SSB and PRACH occasion (configured by RRC) in TDD operation. To this end, we suggest the following TPs for Clause 17.1 of TS 38.213: - For unpaired spectrum, a RedCap UE does not expect the set of symbols indicated as uplink by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated to overlap with the set of symbols indicated presence of SS/PBCH blocks by NonCellDefiningSSB within an active DL BWP. - For unpaired spectrum, if a RedCap UE is configured with PRACH resources in an active UL BWP, which are associated with SS/PBCH blocks indicated by NonCellDefiningSSB in an active DL BWP, a valid PRACH occasion for RedCap UE does not precede a SS/PBCH block indicated by NonCellDefiningSSB in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block symbol indicated by NonCellDefiningSSB, where Ngap is provided in Clause 8.1 of TS 38.213. |  |
| vivo | Support |  |
| ZTE, Sanechips | We are thinking the forward compatibility issues, e.g., NR UE support the NCD-SSB, may also be considered. So, if we have the above modification, it is fine for RedCap, but not suitable for NR UE. Once NR UE supports the NCD-SSB, then the description would be quite complicated here. So, we think we can come back this spec change after we have the conclusion for NR UE supporting NCD-SSB issue. |  |
| Sharp | Support |  |
| Nordic | Would be covered by A RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB, handles SS/PBCH block as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses. |  |
| CATT | We are OK with FL’s proposal. |  |
| Nokia, NSB | Support |  |
| Intel | To better align with the presentation in the current specs, we prefer the following TP (from R1-2206549 [20]): ----start of changes (TS 38.213, v17.2.0) ---- For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon or NonCellDefiningSSB if provided for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1, the UE is not required to monitor the PDCCH candidate. For monitoring of a PDCCH candidate by a UE, if the UE - has received ssb-PositionsInBurst in ServingCellConfigCommon or NonCellDefiningSSB if provided for a serving cell, and - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon or NonCellDefiningSSB, the UE is not required to monitor the PDCCH candidate. ----end of changes (TS 38.213, v17.2.0) ---- In addition, we think the case of PUCCH repetitions need to be addressed as well. A TP is provided from [20] : ----start of changes (TS 38.213, v17.2.0) ---- ### 9.2.6 PUCCH repetition procedure A SS/PBCH block symbol is a symbol of an SS/PBCH block with candidate SS/PBCH block index corresponding to the SS/PBCH block index indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon or ssb-PositionsInBurst in NonCellDefiningSSB if provided or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states. ----end of changes (TS 38.213, v17.2.0) ---- |  |
| CMCC | Considering ZTE’s comment, if we delete “reduced capability” in this sentence, will it be forward compatible? For monitoring of a PDCCH candidate by a reduced capability UE configured with NonCellDefiningSSB |  |
| Huawei, HiSilicon | We think it is not necessary for revising everywhere related to NCD-SSB if current spec can already cover. |  |
| NEC | OK. |  |
| Ericsson | We support Intel’s proposal. |  |
| FL4 FL5 | There seems to be no fundamental concerns with the TP. However, concerns were raised with regards to applicability of NCD-SSB for non-RedCap UEs. Based on the received responses the following updated proposal can be considered: High Priority Proposal 2.7-1c: For the relation between PDCCH and NCD-SSB for RedCap UEs, adopt the following TP in 38.213 clause 17.1 (‘RedCap UE procedures’). For monitoring of a PDCCH candidate by a reduced capability UE configured with NonCellDefiningSSB, if the UE - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by NonCellDefiningSSB, the UE is not required to monitor the PDCCH candidate.. |
| FL6 | RAN1 made the following agreements in an online session on Thursday 25th August. The agreed TPs were incorporated in a 38.213 CR in R1-2208247. Agreement: For the relation between PDCCH and NCD-SSB for RedCap UEs, adopt the following TP in R17 38.213 clause 17.1 (‘RedCap UE procedures’). For monitoring of a PDCCH candidate by a reduced capability UE configured with NonCellDefiningSSB, if the UE - does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and - at least one RE for a PDCCH candidate overlaps with at least one RE of a candidate SS/PBCH block corresponding to a SS/PBCH block index provided by NonCellDefiningSSB, the UE is not required to monitor the PDCCH candidate.. Agreement: The following TP to TS 38.213, v17.2.0, clause 9.2.6 is endorsed in principle. ----start of changes (TS 38.213, v17.2.0) ---- ### 9.2.6 PUCCH repetition procedure A SS/PBCH block symbol is a symbol of an SS/PBCH block with candidate SS/PBCH block index corresponding to the SS/PBCH block index indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon or by NonCellDefiningSSB if provided or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states. ----end of changes (TS 38.213, v17.2.0) ---- |

## 2.8	DCI format 0_0 size determination in 38.212

Contribution [27] proposes to clarify the DCI format 0_0 size determination in 38.212 clause 7.3.1.0.

FL1 Question 2.8-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 1 | The initial UL BWP may include the BWP provide by initialUplinkBWP or initialUplinkBWP-RedCap |
| Nordic | 1 | We believe there is no ambiguity, as it is clear which initial UL BWP is used by the RedCap UE. |
| Vivo | 3 | For DCI format 1_0 size determination in CSS, we made one conclusion in RAN1#108-e that DCI format 1_0 size in CSS always depends on size of CORESET#0. But there is no discussion about DCI size determination for DCI format 0_0 in CSS before aligning its size to DCI format 1_0 monitored in CSS. It should be clarified in 38.212. |
| Intel | 1 | Same view as Spreadtrum and Nordic. |
| Qualcomm | 1 |  |
| CATT | 1 |  |
| ZTE, Sanechips | 1 | No need to be discussed. Initial UL BWP can refer to both initialUplinkBWP or initialUplinkBWP-RedCap |
| Sharp | 1 | Agree with Nordic. There should be no ambiguity on the initial UL BWP definition for RedCap Ues. As clarified in 38.331, if initialUplinkBWP-RedCap is present, RedCap Ues use the UL BWP instead of initialUplinkBWP. |
| Samsung | 1 |  |
| CMCC | 1 |  |
| Nokia, NSB | 1 |  |
| Ericsson | 1 |  |
| NEC | 1 |  |
| OPPO | 1 |  |

## 2.9	Msg1/MsgA retransmission timeline in 38.213

Contributions [42, 43] propose to make the text about the Msg1/MsgA retransmission timeline in 38.213 clauses 8.2 and 8.2A applicable to non-RedCap UEs only, whereas contribution [36 (section 2)] proposes to add corresponding text in 38.213 clause 17.1 for the case when a RedCap UE performs random access on an active DL BWP with SSB.

FL1 Question 2.9-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 2 | It is important for UE implementation, but companies may have time to check the actual time line. |
| Nordic | 3 | We should discuss whether timeline is extended for RedCap due to RF returning. |
| Vivo | 1 | As clarified, “the UE shall be ready to transmit a PRACH” does not mandate the UE must transmit. There are other cases that after the timeline, the UE cannot transmit the PRACH e.g., no valid/available RO or collision happens between RO and DL receptions. Neither correction from [42] or [36] can solve the ‘problem’ in case the active/separate initial BWP without SSB. If it is really issue, the legacy UE supporting FG6-1a also have timeline problem in case the active BWP does not contain SSB? |
| Intel | 1 | Share same view as vivo – this aspect was discussed earlier during the WI and clarified as explained by vivo. |
| Qualcomm | 3 | The R16 CR on msg1/msg3 retransmission was discussed in RAN1#109 meeting. The FL of the R16 CR (Lihui, vivo) has clarified the R16 CR applies to non-RedCap UE only. Due to the potential impacts on RedCap UE timeline/implementation, we think it is a high priority issue. As proposed in our Tdoc, a minor change to 213 spec (adding “if SSB is present in the initial DL BWP of RedCap UE” to the relevant descriptions in Clause 8.2/8.2A, or in Clause 17.1) should be able to fix the issue. |
| CATT | 1 |  |
| ZTE, Sanechips | 1 | It is up to UE implementation in different cases, no need to discuss it again. |
| Samsung | 2 | Open to have some clarification. |
| FUTUREWEI | 1 | This aspect was discussed earlier in the WI |
| CMCC | 1 | Share similar view as vivo. |
| Nokia, NSB | 1 | Share similar view as vivo. |
| Ericsson | 2 |  |
| OPPO | 1 |  |
| Huawei. HiSilicon |  | We think the current specification is even less requiring any timeline for UE. |

# 3	HD-FDD operation

## 3.1	PUSCH repetition corrections in 38.214

Contributions [13 (section 3), 16 (issue 3), 19, 28, 29, 37, 38] propose various PUSCH repetition related corrections for HD-FDD in subclauses to 38.214 clause 6.1.2.

FL1 Question 3.1-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 3 | Based on the agreements for Type B repetition in the last meeting, CR is needed. |
| Nordic | 3 |  |
| vivo | 3 | The agreements made for HD-FDD collision handling and/or insufficient switching time for PUSCH repetition Type A with/without enabling valid slot counting and PUSCH repetition Type B should be correctly captured in the spec |
| Intel | 3 |  |
| Qualcomm | 3 |  |
| CATT | 3 |  |
| ZTE | 3 |  |
| Sharp | 3 |  |
| Samsung | 3 |  |
| CMCC | 3 |  |
| Nokia, NSB | 3 |  |
| Ericsson | 3 |  |
| OPPO | 3 |  |
| FL2 | Based on received responses, the following proposal can be considered. High Priority Proposal 3.1-1a: - Agree the draft 38.214 CR on PUSCH repetition type A for HD-UE in R1-2207272 in principle. - Agree the draft 38.214 CR on PUSCH repetition type B for HD-UE in R1-2207273 in principle. |  |
| FL3 | The proposal was discussed in an online session on Tuesday 23rd August. High Priority Question 3.1-1b: Companies are invited to comment further on the following proposal and propose potential resolutions in the Comments field. - Agree the draft 38.214 CR on PUSCH repetition type A for HD-UE in R1-2207272 in principle. - Agree the draft 38.214 CR on PUSCH repetition type B for HD-UE in R1-2207273 in principle. |  |
| Qualcomm | Y |  |
| vivo | For R1-2207272, fine in principle, with deleting the or by NonCellDefiningSSB since there is no parent IE mentioned in the text e.g. ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon in order to cover SSB-MTC-AdditionalPCI (for m-TRP). For R1-2207273, we would like to ask company check the similar correction in R1-2206751, which is preferred for its simplicity. |  |
| ZTE, Sanechips | Agree with vivo’s clarification. ssb-PositionsInBurst can refer to NCD-SSB, since NCD-SSB has the same prosperity and ssb-PositionsInBurst is not a IE or field of NonCellDefiningSSB . If we need to differentiate the types of SSB, then ‘in SIB1’or ‘in ServingCellConfigCommon’ or ‘NonCellDefiningSSB ’ should be differentiated. So,we suggest to have a simple way as follows: - For the case of a reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 when AvailableSlotCounting is enabled and K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or by NonCellDefiningSSB or if the symbols indicated by the indexed row of the used resource allocation table in the slot would not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or by NonCellDefiningSSB. And - For a reduced capability half-duplex UE in paired spectrum and for PUSCH repetition Type B transmission, symbols indicated by ssb-PositionsInBurst in SIB1, ssb-PositionsInBurst in ServingCellConfigCommon or by NonCellDefiningSSB for reception of SS/PBCH blocks within the active DL BWP are considered as invalid symbols for PUSCH repetition Type B transmission, and symbol(s) starting earlier or ending larter than NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of a set of symbols indicated by ssb-PositionsInBurst in SIB1, ssb-PositionsInBurst in ServingCellConfigCommon or by NonCellDefiningSSB for reception of SS/PBCH blocks within the active DL BWP are considered as invalid symbols for PUSCH repetition Type B transmission. |  |
| Sharp | - We agree with vivo’s comment, that is, an SS/PBCH block with index provided by ssb-PositionsInBurst can cover CD-SSB and NCD-SSB. So, we suggest removing ‘or by NonCellDefiningSSB’ as below. Otherwise, we have to add ‘in SIB or in ServingCellConfigCommon’ right in front of ‘or by NonCellDefiningSSB’. Therefore, we support the draft CR in R1-2207272 with removing ‘or by NonCellDefiningSSB’. with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or by NonCellDefiningSSB or if the symbols indicated by the indexed row of the used resource allocation table in the slot would not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or by NonCellDefiningSSB - We support the draft CR in R1-2207273. |  |
| Nordic | This CR should be also based on assumption “A RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB, handles SS/PBCH block as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses.” |  |
| CATT | Agree with vivo’s view on the first bullet. No strong view between 7273 and 6751. |  |
| Intel | Same view as vivo and CATT on PUSCH type A repetitions that NCD-SSB is already covered. For PUSCH type B, we would prefer either the version in 6751 or 6548 as they are more compact. |  |
| CMCC | For the first TP, support the suggestion to remove “or by NonCellDefiningSSB’. For the second TP, we are fine. |  |
| Ericsson | We are fine with both draft CRs. However, the draft CR on PUSCH repetition type A could be improved. Current spec already captures PUSCH overlapping with SSB as unavailable slot. The TP in R1-2207272 adds new agreement about Tx/Rx switching time. However, as the new sentence fully covers the old sentence about PUSCH overlapping with SSB, the original one can be removed. Intel submitted a similar CR for TBoMS, R1-2206555, where the new sentence replaces the old one. - For the case of a reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 when AvailableSlotCounting is enabled and K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or by NonCellDefiningSSB or if the symbols indicated by the indexed row of the used resource allocation table in the slot would not start or end at least NRx-Tx⋅Tc or NTx-Rx⋅Tc, respectively, from the last or first symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or by NonCellDefiningSSB. |  |
| FL4 | FL’s suggestion is to consider this proposal based on the outcome of Proposal 2.5-1c. |  |

## 3.2	PUSCH repetition corrections in 38.213

Contribution [13 (section 2)] proposes PUSCH repetition related corrections for HD-FDD in 38.213 clause 17.2.

FL1 Question 3.2-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 1 | In our understanding, 214 describes the available slots for Msg3 repetition, while 213 describes the UE behavior, i.e., whether to drop a transmission in an available slot if collision happens (overlapped with SSB). It seems no conflicts… |
| vivo | 1 | We think the correction is not needed. Aavailable slot determination and whether to perform the transmission on the determined available slot is different thing. There are no conflict behaviours for HD-FDD UE for Msg.3 repetitions. |
| Intel | 1 | No conflicts as explained by Spreadtrum and vivo. |
| Qualcomm | 1 |  |
| CATT | 1 | In R17 CE topic, it is agreed that for Msg3 repetition in HD-FDD, the available slot determination is the same as FD-FDD. Msg3 repetition in available slots will be dropped due to SSB collision (decision made in RedCap topic). This makes Msg3 repetition different from other PUSCH repetition. Current spec should be correct. |
| ZTE | 1 |  |
| Sharp | 1 | We do not see the inconsistence given that 38.214 just describes the consecutive slots are applied to for slot determination, not about the actual transmission. |
| Samsung | 1 | Share a view from other companies that available slot determination and transmission itself are separately specified. |
| CMCC | 1 | Share similar view with CATT. |
| Nokia, NSB | 1 |  |
| Ericsson | 1 |  |
| OPPO | 3 | That is not only inconsistent issue as CATT mention. We would like to let companies consider the situation that gNB have to unnecessarily try to blind detect HD-FDD UE as this will not be earlier identified through PRACH. FD-FDD UE: Transmit all N*K slots for msg3 PUSCH. HD-FDD UE: Drop some of SSB overlapped slot among N*K. gNB supporting HD-FDD should at least try to decoded based on 2 different assumptions of actually transmitted slot. This dropping is unnecessarily applied to RAR msg3 PUSCH of HD-FDD UE. When it msg3 transmission, it doesn’t need to measure SSB. We don’t think the earlier agreement intended for msg3. |

## 3.3	UE processing capability clarification in 38.213

Contribution [23] proposes clarifications related to UE processing capability for HD-FDD in 38.213 clause 17.2.

FL1 Question 3.3-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Nordic | 1 | It does not make sense to support faster capability 2 for RedCap UE. |
| Vivo | 1 | For HD-FDD RedCap UE, it may not be necessary or essential to support UE processing capability 2. |
| Intel | 1 | Not essential. |
| CATT | 1 | This seems technically right since we did not agree to always use UE capability 1 to determine the available/invalid slot or symbols. Having said this, we also feel that RedCap UE is unlikely to support capability 2, especially for a HD-FDD UE. |
| ZTE | 2 | If UE processing capability 2 is supported for RedCap UE, the CR is necessary. If UE processing capability 2 is not supported, then as mentioned by other companies, there is no need to have this kind of correction. Therefore, we may need to determine whether UE processing capability 2 is supported for RedCap firstly. |
| Samsung | 1 | Seems not essential. |
| CMCC | 1 | Share the same view as above companies. If the common understanding is no support of UE processing capability 2, a clarification can be made, then no need for such modification. |
| Nokia, NSB | 1 | Similar view as ZTE. |
| Ericsson | 1 |  |
| OPPO | 1 |  |

# 4	SDT operation

Contribution [12] contains several proposals related to small data transmission (SDT) operation for RedCap UEs. Contribution [32 (section 2.1)] proposes that it should be up to the UE implementation whether and how a UE monitors SI change indication during an SDT procedure in a separate initial DL BWP not containing CD-SSB. The FL suggestion is to postpone these proposals for the combination of RedCap and SDT until the RedCap specifications on one hand and the SDT specifications on the other hand are a bit more stable.

FL1 Question 4-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 3 | It is important for clarification since RedCap+SDT is not complete in the spec. Moderator’s suggestion is also reasonable. |
| Nordic | 3 | We fine with proposal |
| vivo | 1 | We are fine with FL suggestion to wait when the specification for RedCap and SDT are more stable. |
| Intel | 1 | Support recommendation from the FL. |
| Qualcomm |  | Agree with FL proposal |
| CATT | 1 | Agree with FL. |
| ZTE, Sanechips | 3 | Based on current agreement for SDT and RedCap, further clarification is needed in this meeting, since SDT is supported for RedCap UE and SDT also already supports separate initial BWP for BWP, and our discussion can fascinate the discussion for SDT also. |
| Samsung | 1 | Agree with FL. |
| FUTUREWEI | 1 | Ok with FL proposal |
| CMCC | 1 | Fine with FL suggestion. |
| Nokia, NSB | 1 | Agree with FL. |
| Ericsson | 1 | Agree with FL. |
| NEC | 1 | Agree with FL. |
| OPPO | 1 |  |
| Huawei, HiSilicon | 3 | The issue is also raised in SDT from our side. We are fine to discuss it either side but prefer not to delay further. |

# 5	SSB-less BWP

## 5.1	Measurements gaps

Contribution [36 (section 6)] proposes to update 38.213 and 38.822 to capture a RedCap UE’s need for measurement gaps to use SSB outside its BWP based on a potential LS reply from RAN4.

FL1 Question 5.1-1: Companies are invited to provide comments and suggested priority (1=Low, 2=Med, 3=High).

| Company | Priority | Comments |
| --- | --- | --- |
| Spreadtrum | 1 | It seems to be resolved in the LS reply for BWP operation without restriction. There are several options, like CSI-RS based measurement and measurement gap. Whether to introduce measurement gap needs to be discussed and concluded by RAN1/RAN2/RAN4, and for now RAN1 only needs to reply the LS. |
| Vivo | 1 | We are fine with the proposal. But we think the related spec impact should be in RAN4 given the following conclusion we made: Conclusion: - From RAN1 perspective, whether and under what conditions a RedCap UE requires to be configured with existing measurement gaps to support operation without SSB in an RRC-configured active BWP, and its related UE feature discussion (including measurement gaps) is up to RAN4. - Send an LS to RAN4 to inform them about the conclusion. |
| Intel | 1 | Same view as vivo. We already agreed to leave this up to RAN4. |
| CATT | 1 | Agree with vivo and Intel. |
| ZTE, Sanechips | 1 | Better to wait for the current discussion of BWP operation without restriction, and then consider whether we need to modify the corresponding conclusion or just reuse it. |
| CMCC | 1 | Better to leave it to RAN4. |
| Nokia, NSB | 1 | Agree with Vivo. |
| Ericsson | 1 |  |
| OPPO | 1 |  |
| Huawei, HiSilicon | 1 | Better to wait for real RAN4 LS not potential LS. |

## 5.2	CSI-RS based RLM

Contribution [15] proposes to include capability of CSI-RS based RLM (FG 1-7) into FG 28-1a and to reuse existing specifications for RLM on PCell. The FL suggests treating this topic under UE feature list agenda item 8.16.5 instead.

# 6	LS response on NCD-SSB time offset parameter

RAN1 and RAN4 have received an LS from RAN2 in [46] with the following overall description and actions:

1. Overall Description: RAN2 would like to thank RAN1 and RAN4 for their reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times. RAN2 agreed to specify the offset with the following value range {sf5, sf10, sf15, spare5, spare4, spare3, spare2, spare1} and the definition below: ssb-TimeOffset The time offset between CD-SSB of the serving cell and this Non-Cell Defining SSB. Value sf5 means the first burst of Non-Cell Defining SSB is transmitted 5ms later than the first burst of CD-SSB transmitted after the first symbol of SFN=0 of the serving cell, value sf10 means the first burst of Non-Cell Defining SSB is transmitted 10ms later than the first burst of CD-SSB transmitted after the first symbol in SFN=0 of the serving cell, and so on. If the field is absent, RedCap UE considers that the time offset between the first burst of CD-SSB transmitted in the serving cell and the first burst of this Non-Cell Defining SSB transmitted is zero.. RAN2 would like to ask RAN1 and RAN4 to take the above into consideration and provide feedback on the values, i.e., confirm and/or indicate whether additional values are needed. 2. Actions: To RAN1 and RAN4 ACTION: RAN2 kindly asks RAN1 and RAN4 to take the above into consideration and provide feedback on the values, i.e., confirm and/or indicate whether additional values are needed.

Contribution [47] proposes to add values {sf20, sf40, sf60}, whereas contribution [51] questions the necessity of value sf15, and contributions [48, 49, 50, 52, 53] express that the current values {sf5, sf10, sf15} are sufficient from RAN1 perspective.

The ongoing RAN4 meeting has already made the following agreement:

- For NCD-SSB time offset, add the additional MGRP values of 20ms and 40ms, and further discuss whether and what other values are needed.

FL2 High Priority Proposal 6-1: Reply to RAN2 that the current NCD-SSB time offset values {sf5, sf10, sf15} are sufficient from RAN1 perspective.

| Company | Y/N | Comments |
| --- | --- | --- |
| FL3 | RAN1 made the following agreement on Tuesday 23rd August: Agreement: RAN1 understands RAN4 has defined 20 and 40 ms periodicity, and RAN1 think that the NCD-SSB time offset values {sf5, sf10, sf15} are sufficient from RAN1 perspective, and {sf20, sf40} are also feasible. High Priority Proposal 6-1a: Agree the draft LS in RedCapDraftLs-v000.docx. |  |
| Qualcomm |  | In the LS to RAN2, it is important to include zero time offset between CD-SSB and NCD-SSB as a feasible option. |
| vivo | Y |  |
| ZTE, Sanechips | With updates | We would suggest the following updates: RAN1 has discussed the question in the LS and would like to provide the following answer reply: RAN1 understands RAN4 has defined 20 and 40 ms periodicity offset, and RAN1 thinks that the NCD-SSB time offset values {sf5, sf10, sf15} are sufficient from RAN1 perspective, and {sf20, sf40} are also feasible. |
| Nordic | Y | With fixing typos, I hope it is also common understanding that offset should be configured smaller than periodicity |
| CATT | Y |  |
| Intel | Y |  |
| CMCC | Y |  |
| NEC | Y |  |
| Ericsson | Y | Agree with ZTE’s updates |
| FL4 | The draft LS in R1-2207979was endorsed and the final LS in R1-2207980 approved in an online session on Thursday 25th August. |  |

# References

| [1] | RP-220966 | Revised WID on support of reduced capability NR devices | Ericsson |
| --- | --- | --- | --- |
| [2] | R1-221163 | Summary of WI on support of reduced capability (RedCap) NR devices | Ericsson |
| [3] | R1-2205427 | RAN1 agreements for Rel-17 NR RedCap | Rapporteur (Ericsson) |
| [4] | R1-2205107 | FL summary for preparatory phase for Rel-17 RedCap maintenance | Moderator (Ericsson) |
| [5] | R1-2205428 | FL summary for maintenance on UE bandwidth reduction for RedCap | Moderator (Ericsson) |
| [6] | R1-2205429 | FL summary for incoming LS (R1-2203046) on introduction of an offset to transmit CD-SSB and NCD-SSB at different times | Moderator (Ericsson) |
| [7] | R1-2205364 | FL summary #1 for maintenance on HD-FDD for RedCap | Moderator (Qualcomm) |
| [8] | R1-2205442 | FL summary #2 for maintenance on HD-FDD for RedCap | Moderator (Qualcomm) |
| [9] | R1-2205738 | Corrections and clarifications of RedCap UE procedures | Ericsson |
| [10] | R1-2205788 | Correction on separate initial DL/UL BWP for RedCap UEs | Huawei, HiSilicon |
| [11] | R1-2205789 | Corrections related to NCD-SSB for RedCap UEs | Huawei, HiSilicon |
| [12] | R1-2205974 | Remaining issues on support of Reduced Capability NR Devices | Spreadtrum Communications |
| [13] | R1-2206298 | Other remaining issues for Reduced Capability NR Devices | OPPO |
| [14] | R1-2206369 | Correction on Type2-PDCCH CSS configuration in separate initial DL BWP | CATT |
| [15] | R1-2206416 | Remaining details on BWP operation for RedCap | NEC |
| [16] | R1-2206442 | Maintenance Issues on Complexity Reduction for RedCap | Nokia, Nokia Shanghai Bell |
| [17] | R1-2206546 | Draft CR on corrections to BWP operations for RedCap UEs | Intel Corporation |
| [18] | R1-2206547 | Remaining details on BWP operations for RedCap UEs | Intel Corporation |
| [19] | R1-2206548 | Draft CR on correction to handling of Types A and B PUSCH repetitions for HD-FDD RedCap UEs | Intel Corporation |
| [20] | R1-2206549 | Draft CR on corrections for handling of NCD-SSB for RedCap UEs | Intel Corporation |
| [21] | R1-2206550 | Draft CR on corrections for PDSCH reception in BWP configured with NCD-SSB for RedCap UEs | Intel Corporation |
| [22] | R1-2206551 | Discussion on NCD-SSB handling for RedCap UEs | Intel Corporation |
| [23] | R1-2206616 | Corrections on Half-duplex FDD operation in paired spectrum in TS 38.213 | Xiaomi |
| [24] | R1-2206746 | Corrections for RedCap UE behavior on BWP operation | vivo |
| [25] | R1-2206747 | Correction on PDSCH resource mapping around NCD-SSB for RedCap UE | vivo |
| [26] | R1-2206748 | Correction on PDCCH monitoring for RedCap UE | vivo |
| [27] | R1-2206749 | Corrections on DCI format 0_0 size determination for RedCap UE | vivo |
| [28] | R1-2206750 | Correction on available slot determination for PUSCH repetition type A for HD-FDD | vivo |
| [29] | R1-2206751 | Correction on invalid symbol determination for PUSCH repetition type B for HD-FDD | vivo |
| [30] | R1-2206888 | Correction on SSB transmission for initial DL BWP | CMCC |
| [31] | R1-2207000 | Correction for PUCCH resource set indication for RedCap | MediaTek Inc. |
| [32] | R1-2207045 | Discussion on RedCap remaining issues | ZTE, Sanechips |
| [33] | R1-2207046 | Correction on NCD-SSB related spec for RedCap in TS38.213 | ZTE, Sanechips |
| [34] | R1-2207047 | Correction on NCD-SSB related spec for RedCap in TS38.214 | ZTE, Sanechips |
| [35] | R1-2207048 | Correction on SSB and CORESET#0 presence for RedCap | ZTE, Sanechips |
| [36] | R1-2207196 | Maintenance on NR R17 RedCap UE | Qualcomm Incorporated |
| [37] | R1-2207272 | Corrections on available slot counting for PUSCH repetition type A for HD-UE | Sharp |
| [38] | R1-2207273 | Corrections on inclusion of NCD-SSB and switching gap for determining invalid symbols for PUSCH repetition type B for HD-UE | Sharp |
| [39] | R1-2207274 | Corrections on collision handling between NCD-SSB and UL transmission in TS38.213 for RedCap UE in unpaired spectrum | Sharp |
| [40] | R1-2207275 | Corrections on inclusion of NCD-SSB in TS38.214 for RedCap UE | Sharp |
| [41] | R1-2207276 | Correction on RRC parameter alignment for additional PRB offset in TS38.213 for RedCap UE | Sharp |
| [42] | R1-2207383 | Draft CR on timeline requirement for retransmitting MSG1/MSGA for RedCap | NTT DOCOMO, INC. |
| [43] | R1-2207384 | Discussion on timeline requirement for retransmitting MSG1/MSGA for RedCap | NTT DOCOMO, INC. |
| [44] | R1-2207494 | On PUCCH resource set indication for RedCap | MediaTek Beijing Inc. |
| [45] | R1-2207669 | Correction on separate initial UL BWP for RedCap UEs | Huawei, HiSilicon |
| [46] | R1-2205734 | Reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times | RAN2, Ericsson |
| [47] | R1-2205761 | On the offset between CD-SSB and NCD-SSB | Huawei, HiSilicon |
| [48] | R1-2206415 | Discussion on LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times | NEC |
| [49] | R1-2206441 | Discussion on reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB | Nokia, Nokia Shanghai Bell |
| [50] | R1-2206483 | On the Reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times | Ericsson |
| [51] | R1-2206704 | Draft reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times | vivo |
| [52] | R1-2207044 | Discussion on LS reply for time offset between CD-SSB and NCD-SSB | ZTE, Sanechips |
| [53] | R1-2207614 | Draft Reply LS on introduction of an offset to transmit CD-SSB and NCD-SSB at different times | Ericsson |
| [54] | R1-2207727 | FL summary #1 for Rel-17 RedCap maintenance | Moderator (Ericsson) |
| [55] | R1-2207728 | FL summary #2 for Rel-17 RedCap maintenance | Moderator (Ericsson) |