3GPP TSG RAN WG1 Meeting #110	R1-2207736

Toulouse, France, August 22 – 26, 2022

Agenda Item:	9.12.4

Source:	Moderator (MediaTek)

Title:	Feature lead summary #1 of AI 9.12.4 on improved GNSS operations

Document for:	Discussion

0 Introduction

0.1 Background

In RAN#96e, the revised WID on IoT NTN enhancements has been endorsed for Release 18 [1].

The work item aims to specify further enhancements for E-UTRA (LTE-RAN) based NTN (non-terrestrial networks) according to the following assumptions:

-	GEO and NGSO (LEO and MEO).

-	Earth fixed Tracking area. Earth fixed & Earth moving cells for NGSO

-	FDD mode

-	UEs with GNSS capabilities

The detailed objectives are to specify enhanced NB-IoT NTN and eMTC NTN radio interfaces and E-UTRAN/NG-RAN as follows:

4.1.1	IoT-NTN Performance Enhancements in Rel-18 to address remaining issues from Rel-17

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below:

-	Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

-	Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1]

- NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

In this meeting, company views on improved GNSS operations for IoT NTN are summarized proposals on identified issues are made.

0.2 Contact Information

Please help to fill in the contact information for the FL summary. (If any change, please revise.)

| Company | Name | E-mail |
| --- | --- | --- |
| OPPO | Hao Lin | lin.hao@oppo.com |
| Lockheed | Robert Olesen | robert.l.olesen@lmco.com |
| Lenovo | Zhi, Yan | yanzhi1@lenovo.com |
| ZTE | Fangyu Cui | cui.fangyu@zte.com.cn |
| Qualcomm | Ayan Sengupta | asengupt@qti.qualcomm.com |
| CATT | Deshan Miao | miaodeshan@catt.cn |
| Xiaomi | Yajun Zhu | zhuyajun@xiaomi.com |
| Nokia, NSB | Jingyuan Sun | Jingyuan.sun@nokia-sbell.com |
| Samsung | Min Wu | min1.wu@samsung.com |
| Samsung | Carmela Cozzo | carmela.c@samsung.com |
| CMCC | Wei Qin | qinwei@chinamobile.com |
| Nordic | Mauri Nissila | mauri.nissila@nordicsemi.no |
| Huawei, HiSilicon | Xiaolei TIE | tiexiaolei@huawei.com |
| Huawei, HiSilicon | Xinghua Song | songxinghua@huawei.com |
| SONY | Martin Beale | martin.beale@sony.com |
| Apple | Chunxuan Ye | Chunxuan_ye@apple.com |
| Apple | Chunhai Yao | Chunai_yao@apple.com |
| Spreadtrum | Zhenzhu Lei | reven.lei@unisoc.com |
| MediaTek | Wen Tang | WenT.Tang@mediatek.com |
| Ericsson | Talha Khan | talha.khan@ericsson.com |
| Ericsson | Olof Liberg | olof.liberg@ericsson.com |

- Issue #1: GNSS measurement triggering in connected

Agreement (RAN1 109e)

Further study on whether there is a need for potential enhancements on the following for long connection time

- UE triggered GNSS measurement.

- Network triggered GNSS measurement.

Conclusion(RAN1 109e)

IoT NTN UE may need to re-acquire a valid GNSS position fix in long connection time.

FFS: Whether and how to update or reduce the need to update GNSS position fix in long connection time

  - Company contributing views

| Contribution | Observation/Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 2: Support eNB aperiodically trigger GNSS measurement gap according to the validity duration reported by UE. Proposal 4: Support UE to update and report GNSS validity duration during connection. |
| Spreadtrum | Proposal 2: Network triggered GNSS measurement. should be supported. |
| MediaTek | Observation 8: RAN1 should further discuss how the UE can be kept in connected after GNSS validity expiration if residual transmit timing error after closed-loop timing correction is within the transmit timing error requirements. Observation 9: RAN1 should further discuss how the UE can be kept in connected after GNSS validity expiration and make a short GNSS measurement sufficient for a typical hot GSM position fix of 1 – 2 seconds if residual transmit timing error after closed-loop timing correction may exceed the transmit timing error requirements. |
| OPPO | Observation 1: For GNSS position TTFF, hot start requires about 1~2 seconds, warm start requires several seconds, and cold start requires about 30 seconds. Proposal 1: RAN1 to agree that UE performing GNSS position fix updating is under gNB control Proposal 2: GNSS validity duration expiration is not the only condition for GNSS position fix updating |
| Samsung | Proposal 2: Specify the condition of UE triggered GNSS measurement, e.g., GNSS validity timer is expired, there are UL data to transmit, and the UL synchronization is lost. Proposal 3: Specify the condition of eNB triggered GNSS measurement, e.g., GNSS validity timer is expired, there are DL data to transmit and the UL synchronization is lost. |
| Nordic Semiconductor ASA | Proposal 2: Consider a need of signalling mechanism for the network to dynamically allocate GNSS measurement gap to the UE in RRC connected mode. Proposal 3: Consider a need of signaling mechanism for UE to request a GNSS measurement gap whenever the need for it emerge. |
| InterDigital | Observation-3: Network triggered GNSS measurement is not necessary if a new gap for GNSS position fix is introduced in Rel-18 since a UE may perform GNSS measurement in the gap configured by the eNB. Observation-4: UE triggered GNSS measurement is not necessary considering that UE already reports GNSS validity duration as an assistance information for GNSS operation. Proposal-3: no support of additional mechanism to trigger GNSS measurement. |
| Ericsson | Proposal 1 Network can potentially indicate to an IoT NTN UE in connected mode if it can continue its uplink transmission after GNSS validity timer expiry. Proposal 2 Network can optionally indicate service link timing drift parameters to an IoT NTN UE in connected mode. Proposal 3 Upon GNSS timer expiry in connected mode, an IoT NTN UE may use service link drift information (in addition to common TA parameters) to calculate TA values before receiving the next TA command. Observation 2 The network can use GNSS assistance information (which the UE will have to report anyways if configured by the network) to trigger a GNSS measurement by scheduling a gap. Observation 3 The need of a UE-triggered GNSS measurement is not clear if the GNSS assistance information is reported anyways to the network. Proposal 6 Support network-triggered GNSS measurements in connected mode for IoT NTN. |
| Nokia | Proposal 1: GNSS measurement gap in CONNECTED mode should be specified for a new GNSS measurement when GNSS position is about to outdated. Proposal 9: RAN1 to discuss the possibility of network extending a UE’s GNSS validity timer. |
| Lenovo | Proposal 4: eNB configures the GNSS position fix gap periodically and UE report the update of GNSS validity duration X and GNSS position fix measurement time instead of directly trigger a GNSS measurement to eNB. |
| Apple | Proposal 1: RAN1 considers network schedules new periodic GNSS measurement gap for UE to re-acquire GNSS position for long connection times. Proposal 4: RAN1 is to study further aperiodically triggered GNSS measurement. |

When and how to start a GNSS measurement are discussed by contributing companies.

On how to start a GNSS measurement, several options are proposed:

Option 1: eNB/Network trigger GNSS measurement

- Huawei, HiSilicon proposed eNB aperiodically trigger GNSS measurement gap according to the validity duration reported by UE.

- Spreadtrum proposed Network triggered GNSS measurement should be supported.

- Samsung proposed to specify the condition of eNB triggered GNSS measurement, e.g., GNSS validity timer is expired, there are DL data to transmit and the UL synchronization is lost.

- Nordic proposed to consider a need of signalling mechanism for the network to dynamically allocate GNSS measurement gap to the UE in RRC connected mode.

- Ericsson proposed network-triggered GNSS measurements in connected mode using GNSS assistance information (which the UE will have to report anyways if configured by the network and the need of a UE-triggered GNSS measurement is not clear if the GNSS assistance information is reported anyways to the network).

- Apple mentioned RAN1 considers network schedules new periodic GNSS measurement gap for UE to re-acquire GNSS position for long connection times and studies further aperiodically triggered GNSS measurement.

Option 2: UE trigger GNSS measurement

- Samsung proposed to specify the condition of UE triggered GNSS measurement, e.g., GNSS validity timer is expired, there are UL data to transmit, and the UL synchronization is lost.

- Nordic proposed to consider a need of signaling mechanism for UE to request a GNSS measurement gap whenever the need for it emerge.

Option 3: eNB/Network control GNSS measurement

- OPPO proposed UE performing GNSS position fix updating is under eNB control.

- InterDigital observed that Network triggered GNSS measurement is not necessary if a new gap for GNSS position fix is introduced in Rel-18 since a UE may perform GNSS measurement in the gap configured by the eNB and UE triggered GNSS measurement is not necessary considering that UE already reports GNSS validity duration as an assistance information for GNSS operation and proposed no support of additional mechanism to trigger GNSS measurement.

- Lenovo proposed eNB configures the GNSS position fix gap periodically and UE report the update of GNSS validity duration X and GNSS position fix measurement time instead of directly trigger a GNSS measurement to eNB.

On when to start a GNSS measurement:

- Huawei, Hisilicon proposed UE to update and report GNSS validity duration during connection.

- MediaTek made observations that RAN1 should further discuss how the UE can be kept in connected after GNSS validity expiration if residual transmit timing error after closed-loop timing correction is within the transmit timing error requirements and how the UE can be kept in connected after GNSS validity expiration and make a short GNSS measurement sufficient for a typical hot GSM position fix of 1 – 2 seconds if residual transmit timing error after closed-loop timing correction may exceed the transmit timing error requirements.

- OPPO proposed GNSS validity duration expiration is not the only condition for GNSS position fix updating.

- Ericsson proposed Network can potentially indicate to an IoT NTN UE in connected mode if it can continue its uplink transmission after GNSS validity timer expiry, with Network optionally indicating service link timing drift parameters to an IoT NTN UE in connected mode and upon GNSS timer expiry in connected mode, an IoT NTN UE may use service link drift information (in addition to common TA parameters) to calculate TA values before receiving the next TA command.

- Nokia proposed RAN1 to discuss the possibility of network extending a UE’s GNSS validity timer and a GNSS measurement gap starts when GNSS position is about to become outdated.

Moderator View: To the moderator understanding, the issue for discussion is to align understanding on when and how to start a GNSS measurement in connected. Three options are mentioned for how to start a GNSS measurement. For Option 1 (eNB/Network trigger GNSS measurement), eNB/Network can periodically/ aperiodically trigger GNSS measurement, may be based on GNSS validity duration and residual transmit timing error, etc. For Option 2 (UE trigger GNSS measurement), UE may need to send a GNSS measurement request to eNB or UE may directly start a new timer without request, that depends on which option is utilized for Issue #2. For Option 3 (eNB/Network control GNSS measurement), eNB/Network controls when UE should do GNSS measurement (i.e. via scheduling gap). There seems to be reasonable consensus of companies for GNSS triggered by eNB, with support from Huawei, HiSilicon, Spreadtrum, Nordic Semiconductor ASA, Samsung, Ericsson, and Apple. GNSS triggered by UE also mentioned by Samsung and Nordic Semiconductor ASA. RAN1 can discuss whether GNSS measurement for GNSS position fix can be triggered by UE itself or by network with GNSS validity duration is known to both UE and whether GNSS measurement triggered by UE itself or by network both are under network control. Besides, RAN1 can discuss the relationship of GNSS validity duration and residual transmit timing error to decide whether GNSS validity timer extension is needed and when to start a GNSS measurement in connected.

  - First Round Discussion

Initial Proposal 1-1:

IoT NTN UE makes a GNSS measurement for new GNSS position fix in connected triggered by:

- eNB/Network (FFS: periodically or aperiodically)

FFS Whether GNSS measurements is controlled by the network

FFS GNSS position fix triggered by UE

Initial Proposal 1-2:

When GNSS validity duration expires, additional signalling can be used by network to keep UE in connected,

- Option 1: eNB extend a UE’s GNSS validity duration

- Option 2: eNB signals timing drift to connected UE to calculate TA values before receiving the next TA command

FFS Without additional signalling, the UE may re-acquire a GNSS position fix (as discussed in Issue#2)

Companies are encouraged to provide comments within the following table:

| Companies | Comments |
| --- | --- |
| OPPO | Initial proposal 1-1: fine Initial proposal 1-2: we suggest to discuss first if the GNSS validity duration will be configured by eNB, which can be different from the duration reported from the UE. If this is allowed, then we merely see the motivation for the eNB to further extend the duration. But if the GNSS validity duration is reported by the UE only, then we agree that the eNB may need to extend it. |
| Lenovo | We are fine with proposal 1-1 in general. But hope it is better to list two options similar as Option 1: eNB with periodical or aperiodical manner Option 2: UE with periodical or aperiodical manner For proposal 1-2, It seems that if the GNSS is triggered by eNB, there is no need the additional signalling for UE. By the way, Option 2 in proposal 1-2 has already supported in Rel.17, I am not sure what kind of the additional signal is. |
| ZTE | For proposal 1-1, support For proposal 1-2, since UE can report GNSS validity duration, eNB can properly trigger the GNSS measurement to minimize the probability that GNSS validity duration is expired in connected mode. Therefore, when GNSS validity duration is expired in connected mode, we can simply follow the legacy procedure, i.e., trigger RLF failure and re-acquire GNSS position, instead of extending the validity duration. |
| Ericsson | 1-1: We support network-triggered GNSS measurement. With closed_loop time/frequency correction, the network should try to maintain a UE’s transmit timing/frequency upon GNSS validity timer expiry. However, when/if GNSS position fix becomes necessary, it may trigger the UE to make a measurement. However, if there are some scenarios where the UE may need to make a GNSS measurement earlier than what is indicated by its validity timer (e.g., due to sudden changes in its mobility), then UE-triggered GNSS measurement may also be relevant (e.g., UE indicates to the network that it is about to make a measurement or UE may request a GNSS gap from the network). 1-2: In principle, we support closed-loop timing/frequency correction to reduce the need of GNSS reacquisition. Our view is that both option 1 and option 2, in some flavor, will be needed. Option 1: The network needs to keep the UE from switching to idle mode or initiating RLF procedure upon the expiry of GNSS validity timer. However, we are not convinced if extending the timer is really needed. Option 2: Support. If the timing/frequency drift is large, then the network may need to send frequent TA commands, resulting in a high signalling overhead. However, if the network shares UE-specific drift information to the UE, the network need not send frequent TA commands as the UE can use drift info to calculate TA values while it waits to receive a new TA command. |
| Spreadtrum | We support proposal 1-1. For proposal 1-2, in our view, when GNSS validity duration expires, UE should follow the legacy procedure, i.e., trigger RLF failure and re-acquire GNSS position. |
| Samsung | We support proposal 1-1. For proposal 1-2, when GNSS validity duration expires, whether or not to re-acquire GNSS position can be dependent on synchronization and/or data arriving status. For example, if no data to be transmitted, and/or, if synchronization is not lost, re-acquire GNSS position is not needed. |
| Huawei, HiSilicon | We are fine with main body of the proposal 1-1. - As for the 1st FFS, we are not sure whether it contradicts with the main body. If GNSS measurement is triggered by eNB/network, it implies eNB/network control the time instance of measurement. - For the 2nd FFS, it should be further clarified what is UE triggered measurement. According to my reading proponent companies’ paper, there are two alternatives. Alt 1, UE do measurement by its own. Alt 2, UE just send measurement request to eNB and let eNB to schedule a measurement. To my understanding, Alt 2 is still controlled by network. For proposal 1-2, we would suggest go through existing mechanism at first before introducing additional signalling. From our perspective, if UE can update or report new validity duration before the old one expires, no additional signalling is required. Also, it is not that straightforward for eNB to extend the validity duration reported by UE as eNB may not have knowledge about UE’s velocity and movement status. |
| Qualcomm | - is not clear. We do not believe the network should control this. As we have mentioned, the need for a new GNSS measurement should be “UE driven”. - Is an OK starting point. But we must keep in mind “frequency corrections” in addition to TA corrections. |
| MediaTek | Proposal 1-1: Support. Our understanding that the eNB based on measurements of transmit timing error or frequency error at its receiver (and after sending closed loop MAC TAC and potentially frequency compensation indication) still see an issue for its receiver demodulation performance and decides UEs need a new GNSS measurement and triggers UE accordingly. The UE has no way of knowing what the impact of its transmit timing error and fequency error even after applying the Closed loop corrections could be on the eNB receiver performance.. Proposal 1-2: - Option 1 can be further discussed. Our understanding is that eNB based on measurements of transmit timing error, frequency error at its receiver may allow UE to keep in RRC connected without re-acquiring GNSS. It is not clear how eNB can determine how long the GNSS can be extended. It would seem simpler if eNB trigger GNSS measurements if needed, and no need to extend GNSS dutaion - Option 2: To our understanding this would allow eNB to reduce the signalling for MAC CE TAC as UE can predict what its transmit timing error is after GNSS validity expires and correct it. Some evaluation for timing drift would be needed. |
| Nokia, NSB | For Initial Proposal 1-1: We think UE’s new GNSS measurement in CONNECTED mode should be in control of eNB/network while it can be FFS who trigger it. So we propose it to be modified to IoT NTN UE makes a GNSS measurement for new GNSS position fix in connected triggered controlled by: - eNB/Network (FFS: periodically or aperiodically) FFS Whether GNSS measurements is controlled triggered by the network FFS GNSS position fix triggered by UE For Initial Proposal 1-2: We think it should be “When Before GNSS validity duration expires” instead of “When GNSS validity duration expires” as when it expire, UE may be UL insync. While for option 1, there can be eNB to control how long the remaining GNSS validity duration, it can be multiple different way to consider, e.g. to extend the GNSS validity duration, to restart a new GNSS validity duration, etc. depends on e.g. eNB detected synchronization of the UL signal. |
| Xiaomi | For the proposal 1-2, we share the view the UE should be able to adjust the timing based on closed loop control before the expire of GNSS validity duration |
| Apple | We are ok with proposal 1-1. For Proposal 1-2, our understanding is we need to discuss the new GNSS measurement mechanism first, then discussing the further enhancement. For Option 1 of Proposal 1-2, if the periodic GNSS measurement is configured properly, additional signalling is not needed. If aperodic GNSS measurement is triggered, and GNSS validity duration is expired, then UE goes back idle mode. |
| Nordic | Ok with proposal 1-1. The second proposal needs more clarity as pointed out many companies |
| CMCC | We are generally fine with the Initial Proposal 1-1. For Initial Proposal 1-2, in our view, the GNSS validity duration is mainly determined by UE according to the mobility and position accuracy requirements. In case of GNSS validity duration expires and no GNSS position fix can be re-acquired before GNSS expired, UE can follow the legacy procedure as companies mentioned above. |
| CATT | For initial proposal 1-1，we think when UE has started to make GNSS measurement, UE can inform the network, rather than network to trigger one GNSS measurement. Since UE is aware of UE speed and GNSS status, UE should be able to determine when to start the GNSS measurement. For initial proposal 1-2, before the expiring, UE or network can take the action to prevent its occurrence. For example, UE can trigger one new GNSS measurement. Regarding the close-loop indication, it needs further study. |

  - Summary of First Round Discussion

15 companies provided comments on Initial Proposal 1-1 and Initial Proposal 1-2 in first round discussion.

On Initial Proposal 1-1:

- 9 companies [OPPO, Lenovo, ZTE, Spreadtrum, Samsung, MediaTek, Apple, Nordic, CMCC] supported Initial Proposal 1-1.

- Lenovo was fine with proposal 1-1 in general, and further suggested to update as two options, Option 1: eNB with periodical or aperiodical manner; and Option 2: UE with periodical or aperiodical manner.

- Ericsson supported network-triggered GNSS measurement (the main bullet of Initial Proposal 1-1). Ericsson clarified with closed_loop time/frequency correction, the network should try to maintain a UE’s transmit timing/frequency upon GNSS validity timer expiry, when/if GNSS position fix becomes necessary, it may trigger the UE to make a measurement. Ericsson further commented UE-triggered GNSS measurement may also be relevant if there are some scenarios where the UE may need to make a GNSS measurement earlier than what is indicated by its validity timer (e.g., UE indicates to the network that it is about to make a measurement or UE may request a GNSS gap from the network).

- Huawei, HiSilicon supported the main body. As for the 1st FFS, if GNSS measurement is triggered by eNB/network, it implies eNB/network control the time instance of measurement. For the 2nd FFS, it should be further clarified what is UE triggered measurement. And mentioned there are two alternatives. Alt 1, UE do measurement by its own. Alt 2, UE just send measurement request to eNB and let eNB to schedule a measurement which is under eNB/network control.

- Qualcomm mentioned the need for a new GNSS measurement should be “UE driven”.

- MediaTek further commented the eNB based on measurements of transmit timing error or frequency error at its receiver (and after sending closed loop MAC TAC and potentially frequency compensation indication) can trigger GNSS position fix when there is an issue for its receiver demodulation performance. And UE has no way of knowing what the impact of it transmit timing error and frequency error could be on the eNB receiver performance, even after applying the Closed loop corrections.

- Nokia commented UE’s new GNSS measurement in CONNECTED mode should be in control of eNB/network and who trigger it can be FFS with a revised version of proposal.

- CATT mentioned when UE has started to make GNSS measurement, UE can inform the network, rather than network to trigger one GNSS measurement. Since UE is aware of UE speed and GNSS status, UE should be able to determine when to start the GNSS measurement.

On Initial Proposal 1-2:

- OPPO suggested to discuss first if the GNSS validity duration will be configured by eNB, which can be different from the duration reported from the UE.

- Lenovo commented if the GNSS is triggered by eNB, there is no need the additional signalling for UE and suggested to clarify additional signalling. To the moderator understanding, additional signalling can be used by network to keep UE in connected after UE reported GNSS validity duration expired with no need to re-acquire GNSS position fix.

- ZTE mentioned eNB can properly trigger the GNSS measurement in connected mode before UE reported GNSS validity duration expired. The legacy procedure i.e., trigger RLF failure and re-acquire GNSS position, can be utilized when GNSS validity duration expired, instead of extending the validity duration. Spreadtrum shared similar view that when GNSS validity duration expires, UE should follow the legacy procedure, i.e., trigger RLF failure and re-acquire GNSS position. CMCC also mentioned GNSS validity duration is mainly determined by UE according to the mobility and position accuracy requirements, in case of GNSS validity duration expires and no GNSS position fix can be re-acquired before GNSS expired, UE can follow the legacy procedure as companies mentioned above.

- Ericsson commented both option 1 and option 2, in some flavor, would be needed and supported Option 2. Option 1: The network needs to keep the UE from switching to idle mode or initiating RLF procedure upon the expiry of GNSS validity timer. However, Ericsson was not convinced if extending the timer was really needed. Option 2: If the timing/frequency drift is large, then the network may need to send frequent TA commands, resulting in a high signalling overhead. However, if the network shares UE-specific drift information to the UE, the network need not send frequent TA commands as the UE can use drift info to calculate TA values while it waits to receive a new TA command.

- Samsung commented whether or not to re-acquire GNSS position can be dependent on synchronization and/or data arriving status when GNSS validity duration expires.

- Huawei, HiSilicon suggested to go through existing mechanism at first before introducing additional signalling. If UE can update or report new validity duration before the old one expires, no additional signalling is required. Also, it is not that straightforward for eNB to extend the validity duration reported by UE as eNB may not have knowledge about UE’s velocity and movement status.

- Qualcomm supported Initial Proposal 1-2 to be starting point. And mentioned “frequency corrections” in addition to TA corrections should be considered.

- MediaTek mentioned Option 1 can be further discussed. And mentioned eNB based on measurements of transmit timing error, frequency error at its receiver may allow UE to keep in RRC connected without re-acquiring GNSS. It would seem simpler if eNB trigger GNSS measurements if needed, and no need to extend GNSS duration. For Option 2: this would allow eNB to reduce the signalling for MAC CE TAC as UE can predict what transmit timing error is after GNSS validity expires and correct it.

- Nokia commented when GNSS validity duration expires, UE may be UL out of sync. While for option 1, there can be eNB to control how long the remaining GNSS validity duration, it can be multiple different way to consider, e.g. to extend the GNSS validity duration, to restart a new GNSS validity duration, etc. depends on e.g. eNB detected synchronization of the UL signal.

- Xiaomi mentioned UE should be able to adjust the timing based on closed loop control before the expire of GNSS validity duration.

- Apple mentioned RAN1 need to discuss the new GNSS measurement mechanism first, then discussing the further enhancement. For Option 1 of Proposal 1-2, if the periodic GNSS measurement is configured properly, additional signalling is not needed. If aperodic GNSS measurement is triggered, and GNSS validity duration is expired, then UE goes back to idle mode.

- CATT mentioned before the expiring, UE or network can take the action to prevent its occurrence. For example, UE can trigger one new GNSS measurement. Regarding the close-loop indication, it needs further study.

Moderator View: For first round discussion, 15 companies provided comments. On Initial Proposal 1-1, majority companies supported eNB triggers to make a GNSS measurement. To the moderator understanding, RAN1 can provide further comments on when GNSS measurement will be triggered and the configuration of signalling for the trigger. RAN1 can further discuss on additional signalling provided to connected UE to calculate TA values before receiving the next TA command.

  - Second Round Discussion

Second Round Proposal 1-1a:

eNB triggers connected UE to make a GNSS measurement for new GNSS position fix

- Before GNSS validity duration expires

- After GNSS validity duration expires and closed-loop corrections are applied

FFS Additional signalling

Second Round Proposal 1-2a:

eNB provides additional signalling to connected UE to calculate TA values before receiving the next TA command

- FFS Signalling of UE transmit timing drift determined at eNB after closed-loop timing correction is applied

Companies are encouraged to provide comments within the following table:

| Companies | Comments |
| --- | --- |
| Ericsson | 1-1a: If GNSS expires, the network and UE should try to maintain UL sync via closed loop TA commands. If a new GNSS fix is needed, it should be done after this mechanism fails to maintain uplink sync. Otherwise, closed-loop TA mechanism is rendered irrelevant if the UE will necessarily perform a GNSS position fix every time its timer expires. 1-2a: Support. Similar to common TA drift parameters, the network may optionally signal UE-specific timing drift parameters for the service link. This will help reduce the signalling overhead due to TA commands, as the UE can continue to calculate its TA in a semi-autonomous manner until it receives a new TA command from the network. |
| Xiaomi | For proposal 1-1a, it is confused why we need to discuss whether eNB triggers connected UE to make a GNSS measurement before or after GNSS validity duration expires. This is anywhere under the eNB’s control. |

- Issue #2: Schemes for GNSS measurement in connected

RAN2-116bis

UE need to have a valid GNSS fix before going to connected. RAN2 assumes that the UE may need to re-acquire the GNSS fix right before establishing the connection (regardless if previously valid or not), if needed to avoid interruption during the connection.

When the GNSS fix becomes outdated in RRC_CONNECTED mode, the UE goes to IDLE mode.

Agreement (RAN1 109e)

At least the following options can be considered on GNSS measurement in connected for potential enhancements for improved GNSS operations:

- Option 1: UE re-acquires GNSS position fix during RLF procedure

- Option 2: UE re-acquires GNSS position fix with a new gap

Note: this does not imply that a Rel-18 IoT NTN UE is mandated to support one or both of the options.

  - Company contributing views

| Contribution | Observation/Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 1: Support for UE to re-acquire GNSS position fix during a new gap, which is assigned by eNB so as to align GNSS position fix between eNB and UE. |
| Spreadtrum | Proposal 1: UE re-acquires GNSS position fix with a new gap should be supported on GNSS measurement in connected. |
| ZTE | - Observation 1: Enable the reacquisition of GNSS position fix without changes of UE’s state is beneficial to improve the UE’s power consumption and reduce the signalling overhead. - Proposal 1: Option 2 is preferred for GNSS measurement in connected with consideration of lower complexity and cost. |
| MediaTek | Proposal 2: Option 1 “UE re-acquire GNSS position fix during RLF procedure” is baseline for improved GNSS operations. |
| OPPO | Proposal 3: For the scheme of GNSS measurement, the option 2: UE to re-acquire GNSS position fix in a new gap, can be prioritized in this release. |
| CATT | Proposal 1: Specify one common GNSS measurement gap as minimum measurement duration for the all UEs to minimize the standard impact. Proposal 3: UE should recover DL synchronization and send the UL PRACH signal to network for UL re-synchronization and the indication of GNSS measurement completion. |
| xiaomi | Proposal 2: The IoT UE can be configured with measurement gap to perform GNSS measurement depending on the UE’s capability. |
| Samsung | Proposal 4: For eNB triggered GNSS measurement, a GNSS measurement window needs to be defined. |
| Nordic Semiconductor ASA | Proposal 2: Consider a need of signalling mechanism for the network to dynamically allocate GNSS measurement gap to the UE in RRC connected mode. Proposal 3: Consider a need of signaling mechanism for UE to request a GNSS measurement gap whenever the need for it emerge. |
| CMCC | Proposal 1. During long connection times, the GNSS measurement gaps can be applied to update the GNSS position fix in case the GNSS becomes outdated. Proposal 2. If GNSS measurement gap is applied to enable a new GNSS position fix during long connection times, the common understanding between UE and eNB that UE may automatically perform GNSS operation and NTN NB-IoT/eMTC operation on non-overlapping slot/frame should be achieved. Proposal 3. The GNSS measurement gaps can be configured individually per UE level. Proposal 4. Considering the mechanisms to support UE perform GNSS operation and NTN NB-IoT/eMTC operation on non-overlapping slot/frame, the following configuration can be further studied: - Option 1. Semi-static indication of GNSS measurement gap period through RRC configuration. - Option 2. Indicate the GNSS measurement gap by DCI format. - Option 3. Signal the GNSS measurement by MAC CE command. |
| InterDigital | Proposal-1: Support a new gap for a UE to re-acquire GNSS position fix (i.e., Option 2). |
| Qualcomm | Observation 8: GNSS measurement in connected mode should be considered a fallback procedure (as opposed to a routine, expected occurrence), e.g., when closed-loop time and frequency commands fail to maintain uplink synchronization in time and frequency within stipulated requirements. Proposal 5: A UE may re-acquire GNSS as part of a “recovery” procedure before the declaration of an eventual RLF (if the recovery fails within a certain time), in a similar fashion to Release 17 recovery procedures for ephemeris and common TA parameters upon validity expiry. Proposal 6: The network shall indicate whether it supports UE behavior associated with connected mode GNSS measurements, as part of a recovery procedure upon expiry of GNSS validity. - This avoids backwards compatibility issues w.r.t a Rel 17 base-station, that always expects UEs to go to IDLE mode upon GNSS validity expiry. |
| Ericsson | Observation 1 It is not obvious if there are any technical benefits in performing GNSS acquisition during RLF procedure over using a GNSS measurement gap. Proposal 5 Support GNSS measurement gaps for IoT NTN UEs in connected mode. |
| Nokia | Proposal 1: GNSS measurement gap in CONNECTED mode should be specified for a new GNSS measurement when GNSS position is about to outdated. Observation 2: Common understanding on GNSS measurement gap in time domain between UE and network is needed. Observation 3: Multiple IoT UEs with different capability, mobility state and channel status may request different GNSS measurement gaps. Observation 6: UE may be aware of its own movement based on RRC Idle mobility state or GNSS positions changing. Proposal 6: RAN1 to discuss how the network can handle UEs, which are aware they will need a GNSS measurement gap in an upcoming long RRC Connection. Observation 7: network can provide common GNSS measurement gap configurations for different UE mobility states. Proposal 7: RAN1 to discuss use of common GNSS measurement gap configurations to effectively support many moving UEs. Proposal 11: Enabling a GNSS measurement gap during the long repetition for IoT should be studied. |
| Lenovo | Proposal 1: Scheduling new gap solution can be supported for UE with long uplink transmission and solution of scheduling new gap to allow UE to refresh its GNSS position fix needs further study. |
| Apple | Proposal 1: RAN1 considers network schedules new periodic GNSS measurement gap for UE to re-acquire GNSS position for long connection times. |

Two options are mentioned in last meeting on GNSS measurement schemes in connected.

Option 1: UE re-acquires GNSS position fix during RLF procedure

MediaTek, Qualcomm, Ericsson provided proposals/observations for UE re-acquires GNSS position fix during RLF procedure.

- MediaTek proposed Option 1 is baseline for improved GNSS operations. And mentioned in the RLF procedure, a new timer is utilized, UE may switch off IoT module to re-acquire GNSS position fix and switch on IoT module to re-synchronize on DL. Then UE can reset the timer and exit RLF procedure. During all that time the UE is kept in RRC_CONNECTED, which is similar to the procedure in Rel-17 when UL ephemeris validity expired. Assuming a hot fix of 1-2 seconds and re-sycnhronization time in the order of several hundred ms, the total time will be consistent with connected DRX of 2.56 s. In Rel-17, when UE initiates RLF procedure it is kept in RRC_CONNECTED and upon recovery from loss of pre-compensation synchronization while the Timing Alignment Timer (TAT) has not expired, UE resumes UL operation, and no RACH is needed.

- Qualcomm proposed a UE may re-acquire GNSS as part of a “recovery” procedure before the declaration of an eventual RLF (if the recovery fails within a certain time), in a similar fashion to Release 17 recovery procedures for ephemeris and common TA parameters upon validity expiry and network shall indicate whether it supports UE behavior associated with connected mode GNSS measurements, as part of a recovery procedure upon expiry of GNSS validity (This avoids backwards compatibility issues w.r.t a Rel 17 base-station, that always expects UEs to go to IDLE mode upon GNSS validity expiry.) And clarified that GNSS measurement in connected mode should be considered a fallback procedure (as opposed to a routine, expected occurrence), e.g., when closed-loop time and frequency commands fail to maintain uplink synchronization in time and frequency within stipulated requirements.

- Ericsson mentioned it is not obvious if there are any technical benefits in performing GNSS acquisition during RLF procedure over using a GNSS measurement gap

Option 2: UE re-acquires GNSS position fix with a new gap

Huawei, HiSilicon, Spreadtrum, ZTE, OPPO, CATT, xiaomi, Samsung, Nordic, CMCC, InterDigital, Ericsson, Nokia, Lenovo, Apple proposed proposals for GNSS measurement gap.

- ZTE mentioned enable the reacquisition of GNSS position fix without changes of UE’s state is beneficial to improve the UE’s power consumption and reduce the signalling overhead and Option 2 is preferred for GNSS measurement in connected with consideration of lower complexity and cost.

- CATT mentioned one common GNSS measurement gap as minimum measurement duration for all UEs to minimize the standard impact. And UE should recover DL synchronization and send the UL PRACH signal to network for UL re-synchronization and the indication of GNSS measurement completion.

- Xiaomi proposed IoT UE can be configured with measurement gap to perform GNSS measurement depending on the UE’s capability.

- Nordic proposed signalling mechanism for the network to dynamically allocate GNSS measurement gap.

- CMCC proposed if GNSS measurement gap is applied to enable a new GNSS position fix during long connection times, the common understanding between UE and eNB that UE may automatically perform GNSS operation and NTN NB-IoT/eMTC operation on non-overlapping slot/frame should be achieved. And the GNSS measurement gaps can be configured individually per UE level with the following configuration can be further studied: Option 1. Semi-static indication of GNSS measurement gap period through RRC configuration; Option 2. Indicate the GNSS measurement gap by DCI format; Option 3. Signal the GNSS measurement by MAC CE command.

- Nokia proposed GNSS measurement gap in CONNECTED mode should be specified for a new GNSS measurement when GNSS position is about to outdated. And mentioned Common understanding on GNSS measurement gap in time domain between UE and network is needed, where multiple IoT UEs with different capability, mobility state and channel status may request different GNSS measurement gaps. UE may be aware of its own movement based on RRC Idle mobility state or GNSS positions changing, and network can provide common GNSS measurement gap configurations for different UE mobility states. Nokia proposed RAN1 to discuss how the network can handle UEs, which are aware they will need a GNSS measurement gap in an upcoming long RRC Connection, use of common GNSS measurement gap configurations to effectively support many moving UEs, enabling a GNSS measurement gap during the long repetition for IoT.

- Lenovo proposed scheduling new gap solution can be supported for UE with long uplink transmission and solution of scheduling new gap to allow UE to refresh its GNSS position fix needs further study.

- Apple proposed periodic GNSS measurement gap for UE to re-acquire GNSS position for long connection times.

Moderator View: As mentioned by MediaTek and Qualcomm, Option 1 is close to recovery procedures for ephemeris and common TA parameters upon validity expiry in Rel-17, the UE is kept in RRC_CONNECTED within a certain timer. Option 2 supported by Huawei, HiSilicon, Spreadtrum, ZTE, OPPO, CATT, xiaomi, Samsung, Nordic, CMCC, InterDigital, Ericsson, Nokia, Lenovo, Apple, which may have specification impact for configuration of UE-level/common GNSS measurement gap and related signalling.

  - First Round Discussion

Initial Proposal 2:

For GNSS measurement scheme in connected:

- Option 1: UE re-acquires GNSS position fix in a “recovery” procedure during RLF with a new timer for GNSS measurement.

  - FFS: Details of GNSS measurement timer.

- Option 2: UE re-acquires GNSS position fix with a new gap

  - FFS: UE-level/common gap length

  - FFS: Signalling for configuration of the GNSS measurement gap.

Companies are encouraged to provide comments within the following table:

| Companies | Comments |
| --- | --- |
| OPPO | We support option 2. From our opinion, the GNSS recovery procedure should not be related to the RLF procedure. If companies want to use a timer that controls the GNSS measurement, we suggest to follow the R17 principle, i.e. introducing a GNSS measurement timer, when the UE does not complete GNSS recovery, the UE should go to idle. Thus, we suggest the following option 3 Option 1: UE re-acquires GNSS position fix within a new timer, and the UE goes to idle after the new timer expires - FFS: Details of GNSS measurement timer. |
| Lenovo | For the main bullet, will we consider the two options or downselect from the two options? For option 2, should the first FFS be the sub-bullet of the second FFS? The gap length is configured by cell-specific or UE-specific? |
| ZTE | Support option 2. With option 2, BS and UE can achieve consensus on the time for GNSS measurement and avoid scheduling new transmission during this period. The connection re-establishment related signaling cost can be also saved. |
| Ericsson | With Option 1, it seems that a common timer may be needed for all UEs regardless of their GNSS TTFF requirement, which further suggests that a relatively large value for the GNSS measurement timer may be needed to accommodate different GNSS TTFF values. Option 2 seems more flexible since the network can configure a GNSS gap depending on the TTFF value reported by the UE. We tend to lean towards Option 2 but are open to further discussing both options. |
| Spreadtrum | We support Option 2. |
| Samsung | We support Option 2. |
| Huawei, HiSilicon | Comparing with the agreement in last meeting, the proposal 2 do not provide obvious progress from our perspective. Considering the clear majority, we should put our effort option 2 with higher priority. We also share the view from OPPO that option 1 mentioned by proponent companies looks not the RLF procedure, which is clear defined in higher layers specification. |
| Qualcomm | We support Option 1, since GNSS is “inherently” UE-driven. We do not see any need for a “network triggered gap”. Recall that scheduling gaps in legacy are for much smaller durations than it would be for GNSS. Also, anyway, Option 1 must be defined as a “fallback” in case something like Option 2 fails. Also Option 1 is consistent with the ephemeris and common TA recovery procedures defined in Release 17. |
| MediaTek | Both options can be considered for this meeting, and potential gains of Option 2 should be further discussed. Option 1can be the baseline. |
| Nokia, NSB | We agree option 2 can be further studied as it can provide good common understanding between UE and network with no unnecessary additional power/latency/complexity, with similar requirement on spec update as option 1. But option 1 will need unnecessary procedure for RRC reconnection, which is not beneficial for IoT UE. |
| Xiaomi | Support option 2 to make common understanding between UE and eNB. |
| Apple | We support Option2. |
| Nordic | First we should decide whether closed loop time and frequency correction works effectively to compensate time and frequency drifting due to UE movement. Then we can discuss about what kind of recovery procedures are needed if UL synch is lost |
| CMCC | We support Option 2, and configuration of the GNSS measurement gap can be further discussed together with the consideration of impact on power consumption and complexity. |
| CATT | For Option 1, UE has to perform lots of procedure that is not necessary at this scenario. For example, cell reselection and RRC re-establishment. So for this option, we suggest introducing RAN2 for the impacts evaluation. For Option 2, maybe no new gap is needed, for example, the UE perform GNSS measurement when the existing validity duration expires, and UE recovers DL synchronization and sends the UL PRACH signal to network for UL re-synchronization and the indication of GNSS measurement completion. The common gap we mentioned in our paper, means that, there should be a minimum duration start at the time of validity duration expires, which defines the minimum time a UE needed for GNSS measurement. |

  - Summary of First Round Discussion

15 companies provided comments on Initial Proposal 2 in first round discussion.

Option 1: UE re-acquires GNSS position fix in a “recovery” procedure during RLF with a new timer for GNSS measurement.

- Qualcomm supported Option 1. Qualcomm further mentioned Option 1 must be defined as a “fallback” in case something like Option 2 fails and Option 1 is consistent with the ephemeris and common TA recovery procedures defined in Release 17.

- MediaTek mentioned both options can be considered and Option 1can be the baseline.

- OPPO, Huawei, HiSilicon commented that the Option 1 GNSS recovery procedure should not be related to the RLF procedure. OPPO further suggested to follow the R17 principle, i.e. introducing a GNSS measurement timer, when the UE does not complete GNSS recovery, the UE should go to idle. And OPPO suggested a option 3: UE re-acquires GNSS position fix within a new timer, and the UE goes to idle after the new timer expires (FFS: Details of GNSS measurement timer).

- Ericsson mentioned a common timer may be needed for all UEs regardless of their GNSS TTFF requirement for Option 1. And Ericsson mentioned to open to further discussing both options

- Nokia mentioned Option 1 will need unnecessary procedure for RRC reconnection, which is not beneficial for IoT UE.

- CATT mentioned UE has to perform lots of procedure that is not necessary at this scenario, e.g., cell reselection and RRC re-establishment.

Option 2: UE re-acquires GNSS position fix with a new gap

OPPO, ZTE, Ericsson, Spreadtrum, Samsung, Nokia, Xiaomi, Apple, CMCC supported Option 2.

- Lenovo mentioned to clarify whether the gap length is configured by cell-specific or UE-specific.

- ZTE, Xiaomi mentioned with option 2, BS and UE can achieve consensus on the time for GNSS measurement and avoid scheduling new transmission during this period. ZTE further mentioned the connection re-establishment related signaling cost can be also saved.

- Ericsson mentioned Option 2 seems more flexible since the network can configure a GNSS gap depending on the TTFF value reported by the UE.

- Huawei, HiSilicon mentioned to put effort on option 2 with higher priority.

- MediaTek mentioned both options can be considered, and potential gains of Option 2 should be further discussed.

- Nokia mentioned option 2 can be further studied as it can provide good common understanding between UE and network with no unnecessary additional power/latency/complexity, with similar requirement on spec update as option 1.

- CMCC mentioned configuration of the GNSS measurement gap can be further discussed together with the consideration of impact on power consumption and complexity.

- CATT mentioned maybe no new gap is needed, for example, the UE perform GNSS measurement when the existing validity duration expires, and UE recovers DL synchronization and sends the UL PRACH signal to network for UL re-synchronization and the indication of GNSS measurement completion. And clarified the common gap means that, there should be a minimum duration start at the time of validity duration expires, which defines the minimum time a UE needed for GNSS measurement.

Nordic mentioned RAN1 can decide whether closed loop time and frequency correction works effectively to compensate time and frequency drifting due to UE movement, then discuss about what kind of recovery procedures are needed if UL synch is lost

Moderator View: For first round discussion, 15 companies provided comments. OPPO, ZTE, Ericsson, Spreadtrum, Samsung, Nokia, Xiaomi, Apple, CMCC supported Option 2 UE re-acquires GNSS position fix with a new gap. To the moderator understanding, with Option 2, eNB can flexibly schedule on the new gap to save power/latency; while for Option 1, RAN1 should first discuss whether a “fallback” procedure is needed in case something like Option 2 fails, then RAN1 can discuss the detailed of the procedure.

  - Second Round Discussion

Second Round Proposal 2-1a:

If eNB triggers connected UE to make GNSS measurement, UE re-acquires GNSS position fix with a new gap

Second Round Proposal 2-2a:

If eNB does not trigger UE to make GNSS measurements, UE can re-acquire GNSS position fix based on a new GNSS measurement timer

- FFS Configuration of new GNSS measurement timer

- FFS When the UE starts new GNSS measurement timer

Companies are encouraged to provide comments within the following table:

| Companies | Comments |
| --- | --- |
| Ericsson | 2-1a: Support. If the eNB has triggered a GNSS measurement, our understanding is that it will also configure a gap. 2-2a: Support. If the eNB does not trigger a GNSS measurement, then it makes sense to enable the UE to acquire GNSS position during RLF procedure. |
| Xiaomi | 2-1a support. 2-2a. our understanding is that for UEs with low capability, if the eNB does not trigger UE to make GNSS measurements, the UE could either enter IDLE to require GNSS or relies on the closed loop adjustment. |

- Issue #3: Report GNSS assistance information

The moderator recalls the agreements for GNSS assistance information.

Agreement (RAN1 107-e):

The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling.

- X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

Send LS to RAN2 to take the following RAN1 agreements into consideration to specify the aspects related to GNSS position validity:

- For sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated

- The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling.

  - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

- Note: The duration of the short transmission is not longer than the “validity timer for UL synchronization” referred to in the WID objective (but which still needs further discussion for specifying further details)

Agreement (RAN1 109-e):

UE reports additional GNSS assistance information and further study the detailed GNSS assistance information, including e.g. GNSS position fix measurement time

- Note: Since RAN1 agreed that GNSS validity duration is reported by UE in Rel-17, it is already included in GNSS assistance information.

RAN2-118

A new parameter for remaining GNSS validity duration is introduced in Msg5, e.g. RRCConnectionResumeComplete, RRCConnectionSetupComplete and RRCreestablishmentComplete messages, and the parameter refers to the time of message transmission.

Can discuss offline whether to expand the granularity of the value range, but if no convergence will implement the R1 proposal from the LS.

P1: The value range of the remaining GNSS validity duration should include the values proposed by RAN1 , i.e. {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}.

P3: The new parameter for remaining GNSS validity duration is introduced in the following Msg5 messages: RRCConnectionResumeComplete, RRCConnectionSetupComplete, RRCreestablishmentComplete RRCConnectionResumeComplete-NB, RRCConnectionSetupComplete-NB, RRCreestablishmentComplete-NB.

P4: The new parameter for remaining GNSS validity duration is introduced in RRCConnectionReconfigurationComplete for MTC Handover.

P5: No new RRC release cause “GNSS invalidity” is introduced in RRC Release.

This mechanism is not configurable, and the UE always reports.

  - Company contribution views

| Contribution | Observation/Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 4: Support UE to update and report GNSS validity duration during connection. Proposal 5: Support UE report the minimum duration of measurement gap for GNSS position fix. Proposal 6: The minimum value of measurement gap reported by UE can be zero, implying the capability of simultaneous cellular and GNSS operation. |
| Spreadtrum | Proposal 3: UE reports GNSS position fix measurement time should be supported. |
| ZTE | - Proposal 2: Reporting the time length of a new gap for option 2 should be supported to ensure common understanding between BS and UE. - Proposal 3: The previous validity duration length and time window length should be applied after a new GNSS position fix if there is no corresponding report. |
| MediaTek | Observation 7: Network can make better scheduling decision for long connection times with knowledge of GNSS validity duration and GNSS position fix measurement length. Proposal 1: GNSS assistance information consists of: - GNSS validity duration. - GNSS position fix measurement length. |
| OPPO | Proposal 4: UE needs to report the GNSS measurement duration to the gNB. |
| CATT | Proposal 2: UE can report one indication to confirm whether a new GNSS measurement is needed when the GNSS position fix becomes out-of-date. |
| xiaomi | Proposal 1: UE reports the capability related to the non-simultaneously GNSS and NTN NB-IoT/eMTC operation. |
| Samsung | Proposal 5: Following information can be reported to eNB to assist GNSS operation: - Whether UE’s GNSS position is fixed or not; - Whether UE’s GNSS position is available in IoT application layer or not; - UE capability on GNSS measurement, e.g., preferred length of GNSS measurement window; |
| InterDigital | Observation-1: reporting of the required time gap for GNSS position fix measurement is beneficial when the new time gap is introduced as the required time gap could be largely different up to UE implementation and it can avoid penalizing UEs with higher capability. Observation-2: UE capability of simultaneous GNSS and IoT operations can be indicated implicitly by reporting ‘0’ value of the required time gap for GNSS position fix measurement. Proposal-2: support reporting of the required time gap for GNSS position fix as a part of GNSS assistance information if the new time gap is introduced. |
| Ericsson | Observation 4 NTN UEs are required to have a valid GNSS position before accessing the network. Proposal 7 UE should report the GNSS assistance information during connection establishment phase. Observation 5 GNSS assistance information may also need to be reported if a GNSS validity timer expires during a connection. Proposal 8 Upon GNSS timer expiry, a UE in connected mode should report the GNSS assistance information after GNSS reacquisition using RRC signalling if configured by the network. Proposal 9 GNSS assistance information may optionally include Time-to-first-fix (TTFF) for the present and/or the next GNSS position. Proposal 10 Network should configure the type of GNSS assistance information to be included in the UE report. |
| Nokia | Proposal 2: Overhead reduction should be considered for selection of GNSS measurement gap and coordination between UE and eNB. Proposal 3: RAN1 to specify UE can report the required GNSS measurement gap, to keep a low signaling overhead. |
| Lenovo | Proposal 2: UE reports GNSS position fix measurement time information and eNB configures the gap duration based on the assistance information. |
| Apple | Proposal 2: UE reports the residual GNSS validity duration and GNSS measurement time to network to facilitate the GNSS measurement gap scheduling. |

On detailed information of UE reported GNSS assistance information, several options are proposed by contributing companies.

Option 1: GNSS validity duration

- Huawei, HiSilicon proposed UE to update and report GNSS validity duration during connection.

- MediaTek proposed GNSS assistance information consists of GNSS validity duration and GNSS position fix measurement length.

Option 2: GNSS position fix measurement time

- Spreadtrum, OPPO, Lenovo, Apple proposed UE to report GNSS position fix measurement time.

- MediaTek proposed UE to report GNSS position fix measurement time, so that Network can make better scheduling decision for long connection times with knowledge of GNSS validity duration and GNSS position fix measurement length

- Ericsson proposed GNSS assistance information may optionally include Time-to-first-fix (TTFF) for the present and/or the next GNSS position, and Network should configure the type of GNSS assistance information to be included in the UE report.

Option 3: GNSS measurement gap

- Huawei, HiSilicon proposed UE to report the minimum duration of measurement gap for GNSS position fix.

- ZTE proposed UE to report the time length of a new gap for option 2 to ensure common understanding between BS and UE. And proposed the previous validity duration length and time window length should be applied after a new GNSS position fix if there is no corresponding report.

- Samsung proposed UE can report UE capability on GNSS measurement, e.g., preferred length of GNSS measurement window.

- InterDigital proposed UE reporting of the required time gap for GNSS position fix as a part of GNSS assistance information if the new time gap is introduced.

- Nokia proposed UE can report the required GNSS measurement gap, to keep a low signaling overhead.

Option 4: UE capability of simultaneous cellular and GNSS operation

- Huawei, HiSilicon proposed UE to imply the capability of simultaneous cellular and GNSS operation with the minimum value of measurement gap reported by UE be zero.

- InterDigital proposed UE capability of simultaneous GNSS and IoT operations can be indicated implicitly by reporting ‘0’ value of the required time gap for GNSS position fix measurement

- Xiaomi proposed UE reports the capability related to the non-simultaneously GNSS and NTN NB-IoT/eMTC operation. And mentioned different understandings on the simultaneously GNSS and NTN NB-IoT/eMTC operation. One interpretation could be when a UE performs GNSS measurement, the cellular connection is completely released. While the other one interpretation could be when a UE performs GNSS measurement, it cannot perform data exchange while it is capable to maintain the cellular connection.

Option 5: one indication to confirm whether a new GNSS measurement is needed

- CATT proposed UE can report one GNSS indication to inform the GNSS measurement is needed when the GNSS position become out-of-date. Based on this indication, gNB will confirm that the UE will not leave RRC_CONNECTED when the existing GNSS position become out-of-date. The gNB can also determine that a long connection is needed for a UE based on the UE’s service traffic status and send an indication to UE.

Option 6: whether UE’s GNSS position is fixed or not

- Samsung proposed UE can report that its GNSS position is fixed, then GNSS measurement is not needed during a long connection time, and eNB does not need to trigger a new GNSS position fix.

Option 7: whether UE’s GNSS position is available in IoT application layer or not

- Samsung proposed UE can report that its GNSS position can be available in IoT application layer, then UE can obtain new GNSS position by delivering GNSS position information between layers, and no need to trigger GNSS measurement. When eNB triggers a new GNSS position fix, a small GNSS measurement window may be enough, or GNSS measurement window may not be needed.

On when UE should report the GNSS assistance information:

- Ericsson proposed UE should report the GNSS assistance information during connection establishment phase, considering NTN UEs are required to have a valid GNSS position before accessing the network. Besides, Ericsson proposed upon GNSS timer expiry, a UE in connected mode should report the GNSS assistance information after GNSS reacquisition using RRC signalling if configured by the network, considering GNSS assistance information may also need to be reported if a GNSS validity timer expires during a connection.

Moderator View: For Option 1, it has been agreed in RAN1 109e that “Note: Since RAN1 agreed that GNSS validity duration is reported by UE in Rel-17, it is already included in GNSS assistance information.” For Option 2, GNSS position fix measurement time may be beneficial to help UE and Network have same understanding on which GNSS measurement type UE will utilize (e.g. hot start / warm start / cold start). For Option 3, we can discuss Issue #2 first. For Option 4, in case new GNSS measurement gap is configured, the capability of simultaneous cellular and GNSS operation with the minimum value of measurement gap can be indicated by UE reporting zero. Whether a new GNSS measurement gap is needed can be first discussed (in Issue #2). For Option 5, whether there is a need for UE to report one GNSS indication to inform the GNSS measurement is unclear as UE has reported GNSS validity duration as specified in Rel-17. For Option 6, it has already been discussed last meeting that this has been covered by Rel-17 GNSS validity timer as infinity. For Option 7, whether UE should report if UE’s GNSS position is available in IoT application layer or not can be further discussed. Rel-17 has agreed that the UE always reports GNSS Validity duration and agreed to introduce it in Msg5. For when to report GNSS assistance information in connected can be discussed after the detailed GNSS assistance information that UE reports to eNB has finished.

  - First Round Discussion

Initial Proposal 3:

GNSS assistance information that UE reports to eNB consists of:

- GNSS position fix measurement time

- UE’s GNSS position is available in IoT application layer

- GNSS validity duration as specified in Rel-17

Companies are encouraged to provide comments within the following table:

| Companies | Comments |
| --- | --- |
| OPPO | 1st bullet: we agree 2nd bullet: we think it is not needed, the UE can simply report an infinite GNSS validity duration 3rd bullet: already supported in the spec, thus may not need an agreement again |
| Lenovo | At least 3rd bullet is not needed since the spec is already supported. We don’t need to discuss the assistance information anymore. |
| ZTE | We support bullet 1 and 3. For bullet 2, we do not see the need to have this report. Even if a UE can reduce measurement time by GNSS information from higher layer, it can just report a shorter GNSS position measurement time. That is, the function of bullet 2 can be covered by bullet 1. There is no need to define additional signaling. |
| Ericsson | Support bullet 1 and 3. Simultaneous GNSS and NB-IoT/eMTC operation is not assumed in the WI. However, if a UE does not need a measurement gap, it may indicate it using the GNSS position fix measurement time e.g., using “0”. Therefore, a separate field for indicating “GNSS position available in IoT application layer” is not needed. |
| Spreadtrum | We support bullet 1 and 3. |
| Samsung | We support bullet 1, 2 and 3. |
| Huawei, HiSilicon | We are wondering whether all these three information are going to be reported and how about the rest of the options listed in the summary. At least, the time duration for GNSS measurement (either GNSS position fix measurement time or measurement gap) and GNSS validity duration (option 2) are needed. Both information might be used as reference by eNB to determine when to trigger the GNSS position fix measurement if network triggered scheme is adopted. As for the 2nd information in the proposal, it is by UE implementation. |
| Qualcomm | Generally OK, but need to see that there is no overlap with existing signalling in Release 17. |
| MediaTek | Support bullets 1 and 3. On bullet 2., it can be further discussed how this information will be used and whether it is needed. |
| Nokia, NSB | We think the first and the third information are needed. However, the second bullet is not related to RAN1 but can be represented by first and third bullet, e.g. if UE’s GNSS position is available, then UE may report 0ms measurement time for the next reading. |
| Xiaomi | Support bullet 1 and 3.. |
| Apple | We support bullet 1 and 3. For second bullet, we share the views with Nokia, it’s out of RAN1 scope. |
| Nordic | Generally ok, but first we need to agree on the mechanism for re-acquiring of GNSS position fix |
| CMCC | We support bullet 1 and 3, at least GNSS position fix measurement time and GNSS validity duration are necessary for the determination of triggering GNSS measurement and how long the GNSS measurement gap is configured. |
| CATT | We think “GNSS position fix measurement time” is not needed. And a minimum measurement time can be defined, which is used to define a duration after the current validity duration expires, which starts at the time of validity duration expires, and defines the minimum time a UE needed for GNSS measurement. Additionally, an indication that “UE’s GNSS position is available in IoT application layer” can be used. |

  - Summary of First Round Discussion

15 companies provided comments on Initial Proposal 3 in first round discussion.

Bullet 1: GNSS position fix measurement time

- OPPO, ZTE, Ericsson, Spreadtrum, Samsung, Qualcomm, MediaTek, Nokia, Xiaomi, Apple, Nordic, CMCC supported Bullet 1.

- Huawei, HiSilicon mentioned the time duration for GNSS measurement (either GNSS position fix measurement time or measurement gap) and GNSS validity duration are needed.

- CATT mentioned “GNSS position fix measurement time” is not needed. And a minimum measurement time can be defined, which is used to define a duration after the current validity duration expires, which starts at the time of validity duration expires, and defines the minimum time a UE needed for GNSS measurement.

Bullet 2: UE’s GNSS position is available in IoT application layer

- Samsung supported Bullet 2 and Qualcomm, Nordic was OK for Bullet 2.

- OPPO, ZTE, Ericsson mentioned no need of Bullet 2. OPPO further mentioned the UE can simply report an infinite GNSS validity duration with Bullet 3. ZTE, Ericsson further commented that the function of bullet 2 can be covered by bullet 1 with a shorter GNSS position measurement time, e.g., using “0”.

- Nokia mentioned he second bullet is not related to RAN1 but can be represented by first and third bullet, e.g. if UE’s GNSS position is available, then UE may report 0ms measurement time for the next reading. Apple share the views with Nokia, it’s out of RAN1 scope.

- Huawei, HiSilicon mentioned Bullet 2 information is by UE implementation.

- MediaTek mentioned Bullet 2 can be further discussed on how this information will be used and whether it is needed.

Bullet 3: GNSS validity duration as specified in Rel-17

- ZTE, Ericsson, Spreadtrum, Samsung, Huawei, HiSilicon, Qualcomm, MediaTek, Nokia, Xiaomi, Apple, Nordic, CMCC supported Bullet 3.

- OPPO, Lenovo mentioned Bullet 3 already supported in the spec.

Moderator View: RAN1 has made agreements in RAN1 109e “UE reports additional GNSS assistance information and further study the detailed GNSS assistance information, including e.g. GNSS position fix measurement time. Note: Since RAN1 agreed that GNSS validity duration is reported by UE in Rel-17, it is already included in GNSS assistance information.” Hence, the moderator intends to discuss the detailed information of GNSS assistance information. Majority companies are supportive of Bullet 1 (GNSS position fix measurement time) and Bullet 3 (GNSS validity duration as specified in Rel-17). For Bullet 2 (UE’s GNSS position is available in IoT application layer), it can help eNB have better understanding on UE implementation. However, function of Bullet 2 can be covered by Bullet 1 or Bullet 3, whether cover Bullet 2 with a shorter GNSS position measurement time, e.g., using “0” or infinite GNSS validity duration can be further discussed.

  - Second Round Discussion

Second Round Proposal 3-1a:

GNSS assistance information that UE reports to eNB consists of:

- GNSS position fix measurement time

- GNSS validity duration as specified in Rel-17

FFS Signalling to report UE’s GNSS position available in IoT application layer

Second Round Proposal 3-2a:

When eNB triggers UE to make GNSS measurements, UE re-acquires GNSS position fix, signals new GNSS validity duration and GNSS position fix measurement time

  - Updated Second Round Discussion

Based on the 1st offline discuss, the following proposals are updated:

Second Round Proposal 3-1a:

GNSS assistance information that UE reports to eNB at least consists of:

- GNSS position fix time duration for measurement time

- GNSS validity duration as specified in Rel-17

FFS Signalling to report UE’s GNSS position available in IoT application layer

Second Round Proposal 3-2a:

When eNB triggers UE to make GNSS measurements after GNSS validity duration expires, UE re-acquires GNSS position fix, signals new GNSS assistance information as configured by the network validity duration and GNSS position fix measurement time

FFS details of signalling

Companies are encouraged to provide comments within the following table:

| Companies | Comments |
| --- | --- |
| Ericsson | 3-1a: Support 3-2a: Partially Support. It is not clear why “after GNSS validity duration expires” is needed in this proposal. As long as the eNB triggers the UE to performs GNSS position fix in connected, it should report assistance info to the network if configured. The modified proposal is: When eNB triggers UE to make GNSS measurements after GNSS validity duration expires, UE re-acquires GNSS position fix, signals new GNSS assistance information as configured by the network |
| Xiaomi | We are fine with Ericsson’s update. |

4 Issue #4: Closed Loop control

Agreement (RAN1 106e):

For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for IoT-NTN

Agreement (RAN1 109e):

Closed loop time and frequency correction, with potential enhancements, for IoT-NTN is considered to reduce the need for UE to update GNSS position fix in long connection time

  - Company contributing views

| Contribution | Observation/Proposals |
| --- | --- |
| Spreadtrum | Proposal 4: Closed loop frequency correction should not be supported. |
| MediaTek | Observation 1: The TA error due to the position error of a UE that is moving without GNSS measurements can be approximately 5.8 us and 11.6 us within 30 seconds and 60 seconds at UE velocity of 120 km/h in IoT NTN for LEO 600km. Observation 2: For UE velocity above 30 km/h, if no closed loop time correction exists, UE needs to do GNSS position fix within 60 seconds to make TA error due to the position error of a UE smaller than transmit time error 2.6 us in NB-IoT NTN for LEO 600km. Observation 3: TA error caused by UE mobility in TN is larger than that in NTN. Observation 4: The TA error caused by UE mobility in NTN can be taken care of by legacy closed loop time correction in Rel-18. Observation 5: The maximum Doppler shift error due to the position error of a UE that is moving without GNSS measurements at Nadir can be approximately 168.8Hz and 337.7Hz within 60 seconds and 120 seconds at UE velocity of 120 km/h for LEO 600km. At the lower elevation angle 30 degree, the frequency error is in the order of a few Hz for LEO 600km. Observation 6: When GNSS validity duration is below 60 seconds, the frequency error from NTN is smaller than that of TN. RAN1 should first discuss on whether closed loop frequency correction is needed for NTN since there is no such mechanism in legacy TN system. |
| Samsung | Proposal 1: Introduce closed loop pre-compensated frequency offset command signaling, e.g., using a MAC CE similar to current TA command. |
| Nordic Semiconductor ASA | Proposal 1: In RRC connected mode, the closed loop time and frequency correction loops could be considered as a complementary mechanism to GNSS gaps. |
| CMCC | Proposal 5. For long connection, the power consumption of potential mechanisms that enable alternate GNSS measurement and data transmission needs to be evaluated. |
| Qualcomm | Observation 1: Under the studied scenario of short, sporadic connections, a GNSS fix before every connection consumes approximately 34% of the UE’s total available energy. Observation 2: Under the studied scenario of a long connection employing connected mode DRX (with a DRX cycle of ~10s), a GNSS fix before every uplink transmission consumes approximately 45% of the UE’s total available energy without additional enhancements w.r.t uplink synchronization. - This is especially true for mobile UEs that cannot depend on a prior acquired GNSS fix Observation 3: For long connections in eMTC and NB-IoT over NTN, NPRACH-driven closed-loop time and frequency corrections lowers the GNSS power penalty from 45% to 17% (with a GNSS relaxation factor of 4), w.r.t a baseline without closed-loop corrections. Proposal 1: RAN1 to specify NPRACH-driven closed-loop time and frequency corrections, to mitigate UE power consumption on account of GNSS fixes. Observation 4: According to current specifications, any time a UE transmits a NPRACH, it uses a value of NTA=0 in calculating the TA for the NPRACH transmission. Observation 5: If a considerable amount of time has passed since the last GNSS position fix, e.g., for a mobile UE, the accuracy of NTA, adjUE becomes progressively worse over time, i.e., the error in NTA, adjUE increases over time. Observation 6: Although the eNB can progressively correct (by issuing TA commands) the timing error due to a stale UE location, this correction is not applied when transmitting NPRACH (which currently uses NTA=0). This may cause the timing error to go beyond the NPRACH correction capability. Proposal 2: RAN1 to discuss solutions such that the residual TA that is to be corrected by a “closed loop” command (upon transmission of a NPRACH in connected mode), does not exceed the correction capability afforded by such a NPRACH transmission. Candidate solutions include: - Updating the NTA, adjUE term with every TA command received in response to transmitting a NPRACH in connected mode - Accumulating prior TA commands into the NTA term, as opposed to universally setting it to 0 for a NPRACH transmission Proposal 3: Proposal 2—related to TA—shall also translate to an analogous solution for closed-loop residual (doppler) frequency correction. Proposal 4: RAN1 to consider specifying (at least a subset of) NPRACH resources with increased robustness to time and frequency errors, to facilitate: - Accessing a cell from IDLE mode, while relaxing the requirement of an “immediately preceding” GNSS fix in all instances. - Closed-loop corrections (e.g., after periods of UE inactivity), thereby reducing the number of GNSS fixes required during a connection. Observation 7: Restricting alternate starting subcarriers for NPRACH transmissions allows to correct for potentially large initial uplink frequency synchronization errors (e.g., up to 1 kHz) - Such a scheme may facilitate UE power savings by relaxing the frequency and accuracy of GNSS fixes required. - Such a scheme may also facilitate NPRACH-driven closed-loop corrections of time and frequency errors in connected mode, thereby reducing the power penalty from frequent GNSS fixes. |
| Ericsson | Proposal 4 Closed loop frequency correction mechanism should optionally be supported in IoT NTN. |
| Nokia | Observation 1: the release 18 assumption on “long” connection requires the UE can obtain a new GNSS position fix during the connection to minimize the impact on device energy consumption, signaling overhead and latency. Observation 4: UE may not be aware that it is moving during a long connection (uplink repetitions), because it cannot use the GNSS simultaneously. The UE movement will result in misaligned transmission timing. Proposal 4: GNSS error caused by UE movement should be studied and solved. Proposal 5: RAN1 to discuss network detection of UE movement based on uplink transmission drift. Observation 5: Network can attempt closed-loop correction of transmission errors, caused by inacurrate UE location information, but it is not feasible to correct frequency-domain errors. Proposal 10: To save power consumption and latency, keeping RRC connection and new UL synchronization after re-acquiring GNSS should be considered for long term connection, instead of going back to IDLE mode. |
| Lenovo | Proposal 3：Closed loop time and frequency correction can be studied as implementation solution for UL synchronization during long connection. |

Power consumption of GNSS position fix, TA and Doppler tracking error and Closed Loop control are discussed by contributing companies.

On power consumption of GNSS position fix:

- CMCC proposed for long connection, the power consumption of potential mechanisms that enable alternate GNSS measurement and data transmission needs to be evaluated.

- Qualcomm modelled a short, sporadic connection according to Fig. 1 below and a long connection according to Fig. 2 below in R1-2207258.

Figure 1: Short, sporadic transmissions for IoT over NTN.

Figure 2: Long connection with connected mode DRX for IoT over NTN.

And Qualcomm observed that Under the studied scenario of short, sporadic connections, a GNSS fix before every connection consumes approximately 34% of the UE’s total available energy. Under the studied scenario of a long connection employing connected mode DRX (with a DRX cycle of ~10s), a GNSS fix before every uplink transmission consumes approximately 45% of the UE’s total available energy without additional enhancements w.r.t uplink synchronization (This is especially true for mobile UEs that cannot depend on a prior acquired GNSS fix).

- Nokia proposed to save power consumption and latency, keeping RRC connection and new UL synchronization after re-acquiring GNSS should be considered for long term connection, instead of going back to IDLE mode.

On TA and Doppler tracking error of UE mobility:

- MediaTek summarized Table 1-4 from R1-2206140 to show the TA error and frequency error due to the position error of a UE that is moving without GNSS measurements.

Table 1: TA tracking error due to UE mobility for elevation angle 30 degrees

| Validity of UE location | 30 s |  |  | 60 s |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE Velocity | UEpos,error | β | TAerror | UEpos,error | β | TAerror |
| 3 km/h | 25 m | 30 deg | 0.14 us | 50 m | 30 deg | 0.29 us |
| 30 km/h | 250 m | 30 deg | 1.4 us | 500 m | 30 deg | 2.9 us |
| 40 km/h | 333 m | 30 deg | 1.9 us | 666 m | 30 deg | 3.8 us |
| 60 km/h | 500 m | 30 deg | 2.9 us | 1000 m | 30 deg | 5.8 us |
| 120 km/h | 1000 m | 30 deg | 5.8 us | 2000 m | 30 deg | 11.6 us |

Table 2: TA tracking error due to UE mobility for TN

| Validity of UE location UE Velocity | 30 s |  | 60 s |  |
| --- | --- | --- | --- | --- |
|  | UEpos,error | TAerror | UEpos,error | TAerror |
| 3 km/h | 25 m | 0.16 us | 50 m | 0.33 us |
| 30 km/h | 250 m | 1.6 us | 500 m | 3.3 us |
| 40 km/h | 333 m | 2.2 us | 666 m | 4.4 us |
| 60 km/h | 500 m | 3.3 us | 1000 m | 6.6 us |
| 120 km/h | 1000 m | 6.6 us | 2000 m | 13.3 us |

Table 3: Doppler shift tracking error due to UE mobility at Nadir

| Validity of UE location | 10 s |  |  | 30 s |  |  | 60 s |  |  | 120 s |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE Velocity | UEpos,error | θ | Fderror | UEpos,error | θ | Fderror | UEpos,error | θ | Fderror | UEpos,error | θ | Fderror |
| 3 km/h | 8m | 89.999deg | 0.7Hz | 25m | 89.997 deg | 2.1Hz | 50 m | 89.99 deg | 4.2Hz | 100 m | 89.989deg | 8.4Hz |
| 30 km/h | 83m | 89.99deg | 7Hz | 250m | 89.97 deg | 21Hz | 500 m | 89.95 deg | 42Hz | 1000 m | 89.895deg | 84.1Hz |
| 40 km/h | 111m | 89.988deg | 9.3Hz | 333m | 89.96 deg | 28Hz | 666 m | 89.93 deg | 56.1Hz | 1333 m | 89.86deg | 112.2Hz |
| 60 km/h | 166m | 89.98deg | 14Hz | 500m | 89.95 deg | 42.1Hz | 1000 m | 89.9 deg | 84.2Hz | 2000 m | 89.79 deg | 168.5Hz |
| 120 km/h | 333m | 89.96deg | 28.1Hz | 1000m | 89.9 deg | 84.4Hz | 2000 m | 89.79 deg | 168.8Hz | 4000 m | 89.58 deg | 337.7Hz |

Table 4: Doppler shift tracking error due to UE mobility for TN

| Validity of UE location | 10 s |  |  | 30 s |  |  | 60 s |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE Velocity | UEpos,error | θ | Fderror | UEpos,error | θ | Fderror | UEpos,error |  | θ | Fderror |
| 3 km/h | 8 m | 76.6deg | 1.2Hz | 25 m | 54.5deg | 3.2Hz | 50 m |  | 35deg | 4.5Hz |
| 30 km/h | 83m | 22.7deg | 51.2Hz | 250 m | 8deg | 55Hz | 500 m |  | 4deg | 55.4Hz |
| 40 km/h | 111m | 17.5deg | 70.6Hz | 333 m | 6deg | 73.7Hz | 666 m |  | 3deg | 73.9Hz |
| 60 km/h | 166m | 11.8deg | 108.7Hz | 500 m | 4deg | 110.8Hz | 1000 m |  | 2deg | 111Hz |
| 120 km/h | 333m | 6deg | 221Hz | 1000 m | 2deg | 222.1Hz | 2000 m |  | 1deg | 222.2Hz |

And MediaTek observed that TA error due to the position error of a UE that is moving without GNSS measurements can be approximately 5.8 us and 11.6 us within 30 seconds and 60 seconds at UE velocity of 120 km/h in IoT NTN for LEO 600km. For UE velocity above 30 km/h, if no closed loop time correction exists, UE needs to do GNSS position fix within 60 seconds to make TA error due to the position error of a UE smaller than transmit time error 2.6 us in NB-IoT NTN for LEO 600km. TA error caused by UE mobility in TN is larger than that in NTN. The TA error caused by UE mobility in NTN can be taken care of by legacy closed loop time correction in Rel-18. The maximum Doppler shift error due to the position error of a UE that is moving without GNSS measurements at Nadir can be approximately 168.8Hz and 337.7Hz within 60 seconds and 120 seconds at UE velocity of 120 km/h for LEO 600km. At the lower elevation angle 30 degree, the frequency error is in the order of a few Hz for LEO 600km. When GNSS validity duration is below 60 seconds, the frequency error from NTN is smaller than that of TN. RAN1 should first discuss on whether closed loop frequency correction is needed for NTN since there is no such mechanism in legacy TN system.

- Nokia proposed GNSS error caused by UE movement should be studied and solved, and to discuss network detection of UE movement based on uplink transmission drift.to consider a need of signaling mechanism for UE to request a GNSS measurement gap whenever the need for it emerge. Nokia also observed that UE may not be aware that it is moving during a long connection (uplink repetitions), because it cannot use the GNSS simultaneously, then the UE movement will result in misaligned transmission timing.

- Qualcomm observed that According to current specifications, any time a UE transmits a NPRACH, it uses a value of NTA=0 in calculating the TA for the NPRACH transmission, and if a considerable amount of time has passed since the last GNSS position fix, e.g., for a mobile UE, the accuracy of NTA, adjUE becomes progressively worse over time, i.e., the error in NTA, adjUE increases over time, and although the eNB can progressively correct (by issuing TA commands) the timing error due to a stale UE location, this correction is not applied when transmitting NPRACH (which currently uses NTA=0). This may cause the timing error to go beyond the NPRACH correction capability. Then proposed to discuss solutions such that the residual TA that is to be corrected by a “closed loop” command (upon transmission of a NPRACH in connected mode), does not exceed the correction capability afforded by such a NPRACH transmission. Candidate solutions include:

- Updating the NTA, adjUE term with every TA command received in response to transmitting a NPRACH in connected mode

- Accumulating prior TA commands into the NTA term, as opposed to universally setting it to 0 for a NPRACH transmission

On Closed Loop control correction:

- Spreadtrum proposed closed loop frequency correction should not be supported.

- Samsung proposed to introduce closed loop pre-compensated frequency offset command signaling, e.g., using a MAC CE similar to current TA command.

- Nordic proposed in RRC connected mode, the closed loop time and frequency correction loops could be considered as a complementary mechanism to GNSS gaps.

- Qualcomm observed for long connections in eMTC and NB-IoT over NTN, NPRACH-driven closed-loop time and frequency corrections lowers the GNSS power penalty from 45% to 17% (with a GNSS relaxation factor of 4), w.r.t a baseline without closed-loop corrections. And proposed NPRACH-driven closed-loop time and frequency corrections, to mitigate UE power consumption on account of GNSS fixes, a strategy is (approximately) schematically depicted in Fig. 3 below and a robust NPRACH design in Fig. 4 in R1-2207258

Figure 3: Relaxed GNSS fixing using NPRACH-based closed loop corrections.

Figure 4: Example of "restrictions" on starting NPRACH subcarriers for CBRA. Alternate starting subcarriers may be selected for NPRACH transmission by a UE.

Moderator View: Closed-loop time correction is aligned with previous RAN1 agreements. Closed-loop frequency correction is not supported in legacy IoT. RAN1 has agreed that Closed loop time and frequency correction, with potential enhancements, for IoT-NTN is considered to reduce the need for UE to update GNSS position fix in long connection time. The motivation for closed-loop corrections is to improve power consumption with reduced use of GNSS module to re-acquire GNSS position fix as observed by Qualcomm in R1-2207258. And RAN1 should first discuss the impact of GNSS position error to TA error and Frequency error as summarized by MediaTek in Table 1-4 from R1-2206140. Details on whether there are specific solutions for closed loop time/frequency correction to reduce UE power consumption can be further study.

  - First Round Discussion

Initial Proposal 4-1:

RAN1 should first align understanding and analysis on the impact of GNSS position error to TA error and Frequency error on UL synchronization:

- TA error caused by UE mobility in TN and in NTN.

- Frequency error caused by UE mobility in TN and in NTN.

Initial Proposal 4-2:

Companies are encouraged to comment on NPRACH enhancements for NPRACH-driven closed-loop time and frequency corrections:

- Accumulating prior TA commands into the NTA term instead of setting it to 0 as in Rel-17

- Specifying (at least a subset of) NPRACH resources with increased robustness to time and frequency errors

Companies are encouraged to provide comments within the following table:

| Companies | Comments |
| --- | --- |
| Lenovo | For proposal 4-1, OK for further discuss and OK with the way forward. For proposal 4-2, it seems NPRACH enhancement can be discussed after the fundamental issues are stable. |
| ZTE | For proposal 4-1, support. For proposal 4-2, we do not see the need of discussing NPRACH related enhancement before agreeing the fundamental solution. |
| Ericsson | 4-1: Support. 2-2: The specific solutions can be considered at a later stage once the company understanding on TA/frequency error is aligned. |
| Spreadtrum | We support proposal 4-1. |
| Samsung | We support proposal 4-1. |
| Huawei, HiSilicon | For close loop control, including TA and frequency offset compensation, it can reduce the attempts of GNSS position fix to some extent. However, we think it cannot replace GNSS position fix in connected mode as the TA error caused by UE’s movement between two TACs may still beyond the TAC range. We are fine to further evaluate the impact of TA error and Frequency error mentioned in proposal 4-1. For proposal 4-2, it is detail design we could determine after the which close loop control(s) are selected. |
| Qualcomm | Support 4-1 and 4-2. At least w.r.t defining a new “base TA” for NPRACH, we believe companies will realize that this is an essential issue, since the accumulated TA can exceed the rang that can be corrected by a NPRACH. |
| MediaTek | Proposal 4-1: Support Proposal 4-2: Generally supportive of allowing UE to transmit RACH after GNSS validity expires to reduce impact of GNSS on UE power consumption. More understanding of need for potential enhancements for RACH in RAN1 is needed, It can be further discussed. |
| Nokia, NSB | For Initial Proposal 4-1, OK. For Initial Proposal 4-2, we do not think PRACH-driven close-loop procedure can solve the frequency correction. Additionally, PRACH-driven solution will cause much pressure to PRACH capacity, especially when many UE processing the procedure at similar time, which may impact on system performance for both initial access and close-loop adjustment. |
| Xiaomi | Support proposal 4-1. For proposal 4-2, more clarification is needed. |
| Apple | Proposal 4-2 is ok. |
| Nordic | We support both proposals |
| CMCC | For Initial Proposal 4-1, we are fine. For Initial Proposal 4-2, we are open to further discuss the closed-loop time and frequency corrections methods. |
| CATT | For proposal 4-1, we support. |

  - Summary of First Round Discussion

14 companies provided comments on Initial Proposal 4-1 and Initial Proposal 4-2 in first round discussion.

On Initial Proposal 4-1:

- 13 companies supported Initial Proposal 4-1.

- Huawei, HiSilicon mentioned for close loop control, including TA and frequency offset compensation, it can reduce the attempts of GNSS position fix to some extent. However, it cannot replace GNSS position fix in connected mode as the TA error caused by UE’s movement between two TACs may still beyond the TAC range.

On Initial Proposal 4-2:

- Qualcomm, Apple, Nordic supported Initial Proposal 4-2. Qualcomm further mentioned at least w.r.t defining a new “base TA” for NPRACH, since the accumulated TA can exceed the rang that can be corrected by a NPRACH.

- MediaTek Generally supportive of allowing UE to transmit RACH after GNSS validity expires to reduce impact of GNSS on UE power consumption. More understanding of need for potential enhancements for RACH in RAN1 is needed.

- Lenovo, ZTE, Ericsson, Huawei, HiSilicon mentioned NPRACH enhancement can be discussed later, after aligning understanding of TA/frequency error and agreeing the fundamental solution in other issues.

- Nokia mentioned PRACH-driven close-loop procedure cannot solve the frequency correction. Additionally, PRACH-driven solution will cause much pressure to PRACH capacity, especially when many UE processing the procedure at similar time, which may impact on system performance for both initial access and close-loop adjustment.

- CMCC mentioned open to further discuss the closed-loop time and frequency corrections methods.

Moderator View: Almost all contributing companies in first round discussion are supportive of Initial Proposal 4-1. Hence RAN1 should first discuss the impact of GNSS position error to TA error and Frequency error. Whether there are specific solutions for closed loop time/frequency correction, e.g. PRACH-driven enhancements, to reduce UE power consumption can be further study.

  - Second Round Discussion

Second Round Proposal 4-1a:

RAN1 should first align understanding and analysis on the impact of GNSS position error to TA error and Frequency error on UL synchronization:

- TA error caused by UE mobility in TN and in NTN.

- Frequency error caused by UE mobility in TN and in NTN.

Second Round Proposal 4-2a:

If GNSS validity expires and closed-loop corrections are applied, without eNB triggering UE to make GNSS measurements, study whether NPRACH enhancements for NPRACH-driven closed-loop time and frequency corrections may be needed

Companies are encouraged to provide comments within the following table:

| Companies | Comments |
| --- | --- |
| Ericsson | 4-1a: Support. 4-2a: The specific solutions can be discussed at a later stage once RAN1 has made progress as suggested in 4-1a. |
| Xiaomi | 4-1a, not sure how this proposal is related to the TN. |

5 MISC [Low Priority]

The WID objective is copied below for reminder

Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1]

Based on the moderator understanding of the Rel-18 IoT NTN WID objectives and conclusions / agreements in Rel-17 IoT NTN Work Item, the following FL recommendations are made for sub-sections 5.1, 5.2, 5.3, 5.4.

5.1 GNSS measurement in idle mode

5.1.1 Company contributing views

| Contribution | Observation/Proposals |
| --- | --- |
| Huawei, HiSilicon | Proposal 3: Support UE trigger GNSS measurement during DL/UL idle period and report updated validity duration. |
| Nokia | Proposal 8: RAN1 to define when GNSS reacquisition is needed for short RRC Idle periods during a long RRC connection. |
| Apple | Proposal 3: RAN1 is to consider the interaction between GNSS measurement and C-DRX. |

Huawei, HiSilicon proposed to support UE trigger GNSS measurement during DL/UL idle period and report updated validity duration. For example, GNSS position fix may happen during the inactive state of UE’s C-DRX. It will be a waste of resources if GNSS position fix is taken too early as GNSS information is not needed during the remaining time of inactive state. UE can take GNSS position fix just before it enters into active sate as illustrated in Figure 2 form R1-2205858. And accordingly, the validity of GNSS can be considered to have started at the time when UE enter into active state. MediaTek mentioned in their contribution assuming a hot fix of 1-2 seconds and re-sycnhronization time in the order of several hundred ms, the total time will be consistent with connected DRX of 2.56 s. With this scheme, the resources for GNSS position fix can be saved especially when the duration of inactive sate is much longer than the validity of GNSS position fix.

Figure 2 GNSS measurement during DL/UL idle period

Nokia proposed to define when GNSS reacquisition is needed for short RRC Idle periods during a long RRC connection.

Apple proposed to consider the interaction between GNSS measurement and C-DRX.

Moderator View: On long connections also imply that there may be “periods of inactivity”, after which the UE reconnects to the cell. For such cases, RAN1 should align understanding on whether UE can re-use a previously acquired GNSS fix.

5.1.2 First Round Discussion

FL recommendation – 5.1:

Companies can further discuss to align understanding on long connections also imply that there may be “periods of inactivity” during C-DRX, after which the UE re-enters active state. And further discuss on whether for such cases, UE can re-use a previously acquired GNSS fix.

Companies are encouraged to provide comments on the FL recommendation within the following table:

| Companies | Comments |
| --- | --- |
| Ericsson | Why should the UE not use a previously acquired GNSS fix if it is valid? On when to acquire GNSS position fix during C-DRX, it makes sense if the UE acquires it before transitioning to active mode, but the GNSS timer should be started when the UE acquires the GNSS position fix. |
| Nokia, NSB | Agree. We think the long connection may imply some inactivity duration which can be short idle/connection duration. |
| Apple | Agree the understanding, whether UE needs to perform the GNSS measurement before re-entering the active state, it’s depending on whether UE has the valid GNSS fix. |

5.2 Acquisition of assistance information in Connected

The moderator recalls the RAN1 and RAN2 agreements for acquisition of assistance information for UL time/frequency synchronization in Connected.

Agreement (RAN1 107-e):

The serving satellite ephemeris and common TA related parameters are signalled in the same SIB message and have the same epoch time.

Agreement (RAN1 107-e):

A single validity duration for both serving satellite ephemeris and common TA related parameters is broadcast on the SIB.

Agreement (RAN1 107-e):

NTN validity duration is configured per cell and indicated to the UE in X bits with:

- Value range { 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240, Infinity}

- Unit is second

- FFS (to be resolved in current meeting): Additional values for GEO

Agreement (RAN1 108-e):

First discuss for additional values of validity timer for GEO in NR NTN AI 8.4.2. For IoT NTN, adopt the NR NTN agreement without modification for additional values of validity timer for GEO.

Agreement (RAN1 NR NTN 108-e):

Add one additional NTN validity duration value for GEO i.e. 900 seconds. X = 4 bits.

RAN2-116bis

When SI used for UL synch (pre-compensation) is no longer valid, the UE autonomously tunes away and re-aquires the required SI, and then comes back. FFS whether anything additional is needed.

RAN2-117

FFS if we Will have a guard timer to handle the case where the UE takes ‘forever’ reacquire the SIB. At timer expiry UE triggers RLF handling. (Note that it is expected that the timer will not expire in the normal case, and the UE can just come back acc to previous decision).

RAN2 assumes Upon recovery from loss of precomp synch while TAT has not expired, UE resumes UL operation, no RACH is needed.

When the UE tunes away, it is assumed that the UE may not receive DL dedicated transmissions, actions in the DL can be left to UE implementation.

There is some support for enhancements for long data transmissions, which could be Rel-18.

Introduce a guard timer TXXXX for SIBXX acquisition in connected mode. At TXXX expiry, UE triggers RLF (if it can be shown in Q2 that UE will loose RLM when UE tunes away, it can be discussed to skip this timer)

Upon timer expiry (or UE tune away), UE stops all UL transmissions, flushes all HARQ buffers and maintains all UL resources.

5.2.1 Company contributing views

| Contribution | Observation/Proposals |
| --- | --- |
| MediaTek | Observation 10: For acquisition of assistance information in Connected, Rel-17 solutions can be utilized in Rel-18 with no enhancement. |
| xiaomi | Proposal 3: The IoT UE can acquire the assistant information from SIBx in connected mode. |
| Nokia | Observation 10: The ephemeris validity timer may expire during a UE’s uplink transmission using repetitions. Proposal 14: How to acquire ephemeris and update TA during a long sequence of repetitions should be studied. |

MediaTek observed for acquisition of assistance information in Connected, Rel-17 solutions can be utilized in Rel-18 with no enhancement. RAN2 had discussed on acquisition of assistance information, and related agreements were made. Based on RAN2 agreements, the UE may ‘forever’ reacquire the SIB with the new introduced guard timer TXXXX for SIBXX acquisition in connected mode. In Rel-18, UE can utilize the same procedure to forever’ reacquire the SIB during long connection times.

RAN2-116bis - When SI used for UL synch (pre-compensation) is no longer valid, the UE autonomously tunes away and re-aquires the required SI, and then comes back. FFS whether anything additional is needed. RAN2-117 - FFS if we Will have a guard timer to handle the case where the UE takes ‘forever’ reacquire the SIB. At timer expiry UE triggers RLF handling. (Note that it is expected that the timer will not expire in the normal case, and the UE can just come back acc to previous decision). - RAN2 assumes Upon recovery from loss of precomp synch while TAT has not expired, UE resumes UL operation, no RACH is needed. - When the UE tunes away, it is assumed that the UE may not receive DL dedicated transmissions, actions in the DL can be left to UE implementation. - There is some support for enhancements for long data transmissions, which could be Rel-18. - Introduce a guard timer TXXXX for SIBXX acquisition in connected mode. At TXXX expiry, UE triggers RLF (if it can be shown in Q2 that UE will loose RLM when UE tunes away, it can be discussed to skip this timer) - Upon timer expiry (or UE tune away), UE stops all UL transmissions, flushes all HARQ buffers and maintains all UL resources.

Xiaomi proposed the IoT UE can acquire the assistant information from SIBx in connected mode.

Nokia proposed to study how to acquire ephemeris and update TA during a long sequence of repetitions, considering the ephemeris validity timer may expire during a UE’s uplink transmission using repetitions.

Moderator View: This issue was discussed in RAN1 109e, there was FL recommendation “Companies can further align understanding on whether and how re-acquiring the NTN SIB with ephemeris and common TA parameters could improve GNSS operations, and whether there is need and gains for further potential enhancements for re-acquiring NTN SIB in addition to Rel-17.” Proponents of further enhancements on the topic are encouraged to further discuss offline with companies to get more support.

5.2.2 First Round Discussion

FL recommendation – 5.2:

Proponents of further potential enhancements for re-acquiring NTN SIB in addition to Rel-17 are encouraged to further discuss offline with companies to get more support.

Companies are encouraged to provide comments on the FL recommendation within the following table:

| Companies | Comments |
| --- | --- |
| Nokia, NSB | We suggest companies to consider the long repetition case that may happen in legacy TN and even with higher probability in NTN, which may request the UE to read the NTN SIB in the repetition, or the long repetition may never be supported or with large latency to wait for the SIB reading and cause reduced serving time from the serving cell. |

5.3 Enhanced UL segmented transmission

5.3.1 Company contributing views

| Contribution | Observation/Proposals |
| --- | --- |
| Nokia | Observation 8: Over a long uplink transmission the elevation angle change will cause large variation of TA drift rate. Observation 9: Different segment sizes may be needed depending on the TA drift rate. Proposal 12: RAN1 to discuss how to configure multiple segment sizes for an uplink transmission. Proposal 13: How to reduce the TA error for repetitions in the segment should be considered and discussed. |

Nokia proposed to discuss how to configure multiple segment sizes for an uplink transmission and how to reduce the TA error for repetitions in the segment. Over a long uplink transmission, the elevation angle change will cause large variation of TA drift rate and different segment sizes may be needed depending on the TA drift rate.

Moderator View: FL recommendation in RAN1#109 was UL segmented transmission has been discussed in Rel-17. RAN1 are specifying UL segmented transmission in Rel-17. Enhancements to UL segmented transmissions are not in scope of Rel-18 IoT NTN WID. It can be de-prioritized in Rel-18. UL segmented transmission has been discussed in Rel-17. RAN1 are specifying UL segmented transmission in Rel-17. Enhancements to UL segmented transmissions are not in scope of Rel-18 IoT NTN WID. It can be de-prioritized in Rel-18. Proponents of further enhancements on the topic are encouraged to further discuss offline with companies to get more support.

5.3.2 First Round Discussion

FL recommendation- 5.3:

Proponents of further UL segmented transmission enhancements are encouraged to further discuss offline with companies to get more support.

Companies are encouraged to provide comments on the FL recommendation within the following table:

| Companies | Comments |
| --- | --- |
| Nokia, NSB | We suggest companies to consider the segment changing during the long repetition that may last for e.g. 40s for the coverage of IoT UE. If not supported, always the largest segment size for the duration should be selected, causing performance loss and even packet error. How to reduce the time error should be also considered considering the system performance. |

5.4 Repetition continuation between two NTN cells

5.4.1 Company contributing views

| Contribution | Observation/Proposals |
| --- | --- |
| Nokia | Proposal 15: RAN1 should discuss the issue of repetition continuation between two NTN cells. |

Nokia proposed discussing the issue of repetition continuation between two NTN cells. The transmission times of 10s and 40s, can be larger than the time the UE is served by a single cell in the LEO scenarios. For example, the maximum coverage time of one cell may be 50 km / 7.56 km/s =6.6 s based on the assumption of 50km satellite beam diameter for set 1, or 234 km /7.56 km/s = 31s for set 3 with 234km satellite beam diameter. When considering the transparent scenario, at least intra-satellite mobility would entail the two cells most likely originate from the same eNB and thus transfer of received bits would be an eNB-internal process. The target would be to enable the data transfer to continue after a cell reselection instead of restarting. Likewise, the procedure could be for handovers in eMTC and should support both uplink and downlink data transfer. Thus, it is suggested that RAN1 can consider if such continuation is feasible on the PHY layer in terms of keeping soft bits/repetition data, while solutions on higher layer RLC can also be envisioned. This is required to support the long connection times, which are envisioned by the objective in release 18

Moderator View: FL recommendation in RAN1#109 was de-prioritize repetition continuation between two NTN cells as it is not in scope of Rel-18 IoT NTN WID. RAN2 has made agreements “No enhancement to R16 CHO are introduced in R17.” in RAN2 116e. It needs to be clarified that in Rel-17 IoT NTN, HO is only for eMTC. In NB-IoT, there is no HO. Further, when there is HO the HARQ buffers are flushed (for eMTC). Repetition continuation between two NTN cells is not in scope of Rel-18 in moderator view. Impact on RAN1/RAN2 specs is likely to be significant. Proponents of further enhancements on the topic are encouraged to further discuss offline with companies to get more support

5.4.2 First Round Discussion

FL Recommendation – 5.4:

Proponents of further repetition continuation between two NTN cells enhancements are encouraged to further discuss offline with companies to get more support.

Companies are encouraged to provide comments on the FL recommendation within the following table:

| Companies | Comments |
| --- | --- |
| Nokia, NSB | We suggest companies to consider how to reduce the power consumption of IoT NTN UE when a long repetition may be wasted before handover/cell reselection or end of serving time from the serving cell, which may cause the UE to be active for more time and more accessing to next cell for the remaining data transmission. |

5.5 Summary of First Round Discussion

Given the limited views expressed during first round, Companies can provide further comments on the FL recommendation within the First Round Discussion table.

6 Proposals for online/offline section

6.1 Proposals for online section Wednesday 11:00-12:00

Issue #3: Report GNSS assistance information

Second Round Proposal 3-1a:

GNSS assistance information that UE reports to eNB at least consists of:

- GNSS position fix time duration for measurement

- GNSS validity duration

Second Round Proposal 3-2a:

When eNB triggers UE to make GNSS measurements after GNSS validity duration expires, UE re-acquires GNSS position fix, signals GNSS assistance information as configured by the network

FFS details of signalling

Issue #4: Closed Loop control

Second Round Proposal 4-1a:

RAN1 should first align understanding and analysis on the impact of GNSS position error to TA error and Frequency error on UL synchronization:

- TA error caused by UE mobility in TN and in NTN.

- Frequency error caused by UE mobility in TN and in NTN.

Issue#1: GNSS measurement triggering in connected

Second Round Proposal 1-1a:

eNB triggers connected UE to make a GNSS measurement for new GNSS position fix

- Before GNSS validity duration expires

- After GNSS validity duration expires and closed-loop corrections are applied

FFS Additional signalling

Issue #2: Schemes for GNSS measurement in connected

Second Round Proposal 2-1a:

If eNB triggers connected UE to make GNSS measurement, UE re-acquires GNSS position fix with a new gap

Second Round Proposal 2-2a:

If eNB does not trigger UE to make GNSS measurements, UE can re-acquire GNSS position fix based on a new GNSS measurement timer

- FFS Configuration of new GNSS measurement timer

- FFS When the UE starts new GNSS measurement timer

6.2 Proposal for offline section Tuesday 14:30-15:30

Issue #3: Report GNSS assistance information

Second Round Proposal 3-1a:

GNSS assistance information that UE reports to eNB consists of:

- GNSS position fix measurement time

- GNSS validity duration as specified in Rel-17

FFS Signalling to report UE’s GNSS position available in IoT application layer

Second Round Proposal 3-2a:

When eNB triggers UE to make GNSS measurements, UE re-acquires GNSS position fix, signals new GNSS validity duration and GNSS position fix measurement time

Issue #4: Closed Loop control

Second Round Proposal 4-1a:

RAN1 should first align understanding and analysis on the impact of GNSS position error to TA error and Frequency error on UL synchronization:

- TA error caused by UE mobility in TN and in NTN.

- Frequency error caused by UE mobility in TN and in NTN.

Issue#1: GNSS measurement triggering in connected

Second Round Proposal 1-1a:

eNB triggers connected UE to make a GNSS measurement for new GNSS position fix

- Before GNSS validity duration expires

- After GNSS validity duration expires and closed-loop corrections are applied

FFS Additional signalling

Issue #2: Schemes for GNSS measurement in connected

Second Round Proposal 2-1a:

If eNB triggers connected UE to make GNSS measurement, UE re-acquires GNSS position fix with a new gap

Second Round Proposal 2-2a:

If eNB does not trigger UE to make GNSS measurements, UE can re-acquire GNSS position fix based on a new GNSS measurement timer

- FFS Configuration of new GNSS measurement timer

- FFS When the UE starts new GNSS measurement timer

7 Conclusion

The following conclusion and agreements were made in RAN1#110

8 References

RP-221806, Moderator (MediaTek), Revised WID on IoT NTN enhancements, June 6th – 9th, 2022

R1-2205858, Discussion on improved GNSS operations for IoT NTN, Huawei, HiSilicon

R1-2206011, Discussion on improved GNSS operations for IoT NTN, Spreadtrum Communications

R1-2206023, Discussion on improved GNSS operation for IoT-NTN, ZTE

R1-2206140, Improved GNSS operations for IoT NTN, MediaTek Inc.

R1-2206313, Discussion on improved GNSS operations for IoT NTN, OPPO

R1-2206389, Improved GNSS operations for IoT NTN, CATT

R1-2206633, Discussion on improved GNSS operation for IoT NTN, Xiaomi

R1-2206851, Improved GNSS operations for IoT NTN, Samsung

R1-2206882, Improved GNSS operations for IoT NTN, Nordic Semiconductor ASA

R1-2206934, Discussion on improved GNSS operations for IoT NTN, CMCC

R1-2207151, On improved GNSS operation for IoT-NTN, InterDigital, Inc.

R1-2207258, Improved GNSS Operations for IoT-NTN, Qualcomm Incorporated

R1-2207259, On improved GNSS operation in IoT NTN, Ericsson Limited

R1-2207292, Enhancements for long connections in NB-IoT/eMTC over NTN, Nokia, Nokia Shanghai Bell

R1-2207296, Improved GNSS operations for IoT NTN, Lenovo

R1-2207356, On Improved GNSS Operations for IoT NTN, Apple