3GPP TSG RAN WG1 Meeting #110					                     R1-2207767

Toulouse, France, August 22nd – 26th, 2022

Agenda item:	8.11

Source:		Moderator (LG Electronics)

Title:	Feature lead summary #2 for AI 8.11 Inter-UE coordination for Mode 2 enhancements

Document for:	Discussion and information

- Introduction

The inter-UE coordination issues in contributions [1-21] submitted to RAN1#110 meeting are summarized in the table of Sections 3. Also an initial assessment on each issue is provided based on the following classification:

- High priority (H):

  - High-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N):

  - All other purposes such as spec optimization and low priority issues

- Editorial (E):

  - Editorial issues that will be handled as editorial CRs (to be communicated to the editors/chairs)

- Preparation for maintenance

  - Collection of issues for Inter-UE coordination for Mode 2 enhancements

| Issue# | Issue | References | FL initial assessment |
| --- | --- | --- | --- |
| 1 | [Scheme 1] Addition of clarification for the relationship between start/end slots of resource selection window used for SL transmission carrying IUC information and start/end slots of resource selection window for determining the set of resources | [2], [3], [5], [6], [7] | H |
| 2 | [Scheme 1] Addition of clarification in the specification that UE-A is the destination UE of a TB transmitted by UE-B for the case when IUC information is triggered by an explicit request from UE-B and UE-A determines the set of non-preferred resources for UE-B | [2] | H |
| 3 | [Scheme 1] Further clarification on the resource pool used for determining the set of resources and transmitting IUC information | [4], [15] | H |
| 4 | [Scheme 1] Further clarification on how to deal with the different IUC information decoded by SCI format 2-C and MAC CE | [4] | N |
| 5 | [Scheme 1] Further clarification on conditions under which Option B can be used for the received preferred resource set | [4], [19] | N |
| 6 | [Scheme 1] Further clarification on obtaining the set of resources based on the received resource combination indication | [7] | N |
| 7 | [Scheme 1] Modification to UE-B’s behavior of excluding the non-preferred resource set from candidate single-slot resources | [7] | N |
| 8 | [Scheme 1] Further clarification on UE-B’s behavior of performing the random resource selection with the received non-preferred resource set - Note that how to handle this issue in RAN2 LS of R1-2205728 will be dependent on the outcome of online discussion for AI 5 | [7], [11], [13], [21] | H |
| 9 | [Scheme 1] Further clarification on UE-A's behavior when it decodes one container (e.g., SCI format 2C) successfully but fails to decode the other container (e.g., MAC CE) | [8] | N |
| 10 | [Scheme 1] Further clarification on UE’s behavior related to RRC parameters of sl-TriggerConditionRequest and sl-Condition1-A-2 - Note that how to handle this issue in RAN2 LS of R1-2205727 will be dependent on the outcome of online discussion for AI 5 | [10] | H |
| 11 | [Scheme 1] Further clarification on UE's behavior regarding the transmissions of explicit request and IUC information based on the latency bound (e.g., conditions under which UE-B is allowed to transmit new explicit request, whether to allow condition-based IUC information transmission within this latency bound) | [12] | N |
| 12 | [Scheme 1] Introduction of additional conditions that UE-B uses the received non-preferred resource set for its resource selection | [12] | N |
| 13 | [Scheme 1] Addition of clarification on missing field descriptions and functions of SCI format 2-C | [12], [20] | H |
| 14 | [Scheme 1] Further clarification on how IUC information related fields in SCI format 2-C indicates each unit/tuple | [12] | N |
| 15 | [Scheme 1] Addition of clarification for cast type(s) of Scheme 1 and/or SCI format 2-C to the specification | [13], [19] | H |
| 16 | [Scheme 1] Further clarification on how UE-A determines the set of preferred resources when the time gap from IUC transmission to the preferred resources is larger than (Tproc,0SL+Tproc,1SL+Tproc,2SL) | [15] | N |
| 17 | [Scheme 1] Introduction of procedure that allows UE-B to distinguish between non-preferred resource generated based on different conditions (i.e., Condition 1-B-1, Condition 1-B-2) | [17] | N |
| 18 | [Scheme 1] Introduction of field of priority value in MAC CE carrying IUC information with the preferred resource set | [18] | N |
| 19 | [Scheme 1] Further clarification on the use of preferred resource set in the resource reselection due to re-evaluation/pre-emption/conflict information | [19] | H |
| 20 | [Scheme 1] Correction for the reference of the priority field of SCI format 2-C | [20] | N |
| 21 | [Scheme 2] Further clarification on conditions for UE to be UE-B when at least one of UEs scheduling conflicting TBs does not set indicationUEB flag to 1 | [1], [5], [14], [16], [18], [19] | H |
| 22 | [Scheme 2] Further clarification on the description of “valid PSFCH occasion” in the current specification | [2], [21] | N |
| 23 | [Scheme 2] Addition of clarification in the specification about the RSRP threshold(s) used for determining the expected/potential conflict(s) in Option 1 and/or Option 4. | [4], [9], [18], [19] | H |
| 24 | [Scheme 2] Further clarification on conditions for UE to be UE-B in Condition 2-A-2 | [5] | H |
| 25 | [Scheme 2] Further clarification on how to determine the priority for the resource conflict indication with Condition 2-A-2 | [12], [18] | N |
| 26 | [Scheme 2] Further clarification on PSFCH occasion indication in terms of determines a number of PSFCH resources available for multiplexing conflict information in a PSFCH transmission | [12] | N |
| 27 | [Scheme 2] Further clarification on PSFCH occasion validity of UEs scheduling the conflicting TBs in order for UE-A to determine whether to transmit the conflict indication | [19] | N |
| 28 | [Scheme 2] Further clarification on UE-A’s behavior of transmitting PSFCH for the conflict indication when multiple reserved resources are indicated by UE-B’s SCI under the situation where the PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted | [19] | N |
| 29 | [Scheme 2] Further clarification on determining UE-B when sl-TypeUE-A is enabled or not enabled | [20] | N |
| 30 | [Scheme 2] Further clarification on PSFCH prioritization related to the conflict indication | [20] | N |
| 31 | [Scheme 2] Further clarification on priority definition for Tx and Rx of PSFCH with conflict information | [21] | N |
| 32 | [Scheme 2] Further clarification on Rx UE behavior if the timeline for conflict indication is not fulfilled | [21] | N |
| 33 | [Scheme 1/2] Correction on RRC parameter name and value misalignment | [9], [15], [19], [20] | H |

  - Summary of companies’ inputs on the maintenance issues to be treated

| Issue# | FL initial assessment | Companies’ inputs |
| --- | --- | --- |
| 1 | H | - H : 14 - N : 1 - E: |
| 2 | H | - H : 8 - N : 7 - E: |
| 3 | H | - H : 11 - N : 1 - E: 1 |
| 4 | N | - H : 1 - N : 14 - E: |
| 5 | N | - H : 1 - N : 14 - E: |
| 6 | N | - H : 1 - N : 14 - E: |
| 7 | N | - H : 1 - N : 13 - E: 1 |
| 8 | H | - H : 13 - N : 1 - E: |
| 9 | N | - H : 2 - N : 13 - E: |
| 10 | H | - H : 14 - N : - E: |
| 11 | N | - H : 1 - N : 14 - E: |
| 12 | N | - H : 1 - N : 14 - E: |
| 13 | H | - H : 14 - N : - E: |
| 14 | N | - H : 1 - N : 14 - E: |
| 15 | H | - H : 13 - N : 2 - E: |
| 16 | N | - H : 1 - N : 14 - E: |
| 17 | N | - H : 2 - N :13 - E: |
| 18 | N | - H : - N : 15 - E: |
| 19 | H | - H : 10 - N : 4 - E: |
| 20 | N | - H : - N : 12 - E: 3 |
| 21 | H | - H : 11 - N : 4 - E: |
| 22 | N | - H : 4 - N : 11 - E: |
| 23 | H | - H : 15 - N : - E: |
| 24 | H | - H : 14 - N : 1 - E: |
| 25 | N | - H : 2 - N : 13 - E: |
| 26 | N | - H : 1 - N : 14 - E: |
| 27 | N | - H : 1 - N : 14 - E: |
| 28 | N | - H : 1 - N : 14 - E: |
| 29 | N | - H : - N : 15 - E: |
| 30 | N | - H : - N : 15 - E: |
| 31 | N | - H : 2 - N : 13 - E: |
| 32 | N | - H : 1 - N : 13 - E: 1 |
| 33 | H | - H : 8 - N : - E: 6 |

  - FL's recommendation

Based on the summary in Section 2.2, the following maintenance issues of Inter-UE coordination for Mode 2 enhancements are recommended to be addressed in this meeting. According to the decisions made in AI 5, Issue 8 and 10 for Scheme 1 and Issue 23 for Scheme 2 will be dealt with as separate email discussions. Also Issue 3 for Scheme 1 is already covered in the email discussion of [AT119-e][510][V2X/SL] in RAN2.

- Recommendation of maintenance issues for “Inter-UE coordination for Mode 2 enhancements” to be addressed in RAN1#110 meeting

  - Scheme 1

    - [H] Issue 1

    - [H] Issue 13

    - [H] Issue 15

    - [[H] Issue 19]

  - Scheme 2

    - [H] Issue 21

    - [H] Issue 24

  - Scheme 1 and 2

    - [[E] Issue 33]

- Discussion

  - Scheme 1

    - [CLOSED] Issue#1: Addition of clarification for the relationship between start/end slots of resource selection window used for SL transmission carrying IUC information and start/end slots of resource selection window for determining the set of resources

      - Background

Five contributions [2][3][5][6][7] proposed that it is necessary to capture the missing agreement below in the specification.

- Agreement:

  - X1, X2, and X3 are determined by UE-A’s implementation under the constraints defined in the specification (e.g., SL-LatencyBoundIUC-Report-r17, requirement of T_2min)

    - UE-B can choose to not use any resource from the preferred/non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission

      - For Tproc,2,

        - When only MAC CE is used for inter-UE coordination information transmission, it is equal to (Tproc,0+Tproc,1)

        - When MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission, it is equal to Tproc,0

          - Note: this is assuming that SCI format 2-C is received

    - Whether or not to make the time gap from the resource of inter-UE coordination information transmission to preferred/non-preferred resource in the inter-UE coordination information larger than (Tproc,0+Tproc,1+Tproc,2) is up to UE-A implementation

[Huawei, R1-2205766] Summary of change: Specify the permitted UE behaviour subject to timing relationships between the resource selection window for determining the set of resources and the resource selection window for transmitting IUC information, for use of non-preferred resource set. Consequence if not approved: The permitted UE behaviors subject to timing relationships between the resource selection window for determining the set of resources and the resource selection window for transmitting IUC information is incompletely specified for non-preferred resources. Proposal 4: For scheme 1 inter-UE coordination information, for both triggered by UE-B’s explicit request and triggered by a condition other than explicit request reception, relationship between start/end slots of resource selection window for IUC transmission and that for determining the set of resources should be captured, as per the proposed TP from Editor of TS 38.214. We provide the Text Proposal for section 8.1.4C of TS 38.214 below. ---------------------------- Start of Text Proposal for TS 38.214 ----------------------------- < Unchanged parts are omitted > #### 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. Note 2: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than Tproc,0+Tproc, 1+Tproc, 2 after the resource of inter-UE coordination information transmission, where Tproc, 2 is equal to Tproc,0+Tproc, 1 when only MAC CE is used for inter-UE coordination information transmission, or Tproc, 2 is equal to Tproc,0 when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission assuming SCI format 2-C is received. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ---------------------------------- [LG Electronics, R1-2205848] Proposal: Adopt the following text proposal to TS 38.214 v17.2.0: - Reason for change: - The RAN1 agreement related to UE-B's behaviour of determining a resource used for its resource (re-)selection among the non-preferred resource set received from UE-A is not captured in the specification. - Summary of change: - The relevant UE-B's behaviour is described in Section 8.1.4C of TS 38.214. - Consequences if not approved: - The specification text is unclear as to which resource UE-B uses for its resource (re-)selection among the non-preferred resource set received from UE-A --------------------------- Start of text proposal to TS 38.214 v17.2.0 --------------------------- 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. Note: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission, where Tproc,2 is equal to (Tproc,0+Tproc,1) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2 is equal to Tproc,0 when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission and SCI format 2-C is received. --------------------------- End of Text proposal to TS 38.214 v17.2.0 ---------------------------. [ZTE, R1-2206096] Proposal 5: Adopt the following TP to implement the agreement on the use of resource in a non-preferred resource set 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. Note2: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission, where Tproc,2 is equal to (Tproc,0+Tproc,1) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2 is equal to Tproc,0 when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission assuming SCI format 2-C is received. [OPPO, R1-2206283] -------------------------------- Start of Text Proposal for TS 38.214 --------------------------------- 8.1.4C UE procedure for using a received non-preferred resource set [OPPO, R1-2206283] -------------------------------- Start of Text Proposal for TS 38.214 --------------------------------- 8.1.4C UE procedure for using a received non-preferred resource set <Unchanged parts omitted> Note2: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission, where Tproc,2 is equal to (Tproc,0+Tproc,1) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2 is equal to Tproc,0 when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission assuming and SCI format 2-C is received. --------------------------------------------End of Text Proposal ----------------------------------------- Proposal 8: Endorse one of the TPs in Section 3 for the agreement of last meeting on upper/lower bounds of start/end slot of resource selection window used for sidelink transmission carrying inter-UE coordination information. Proposal 9: Send an LS to RAN2 to capture the preferred resource set part of the agreement after the CR for 38.214 is decided. [CATT, R1-2206360] - TP#5 Reason for change:; Corrections on NR Sidelink enhancements. Summary of change:; In clause 8.1.4C, capture the agreement regarding the criterion of using non-preferred resource set achieved in RAN1#109-e.. Consequences if not approved:; Do not capture RAN1 previous agreements. ---------------------------------------------- Start of text proposal to TS 38.214 v17.2.0---------------------------------------------- 8.1.4C UE procedure for using a received non-preferred resource set <<< UNCHANGED PARTS OMITTED >>> Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. Note2: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission, where Tproc,2 is equal to (Tproc,0+Tproc,1) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2 is equal to Tproc,0 when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission assuming SCI format 2-C is received. <<< UNCHANGED PARTS OMITTED >>> ----------------------------------------------- End of Text proposal to TS 38.214 v17.2.0----------------------------------------------

      - Draft Proposals

Draft Proposal 1:

- Reason for change:

- The RAN1 agreement related to UE-B's behaviour of determining a resource used for its resource (re-)selection among the non-preferred resource set received from UE-A is not captured in the specification.

- Summary of change:

- The relevant UE-B's behaviour is described in Section 8.1.4C of TS 38.214.

- Consequences if not approved:

- The specification text is unclear as to which resource UE-B uses for its resource (re-)selection among the non-preferred resource set received from UE-A

--------------------------- Start of text proposal to TS 38.214 v17.2.0 --------------------------- 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. Note: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission, where Tproc,2 is equal to (Tproc,0+Tproc,1) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2 is equal to Tproc,0 when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission and SCI format 2-C is received. Note: the SCI format 2-C case is assuming that SCI format 2-C is received. --------------------------- End of Text proposal to TS 38.214 v17.2.0 ---------------------------

    - Issue#13: Addition of clarification on missing field descriptions and functions of SCI format 2-C

      - Background

Two contributions [12] [20] proposed that it is necessary to have an addition of clarification on missing field descriptions and functions of SCI format 2-C.

[vivo, R1-2206763] The SCI format 2-C includes all the fields present in SCI format 2-A except cast type indicator, thus similar behaviors of SCI 2-A should also be applied to SCI 2-C. However, in current spec, SCI format 2-C is missing when describe the related fields or functions of SCI. Hence, the following TP for TS 38.214 is proposed. - TP#7 for TS 38.214 ### 8.1.3 Modulation order, target code rate, redundancy version and transport block size determination The redundancy version is given by the "Redundancy version" field in SCI format 2-A, or 2-B or SCI format 2-C. ===omitted=== ### 8.2.1 CSI-RS transmission procedure A UE transmits sidelink CSI-RS within a unicast PSSCH transmission if the following conditions hold: - CSI reporting is enabled by higher layer parameter sl-CSI-Acquisition; and - the 'CSI request' field in the corresponding SCI format 2-A or SCI format 2-C is set to 1. ===omitted=== ## 8.3 UE procedure for receiving the physical sidelink shared channel For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, and 2-B and 2-C, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, and 2-B and 2-C, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. A UE is required to decode neither the corresponding SCI formats 2-A, and 2-B and 2-C nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support. ===omitted=== #### 8.5.1.2 Triggering of sidelink CSI reports The CSI-triggering UE is not allowed to trigger another aperiodic CSI report for the same UE before the last slot of the expected reception or completion of the ongoing aperiodic CSI report associated with the SCI format 2-A or SCI format 2-C with the 'CSI request' field set to 1, where the last slot of the expected reception of the ongoing aperiodic CSI report is given by [10, TS38.321]. An aperiodic CSI report is triggered by an SCI format 2-A or SCI format 2-C with the 'CSI request' field set to 1. ===omitted=== Sidelink CSI-RS shall be transmitted according to [4, TS 38.211] in the resource blocks used for the PSSCH associated with the SCI format 2-A or SCI format 2-C triggering a report. ===omitted=== - The number of PSSCH and DM-RS symbols is equal to sl-LengthSymbols‒2. - Assume no REs allocated for sidelink CSI-RS. - Assume no REs allocated SCI format 2-A, or SCI format 2-B, or SCI format 2-C . ===omitted===. Proposal 16: Adopt the TP#7 for 38.214. [ASUSTeK, R1-2207483] #### 2.1 Missing SCI format 2C in TS38.214 Reason for change:; In the current 38.214, SCI format 2-C is missing in clauses of RV, PSSCH reception and SL CSI.. Summary of change:; SCI format 2-C is added in corresponding clauses of RV, PSSCH reception and SL CSI.. Consequences if not approved:; SCI format 2-C cannot support functions of RV, PSSCH reception and SL CSI.. Clauses affected:; 8.1.3, 8.2.1, 8.3, 8.5.1.2, 8.5.2.2, 8.5.2.3 in TS 38.214.. ---------------------------------------< Text Proposal 1 for 38.214> -------------------------------------- ### 8.1.3 Modulation order, target code rate, redundancy version and transport block size determination The redundancy version is given by the "Redundancy version" field in SCI format 2-A, 2-B or 2-C. … ## 8.2 UE procedure for transmitting sidelink reference signals ### 8.2.1 CSI-RS transmission procedure A UE transmits sidelink CSI-RS within a unicast PSSCH transmission if the following conditions hold: - CSI reporting is enabled by higher layer parameter sl-CSI-Acquisition; and - the 'CSI request' field in the corresponding SCI format 2-A or 2-C is set to 1. … ## 8.3 UE procedure for receiving the physical sidelink shared channel For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B or 2-C, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B or 2-C, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. A UE is required to decode neither the corresponding SCI formats 2-A, 2-B and 2-C nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support. … #### 8.5.1.2 Triggering of sidelink CSI reports The CSI-triggering UE is not allowed to trigger another aperiodic CSI report for the same UE before the last slot of the expected reception or completion of the ongoing aperiodic CSI report associated with the SCI format 2-A or 2-C with the 'CSI request' field set to 1, where the last slot of the expected reception of the ongoing aperiodic CSI report is given by [10, TS38.321]. An aperiodic CSI report is triggered by an SCI format 2-A or 2-C with the 'CSI request' field set to 1. A UE is not expected to transmit a sidelink CSI-RS and a sidelink PT-RS which overlap. … #### 8.5.2.2 Reference signal (CSI-RS) The UE can be configured with one CSI-RS pattern as indicated by the higher layer parameters sl-CSI-RS-FreqAllocation, sl-CSI-RS-FirstSymbol in SL-CSI-RS-Config. Parameters for which the UE shall assume non-zero transmission power for CSI-RS are configured according to clause 8.2.1. A UE is not expected to be configured such that a CSI-RS and the corresponding PSCCH can be mapped to the same resource element. A UE is not expected to receive sidelink CSI-RS and PSSCH DM-RS, nor CSI-RS and 2nd-stage SCI, on the same symbol. Sidelink CSI-RS shall be transmitted according to [4, TS 38.211] in the resource blocks used for the PSSCH associated with the SCI format 2-A or 2-C triggering a report. #### 8.5.2.3 CSI reference resource definition The CSI reference resource in sidelink is defined as follows: - In the frequency domain, the CSI reference resource is defined by the group of sidelink physical resource blocks containing the sidelink CSI-RS to which the derived CSI relates. - In the time domain, the CSI reference resource for a CSI reporting in sidelink slot n is defined by a single sidelink slot nCSI_ref where nCSI_ref is the same sidelink slot as the corresponding CSI request. If configured to report CQI index and RI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index and RI index: - The reference resource uses the CP length and subcarrier spacing configured for the SL BWP. - Redundancy Version 0. - PSCCH occupies 2 OFDM symbols. - The number of PSSCH and DM-RS symbols is equal to sl-LengthSymbols‒2. - Assume no REs allocated for sidelink CSI-RS. - Assume no REs allocated SCI format 2-A, SCI format 2-B, or SCI format 2-C. - Assume the same number of DM-RS symbols as the smallest one configured by the higher layer parameter sl-PSSCH-DMRS-TimePatternList. - Assume no REs allocated for sidelink PT-RS. -------------------------------------------< End of Text Proposal 1> --------------------------------------- #### Proposal 1: Adopt text proposal 1 in updating of TS 38.214.

      - Draft Proposals

Draft Proposal 1:

- Reason for change:

- In the current 38.214, SCI format 2-C is missing in clauses of RV, PSSCH reception and SL CSI.

- Summary of change:

- SCI format 2-C is added in corresponding clauses of RV, PSSCH reception and SL CSI.

- Consequences if not approved:

- SCI format 2-C cannot support functions of RV, PSSCH reception and SL CSI.

---------------------------------------------- Start of text proposal to TS 38.214 v17.2.0---------------------------------------------- ### 8.1.3 Modulation order, target code rate, redundancy version and transport block size determination The redundancy version is given by the "Redundancy version" field in SCI format 2-A, or 2-B or SCI format 2-C. ===omitted=== ### 8.2.1 CSI-RS transmission procedure A UE transmits sidelink CSI-RS within a unicast PSSCH transmission if the following conditions hold: - CSI reporting is enabled by higher layer parameter sl-CSI-Acquisition; and - the 'CSI request' field in the corresponding SCI format 2-A or SCI format 2-C is set to 1. ===omitted=== ## 8.3 UE procedure for receiving the physical sidelink shared channel For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, and 2-B and 2-C, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, and 2-B and 2-C, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. A UE is required to decode neither the corresponding SCI formats 2-A, and 2-B and 2-C nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support. ===omitted=== #### 8.5.1.2 Triggering of sidelink CSI reports The CSI-triggering UE is not allowed to trigger another aperiodic CSI report for the same UE before the last slot of the expected reception or completion of the ongoing aperiodic CSI report associated with the SCI format 2-A or SCI format 2-C with the 'CSI request' field set to 1, where the last slot of the expected reception of the ongoing aperiodic CSI report is given by [10, TS38.321]. An aperiodic CSI report is triggered by an SCI format 2-A or SCI format 2-C with the 'CSI request' field set to 1. ===omitted=== Sidelink CSI-RS shall be transmitted according to [4, TS 38.211] in the resource blocks used for the PSSCH associated with the SCI format 2-A or SCI format 2-C triggering a report. ===omitted=== - The number of PSSCH and DM-RS symbols is equal to sl-LengthSymbols‒2. - Assume no REs allocated for sidelink CSI-RS. - Assume no REs allocated SCI format 2-A, or SCI format 2-B, or SCI format 2-C . ===omitted=== --------------------------- End of Text proposal to TS 38.214 v17.2.0 ---------------------------

    - Issue#15: Addition of clarification for cast type(s) of Scheme 1 and/or SCI format 2-C to the specification

      - Background

Two contributions [13] [19] proposed that it is necessary to have an addition of clarification for cast type(s) of Scheme 1 and/or SCI format 2-C to the specification.

[Samsung, R1-2206804] Summary of change Clarify in TS 38.212 that the SCI format 2-C can be used for inter-UE coordination information transmission only when the cast type is unicast. Consequence if not approved RAN1 agreement is not fully captured, i.e., it’s missing in specification that the SCI format 2-C can be used in addition to MAC-CE for inter-UE coordination information transmission only when the cast type is unicast. Text proposal 1 We provide the Text Proposal for section 8.4.1.3 of TS 38.212 [2] as below. ---------------------------- Start of Text Proposal for TS 38.212 ----------------------------- #### 8.4.1.3 SCI format 2-C SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information for unicast or requesting inter-UE coordination information. < Unchanged parts are omitted > --------------------------------------- End of Text Proposal ----------------------------------. [DOCOMO, R1-2207386] #### 2.2.1.1 Cast type for IUC scheme 1 Agreement For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request Working Assumption For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast - FFS: Under which conditions unicast can be supported Conclusion For cast type(s) of inter-UE coordination information with preferred resource set triggered by a condition other than explicit request reception, there is no consensus in RAN1 on the support of groupcast or broadcast for preferred resource set Agreement For inter-UE coordination information transmission, only when the cast type of inter-UE coordination information is unicast regardless of whether or not it is multiplexed with other data, a SCI format 2-C can be used in addition to MAC CE. Reason for change Regarding cast type for scheme 1, the above agreements/conclusion were reached. However, it seems that there is no corresponding text in RAN1/RAN2 spec. Basically cast type determination should be specified in RAN2 spec, thus an LS to inform these agreements should be sent to RAN2. Meanwhile SCI format 2-C is a PHY feature. Corresponding text should be added to RAN1 spec. Summary of change Clarify that SCI format 2-C is used only for unicast. Consequences if not approved UE may use SCI format 2-C for multicast/broadcast. Text proposal 8: 38.212 8.4.1.3 SCI format 2-C SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information. SCI format 2-C can be used only for unicast. The following information is transmitted by means of the SCI format 2-C: <Unchanged parts omitted>. Proposal 1: - Send an LS to RAN2 to inform agreements/conclusion on cast type related to IUC scheme 1 and ask to update their specification accordingly.

      - Draft Proposals

Draft Proposal 1:

- Reason for change

- Regarding the cast type for SCI format 2-C, the following agreement was reached. However, it seems that there is no corresponding text in the specification.

Agreement For inter-UE coordination information transmission, only when the cast type of inter-UE coordination information is unicast regardless of whether or not it is multiplexed with other data, a SCI format 2-C can be used in addition to MAC CE

- Summary of change

- Clarify that SCI format 2-C is used only for unicast.

- Consequences if not approved

- A UE may use SCI format 2-C for multicast/broadcast.

--------------------------- Start of text proposal to TS 38.212 v17.2.0 --------------------------- 8.4.1.3 SCI format 2-C SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information. SCI format 2-C is can be used only for unicast. The following information is transmitted by means of the SCI format 2-C: <Unchanged parts omitted> --------------------------- End of Text proposal to TS 38.212 v17.2.0 ---------------------------

Draft Proposal 2:

- Send an LS to RAN2 to inform agreements/conclusion on cast type related to IUC scheme 1 and ask to update their specification accordingly.

Agreement: For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request Working Assumption: For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast - FFS: Under which conditions unicast can be supported Conclusion: For cast type(s) of inter-UE coordination information with preferred resource set triggered by a condition other than explicit request reception, there is no consensus in RAN1 on the support of groupcast or broadcast for preferred resource set Agreement: For inter-UE coordination information transmission, only when the cast type of inter-UE coordination information is unicast regardless of whether or not it is multiplexed with other data, a SCI format 2-C can be used in addition to MAC CE

    - Issue#19: Further clarification on the use of preferred resource set in the resource reselection due to re-evaluation/pre-emption/conflict information

      - Background

One contribution [19] proposed that it is necessary to have further clarification on the use of preferred resource set in the resource reselection due to re-evaluation/pre-emption/conflict information.

[DOCOMO, R1-2207386] #### 2.2.1.3 Preferred resources for resource reselection 38.321 5.22.1.1 SL Grant reception and SCI transmission ... 3> if configured by RRC, sl-InterUE-CoordinationScheme1 enabling reception of preferred resource set and non-preferred resource set and when the UE does not have own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity from the resources belonging to the received preferred resource set for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 3> if configured by RRC, sl-InterUE-CoordinationScheme1 enabling reception of preferred resource set and non-preferred resource set and when the UE has own sensing result as specified in clause 8.1.4 of TS 38.214 [7] and if a preferred resource set is received from a UE: 4> randomly select the time and frequency resources for one transmission opportunity within the intersection of the received preferred resource set and the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for a MAC PDU to be transmitted to the UE providing the preferred resource set, according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 4> if there are no resources within the intersection that can be selected as the time and frequency resources for the one transmission opportunity according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 5> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) allowed on the carrier. 4> use the randomly selected resource to select a set of periodic resources spaced by the resource reservation interval for transmissions of PSCCH and PSSCH corresponding to the number of transmission opportunities of MAC PDUs determined in TS 38.214 [7]. … 5.22.1.2a Re-evaluation and Pre-emption ... 2> randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for either the removed resource or the dropped resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9]; … 2> if one or multiple SL DRX is configured: 3> randomly select the time and frequency resource from the resources later than the resources for either the removed resource or the dropped resource indicated by a prior SCI, from the resource indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] which occur within the SL DRX active time as specified in clause 5.28.3 of the destination UE selected for indicating to the physical layer the SL DRX active time above, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9]. 2> else: 3> randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] for either the removed resource or the dropped resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9]. … 5.22.1.2b Re-selection for using a received resource conflict indication ... 2> randomly select the time and frequency resource from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7] excluding the conflict resource(s) for the removed resource, according to the amount of selected frequency resources, the selected number of HARQ retransmissions and the remaining PDB of either SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources, and that a resource can be indicated by the time resource assignment of an SCI for a retransmission according to clause 8.3.1.1 of TS 38.212 [9];. In the current specification, non-preferred resources are excluded from S_A at PHY layer and preferred resources are preferentially selected in selection procedure from S_A at MAC layer. Once we see MAC spec carefully, it can be found that preferred resources are used in resource selection procedure but not in resource reselection procedure due to re-evaluation/pre-emption/conflict information. However, ‘resource (re)selection’ in RAN1 agreements include the resource reselection procedure due to re-evaluation/pre-emption/conflict information. MAC spec should be updated such that preferred resources, if available, are used when resource reselection is performed due to re-evaluation/pre-emption/conflict information. One note is that non-preferred resource is used also for this purpose in PHY layer. There would be no reason to skip using preferred resources. Proposal 3: - Send an LS to ask RAN2 to use preferred resources for resource reselection due to re-evaluation/pre-emption/conflict information, if available.

      - Draft Proposals

Draft Proposal 1:

- Preferred resources are used for resource reselection due to re-evaluation/pre-emption/conflict information, if available.

- Send an LS to inform RAN2 about it

  - Scheme 2

    - Issue#21: Further clarification on conditions for UE to be UE-B when at least one of UEs scheduling conflicting TBs does not set indicationUEB flag to 1

      - Background

Six contributions [1][5][14][16][18][19] proposed that it is necessary to have further clarification on conditions for UE to be UE-B when at least one of UEs scheduling conflicting TBs does not set indicationUEB flag to 1.

[FUTUREWEI, R1-2205742] Proposal: Agree to Updated Proposal 5-2b from the moderator with the addition of “for each pair of UEs”. Updated Proposal 5-2b (Moderator) - For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A, - For each pair of UEs, If there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and, if sl-IndicationUE-B is (pre)configured to ‘Enabled’, ‘Conflict information receiver flag’ field is set to 1, that UE is UE-B. Reason for change:; The specification on the determination of UE-B does not cover the case when at least one of UEs scheduling conflicting TBs does not support Scheme 2. Summary of change:; Add the determination of UE-B when at least one of UEs scheduling conflicting TBs does not support Scheme 2. Consequences if not approved:; Ambiguous specifications which may limit the use cases of inter UE coordination Scheme 2 and impact the performance. ## Text proposal for determination of UE-B in TS38.213 Section 16.3.0 --------------------------------- Text proposal begins ----------------------------------------------- ### 16.3.0 UE procedure for transmitting PSFCH with control information ## *** Unchanged text is omitted *** A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 - determines to transmit to the third UE the PSFCH with the conflict information regardless of the values of p1,p2, if the indicationUEB flag in SCI Format 1-A from the second UE is set to 0 and the indicationUEB flag in SCI Format 1-A from the third UE is set to 1, and indicationUEBScheme2 = 'enabled' ## *** Unchanged text is omitted *** --------------------------------- Text proposal ends -----------------------------------------------. [ZTE, R1-2206096] Proposal 1: Support the following updated proposal 5-2b at RAN1#109e - For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A, - If there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and, if sl-IndicationUE-B is (pre)configured to ‘Enabled’, ‘Conflict information receiver flag’ field is set to 1, that UE is UE-B. Proposal 2: To reflect proposal 1 for Condition 2-A-1 in scheme 2, TPs for TS38.213 section 16.3.0 are given as following: - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 - determines to transmit to the second UE the PSFCH with the conflict information, if only for the second UE, the PSFCH occasion for resource conflict information is valid and the indicationUEB flag in SCI Format 1-A is set to 1, if indicationUEBScheme2 = 'enabled' < Unchanged parts are omitted >. [CMCC, R1-2206890] Proposal 6: RAN 1 should take the following proposal as an agreement in RAN1#110-e meeting: Updated Proposal 5-2b (Moderator) - For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A, - If there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and, if sl-IndicationUE-B is (pre)configured to ‘Enabled’, ‘Conflict information receiver flag’ field is set to 1, that UE is UE-B.. [InterDigital, R1-2207146] Proposal 3: Agree on Proposal 5-2b finalized in RAN1 #109e meeting. Proposal 4: Agree on changes to the TS 38.213 specification in the Appendix-B. Appendix-B – Proposed TS 38.213 Specification Change Reason for change:; Corrections on NR Sidelink enhancements. Summary of change:; In clause 16.3.0, capture the steps to determine the UE-B to send conflict indication when the indicationUEB flag of one UE scheduling the conflicting TB is 0.. Consequences if not approved:; UE-A doesn’t know which UE to send conflict indication in this scenario.. ----------------------------------------------Start of TP to TS 38.213 clause 16.3.0--------------------------------------- 16.3.0 UE procedure for transmitting PSFCH with control information A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - determines to transmit to the second UE the PSFCH with the conflict information. A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are validnot passed - when the indicationUEB flag in SCI Format 1-A from the second UE is set to 1 and the third UE is set to 0, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - when the indicationUEB flag in SCI Format 1-A from the second UE is set to 0 and the third UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the third UE the PSFCH with the conflict information - when the indicationUEB flag in SCI Format 1-A from the second UE and the third UE are bothis set to 1, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 ---------------------------------------------- End of TP to 38.213 clause 16.3.0 ------------------------------------------ [Apple, R1-2207313] Proposal 8: For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A, - If there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and, if sl-IndicationUE-B is (pre)configured to ‘Enabled’, ‘Conflict information receiver flag’ field is set to 1, that UE is UE-B. Proposal 9: Adopt the following text proposal for the new rule of determining UE-B. TS38.213 [3] ### 16.3.0 UE procedure for transmitting PSFCH with control information *** < Unchanged parts are omitted> *** A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = ‘enabled’ - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE is set to 1 and the indicationUEB flag in SCI Format 1-A from the third UE is set to 0, if indicationUEBScheme2 = ‘enabled’ - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information *** < Unchanged parts are omitted> ***. [DOCOMO, R1-2207386] Reason for change In the current specification, a UE with higher priority value is set as UE-B when the two UEs scheduling the conflicting TBs support UE-B behavior of scheme 2 (and with further conditions). However, an important case is missed: only either of the two UEs supports UE-B behavior of scheme 2 and the other UE does not support. Rather, this situation is the motivation to introduce ‘Conflict information receiver flag’ field; thus, the correction is an essential feature. That is, when only either of the two UEs indicating overlapping resource(s) supports scheme 2, UE-A sets the capable UE as UE-B. For whole procedure, UE-A should firstly apply the above capability-based UE-B determination and then the priority-based UE-B determination; otherwise, i.e. in the opposite order, none of UE might be set as UE-B. There was an argument that collision indication might be sent to UE transmitting a TB with lower priority, which leads to worse resource efficiency. However, this argument is invalid definitely. Firstly, when the two UEs do not detect each other, which is referred as hidden-node issue and is one of the main situations to be solved in this agenda, there is no impact on resource efficiency and better reliability/latency can be obtained. Secondly, this agenda focuses on better reliability and latency. It is quite strange if we aim to better efficiency at a cost of reliability/latency. Of course, both UEs might perform reselection due to conflict indicator and pre-emption, but this should be OK for better reliability/latency. Thirdly, appropriate (pre-)configuration of collision detection, e.g. SIR-based conflict detection, can handle such a non-preferred situation from resource efficiency perspective. Summary of change For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A, if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and ‘Conflict information receiver flag’ field is set to 1 if sl-IndicationUE-B is (pre)configured to ‘Enabled’, UE-A sets the UE as UE-B. Consequences if not approved Resource conflict in much less situations can be detected and as the result inter-UE coordination scheme 2 achieves little gain. Text proposal 9: 38.213 16.3.0 UE procedure for transmitting PSFCH with control information <Unchanged parts omitted> A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - determines that one of the following conditions are met - Conflict information receiver flag field in SCI Format 1-A from the second UE is set to 1, Conflict information receiver flag field in SCI Format 1-A from the third UE is set to 0, and if sl-IndicationUE-B = 'enabled' - the indicationUEB flag Conflict information receiver flag field in SCI Format 1-A from the second UE and the third UE is set to 1, p2<p1, and if indicationUEBScheme2 sl-IndicationUE-B = 'enabled' - p2<p1 if sl-IndicationUE-B = 'disabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 <Unchanged parts omitted>.

      - Draft Proposals

Draft Proposal 1:

- For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A,

- If there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and, if sl-IndicationUE-B is (pre)configured to ‘Enabled’, ‘Conflict information receiver flag’ field is set to 1, that UE is UE-B.

Draft Proposal 2 (for the case when Draft Proposal 1 above is adopted):

- Reason for change

- The specification on the determination of UE-B does not cover the case when there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and, if sl-IndicationUE-B is (pre)configured to ‘Enabled’, ‘Conflict information receiver flag’ field is set to 1.

- Summary of change

- Capture the steps in clause 16.3.0 to determine the UE-B for this scenario.

- Consequences if not approved

- UE-A doesn’t know which UE to send conflict indication in this scenario.

--------------------------- Start of text proposal to TS 38.213 v17.2.0 --------------------------- - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 - determines to transmit to the second UE the PSFCH with the conflict information, if only for the second UE, the PSFCH occasion for resource conflict information is valid and the indicationUEB flag in SCI Format 1-A is set to 1, if indicationUEBScheme2 = 'enabled' < Unchanged parts are omitted > --------------------------- End of Text proposal to TS 38.213 v17.2.0 ---------------------------

    - Issue#24: Further clarification on conditions for UE to be UE-B in Condition 2-A-2

      - Background

One contribution [5] proposed that it is necessary to have further clarification on conditions for UE to be UE-B in Condition 2-A-2.

[ZTE, R1-2206096] Condition 2-A-2 for scheme 2, i.e., conflict indication transmission due to half-duplex operation, is supported. Current text in TS38.213 can be seen below: A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - determines to transmit to the second UE the PSFCH with the conflict information.. Similar as Condition 2-A-1, it is unfeasible to transmit the collision indication if the PSFCH occasion for resource conflict information of the second UE is passed, nor should the collision indication be transmitted if the indicationUEB flag in SCI Format 1-A from the second UE is set to 0, if indicationUEBScheme2 = 'enabled'. Based on this, we propose to add these conditions for Condition 2-A-2. Proposal 3: For Condition 2-A-2, only UE whose PSFCH occasion for resource conflict indication is not yet passed and, if sl-IndicationUE-B is (pre)configured to ‘Enabled’, ‘Conflict information receiver flag’ field is set to 1, can be UE-B. Proposal 4: To reflect proposal 3 for Condition 2-A-2 in scheme 2, TPs for TS38.213 section 16.3.0 are given as following: - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - the PSFCH occasion for resource conflict information of the second UE is valid - the indicationUEB flag in SCI Format 1-A from the second UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines to transmit to the second UE the PSFCH with the conflict information. < Unchanged parts are omitted >.

      - Draft Proposals

Draft Proposal 1:

- For Condition 2-A-2, only UE whose PSFCH occasion for resource conflict indication is not yet passed and, if sl-IndicationUE-B is (pre)configured to ‘Enabled’, ‘Conflict information receiver flag’ field is set to 1, can be UE-B.

Draft Proposal 2 (for the case when Draft Proposal 1 above is adopted):

- Reason for change

- For Condition 2-A-2, the conditions under which UE-A is actually allowed to transmit conflict indication to UE-B is not clear in the current specification.

- Summary of change

- Capture the relevant conditions mentioned above in clause 16.3.0

- Consequences if not approved

- UE-A’s behaviour related to Condition 2-A-2 is unclear.

--------------------------- Start of text proposal to TS 38.213 v17.2.0 --------------------------- - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - the PSFCH occasion for resource conflict information of the second UE is valid - the indicationUEB flag in SCI Format 1-A from the second UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines to transmit to the second UE the PSFCH with the conflict information. < Unchanged parts are omitted > --------------------------- End of Text proposal to TS 38.213 v17.2.0 ---------------------------

  - Scheme 1 and 2

    - Issue#33: Correction on RRC parameter name /value misalignment

      - Background

Four contribution [9][15][19][20] proposed that it is necessary to have correction on RRC parameter name/value misalignment.

[NEC, R1-2206468] Proposal 2: Adopt the editorial corrections for IUC in TS 38.213 and TS 38.214 in section 5. 5. Text proposal 5.1 TS 38.213 V17.2.0 ### 16.3 UE procedure for reporting and obtaining control information in PSFCH Control information provided by a PSFCH transmission includes HARQ-ACK information or conflict information. #### 16.3.0 UE procedure for transmitting PSFCH with control information A UE can be indicated by an SCI format scheduling a PSSCH reception to transmit a PSFCH with HARQ-ACK information in response to the PSSCH reception. The UE provides HARQ-ACK information that includes ACK or NACK, or only NACK. A UE can be provided, by sl-PSFCH-Period, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled. A UE can be enabled, by sl-InterUE-CoordinationScheme2-r17, to transmit a PSFCH with conflict information in a resource pool. The UE can determine, based on an indication by a SCI format 1-A, a set of resources that includes one or more slots and resource blocks that are reserved for PSSCH transmission. If the UE determines a conflict for a reserved resource for PSSCH transmission, the UE provides conflict information in a PSFCH. A UE expects that a slot t'kSL (0≤k<T'max) has a PSFCH transmission occasion resource if k mod NPSSCHPSFCH=0, where t'kSL is defined in [6, TS 38.214], and T'max is a number of slots that belong to the resource pool within 10240 msec according to [6, TS 38.214], and NPSSCHPSFCH is provided by sl-PSFCH-Period. A UE may be indicated by higher layers to not transmit a PSFCH that includes HARQ-ACK information in response to a PSSCH reception [11, TS 38.321]. If a UE receives a PSSCH in a resource pool and the HARQ feedback enabled/disabled indicator field in an associated SCI format 2-A/2-B/2-C has value 1 [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. The UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH-r16, of the resource pool after a last slot of the PSSCH reception. A UE is provided by sl-PSFCH-RB-Set a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information in a PRB of the resource pool. A UE can be provided by sl-RB-SetPSFCH-r17 a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with conflict information in a PRB of the resource pool. A UE expects that different PRBs are (pre)configured for conflict information and HARQ-ACK information. For a number of Nsubch sub-channels for the resource pool, provided by sl-NumSubchannel, and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH. The second OFDM symbol l' of PSFCH transmission in a slot is defined as l'=sl-StartSymbol+sl-LengthSymbols-2. A UE determines a number of PSFCH resources available for multiplexing HARQ-ACK or conflict information in a PSFCH transmission as RPRB, CSPSFCH=Ntype PSFCH⋅Msubch, slotPSFCH⋅NCSPSFCH where NCSPSFCH is a number of cyclic shift pairs for the resource pool provided by sl-NumMuxCS-Pair and, based on an indication by sl-PSFCH-CandidateResourceType, - if sl-PSFCH-CandidateResourceType is configured as startSubCH, Ntype PSFCH=1 and the Msubch, slotPSFCH PRBs are associated with the starting sub-channel of the corresponding PSSCH - if sl-PSFCH-CandidateResourceType is configured as allocSubCH, Ntype PSFCH=Nsubch PSSCH and the Nsubch PSSCH⋅Msubch, slotPSFCH PRBs are associated with the Nsubch PSSCH sub-channels of the corresponding PSSCH - for conflict information, the corresponding PSSCH is determined based on sl-PSFCH-Occasion-r17 The PSFCH resources are first indexed according to an ascending order of the PRB index, from the Ntype PSFCH⋅Msubch, slotPSFCH PRBs, and then according to an ascending order of the cyclic shift pair index from the NCSPSFCH cyclic shift pairs. A UE determines an index of a PSFCH resource for a PSFCH transmission with HARQ-ACK information in response to a PSSCH reception or with conflict information corresponding to a reserved resource as PID+MIDmodRPRB, CSPSFCH where PID is a physical layer source ID provided by SCI format 2-A/2-B/2-C [5, TS 38.212] scheduling the PSSCH reception, or by SCI format 2-A/2-B/2-C with corresponding SCI format 1-A reserving the resource from another UE to be provided with the conflict information. For HARQ-ACK information, MID is the identity of the UE receiving the PSSCH as indicated by higher layers if the UE detects a SCI format 2-A with Cast type indicator field value of "01"; otherwise, MID is zero. For conflict information, MID is zero. For a PSFCH transmission with HARQ-ACK information or conflict information, a UE determines a m0 value, for computing a value of cyclic shift α [4, TS 38.211], from a cyclic shift pair index corresponding to a PSFCH resource index and from NCSPSFCH using Table 16.3-1. Table 16.3-1: Set of cyclic shift pairs NCSPSFCH; m0. Cyclic Shift Pair Index 0; Cyclic Shift Pair Index 1; Cyclic Shift Pair Index 2; Cyclic Shift Pair Index 3; Cyclic Shift Pair Index 4; Cyclic Shift Pair Index 5. 1; 0; -; -; -; -; -. 2; 0; 3; -; -; -; -. 3; 0; 2; 4; -; -; -. 6; 0; 1; 2; 3; 4; 5. For a PSFCH transmission with HARQ-ACK information, a UE determines a mcs value, for computing a value of cyclic shift α [4, TS 38.211], as in Table 16.3-2 if the UE detects a SCI format 2-A with Cast type indicator field value of "01" or "10" or a SCI format 2-C, or as in Table 16.3-3 if the UE detects a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11". For a PSFCH transmission with conflict information, a UE determines a mcs value for computing a value of cyclic shift α [4, TS 38.211] as in Table 16.3-4. The UE applies one cyclic shift from a cyclic shift pair to a sequence used for the PSFCH transmission [4, TS 38.211]. Table 16.3-2: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission when HARQ-ACK information includes ACK or NACK HARQ-ACK Value; 0 (NACK); 1 (ACK). Sequence cyclic shift; 0; 6. Table 16.3-3: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission when HARQ-ACK information includes only NACK HARQ-ACK Value; 0 (NACK); 1 (ACK). Sequence cyclic shift; 0; N/A. Table 16.3-4: Mapping of conflict information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission Conflict information; Conflict information for a next in time reserved resource indicated in SCI. Sequence cyclic shift; 0. A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - determines to transmit to the second UE the PSFCH with the conflict information. A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, sl-TypeUE-A-r17 is not provided, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool sl-TypeUE-A-r17is provided, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if sl-IndicationUE-B-r17 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH-r16 before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 The first UE can be provided conditions by sl-OptionForCondition2-A-1-r17 to determine conflict of reserved resources in a resource pool - if sl-OptionForCondition2-A-1-r17 = 0, the first UE can be provided by, ThresPSSCH-RSRP-List Thpi,pj, a list of RSRP thresholds for each priority combination pi,pj [6, TS 38.214] - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if the RSRP [6, TS 38.214] of the third UE is above a threshold Thp2,p1 - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if the RSRP of the second UE is above a threshold Thp1,p2 - if sl-OptionForCondition2-A-1-r17 = 1, the first UE can be provided a value Delta_Th by deltaRSRPThresh - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if RSRP2>RSRP1+Delta_Th, where RSRP1 and RSRP2 are the RSRP measurements from the first UE for the second UE and the third UE, respectively - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if RSRP1>RSRP2+Delta_Th If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on sl-PSFCH-Occasion-r17 - If sl-PSFCH-Occasion-r17 = 0, the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH-r16, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information. - If sl-PSFCH-Occasion-r17 = 1, the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH-r16 slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information. #### 16.3.1 UE procedure for receiving PSFCH with control information A UE that transmitted a PSSCH scheduled by a SCI format 2-A/2-B/2-C that indicates HARQ feedback enabled, attempts to receive associated PSFCHs with HARQ-ACK information according to PSFCH resources determined as described in clause 16.3.0. The UE determines an ACK or a NACK value for HARQ-ACK information provided in each PSFCH resource as described in [8-4, TS 38.101-4]. The UE does not determine both an ACK value and a NACK value at a same time for a PSFCH resource. For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report to higher layers. For generating the HARQ-ACK information, the UE can be indicated by a SCI format to perform one of the following - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10" or a SCI format 2-C - report to higher layers HARQ-ACK information with same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01" - report an ACK value to higher layers if the UE determines an ACK value from at least one PSFCH reception occasion from the number of PSFCH reception occasions in PSFCH resources corresponding to every identity MID of UEs that the UE expects to receive corresponding PSSCHs as described in clause 16.3; otherwise, report a NACK value to higher layers - if the PSFCH reception occasion is associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11" - report to higher layers an ACK value if the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, report a NACK value to higher layers A UE that transmitted SCI format 1-A, indicating one or more reserved resources in a resource pool enabled by inter- sl-InterUE-CoordinationScheme2-r17, attempts to receive associated PSFCH with conflict information in a resource pool with PSFCH resources that the UE determines as described in clause 16.3.0. If the UE determines presence of a resource conflict based on conflict information in a PSFCH reception, the UE reports the resource conflict to higher layers - if sl-SlotLevelResourceExclusion-r17 is not provided, the UE reports resources overlapping with a next in time reserved resource indicated by the SCI format 1-A - if sl-SlotLevelResourceExclusion-r17 is provided, the UE reports resources in a slot of a next in time reserved resource indicated by the SCI format 1-A If a UE receives a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE receives the PSFCH in the resource pool in a slot determined based on sl-PSFCH-Occasion-r17 - If sl-PSFCH-Occasion-r17 = '0', the UE receives the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH-r16, of the resource pool after a slot of a PSCCH transmission that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not receive the PSFCH with conflict information - If sl-PSFCH-Occasion-r17 = '1', the UE receives the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH-r16 slots after a slot of a PSCCH transmission that provides the SCI format 1-A; otherwise, the UE does not receive the PSFCH with conflict information 5.2 TS 38.214 V17.2.0 ### 8.1.4A UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission When this procedure is triggered, the following parameters are provided by the higher layer: - the resource pool from which the preferred or non-preferred resources are to be determined; - the resource selection window [n+T1,n+T2] within which the preferred or non-preferred resources are to be determined; - the resource set type (either preferred or non-preferred resource set); - if the resource set type indicates preferred set, then the higher layer additionally provides the following parameters: - L1 priority, prioTX; - the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH; - the resource reservation period, Prsvp_TX, if present. The value of Cresel is determined by the UE according to clause 8.1.5. When this procedure is triggered by another UE's explicit request, the fields in the request are interpreted as follows: - The field 'Resource selection window location' is the concatenation of the starting time location and the ending time location of the resource selection window. The starting and ending time locations of the resource selection window are each encoded in the same way as the reference slot as described in clause 8.1.5A. - The field 'Resource reservation period' is encoded in the same way as the field of the same name in SCI format 1-A. When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4 with the above parameters and the following modifications: - Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met: - the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined; - the higher layer parameter sl-Condition1-A-2-r17 is not set to 'Disabled'. When determining a non-preferred resource set, the UE considers any resource(s) within the resource selection window, if indicated by a received explicit request, and satisfying at least one of the following conditions as non-preferred resource(s): - resource(s) indicated by a received [SCI format 1-A], satisfying at least one of the following criteria: - the RSRP measurement performed, according to clause 8.4.2.1, for the received [SCI format 1-A], is higher than ThprioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in sl-ThresholdRSRP-Condition1-B-1-Option1List-r17, where k=pi. - the UE is a destination UE of a TB associated with the received [SCI format 1-A] and the RSRP measurement performed, according to clause 8.4.2.1 for the received [SCI format 1-A], is lower than Th'prioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th'(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in sl-ThresholdRSRP-Condition1-B-1-Option2List-r17, where k=pi. - resources(s) in slot(s) in which the UE does not expect to perform SL reception due to half duplex operation, if the UE is a destination UE of a TB for whose transmission the non-preferred resource set is being determined. ### 8.1.4B UE procedure for determining a resource conflict A UE configured with the higher layer parameter sl-InterUE-CoordinationScheme2-r17 enabling transmission of a resource conflict indication considers that a resource conflict occurs on a first reserved resource r1 indicated by a first received SCI format if at least one of the following conditions is satisfied: - the first reserved resource r1 overlaps with a second reserved resource r2 indicated by a second received SCI format, and - if [the higher layer parameter for enabling Options 1/4 in Condition 2-A-1] indicates [Option 1 enabled], - if the UE is a destination UE of a TB to be transmitted in r1, the RSRP measurement performed for the second received SCI format RSRP2 is higher than Thprio2,prio1 where prio1 and prio2 are the priorities indicated in the first and second received SCI format, respectively. - if the UE is a destination UE of a TB to be transmitted in r2, the RSRP measurement performed for the first received SCI format RSRP1 is higher than Thprio1,prio2 where prio1 and prio2 are the priorities indicated in the first and second received SCI format, respectively. - if [the higher layer parameter for enabling Options 1/4 in Condition 2-A-1] indicates [Option 4 enabled] and the UE supports [Option 4], - if the UE is a destination UE of a TB to be transmitted in r1, RSRP2-RSRP1 is higher than a (pre)configured RSRP threshold. - if the UE is a destination UE of a TB to be transmitted in r2, RSRP1-RSRP2 is higher than a (pre)configured RSRP threshold. - where the RSRP1, RSRP2 measurements are performed according to clause 8.4.2.1 and the parameter Th(pi,pj) is determined [as described in step 3 of clause 8.1.4]. - the first reserved resource r1 occurs in a slot in which the UE does not expect to perform SL reception due to half-duplex operation and the UE is a destination UE of a TB to be transmitted in resource r1. ### 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter sl-InterUE-CoordinationScheme1-r17 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. ### 8.1.5 UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields 'Frequency resource assignment', 'Time resource assignment' of the associated SCI format 1-A as described below. 'Time resource assignment' carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerReserve is 2, and N = 1 or 2 or 3 actual resources when sl-MaxNumPerReserve is 3, in a form of time RIV (TRIV) field which is determined as follows: if N=1 TRIV=0 elseif N=2 TRIV=t1 else if t2-t1-1≤15 TRIV=30t2-t1-1+t1+31 else TRIV=3031-t2+t1+62-t1 end if end if where the first resource is in the slot where SCI format 1-A was received, and ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤t1≤31; and for N = 3, 1≤t1≤30, t1<t2≤31. The starting sub-channel nsubCH,0start of the first resource is determined according to clause 8.1.2.2. The number of contiguously allocated sub-channels for each of the N resources LsubCH≥1 and the starting sub-channel indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where. If sl-MaxNumPerReserve is 2 then FRIV=nsubCH,1start+i=1LsubCH-1N subchannel SL+1-i If sl-MaxNumPerReserve is 3 then FRIV=nsubCH,1start+nsubCH,2start⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2 where - nsubCH,1start denotes the starting sub-channel index for the second resource - nsubCH,2start denotes the starting sub-channel index for the third resource - N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter sl-NumSubchannel If TRIV indicates N < sl-MaxNumPerReserve, the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used. The number of slots in one set of the time and frequency resources for transmission opportunities of PSSCH is given by Cresel where Cresel= 10*SL_RESOURCE_RESELECTION_COUNTER [10, TS 38.321] if configured else Cresel is set to 1. If a set of sub-channels in slot t'mSL is determined as the time and frequency resource for PSSCH transmission corresponding to the selected sidelink grant (described in [10, TS 38.321]), the same set of sub-channels in slots t'm+j×Prsvp_TX'SL are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t'0SL, t'1SL, t'2SL,… is determined by Clause 8. Here, Prsvp_TX is the resource reservation interval indicated by higher layers. ### 8.1.5A UE procedure for determining slots and resource blocks indicated by a preferred or non-preferred resource set The set of slots and resource blocks indicated by a set of preferred or non-preferred resource(s) is determined as described below. The set of preferred or non-preferred resources r0,r1,r2,…, is indicated by a reference slot tref and M tuples (TRIVm,FRIVm,Prsvp,m), 1≤m≤M indicated by the 'resource combination(s)' field, where for each tuple TRIVm is indicated by the 9 MSBs, followed by FRIVm and Prsvp,m (if present). The reference slot tref is indicated by the 'reference slot location' field as a combination of DFN index and slot index [5, TS 38.212], with the 10 MSBs indicating the DFN index. TRIVm and FRIVm are interpreted according to clause 8.1.5, with the following modifications: - the value of sl-MaxNumPerReserve is fixed to 3. - "slot where SCI format 1-A was received" is replaced by slot indicated as the first resource location of a TRIVm. - the first resource location of each TRIVm for m>1 is indicated by a slot offset tm in logical slots with respect to the reference slot tref; the first resource location of TRIV1 is at slot offset 0 with respect to the reference slot. - "the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received" is replaced by "each tuple" - the starting sub-channel nsubCH,0start of the first resource of each tuple is separately indicated. The starting sub-channel nsubCH,0start of the first resource of each tuple is indicated by the '[lowest subchannel index for the first resource location of each TRIV]' field. The resource reservation period Prsvp,m is encoded as in SCI format 1-A. If the set is indicated by an SCI format 2-C, the number of tuples is M=2. A UE forms the union of the subsets indicated by each tuple (TRIVm,FRIVm,Prsvp,m) to obtain the set r0,r1,r2,…. [Sharp, R1-2206936] TP6 Reason for change:; Misalignment of RRC parameter name and value between TS38.213 and TS38.331.. Summary of change:; - Change ‘typeAUEScheme2’ to ‘sl-TypeUE-A’. - Change ‘indicationUEBScheme2’ to ‘sl-IndicationUE-B’. - Change ‘optionForCondition2A1Scheme2’ to ‘sl-OptionForCondition2-A-1’. - Change ‘RSRP-ThresPerPriorities’ to ‘value 0’. - Change ‘RSRP-ThresWithRsrpMeasurement’ to ‘value 1’. - Change ‘PSFCHOccasionScheme2’ to ‘sl-PSFCH-Occasion’. - Change ‘followSCI’ to ‘value 0’. - Change ‘followReservedResource’ to ‘value 1’.. Consequences if not approved:; Misaligned RRC parameter name and value between TS38.213 and TS38.331.. ### 16.3.0 UE procedure for transmitting PSFCH with control information <Unchanged text omitted> A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - determines to transmit to the second UE the PSFCH with the conflict information. A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, sl-TypeUE-A is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool sl-TypeUE-A is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if sl-IndicationUE-B = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 The first UE can be provided conditions by sl-OptionForCondition2-A-1 to determine conflict of reserved resources in a resource pool - if sl-OptionForCondition2-A-1 = 'value 0', the first UE can be provided by, ThresPSSCH-RSRP-List Thpi,pj, a list of RSRP thresholds for each priority combination pi,pj [6, TS 38.214] - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if the RSRP [6, TS 38.214] of the third UE is above a threshold Thp2,p1 - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if the RSRP of the second UE is above a threshold Thp1,p2 - if sl-OptionForCondition2-A-1 = 'value 1', the first UE can be provided a value Delta_Th by deltaRSRPThresh - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if RSRP2>RSRP1+Delta_Th, where RSRP1 and RSRP2 are the RSRP measurements from the first UE for the second UE and the third UE, respectively - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if RSRP1>RSRP2+Delta_Th If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on sl-PSFCH-Occasion - If sl-PSFCH-Occasion = 'value 0', the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information. - If sl-PSFCH-Occasion = 'value 1', the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information. <Unchanged text omitted>. Proposal 8: Adopt TP6 as above. [DOCOMO, R1-2207386] Reason for change / Summary of change Names of higher layer parameters are aligned with those in 38.331. There is a duplicated part on resource conflict detection. The corresponding part in 38.214 is removed. Consequences if not approved UE might refer to different parameters. Text proposal 12: 38.212 8.3.1.1 SCI format 1-A <Unchanged parts omitted> - Reserved – a number of bits as determined by the following: - Nreserved bits as configured by higher layer parameter sl-NumReservedBits, with value set to zero, if higher layer parameter indicationUEBScheme2 sl-IndicationUE-B is not configured, or if higher layer parameter indicationUEBScheme2 sl-IndicationUE-B is configured to 'Disabled'; - (Nreserved-1) bits otherwise, with value set to zero. - Conflict information receiver flag – 0 or 1 bit - 1 bit if higher layer parameter indicationUEBScheme2 sl-IndicationUE-B is configured to 'Enabled', where the bit value of 0 indicates that the UE cannot be a UE to receive conflict information and the bit value of 1 indicates that the UE can be a UE to receive conflict information as defined in Clause 16.3.0 of [5, TS 38.213]; - 0 bit otherwise. <Unchanged parts omitted> 8.4.1.3 SCI format 2-C <Unchanged parts omitted> - Resource set type – 1 bit, where value 0 indicates a request for inter-UE coordination information providing preferred resource set and value 1 indicates a request for inter-UE coordination information providing non-preferred resource set, if higher layer parameter determineResourceSetTypeScheme1 sl-DetermineResourceType is configured to 'UE-B's request ueb'; otherwise, 0 bit. <Unchanged parts omitted> 38.213 16.3.0 UE procedure for transmitting PSFCH with control information <Unchanged parts omitted> A UE can be enabled, by sl-iInter-UE-CoordinationScheme2, to transmit a PSFCH with conflict information in a resource pool. The UE can determine, based on an indication by a SCI format 1-A, a set of resources that includes one or more slots and resource blocks that are reserved for PSSCH transmission. If the UE determines a conflict for a reserved resource for PSSCH transmission, the UE provides conflict information in a PSFCH. <Unchanged parts omitted> A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 sl-TypeUE-A is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 sl-TypeUE-A is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 sl-IndicationUE-B = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 The first UE can be provided conditions by optionForCondition2A1Scheme2 sl-OptionForCondition2-A-1 to determine conflict of reserved resources in a resource pool - if optionForCondition2A1Scheme2 sl-OptionForCondition2-A-1 = 'RSRP-ThresPerPriorities' 0, the first UE can be provided by, ThresPSSCH-RSRP-List Thpi,pj, a list of RSRP thresholds for each priority combination pi,pj [6, TS 38.214] - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if the RSRP [6, TS 38.214] of the third UE is above a threshold Thp2,p1 - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if the RSRP of the second UE is above a threshold Thp1,p2 - if optionForCondition2A1Scheme2 sl-OptionForCondition2-A-1 = 'RSRP-ThresWithRsrpMeasurement' 1, the first UE can be provided a value Delta_Th by deltaRSRPThresh - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if RSRP2>RSRP1+Delta_Th, where RSRP1 and RSRP2 are the RSRP measurements from the first UE for the second UE and the third UE, respectively - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if RSRP1>RSRP2+Delta_Th If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on PSFCHOccasionScheme2 sl-PSFCH-Occasion - If PSFCHOccasionScheme2 sl-PSFCH-Occasion = 'followSCI' 0, the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information. - If PSFCHOccasionScheme2 sl-PSFCH-Occasion = 'followReservedResource' 1, the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information. <Unchanged parts omitted> 16.3.1 UE procedure for receiving PSFCH with control information <Unchanged parts omitted> A UE that transmitted SCI format 1-A, indicating one or more reserved resources in a resource pool enabled by sl-iInter-UECoordinationScheme2, attempts to receive associated PSFCH with conflict information in a resource pool with PSFCH resources that the UE determines as described in clause 16.3.0. If the UE determines presence of a resource conflict based on conflict information in a PSFCH reception, the UE reports the resource conflict to higher layers - if slotLevelResourceExclusionScheme2 sl-SlotLevelResourceExclusion is not provided, the UE reports resources overlapping with a next in time reserved resource indicated by the SCI format 1-A - if slotLevelResourceExclusionScheme2 sl-SlotLevelResourceExclusion is provided, the UE reports resources in a slot of a next in time reserved resource indicated by the SCI format 1-A If a UE receives a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE receives the PSFCH in the resource pool in a slot determined based on PSFCHOccasionScheme2 sl-PSFCH-Occasion - If PSFCHOccasionScheme2 sl-PSFCH-Occasion = 'followSCI' 0, the UE receives the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH transmission that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not receive the PSFCH with conflict information - If PSFCHOccasionScheme2 sl-PSFCH-Occasion = 'followReservedResource' 1, the UE receives the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH transmission that provides the SCI format 1-A; otherwise, the UE does not receive the PSFCH with conflict information <Unchanged parts omitted> 38.214 8.1 UE procedure for transmitting the physical sidelink shared channel <Unchanged parts omitted> - if [Providing/Requesting indicator] indicates SCI format 2-C is used to convey an explicit request for inter-UE coordination information: - the UE shall set value of the ['Priority'] field as indicated by higher layers. - the UE shall set value of the ['Number of subchannels'] field as indicated by higher layers. - the UE shall set value of the ['Resource reservation period'] field as indicated by higher layers. - the UE shall set value of the ['Resource selection window location'] field as indicated by higher layers. - the UE shall set value of the ['Resource set type'] field as indicated by higher layers if higher layer parameter determineResourceSetTypeScheme1 sl-DetermineResourceType is configured to 'UE-B's request ueb'; otherwise this field is omitted. - if [Providing/Requesting indicator] indicates SCI format 2-C is used to convey inter-UE coordination information: - the UE shall set value of the ['Resource set type'] field as indicated by higher layers. - the UE shall set value of the ['Resource combination(s) '] field (clause 8.1.5A) as indicated by higher layers. - the UE shall set value of the ['Lowest subchannel indices'] as indicated by higher layers - the UE shall set value of the ['First resource location'] as indicated by higher layers - the UE shall set value of the ['Reference slot location'] as indicated by higher layers <Unchanged parts omitted> 8.1.4A UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission <Unchanged parts omitted> When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4 with the above parameters and the following modifications: - Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met: - the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined; - the higher layer parameter condition1A2Scheme1Disabled sl-Condition1-A-2 is not set to 'Disabled'. When determining a non-preferred resource set, the UE considers any resource(s) within the resource selection window, if indicated by a received explicit request, and satisfying at least one of the following conditions as non-preferred resource(s): - resource(s) indicated by a received [SCI format 1-A], satisfying at least one of the following criteria: - the RSRP measurement performed, according to clause 8.4.2.1, for the received [SCI format 1-A], is higher than ThprioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in thresholdRSRPCondition1B1Option1Scheme1 sl-ThresholdRSRP-Condition1-B-1-Option1List, where sl-Priority in the k-th field indicates k=pi. - the UE is a destination UE of a TB associated with the received [SCI format 1-A] and the RSRP measurement performed, according to clause 8.4.2.1 for the received [SCI format 1-A], is lower than Th'prioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th'(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in thresholdRSRPCondition1B1Option2Scheme1 sl-ThresholdRSRP-Condition1-B-1-Option2List, where sl-Priority in the k-th field indicates k=pi. - resources(s) in slot(s) in which the UE does not expect to perform SL reception due to half duplex operation, if the UE is a destination UE of a TB for whose transmission the non-preferred resource set is being determined. 8.1.4B UE procedure for determining a resource conflict Void A UE configured with the higher layer parameter sl-iInter-UE-CoordinationScheme2 enabling transmission of a resource conflict indication considers that a resource conflict occurs on a first reserved resource r1 indicated by a first received SCI format if at least one of the following conditions is satisfied: - the first reserved resource r1 overlaps with a second reserved resource r2 indicated by a second received SCI format, and - if [the higher layer parameter for enabling Options 1/4 in Condition 2-A-1] indicates [Option 1 enabled], - if the UE is a destination UE of a TB to be transmitted in r1, the RSRP measurement performed for the second received SCI format RSRP2 is higher than Thprio2,prio1 where prio1 and prio2 are the priorities indicated in the first and second received SCI format, respectively. - if the UE is a destination UE of a TB to be transmitted in r2, the RSRP measurement performed for the first received SCI format RSRP1 is higher than Thprio1,prio2 where prio1 and prio2 are the priorities indicated in the first and second received SCI format, respectively. - if [the higher layer parameter for enabling Options 1/4 in Condition 2-A-1] indicates [Option 4 enabled] and the UE supports [Option 4], - if the UE is a destination UE of a TB to be transmitted in r1, RSRP2-RSRP1 is higher than a (pre)configured RSRP threshold. - if the UE is a destination UE of a TB to be transmitted in r2, RSRP1-RSRP2 is higher than a (pre)configured RSRP threshold. - where the RSRP1, RSRP2 measurements are performed according to clause 8.4.2.1 and the parameter Th(pi,pj) is determined [as described in step 3 of clause 8.1.4]. - the first reserved resource r1 occurs in a slot in which the UE does not expect to perform SL reception due to half-duplex operation and the UE is a destination UE of a TB to be transmitted in resource r1. 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter sl-iInter-UE-CoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. <Unchanged parts omitted>. [ASUSTeK, R1-2207483] Reason for change:; In the current 38.213, several RRC parameters for inter-UE coordination scheme 2 are used, which are not aligned to current RRC specification. Field name in SCI format 1-A is not aligned between 38.212 and 38.213.. Summary of change:; Change “inter-UECoordinationScheme2” to “sl-IUC-Scheme2”, change “sl-PSFCH-Conflict-RB-Set” to “sl-RB-SetPSFCH”, change “typeAUEScheme2” to “sl-TypeUE-A”, change “indicationUEBScheme2” to “sl-IndicationUE-B”, change “optionForCondition2A1Scheme2” to “sl-OptionForCondition2-A-1” and corresponding value setting to '0', '1', change “PSFCHOccasionScheme2” to “sl-PSFCH-Occasion” and corresponding value setting to '0', '1', change “slotLevelResourceExclusionScheme2” to “sl-SlotLevelResourceExclusion” and “provided” is changed to “enabled”. Change “indicationUEB flag” to “Conflict information receiver flag” in 38.213.. Consequences if not approved:; RRC parameters are used incorrectly which causes confusion on performing inter-UE coordination scheme 2.. ---------------------------------------< Text Proposal 3-1 for 38.213> -------------------------------------- ### 16.3.0 UE procedure for transmitting PSFCH with control information A UE can be indicated by an SCI format scheduling a PSSCH reception to transmit a PSFCH with HARQ-ACK information in response to the PSSCH reception. The UE provides HARQ-ACK information that includes ACK or NACK, or only NACK. A UE can be provided, by sl-PSFCH-Period, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled. A UE can be enabled, by sl-IUC-Scheme2, to transmit a PSFCH with conflict information in a resource pool. The UE can determine, based on an indication by a SCI format 1-A, a set of resources that includes one or more slots and resource blocks that are reserved for PSSCH transmission. If the UE determines a conflict for a reserved resource for PSSCH transmission, the UE provides conflict information in a PSFCH. … A UE is provided by sl-PSFCH-RB-Set a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information in a PRB of the resource pool. A UE can be provided by sl-RB-SetPSFCH a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with conflict information in a PRB of the resource pool. A UE expects that different PRBs are (pre)configured for conflict information and HARQ-ACK information. … A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - determines to transmit to the second UE the PSFCH with the conflict information. A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, sl-TypeUE-A is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool sl-TypeUE-A is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the Conflict information receiver flag flag in SCI Format 1-A from the second UE and the third UE is set to 1, if sl-IndicationUE-B = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 The first UE can be provided conditions by sl-OptionForCondition2-A-1 to determine conflict of reserved resources in a resource pool - if sl-OptionForCondition2-A-1 = '0', the first UE can be provided by, ThresPSSCH-RSRP-List Thpi,pj, a list of RSRP thresholds for each priority combination pi,pj [6, TS 38.214] - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if the RSRP [6, TS 38.214] of the third UE is above a threshold Thp2,p1 - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if the RSRP of the second UE is above a threshold Thp1,p2 - if sl-OptionForCondition2-A-1 = '1', the first UE can be provided a value Delta_Th by deltaRSRPThresh - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if RSRP2>RSRP1+Delta_Th, where RSRP1 and RSRP2 are the RSRP measurements from the first UE for the second UE and the third UE, respectively - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if RSRP1>RSRP2+Delta_Th If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on sl-PSFCH-Occasion - If sl-PSFCH-Occasion = '0', the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information. - If sl-PSFCH-Occasion = '1', the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information. ### 16.3.1 UE procedure for receiving PSFCH with control information … A UE that transmitted SCI format 1-A, indicating one or more reserved resources in a resource pool enabled by sl-IUC-Scheme2, attempts to receive associated PSFCH with conflict information in a resource pool with PSFCH resources that the UE determines as described in clause 16.3.0. If the UE determines presence of a resource conflict based on conflict information in a PSFCH reception, the UE reports the resource conflict to higher layers - if sl-SlotLevelResourceExclusion is not enabled, the UE reports resources overlapping with a next in time reserved resource indicated by the SCI format 1-A - if sl-SlotLevelResourceExclusion is enabled, the UE reports resources in a slot of a next in time reserved resource indicated by the SCI format 1-A If a UE receives a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE receives the PSFCH in the resource pool in a slot determined based on sl-PSFCH-Occasion - If sl-PSFCH-Occasion = '0', the UE receives the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH transmission that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not receive the PSFCH with conflict information - If sl-PSFCH-Occasion = '1', the UE receives the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH transmission that provides the SCI format 1-A; otherwise, the UE does not receive the PSFCH with conflict information -----------------------------------------< End of Text Proposal 3-1> --------------------------------------- ---------------------------------------< Text Proposal 3-2 for 38.214> -------------------------------------- ### 8.1.4B UE procedure for determining a resource conflict A UE configured with the higher layer parameter sl-IUC-Scheme2 enabling transmission of a resource conflict indication considers that a resource conflict occurs on a first reserved resource r1 indicated by a first received SCI format if at least one of the following conditions is satisfied: -----------------------------------------< End of Text Proposal 3-2> --------------------------------------- ---------------------------------------< Text Proposal 3-3 for 38.212> -------------------------------------- #### 8.3.1.1 SCI format 1-A SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH The following information is transmitted by means of the SCI format 1-A: - … - Reserved – a number of bits as determined by the following: - Nreserved bits as configured by higher layer parameter sl-NumReservedBits, with value set to zero, if higher layer parameter sl-IndicationUE-B is not configured, or if higher layer parameter sl-IndicationUE-B is configured to 'Disabled'; - (Nreserved-1) bits otherwise, with value set to zero. - Conflict information receiver flag – 0 or 1 bit - 1 bit if higher layer parameter sl-IndicationUE-B is configured to 'Enabled', where the bit value of 0 indicates that the UE cannot be a UE to receive conflict information and the bit value of 1 indicates that the UE can be a UE to receive conflict information as defined in Clause 16.3.0 of [5, TS 38.213]; - 0 bit otherwise. -----------------------------------------< End of Text Proposal 3-3> --------------------------------------- Proposal 3: Adopt text proposals 3-1~3-3 in updating of TS38.213, 38.214, 38.212.

      - Draft Proposals

Draft Proposal 1:

- The followings (e.g., related to the misalignment of RRC parameter names/values between TS 38.213/TS 38.214/TS 38.212 and TS 38.331) will provided to the specification editor, and companies can check them out in the CR discussion.

--------------------------- Start of text proposal to TS 38.213 v17.2.0 --------------------------- ### 16.3.0 UE procedure for transmitting PSFCH with control information A UE can be indicated by an SCI format scheduling a PSSCH reception to transmit a PSFCH with HARQ-ACK information in response to the PSSCH reception. The UE provides HARQ-ACK information that includes ACK or NACK, or only NACK. A UE can be provided, by sl-PSFCH-Period, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled. A UE can be enabled, by sl-IUC-Scheme2, to transmit a PSFCH with conflict information in a resource pool. The UE can determine, based on an indication by a SCI format 1-A, a set of resources that includes one or more slots and resource blocks that are reserved for PSSCH transmission. If the UE determines a conflict for a reserved resource for PSSCH transmission, the UE provides conflict information in a PSFCH. … A UE is provided by sl-PSFCH-RB-Set a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information in a PRB of the resource pool. A UE can be provided by sl-RB-SetPSFCH a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with conflict information in a PRB of the resource pool. A UE expects that different PRBs are (pre)configured for conflict information and HARQ-ACK information. … A UE determines a number of PSFCH resources available for multiplexing HARQ-ACK or conflict information in a PSFCH transmission as RPRB, CSPSFCH=Ntype PSFCH⋅Msubch, slotPSFCH⋅NCSPSFCH where NCSPSFCH is a number of cyclic shift pairs for the resource pool provided by sl-NumMuxCS-Pair and, based on an indication by sl-PSFCH-CandidateResourceType, - if sl-PSFCH-CandidateResourceType is configured as startSubCH, Ntype PSFCH=1 and the Msubch, slotPSFCH PRBs are associated with the starting sub-channel of the corresponding PSSCH - if sl-PSFCH-CandidateResourceType is configured as allocSubCH, Ntype PSFCH=Nsubch PSSCH and the Nsubch PSSCH⋅Msubch, slotPSFCH PRBs are associated with the Nsubch PSSCH sub-channels of the corresponding PSSCH - for conflict information, the corresponding PSSCH is determined based on sl-PSFCH-Occasion … A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - determines to transmit to the second UE the PSFCH with the conflict information. A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, sl-TypeUE-A is not provided, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool sl-TypeUE-A is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the Conflict information receiver flag flag in SCI Format 1-A from the second UE and the third UE is set to 1, if sl-IndicationUE-B = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 The first UE can be provided conditions by sl-OptionForCondition2-A-1 to determine conflict of reserved resources in a resource pool - if sl-OptionForCondition2-A-1 = '0', the first UE can be provided by, ThresPSSCH-RSRP-List Thpi,pj, a list of RSRP thresholds for each priority combination pi,pj [6, TS 38.214] - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if the RSRP [6, TS 38.214] of the third UE is above a threshold Thp2,p1 - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if the RSRP of the second UE is above a threshold Thp1,p2 - if sl-OptionForCondition2-A-1 = '1', the first UE can be provided a value Delta_Th by deltaRSRPThresh - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if RSRP2>RSRP1+Delta_Th, where RSRP1 and RSRP2 are the RSRP measurements from the first UE for the second UE and the third UE, respectively - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if RSRP1>RSRP2+Delta_Th If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on sl-PSFCH-Occasion - If sl-PSFCH-Occasion = '0', the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information. - If sl-PSFCH-Occasion = '1', the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information. ### 16.3.1 UE procedure for receiving PSFCH with control information … A UE that transmitted SCI format 1-A, indicating one or more reserved resources in a resource pool enabled by sl-IUC-Scheme2, attempts to receive associated PSFCH with conflict information in a resource pool with PSFCH resources that the UE determines as described in clause 16.3.0. If the UE determines presence of a resource conflict based on conflict information in a PSFCH reception, the UE reports the resource conflict to higher layers - if sl-SlotLevelResourceExclusion is not provided, the UE reports resources overlapping with a next in time reserved resource indicated by the SCI format 1-A - if sl-SlotLevelResourceExclusion is enabled, the UE reports resources in a slot of a next in time reserved resource indicated by the SCI format 1-A If a UE receives a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE receives the PSFCH in the resource pool in a slot determined based on sl-PSFCH-Occasion - If sl-PSFCH-Occasion = '0', the UE receives the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH transmission that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not receive the PSFCH with conflict information - If sl-PSFCH-Occasion = '1', the UE receives the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH transmission that provides the SCI format 1-A; otherwise, the UE does not receive the PSFCH with conflict information --------------------------- End of Text proposal to TS 38.213 v17.2.0 ---------------------------

----------------------------- Start of text proposal to TS 38.214 v17.2.0---------------------------- ### 8.1 UE procedure for transmitting the physical sidelink shared channel <Unchanged parts omitted> The UE shall set the contents of the SCI format 2-C as follows: - the UE shall set value of the 'HARQ process number' field as indicated by higher layers. - the UE shall set value of the 'NDI' field as indicated by higher layers. - the UE shall set value of the 'Redundancy version' field as indicated by higher layers. - the UE shall set value of the 'Source ID' field as indicated by higher layers. - the UE shall set value of the 'Destination ID' field as indicated by higher layers. - the UE shall set value of the 'HARQ feedback enabled/disabled indicator' field as indicated by higher layers. - the UE shall set value of the 'CSI request' field as indicated by higher layers. - the UE shall set value of [request/coordination information flag] field as indicated by higher layers. - if [Providing/Requesting indicator] indicates SCI format 2-C is used to convey an explicit request for inter-UE coordination information: - the UE shall set value of the ['Priority'] field as indicated by higher layers. - the UE shall set value of the ['Number of subchannels'] field as indicated by higher layers. - the UE shall set value of the ['Resource reservation period'] field as indicated by higher layers. - the UE shall set value of the ['Resource selection window location'] field as indicated by higher layers. - the UE shall set value of the ['Resource set type'] field as indicated by higher layers if higher layer parameter determineResourceSetTypeScheme1 sl-DetermineResourceType is configured to 'UE-B's request ueb'; otherwise this field is omitted. - if [Providing/Requesting indicator] indicates SCI format 2-C is used to convey inter-UE coordination information: - the UE shall set value of the ['Resource set type'] field as indicated by higher layers. - the UE shall set value of the ['Resource combination(s) '] field (clause 8.1.5A) as indicated by higher layers. - the UE shall set value of the ['Lowest subchannel indices'] as indicated by higher layers - the UE shall set value of the ['First resource location'] as indicated by higher layers - the UE shall set value of the ['Reference slot location'] as indicated by higher layers <Unchanged parts omitted> … ### 8.1.4A UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission … When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4 with the above parameters and the following modifications: - Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met: - the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined; - the higher layer parameter sl-Condition1-A-2 is not set to 'Disabled'. When determining a non-preferred resource set, the UE considers any resource(s) within the resource selection window, if indicated by a received explicit request, and satisfying at least one of the following conditions as non-preferred resource(s): - resource(s) indicated by a received [SCI format 1-A], satisfying at least one of the following criteria: - the RSRP measurement performed, according to clause 8.4.2.1, for the received [SCI format 1-A], is higher than ThprioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in sl-ThresholdRSRP-Condition1-B-1-Option1List, where sl-Priority in the k-th field indicates k=pi - the UE is a destination UE of a TB associated with the received [SCI format 1-A] and the RSRP measurement performed, according to clause 8.4.2.1 for the received [SCI format 1-A], is lower than Th'prioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th'(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in sl-ThresholdRSRP-Condition1-B-1-Option2List, where sl-Priority in the k-th field indicates k=pi. - resources(s) in slot(s) in which the UE does not expect to perform SL reception due to half duplex operation, if the UE is a destination UE of a TB for whose transmission the non-preferred resource set is being determined. ### 8.1.4B UE procedure for determining a resource conflict A UE configured with the higher layer parameter sl-IUC-Scheme2 enabling transmission of a resource conflict indication considers that a resource conflict occurs on a first reserved resource r1 indicated by a first received SCI format if at least one of the following conditions is satisfied: … ### 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter sl-InterUE-CoordinationScheme1-r17 uses a received non-preferred resource set as follows when performing resource (re-)selection: … --------------------------- End of Text proposal to TS 38.214 v17.2.0 ---------------------------

--------------------------- Start of text proposal to TS 38.212 v17.2.0 --------------------------- #### 8.3.1.1 SCI format 1-A SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH The following information is transmitted by means of the SCI format 1-A: - … - Reserved – a number of bits as determined by the following: - Nreserved bits as configured by higher layer parameter sl-NumReservedBits, with value set to zero, if higher layer parameter sl-IndicationUE-B is not configured, or if higher layer parameter sl-IndicationUE-B is configured to 'Disabled'; - (Nreserved-1) bits otherwise, with value set to zero. - Conflict information receiver flag – 0 or 1 bit - 1 bit if higher layer parameter sl-IndicationUE-B is configured to 'Enabled', where the bit value of 0 indicates that the UE cannot be a UE to receive conflict information and the bit value of 1 indicates that the UE can be a UE to receive conflict information as defined in Clause 16.3.0 of [5, TS 38.213]; - 0 bit otherwise. …. #### 8.4.1.3 SCI format 2-C <Unchanged parts omitted> - Resource set type – 1 bit, where value 0 indicates a request for inter-UE coordination information providing preferred resource set and value 1 indicates a request for inter-UE coordination information providing non-preferred resource set, if higher layer parameter determineResourceSetTypeScheme1 sl-DetermineResourceType is configured to 'UE-B's request ueb'; otherwise, 0 bit. <Unchanged parts omitted> --------------------------- End of Text proposal to TS 38.212 v17.2.0 ---------------------------

- Proposals for discussion in online sessions

TBD

- List of RAN1 agreements on IUC for Mode 2 enhancements

## 1) RAN1#103-e meeting

- Conclusion:

  - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B:

    - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

        - e.g., based on its sensing result

    - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

        - e.g., based on its sensing result and/or expected/potential resource conflict

    - UE-A sends to UE-B the set of resource where the resource conflict is detected

    - FFS: details of resource conflict, e.g., including type of resource conflict

    - FFS: details of sensing operation at UE-A side

    - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s)

    - Note: these different types may be used in combination with each other

  - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required

  - Send an LS to RAN plenary

    - Final LS in R1-2009841

- Conclusion:

  - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed.

    - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling

    - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

    - How UE-A and UE-B are determined

    - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

    - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

    - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

## 2) RAN1#104-e meeting

- Conclusion:

  - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature.

    - The detailed observations can be found in the attachment of the LS

- Draft LS in R1-2102165, along with the attachment R1-2102166, is approved (with a typo fix)

  - Final LS in R1-2102168

## 3) RAN1#104bis-e meeting

- Agreement:

  - Support the following schemes of inter-UE coordination in Mode 2:

    - Inter-UE Coordination Scheme 1:

      - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission

        - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information

      - FFS condition(s) in which Scheme 1 is used

    - Inter-UE Coordination Scheme 2:

      - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI

        - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict

      - FFS condition(s) in which Scheme 2 is used

- Agreement:

  - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination:

    - Details include applicable scenario(s)/inter-UE coordination scheme(s)

    - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc.

      - Including the possibility of being subject to certain conditions and/or capability

- Agreement:

  - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission

    - For scheme 1:

      - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

      - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

      - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information

      - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information

    - For scheme 2:

      - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

      - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

## 4) RAN1#106-e meeting

- Agreement:

  - For scheme 1, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B.

    - Set of resources preferred for UE-B’s transmission

    - Set of resources non-preferred for UE-B’s transmission

- Agreement:

  - For scheme 2, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B

    - Presence of expected/potential resource conflict on the resources indicated by UE-B’s SCI

      - FFS: UE behaviour when the presence of expected/potential resource conflict is detected by the transmitter

    - FFS: Whether to additionally support the presence of detected resource conflict on the resources indicated by UE-B’s SCI

- Agreement:

  - In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2:

    - A UE that sends an explicit request for inter-UE coordination information can be UE-B

    - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A

    - (Working assumption) At least a destination UE of a TB transmitted by UE-B can be UE A

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

  - (Working Assumption) In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by a condition other than explicit request reception in Mode 2:

    - A UE that satisfies the condition mentioned in the main bullet and sends inter-UE coordination information is UE-A

    - A UE that received inter-UE coordination information from UE-A and uses it for resource (re-)selection is UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

- Agreement:

  - In scheme 2, at least the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination transmission triggered by a detection of expected/potential resource conflict(s) in Mode 2:

    - A UE that transmitted PSCCH/PSSCH with SCI indicating reserved resource(s) to be used for its transmission, received inter-UE coordination information from UE-A indicating expected/potential resource conflict(s) for the reserved resource(s), and uses it to determine resource re-selection is UE-B

    - A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A

      - (Working assumption) At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s)

        - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured

      - FFS: Additional details and condition(s) on UE-A and UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Definition of expected/potential resource conflict(s) and other details (if any)

- Agreement:

  - In scheme 2, the following UE-B’s behavior in its resource (re)selection is supported when it receives inter-UE coordination information from UE-A:

    - UE-B can determine resource(s) to be re-selected based on the received coordination information

      - UE-B can reselect resource(s) reserved for its transmission when expected/potential resource conflict on the resource(s) is indicated

        - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

    - For preferred resource set, the following two options are supported:

      - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result

          - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met

            - FFS: Details of condition(s)

          - This option is supported when UE-B performs sensing/resource exclusion

          - FFS: Other details (if any)

      - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set

          - This option is supported at least when UE-B does not support sensing/resource exclusion

            - FFS: Whether the support is conditional or UE capability

          - FFS: Other details (if any)

      - FFS: Other option(s), and other details (if any)

    - For non-preferred resource set,

      - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

          - FFS: Details including

            - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any)

            - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

        - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set

      - FFS: Other option(s), and other details (if any)

- Agreement:

  - In scheme 2, at least the following is supported to determine inter-UE coordination information:

    - Among resource(s) indicated by UE-B’s SCI, UE-A considers that expected/potential resource conflict occurs on the resource(s) satisfying at least one of the following condition(s):

      - Condition 2-A-1:

        - Other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency

        - FFS: Other details (if any)

        - FFS: Whether/how to specify additional criteria and other details (if any) including signaling details of conflict indication

      - (Working Assumption) Condition 2-A-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of preferred resource set:

    - UE-A considers any resource(s) satisfying all the following condition(s) as set of resource(s) preferred for UE-B’s transmission

      - Condition 1-A-1:

        - Resource(s) excluding those overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than a RSRP threshold

          - FFS: Other details (if any)

      - FFS: Condition 1-A-2:

        - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Condition 1-A-3:

        - Resource(s) satisfying UE-B’s traffic requirement (if available)

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of non-preferred resource set:

    - UE-A considers any resource(s) satisfying at least one of the following condition(s) as set of resource(s) non-preferred for UE-B’s transmission

      - Condition 1-B-1:

        - Reserved resource(s) of other UE identified by UE-A from other UEs’ SCI (including priority field) and RSRP measurement

          - FFS: Other details (if any)

      - FFS: Condition 1-B-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

## 5) RAN1#106bis-e meeting

- Agreement:

  - For Scheme 2, PSFCH format 0 is used to convey the presence of expected/potential resource conflict on reserved resource(s) indicated by UE-B’s SCI

- Agreement:

  - For Condition 2-A-1 of Scheme 2, down-select one or more of following additional criteria to determine resource(s) where expected/potential resource conflict occurs

    - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

      - Strive to reuse Rel-16 specification wherever possible

    - Option 2: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is within a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - Option 3: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) and the other UE is within a distance threshold of UE-B as determined by both UEs’ SCIs.

    - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - FFS: In case of collisions of resources for two UEs having TBs with UE A as destination UE, if needed

- Working Assumption

  - For Condition 1-B-1 of Scheme 1, the following two options are supported

    - Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s)

    - Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s)

- Working Assumption

  - For Scheme 1 with non-preferred resource set, support following condition:

    - Condition 1-B-2:

      - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

- Agreement:

  - For Condition 1-A-1 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - When the inter-UE coordination information transmission is triggered by UE-B’s explicit request, the candidate single-slot resource(s) are determined in the same way according to Rel-16 TS 38.214 Section 8.1.4 with at least following parameters provided by signaling from UE-B. FFS whether or not to apply RSRP threshold increase in Step 7) of Rel-16 TS 38.214 Section 8.1.4.

      - Priority value to be used for PSCCH/PSSCH transmission

        - It replaces prio_TX

      - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

        - It replaces L_subCH

      - Resource reservation interval

        - It replaces P_rsvp_TX

      - FFS: Starting/ending time location of resource selection window

    - FFS : In addition to Rel-16 procedure, use inter-UE coordination information from other UEs

      - If there is no consensus in RAN1#106bis-e, no further discussions for Rel-17

- Conclusion:

  - No consensus that UE-A uses inter-UE coordination information from other UEs when it determines the preferred resource set for Condition 1-A-1 of Scheme 1.

- Working Assumption

  - For Scheme 1 with preferred resource set, support following condition:

    - Condition 1-A-2:

      - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

      - This can be disabled by RRC (pre-)configuration

- Agreement:

  - For allocating PSFCH resources in Scheme 2, at least following can be (pre)configured separately from those for SL HARQ-ACK feedback.

    - Set of PRBs for PSFCH transmission/reception (sl-PSFCH-RB-Set)

- Agreement:

  - For Scheme 2,

    - Index of a PSFCH resource for inter-UE coordination information transmission is determined in the same way according to Rel-16 TS 38.213 Section 16.3 with at least following modification

      - P_ID is L1-Source ID indicated by UE-B’s SCI

      - M_ID is 0

    - FFS: How to set m_CS

    - FFS: How to set m_0

    - FFS: Whether M_ID can be (pre)configured

## 6) RAN1#107-e meeting

- Agreement:

  - A resource pool level (pre-)configuration uses either of the following options

    - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted

      - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication

      - Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3

    - Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI

      - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least T_3 slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs

      - FFS: How to account for processing timeline

    - Note that it is possible not to configure either option1 or option 2.

- Agreement:

  - For Condition 1-A-2 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - UE-A excludes candidate single-slot candidate(s) belonging to “slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation” after Step 6) of TS 38.214 Section 8.1.4

- Agreement:

  - When PSFCH TX/RX for Scheme 2 is overlapping with LTE SL TX/RX and/or UL in a UE, reuse prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

- Conclusion:

  - For Scheme 2, the values of the following parameters are the same as those for SL HARQ-ACK feedback in the same resource pool

    - Period of PSFCH resources (sl-PSFCH-Period)

    - Number of cyclic shift pairs used for a PSFCH transmission that can be multiplexed in a PRB (sl-NumMuxCS-Pair)

    - Number of PSFCH resources available for multiplexing information in a PSFCH transmission (sl-PSFCH-CandidateResourceType)

- Agreement:

  - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

    - Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

    - Alt 2: MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

    - FFS: Whether/How to use resource reservation information as coordination information

- Working Assumption:

  - A resource pool level (pre-)configuration can enable one of the following options:

    - Option 1:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively

    - Option 4:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s).

      - Support of Option 4 is subject to UE capability

    - FFS: Whether/how RSRP threshold depends on priority, MCS, overlap

- Agreement:

  - For Scheme 1 with non-preferred resource set,

    - Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

- Agreement:

  - For Condition 1-A-1 of Scheme 1, when UE-A determines the set of resources preferred for UE-B’s transmission, apply RSRP threshold increase in the same way according to Rel-16 TS 38.214 Section 8.1.4.

    - FFS: Whether/how to introduce the maximum limit of RSRP threshold increase

- Agreement:

  - For Scheme 1, at least following parameters are provided by UE-B’s request:

    - Priority value to be used for PSCCH/PSSCH transmission

    - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

    - Resource reservation interval

- Agreement:

  - For Scheme 2, when PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI,

    - Time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value.

      - FFS: Details of X

- Working Assumption:

  - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value is UE-B.

    - FFS whether/how to set additional condition for UE-A to send PSFCH.

    - Conclude on whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2 at the subsequent meetings

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1,

    - UE-A uses a TX resource pool used for UE-B’s request transmission to determine the set of resources and to transmit the set of resources to UE-B

- Agreement:

  - For inter-UE coordination information triggered by a condition rather than request reception in Scheme 1,

    - UE-A transmitting in a resource pool provides inter-UE coordination information associated with the same resource pool

## 7) RAN1#107bis-e meeting

- Agreement:

  - For Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

    - Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request

      - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index

- Agreement:

  - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value

    - X = sl-MinTimeGapPSFCH

  - UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied

- Agreement:

  - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

    - (Working assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A

      - A single format SCI 2-C is used for inter-UE coordination information and request

        - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information

      - SCI 2-C is UE RX optional

      - It is up to UE implementation to additionally use 2nd SCI (for UE-B).

    - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A

- Conclusion:

  - For Scheme 2, there is no consensus to support indication of the following

    - Condition type of a resource conflict

    - Time location of a resource conflict

- Agreement:

  - For Scheme 2,

    - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

    - When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

      - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

    - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B

- Agreement:

  - For PSFCH TX/RX or TX/TX prioritization in Scheme 2,

    - Priority value of PSFCH TX for a resource conflict indication is the smallest priority value of the conflicting TBs

    - Priority value of PSFCH RX for a resource conflict indication is priority value indicated by UE-B’s SCI

    - For PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s), PSFCH TX/RX for SL HARQ-ACK feedback is always prioritized over PSFCH TX/RX for a resource conflict indication

- Agreement:

  - For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information

    - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request

- Working Assumption:

  - For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception

    - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set

      - FFS: Under which conditions groupcast/broadcast can be supported

    - Unicast

      - FFS: Under which conditions unicast can be supported

- Agreement:

  - For determining preferred resource set in Scheme 1, the value of Cresel is determined by UE-A according to Rel-16 procedure.

    - This information is not conveyed to/from UE-B

    - When inter-UE coordination information is triggered by UE-B’s request, P_rsvp_TX used for determining SL_RESOURCE_RESELECTION_COUNTER according to Rel-16 procedure is provided by resource reservation interval indicated by UE-B’s request

- Agreement:

  - For the indication of resource set in Scheme 1, the value of Sl-MaxNumPerReserve is fixed to 3.

- Agreement:

  - The following working assumption is confirmed with modification in RED.

    - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

          - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

- Agreement:

  - For inter-UE coordination information transmission in Scheme 1,

    - Inter-UE coordination information can be multiplexed with other data only if the source/destination ID pair is the same

      - Retransmission of the TB carrying inter-UE coordination information is supported

  - For explicit request transmission in Scheme 1,

    - Explicit request can be multiplexed with other data only if the source/destination ID pair is the same

      - Retransmission of the TB carrying request is supported

- Agreement:

  - For inter-UE coordination triggered by an explicit request in Scheme 1, whether or not to transmit the inter-UE coordination information upon the request reception is determined by UE-A’s implementation subject to the following procedures.

    - Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control

- Agreement:

  - For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation.

      - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B

    - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

- Agreement:

  - For inter-UE coordination triggered by UE-B’s explicit request in Scheme 1,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1: it is up to UE-B’s implementation whether or not to trigger the request generation

      - Alt 2: the request generation can be triggered only when UE-B has data to be transmitted to UE-A

    - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

- Agreement:

  - For Scheme 1 with preferred resource set Option A,

    - MAC layer selects resources using S_A and the received preferred resource set

      - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

        - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

      - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A under the constraint defined in Rel-16.

- Agreement:

  - For Scheme 1 with preferred resource set Option B,

    - MAC layer selects resources belonging to the received preferred resource set under the constraint defined in Rel-16

      - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request.

    - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B.

    - For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data

- Agreement:

  - For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration.

    - FFS: Otherwise, the priority value is determined by UE-A’s implementation.

    - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

- Agreement:

  - For sidelink transmission carrying inter-UE coordination information in Scheme 1,

    - UE-A performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B.

  - For sidelink transmission carrying request in Scheme 1,

    - UE-B performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion. Otherwise, at least UE-B can perform random selection

  - Note: RAN1 does not pursue specific enhancement of Rel-17 resource (re)selection for the transmission of inter-UE coordination information and its request.

- Working assumption:

  - First resource location of each TRIV is a slot offset with respect to a reference slot

    - Alt 2:

      - The slot offset is the number of logical slots from the reference slot

        - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256]

          - FFS: The detailed value range including granularity

        - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

    - For the reference slot,

      - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

- Agreement:

  - For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

    - Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters

      - prio_TX

      - L_subCH

      - P_rsvp_TX

    - UE-A determines by its implementation values of following parameters

      - n+T_1, n+T_2

    - FFS: Whether/how to support (pre)configuration of n+T_1 and n+T_2

    - Note that it is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the preferred resource set

- Agreement:

  - For inter-UE coordination information is triggered by UE-B’s request,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1:

        - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

          - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

      - Alt 2:

        - Resource set type to be provided by inter-UE coordination information transmission is indicated by UE-B’s request

          - UE-B’s request indicates either preferred resource set or non-preferred resource set

    - Note that it is up to RAN2 decision whether/how UE-B provides its support of sensing/resource exclusion to UE-A via PC5-RRC signaling and UE-A uses the received information to determine the type of resource set to be transmitted to UE-B

- Agreement:

  - For inter-UE coordination information is triggered by a condition other than explicit request reception,

    - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

      - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

- Working assumption:

  - For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not.

    - FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication

## 8) RAN1#108-e meeting

- Agreement:

  - For a slot offset that is (pre)configured to indicate the first resource location of each TRIV with respect to a reference slot,

    - Granularity of the slot offset is 1 logical slot

    - (Pre)configured maximum value of the slot offset is up to 8000

      - When both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the maximum value of the slot offset is 255

      - When MAC CE only is used as the container of inter-UE coordination information, the maximum value of the slot offset is the (pre)configured maximum value

- Agreement:

  - A SCI format 2-C includes all the fields present in SCI format 2-A except cast type indicator

- Conclusion:

  - For cast type(s) of inter-UE coordination information with preferred resource set triggered by a condition other than explicit request reception, there is no consensus in RAN1 on the support of groupcast or broadcast for preferred resource set

- Agreement

  - For Scheme 2, m_CS for a resource conflict indication for the next reserved resource indicated by the corresponding UE-B’s SCI for either current TB transmission or next TB transmission is 0

- Agreement

  - For Scheme 2, when UE-B receives a conflict indicator for resource(s) indicated by its SCI, it up to UE-B’s implementation whether/how to set the reservation periodicity in the re-selected resource.

- Agreement

  - For Scheme 2,

    - m_0 for a resource conflict indication is derived in the same way as specified for HARQ-ACK information in TS 38.213 Section 16.3

    - A UE expects that different PRBs are (pre)configured between conflict indication and HARQ-ACK information

- Agreement:

  - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

- Agreement:

  - For Scheme 1, when MAC CE only is used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

- Conclusion:

  - For inter-UE coordination operation in Rel-17, RAN1 understands that only UE(s) in mode 2 can be UE-A

    - Note that RAN1 does not pursue specific enhancement of Rel-17 inter-UE coordination operation for handling the case where UE(s) in mode 1 can be UE-A

- Agreement

  - Confirm the following working assumption with modification in RED

- Working assumption made in RAN1#107bis-e: - First resource location of each TRIV is a slot offset with respect to a reference slot - Alt 2: - The slot offset is the number of logical slots from the reference slot - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [8000256] - FFS: The detailed value range including granularity - Slot offset for each TRIV except for first TRIV to indicate the set of resources is separately indicated by inter-UE coordination information - Slot offset for first TRIV is 0 - For the reference slot, - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

- Agreement

  - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

    - For the indication of resource set, the following is supported:

      - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

        - First resource location of each TRIV is separately indicated by the inter-UE coordination information

      - If N <= 2, MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If N > 2, only MAC CE is used.

        - FFS: UE capability details

        - 2nd SCI is UE RX optional

        - The field size of the indication of resource set in a SCI format 2-C is determined by N=2

- Agreement

  - For Scheme 1, each bit field size of a SCI format 2-C for inter-UE coordination information is given by following table:

    - Note that lowest subchannel index for the first resource location of each TRIV is separately indicated by inter-UE coordination information

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | 2*log2(N subchannel SLNsubchannel SL + 12Nsubchannel SL + 16)+9+Y Where N subchannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_period with that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; Y=0 otherwise. |
| First resource location(s) | 8 |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | 2*log2NsubchannelSL where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel |
| (FFS) Actual number of resource combination | 1 Note: Support of this field is to be concluded by Feb 28. |

- Agreement

  - For Scheme 1, each bit field size of a SCI format 2-C for an explicit request for inter-UE coordination information is given by following table:

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Priority | 3 |
| Number of subchannels | log2(N subChannel SL) Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel |
| Resource reservation period | Y Where Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| Resource selection window location | 210+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 bit if determineResourceSetTypeScheme1 is set to ‘UE-B’s request’, otherwise, 0 bit |

  - This agreement does not imply that new field requested by RAN2 cannot be further added.

- Agreement

  - For Scheme 1, when MAC CE only is used as the container of inter-UE coordination information, each bit field size for inter-UE coordination information is given by following table from RAN1’s perspective, and RAN1 understands that the maximum value of N resource combinations to be conveyed in inter-UE coordination information is bounded so that the total payload size of inter-UE coordination information leads not to exceed the size of TB including the MAC CE

    - Details (e.g., whether/how to separately indicate the value of N in the inter-UE coordination information, how to put the following fields into MAC CE and the related field sizes in MAC CE) are up to RAN2

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | N*log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| First resource location(s) | N-1*log2(X) Where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | N*log2NsubchannelSL Where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel. |

- Conclusion:

  - There is no consensus in RAN1 on indicating actual number of resource combination in a SCI format 2-C for inter-UE coordination information.

    - Note: Different resource combinations can indicate the same set of resources for the case when only one resource combination is actually used

- Agreement

  - For Scheme 2,

    - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

    - When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

      - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

- Agreement

  - Confirm the following working assumption with modification in RED. Note that the terminology of “indicationUEB flag” means the indication of whether UE scheduling a conflict TB can be UE-B or not.

    - Working Assumption:

      - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A,

        - for each pair of UEs scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, a UE with the higher priority value is UE-B. When the UEs in the pair have the same priority value, UE-A determines one of the UEs to be UE-B by its implementation.

          - UE-A considers the SCIs received earlier than or equal to sl-MinTimeGapPSFCH before the PSFCH occasion for conflict indication when determining UE-B.

- Agreement

  - A UE performs PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s) first, and then the UE performs prioritization between prioritized PSFCH TX(s) or RX(s) and LTE SL TX/RX or UL by reusing prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

- Conclusion:

  - RAN1 does not pursue specific enhancement of Rel-17 inter-UE coordination operation for handling the overlapping between UL with SL-HARQ-ACK information and PSFCH for a conflict indication, i.e., there is no case in Rel-17 where the overlapping between UL with SL-HARQ-ACK information and PSFCH for a conflict indication occur at a UE performing inter-UE coordination operation

- Conclusion:

  - There is no consensus in RAN1 to further introduce enhancement in Rel-17 on Mode 2 resource selection procedure to ensure the timeline (i.e., minimum time gap between PSFCH and a slot where a SCI is transmitted of sl-MinTimeGapPSFCH, minimum time gap between PSFCH and a slot where expected/potential resource conflict occurs on PSSCH resource indicated by a SCI of T_3) for a conflict indication.

- Agreement

  - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same inter-UE coordination information is indicated in the SCI format 2-C and the MAC CE

    - Details (e.g., how to put the fields of SCI format 2C for inter-UE coordination information into MAC CE and the related field sizes in MAC CE) are up to RAN2

- Conclusion:

  - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted,

    - if there is a PSFCH occasion satisfying “the minimum time gap (sl-MinTimeGapPSFCH) between the PSFCH occasion and a slot where the SCI is transmitted” but not satisfying “the minimum time gap (T_3) between the PSFCH occasion and a slot of the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion”,

      - the PSFCH occasion cannot be used by UE-A for a conflict indication for reserved PSSCH resource other than the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion

- Agreement

  - (Pre)configuration of parameters related to n+T_1 and n+T_2 for determining the set of preferred resources in inter-UE coordination information triggered by a condition other than explicit request reception is not supported.

    - Note that T_2 is no smaller than T_2,min and 0 <= T_1 <= Tproc,1 as specified in TS 38.214 section 8.1.4.

- Agreement

  - For inter-UE coordination information transmission, only when the cast type of inter-UE coordination information is unicast regardless of whether or not it is multiplexed with other data, a SCI format 2-C can be used in addition to MAC CE

- Agreement

  - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A

    - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

  - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A

    - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e

  - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A

    - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

- Agreement

  - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As,

    - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set.

  - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As.

    - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline)

  - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

    - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

- Agreement

  - Notations:

    - (n+T_1) – Start slot of resource selection window for determining the set of resources

      - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_1) is provided by UE-B’s request as per the existing agreement

      - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_1) is determined by UE-A’s implementation as per the existing agreement

    - (n+T_2) – End slot of resource selection window for determining the set of resources

      - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_2) is provided by UE-B’s request as per the existing agreement

      - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_2) is determined by UE-A’s implementation as per the existing agreement

    - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information

    - (n’+T’_2) – End slot of resource selection window used for sidelink transmission carrying inter-UE coordination information

    - n' is the slot where UE procedure of determining TX resources of sidelink transmission carrying inter-UE coordination information is triggered

  - For inter-UE coordination information triggered by UE-B’s explicit request

    - Alt 1-1:

      - X1 ≤ (n’+T’_1)

      - (n’+T’_2) ≤ X2

  - For inter-UE coordination information triggered by a condition other than explicit request reception,

    - Alt 2-2:

      - (n’+T’_2) < X3

  - FFS: Values for X1, X2, X3

- Agreement

  - For sensing window for determining the set of resources in Scheme 1,

    - Notations:

      - For inter-UE coordination information triggered by UE-B’s explicit request, the values of (n+T_1) and (n+T_2) are provided by the request as per the existing agreement.

      - For inter-UE coordination information triggered by a condition other than explicit request reception, the values of (n+T_1) and (n+T_2) are determined by UE-A’s implementation as per the existing agreement.

      - T’’_1 is up to UE-A’s implementation under 0 <= T’’_1 <= Tproc,1

      - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information

      - n' is the slot where UE procedure of determining TX resources of inter-UE coordination information is triggered

    - Alt 1:

      - No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location (n+T_1) and (n+T_2) used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [(n+T_1) – T_0 – T’’_1, (n+T_1) – T_proc,0 – T’’_1].

- Agreement

  - For the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total in step 7),

    - It is up to UE-B’s implementation whether to take the received non-preferred resource set in its resource selection after step 6) to meet this requirement

## 8) RAN1#109-e meeting

- Agreement:

  - TP#1 (for TS38.213 v17.1.0, clause 16.4) in section 8.1 of R1-2203717 is endorsed.

  - TP#2 (for TS38.213 v17.1.0, clause 16.2.3) in section 8.2 of R1-2203717 is endorsed.

- Agreement:

  - When UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A or different UE-As, when UE-B has own sensing results

    - No RAN1 specification change to TS 38.214 is deemed necessary on how it uses the non-preferred resource set in its resource (re)selection

    - It is up to UE-B implementation to use the preferred resource set in its resource (re)selection for transmissions to the UE A providing the preferred resource set

- Agreement:

  - Remove the sentence below in Section 8.1.5A of TS 38.214

[When a preferred resource set is indicated by an SCI format 2-C, if the transmission of the set was triggered by an explicit request, the value of the resource reservation interval P_(rsvp,m) is set to 0.]

- Agreement:

  - X1, X2, and X3 are determined by UE-A’s implementation under the constraints defined in the specification (e.g., SL-LatencyBoundIUC-Report-r17, requirement of T_2min)

    - UE-B can choose to not use any resource from the preferred/non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission

      - For Tproc,2,

        - When only MAC CE is used for inter-UE coordination information transmission, it is equal to (Tproc,0+Tproc,1)

        - When MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission, it is equal to Tproc,0

          - Note: this is assuming that SCI format 2-C is received

    - Whether or not to make the time gap from the resource of inter-UE coordination information transmission to preferred/non-preferred resource in the inter-UE coordination information larger than (Tproc,0+Tproc,1+Tproc,2) is up to UE-A implementation

- Agreement:

  - Adopt the following text proposal to TS 38.214 v17.1.0:

    - Reason for change:

      - RAN1 discussed how to alleviate an ambiguity of UE-B’s behavior in terms of handling the field of resource reservation period in the inter-UE coordination information considering that there is no special mechanism that makes UE-B distinguishable whether the inter-UE coordination information is triggered by an explicit request or triggered by a condition, and agreed that this field is always valid.

    - Summary of change:

      - Delete the sentence of “[When a preferred resource set is indicated by an SCI format 2-C, if the transmission of the set was triggered by an explicit request, the value of the resource reservation interval P_(rsvp,m) is set to 0.]” in Section 8.1.5A of TS 38.214.

    - Consequences if not approved:

      - The specification is not clear regarding UE-B’s behavior on handling the field of resource reservation period in the inter-UE coordination information.

----------------------- Start of text proposal to TS 38.214 v17.1.0 ----------- 8.1.5A UE procedure for determining slots and resource blocks indicated by a preferred or non-preferred resource set The set of slots and resource blocks indicated by a set of preferred or non-preferred resource(s) is determined as described below. The set of preferred or non-preferred resources r0,r1,r2,…, is indicated by a reference slot tref and M tuples (TRIVm,FRIVm,Prsvp,m), 1≤m≤M indicated by the 'resource combination(s)' field, where for each tuple TRIVm is indicated by the 9 MSBs, followed by FRIVm and Prsvp,m (if present). The reference slot tref is indicated by the 'reference slot location' field as a combination of DFN index and slot index [5, TS 38.212], with the 10 MSBs indicating the DFN index. TRIVm and FRIVm are interpreted according to clause 8.1.5, with the following modifications: - - the value of sl-MaxNumPerReserve is fixed to 3. - - "slot where SCI format 1-A was received" is replaced by slot indicated as the first resource location of a TRIVm. - - the first resource location of each TRIVm for m>1 is indicated by a slot offset tm in logical slots with respect to the reference slot tref; the first resource location of TRIV1 is at slot offset 0 with respect to the reference slot.- "the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received" is replaced by "each tuple" - - the starting sub-channel nsubCH,0start of the first resource of each tuple is separately indicated. The starting sub-channel nsubCH,0start of the first resource of each tuple is indicated by the '[lowest subchannel index for the first resource location of each TRIV]' field. The resource reservation period Prsvp,m is encoded as in SCI format 1-A. If the set is indicated by an SCI format 2-C, the number of tuples is M=2. A UE forms the union of the subsets indicated by each tuple (TRIVm,FRIVm,Prsvp,m) to obtain the set r0,r1,r2,…. Prsvp,m ------------------------ End of Text proposal to TS 38.214 v17.1.0 ----------

## 8) RAN1#110 meeting

- Agreement

  - Reason for change:

    - The RAN1 agreement related to UE-B's behaviour of determining a resource used for its resource (re-)selection among the non-preferred resource set received from UE-A is not captured in the specification.

  - Summary of change:

    - The relevant UE-B's behaviour is described in Section 8.1.4C of TS 38.214.

  - Consequences if not approved:

    - The specification text is unclear as to which resource UE-B uses for its resource (re-)selection among the non-preferred resource set received from UE-A

--------------------------- Start of text proposal to TS 38.214 v17.2.0 --------------------------- 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. Note: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission, where Tproc,2 is equal to (Tproc,0+Tproc,1) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2 is equal to Tproc,0 when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission. Note: the case when Tproc,2 is equal to Tproc,0 is assuming that SCI format 2-C is received. --------------------------- End of Text proposal to TS 38.214 v17.2.0 ---------------------------

Agreement

  - Reason for change

    - Regarding the cast type for SCI format 2-C, the following agreement was reached. However, it seems that there is no corresponding text in the specification.

Agreement For inter-UE coordination information transmission, only when the cast type of inter-UE coordination information is unicast regardless of whether or not it is multiplexed with other data, a SCI format 2-C can be used in addition to MAC CE

  - Summary of change

    - Clarify that SCI format 2-C can be used only for unicast.

  - Consequences if not approved

    - The cast type for which SCI format 2-C was agreed is not explicitly reflected in the specification.

--------------------------- Start of text proposal to TS 38.212 v17.2.0 --------------------------- 8.4.1.3 SCI format 2-C SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information. SCI format 2-C can be used only for unicast. The following information is transmitted by means of the SCI format 2-C: <Unchanged parts omitted> --------------------------- End of Text proposal to TS 38.212 v17.2.0 ---------------------------

- Reference

- R1-2205742	Correction for inter-UE coordination Scheme 2 determination of UE-B	FUTUREWEI

- R1-2205766	Remaining issues on maintenance of Rel-17 sidelink enhancements	Huawei, HiSilicon

- R1-2205848	Discussion on maintenance on NR sidelink enhancement	LG Electronics

- R1-2205977	Remaining issues on NR Sidelink enhancement	Spreadtrum Communications

- R1-2206096	Maintenance on NR SL enhancement	ZTE, Sanechips

- R1-2206283	Remaining essential issues in R17 NR sidelink enhancement	OPPO

- R1-2206360	Maintenance on NR sidelink enhancement	CATT, GOHIGH

- R1-2206447	Remaining issues on resource allocation for sidelink enhancement	Lenovo

- R1-2206468	Editorial correction for inter-UE coordination	NEC

- R1-2206562	Discussion on LS Related to RRC Parameters for IUC Scheme 1 and Default CBR Configuration	Intel Corporation

- R1-2206563	Discussion on LS on IUC with Non-Preferred Resource Set	Intel Corporation

- R1-2206763	Maintenance on NR Sidelink enhancement	vivo

- R1-2206804	Maintenance on NR sidelink enhancement	Samsung

- R1-2206890	Maintenance on NR Sidelink enhancement	CMCC

- R1-2206936	Remaining issues on NR sidelink enhancement	Sharp

- R1-2207146	Remaining issues on R17 SL Enhancement	InterDigital, Inc.

- R1-2207206	Remaining issues in Sidelink	Qualcomm Incorporated

- R1-2207313	On Maintenance of NR Sidelink Enhancement	Apple

- R1-2207386	Maintenance of sidelink enhancement	NTT DOCOMO, INC.

- R1-2207483	Maintenance on Inter-UE coordination	ASUSTeK

- R1-2207563	Critical corrections and remaining issues on NR SL enhancement	Ericsson