3GPP TSG-RAN WG1 Meeting #110	R1-22xxxxx

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.6.2

Title:	FL summary #1 on evaluation of coverage impact

Source:	Moderator (MediaTek Inc.)

Document for:	Discussion, Decision

# Coverage impact

  - Introduction to coverage impact

  - Coverage impact evaluation

### Urban scenario at 2.6 GHz

Editor’s Note: the following table is summarized based on eRedCapCoverage-2.6GHz-11PRBs-v023-Panasonic-FL.xlsx on August 18 in the following link:

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_110/Inbox/drafts/9.6(FS_NR_redcap_enh)/Evaluation/Coverage-2.6GHz-11PRBs/eRedCapCoverage-2.6GHz-11PRBs-v023-Panasonic-FL.xlsx

Table 8.2.1-[2] (Part 1): In Urban scenario at 2.6GHz, coverage margins of Rel-18 eRedCap UE with 11-PRB BW without antenna efficiency loss compared to the bottleneck channel for the reference R15 NR UE in Table 8.2.1-1.

| Coverage margin (dB) | PBCH target BLER1% | PBCH target BLER10% | PDCCH CSS (BW1, 11 PRBs; CORESET: 2 symbols, 48 PRBs; AL16) | PDCCH CSS (BW1, 11 PRBs; CORESET: 2 symbols, 24 PRBs; AL8) | PDCCH CSS (BW1, 11 PRBs; CORESET: 3 symbols, 6 PRBs; AL2) | SIB1 (BW1, 11 PRBs; SIB1 BW > 5 MHz; TBS 1256 bits) | SIB1 (BW1, 11 PRBs; SIB1 BW < 5 MHz; TBS 1256 bits) | Msg2 (BW1, 11 PRBs; TBS 72 bits) | Msg2 (BW1, 11 PRBs; TBS scaling factor 0.5) | Msg4 (BW1, 11 PRBs; TBS 1040 bits) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MediaTek | 16.31 | - | 11.21 | 10.61 | 6.51 | 5.61 | 8.41 | 13.01 | 16.61 | 9.21 |
| Spreadtrum | - | 7.23 | 4.73 | 4.03 | 0.53 | -0.57 | 0.83 | 11.23 | - | 4.83 |
| ZTE | - | 10.90 | 0.90 | -4.00 | -2.80 | -1.70 | 4.10 | 10.60 | - | 5.50 |
| Ericsson | 8.53 | - | 6.53 | 6.03 | -0.67 | -2.23 | 1.04 | 6.69 | 10.65 | 2.01 |
| Samsung | - | - | - | - | 7.43 | - | 5.53 | 14.13 | - | 7.33 |
| vivo | -4.29 | 0.83 | 11.29 | 9.30 | 7.27 | 8.83 | 8.44 | - | - | - |
| Nokia | 18.19 | - | 15.19 | 15.19 | 11.19 | 11.89 | 11.39 | 24.59 | - | 10.69 |
| CMCC | 12.62 | - | 3.44 | 3.44 | -0.32 | 5.35 | 7.85 | 18.75 | - | 14.95 |
| NTT DOCOMO | 10.33 | - | 6.02 | 6.02 | 1.50 | -3.64 | 3.04 | 8.96 | - | 4.16 |
| CATT | 10.93 | - | 5.53 | 3.13 | -0.17 | 0.93 | 2.73 | 6.23 | - | 3.93 |
| OPPO | 10.81 | - | 10.91 | 10.31 | 6.91 | 9.01 | 12.21 | 13.53 | - | 13.21 |
| Panasonic | - | - | 0.31 | -0.49 | -6.09 | -0.79 | -1.49 | - | - | - |
| Mavenir | - | - | - | 7.05 | 5.40 | 2.31 | 3.80 | - | - | - |
| Xiaomi | 4.30 | - | - | 4.37 | -1.78 | -3.58 | -1.08 | 5.72 | - | 1.79 |
| Huawei | 13.81 | - | 9.61 | 9.21 | 5.31 | 15.61 | 11.11 | 12.31 | - | 7.81 |
| Lenovo | -2.54 | - | - | - | -6.23 | - | -6.94 | - | - | - |
| Intel | - | 17.43 | 11.13 | 9.93 | 6.43 | 13.18 | 14.13 | 12.53 | - | 7.43 |
| Qualcomm | 16.77 | - | 5.25 | - | 2.55 | 2.15 | 2.45 | 8.95 | - | 3.85 |
| Representative value 1 | 11.04 | [10.90] | 6.53 | 5.77 | 1.64 | 2.83 | 4.46 | 11.04 | [10.65] | 5.76 |
| # of samples 1 | 9 | 3 | 10 | 10 | 13 | 11 | 13 | 12 | 1 | 12 |
| Representative value 2 | 10.19 | 9.07 | 7.21 | 6.38 | 2.38 | 3.60 | 5.02 | 11.41 | [13.63] | 6.67 |
| # of samples 2 | 12 | 4 | 14 | 15 | 18 | 16 | 18 | 14 | 2 | 14 |

Table 8.2.1-[2] (Part 2): In Urban scenario at 2.6GHz, coverage margins of Rel-18 eRedCap UE with 11-PRB BW without antenna efficiency loss compared to the bottleneck channel for the reference R15 NR UE in Table 8.2.1-1.

| Coverage margin (dB) | PDCCH USS (BW1, 11 PRBs; CORESET: 3 symbols, 6 PRBs; AL2; baseline) | PDCCH USS (BW1, 11 PRBs; CORESET: 3 symbols, 12 PRBs; AL4) | PDSCH (BW1, 11 PRBs; 0.5 Mbps) | PRACH B4 (BW1, 11 PRBs) | PRACH C2 (BW1, 11 PRBs) | Msg3 (BW1, 11 PRBs) | PUSCH (BW1, 11 PRBs; 0.25 Mbps) | PUCCH 2bit (BW1, 11 PRBs) | PUCCH 11bit (BW1, 11 PRBs) | PUCCH 22 bits (BW1, 11 PRBs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MediaTek | 8.91 | 15.11 | 18.81 | - | - | - | - | - | - | - |
| Spreadtrum | 4.53 | 9.93 | 12.53 | 7.06 | 5.06 | 7.76 | 4.06 | 11.47 | 12.37 | 9.97 |
| ZTE | 0.33 | 10.43 | 21.03 | - | - | 13.76 | 5.06 | 21.27 | 18.67 | 16.67 |
| Ericsson | 3.33 | 8.13 | 11.98 | 17.96 | - | 14.06 | 5.46 | 13.07 | 18.56 | 16.63 |
| Samsung | 11.53 | 16.73 | 17.33 | - | - | 10.56 | 4.97 | 20.17 | 15.37 | 13.17 |
| vivo | - | - | - | - | - | - | - | - | - | - |
| Nokia | 11.19 | 15.69 | 18.59 | - | - | 11.86 | 3.94 | - | - | - |
| CMCC | 5.68 | 11.27 | 19.35 | 14.74 | 12.77 | 11.70 | 4.44 | 15.60 | 13.75 | 11.84 |
| NTT DOCOMO | 5.50 | - | 14.17 | - | - | 9.37 | 5.35 | 17.53 | 17.58 | - |
| CATT | 3.83 | 8.03 | 12.43 | 10.56 | 10.26 | 8.46 | 4.36 | 15.87 | 14.47 | 12.47 |
| OPPO | - | - | - | - | - | - | - | - | - | - |
| Panasonic | - | - | - | - | - | - | - | - | - | - |
| Mavenir | - | - | - | - | - | - | - | - | - | - |
| Xiaomi | 2.22 | - | 10.90 | - | - | 8.19 | 5.04 | - | - | - |
| Huawei | 8.31 | 13.41 | 15.91 | 6.28 | 6.38 | 11.66 | 3.68 | 21.63 | 19.33 | - |
| Lenovo | - | - | - | - | - | 0.00 | - | - | - | - |
| Intel | 7.69 | 13.09 | 14.19 | - | - | 7.14 | 5.63 | - | - | - |
| Qualcomm | - | - | 14.65 | - | - | 7.04 | 4.51 | - | - | 6.55 |
| Representative value 1 | 5.81 | 11.71 | 15.12 | 10.79 | 8.32 | 10.05 | 4.72 | 17.25 | 16.40 | 12.82 |
| # of samples 1 | 11 | 9 | 12 | 5 | 4 | 12 | 12 | 8 | 8 | 7 |
| Representative value 2 | 6.12 | 12.13 | 15.45 | 10.79 | 8.32 | 9.77 | 4.72 | 17.25 | 16.40 | 12.82 |
| # of samples 2 | 12 | 10 | 13 | 5 | 4 | 13 | 12 | 8 | 8 | 7 |

### Rural scenario at 0.7 GHz

### Urban scenario at 4 GHz