3GPP TSG-RAN WG1 Meeting #110	R1-22xxxxx

Toulouse, France, August 22nd – 26th, 2022

Agenda Item:	9.6.2

Title:	FL summary #1 on evaluation of coverage impact

Source:	Moderator (MediaTek Inc.)

Document for:	Discussion, Decision

# Coverage impact

  - Introduction to coverage impact

  - Coverage impact evaluation

### Urban scenario at 2.6 GHz

Editor’s Note: the following table is summarized based on eRedCapCoverage-2.6GHz-11PRBs-v023-Panasonic-FL.xlsx on August 18 in the following link:

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_110/Inbox/drafts/9.6(FS_NR_redcap_enh)/Evaluation/Coverage-2.6GHz-11PRBs/eRedCapCoverage-2.6GHz-11PRBs-v023-Panasonic-FL.xlsx

Table 8.2.1-[3] (Part 1): In Urban scenario at 2.6GHz, coverage margins of Rel-18 eRedCap UE with 11-PRB BW and with 3dB antenna efficiency loss compared to the bottleneck channel for reference R15 NR UE in Table 8.2.1-1.

| Coverage margin (dB) | PBCH target BLER1% | PBCH target BLER10% | PDCCH CSS (BW1, 11 PRBs; CORESET: 2 symbols, 48 PRBs; AL16) | PDCCH CSS (BW1, 11 PRBs; CORESET: 2 symbols, 24 PRBs; AL8) | PDCCH CSS (BW1, 11 PRBs; CORESET: 3 symbols, 6 PRBs; AL2) | SIB1 (BW1, 11 PRBs; SIB1 BW > 5 MHz; TBS 1256 bits) | SIB1 (BW1, 11 PRBs; SIB1 BW < 5 MHz; TBS 1256 bits) | Msg2 (BW1, 11 PRBs; TBS 72 bits) | Msg2 (BW1, 11 PRBs; TBS scaling factor 0.5) | Msg4 (BW1, 11 PRBs; TBS 1040 bits) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MediaTek | 13.31 | - | 8.21 | 7.61 | 3.51 | 2.61 | 5.41 | 10.01 | 13.61 | 6.21 |
| Spreadtrum | - | 4.23 | 1.73 | 1.03 | -2.47 | -3.57 | -2.17 | 8.23 | - | 1.83 |
| ZTE | - | 7.90 | -2.10 | -7.00 | -5.80 | -4.70 | 1.10 | 7.60 | - | 2.50 |
| Ericsson | 5.53 | - | 3.53 | 3.03 | -3.67 | -5.23 | -1.96 | 3.69 | 7.65 | -0.99 |
| Samsung | - | - | - | - | 4.43 | - | 2.53 | 11.13 | - | 4.33 |
| vivo | -7.29 | -2.17 | 8.29 | 6.30 | 4.27 | 5.83 | 5.44 | - | - | - |
| Nokia | 15.19 | - | 12.19 | 12.19 | 8.19 | 8.89 | 8.39 | 21.59 | - | 7.69 |
| CMCC | 9.62 | - | 0.44 | 0.44 | -3.32 | 2.35 | 4.85 | 15.75 | - | 11.95 |
| NTT DOCOMO | 7.33 | - | 3.02 | 3.02 | -1.50 | -6.64 | 0.04 | 5.96 | - | 1.16 |
| CATT | 7.93 | - | 2.53 | 0.13 | -3.17 | -2.07 | -0.27 | 3.23 | - | 0.93 |
| OPPO | 7.81 | - | 7.91 | 7.31 | 3.91 | 6.01 | 9.21 | 10.53 | - | 10.21 |
| Panasonic | - | - | -2.69 | -3.49 | -9.09 | -3.79 | -4.49 | - | - | - |
| Mavenir | - | - | - | 4.05 | 2.40 | -0.69 | 0.80 | - | - | - |
| Xiaomi | 1.30 | - | - | 1.37 | -4.78 | -6.58 | -4.08 | 2.72 | - | -1.21 |
| Huawei | 10.81 | - | 6.61 | 6.21 | 2.31 | 12.61 | 8.11 | 9.31 | - | 4.81 |
| Lenovo | -5.54 | - | - | - | -9.23 | - | -9.94 | - | - | - |
| Intel | - | 14.43 | 8.13 | 6.93 | 3.43 | 10.18 | 11.13 | 9.53 | - | 4.43 |
| Qualcomm | 13.77 | - | 2.25 | - | -0.45 | -0.85 | -0.55 | 5.95 | - | 0.85 |
| Representative value 1 | 8.04 | [7.90] | 3.53 | 2.77 | -1.36 | -0.17 | 1.46 | 8.04 | [7.65] | 2.76 |
| # of samples 1 | 9 | 3 | 10 | 10 | 13 | 11 | 13 | 12 | 1 | 12 |
| Representative value 2 | 7.19 | 6.07 | 4.21 | 3.38 | -0.62 | 0.60 | 2.02 | 8.41 | [10.63] | 3.67 |
| # of samples 2 | 12 | 4 | 14 | 15 | 18 | 16 | 18 | 14 | 2 | 14 |

Table 8.2.1-[3] (Part 2): In Urban scenario at 2.6GHz, coverage margins of Rel-18 eRedCap UE with 11-PRB BW and with 3dB antenna efficiency loss compared to the bottleneck channel for reference R15 NR UE in Table 8.2.1-1.

| Coverage margin (dB) | PDCCH USS (BW1, 11 PRBs; CORESET: 3 symbols, 6 PRBs; AL2; baseline) | PDCCH USS (BW1, 11 PRBs; CORESET: 3 symbols, 12 PRBs; AL4) | PDSCH (BW1, 11 PRBs; 0.5 Mbps) | PRACH B4 (BW1, 11 PRBs) | PRACH C2 (BW1, 11 PRBs) | Msg3 (BW1, 11 PRBs) | PUSCH (BW1, 11 PRBs; 0.25 Mbps) | PUCCH 2bit (BW1, 11 PRBs) | PUCCH 11bit (BW1, 11 PRBs) | PUCCH 22 bits (BW1, 11 PRBs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MediaTek | 5.91 | 12.11 | 15.81 | - | - | - | - | - | - | - |
| Spreadtrum | 1.53 | 6.93 | 9.53 | 4.06 | 2.06 | 4.76 | 1.06 | 8.47 | 9.37 | 6.97 |
| ZTE | -2.67 | 7.43 | 18.03 | - | - | 10.76 | 2.06 | 18.27 | 15.67 | 13.67 |
| Ericsson | 0.33 | 5.13 | 8.98 | 14.96 | - | 11.06 | 2.46 | 10.07 | 15.56 | 13.63 |
| Samsung | 8.53 | 13.73 | 14.33 | - | - | 7.56 | 1.97 | 17.17 | 12.37 | 10.17 |
| vivo | - | - | - | - | - | - | - | - | - | - |
| Nokia | 8.19 | 12.69 | 15.59 | - | - | 8.86 | 0.94 | - | - | - |
| CMCC | 2.68 | 8.27 | 16.35 | 11.74 | 9.77 | 8.70 | 1.44 | 12.60 | 10.75 | 8.84 |
| NTT DOCOMO | 2.50 | - | 11.17 | - | - | 6.37 | 2.35 | 14.53 | 14.58 | - |
| CATT | 0.83 | 5.03 | 9.43 | 7.56 | 7.26 | 5.46 | 1.36 | 12.87 | 11.47 | 9.47 |
| OPPO | - | - | - | - | - | - | - | - | - | - |
| Panasonic | - | - | - | - | - | - | - | - | - | - |
| Mavenir | - | - | - | - | - | - | - | - | - | - |
| Xiaomi | -0.78 | - | 7.90 | - | - | 5.19 | 2.04 | - | - | - |
| Huawei | 5.31 | 10.41 | 12.91 | 3.28 | 3.38 | 8.66 | 0.68 | 18.63 | 16.33 | - |
| Lenovo | - | - | - | - | - | -3.00 | - | - | - | - |
| Intel | 4.69 | 10.09 | 11.19 | - | - | 4.14 | 2.63 | - | - | - |
| Qualcomm | - | - | 11.65 | - | - | 4.04 | 1.51 | - | - | 3.55 |
| Representative value 1 | 2.81 | 8.71 | 12.12 | 7.79 | 5.32 | 6.77 | 1.72 | 14.25 | 13.40 | 9.82 |
| # of samples 1 | 11 | 9 | 12 | 5 | 4 | 13 | 12 | 8 | 8 | 7 |
| Representative value 2 | 3.12 | 9.13 | 12.45 | 7.79 | 5.32 | 6.77 | 1.72 | 14.25 | 13.40 | 9.82 |
| # of samples 2 | 12 | 10 | 13 | 5 | 4 | 13 | 12 | 8 | 8 | 7 |

### Rural scenario at 0.7 GHz

### Urban scenario at 4 GHz