3GPP TSG RAN WG1 #110           	          R1-2207947

Toulouse, France, August 22nd – 26th, 2022

Source: 	vivo

Title:	FL summary#2 on CRs for intercell-mTRP

Agenda Item:	8.1

Document for:	Discussion and Decision

FL proposals:

Proposal 1: CRs in R1-2207132 and R1-2207134 are agreed as editorial.

Proposal 2: Merge the following draft CRs R1-2207177, R1-2207178 and R1-2205935, and FL to provide draft CR

Proposal 3: agree the CR in R1-2206221

Proposal 4: agreed the CR in R1-2206258

| company | comments |
| --- | --- |
| NTT DOCOMO | For Proposal 1, OK with the text proposal in R1-2207132 and R1-2207134. For Proposal 2, OK with R1-2205935. For R1-2207177 and R1-2207178, if all SSBs indicated by ssb-PositionsInBurst in SSB-MTCAdditionalPCI are considered for invalid symbols for UL transmission, the scheduling flexibility will be largely degraded. On the other hand, we see the issue for SSBs for L1 measurement from PCIs not associated with the active TCI states. How about considering a middle way: to consider SSBs associated with the active TCI states, as well as SSBs configured for L1 beam measurement/reporting? For Proposal 3, OK with the text proposal. For Proposal 4, OK with the text proposal. |
| OPPO | We are fine with proposal 1 and 4. For proposal 2, we intend to agree with DOCOMO. If all the SSBs associated with RRC configured additional PCI are invalid for uplink transmission, the impact to uplink transmission is too significant. For proposal 3, maybe we can simply use “the serving cell physical cell ID” to replace “one physical cell ID”. |
| LG | OK with Proposal 1,3,4. For proposal 2, we support R1-2205935. R1-2207177/2207178 reverts previous agreement in which SSB is limited as “active” SSB. Agreement For inter-cell mTRP , UE does not transmit PUCCH/PUSCH/PRACH in a slot or SRS in the symbols if in time domain the PUCCH/PUSCH/PRACH/SRS overlaps with an SSB of a serving cell PCI or an SSB associated with the active additional PCI. |
| Lenovo | For proposal 3, we are fine with either the text proposal in R1-2206221 or OPPO’s suggestion. Fine with proposal 1 and proposal 4. For proposal 2, fine with R1-2205935. For R1-2207177 and R1-2207178, we share similar with NTT DOCOMO and OPPO. |

Based on comments received, following is considered offline agreement

Offline agreement:

- CRs in R1-2207132 and R1-2207134 are agreed as editorial.

- CR in R1-2206258 is agreed

- CR in R1-2205935 is agreed

- Revised TP corresponding to R1-2206221 as below, in sections 5.1 and 5.1.5 in 38.214

When a set of TCI state IDs are activated for a coresetPoolIndex, the activated TCI states corresponding to one coresetPoolIndex can be associated with the serving cell one physical cell ID and activated TCI states corresponding to another coresetPoolIndex can be associated with another physical cell ID.