3GPP TSG RAN WG1 #110 							    R1-2207985

Toulouse, France, 22 – 26. August 2022

Source:	MediaTek

Title:	[110-NR-7.1-CRs] Summary of comments on R1-2207004 “On PUSCH power initialization during BWP switch”

Agenda item:	7.1

Document for:	Discussion and Decision

Introduction

In RAN1#110 meeting, one contribution [1, MTK] is submitted to clarify the PUSCH power initialization during BWP switch.

As guided by the Chairman, to collect company views, this contribution provides summary of the submitted contributions (Section 4), discussion points (Section 2), and possible RAN1 consensus during this meeting (Section 3, TBD).

Discussion points

Based on the submitted contribution [1, MTK], it is mentioned that

Observation 1: In 38.213 V16.10.0 7.1.1, when UE is doing PUSCH power initialization, some used parameters, including preambleReceivedTargetPower/msg3-DeltaPreamble/msgA-preambleReceivedTargetPower/msgA-DeltaPreamble, are configured under RACH/PUSCH configuration under UL BWP. If UE establishes RACH process in BWP0, and then switches to BWP1, it is not clear the aforementioned parameters under which BWP should be used for PUSCH power initialization in BWP1.

The view from [1] on this issue is:

- The PUSCH power initialization should be based on the RACH process which established the connection; UE applies the parameters UE used to establishes RACH process as a starting point for PUSCH power initialization, irrelevant of which BWP UE switches to.

which means UE should use the parameters mentioned in the example of Observation 1 under BWP0 to do PUSCH power initialization in BWP1.

Hence, [1] has the following text proposal to 38.213 to clarify that “The PUSCH power initialization should be based on the RACH process which established the initial connection”:

------------------------------------------------------------ TP begins-----------------------------------------------------------------------

- 7.1	Physical uplink shared channel

<unchanged parts are omitted>

- 7.1.1 UE behavior

<unchanged parts are omitted>

- PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1.

-	If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in clause 8.3,

j=0, PO_UE_PUSCH,b,f,c0=0, and PO_NOMINAL,PUSCH,f,c0=PO_PRE+∆PREAMBLE,Msg3,

where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] corresponding to the Type-1 random access procedure and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble corresponding to the Type-1 random access procedure, or ∆PREAMBLE,Msg3=0 dB if msg3-DeltaPreamble is not provided, for carrier f of serving cell c

-	If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A,

j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH,

where PO_PRE is provided by msgA-preambleReceivedTargetPower corresponding to the Type-2 random access procedure, or by preambleReceivedTargetPower corresponding to the Type-2 random access procedure if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble corresponding to the Type-2 random access procedure, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c

<unchanged parts are omitted>

------------------------------------------------------------ TP ends------------------------------------------------------------------------

Discussion point 1:

Some parameters used to initialize PUSCH power in UE side, including preambleReceivedTargetPower/msg3-DeltaPreamble/msgA-preambleReceivedTargetPower/msgA-DeltaPreamble, are configured under RACH/PUSCH configuration under UL BWP.

If UE establishes RACH process in BWP0, and then switches to BWP1, the aforementioned parameters under which BWP (BWP0 or BWP1), do you think should be used for PUSCH power initialization in BWP1?

| Company | BWP0 or BWP1 | Comment |
| --- | --- | --- |
| MTK | BWP0 | The PUSCH power initialization should be based on the RACH process which established the initial connection; UE applies the parameters UE used to establishes RACH process as a starting point for PUSCH power initialization, irrelevant of which BWP UE switches to. |
| Samsung | BWP0 |  |
| ZTE | BWP0 |  |
| CATT | BWP0 |  |
| Apple | BWP0 | This is on condition that UE ‘is not provided P0-PUSCH-AlphaSet’ on BWP#1, as explicitly written in spec. Otherwise, UE should use the RRC-configured P0-PUSCH-AlphaSet for BWP#1. |
| Intel | BWP0 |  |
| OPPO | BWP0 |  |
| vivo | BWP0 |  |
| Huawei |  | If there is no information provided in another BWP (e.g. #1), the UE naturally use the configuration of BWP0. Otherwise, it might need to clarify whether the UE can use the configuration of BWP#1,e.g from Scell. On the other hand, for RACH procedure from SCell, the parameter is associated with the carrier f of a serving c, so it does not seem to have ambiguity. For RACH procedure from a same serving cell, except for RedCap where two shared BWP is possible, we are not sure if there is such a case that may cause ambiguity |

Discussion point 2:

If your answer to Discussion point 1 is BWP0, can you accept the TP to 38.213 from [1]? If your answer is no, please further elaborate why you think current spec is clear enough or you have a different TP to address this issue.

| Company | Yes or No | Comment |
| --- | --- | --- |
| MTK | Yes | We see the need to clarify the spec. |
| Samsung | No | Current spec is clear. We think the yellow highlighted sentence, like “If a UE established dedicated RRC connection using a Type-1 random access procedure” already implies the proposed point in this CR. Hence, no need editorial clarification. |
| ZTE | No | We share the same views with Samsung that the current description seems sufficient. If required, we can live with sa conclusion for concluding this issue. |
| CATT | No | Current spec is clear. |
| Apple |  | We also think current spec is clear. It would be helpful if MTK can clarify where the ambiguity comes from. Note that ‘…provided by preambleReceivedTargetPower [11, TS 38.321]’, this parameter is referring to TS 38.321 and not TS 38.331, which implies that it is the one that is used by UE for RACH procedure. We are also open for a conclusion if majority companies prefer. |
| Intel | No | Current spec is clear. |
| MTK |  | Thanks for companies’ informative inputs. If seems companies do not see a spec change required except for MTK. In this case, we try to propose the following conclusion, just to give a clarification for this issue with companies’ inputs combined: Conclusion: According to 38.213 7.1.1: - “If a UE established dedicated RRC connection using a Type-1 random access procedure , as described in clause 8, and is not provided P0-PUSCH-AlphaSet…, PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and …” If UE is not provided P0-PUSCH-AlphaSet, the PUSCH power initialization should be based on the RACH process (either Type 1 or Type 2) which established the connection and irrelevant of which BWP UE switches to. |
| OPPO | No | We have the same understanding as Samsung. |
| vivo | No | During random access procedure, we believe that the occurrence of BWP switching is impossible. After that, when the UE receives a BWP switching indication, it means that RRC connection establishment has been complete and BWP information has been configured. It seems that the case of RRC only configures BWP information but not configure PUSCH power control parameters is a corner case. If majority of companies consider this case is valid, we are open to make a conclusion. |
| Huawei, HiSilicon |  | We are open to clarify this if any issue is found. |

Discussion point 3:

If your answer to Discussion point 1 is BWP1, please elaborate on why you think it should be BWP1 and whether a spec change to 38.213 or RAN1 conclusion is needed to make it clear.

| Company | Comment |
| --- | --- |

Resulted RAN1 conclusion/agreement (phase 2)

No conclusion/agreement achieved for this issue.

From Discussion point 1, all companies agree that

- If UE establishes RACH process in BWP0, and then switches to BWP1:

  - If UE is not provided P0-PUSCH-AlphaSet in BWP1, the PUSCH power initialization parameters UE used for BWP1 should be based on BWP0

During online discussion, the following Conclusion is proposed by MediaTek but NOT agreed, due to the reason that companies deem it is not needed to repeat what spec already says with a new conclusion.

Conclusion: According to 38.213 7.1.1:

- “If a UE established dedicated RRC connection using a Type-1 random access procedure , as described in clause 8, and is not provided P0-PUSCH-AlphaSet…, PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and …”

If UE is not provided P0-PUSCH-AlphaSet, the PUSCH power initialization should be based on the RACH process (either Type 1 or Type 2) which established the connection and irrelevant of which BWP UE switches to.

Summary of contribution inputs

Summary for [1, MTK]:

In [1], it is mentioned that, in 38.213 V16.10.0 7.1.1, when UE is doing PUSCH power initialization, some used parameters, including preambleReceivedTargetPower/msg3-DeltaPreamble/msgA-preambleReceivedTargetPower/msgA-DeltaPreamble, are configured under RACH/PUSCH configuration under UL BWP, as shown below in highlighted sentences:

- 7.1	Physical uplink shared channel

<unchanged parts are omitted>

- 7.1.1 UE behavior

<unchanged parts are omitted>

- PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1.

-	If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in clause 8.3,

j=0, PO_UE_PUSCH,b,f,c0=0, and PO_NOMINAL,PUSCH,f,c0=PO_PRE+∆PREAMBLE,Msg3,

where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble, or ∆PREAMBLE,Msg3=0 dB if msg3-DeltaPreamble is not provided, for carrier f of serving cell c

-	If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A,

j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH,

where PO_PRE is provided by msgA-preambleReceivedTargetPower, or by preambleReceivedTargetPower if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c

<unchanged parts are omitted>

;

The RRC configuration hierarchy of preambleReceivedTargetPower/msg3-DeltaPreamble/msgA-preambleReceivedTargetPower/msgA-DeltaPreamble under UL BWP from 38.331 are shown below for ease of reference:

- BWP-UplinkCommonRACH-ConfigCommonRACH-ConfigGenericpreambleReceivedTargetPower

- BWP-UplinkCommonPUSCH-ConfigCommonmsg3-DeltaPreamble

- BWP-UplinkCommonMsgA-ConfigCommon-r16RACH-ConfigCommonTwoStepRA-r16RACH-ConfigGenericTwoStepRA-r16msgA-preambleReceivedTargetPower-r16

- BWP-UplinkCommonMsgA-ConfigCommon-r16MsgA-PUSCH-Config-r16msgA-DeltaPreamble-r16

If UE establishes RACH process in BWP0, and then switches to BWP1, it is not clear the aforementioned parameters under which BWP should be used for PUSCH power initialization in BWP1.

Observation 1: In 38.213 V16.10.0 7.1.1, when UE is doing PUSCH power initialization, some used parameters, including preambleReceivedTargetPower/msg3-DeltaPreamble/msgA-preambleReceivedTargetPower/msgA-DeltaPreamble, are configured under RACH/PUSCH configuration under UL BWP. If UE establishes RACH process in BWP0, and then switches to BWP1, it is not clear the aforementioned parameters under which BWP should be used for PUSCH power initialization in BWP1.

Intuitively, the PUSCH power initialization should be based on the RACH process; UE applies the parameters UE used to establishes RACH process as a starting point for PUSCH power initialization, irrelevant of which BWP UE switches to.

Observation 2: The PUSCH power initialization should be based on the RACH process; UE applies the parameters UE used to establishes RACH process as a starting point for PUSCH power initialization, irrelevant of which BWP UE switches to.

And [1] has the following proposal:

Proposal 1: Adopt the CR below in 38.213 V16.10.0 to clarify the used parameters for PUSCH power initialization

- 7.1	Physical uplink shared channel

<unchanged parts are omitted>

- 7.1.1 UE behavior

<unchanged parts are omitted>

- PO_PUSCH,b,f,c(j) is a parameter composed of the sum of a component PO_NOMINAL,PUSCH,f,c(j) and a component PO_UE_PUSCH,b,f,c(j) where j∈0,1,…,J-1.

-	If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in clause 8.3,

j=0, PO_UE_PUSCH,b,f,c0=0, and PO_NOMINAL,PUSCH,f,c0=PO_PRE+∆PREAMBLE,Msg3,

where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] corresponding to the Type-1 random access procedure and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble corresponding to the Type-1 random access procedure, or ∆PREAMBLE,Msg3=0 dB if msg3-DeltaPreamble is not provided, for carrier f of serving cell c

-	If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A,

j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH,

where PO_PRE is provided by msgA-preambleReceivedTargetPower corresponding to the Type-2 random access procedure, or by preambleReceivedTargetPower corresponding to the Type-2 random access procedure if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble corresponding to the Type-2 random access procedure, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c

<unchanged parts are omitted>

- References

[1] R1-2207004	On PUSCH power initialization during BWP switch, MediaTek, RAN1 #110