# 3GPP TSG RAN WG1 #110			                                                                    R1-2208028

# Toulouse, France, August 22nd – 26th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR |  | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on determination of TBS for NPDSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ASUSTeK |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2022-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Release 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the phase of introducing Rel-14 feature in v.14.2.0 in RAN1#88, some changes on Rel-13 feature were accidentally conducted by changing “carries” to “carriers”. Note that an initial version of CR provided by the rapporteur under the thread ” [88-04-213] NB-IoT 213 CR” does not include such change, while it was reflected in the final version even though comments provided over emails do not touch the concerned section. Currently Rel-13 spec still reads ”carries” while the releases including Rel-14 and beyond read “carriers” due to above mentioned accident. The condition for determining NPDSCH TBS becomes unclear since the term ”carriers” is ambiguous, e.g. whether it’s referring NB-IoT carrier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change “carriers” to “carries”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The term “carriers” under” condition for determining NPDSCH TBS is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.4.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis It’s RAN1’s common understanding that the changes in this CR comply with previous releases |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

16.4.1.5	Modulation order and transport block size determination

To determine the modulation order in the NPDSCH, the UE shall

-	if the UE is configured with higher layer parameter npdsch-16QAM-Config and the DCI is mapped onto the UE specific search space given by C-RNTI, or the UE is configured with higher layer parameter pur-DL-16QAM-Config and the DCI is mapped onto the UE specific search space given by PUR-RNTI,

-	If the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’,

-	use modulation order, = 4

-	otherwise

-	use modulation order, = 2

-	otherwise

-	use modulation order, = 2.

To determine the transport block size in the NPDSCH, the UE shall first,

-	if NPDSCH carries SystemInformationBlockType1-NB

-	set  to the value of the parameter schedulingInfoSIB1 configured by higher-layers

-	else if NPDSCH with 16QAM

-	read the 4-bit "modulation and coding scheme for 16QAM" (IMCS') in the DCI

-	If for the carrier on which NPSS/NSSS/NPBCH are detected the value of the higher layer parameter operationModeInfo is set to '00' or '01', or if the value of the higher layer parameter inbandCarrierInfo-r13 is configured for a higher layer configured carrier if any, set ITBS=IMCS'+11, otherwise set ITBS=IMCS'+14

-	otherwise

-	read the 4-bit "modulation and coding scheme" field () in the DCI and set .

and second,

-	if NPDSCH carries SystemInformationBlockType1-NB

-	use Clause 16.4.1.5.2 for determining its transport block size.

-	otherwise,

-	read the 3-bit "resource assignment" field () in the DCI and determine its TBS by the procedure in Clause 16.4.1.5.1.

For a NPDCCH UE-specific search space, if the UE is configured with higher layer parameter twoHARQ-ProcessesConfig, or the UE is configured with higher layer parameter npdsch-MultiTB-Config and single TB is scheduled in the corresponding DCI

-	the NDI and HARQ process ID as signalled on NPDCCH, and the TBS, as determined above, shall be delivered to higher layers,

otherwise

-	the NDI as signalled on NPDCCH, and the TBS, as determined above, shall be delivered to higher layers. If the UE is configured with higher layer parameter npdsch-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the HARQ process ID of 0 is for the first TB and HARQ process ID of 1 shall be assumed for the second TB, otherwise, HARQ process ID of 0 shall be assumed.