3GPP TSG RAN WG1 meeting #110bis-e		                                   R1-2209198

E-meeting, Oct. 10th – 19th, 2022

Title: 	XR specific capacity enhancements

Source: 	ZTE, Sanechips

Agenda item:	9.10.2

Document for:	Discussion and decision

Introduction

In RAN1#110 meeting, the study scope of candidate capacity enhancement techniques for XR traffic based on SPS/CG/DG has been agreed as follows [1]:

Agreement RAN1 to make decision on the following in RAN1#110bis-e - Support single DCI scheduling multi-PDSCHs/PUSCHs which is currently supported for FR2-2 to other SCS in FR1/FR2 Agreement When DG is used as the baseline scheme, for the performance evaluation scheduling, after SR is triggered, both BSR and UL data can be transmitted using the UL grant after SR. - Companies are encouraged to provide the size of resources by the first UL grant after SR Agreement Whether/how to enhance BSR to improve capacity performance of XR traffic is within RAN2 scope and is not handled by RAN1. - Note that companies should indicate if and what BSR enhancement is assumed in their RAN1 proposals on CG and DG enhancements. - RAN1 can evaluate BSR enhancement to improve capacity performance ...

In this paper, we present the evaluation results of following candidate capacity enhancement techniques: multiple CG PUSCHs with unused resource release and adaptive parameter adjustment, one DCI scheduling multiple PDSCHs, and BSR enhancement, wherein updated dynamic grant baseline are added, and more realistic assumptions are considered for one DCI scheduling multiple PDSCHs.

Discussion on uplink CG enhancement

CG transmission for XR traffic

CG mechanism in NR is adopted to support URLLC services. The key reasons are that CG transmission is capable of reducing overhead of PDCCH, more importantly, extra latency caused by both SR transmission and BSR procedure.

Then for XR services which require low latency (e.g. 10ms,15ms,30ms), the CG-based solutions were considered and evaluated to meet the requirement. On the other hand, when conserved dynamic grant with SR-BSR-PUSCH procedure was adopted, a lot of packets transmission might exceed PDB and therefore got dropped, which would deteriorate the performance seriously.

In the case of Pose/Control traffic model transmission, since the packets arrival is periodic and the packet size is constant, CG mechanism can be applied directly without any enhancements.

The Pose/Control packets arrival is periodic and the packet size is constant, thus CG is necessary for Pose/Control traffic transmissions.

For AR video traffic model, i.e., AR aggregating scene/video/data/audio, which is characterized by large and various packet size. In this case, one solution is to use uplink dynamic grant for flexible resource scheduling. However, more time would be needed for transmitting the large packet for a UE. Thus DG would encounter performance penalty, due to the procedure of SR-BSR-PUSCH transmission, especially in the scenario of more stringent latency requirement. As to CG, although it’s beneficial from undoing the procedure of SR-BSR-PUSCH, the problem is CG PUSCH resource may not be able to transmit the whole packet. To this end, multiple CG PUSCHs is considered as the solution to transmit the XR packet. For example, multiple transmission occasions in a period are pre-configured, when a packet arrives, UE can transmit the packet in these occasions directly.

For AR video traffic model, uplink dynamic grant would face performance penalty especially in the scenario of stringent latency requirement.

Potential XR-specific CG enhancement

Towards the issue in [2] for multiple CG PUSCHs in a period, i.e., low resource efficiency, in our opinion, unused resource release can improve resource usage efficiency. In this case, the redundant CG PUSCHs can be recycled.

Figure 1 Unused resource release

For example, as shown in Figure 1, the 1st CG PUSCH and the 2nd CG PUSCH are used for packet 1 transmission, while the 3rd CG PUSCH and the 4th CG PUSCH are redundant resources. In this situation, a release indication would be sent to inform that the 3rd and 4th CG PUSCHs are released, which can be used for other XR service transmission.

Moreover, the UCI or the MAC CE can be considered to carry the release indication in the first or last CG PUSCH in a period. The UCI or MAC CE can be transmitted via PUCCH resource or multiplexed/transmitted on PUSCH, which just follows the current specification. And the change is on what information for release indication is conveyed in UCI or MAC CE.

For multiple CG PUSCHs, release indication for unused CG PUSCHs can be carried by L1/L2 signaling to address the problem of low resource efficiency and improve capacity performance, which has minor specification impact compared to legacy CG mechanism.

In addition, dynamic adjustment of CG parameters is considered as another enhancement. The adaptive parameters such as MCS level, number of layers, are beneficial for UE transmitting data. As depicted in Figure 2, when the indication of updating parameters is received by UE, the parameters of following CG PUSCHs, such as MCS level, the number of layers, etc, would be updated.

Figure 2 Parameter update for CG PUSCH

For multiple CG PUSCHs, dynamic parameter adjustment can be considered for capacity performance enhancement.

Regarding the issue of inter-UE resource collision for XR services with multi-users, it is more likely to assign each CG PUSCHs burst with a large time and frequency domain resources. That means small number of UEs can be supported due to the limited uplink resource in unpaired spectrum.

To this end, non-orthogonal/overlapped CG configurations for different UEs in one cell are considered. In a meanwhile a resource indication is applied to handle the inter-UE resource collisions.

Figure 3 Procedure of uplink resource indication

As Figure 3 illustrated, firstly, multiple resources are configured for multiple UEs at the same time. Then, assistance information about occupied PUSCH occasions can be reported by UEs. When the information is received by gNB, gNB can coordinate resources for these UEs’ transmissions via a resource indication.

For multiple CG PUSCHs, dynamic resource indication can be considered for capacity performance enhancement.

As to periodicity misalignment, both semi-static indication and dynamic indication can adjust the location of transmission occasions in timeline as shown in Figure 4. In this way, the accumulated delay can be reduced to an acceptable range.

Figure 4 Periodicity alignment illustration

For multiple CG PUSCHs, both semi-static indication and dynamic indication can be used to align the periodicity of XR packet arrival and periodicity of CG PUSCH.

Evaluation results for CG enhancement

Evaluation assumptions and evaluation results

In this part, we present the evaluation results for multiple CG PUSCHs with adaptive parameter adjustment. And we assume that the periodicities of packet arrival and CG PUSCH have been aligned in our simulations. Moreover, the simulation assumptions are shown as below:

- Traffic model: use the uplink AR video traffic model in TR 38.838 [3]

Table 1 Traffic model for uplink simulation

| Traffic Model No. | Bit rate (Mbps) | Periodicity (FPS) | Mean Packet size (Byte) | PDB (ms) |
| --- | --- | --- | --- | --- |
| Traffic Model 1 | 20 | 60 | 41667 | 30 |
| Traffic Model 2 | 20 | 60 | 41667 | 15 |
| Traffic Model 3 | 20 | 60 | 41667 | 10 |
| Traffic Model 4 | 10 | 60 | 20833 | 15 |
| Traffic Model 5 | 10 | 60 | 20833 | 10 |

- Simulation cases: Following case 1~ case 3 include the legacy CG, dynamic grant, and enhanced CG.

  - Case 1: legacy CG

Figure 5 Legacy CG

In Figure 5, orthogonal CG configurations are configured among different UEs in one cell. Moreover, fixed MCS level and the number of layers are adopted.

  - Case 2: uplink dynamic grant

- Agreed UL dynamic grant baseline (Case 2-1)

- Another UL dynamic grant baseline (Case 2-2)

Figure 6 Dynamic grant with Tx delay (Note that DCI-x schedules PUSCH-x, while DCI-B schedules PUSCH only for BSR)

In last meeting, we agreed when DG is used as the baseline scheme, both BSR and UL data can be transmitted using the UL grant after SR, the procedure is depicted in Figure 6(a). In our opinion, current specification supports this kind of mechanism by padding BSR, in this case, the redundant resource of the UL grant can be used for BSR reporting.

While in for XR use case, it seems challenging for gNB to predict the size of resources by the first UL grant. For simplification, we assume accurate size of resource for the first UL grant can be obtained with the ability of XR-awareness, while the allocated resource in the UL grant for a UE depends on the PF scheduler.

  - Case 3: CG enhancement

Figure 7 multiple CG PUSCHs configuration with adaptive parameter adjustment

As shown in Figure 7, the assumptions are applied for multiple CG PUSCHs:

- Overlapped resources can be configured among different multiple UEs

- Unused resource is recycled.

- Adaptive MCS level and the number of layers are applied.

Other evaluation assumptions have been listed in Appendix A.

Then the simulation results with different traffic models are shown below:

| (a) Traffic model 1: 20Mbps@60fps, 30ms PDB | (b) Traffic model 2: 20Mbps@60fps, 15ms PDB | (c) Traffic model 3: 20Mbps@60fps, 10ms PDB |
| --- | --- | --- |

Figure 8 Capacity comparison among Case 1, Case 2 and Case 3 for 20Mbps@60fps traffic model with different PDB in Dense Urban scenario

| - Traffic model 4: 10Mbps@60fps, 15ms | (b) Traffic model 5: 10Mbps@60fps, 10ms |
| --- | --- |

Figure 9 Capacity comparison among Case 1, Case 2 and Case 3 for 10Mbps@60fps traffic model with different PDB in Dense Urban scenario

It can be observed in Figure 8 that with traffic model of 20Mbps@60fps, the system capacity of CG enhancement is [3.9], [3.5] and [2.6] UEs for 30ms PDB, 15ms PDB and 10ms PDB, respectively. The system capacity of uplink DG with the assumption of both BSR and PUSCH are transmitted after SR is triggered is [3.6], [2.1] and [0] UEs for 30ms PDB, 15ms PDB and 10ms PDB, respectively. While the system capacity of uplink DG with the assumption of only BSR is transmitted after SR is triggered is [3.5], [0] and [0] UEs for 30ms PDB, 15ms PDB and 10ms PDB, respectively.

It can be observed in Figure 9 that with traffic model of 10Mbps@60fps, the system capacity of CG enhancement is [8.2] and [5.4] UEs for 15ms PDB and 10ms PDB, respectively. The system capacity of uplink DG with the assumption of both BSR and PUSCH are transmitted after SR is triggered is [7.0] and [0] UEs for 15ms PDB and 10ms PDB, respectively. While the system capacity of uplink DG with the assumption of only BSR is transmitted after SR is triggered is [4.0] and [0] UEs for 15ms PDB and 10ms PDB, respectively.

For easier comparing these cases the above simulation results are summarized in Figure 10.

| - 20Mbps@60fps | (b) 10Mbps@60fps |
| --- | --- |

Figure 10 Capacity summary of Figure 8 and Figure 9

It can be observed in Figure 10 that CG enhancement is beneficial to increase capacity gain compared to UL DG and legacy CG. Moreover, the capacity gain increases when PDB requirement becomes stringent with the same data rate. And the capacity also increases when data rate increases with the same PDB limitation.

CG enhancement is capable of bringing significant capacity gain compared to UL dynamic grant and legacy configured grant. Moreover, capacity gain increases when the required PDB decreases or the data rate increases.

Analysis for capacity evaluation results

In this section, we analyze the reasons why the considerable capacity performance gain of Multiple CG PUSCHs can be obtained, compared to DG or legacy CG.

Resource occupation

From resource occupation’s perspective, DG transmission scheme is capable of adjusting time and frequency domain resource, MCS level and so on. Hence, time and frequency domain resources of DG transmission are flexible for each transport block. While for multiple CG PUSCHs configuration, a few parameters such as MCS level and the number of layers are adjusted; most parameters are pre-configured. And the adjusted MCS level and the number of layers only affect the number of slots in the multiple CG PUSCHs configurations.

In case of Traffic model 1, i.e. 20Mbps@60fps, 30ms, the time and frequency domain resource occupations for Case 1~Case 3 are shown as follows.

| - Slot occupation | (b) Resource block occupation |
| --- | --- |

Figure 11 Resource occupation for Case 1(legacy CG), Case 2(DG) and Case 3(Multiple CG PUSCHs).

As shown in Figure 11(a), Case 2, i.e., including both Case 2-1 and Case 2-2, occupies average [3.6] slots in time domain, while Case 3 occupies average [4.9] slots in time domain. Case 3 occupies less resources than case 1 in time domain, and the difference between Case 2 and Case 3 is around 1 slot.

As shown in Figure 11(b), both Case 1 and Case 3 occupy the whole bandwidth, i.e., [272] RBs in frequency domain and even for Case 2, more than half of the bandwidth are used. It means due to large packet size service transmitting in the uplink transmission, most frequency domain resource would be used. Thus the scheduling flexibility of DG transmission would be rather limited.

For multiple CG PUSCHs, the enhancements e.g., unused resource recycle and dynamic parameters adjustment are beneficial to achieve similar slots occupation with DG schemes from time domain resource occupation perspective.

For large packet size service, e.g. 20Mbps@60fps, in the uplink transmission, most frequency domain resource would be occupied for both CG and DG solutions.

Transmission delay

From transmission delay’s perspective, Case 2-1, case 2-2 considers the different transmission delay to simulate the procedure of SR-BSR-PUSCH transmission.

| (a) Traffic model 1: 20Mbps@60fps, 30ms | (b) Traffic model 2: 20Mbps@60fps, 15ms | (c) Traffic model 3: 20Mbps@60fps, 10ms |
| --- | --- | --- |

Figure 12 Transmission delay comparison for Case 1&case 3, Case 2-1 and Case 2-2 in Traffic model 1~3 in Dense Urban scenario

According to results in Figure 12, when transmitting packets of 20Mbps@60fps traffic model in Dense Urban scenario, Case 1, Case 3, has average [1.5] ms transmission delay considering UE is capable of transmitting data in the closest CG PUSCH. But Case 2 has average [3.0]~[6.0] ms transmission delay.

For transmitting packets of 20Mbps@60fps traffic model in Dense Urban scenario, CG based solution has average [1.5] ms transmission delay considering UE is capable of transmitting data in the closest CG PUSCH. But DG based solution has average [3.0]~[6.0] ms transmission delay.

Therefore, based on above evaluation as well as the analysis, it can be concluded that multiple CG PUSCHs in a period reduces the transmission delay compared to DG transmission. And unused resource release indication and adaptive parameters adjustment for multiple CG PUSCHs bring capacity performance gain.

Support to take multiple CG PUSCHs in a period as a candidate solution for XR service.

For multiple CG PUSCHs, study and specify solutions of recycling unused CG PUSCH resources to improve resource usage efficiency.

For multiple CG PUSCHs, study and specify solutions of dynamically adjusting CG parameters, including e.g., MCS level and the number of layers, to improve resource usage efficiency.

Discussion on downlink SPS enhancement

SPS transmission is characterized by fixed MCS level, periodic and fixed time-frequency resource, which fits to the periodical service with small packet size and stringent latency, including e.g. URLLC services. Compared to dynamic grant transmission, SPS transmission is capable of reducing overhead of downlink control signaling, as well as transmission latency which is required for PDCCH transmission/decoding in DG mechanism.

XR service is characterized by quasi-periodicity, large and various packet size as well as stringent latency. In order to reduce transmission latency and save overhead of downlink control signaling, SPS transmission may be one of potential transmission schemes for XR service due to the mentioned advantages.

Given the advantages of reducing overhead of downlink control signaling and transmission latency, SPS mechanism can be considered for XR traffic.

According to the conclusion in RAN1#110 meeting [1], SPS enhancement has been deprioritized. However, since SPS has benefit in reducing overhead of downlink control signaling, transmission delay and power consumption, SPS enhancement is valuable for further study.

Suggest to capture SPS enhancement techniques as well as their evaluation results proposed by companies into TR38.835.

Potential XR-specific SPS enhancement

Towards to the all the issues of transmitting XR service by SPS transmission mentioned above, the solution is similar to potential CG enhancement:

- Parameters, such as MCS level and the number of layers, are adjusted for each packet transmission to increase capacity performance by resource efficiency enhancement.

- Non-integer periodicity alignment is used to eliminate the accumulation of delay so that the delay is controlled within the acceptable range.

- Unused resource release is used to release the redundant resources and UE is capable of skipping the redundant resources instead of blindly detecting each resource.

Evaluation results and analysis for SPS enhancement

In this section, we present the evaluation results for multiple SPS PDSCHs configuration with adaptive parameter adjustment. The simulation assumptions are shown as below:

- Traffic model: Downlink AR/VR video which is captured in TR 38.838 [3]

Table 2 Traffic model for downlink simulation

| Traffic Model No. | Bit rate (Mbps) | Periodicity (FPS) | Mean Packet size (Byte) | PDB (ms) |
| --- | --- | --- | --- | --- |
| Traffic Model 6 | 45 | 60 | 93750 | 10 |
| Traffic Model 7 | 60 | 60 | 125000 | 10 |

- Simulation cases: the simulation cases include the legacy SPS configuration and SPS enhancement.

  - Case 1: legacy SPS configuration

  - Case 2: SPS enhancement

Other evaluation assumptions have been listed in Appendix A. Base on the assumptions mentioned above, we present evaluation results of capacity performance in Indoor HotSpot scenario as follows.

| (a) Downlink Traffic model 6: 45Mbps@60fps, 10ms | (b) Downlink Traffic model 7: 60Mbps@60fps, 10ms |
| --- | --- |

Figure 13 Capacity performance comparison for Case 1 and Case 2 for Traffic Model 6 and Traffic Model 7 in Indoor HotSpot scenario

It can be observed in Figure 13, when the traffic model is 45Mbps@60fps, 10ms and 60Mbps@60fps, 10ms in Indoor Hotspot scenario, [0.0] UE can be both supported by legacy SPS transmission, while [4.2] and [2.4] UEs can be supported by SPS enhancement, respectively.

When considering the traffic model of 45Mbps@60fps, 10ms or 60Mbps@60fps, 10ms in Indoor Hotspot, SPS enhancement techniques, including e.g., unused resource release and dynamic parameter adjustment are capable of supporting considerable amount of UEs compared with legacy SPS transmission scheme.

Moreover, for these two traffic models, the evaluation results of power consumption are presented as follows. And the power model is based on Appendix B.

Table 3 Power consumption comparison for Case 1 and Case 2 for Traffic Model 6 and Traffic Model 7 in Indoor HotSpot scenario

| Traffic model No. | Case | Capacity | Power consumption |
| --- | --- | --- | --- |
| Traffic Model 6: 45Mbps@60fps, 10ms | Case 1: Legacy SPS | 0 | 195.43 |
|  | Case 2: MSPS | 4 | 52.88 |
| Traffic Model 7: 60Mbps@60fps, 10ms | Case 1: Legacy SPS | 0 | 195.37 |
|  | Case 2: MSPS | 2.4 | 61.06 |

It can be observed in Table 3 that for Traffic Model 6 (45Mbps@60fps, 10ms) in Indoor Hotspot scenario, unused resource release for multiple SPS PDSCHs in a period reduces the power consumption from [195.43] to [52.88], compared to legacy SPS transmission scheme. While for Traffic Model 7 (60Mbps@60fps, 10ms) in Indoor HotSpot scenario, unused resource release for multiple SPS PDSCHs in a period reduces the power consumption from [195.37] to [61.06], compared to legacy SPS transmission scheme.

Figure 14 Power consumption reduction

As illustrated in Figure 14, assuming that there are 5 SPS PDSCHs pre-configured in a period, but 2 SPS PDSCHs resource are occupied to carry data. All the 5 SPS PDSCHs are blindly detected according to legacy SPS transmission mechanism. While multiple SPS PDSCHs configuration with unused resource release, only 2 SPS PDSCHs are detected and the redundant SPS PDSCHs can be skipped with the cost of additional PDCCH monitoring.

SPS enhancement techniques, including e.g., unused resource release, are capable of reducing power consumption by reducing the blind detection of PDSCH compared with legacy SPS transmission scheme.

Discussion on dynamic scheduling/grant enhancement

Single DCI scheduling multiple PxSCH

Views on extending capability of single DCI scheduling multi-PDSCHs for FR2-2 to FR1/FR2

In RAN1#109-e meeting, it was agreed to study whether to extend the capability of single DCI scheduling multi-PxSCHs to XR use cases. Since the DL stream is pseudo-periodic and with large scale of packet size (e.g., several slots would be scheduled for a packet transmission), single DCI scheduling multiple TBs might be considered to improve network capacity by saving PDCCH overhead. In our contribution [2], single DCI scheduling multiple TBs is capable of improving capacity performance in the assumption that the PDCCH overheads are saved in all the PDSCHs except the first PDSCH, which is elaborated as follows:

- 2 symbols are used for PDCCH, if the slot is the first slot of single TB scheduling or multiple TBs scheduling.

- 0 symbols are used for PDCCH, if the slot is not the first slot of multiple TB scheduling for one UE.

In the assumption that the PDCCH resource are saved in all PDSCHs except the first PDSCH, single DCI scheduling multi-PDSCHs provides marginal capacity performance gain, i.e around 3% performance gain [2].

However, it is too idealistic to assume that all the PDSCHs can save PDCCH overheads for single DCI scheduling multiple TBs, because the slot of PDSCH-only for one UE might include PDCCH monitoring occasion(s) for another UE to schedule its own PDSCH(s). In this regard, it is hard to say the PDCCH overheads can be saved in the PDSCH except the first PDSCH, especially in the case of multiple UEs with random arrival and medium/high system load.

With above consideration of PDCCH overhead saving, we present evaluation results of single DCI scheduling multiple TBs in 30Mbps@60fps traffic model in Indoor Hotspot, where the PDCCH overheads are considered as follows:

- 2 symbols are used for PDCCH, if the slot is the first slot of single TB scheduling or multiple TBs scheduling.

- 2 symbols are used for PDCCH, if the slot is not the first slot of multiple TB scheduling for one UE but is the first slot of multiple TBs scheduling or the slot of single TB scheduling for another UE.

- 0 symbol is used for PDCCH, if the slot is not the first slot of multiple TB scheduling for one UE and is not the first slot of multiple TBs scheduling or the slot of single TB scheduling for any other UEs.

Then, the evaluation results are shown in Figure 15, and the detailed simulation assumptions have been listed in Appendix A.

| - Scenario: Urban Macro | (b) Scenario: Indoor Hotspot |
| --- | --- |

Figure 15 Capacity performance comparison between single DCI scheduling multiple PDSCHs and single DCI scheduling single PDSCH in 30Mbps@60fps, 10ms in Urban Macro/ Indoor HotSpot scenario

It can be observed in Figure 15 that for 30Mbps@60fps, 10ms traffic model, single DCI scheduling multiple PDSCHs can support [7.6] UEs while single DCI scheduling single PDSCHs can support [8.8] UEs in Urban Macro scenario. Moreover, single DCI scheduling multiple PDSCHs can support [7.9] UEs while single DCI scheduling single PDSCHs can support [9.1] UEs in Indoor HotSpot scenario.

According to the evaluation results in the two scenarios, the capacity performance of single DCI scheduling multiple PDSCHs is inferior to that of single DCI scheduling single PDSCH. The reason is that a majority of slots include PDCCH overheads (i.e. slots are usually in the case of (1) and (2), few slots are in the case of (3)) when we consider a more realistic PDCCH overheads. Without a significant PDCCH overhead saving, the capacity performance penalty of single DCI scheduling multiple PDSCHs occurs owing to fixed MCS and FDRA assignment, which leads to incompatible with current channel condition.

Compared to one DCI scheduling one PDSCH, capacity performance of one DCI scheduling multiple PDSCHs degrades, due to limited PDCCH overhead saving as well as inflexible resource assignment, i.e., fixed MCS and FDRA assignment across multiple PDSCHs.

In the assumption that the PDCCH resource are seldom saved in all PDSCHs due to random UE data arrival, single DCI scheduling multi-PDSCHs is not beneficial to the improvement of system capacity and even worse than that of single DCI scheduling single PDSCH.

Besides, for single DCI scheduling multi-PDSCHs, although both initial transmission and re-transmission are supported in current specification by indicating NDI for each slot, there are several issues on re-transmission in single DCI scheduling multi-PDSCHs. Due to fixed MCS and FDRA assignment for all PDSCHs scheduled by one DCI, the same MCS level and PRBs are used for both initial transmission and re-transmission. In such case, the reliability of re-transmitted TBs would degrade if the MCS level is aggressive to serve initial TBs better, or resource efficiency of initial TBs become low if the MCS level is conserve to ensure re-transmitted TBs. While from the perspective of PRB assignment, same PRB assignment for initial TBs and re-transmitted TBs also has an impact on the reliability of re-transmitted TBs, implying the re-transmitted TBs have a risk of encountering frequency-selective fading.

Considering mixed initial transmission and re-transmission in single DCI scheduling multi-PDSCH is supported in current specification, it has a negative impact on capacity performance due to inflexible resource assignment, i.e., fixed MCS and FDRA assignment for each PDSCH.

De-prioritize the study of enhancements related to extending capability of single DCI scheduling multi-PDSCHs for FR2-2 to FR1/FR2 for XR, since it has no benefit to improve capacity performance.

Views on single DCI scheduling multiple PDSCHs enhancements

If single DCI scheduling multiple PDSCHs is supported in RAN1, the valuable study points are as follows.

In current specification, the HARQ-ACK codebook to schedule PDSCHs is generated and transmitted in one slot. It means the HARQ feedback is not transmitted to the gNB until the last scheduled downlink transmission, the re-transmission schedule will be delayed. As a result, such latency may cause degradation of capacity performance.

If HARQ-ACK can be reported in multiple slots according to corresponding groups of the scheduled downlink transmissions. Accordingly, delay of HARQ-ACK for a group of PDSCHs can be reduced. In other words, to improve the capacity of DL transmission, partial DL transmission can be re-transmitted earlier if it is not detected correctly by the UE.

It is beneficial that HARQ-ACK feedback for multiple groups of the scheduled downlink transmissions can be reported in multiple slots.

Moreover, further study for the configurations per PDSCH/PUSCH is needed. According to current specification, per PDSCH/PUSCH indication including SLIV, RV, NDI, can be supported. However, given the channel state of consecutive slots at which conveys PxSCHs, may be similar because of the low mobility of XR devices, the flexibility of bit field as SLIV in DCI format 0-1/ DCI format 1-1 per PDSCH/PUSCH would be not necessary. Meanwhile, since the multiple PxSCH scheduling is more likely to be occurred in initial transmission, instead of re-transmission, it is redundant to convey a fine granularity indication of RV and NDI.

On the other hand, given UEs’ frequency selective fading channel, e.g., for indoor scenarios, if multiple frequency resources were allocated to a UE according to the channel states, it would be beneficial to improve the capacity. Besides, in case of a slot with single TB scheduling where remaining number of RBs is small, then single DCI scheduling multiple PDSCHs can’t be available in this slot because it’s inefficient to schedule a series of small TBs. However, with finer frequency granularity indication, it possible to allocate different RBs numbers in each slot, if using this slot for single DCI scheduling multiple PDSCHs as well, the PDCCH overhead in the following slot can be saved.

The flexibility of some bit fields such as SLIV, RV, etc in DCI format 0-1/ DCI format 1-1 for per PDSCH/PUSCH may be not necessary. However, finer granularity indication of frequency resources deserves further consideration.

Expect single DCI scheduling multiple PDSCHs, TB over multiple slots can be also considered. TBoMS for PUSCH has been studied and specified in Coverage work item in Release 17. The technique can be also extended to PDSCH to realize the purpose of one DCI scheduling a large packet size. Compared with single DCI scheduling multiple PDSCHs, the advantage is that the HARQ process number will not be too large.

However, since only one HARQ process is used for one packet, gNB may need to reschedule the entire packet if the TB does not been decoded correctly. Rescheduling the entire packet may waste the resource, decrease the capacity and increase UE power consumption. CBG-based PDSCH transmission mechanism is capable of reducing the re-transmission resource and further improve the capacity by only re-transmitting the code block groups with erroneous code blocks, which increases the resource efficiency. Therefore, CBG technique can be considered together with the TBoMS to reduce the packet that needs to be re-transmitted. One simple method of CBG division is treat the data in one slot as a CBG, as shown in Figure 16.

Figure 16 CBG based TBoMS

To this end, CBG-based PDSCH transmission in multiple slots can be considered, if single DCI scheduling multiple PDSCHs is supported in RAN1.

Compared with single DCI scheduling multiple PDSCH for FR2-2, TBoMS functionality can be introduced and extended to PDSCH which can reduce the HARQ process number.

Precise buffer status reporting for UL transmission

To further illustrate the size gap and impact on capacity, we study the relationship between the distribution of UL AR traffic model and the value of buffer size level table in [4], [5].

The XR capacity results of Dense Urban scenarios are provided in Figure 17. The traffic models are AR aggregating scene/video/data/audio traffic model: 10Mbps@60fps with 30ms PDB and 20Mbps@60fps with 30ms. The parameters for simulation are listed in Table A. The distribution curves are shown as follows.

| (a)10Mbps@60fps traffic model distribution | (b)20Mbps@60fps traffic model distribution |
| --- | --- |

Figure 17 Distribution of UL AR video traffic model and BS level table.

In Figure 17, the blue line is the PDF of UL AR traffic model, while the black dots represent the value in buffer size level table in [5]. According to Figure 17, for both 10Mbps@60fps traffic model and 20Mbps@60fps traffic model, more than a half proportion of packet sizes are overestimated, where 56% of packets (i.e. packet size larger than 20516 Byte) are overestimated to 28581 Byte based on 10Mbps@60fps traffic model, while 66% of packets (i.e. packet size larger than 39818 Byte) are overestimated to 55474 Byte based on 20Mbps@60fps traffic model.

For both 10Mbps@60fps traffic model and 20Mbps@60fps traffic model, more than a half proportion of packet sizes are overestimated if existing BSR tables are used.

The capacity performance between precise BSR and legacy BSR is depicted as follows:

Figure 18 Capacity comparison between precise BSR and current legacy BSR.

In Figure 18, for 10Mbps@60fps, capacity performances are increased from [9.5] with legacy BSR indication to [10.9] @90% satisfied UE percentage with precise BSR indication by about [14.47%], while for 20Mbps@60fps, capacity performances are increased from [3.4] with legacy BSR indication to [5.1] @90% satisfied UE percentage with precise BSR indication by about [50.00%] using MU-MIMO in UL FR1 dense urban scenario.

Based on Figure 17 and Figure 18, it can be observed that the size gap results in resource waste and capacity performance degradation. To this end, the BSR should be enhanced.

In uplink transmission, overestimated packet sizes may cause capacity performance loss. Therefore, uplink transmission with precise BSR indication can bring capacity performance gain.

According to the agreement in [1], RAN1 is able to evaluate the BSR enhancement for improving capacity performance. Although the granularity enhancement for BSR is more likely RAN2’s work, we believe the evaluation results for proving the capacity gain brought by this kind of enhancement should be captured in TR 38.835. Then, we have the following proposal:

Suggest to capture the evaluation results of BSR enhancement provided by companies in RAN1 into TR 38.835.

Intra/inter UE prioritization/multiplexing enhancements

Inter-UE multiplexing enhancement can be used in the case of coexistence between XR traffic and eMBB or URLLC traffic. While intra-UE multiplexing enhancement can be considered in the case of multi-stream transmission. And some evaluation results are provided in our contribution [6].

According to the conclusion in RAN1#110 meeting [1], intra/inter UE multiplexing enhancements have been deprioritized. However, since intra/inter UE multiplexing enhancements are capable of bringing significant capacity gain in the scenario of multi-stream transmission and mixed traffic transmission, intra/inter UE multiplexing enhancements are valuable for further study. Then, we have the following proposal:

Suggest to capture the evaluation results of adaptive inter-UE/intra-UE multiplexing enhancement provided by companies into TR38.835.

Conclusion

In this contribution, we have the following observations/proposals:

Observation 1: The Pose/Control packets arrival is periodic and the packet size is constant, thus CG is necessary for Pose/Control traffic transmissions.

Observation 2: For AR video traffic model, uplink dynamic grant would face performance penalty especially in the scenario of stringent latency requirement.

Observation 3: For multiple CG PUSCHs, release indication for unused CG PUSCHs can be carried by L1/L2 signaling to address the problem of low resource efficiency and improve capacity performance, which has minor specification impact compared to legacy CG mechanism.

Observation 4: For multiple CG PUSCHs, dynamic parameter adjustment can be considered for capacity performance enhancement.

Observation 5: For multiple CG PUSCHs, dynamic resource indication can be considered for capacity performance enhancement.

Observation 6: For multiple CG PUSCHs, both semi-static indication and dynamic indication can be used to align the periodicity of XR packet arrival and periodicity of CG PUSCH.

Observation 7: CG enhancement is capable of bringing significant capacity gain compared to UL dynamic grant and legacy configured grant. Moreover, capacity gain increases when the required PDB decreases or the data rate increases.

Observation 8: For multiple CG PUSCHs, the enhancements e.g., unused resource recycle and dynamic parameters adjustment are beneficial to achieve similar slots occupation with DG schemes from time domain resource occupation perspective.

Observation 9: For large packet size service, e.g. 20Mbps@60fps, in the uplink transmission, most frequency domain resource would be occupied for both CG and DG solutions.

Observation 10: For transmitting packets of 20Mbps@60fps traffic model in Dense Urban scenario, CG based solution has average [1.5] ms transmission delay considering UE is capable of transmitting data in the closest CG PUSCH. But DG based solution has average [3.0]~[6.0] ms transmission delay.

Observation 11: Given the advantages of reducing overhead of downlink control signaling and transmission latency, SPS mechanism can be considered for XR traffic.

Observation 12: When considering the traffic model of 45Mbps@60fps, 10ms or 60Mbps@60fps, 10ms in Indoor Hotspot, SPS enhancement techniques, including e.g., unused resource release and dynamic parameter adjustment are capable of supporting considerable amount of UEs compared with legacy SPS transmission scheme.

Observation 13: SPS enhancement techniques, including e.g., unused resource release, are capable of reducing power consumption by reducing the blind detection of PDSCH compared with legacy SPS transmission scheme.

Observation 14: In the assumption that the PDCCH resource are saved in all PDSCHs except the first PDSCH, single DCI scheduling multi-PDSCHs provides marginal capacity performance gain, i.e around 3% performance gain [2].

Observation 15: Compared to one DCI scheduling one PDSCH, capacity performance of one DCI scheduling multiple PDSCHs degrades, due to limited PDCCH overhead saving as well as inflexible resource assignment, i.e., fixed MCS and FDRA assignment across multiple PDSCHs.

Observation 16: In the assumption that the PDCCH resource are seldom saved in all PDSCHs due to random UE data arrival, single DCI scheduling multi-PDSCHs is not beneficial to the improvement of system capacity and even worse than that of single DCI scheduling single PDSCH.

Observation 17: Considering mixed initial transmission and re-transmission in single DCI scheduling multi-PDSCH is supported in current specification, it has a negative impact on capacity performance due to inflexible resource assignment, i.e., fixed MCS and FDRA assignment for each PDSCH.

Observation 18: It is beneficial that HARQ-ACK feedback for multiple groups of the scheduled downlink transmissions can be reported in multiple slots.

Observation 19: The flexibility of some bit fields such as SLIV, RV, etc in DCI format 0-1/ DCI format 1-1 for per PDSCH/PUSCH may be not necessary. However, finer granularity indication of frequency resources deserves further consideration.

Observation 20: Compared with single DCI scheduling multiple PDSCH for FR2-2, TBoMS functionality can be introduced and extended to PDSCH which can reduce the HARQ process number.

Observation 21: For both 10Mbps@60fps traffic model and 20Mbps@60fps traffic model, more than a half proportion of packet sizes are overestimated if existing BSR tables are used.

Observation 22: In uplink transmission, overestimated packet sizes may cause capacity performance loss. Therefore, uplink transmission with precise BSR indication can bring capacity performance gain.

Proposal 1: Support to take multiple CG PUSCHs in a period as a candidate solution for XR service.

Proposal 2: For multiple CG PUSCHs, study and specify solutions of recycling unused CG PUSCH resources to improve resource usage efficiency.

Proposal 3: For multiple CG PUSCHs, study and specify solutions of dynamically adjusting CG parameters, including e.g., MCS level and the number of layers, to improve resource usage efficiency.

Proposal 4: Suggest to capture SPS enhancement techniques as well as their evaluation results proposed by companies into TR38.835.

Proposal 5: De-prioritize the study of enhancements related to extending capability of single DCI scheduling multi-PDSCHs for FR2-2 to FR1/FR2 for XR, since it has no benefit to improve capacity performance.

Proposal 6: Suggest to capture the evaluation results of BSR enhancement provided by companies in RAN1 into TR 38.835.

Proposal 7: Suggest to capture the evaluation results of adaptive inter-UE/intra-UE multiplexing enhancement provided by companies into TR38.835.

References

“RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #110 Meeting, Toulouse, France, May 9th–20th, 2022.

R1-2207062, “XR-specific capacity enhancements techniques”, ZTE, Sanechips, 3GPP TSG RAN WG1 meeting #110, Toulouse, France, Aug. 22th – 26th, 2022.

3GPP TR 38.838, Study on XR (Extended Reality) Evaluations for NR.

R1-2111351, “Performance Evaluation Results for XR”, ZTE, Sanechips, 3GPP TSG RAN WG1 #107-e, e-Meeting, e-Meeting, November 11th-19th, 2021.

3GPP TS38.321 V17.0.0, “Medium Access Control (MAC) protocol specification (Release 17)”.

R1-2203607, “Discussion on XR specific capacity enhancements techniques”, ZTE, Sanechips, 3GPP TSG RAN WG1 #109-e, e-Meeting, May 9th–20th, 2022

- Appendix

- A Simulation Parameters

Table A.1 simulation parameters for FR1 DL

| Parameters | Value |  |  |
| --- | --- | --- | --- |
| Scenario | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors | Scenario-3: Urban Macro hexagonal layout with 7, 3 Sectors |
| Inter-BS distance | 20m | 200m | 500m |
| Carrier frequency | 4 GHz |  |  |
| Duplex Mode / Simulation bandwidth | 100 MHz |  |  |
| SCS | 30KHz |  |  |
| TDD pattern | DDDSU |  |  |
| BS Antenna Configuration | 32 Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ); 32TX:(M, N, P, Mg, Ng; Mp, Np) = (4,4, 2, 1, 1; 4, 4); The antenna tilt is 90 degrees. | 64 Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ); 64TX:(M, N, P, Mg, Ng; Mp, Np) = (8,8, 2, 1, 1; 4, 8); The antenna tilt is 12 degrees. |  |
| UE Antenna Configuration | 4 Rx antenna ports, and Panel model 1: dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); | 4 Rx antenna ports, and Panel model 1: dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); |  |
| Transmit Power | 24 dBm per 20M at TRP, and 23 dBm at UE | 44 dBm per 20M at TRP, and 23 dBm at UE |  |
| BS Height | 3 m | 25 m |  |
| UE Height | 1.5m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |  |
| Antenna Element Gain | 5dBi for BS and 0 dBi for UE | 8dBi for BS and 0 dBi for UE |  |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |  |  |
| UE distribution | 100% of users are indoor Use 3km/h for modeling fading channel | 80% of users are indoor,20%of users are outdoor; Use 3km/h for modeling fading channel |  |
| Number of UEs per cell | up to 15 |  |  |
| Scheduling Algorithm | SU-MIMO+ PF |  |  |
| HARQ/repetition | HARQ retransmission |  |  |
| Channel estimation | Realist |  |  |
| Target BLER | 10% for first transmission |  |  |
| PHY processing delay | UE PUSCH processing Capability #1,N1=10 |  |  |
| PDCCH overhead | 1/7(2symbols per 14symbol) |  |  |
| DMRS overhead | 1 symbol per 14symbol |  |  |
| BS receiver | MMSE-IRC |  |  |

Table A.2 Simulation assumption for FR1 UL

| Parameter | value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban |
| Carrier frequency | 4GHz |  |
| Bandwidth | 100 MHz |  |
| Subcarrier spacing | 30 KHz |  |
| Frame structure | DDDSU (S: 10D:2G:2U) for UL interaction/pose information delivering |  |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32R: (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ | For 64R: (8,8, 2, 1, 1; 4, 8) (dH,dV) = (0.5λ, 0.5λ); |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 2T: (1,1,2,1,1;1,1) (dH, dV)=( 0.5, N/A)λ |  |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi | Ceiling-mount pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi | Omnidirectional, 0 dBi |
| UE max Power | 23 dBm |  |
| Power control | P0 = -80 dBm, alpha = 0.8 |  |
| Noise Figure | 5dB for BS and 9dB for UE |  |
| Scheduler | MU-MIMO+PF / SU-MIMO CG |  |
| Device deployment | 100% indoor | 80% of users are indoor,20%of users are outdoor; Use 3km/h for modeling fading channel |
| Down-tilt | 90 degrees | 12 degrees |
| BS receiver | MMSE-IRC |  |
| UE receiver | MMSE-IRC |  |
| Channel estimation | Realistic |  |
| Target BLER | 10% for first transmission |  |
| UE speed | 3 km/h |  |
| HARQ/repetition | HARQ retransmission,Maximum number of retransmissions is 3 |  |
| PHY processing delay | UE PUSCH processing Capability #1, N2=12 |  |
| DMRE overhead | 4RE |  |

- B Power model

Table B.1 Power model for DL

| Power state | Relative Power (1 slot) |
| --- | --- |
| PDCCH-only | 100 |
| PDCCH+PDSCH | 300 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |

Table B.2 Power model for UL

| Power state | Relative Power (1 slot) |  |  |
| --- | --- | --- | --- |
|  | 0dBm | 23dBm | Other Tx power |
| UL(long PUCCH or PUSCH) | 250 | 700 | Obtained by linear function or step function |
| PDCCH+PUSCH | Sum(PDCCH-only, PUSCH) |  |  |
| PDCCH+PDSCH+PUSCH | Sum(PDCCH+PDSCH, PUSCH) |  |  |

Note: Power model of “PDCCH+PUSCH” is the sum of “PDCCH-only” power consumption and “PUSCH” power consumption. Power model of “PDCCH+PDSCH+PUSCH” is the sum of “PDCCH+PDSCH” power consumption and “PUSCH” power consumption.