3GPP TSG RAN WG1 #112		                                         R1-2300983

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	9.1.1.2

Source: 	CMCC

Title: 	Discussion on two TAs for multi-DCI

Document for:	Discussion and Decision

Introduction

In the RAN1#111 meeting, two TA for multiple DCI was discussed on the TA acquisition, TA updates, reference timing and others [1]. The detailed agreements are quoted in the related sections.

In this contribution, we discussed the related issues about two TAs for UL multi-DCI for multi-TRP operation.

Discussion

Requirements to UE implementations

In the RAN1#109e meeting, it was agreed to support the enhancement on two TAs for UL multiple DCI for multiple TRP operation in Rel-18.

Agreement Enhancement on two TAs for UL multi-DCI for multi-TRP operation is supported in Rel-18. Note 1: whether (1) the network signals two TACs or (2) the network signals one TAC and the UE deriving the second TA can be further studied. Note 2: evaluations can be considered on as-needed basis.

Currently the UE could only support one TA at least for one carrier. But the UE could also support multiple TAs across multiple carriers. Unlike in the CA scenario, the carriers that have different TAs may have different RF chains. Those RF chains could work separately. But in the current two TAs under multiple TRP scenario, two TRP work under the same frequency. The UE should support two TAs in the same carrier using a same set of RF and IF unit. It is not clear whether this will put additional requirements for the realization of UEs. If the TA adjustment is related to the clock of UE, the retuning of the clock may require an additional process time between the switching between the TAs. If two TAs do not require to retune the clock, whether the UE can maintain two TAs and how to maintain the two TAs should be clarified.

Proposal 1:

It should be clarified that if the two TAs put additional requirements on the UE realization, such as clock retuning, process time of switching between the two TAs, whether there are additional requirements for UE to maintain two TAs.

Initial TA acquisition

For the initial acquisition of the TA of 2nd TRP, two mechanisms could be considered. One is based on UE’s measurements without PRACH transmission. The other is based on the RACH transmission. For the 1st mechanism, UEs with two TRPs could receive the SSBs or other DL signals from the two TRP and then derive downlink timing difference between the two TRPs. The timing difference could be used to estimate the TA of the 2nd TRP. Although this estimated TA may not be accurate as the one indicated by 2nd TRP, the 2nd TRP could update the TA in the following procedures.

Observation 1:

UEs could measure the DL timing difference between two TRPs which could be used to generate a estimated TA for the uplink transmission to the 2nd TRP. And the 2nd TRP could update the TA after receiving first transmission from this UE.

In RAN1#111 meeting, only PDCCH ordered CFRA was supported for the TA acquisition for the 2nd TRP in RAN1.

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support CFRA triggered by PDCCH order for both intra-cell and inter-cell cases. Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, there is no consensus to support enhancements for CBRA triggered by PDCCH order. Conclusion For multi-DCI based Multi-TRP operation with two TA enhancement, it is up to RAN2 to decide whether there is a need to enhance CBRA procedure to support per TRP UE-initiated RACH procedure.

Although the UE has the information that the propagation delay to the two TRPs are different and a 2nd TA is required, the serving TRP does not have this information. It should be discussed that when and in which condition, the PDCCH ordered RACH for the 2nd TA should be triggered.

Proposal 2:

It should be discussed that when and in which condition, the PDCCH ordered RACH for the 2nd TA should be triggered.

Once a RACH is sent to the 2nd TRP, an RAR with an TA indication should be feedback to the UE. Two alternatives were discussed for the inter-cell multiple TRP scenarios and a down selection should done at this meeting.

Agreement For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support one of the alternatives (down selection to be done in RAN1#111): - Alt 1: PDCCH scheduling RAR will always be received from serving cell there is no need for additional type 1 CSS configuration per additional PCI - Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported additional type 1 CSS configuration per additional PCI needs to be supported

The 1st alternative is that the RAR is always received from the serving cell and no additional type 1 CSS configuration per additional PCI is needed. The 2nd alternative is that the additional type 1 CSS configuration per additional PCI should be supported. The benefits of the 1st alternative is that there is less configurations to be transferred to the 1st TRP or the serving cell. But the issue is that BH between two TRPs could be non-ideal, which is the typical using scenario of multiple DCI based multiple TRP operation. And the transmission of the TA values from the other cells could be delayed and cannot fulfil the requirements. In the 2nd alternative, the TRP can directly send the TA values to the UE via the RAR. There is no latency issue for the 2nd alternative. In addition, the UE could receive the scheduling and DL data from the 2nd TRP, since the DL synchronization and transmission is not a problem in current scenario. During the offline discussion, majority companies think the additional type 1 CSS configuration per additional PCI would increase the UE complexity and overhead of PDCCH monitoring. But actually, only one Type 1 CSS is needed to be monitored, which is related to the PDCCH ordered RACH. The additional configuration would not increase the complexity of the UE. And only one additional PCI would be activated, then UE can only monitor one specific Type 1 CSS associated with additional PCI.

Proposal 3:

The Alternative 2 which has less latency issues is preferred.

- Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported  additional type 1 CSS configuration per additional PCI needs to be supported

Agreement For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support a mechanism to determine which PRACH configuration (i.e., RACH configuration corresponding to serving cell PCI or an additional PCI) to be used in the RACH procedure triggered by PDCCH order - FFS: Explicit indication or implicit indication through PDCCH order

Two cases could be considered for the RACH procedure triggered by PDCCH order. One is that the PDCCH order is from the serving cell or serving TRP. The PDCCH ordering the RACH procedure associated with additional PCI should contain the PCI information in the DCI. If the PDCCH only indicate the SSB index without any PCI information, it means the RACH procedure is for the serving cell. The other case is that, once the PDCCH for ordering the RACH procedure is from the non-serving cell, then UE could identify it through the Coresetpool index.

Proposal 4:

Both PDCCH with additional field indicating the PCI information or through the Coresetpool index could be used to determine which PRACH configuration to be used.

Multiple mechanism are identified for the TAG ID indication for the intra-cell mDCI scenario. a down selection should be done at this meeting.

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111): - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 4: divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 5: divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP - Alt 6: TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order - Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order Note: If Alt 1 or Alt 2 is downselected, then it does not preclude indication of two TAG IDs (if supported)

As previously discussed, an RAR could be sent directly from the 2nd TRP, the TAG ID could be directly indicated as part of the TA command in RAR. But detailed mechanism to indicate TAG id within RAR and with limited specification impact should be discussed. Alternative 6 is also workable, since CORESETPoolindex would be used for the PDCCH order.

Proposal 5:

Both Alt 1 and Alt 6 could be considered to indicate the TAG ID.

The association between TAG and UL transmissions

In the 110bis-e meeting, the association between TAGs and UL transmissions was discussed and the options are updated as below.

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, the four options agreed in RAN1#110bis-e are refined as below (down-selection of one or a combination of the options to be performed in RAN1#111): - Option 1: Associate TAG to TCI-state/spatial relation - Configure TAG ID as part of UL/joint TCI state or spatial relation - for UL transmission, the TAG ID associated with the UL/joint TCI state or spatial relation is utilized - Option 2: Associate TAG to CORESETPoolIndex - for dynamically scheduled/activated PUSCH, TAG associated with the CORESET pool index of the CORESET carrying the scheduling/activating PDCCH is utilized for UL transmission - for Type 1 CG, P/SP-SRS, and P/SP-PUCCH, coresetPoolIndex is RRC-configured. - FFS: Other signals/channels: AP-SRS, and dynamic HARQ-ACK - Option 3: Associate TAG to SSB group (if such an association is agreed in agenda 9.1.1.2). For a UL transmission, UE adopts the TAG associated with the SSB group such that - if the PL RS is an SSB, then the UE adopts the TAG associated with the SSB group which the PL RS of the UL transmission belongs to - if the PL RS is a CSI-RS, then the UE adopts the TAG associated with the SSB group which the QCL source SSB of the PL RS belongs to - Option 4: TAG association performed as follows: - for dynamically scheduled/activated channels/signals, TAG associated with the CORESET pool index of the CORESET carrying the scheduling PDCCH is utilized for UL transmission - for P/SP UL channels / signals (not scheduled or activated by DCI), TAG ID is RRC-configured.

Not only the dynamic scheduled uplink transmission should be associated with the 2nd TA and TAG but also the semi-static configured UL transmissions. For the dynamic scheduled uplink transmission, it should be associated with CORESETPoolIndex which is used for the m-DCI transmissions in MTRPs. UEs could determine the TA or UL transmission timing according to the PDCCH scheduling. And for the other semi-static configured transmissions, they should also be associated with the TAGs and use the right TAs. The CORESETPoolindex is only used for the multiple DCI scenario in the MTRP for differentiating which TRP is the DCI from. Extending the usage of CORESETPoolindex to the semi-static configured transmission seems not necessary. But for the semi-static configured transmission, a TAG ID indicating a different TA should be used is sufficient. This could also be used for the mobility scenario. The option 4 is slightly preferred.

Proposal 6:

For the dynamic scheduled UL transmission, the TAG should be associated with the CORESETPoolIndex. And for the semi-static configured UL transmissions, the TAG should be associated with the targeted channels and signals. The option 4 is slightly preferred.

Reference timing

In Rel-17 mTRP discussion, it was considered that the uplink transmission could work properly with one single TA under two TRPs. But if the timing between the two TRPs are not aligned and the difference between the propagation delay of two TRPs cannot be ignored or covered by CP, then two TAs are needed. Considering above reasons, the DL timings of two TRPs should also different. As illustrated in the figure below, due to different propagation delays, TRP1 and TRP2 have their own reference timings and TA values.

Figure 1 Reference timing and TAs in mTRP scenario

Two reference timing was agreed in the last meeting.

Agreement For multi-DCI multi-TRP operation with two TAs in a CC, two DL reference timings are supported where each DL reference timing is associated with one TAG - baseline assumption is that the Rx timing difference between the two DL reference timings is no larger than CP length - as an optional UE capability, Rx timing difference between the two DL reference timings can be assumed to be larger than CP length - FFS: the maximum Rx timing difference (could be up to RAN4) - Other than UE capability details and relevant configuration, no additional RAN1 specification enhancement specific for this case is expected

In the DL transmission, the UE should follow the DL timing for receiving and measurements. And the uplink transmission should also follow the same timing and calculate the timing advance. Theoretically the TA value should equal to two folds of the propagation delay. In single TRP scenario, the TA is based on the single TRP’s reference timing. And if the propagation delay of the 2nd TRP is different, as discussed above, the reference timing and the 2nd TA from the 2nd TRP should be also different. If the UE has the capability to maintain two reference timings, it could facilitate to reuse the legacy defined TA, such as the 2nd TA in Figure 1, which should be indicated by the TRP2. But in the M-DCI mTRP scenario, the scheduling of one UE from two TRPs are independent. The scheduling timeline could cross between the two TRPs. As in Figure 2, the UE would receive the 2nd DCI from TRP#2, in which the reference timing should follows the DL of TRP#2. Then the UE should transmit the PUSCH#1 according to the scheduling from TRP1. If the UE cannot switch the reference timing from TRP#2 to TRP#1 in time, the UE cannot reuse the traditional TA as illustrated in Figure 1.

Figure 2 The crossed scheduling timeline between two TRPs

Proposal 7:

It should discuss the impact of the two reference timings to the uplink transmission.

Collision and overlapping issues

In Rel-17, the multiple DCI under mTRP was assumed with non-ideal backhaul connections. And the collisions between two PUSCHs scheduled by two DCIs are not allowed. It means that although the coordination between the TRPs is slow due to non-ideal backhaul, it still needs coordination between the two TRPs because UEs are not expected to be scheduled with the overlapped time domain resources. This put additional requirements or the more limitations to the gNBs’ scheduling.

When only one TA is used for the two TRPs transmission, gNBs should try to schedule the UE with different time domain resources. But if two TAs are considered, the overlapping due to different TAs should be taken into account. Currently in the specification, when two slots are overlapped due to different TAs in the CA scenarios, the first slot will be transmitted complete. UE will drop the overlapped part of the 2nd slot. Only the non-overlapped part will be transmitted.

In the last meeting, the handling of the overlapped part between two UL transmissions associated with two TAs was discussed. If the BH link between two TRPs is ideal, the transmission overlapping could be avoided through the scheduling. But if the BH is non-ideal, then the TRPs may not know the overlapped region between the two transmissions. A conclusion is drawn as below.

Conclusion (#110bis) For multi-DCI based Multi-TRP operation with two TA enhancement, it cannot always be assumed that both TRPs have knowledge of the overlapping region between transmissions corresponding to the two TAs. - Note: This doesn’t prevent the network from applying scheduling restrictions even if the TRPs have no knowledge of the overlapping region Agreement (#110) For multi-DCI based multi-TRP operation with two TAs, study how to handle overlapping part between two UL transmissions associated with two TAs, where the study includes: - whether to introduce scheduling restriction in overlapping part - whether to introduce dropping rules - whether specification impact is needed, or if the issue can be handled via implementation - whether to allow overlapped transmission in case the UE supports STxMP transmission (if STxMP feature is agreed in NR Rel-18)

Since it cannot always be assumed that both TRPs have knowledge of the overlapping region between two transmissions, then it cannot be avoided through the scheduling. Then a dropping rule should be introduced, which is helpful for UE implementation. But the overlapping issue may not only happen to PUSCH+PUSCH. Other overlapping issues such as SRS+PUSCH, PUCCH+PUSCH should be further discussed.

Proposal 8:

A dropping rule should be introduced to deal with the overlapping issue.

Proposal 9:

The overlapping issue of PUSCH with other UL transmission should be discussed.

Conclusions

In this contribution, we discussed the related issues about two TAs for UL multi-DCI for multi-TRP operation. The observation and proposals are listed as below.

Observation 1:

UEs could measure the DL timing difference between two TRPs which could be used to generate a estimated TA for the uplink transmission to the 2nd TRP. And the 2nd TRP could update the TA after receiving first transmission from this UE.

Proposal 1:

It should be clarified that if the two TAs put additional requirements on the UE realization, such as clock retuning, process time of switching between the two TAs, whether there are additional requirements for UE to maintain two TAs.

Proposal 2:

It should be discussed that when and in which condition, the PDCCH ordered RACH for the 2nd TA should be triggered.

Proposal 3:

The Alternative 2 which has less latency issues is preferred.

- Alt 2: In addition to PDCCH scheduling RAR being received from serving cell, reception of PDCCH scheduling RAR from a TRP corresponding to an additional PCI for a RACH procedure associated to the additional PCI is supported  additional type 1 CSS configuration per additional PCI needs to be supported

Proposal 4:

Both PDCCH with additional field indicating the PCI information or through the Coresetpool index could be used to determine which PRACH configuration to be used.

Proposal 5:

Both Alt 1 and Alt 6 could be considered to indicate the TAG ID.

Proposal 6:

For the dynamic scheduled UL transmission, the TAG should be associated with the CORESETPoolIndex. And for the semi-static configured UL transmissions, the TAG should be associated with the targeted channels and signals. The option 4 is slightly preferred.

Proposal 7:

It should discuss the impact of the two reference timings to the uplink transmission.

Proposal 8:

A dropping rule should be introduced to deal with the overlapping issue.

Proposal 9:

The overlapping issue of PUSCH with other UL transmission should be discussed.

References

[1] Chair’s notes RAN1#111, 3GPP TSG RAN WG1 #111 Toulouse, France, November 14th – 18th, 2022