3GPP TSG-RAN WG1 Meeting #112 	R1-2301964

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	9.7.1

Source:	Moderator (Huawei)

Title:	FL summary#1 for spatial and power domain techniques for R18 NES

Document for:	Discussion and decision

- Introduction

This summary contains discussion points and proposals based on submitted contributions to agenda item 9.7 and 9.7.1.

For information, the current online schedule (based on v01) is as follows:

| Monday |  | Tuesday |  | Thursday |  |
| --- | --- | --- | --- | --- | --- |
| 17:00 ~ 19:30 (150 min) | R17 8.1 (50) R18 NCR (50) R18 NES (50) | 17:00 ~ 19:30 (150 min) | R17 8.1 (50) R18 NCR (50) R18 NES (50) | 08:00 ~ 10:30 (150 min) | R17 8.1 (50) R18 NCR (50) R18 NES (50) |

- High level issues

There are many valuable considerations and proposals looking into different aspects of this subject. At this moment, there are no specific discussion points set for some of those with the following initial consideration from FL. However, they can be further requested/discussed as the discussion extends into details or more findings are available.

Cross-WGs

[KT] provides proposals about gNBs the need of network coordination and indication. Regarding the indication methods, the details can be discussed following the techniques to be specified in RAN1. The coordination between gNBs including the potential definition of ES mode can also be discussed, and may particularly require higher-layer involvements. It is expected that other WGs will also further discuss these aspects at their need.

Cross-objectives

There is another objective for specifying cell DTX/DRX related enhancements. [Samsung] proposes joint consideration of techniques from spatial/power domain with enhancement of cell DTX/DRX. It is a possible/valid scenario that both two features can be enabled by a network. The specific impact of this scenario may be considered when the picture of each feature is clearer, while companies can take this into account during the discussion of each feature.

Clarification of scope/objectives

With respect to the spatial domain techniques, one clarification question from [Nokia/NSB] is whether multi-TRP scenarios and operations are in the scope of this work item. On one hand this scope related discussion may also be taken in RAN plenary, and on the other hand, FL understanding is that multi-TRP operations and enhancements are dropped from Rel-18 candidate objectives as RAN#98-e discussed carried out.

Furthermore, [Nokia/NSB] proposes to clarify the exactly covered UE CSI/CSI-RS capabilities regarding the “Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements” in the WID. Many contributions submitted to this meeting also describe enhancements/impact analysis on UE capabilities. RAN1 will need to discuss the concrete enhancements related to those capabilities.

Q2-1: Different views or request for a specific discussion can be provided here.

| Company | Comments |
| --- | --- |
| Lenovo | We agree with Nokia that more clarity is needed on the scope of the WI. We suggest that this question is discussed in the online session to seek more guidance from the chair. Our views are as follows: Multi-TRP scenarios: While multi-TRP scenarios have been discussed in the SI and included in TR 38.864, our understanding is that it is not included in the WID scope. However, CSI/BM aspects that are related to multi-TRP/panel can be discussed since CSI enhancements are clearly specified as part of the WID Legacy UE capability note: in our understanding, the note implies that the prospective NES enhancements should be based on the legacy UE capability, e.g., maximum number of CSI reporting config/CSI-RS resources/resource sets that the UE can be configured with. |
| Qualcomm1 | - R18 scope does not include mTRP scenarios. - On the note on UE capability, it includes all CSI related UE capabilities defined in R15/16/17. - The objectives have two important keywords “necessary enhancements” and “efficient adaptation”. Before discussing enhancement proposals in spatial and power domains, it is important that RAN1 discusses the following questions: Efficient adaptation of spatial elements - What does “efficient adaptation” mean in efficient adaptation of spatial elements? - Can the legacy CSI feedback framework already enable efficient adaptation of spatial elements. If it cannot, which part of CSI feedback framework is limiting factor in enabling such adaptation? - Can the legacy beam management framework already enable efficient adaptation of spatial elements? If not, which part of the beam management framework is limiting factor in enabling such adaptation? Efficient adaptation of power offset values between PDSCH and CSI-RS - What does “efficient adaptation” mean in efficient adaptation of power offset values between PDSCH and CSI-RS? - Can the legacy CSI feedback framework already enable efficient adaptation of power offset values between PDSCH and CSI-RS. If it cannot, which part of CSI feedback framework is limiting factor in enabling such adaptation? Good understandings of the above questions can provide good guidance on how RAN1 should discuss enhancements if any. |
| Apple | Multi-TRP scope: According to our understanding, multi-TRP is not included in the WI objective, On UE capability: According to our understanding, this means that CSI enhancements for NES should try to follow the legacy UE CSI/CSI-RS capabilities, enhancements that do not increase UE CSI/CSI-RS measurement and reporting requirement should be considered. Cross-objectives: Not at this stage, it is better to discuss separately before the enhancements for each objective becomes clearer. Cross-WG: Not at this stage, before the enhancements for each objective becomes clearer. |
| Futurewei | Aspects not explicitly stated in the WID should not be assumed to be included, especially on the non-trivial consideration of multi-TRP. On the Note on legacy UE CSI-RS, our understanding is that it should be taken into consideration during discussion. It was not meant to restrict any possible discussions or enhancements. |
| Huawei, HiSilicon | According to our understanding, Multi-TRPs is not included in scope of Rel-18. We agree with Apple, Cross-objectives and cross-WGs can be discussed at later stages. @ Qulacomm1 In our view, efficient adaptation means the adaptation that maximize the NES gain and at the same time guarantee negligible performance loss with respect to bassline schemes. Based on that, Dynamic adaptation (e.g. different number of TxRUs and/or different DL power per UE per slot or even OFDM symbol) can be called efficient adaptation as our simulations shows. In our opinion, Current CSI feedback framework has at least three inefficient aspects regarding Dynamic adaptation. - First, in current specification, to enable CSIs reporting for different spatial/power configuration, gNB can only uses different CSIreportConfigs. However, current CSIreportConfigs can be used for different purposes, such as beam management, CSI measurement and tracking. Multiple CSIreportConfigs used for NES in legacy way seems consuming the number of CSIreportConfigs and squeezing the configuration space of the CSI report for other purposes. And, multiple CSIreportConfigs with common parameters for NES causes a large amount of RRC payload. - Second, configure multiple independent CSI reports for NES in legacy way makes the calculation of CSIs inefficient, since the UE cannot use the correlation between CSIs as we discussed in our contribution. - Third, UE reports multiple independent CSIs makes ineffective utilization of uplink resource considering there is correlation between these CSIs and some compression methods can be used. All above three aspects should be discussed. The first one is necessary due to the legacy UE capability. The second one is necessary in order to decrease the UE burden for CSI calculation. |
| DOCOMO | - For multiple TRP, it is not included in the R18 NES WI. - For UE capability, although it is captured in the WI scope, the details are not fully discussed during the RANP meeting. From our view, the note is targeted for the legacy CSI related UE capabilities in R15/16/17. As R18 NES feature itself is an optional capability for the UE, the extension of the UE capability of CSI/CSI-RS measurement may be acceptable. |
| MTK1 | On Multi-TRP: From the WID, as quoted below, whether multi-TRP is included in the scope by trading TRP as one type of spatial elements may be a bit ambiguous. On the other hand, given there are only 4 meetings (including this one) for specification, prioritizing adaptation on antenna ports and active transceiver chains is more practical way forward. - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2]. On efficient adaptation: We would suggest discussion to clarify the following aspects: - What does the adaptation target? - The specification describes UE behaviors and, with DMRS, the adaptation of BS spatial elements for a data transmission is transparent to UE. What matters to the performance will be whether UE can help provide sufficient CSI information for BS to select the best transmission setting (including spatial elements, MCS, etc.). In this regard, it will be UE CSI-RS and CSI reporting configurations that require enhancement for efficient adaptation. - What are the requirements for efficient adaptation? - NES WI is for BS energy saving, and the operating condition for spatial and power domain NES scheme(s) can be medium (up to 50%) cell loading. If the adaptation requires frequent per-UE RRC reconfiguration, there can induce huge amount of signaling and energy overhead. - Given the WID captures the note of applying legacy UE CSI/CSI-RS capabilities, UE computation complexity will be controlled and frequent adaptation on UE CSI-RS and CSI reporting configurations is not deemed necessary. For BS scheduling complexity if the content of UE reports changes frequently, it also introduces additional complexity to incorporate such variation. By the above, infrequent cell-wise adaptation can be baseline to support, and further optimization, if providing significant benefit, can be perused further. On legacy UE CSI/CSI-RS capabilities: Our understanding on the note is to confine the total CSI processing complexity, including measurements and reports, within legacy UE capabilities. The will help R18 NES to be quickly supported by UEs as no change to existing UE processing capabilities. On cross-WG: We expect some work split discussion for Cell DTX/DRX mechanism should be discussed as suggested on R1-2301600. |
| Intel | From our perspective, mTRP is not included WI scope Regarding UE capability issue, we agree with QC and Apple that legacy UE CSI/CSI-RS capabilities apply. We do not see a need at this stage to consider spatial/power domain adaptation and cell DTX/DRX objectives jointly. They are captured as separate objectives and of course, NW could employ both specified solutions if needed. Cross-WG discussion can be triggered as needed, which is routine work. |
| Nokia | Fine for us, it’s good to clarify and have common understanding on whether Multi-TRP operations/scenarios. On the cross-objectives and cross-WG, we share similar views as Apple. |
| LG Electronics | Multi-TRP scope: We share the view with Qualcomm and Apple that multi-TRP scenario is out of the scope for Rel-18 NES. On UE capability: In our understanding, similar to other companies, legacy Rel-15/16/17 UE capabilities related to CSI/CSI-RS are kept for Rel-18 NES WI. Cross-objectives: It would be better to first discuss each objective separately. |

- Spatial element adaptation including beam management

The objective for spatial element adaptation is as below.

- Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2]

Many companies mentioned that different implementations regarding port virtualization should be considered. In general, it seems two types (or three, as illustrated by [NTT docomo]) of implementations exist, and majority consider that both of the two types can be supported except for [Transsion] who prefers only to consider the type that all spatial elements are associated to a single logical antenna port.

Given the willingness to accommodate all possible implementations, it can be suggested that

P3-1: RAN1 to specify spatial domain adaptations considering both of the below types of spatial element/TRxU mapping implementations

- Type 1: all spatial elements are associated to a logical antenna port

- Type 2: part/subset of spatial elements are associated to a logical antenna port.

| Company | Comments |
| --- | --- |
| Lenovo | Agree with this classification and support including an agreement to indicate this categorization for consideration and evaluation. More discussion is needed on whether one or both types are to be supported |
| Qualcomm1 | We understand that companies would like to cover all possible mappings between logical antenna ports and spatial elements (transceiver chains or physical antenna ports), which is reasonable from our perspectives. However, 3GPP specifications only specify up to the logical antenna port and leave the mapping between logical antenna port and spatial elements to implementation. How to turn on/off Type 1 or Type 2 depends on the choice of the logical antenna port configuration. The enhancements, if necessary, should be transparent regardless of whether type 1 or type 2 is implemented. In other words, it is not expected to have enhancements targeting for type 1 only or type 2 only. Therefore, we don’t think this proposal is necessary. |
| Apple | We are OK to consider both Type 1 and Type 2 in terms of how to specify corresponding resource and report configurations as well as procedures. However, if only one type is to be selected, we would prefer Type 1 since UE could measure on subset of ports of the current CSI-RS resources. For NTT docomo’s Type 3, we think it could be a special case of Type 2. However, we think the wording of the proposal needs to be modified, we are not discussing on how spatial elements are mapped to antenna port (which is a gNB architecture which is only for information), rather we are discussing on how these spatial elements are turned off, so the proposed wording is as follows, P3-1: Both below types of spatial element adaptation can be considered - Type 1: all spatial elements associated to a logical antenna port are disabled for network energy saving. - Type 2: part of spatial elements associated to a logical antenna port are disabled for network energy saving. |
| Futurewei | We support that both Type I and Type II are considered. |
| Huawei, HiSilicon | Both type 1 and type 2 should be supported. Even if the gNB implementation is transparent to UE, the enhancement in Rel-18 should be compatible with type 1 and type 2 to enable NES for networks with different types of gNBs implementations. |
| DOCOMO | Thank you for the classification. From our understanding there are two modes for antenna and virtual port mapping, i.e., sub-array mode and full-connection mode (refer R1-2301505). If network vendor considers that antenna on/off operation for full-connection mode can also be categorized into two types as P3-1, we can agree with P3-1. |
| InterDigital | Support FL’s proposal to consider both Type I and Type II |
| MTK1 | The intention of the proposal is to allow flexible restriction on CSI-RS and CSI report configurations regarding various BS implementation of adapting spatial elements. From UE perspective, if there is no significant CSI processing difference, it may not be necessary to separate Type-1 mapping and Type-2 mapping. We suggest come back on this proposal when companies have better consensus on what parameters to be adapted (related to P3-2) before discussing whether to further categorize different types of adaptations. |
| Intel | Open to consider both at this point. Further discussions on the actual specification impact associated with enhancements required to support option 1 and option 2 will be needed. |
| Nokia | Agree that, the mapping between logical antenna port and spatial elements is up to implementation. For type1 and type 2, the intention is to list all the implementation possibility that the enhancement on spatial muting adaptation may need to consider with, where both Type 1 and Type 2 would need to be considered in the discussions. And we could aim at designing enhancements that are transparent regardless of which Type is implemented at gNB. Regarding Apple’s suggested changes on the proposal, it’s worth noting that muting/disabling of spatial elements is only one part of the adaptation, and on the other hand, the unmuting/enabling is also a possibility. |
| LG Electronics | We support that both Type 1 and Type 2 are considered. In the case of Type 2, CSI-RS power may be changed because the number of TxRUs per port is changed while the number of ports is maintained. Therefore, we would like to clarify whether the meaning of supporting type 2 is that CSI-RS power changes in the power domain are also included in the work item scope. |
| Panasonic | We are okay to study the adaptation of both type 1 and 2, although the wording needs a bit polished to reflect the intention. The below is proposed based on the proposal from Apple: P3-1: Both below types of spatial element adaptation can be considered - Type 1: all spatial elements associated to a logical antenna port are adapted. Type 2: part of spatial elements associated to a logical antenna port are adapted. |

## P3-1-rev1

RAN1 to specify spatial domain adaptations considering the below cases

- Type 1: all spatial elements associated to a logical antenna port is disabled/enabled

- Type 2: part/subset of spatial elements associated to a logical antenna port is disabled/enabled

FFS whether/how to categorize Type 3 as illustrated in R1-2301505, which is shown below.

| Company | Comments |
| --- | --- |

For spatial element adaptation, the understanding of the adapted parameters and content for indication/configuration is needed, e.g. for representing a spatial pattern w.r.t. the ports and/or number of elements, codebook [Nokia/NSB, CATT, China Telecom, Fujitsu, NEC, InterDigital, ETRI, Samsung, MediaTek etc., Apple]. The following proposal can be considered for discussion.

P3-2: Further study necessary parameters in configuration and report, for formulating a corresponding spatial element pattern, e.g.

- Antenna ports

- [Codebook structure]

  - No: CATT, SPD, nokia

- CSI content

- FFS: how to index to a spatial element adaptation pattern.

| Company | Comments |
| --- | --- |
| Lenovo | Support in general, however propose to replace “CSI content” with “CSI parameters” |
| Qualcomm1 | It is premature to discuss this proposal. Please see our comment in Q2-1. |
| Apple | We think this proposal is a bit vague in terms of “spatial element pattern”. We understand the motivation for this proposal is to discuss on what enhancements need to be considered for resource configuration and reporting configuration. We think this should be discussed more in detail and could be different for the two types of spatial elements adaptation. For example, Regarding resource configurations, for Type 1, we think at least a subset of the antenna ports should be indicated, while for Type 2, new resources or resources sets need to be configured. Regarding reporting configurations, for Type 1, the codebookConfig should also be additionally indicated, while for Type 2, this change may not be needed. Note that Type 1 and Type 2 can be only used for discussion purpose and does not need to be reflected in the spec. For “CSI content”, we are not clear what this refers to, does this mean CSI reporting contents, including the all the csi-related quantities? We think for this part, it also has to do with the reporting procedure enhancements, for example, whether we allow multiple CSI results (corresponding to the results before and after the spatial elements adaptation) simultaneously carried in one CSI reports. For now, we think we could separate the discussion for resource configurations and reporting configurations for clarity, and start with the resource configuration part. |
| Futurewei | Our view is that discussion should be separated into DL signaling/configuration and UL reporting/CSI content. At least for DL, deciding on the antenna ports and its ‘arrangement’ as per the Type I and II that constitute an ‘adaptation pattern’ needs further discussion. Other parameters, if needed, would impact the design for the indexing of these patterns. |
| Huawei, HiSilicon | Support in general. Perhaps spatial element pattern in type 1 and type 2 can be designed in a unified way. |
| DOCOMO | The wording of “for formulating a corresponding spatial element pattern” is not clear. If it is talking about the resource configuration, there are many other possible methods as indicated in P3-4. If it is talking about the triggering of multiple CSIs, it has lower priority than discussion of P3-3 and P3-4. We can come back here later if we have conclusion on P3-3 and P3-4. |
| InterDigital | Generally ok with the proposal but further clarification on the parameters related to antenna ports and CSI content is needed. Also, before discussing on indexing it would be useful to also discuss what constitutes a spatial element adaptation pattern. |
| MTK1 | We are supportive of this discussion for identifying what parameter(s) is essential to be adapted and the corresponding UE processing impact. To provide more specific information, we suggest moderator can quote some RRC parameter(s) and the description(s) for ease of companies’ check. |
| Intel | This proposal can be revisited later |
| Nokia | Although we somewhat see the intention of the proposal, however we agree with other companies that this proposal is not fully clear. Better to first agree on how a spatial pattern would be defined, e.g. using subset of antenna ports (what else, if any?) etc. We could then discuss how a report configuration would need to be enhanced: this would require discussions on the resource(s) configuration and what content to consider for CSI report(s). For the first part, we suggest discussing the following proposal: Proposal: Discuss which elements form or identify a spatial pattern, such as whether this can be solely done using an antenna port set/subset. |
| LG Electronics | It is negative to change the codebook structure to adapt the spatial element pattern, but the codebook subset restriction can be considered. |
| Panasonic | We prefer to discuss bullet by bullet, based on the two types of adaptation as in previous proposal, respectively. |

## P3-2-rev1

Identify specification impact that enable dynamic spatial element adaptations at gNB and enable measurement/report at UE for different spatial element adaptation types.

For formulating a corresponding spatial element pattern, further study necessary parameters in configuration and report, e.g.

- Antenna ports

- [Codebook structure]

- CSI content

- FFS: how to index to a spatial element adaptation pattern.

| Company | Comments |
| --- | --- |

For the effective adaptation, there is vast support of dynamic spatial element adaptation, given what has been studied and found in the SI stage.

For CSI measurement, enhancements on CSI resource configuration and association were mentioned. There are quite a few alternatives considering different mapping methods, e.g. multiple CSI resources in one CSI-RS resource set, each corresponding to a spatial element configuration including spatial element adaptation patterns [FUTUREWEI] [Nokia/NSB] [vivo] [Xiaomi] [Apple], multiple CSI measurement for each NZP CSI-RS resource [CATT, InterDigital, Intel, LGE], One CSI-RS resource associated with one or multiple CSI-RS AP sets, each CSI-RS AP set is a subset of the total antenna ports of the CSI-RS resource [vivo][ETRI] [Ericsson].

For CSI report, enhancements on multiple CSIs were mentioned. There are quite a few details for the reporting, e.g. that a UE report including multiple spatial element configurations, or the best N beams for each antenna panel independently in one CSI report, or with selection of one or a subset of all possible spatial patterns [FUTUREWEI] [Nokia/NSB] [vivo] [Lenovo] [LGE] [Fraunhofer] [Qualcomm], multiple CSI feedback in one report [Huawei/HiSi, ZTE, CATT, CMCC, ETRI, NTT docomo] which may be on a need basis with suited triggering/periodicity for AP/P, SP-CSI reporting [Samsung].

Considering the above, the following proposal can be made to reflect major preference and drive the group for further discussion.

P3-3: Specify means that enable dynamic spatial element adaptations at gNB (i.e., in a more efficient manner than static adaptation based on RRC configurations) and enable measurement/report at UE for different spatial element patterns, at least considering the following

- Impact on CSI resource configuration including resource mapping, CSI resource patterns

- Impact on CSI reporting including CSI report configuration, CSI computation, CSI processing complexity

- Impact on beam management and other relevant measurements, including e.g. TCI state, QCL info, RLM, RRM.

Alt: Identify potential specification impact that enable dynamic spatial element adaptations at gNB (i.e., in a more efficient manner than static adaptation based on RRC configurations) and enable measurement/report at UE for different spatial element patterns adaptation types, at least considering the following potential issues

- Impact on CSI resource configuration including resource mapping, CSI resource patterns, new resource indication

  - To address the issues e.g.

- Impact on CSI reporting including CSI report configuration, CSI computation, CSI processing complexity, CSI reporting overhead

- Impact on beam management and other relevant measurements, including e.g. TCI state, QCL info, RLM, RRM, pathloss.

- Impact on UE measurement and reporting complexity, including resource and report activation/deactivation mechanisms, CPU occupation, etc.

| Company | Comments |
| --- | --- |
| Lenovo | Support |
| Qualcomm1 | It is premature to discuss this proposal. We propose to address our questions in Q2-1 first. It should be noted that dynamic spatial element adaptation is already possible today. This can be implemented by many different means e.g., based on aperiodic CSI feedback framework, semi-persistent CSI feedback framework, even with periodic CSI feedback framework, or BWP switching etc. |
| Apple | We support the main bullet and propose the following points to be added. Explanations are provided in the bracket. Specify means that enable dynamic spatial element adaptations at gNB (i.e., in a more efficient manner than static adaptation based on RRC configurations) and enable measurement/report at UE for different spatial element patterns adaptation types, at least considering the following - Impact on CSI resource configuration including resource mapping, CSI resource patterns, new resource indication [Explanation: This is to include the case where additional resources/resource sets are needed for spatial adaptation] - Impact on CSI reporting including CSI report configuration, CSI computation, CSI processing complexity, CSI reporting overhead - Impact on beam management and other relevant measurements, including e.g. TCI state, QCL info, RLM, RRM. - Impact on UE measurement and reporting complexity, including resource and report activation/deactivation mechanisms, CPU occupation, etc. - [ From UE perspective, we think it is important to at least support the case where the UE do not need to measure resources before and after spatial adaptation simultaneously and/or report the CSI result before and after spatial adaptation simultaneously. This would require some enhancements on the activation/deactivation of resources and reports that are configured in a single or multiple ReportConfig. We also would like to draw the attention on the potential impact on CPU occupation once the resource configuration is clear. ] |
| Futurewei | Support |
| Huawei, HiSilicon | Support. @ Qualcomm All the mechanisms mentioned need multiple CSIreportConfig for CSIs reporting. And as we discussed in Q2-1, configured multiple CSIreportConfigs for NES in legacy way is inefficient. For example, for type 1 shutdown, due to the different number of CSI-RS ports, different condebookconfigs are needed. |
| DOCOMO | Support the proposal. |
| InterDigital | Support |
| MTK1 | Currently specification already support dynamic adaptation on spatial elements for BS data transmission, CSI-RS transmission, and UE CSI report contents. We see Qualcomm have good summary in their reply. Therefore, the focus of the proposal should not be “enable dynamic spatial element adaptations”. Instead, the focus can be on how to “enable efficient spatial element adaptations for NES”. In our view, extending current per-UE adaptation framework(s) to cell-wise adaptation would be the most practical design to start with. |
| Intel | Clarification of Spatial element pattern needs more discussion first. We are ok in principle that we need to discuss different aspects related to dynamic spatial element adaptation. |
| Nokia | We are fine with the intention of the proposal. However, we have a few comments: - We don’t think that “(i.e., in a more efficient manner than static adaptation based on RRC configurations)” is needed, so it should be removed from the proposal. - On “enable measurement/report at UE for different spatial element patterns”: we agree that we would need to enable measurements for different spatial patterns, however what to report would require discussions. The current formulation seems to suggest that the UE provides report for different spatial patterns, however this is only one possibility. - The aspects listed in the proposal should be first discussed, so for the moment we cannot conclude on what exactly the ‘means’ to specify. We thus suggest the following updates on the proposal: P3-3: To enable dynamic spatial element adaptations at gNB (i.e., in a more efficient manner than static adaptation based on RRC configurations) and corresponding measurement/report at UE for considering different spatial element patterns, at least considering the following discussions aspects - Impact on CSI resource configuration including resource mapping, CSI resource patterns - Impact on CSI reporting including CSI report configuration, CSI computation, CSI processing complexity - Impact on beam management and other relevant measurements, including e.g. TCI state, QCL info, RLM, RRM. |
| Lenovo2 | We are fine with the updated FL proposal. Also, Nokia’s rewording is fine for us |
| LG Electronics | We are generally fine with the proposal modified by Apple except for the new resource indication in the first bullet which is not clear. For second bullet, we think that priority and omission rules for CSI reports should also be considered. Impact on CSI reporting including CSI report configuration, CSI computation, CSI processing complexity, priority and omission rule for CSI reports |
| Panasonic | We are supportive to study these impacts. |

## Q3-1

For spatial element adaptation, from CSI resource and CSI report configuration perspective, do you consider the current specification is efficient enough? If ‘N’, what enhancement may be needed for CSI resource configuration and/or CSI report configuration?

| Company | Y/N | Comments |
| --- | --- | --- |

## Q3-2

For spatial element adaptation, from UE measuring and reporting CSI perspective, do you consider the current specification is efficient enough? If ‘N’, what enhancement may be needed for CSI measurement and reporting?

| Company | Y/N | Comments |
| --- | --- | --- |

## Q3-3

For spatial element adaptation, from beam management perspective, do you consider the current specification is efficient enough? If ‘N’, what enhancement may be needed for beam management?

| Company | Y/N | Comments |
| --- | --- | --- |

There is also large support that configuration/report overhead should be minimized along with dynamic spatial element adaptation.

In general, one or multiple CSI resources for measurement and one or multiple CSI for report are considered beneficial for network to make proper adaptation decisions. If/when multiple CSI resources corresponding to different spatial element adaptation patterns are configured, multiple CSI feedback are possible such that proper scheduling/adaptation decisions at gNB can be possible based on the multiple CSIs. Differing from what current specification can do, i.e. to be more efficient, many companies mentioned that there is relation among different spatial element patterns that can be utilized. For example, there could be nested structure for CSI resources for measurement of different patterns [Qualcomm] such that a common CSI-RS resource potentially for multiple patterns can be feasible [CATT] [Intel][Fujitsu] etc., then the configuration overhead can be reduced. Or, at UE side, correlation property in CSIs among different patterns [Huawei/HiSi], different CSI reporting hypotheses for different levels of spatial dimensions [AT&T], omitting the duplicated CSI reporting contents [NTT docomo], hypothetical RSRP or hypothetical CQI [NEC] may also be feasible, such that UE report burden reduction as the major motivation can be achieved.

Note that, the above may assume the operation is that the gNB makes propose scheduling/adaptation based on multiple CSIs. Alternatively, another possible approach is that gNB dynamically indicates/applies the configuration corresponding to an adaptation, without knowing the accurate CSI beforehand, but with a same motivation to reduce the UE report burden since only single-CSI is performed.

P3-4: Discuss the properties that could be utilized for reducing the configuration overhead or UE report overhead/processing complexity, considering the following scenarios

- Case 1: corresponding CSI is available at gNB before adaptation, by

  - a) multiple CSI-RS resources associated with multiple spatial adaptation patterns, with multiple CSIs in one report

  - b) a common CSI-RS associated with multiple spatial adaptation patterns, with multiple CSI reports

- Case 2: a common CSI-RS resource or multiple CSI-RS resources are associated with multiple spatial adaptation patterns, with gNB dynamically adapting the spatial elements based on single-CSI report corresponding to one pattern

| Company | Comments |
| --- | --- |
| Lenovo | Can we suggest breaking down this proposal to two sub-proposals that separately discuss the spatial adaptation patterns and the CSI feedback? For spatial adaptation pattern, we also suggest to discuss how a spatial adaptation pattern is identified for a single CSI-RS with multiple patterns, whether as part of NW configuration or UE reporting. We suggest the following proposal update for consideration: P3-4-1: Discuss the properties that could be utilized for reducing the CSI-RS configuration overhead/processing complexity, considering the following scenarios - Case 1: multiple CSI-RS resources associated with multiple spatial adaptation patterns - Case 2: common CSI-RS resource associated with multiple spatial adaptation patterns - FFS: whether/how an index of a spatial adaptation pattern is configured/reported For CSI reporting, we propose to include three alternatives: (i) multiple CSI reports, (ii) multiple CSIs in one CSI report, or (iii) one CSI reported based on UE selection of one of multiple patterns. We suggest the following proposal update for consideration: P3-4-2: Discuss the properties that could be utilized for reducing CSI reporting overhead/processing complexity, considering the following scenarios - Case 1: multiple CSI hypotheses corresponding to multiple configured spatial adaptation patterns are reported with multiple CSIs in one report - Case 2: multiple CSI hypotheses corresponding to multiple configured spatial adaptation patterns are reported with multiple CSI reports - Case 3: one CSI hypothesis corresponding to a selected spatial adaptation pattern is reported, with additional parameter(s) indicating an index of the selected spatial adaptation pattern in the CSI report |
| Qualcomm1 | It is premature to discuss this proposal. We propose to address our questions in Q2-1 first. Benefit of having one CSI report setting contains multiple CSI report configs corresponding to multiple spatial adaptation patterns is unclear: - CSI report configuration overhead reduction: It is understood that some parameters may be shared across different CSI report configs associated with different spatial adaptation patterns. However, codebook config is likely the heaviest one out of all and it is likely needed for each adaptation patterns. Furthermore, CSI report configuration is RRC configured. Hence, the overhead reduction is quite minimal. - Complicate RRC signaling design for CSI report setting: Now RAN2 has to implement a new type of CSI report setting IE. - CSI processing complexity reduction at UE: No, CSI computation is the same as that with multiple CSIs associated with multiple legacy CSI report settings. We understand some correlation between CSIs associated with different adaptation patterns. However, the implementation complexity to leverage such correlation is non-trivial and overall complexity remains the same as the legacy CSI framework. - Require spec change in defining CSI computation/processing requirements. Some counting in CSI-related UE capabilities (e.g., FG 2-33, FG 2-35, etc.) must be scaled up accordingly. Case 1b and Case 2 are already supported in today spec. For example, Case 1b can be achieved by AP/SP/P CSI report framework with multiple CSI report configurations while Case 2 can be achieved by AP/SP CSI report framework. |
| Apple | We don’t see much of the relationship between when the adaptation takes place and design on how to reduce configuration/report overhead/complexity. Rather, it would be clearer we could list out all the subcases regarding how resource configuration and reporting configuration are linked and determine which of the combinations are really needed. We think some terminology alignment should be made here, if we understand correctly, by “multiple CSIs in one report” in case 1a), this actually refers to enhancement to include multiple CSIs configured in one CSI-ReportConfig, while by “multiple CSI reports” in case 1b), this actually refers to that these reports are configured in multiple CSI-ReportConfigs as current spec supports. If above is the correct or common understanding, then we think at least another two cases (case c and d in the following) should be listed for discussion Case a) multiple CSI-RS resources/resource sets/resource settings associated with multiple spatial adaptation patterns, with multiple CSIs in one report Case b) a common CSI-RS resource/resource set associated with multiple spatial adaptation patterns, with multiple CSI reports Case c): multiple CSI-RS resources/resource sets/resource setting associated with multiple spatial adaptation patterns, with multiple CSI reports Case d): a common CSI-RS resource/resource set associated with multiple spatial adaptation patterns, with multiple CSIs in one report Then we could discuss on which of the four cases are really needed and the corresponding configuration overhead reduction, report overhead reduction, UE measurement and report complexity reduction, etc. |
| Futurewei | It is not clear what the two cases are differentiating. Case 1 and 2 should be left up to the gNB implementation |
| Huawei, HiSilicon | We think that case 1 reduce the configuration overhead and could be used as an indication to the UE that the CSI calculations are correlated which both save the UE needed CSI processing and could lead to at the end to UE efficient (compressed/optimized) reports/feedback to the gNB. |
| DOCOMO | From our understanding, the proposal mixes several things as follow. - resource configuration, - reporting configuration - and adaptation procedure. We think that it is better to discuss these issues separately. Then companies could show their preference on each issue. For example, - For resource configuration, we have following options (copy some from APPLE): - multiple CSI-RS resources/resource sets/resource settings associated with multiple spatial adaptation patterns, - a common CSI-RS resource/resource set associated with multiple spatial adaptation patterns - For CSI reporting of multiple CSIs, we have following options - Independ reporting of each CSI (one CSI corresponding to one spatial assumption) - Selected reporting CSI, e.g. gNB configures several, UE selects one or multiple to report - Joint reporting with multiple CSIs, some overhead reduction approaches can be considered, such as considering correlation property, omitting the duplicated CSI reporting contents, etc. - For adaptation procedure, we have following options - corresponding CSI is available at gNB before adaptation, others |
| InterDigital | Share same concerns raised by Apple regarding the cases. We think the suggestion provided by Docomo for separating the issues is quite reasonable. |
| MTK1 | The reasons behind separating the cases will need to be further justified. Having the discussions separately for CSI-RS configuration and CSI reporting, as suggested by Lenovo, will be useful for companies to understand the difference for each case. |
| Nokia | We don’t support the proposal in its current form. We share similar views as Apple on the unclear relationship between when the adaptation takes place and how to reduce configuration/report overhead and complexity. So, the proposal needs to be revisited in order to clarify its scope, e.g., by removing the ‘complexity and overhead’ part from the proposal and listing various possibilities/cases for CSI reporting to assist the gNB and how this CSI is measured and reported … The updated list by Apple could be a good starting point. |
| ‍CEWiT | In the explanation, it is mentioned that the Gnb adapts the spatial muting pattern, without knowing the accurate CSI beforehand. However it is not captured in case 2. Hence the following update is suggested for the case 2 - Case 2: a common CSI-RS resource or multiple CSI-RS resources are associated with multiple spatial adaptation patterns, with gNB dynamically adapting to one pattern and indicating to the UE for single-CSI report corresponding to the pattern |
| Lenovo2 | As discussed in our previous response, we see the need of breaking down the proposal to discuss CSI resource configuration and CSI reporting separately. We also agree with DOCOMO’s comments |
| LG Electronics | We share a similar view with DOCOMO. It would be better to discuss separately by dividing it into CSI framework and CSI reporting enhancement rather than dividing it into Case1 and Case2. For CSI framework enhancements, we have following approaches - Approach 1: different spatial patterns per CSI-ReportConfig - Approach 2: different spatial patterns per CSI-RS resource set - Approach 3: different spatial patterns per CSI-RS resource - Approach 4: different spatial patterns for a CSI-RS resource configuration For CSI reporting enhancements, we have following options - Option 1: report the multiple CSIs in a CSI report corresponding to configured spatial assumptions - Option 2: report the single or multiple CSIs in a CSI report corresponding to a specific spatial assumption among the multiple configured assumptions by gNB’s indication Option 3: report the single or multiple CSIs in a CSI report corresponding to a specific spatial assumption among the multiple configured assumptions by UE with pre-configured criteria |
| Panasonic | We are okay with DOCOMO’s approach, although we think it is pre-mature to discuss this. |

## P3-4-rev1

For spatial element adaptation, further discuss the following cases

- For resource configuration,

A1) multiple CSI-RS resources/resource sets/resource settings associated with multiple spatial adaptation patterns,

A2) a common CSI-RS resource/resource set associated with multiple spatial adaptation patterns

- For CSI reporting of multiple CSIs, we have following options

B1)	Independ reporting of each CSI (one CSI corresponding to one spatial assumption)

B2) Selected reporting CSI, e.g. gNB configures several, UE selects one or multiple to report

B3) Joint reporting with multiple CSIs, where overhead reduction approaches can be considered, such as considering correlation property, omitting the duplicated CSI reporting contents, etc.

- For adaptation procedure,

C1) corresponding CSI is available at gNB before adaptation

C2) gNB makes adaptation before asking UE for reporting the corresponding CSI.

| Company | Comments |
| --- | --- |

Regarding the impacted signals, channels, or codebook types, whether spatial adaptation is applicable to all UE dedicated signals and channels is questioned [Nokia/NSB]. And there is view to start from periodic CSI-RS and joint measurement for CSI-RS and PL-RS can be considered [CMCC], or view to start from persistent or Aperiodic CSI-RS [Spreadtrum][OPPO]. The enhancements may be applicable to R15 Type1, R16 eType2, and R18 eType2 codebook [Google]. From FL perspective it seems simpler to focus on UE-specific PDSCH especially considering the study in SI. With this, it may also become natural that coverage of common control channels with spatial elements adaptation could remain the same, as proposed by [e.g. CATT] or that impact on reference signal performance can be minimized, as proposed by [ZTE]. Furthermore, potential needs to introduce new CSI-RS patterns are also discussed in [China Telecom, NEC, and Qualcomm].

In order to initiate the discussion without precluding other possibilities once clearer picture is obtained, the following can be considered at this stage.

P3-5: Start with focus on UE-specific PDSCH for spatial element adaptation discussion

- FFS: whether the adaptation is also applied to CSI-RS or other transmission.

- FFS: needs to introduce new CSI-RS transmission patterns, e.g.

  - CSI-RS transmission with reduced density

  - CSI-RS patterns that allow more nested spatial patterns.

| Company | Comments |
| --- | --- |
| Lenovo | Support |
| Qualcomm1 | During SI, only adaptation for UE-specific PDSCH was analyzed. Furthermore, the objective clarifying necessary enhancements on CSI related procedure means that the adaptation here is for only PDSCH per our understanding. Hence, we should only discuss enhancements if any to support spatial elements adaptation for PDSCH, and we don’t think the first FFS is needed. For 2nd FFS, we can wait after RAN1 has good understandings on questions we raised in Q2-1. |
| Apple | Our understanding is that the enhancements for CSI framework or procedures are for gNB to get a more accurate CSI to determine what spatial elements adaptation can be applied to UE-specific PDSCH. For other channels, it needs to be further justified the necessity further before we could make further enhancements. For the second FFS bullet, we are not very clear why it is discussed here, we think this FFS should be discussed in P3-3. |
| Futurewei | Support the proposal and on the FFS, our view is that CSI-RS is not impacted by spatial adaptation. gNB can and should maintain the same total CSI-RS power. |
| Huawei, HiSilicon | We think we should focus on the PDSCH. Due to the reference signal are sparse in time domain, the energy saving gain for spatial elements adaptation for reference signal is expected to be negligible. |
| DOCOMO | Delete first FFS, as CSI-RS adaptation for PDSCH CSI reporting is needed. Move the second FFS into P3-4, as it is just discussing on CSI-RS resource configuration. |
| MTK1 | We see adaptation on CSI-RS transmission can be included in the design for adapting UE’s CSI-RS configuration. There is no need for separated discussion. On CSI-RS transmission patterns, we haven’t seen the necessity and cannot agree to FFS without sufficient justification. |
| Intel | Agree with main bullet in principle |
| Nokia | We are fine to first focus on CSI enhancements to enable spatial adaptation for PDSCH. However, impact on other channels/signals (such as CSI-RS for BM, etc.) would need to be discussed. It should be noted that less NES gains would be expected if only adaptation for PDSCH is considered. In addition, we don’t see the intention or need for the second FFS in the proposal, so we suggest removing it. |
| LG Electronics | We are OK to focus on UE-specific PDSCH first for the spatial adaptation discussion. However, we think that the second FFS that changes CSI-RS itself through the new CSI-RS transmission pattern should be removed. |
| Panasonic | We agree and support Nokia’s view. |

## P3-5-rev1:

For spatial element adaptation, in addition to UE-specific PDSCH, further study whether the adaptation needs also to be applied to CSI-RS or other transmission.

| Company | Comments |
| --- | --- |

There could be a need to define a NES mode [KT, Lenovo] to be used for differentiation of a normal operation mode for gNB compared to the operation that gNB performs techniques aiming for energy savings. There could also be a need to allow the gNB occasionally perform full-port transmission [MediaTek] or different CSI feedback for different RSs patterns on demand [Samsung]. Also, the operation or scenario may be activated or deactivated by signalling, e.g. activation of a CSI report from multiple configurations [Google]. A signalling could be semi-static [NEC] or dynamic, e.g. MAC CE or DCI [FUTUREWEI] [InterDigital] [Samsung] [Ericsson] [MediaTek] [OPPO] [Intel] [ETRI] [AT&T], and a signalling could be UE specific DCI [CATT], group common DCI [vivo][Fujitsu] [Intel] or cell-wise indication [MediaTek]. Also, there could be a transition period along with each adaptation [NTT docomo], depending on how fast a change can be made.

P3-6: Discuss the following signalling aspects for spatial element adaptation

- The need of a RRC configured NES mode,

- The need of dynamic signalling based activation and deactivation with e.g.

  - MAC-CE

  - UE specific and/or group common DCI

  - Cell-wise indication

- This does not preclude to use combination of different signalling methods

- Transition time per adaptation can be taken into account

| Company | Comments |
| --- | --- |
| Apple | Could be discussed when P3-3 and P3-4 become clearer. |
| Lenovo | Support |
| Qualcomm1 | It is premature to discuss this proposal. We propose to address our questions in Q2-1 first. |
| Futurewei | Support |
| Huawei, HiSilicon | We are fine to discuss it. The details can be determined after more progress is made for understanding what needs to be signalled. |
| DOCOMO | Agree with Apple that we can touch it later. |
| InterDigital | Support |
| MTK1 | This can be discussed after achieving better consensus on P3-2 and P3-3. |
| Intel | During SI phase, RAN1 preferred to not explicitly use any ‘NES mode’. Among the different signaling options, we prefer DCI based activation/deactivation as it provides the fastest latency for adaptation, and since it is possible to address group of UEs. MAC-CE for a group of UE is not feasible due to the MAC CE signaling architecture. |
| Nokia | There is NO need to specify the NES mode, as we have been discussed during SI phase, and there was no conclusion being reached on this aspect. Thus, we don’t agree to specify the need of a RRC configured NES mode. For the other items, we agree with Apple to discuss the proposal after clarifying at least some of the earlier proposals. |
| LG Electronics | We are open to discuss about introduction of NES mode, but we need to discuss how to define it and the necessity first. |
| Panasonic | Okay. |

[Intel] considers that enhancements should not be based on multiple BWPs, given the commercial support of multiple BWPs framework (as optional UE capability) and the nature of BWP w.r.t. network energy savings. [Huawei/HiSi] mentions that replying on BWP switching may lead to more inefficiency from performance perspective. FL considers this view can also be taken into account in detailed discussion without setting a discussion point or proposal at this moment.

Q3-1: Different views or request for a specific discussion can be provided here.

| Company | Comments |
| --- | --- |
| Lenovo | Prefer to prioritize single BWP scenarios. |
| Qualcomm1 | We propose that “The UE is provided with an indication on the PDSCH transmission change” where the power change can be due to spatial element adaptation or power reduction. We can discuss the need of this aspect when discussing questions in Q2-1. |
| Intel | The exact formulation can be further discussed. One method to move forward could be to agree that we will focus on single BWP scenarios as Lenovo mentioned. |

- DL transmission power adaptation

The objective for transmission power adaptation is as below.

- Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2]

A first question similar to that is asked for spatial adaptation is that what is being adapted for DL transmission power of UE specific channels/signals. At least powerControlOffset, which is the power offset between PDSCH and CSI-RS, can be dynamically indicated to UE, as many companies proposed, e.g. dynamically notifying the UE (e.g. of the powerControlOffset values to use in CSI reporting) [FUTUREWEI] [Nokia/NSB] [Spreadtrum] [Panasonic] [ZTE] [Xiaomi] [Fujitsu] [InterDigital] [CMCC] [Lenovo] [Samsung] [CEWiT], or at least a UE can be provided with an indication on the PDSCH transmission change (which should not be frequent) [Qualcomm].

A few companies also consider the parameter powercontrolOffsetSS needs to be adapted or notified [CATT, Panasonic, MediaTek].

The below proposal is made given the view so far.

P4-1: For DL transmission power adaptation, at least the transmission power offset between UE-specific PDSCH and CSI-RS is applied.

- FFS whether the current parameter powerControlOffset is reused

FFS: indication for power offset among other channels/signals, e.g. between CSI-RS and SSB.

| Company | Comments |
| --- | --- |
| Lenovo | Support in general, however we suggest the following minor update FFS whether/how the current parameter powerControlOffset is reused |
| Qualcomm1 | Per our understanding, objective clearly mentions “efficient adaptation of power offset values between PDSCH and CSI-RS” meaning that the enhancement if any should deal with the offset between PDSCH and CSI-RS. Furthermore, during SI, only power adaptation of UE-specific PDSCH was analyzed. Hence, 2nd FFS is not needed. The main proposal is not clear since today such offset is already applied. We suggest discussing questions we raised in Q2-1 first. |
| Apple | We are not very sure what this main proposal would like to discuss, which, according to our understanding, is already captured in the objective “enable efficient adaptation of power offset values between PDSCH and CSI-RS”. |
| Futurewei | Support. On the FFS. our view is to reuse the parameter powerControlOffset and do not change the power offset between CSI-RS and SSB powerControlOffsetSS . |
| Huawei, HiSilicon | Support in general. Similar to what we discussed in P 3-5, transmission power adaptation for reference signals will not bring notable energy saving gain. |
| DOCOMO | It is not clear on the motivation of the proposal in the main bullet as Apple mentioned above. |
| InterDigital | Generally support |
| MTK1 | Agree with Qualcomm that the WID already restricts to the power offset between PDSCH and CSI-RS, as quoted below. In this regard, we can do analogous discussion as P3-2 and discuss whether/how to include multiple candidate power offset values, which will be more specific to move forward. - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2]. |
| Nokia | We propose to discuss, how the power control offsets between UE-specific PDSCH and CSI-RS shall be enhanced for dynamic power adaptation. Regarding enhancements to power offsets between CSI-RS and SSB (or between other channels/signals) we think that they are not in the scope of the work item. We propose to clarify this in the meeting. |
| CEWiT | As given in our contribution, the intention of the proposal is not clear. The adaptation of power offset should be indicated to the UE. The indication can be either in form of value of power offset or change in the value of power offset. Hence we suggest to update the proposal as follows for discussion: P4-1: For DL transmission power adaptation, at least the adaptation in transmission power offset between UE-specific PDSCH and CSI-RS is dynamically indicated to UE. -FFS how the adaptation in transmission power offset between UE-specific PDSCH and CSI-RS is indicated FFS: indication for power offset among other channels/signals, e.g. between CSI-RS and SSB. |
| LG Electronics | As mentioned in P3-1, this is related to Type 2 partial element/TxRU mapping. Since CSI-RS power may be changed because the number of TxRUs per port is changed while the number of ports is maintained. Therefore, we would like to clarify whether the meaning of supporting type 2 is that CSI-RS power changes in the power domain are also included in the work item scope. |
| ‍Panasonic | We support |

## P4-1-rev1

For DL transmission power adaptation between power offset between UE-specific PDSCH and CSI-RS,

- FFS whether/how the current parameter powerControlOffset is reused

FFS: indication for power offset among other channels/signals, e.g. between CSI-RS and SSB.

| Company | Comments |
| --- | --- |

Furthermore, similar issue is observed as for spatial element adaptation, for accurate CSI information being available at gNB before adaptation, multiple CSI corresponding to different adaptation values may be preferred, therefore CSI measurement with multiple power offset values [MediaTek] [Ericsson], and/or CSI report corresponding to different power offsets [Huawei/HiSi] [Nokia] [ZTE] [xiaomi] [Fujitsu, who also consider single-CSI] can be considered, or to enable an approach such that power adaptation is decided by both NW and UE [China Telecom].

And, to reduce the UE report overhead, again the multiple CSIs can be feedback in one report [Huawei/HiSi] [ZTE], and/or other report optimization can be considered, e.g. an indication of maximum power reduction can be made while the UE still maintains rank/MCS [Nokia/NSB], a UE can report its tolerance of potential PDSCH power reduction [NEC], or a UE reports a power control offset value based on a configured target WB CQI or reporting a differential CQI value [Lenovo].

P4-2: For DL transmission power adaptation, discuss the following scenarios when multiple power offset values configured for CSI measurement and report

- Case 1: multiple CSIs corresponding to different power offset values in one report is performed

- Case 2: one power offset value is dynamically notified to UE based on single-CSI report

- Case 3: other UE report content is included, e.g. maximum power reduction

| Company | Comments |
| --- | --- |
| Lenovo | We are fine with this proposal. We suggest a few wording updates that capture the number of CSI hypotheses reported, as follows: P4-2: For DL transmission power adaptation, discuss the following scenarios when multiple power offset values configured for CSI measurement and report - Case 1: multiple CSIs hypotheses corresponding to different power offset values is reported in one report is performed - Case 2: one CSI hypothesis is reported based on a single power offset value is dynamically notified to UE based on single-CSI report - Case 3: other New UE report content is included, e.g., maximum power reduction for a given power offset/CQI values |
| Qualcomm1 | It is premature to discuss this proposal. We propose to address our questions in Q2-1 first. |
| Apple | We would like to discuss on the cases to be supported similar to P3-4, Case a) multiple CSI-RS resources/resource sets/resource settings associated with multiple power offset values/assumptions, with multiple CSIs in one report Case b) a common CSI-RS resource/resource set associated with multiple power offset values/assumption, with multiple CSI reports Case c): multiple CSI-RS resources/resource sets/resource setting associated with multiple power offset values/assumptions with multiple CSI reports Case d): a common CSI-RS resource/resource set associated with multiple power offset values/assumptions, with multiple CSIs in one report |
| Futurewei | Support the options listed. |
| Huawei, HiSilicon | Preferer Case 1. |
| DOCOMO | A little confusing of the wording. Whether the “report” in the wording “in one report/ single-CSI report” stands for the CSI report configuration? |
| InterDigital | Generally ok with the proposal but for facilitating initial discussions we suggest modifying the proposal as following P4-2: For DL transmission power adaptation, discuss the following scenarios on configuring one or multiple when multiple power offset values configured for CSI measurement and report - Case 1: multiple CSIs corresponding to different power offset values in one report is performed - Case 2: one power offset value is dynamically notified to UE based on single-CSI report - Case 3: other UE report content is included, e.g. maximum power reduction |
| MTK1 | The proposal looks a good starting point. Among Cases 1, 2 and 3, we think Case 1 would be more reasonable to discuss, and the other two cases will induce too frequent BS and UE interactions which may cause concern on increased BS energy overhead. |
| Nokia | We would like to discuss all 3 cases Case 1: single report can be enhanced for multiple CSI hypothesis corresponding to different power offsets Case 2: CSI report for a dynamically indicated power offset. Case 3: CSI report content enhancement |
| LG Electronics | We support the proposal in principal. |
| Panasonic | We are okay with the proposal and intention. The wording from Nokia is also good with us. |

- Others

5.1	Unified/joint framework

A number of companies mention that the enhancements for spatial and power domain can be based on an unified common framework, and/or joint adaptation can be enabled [e.g., ZTE, vivo, CATT, CMCC]. FL considers the following conclusion may be sufficient at this point.

P5-1 Conclusion: Strive for a unified/joint framework for enhancements related to spatial and power domain adaptations.

| Company | Comments |
| --- | --- |
| Lenovo | We see the merit of a unified scheme/mode for spatial and power domain techniques, however, our preference is to discuss this after we have more clarity on the supported techniques |
| Qualcomm1 | It is premature to discuss this proposal. We propose to address our questions in Q2-1 first. |
| Apple | For the unified framework, we think it needs to be stated clearly what are the parts that need to be unified. We think the current design is all under the current CSI Framework which is already unified in some sense. For the joint adaptation, we don’t think it needs to be discussed now since individual adaptation enhancements are not clear yet. After spatial and power domain enhancements are clear respectively, we could consider whether there are still additional enhancements needed for the joint adaptation. |
| MTK1 | We are supportive of this direction and think power offset adaptation can be a simple extension to spatial domain adaptation design, e.g., by including additional power offset candidate value(s) for CSI report(s) based on a CSI-RS configuration. |
| Intel | Given the current signaling structure for power offset and CSI-RS configurations, we think there is merit to consider a joint framework for enhancements for both spatial and power domain adaptation. In fact, CSI reports cannot be computed at the UE without CSI-RS port information nor power offset information, as both information is critical for the UE to compute the CSI. Its not clear to us, if these parameters can be separated from the CSI reporting aspects. So they need to be jointly considered. If from further discussions, RAN1 identifies significant reason to have a separate it framework for spatial and power, then it can be further discussed. |
| Nokia | We agree with Lenovo and Apple’s view. When the enhancements for configuration and reporting are clarified, we can consider to discuss if any further changes would be required for joint adaptation. |
| LG Electronics | Even though aiming at a unified solution for NES techniques in spatial and power domains could be identified as beneficial, we don’t think it can be decided at this stage. Rather, we should first discuss which enhancements are necessary for those techniques. |
| Panasonic | We support this direction or principle. We are also okay to discuss the enhancement individually first and see whether/how to have a unified design. |

- Summary

tbd

Reference

R1-2300065	Spatial and Power Adaptations for Network Energy Savings	 FUTUREWEI

R1-2300073	Power saving techniques in spatial and power domains	Huawei, HiSilicon

R1-2300133	WI Work plan for R18 network energy savings	Rapporteur (Huawei)

R1-2300143	Techniques in spatial and power domains	Nokia, Nokia Shanghai Bell

R1-2300230	Discussion on NES techniques in spatial and power domains	Spreadtrum Communications

R1-2300264	Discussion on techniques in spatial and power domains	OPPO

R1-2300360	Spatial and power domain adaptation for network energy saving	Panasonic

R1-2300372	Discussion on NES techniques in spatial and power domains	ZTE, Sanechips

R1-2300403	Network Energy Saving in Spatial and Power Domain	Google

R1-2300405	Considerations on Network energy savings for NR	KT Corp.

R1-2300465	Discussions on NES techniques in spatial and power domain	vivo

R1-2300587	Discussion on techniques in spatial and power domains	Xiaomi

R1-2300692	Network Energy Saving techniques in spatial and power domain	CATT

R1-2300722	Discussion on NES techniques in spatial and power domains	China Telecom

R1-2300752	Discussion on NW energy saving techniques in spatial and power domains	Fujitsu

R1-2300768	Discussion on network energy saving techniques in spatial and power domains	NEC

R1-2300784	Discussion on techniques in spatial and power domains	InterDigital Communications

R1-2300960	Discussion on NWES techniques in spatial and power domain	Intel Corporation

R1-2301014	Discussion on network energy saving techniques in spatial and power domains	CMCC

R1-2301047	Network energy saving techniques in spatial domain	ETRI

R1-2301107	Discussion on NES techniques in spatial and power domains	LG Electronics

R1-2301163	Spatial Domain Adaptation for NES	Fraunhofer IIS, Fraunhofer HHI

R1-2301206	Network energy saving techniques in spatial and power domains	Lenovo

R1-2301222	Network energy savings techniques in spatial and power domains	AT&T

R1-2301276	Techniques in spatial and power domains	Samsung

R1-2301310	Discussion of NES techniques in spatial domain and power domain	 Transsion Holdings

R1-2301319	Discussion on techniques in spatial and power domains	ITRI

R1-2301358	Discussion on spatial and power domain enhancements to support network energy saving	Apple

R1-2301425	Techniques in spatial and power domains	Qualcomm Incorporated

R1-2301505	Discussion on spatial and power domain enhancements for NW energy savings	NTT DOCOMO, INC.

R1-2301554	NW energy saving techniques in spatial and power domains	 Ericsson

R1-2301599	On NW energy saving techniques in spatial and power domains	 MediaTek Inc.

R1-2301698	Discussion on spatial and power adaptations for network energy saving	CEWiT, Reliance Jio

R1-2301717	Addition of abbreviations and symbols to TR 38.864	Huawei

Appendix

The objectives of the work item are the following: - Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]