3GPP TSG RAN WG1 #112bis-e	R1-2302289

e-Meeting, April 17th - April 26th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Channel Access Mechanism for SL-U

Agenda item:	9.4.1.1

Document for:	Discussion and Decision

# Introduction

The most recent WID on NR sidelink evolution updated in Meeting #99 includes the following objective for operation of sidelink in unlicensed spectrum [1]:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

In this contribution, we continue discussing aspects related channel access mechanism and provide our view also on the issues discussed so far.

# Regulatory requirements and their implications

## Channel Access Procedures

A co-existence mechanism based on energy-detection listen-before-talk (LBT) procedure is required in some regions / bands and, hence, needs to be included in the SL unlicensed concept. Further, beyond regulatory requirements, NR SL-U should operate as a “good neighbor” towards all legacy systems, for example, NR-U, WiFi variants and LTE LAA. Energy detection based LBT procedure is a simple and efficient mechanism for that. As noted in the WID, channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation.

Potential further adjustments for channel access mechanism may be considered during the work item. One difference compared to NR-U is that, while NR-U channel access procedures have been described such that gNB is always either the initiating or the responding device, the same is no longer the case in SL-U. To avoid excessive changes to TS 37.213, we propose to re-use the existing channel access mechanisms as much as possible, such that the sidelink Tx UE follows the DL channel access procedures in 37.213 (i.e. assumes the role of gNB from channel access point of view), and correspondingly, the Rx UE follows the UL channel access procedures. Necessary deviations from this principle can be considered on a per need basis.

Observation 1:	Channel access mechanisms defined for NR-U in TS 37.213 can be largely re-used with SL-U, such that the role of TX UE is similar to gNB, and the role of RX UE is similar to UE.

NR-U specified support for two types of channel access schemes:

-	Dynamic channel access (Load-based Equipment in ETSI EN 301 893)

-	Semi-static channel access (Frame Based Equipment in ETSI EN 301 893)

The two channel access schemes target different deployment scenarios. Dynamic channel access is the more widely deployed option and allows for accessing the channel at any point in time, provided that the channel has been sensed to be vacant according to specific channel access procedures. Semi-static channel access, on the other hand, relies on simpler periodic channel sensing, where during a given fixed frame period there is only one opportunity to access the channel. Due to these fundamental differences, semi-static channel access is usually only deployed in a controlled environment, where interference from other systems is not expected to occur. Considering the wide range of use cases and deployments for SL-U we see that both these channel access mechanisms should also be supported in case of SL-U. However, due to WI time constraints, semi-static channel access should be only supported without specific enhancements for SL channels and resource selection procedure, as well as without changes to channel access procedure defined by NR-U.

Proposal 1: SL-U supports both dynamic (LBE) as well as semi-static (FBE) channel access mechanisms, the latter with minimum specification impact.

### LBT Type 1 aspects and CW size

When a device initiates the communication (i.e. the device takes the role of initiating device), then this device has to acquire the “right” to access the channel for a certain period of time – denoted in the specifications as the Channel Occupancy Time (COT) – by applying an “extended” LBT procedure where the channel must be deemed as free for the entire duration of a Contention Window (CW). This “extended” LBT procedure is commonly known as LBT Type 1 as specified in TS 37.213. This procedure is illustrated in Figure 1.

Figure 1: Acquisition of the COT by an initiating device via LBT Type 1.

The duration of both the COT and CW depends on the Channel Access Priority Class (CAPC) associated with the device’s traffic and is provided in TS 37.213. The device initiating the transmission (the initiating device) upon successfully completing the LBT Type 1 and performing a transmission, acquires the COT with duration associated with the corresponding CAPC.

For Rel-18, RAN1 has been discussing the support of LBT Type 1 for SL-U. In relation to the CAPC definition for Type 1 SL channel access procedure, the following agreement was made in RAN1 Meeting #110bis-e:

Agreement In Type 1 SL channel access procedure, the following table is adopted for channel access priority class (CAPC) for SL. - FFS: the applicability and usage of NOTE1 in the table - FFS: whether mp=1 can be used with p=1, and applicable cases Channel Access Priority Class (p); mp; CWmin,p; CWmax,p; Tslmcot,p; allowed CWp sizes. 1; 2; 3; 7; 2 ms; {3,7}. 2; 2; 7; 15; 4 ms; {7,15}. 3; 3; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. 4; 7; 15; 1023; 6ms [or 10 ms]; {15,31,63,127,255,511,1023}. [NOTE1: Forp=3,4, Tslmcot,p=10ms if the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided, otherwise,Tslmcot,p=6ms.] NOTE 2: When Tslmcot,p=6ms it may be increased to 8ms by inserting one or more gaps. The minimum duration of a gap shall be 100μs. The maximum duration before including any such gap shall be 6ms..

Regarding the applicability of the parameter absenceOfAnyOtherTechnology-r16, in our view, it is beneficial to have this or a similar parameter for SL-U. From RRC specification, “Presence of this field indicates absence on a long term basis (e.g. by level of regulation) of any other technology sharing the carrier; absence of this field indicates the potential presence of any other technology sharing the carrier, as specified in TS 37.213 [48] clauses 4.2.1 and 4.2.3”. This gives flexibility to configure the UEs to transmit higher amount of data before having to acquire a new COT, e.g., by transmitting a larger number of multi-consecutive slots during the longer COT. As can be seen in Figure 2, assuming no other RAT on the channel and absenceOfAnyOtherTechnology is configured, the UPT is improved for different loads and percentiles with Tslmcot=10ms in comparison with Tslmcot=6ms.

Figure 2 – 50%-ile and 5%-ile UPT for different traffic loads per UE, assuming absenceOfAnyOtherTechnology not configured (Tslmcot=6ms) and absenceOfAnyOtherTechnology configured (Tslmcot=10ms). Simulation assumptions in A.1	Appendix.

Observation 2: Simulation results based on the agreed evaluation scenario show that there is a consistent performance improvement for SL-U when absenceOfAnyOtherTechnology parameter is configured.

Proposal 2: The higher layer parameter absenceOfAnyOtherTechnology is supported for SL-U.

Regarding whether mp=1 can be used with p=1, in our understanding this has been proposed with the intention to facilitate the channel acquisition for S-SSB transmissions in order to make it comparable to NR-U SSB. However, is our view that S-SSB can be transmitted based on short control signalling channel access procedure (possibly with a LBT Type 2A), therefore, we don’t see the need of having an exception in the CAPC table for S-SSB transmissions.

Proposal 3: The agreed UL CAPC table is adopted for SL-U without specific exceptions for mp value.

Regarding the Contention Window Size adjustment, in RAN1 Meeting #110-e it was decided to use NR-U DL CW adjustment mechanism as the baseline for SL-U in unicast and FFS any necessary update for SL-U operation. In the DL NR-U CWS adjustment procedure (as defined in TS 37.213 in section 4.1.4), the CWS is increased based on the detection of HARQ NACK (i.e., the intended receiver(s) was unable to decode the TB and the transmitter has to retransmit) and reset based on the detection of HARQ ACK (i.e., the intended receiver(s) was able to decode the TB, therefore no retransmission is needed). This principle of operation is well suited for SL-U when SL-HARQ feedback is enabled. Therefore, regarding FFS any necessary update for SL-U operation from RAN1#109-e, no further enhancement is needed for unicast in our view.

Proposal 4: SL-U adopts NR-U DL CW adjustment mechanism when SL-HARQ feedback is enabled in SCI for unicast, without any further enhancement for SL-U operation.

Further, in RAN1 Meeting #110bis-e, the following agreement was made regarding CW size adjustment and reference duration:

Agreement - RAN1 is to study the definition of a “SL reference duration” following the NR-U principle and RAN1 is to agree on the definition before down-selection to an option for CW adjustment for SL HARQ-ACK feedback enabled/disabled and each cast type - In Type 1 SL channel access procedure, further study the following cases and options. Other options are not precluded. - CW adjustment when SL-HARQ feedback is disabled (at least if all transmissions within the latest SL reference duration have SL-HARQ feedback disabled): - Option 1: For every priority class , use the latest used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class . - Option 2: CW is adjusted according to number blind retransmissions of the TBs within a COT. - Option 3: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U - Option 4: If a is consecutively used times for generation of , is updated for each priority class to the next higher allowed value. - Option 5: If a collision indicator is received, increase for every priority class to the next higher allowed value. - CW adjustment for groupcast option 2 with SL-HARQ feedback enabled (i.e., at least In case only groupcast option 2 PSSCH(s) is (are) transmitted within the latest SL reference duration): - Option 1: Based on a (pre-)configurable ratio of received SL HARQ-ACK feedbacks in the latest SL reference duration, is reset to for every priority class , otherwise increase for every priority class to the next higher allowed value. - FFS: whether the ratio of the received SL HARQ-ACK feedbacks is ‘ACK’, ‘NACK’ or ‘ACK+NACK’ - FFS: how to calculate the ratio - FFS: the (pre-)configuration ratio values - Option 2: If at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class ; otherwise is increased. - FFS whether groupcast option 1 (NACK-only) with SL-HARQ feedback enabled can be supported for SL-U. If supported, further study the following options (at least if all transmissions within the latest SL reference duration are groupcast option 1 transmissions) - Option 1: For every priority class , use the latest used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class . - Option 2: - If ‘NACK’ or a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, increase for every priority class to the next higher allowed value. - When neither ‘NACK’ nor a collision indicator (IUC scheme 2) is received related to any transmissions within the latest SL reference duration, - Option A: is reset to for every priority class . - Option B: For every priority class , use the latest used for any SL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class . - Option 3: An ACK-only procedure is used instead of a NACK-only procedure. In this case, if at least a ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class , otherwise is increased - Option 4: CW is adjusted according to CR/CBR measurement, if CR/CBR is supported for SL-U - Option 5 (option 3+legacy): ACK feedback is performed when a TB is successfully decoded in addition to the legacy NACK-only procedure. In this case, if ACK only is received related to any transmissions within the latest SL reference duration then , otherwise is increased. - CW adjustment for unicast with SL-HARQ feedback enabled (at least In case only unicast PSSCH(s) is (are) transmitted within the latest SL reference duration): - Option 2: If at least one ‘ACK’ is received related to any transmissions within the latest SL reference duration, for each priority class ; otherwise is increased. - FFS the case when UE is operating with different SL-HARQ feedback schemes (e.g., UE has concurrent broadcast transmission + unicast with SL-HARQ enabled, or GC option 1 + GC option 2, etc in the SL reference duration).

And in RAN1#111 it was agreed the following:

Agreement - If , the next higher allowed value for adjusting is . - If the is consecutively used times for generation of , is reset to only for that priority class for which is consecutively used times for generation of . is selected by UE from the set of values {1, 2, …,8} for each priority class .

Regarding the case which HARQ feedback is disabled, assuming DL NR-U CWS adjustment procedure, when a transmission is not associated with HARQ feedback, then the CWS used corresponds to the latest CWS used for any DL transmission procedures associated with the channel access priority of the transmission. In case there is no previous DL transmission with the same channel access priority, then the minimum CWS for that channel access priority is adopted.

Applying this procedure, the SL-U Tx UE would either reuse the latest CWS obtained from other HARQ acknowledged transmissions and, in the absence of these transmissions, the UE would instead use the minimum CWS, i.e. CWp=CWmin,p. However, that procedure may lead to situations where the SL-U UE would keep reusing a large CW unnecessarily. In our view, it should be allowed that the SL-U UE uses other than the latest CWS. The SL-U UE could determine a CWS, for example, based on CBR, e.g., a smaller CWS (than the latest CWS) can be selected if CBR is low.

Proposal 5: SL-U allows using a different CW size other than the latest CWS, when a transmission is not associated with HARQ feedback. CW size can be determined based on , for example, CBR (Option 3 of CW adjustment when SL-HARQ feedback is disabled).

Regarding the CW adjustment for groupcast option 1 (NACK-only) and groupcast option 2, there should be no special difference with regard to CW size adjustment. I.e., as long the Tx UE retransmits then the CWS should be increased, independent of how many ACKs or NACKs it has received. Note that, according to regulations [EN 301 893] the UE sets CWmin,p if the UE does not retransmit.

Proposal 6: In both groupcast option 1 (NACK-only) and groupcast option 2, the SL-U UE increases the CW size if UE retransmits (regardless of how many ACK/NACKs it has received), otherwise it shall set it to CWmin (aligned with ETSI regulations).

In a further point, when there is no HARQ feedback but where there are blind retransmissions it is not clear if there is a need or not to increase the CW size. We note that the ETSI regulations are not clear on this aspect.

Proposal 7: In case of blind retransmissions without HARQ feedback across different COTs, RAN1 to study if CW size should be increased.

In relation to the definition of reference duration, the following was agreed in RAN1#112:

Agreement The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows: - Option 1a - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed

Regarding whether to support another ending timing for MCSt, in our view there is no need of a separated definition as this could also be specified like in NR-U in terms of transmission burst. For example, “the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted, or until the end of the first transmission burst by the UE that contains PSSCH with ACK/NACK HARQ-ACK enabled, whichever occurs earlier”

Proposal 8: Regarding whether to support another ending timing, the definition can be made like in NR-U specs, e.g., “the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted, or until the end of the first transmission burst by the UE that contains PSSCH with ACK/NACK HARQ-ACK enabled, whichever occurs earlier”.

### Channel access for S-SSB

In RAN1 Meeting #110bis-e, the following agreement was made:

Agreement - Type 1 SL channel access procedure is applicable to the following transmissions by a UE: - PSSCH/PSCCH transmission(s) scheduled or configured by a gNB in SL Mode 1 resource allocation. - PSSCH/PSCCH transmission(s) from the UE in SL Mode 2 resource allocation. - Other SL transmissions including S-SSB and PSFCH transmissions from a UE - FFS: how to set CAPC for S-SSB and PSFCH - Note: Type 1 can be used to initiate a COT - A UE uses a channel access priority class applicable to the sidelink user plane data multiplexed in PSSCH for performing the Type 1 channel access procedures to transmit transmission(s) including PSSCH with user plane data and its associated PSCCH. - Note: how to set CAPC for MAC CE multiplexed in PSSCH is up to RAN2 - A UE shall not transmit on a channel for a Channel Occupancy Time that exceeds the maximum COT duration where the channel access procedures are performed based on a channel access priority class p associated with the UE transmissions, as given in CAPC table for SL.

In RAN1#111, it was also agreed the following related to applicability of Type 2A.

Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure

For the transmission of the S-SSB, some options which could be considered include:

1)	allowing transmission as Short Control Signalling transmission (SCSt) (including Type 2A LBT);

2)	applying Type 2 LBT in case of shared COT; or

3)	applying Type 1 LBT.

For option 1), NR-U and LTE-LAA partially make use of this SCSt allowance to transmit discovery bursts / SSBs, but to ensure fair co-existence in all cases, they still apply LBT Type 2A (25 us LBT). For SL-U, it was agreed in RAN1#111 that Type 2A channel access procedure will also be supported for S-SSB, as mentioned above:

Regarding details of EDT, the same principles as NR-U should apply, i.e., EDT is determined based on e.g. maximum transmit power and TX bandwidth.

Proposal 9: Same principles for EDT as NR-U can be applied for S-SSB transmissions with Type 2A LBT, i.e., based on maximum transmit power and TX bandwidth.

In option 2), the applicability depends on how the COT sharing is performed and if the conditions to allow a UE taking the role of a responding device also include the case where that device would utilize the shared COT to do a S-SSB transmission. As it has been agreed in RAN1#111, when performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. Therefore it can apply Type 2A/2B/2C channel access (depending on gap) within any shared COT. Further discussion on COT sharing consideration is performed in section 2.3.

Proposal 10: S-SSB can be transmitted with Type2A/2B/2C (depending on gap from previous transmission) if there is a shared COT from any UE.

Finally, in cases where option 3) is applied (e.g. if S-SSB transmissions do not meet the criteria for short control signaling or there is no available COT), it should be noted that the following agreement was reached in RAN1#112 in relation to CAPC to be used for S-SSB:

Agreement The CAPC level that should be used for S-SSB transmissions: - Option 1: CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for S-SSB transmission

In our understanding, this agreement applies regardless of whether the S-SSB is the legacy S-SSB or is an additional candidate S-SSB occasion (if the latter is supported).

Proposal 11: RAN1 to confirm that additional S-SSB (if supported) is also associated to CAPC with p=1.

### Channel access for PSFCH

The transmission of PSFCH may be done using Short Control Signalling Transmissions (SCSt) or by applying LBT procedure (Type 1 or Type 2 when COT has been acquired) prior to the transmission to determine whether the medium is idle or not. Regarding the RAN1#110bis-e FFS, for the transmission of PSFCH, these options could be considered:

1)	allowing transmission as Short Control Signal (with/without a Type 2A LBT);

2)	applying Type 2 LBT for transmissions within a shared COT;

3)	applying Type 1 LBT in case there is no initiated/shared COT for transmitting PSFCH.

For option 1), we note that the advantage of using SCSt without LBT Type 2A for PSFCH transmission is that there is no possibility of LBT failure that prevents the transmission of the feedback and therefore there is no need for multiple PSFCH opportunities to provide robustness in case of LBT failure. However, this may not be seen as coexistence friendly even if SCSt duty cycle is followed according to regulation. Another option is to use SCSt that is preceded by an LBT Type 2A, In this case there is a possibility for LBT failure, however this is expected to be less likely than when LBT Type 1 is applied (in the absence of shared COT).

Noting that LBT Type 2A is supported for S-SSB under the constraints of transmission duration and duty cycle, in the case LBT Type 2A is also applied for PSFCH, the UE implementation just needs to keep track of how much of the duty cycle is being used for both channels transmissions without a shared COT. Based on periodicity of S-SSB and PSFCH configuration, as well as the number of times that UE has applied Type2A for S-SSB and PSFCH, the UE should be able to determine if it complies with the total duration budget for SCSt within an observation window. If it cannot meet the budget, then a PSFCH transmission without a shared COT should fall back to Type 1 LBT. The constraint on transmit time duration of 1ms can be easily satisfied by PSFCH (at it takes at most 214us assuming a CPE is added).

Proposal 12: RAN1 to support the use of SCSt with LBT Type 2A for PSFCH transmissions. The total duration of S-SSB and PSFCH transmissions by a UE using SCSt with Type 2A LBT is at most 2.5 ms within a 50 ms observation period.

Regarding CAPC for PSFCH, in RAN1#112, the following was agreed:

Agreement The CAPC level that should be used for PSFCH transmission, CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for PSFCH transmission

Since PSFCH is associated with CAPC with p=1, this allows PSFCH to be transmitted even within a COT initiated with CAPC of p>1, i.e., PSFCH can be transmitted in any shared COT at least when there is PSFCH for the COT initiator UE.

In addition, the agreement does not distinguish whether PSFCH transmission is an additional PSFCH or not. So based on the agreement any PSFCH can be associated with CAPC with p=1.

Proposal 13: RAN1 to confirm that additional PSFCH(s) (if supported) are also associated with CAPC with p=1.

In all cases where LBT procedure is applied for PSFCH (Type 1 for acquiring COT, Type 2 with shared COT, or SCSt with Type2A), there is the need to make HARQ feedback process more robust against LBT failures without large impact on PSFCH capacity and still comply with the OCB and PSD requirements. For such purpose, the UE may use the interlaced RBs to, not only providing HARQ feedback for the corresponding PSSCH, but also as additional opportunities for PSFCH of prior PSSCHs. That is further discussed in our companion paper under AI 9.4.1.2 [2].

## CP extension

For a UE to apply Type 2A/2B/2C LBT within a COT, it should comply with transmission gaps of 25, 16, and <16 us respectively. However, the gap durations of 16 and 25 us do not exactly match with the NR symbol durations. For this reason, in NR-U CP extension is used prior to certain UL transmissions to reduce the duration of the gap to exactly 16 or 25 us, such that COT sharing becomes practical.

In RAN1#109-e it was agreed that CP extension is supported also in SL-U. And in RAN1#111, CPe was further discussed, and the following agreements were reached:

Agreement - A CPE is transmitted from a CPE starting position before SL transmission within a COT, select one or both of the two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: within at most 1, 2 or 4 symbols just before the next AGC symbol for 15, 30 or 60 kHz SCS, respectively - FFS: whether Option 1 and Option 2 are both applicable and the conditions (e.g., Option 1 in case of COT sharing and Option 2 in case of initiating a COT) - FFS: which channel access type(s) is applicable for option 1 and option 2 - FFS: other details - A single CPE starting position for PSFCH - FFS CPE starting position and whether it should be (pre-)configured in each RP, pre-defined or indicated - FFS other details (e.g., indication granularity) - Note: value 0 is a candidate - At least one CPE starting position for S-SSB - FFS CPE starting position should be (pre-)configured, pre-defined or indicated - FFS: Whether multiple CPE starting positions should be (pre-)configured, pre-defined or indicated - FFS CPE starting positions for the R16 S-SSB and the additional S-SSBs - Note: value 0 is a candidate - One or multiple CPE starting positions can be (pre-)configured in each resource pool for PSSCH/PSCCH - When multiple CPE starting positions are (pre-)configured, - FFS whether/how to define a criteria for selecting a default CPE starting position (e.g., according to partial/full RB set allocation, resource reservation information, within or outside of a COT, etc.) - FFS criteria for selecting one of the multiple CPE starting positions (e.g., according to priority level (e.g., CAPC or L1), selected randomly by UE from the (pre-)configured set of CPEs, selected by the UE based on channel access result, determined based on indication from the COT initiating UE, etc.) - FFS other details

In RAN1#112, the following was agreed for CP extension starting position:

Agreement A CPE can be transmitted from a CPE starting position before SL transmission for the following two options: - Option 1: within the symbol just before the next AGC symbol - Option 2: - within the symbol just before the next AGC symbol for 15 kHz SCS - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS - FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2

### CP extension for PSSCH

Regarding the CPE starting position before SL transmission within a COT, it is reasonable that a CPE should start within 1 symbol before the next AGC symbol for any subcarrier spacing. RAN1 should then further discuss if other CPE starting positions are relevant e.g. in case of SL configured grant or semi-persistent transmissions, in light that longer CPEs are supported for NR-U CG.

It should be noted that NR-U supports the longer CPE for CG-PUSCH with the intention of having multiple possible starting points for avoiding collisions between multiple UEs transmitting CG PUSCH on a same resource. However, in SL there are pre-emption and reselection mechanisms that can be used to avoid collisions. So, the support for Option 2 should be only considered if RAN1 assumes that SL CG transmissions or SL semi persistent transmissions from different UEs may occur on same PSSCH resources.

Proposal 14: Support CPE starting position within one symbol before the AGC for any subcarrier spacing (Option 1). Option 2 should be only considered if RAN1 assumes that SL CG transmissions or SL semi persistent transmissions from different UEs may occur on same PSSCH resources.

In addition, we note that NR-U supports different CPE starting points configuration for full and partial RB-set allocation based on CG-StartingOffsets for CG PUSCH. Similarly, if multiple CPEs are supported for PSCCH/PSSCH, it should be up to configuration/implementation which of the CPE options should be applied for partial or full RB set allocation. In our view, even with partial allocation, there are some situations in which the UE can apply a longer CPE, e.g., to prevent other transmissions to be initiated in neighboring interlaces that may cause IBE issues over its transmission.

Proposal 15: If multiple CPE are supported for PSCCH/PSSCH, it should be up to configuration/implementation which of the CPE lengths should be used for partial or full RB set allocation.

### CP extension for PSFCH

For PSFCH a single preconfigured or predefined CPE starting position can be specified so that PSFCH can be transmitted with a type 2C LBT, at least when PSFCH is within a COT. Note that even if PSFCH transmission is not in a shared COT, the same CPE should be used in order to enable FDMed PSFCH transmissions from multiple UEs accessing the channel simultaneously.

Proposal 16: Support a single preconfigured or predefined CPE for PSFCH.

The allowed CPE length for PSFCH will depend on the decisions on UE-to-UE COT sharing principles and on whether SCSt channel access with Type 2A can be used for PSFCH. If agreed that PSFCH can be transmitted to any UE during a shared COT, not necessarily including the COT initiating device, so the CPE length should be defined for allowing a gap of 16us in order to enable a Type 2C for any UE transmitting PSFCH within the shared COT. While if RAN1 decides that PSFCH can use SCSt with Type 2A, then the CPE should be designed for a of 25us, so that all UEs transmitting PSFCH with or without COT can perform channel access simultaneously.

Proposal 17: The allowed CPE starting position for PSFCH should be discussed after deciding whether PSFCH can be transmitted to any UE during a shared COT and after deciding if PSFCH can use SCSt with Type 2A.

### CP extension for S-SSB

For S-SSB, at least for the legacy version, it can be useful to configure multiple CPE starting positions. Note that S-SSB is not FDMed with other transmissions since the legacy S-SSB occasion is not part of the resource pool (additional S-SSB is not yet agreed whether should be part or not of the RP). By allowing multiple CPE starting positions for S-SSB, it is possible to configure that a first sync ref UE provides S-SSB of a certain sync priority while other potential sync ref UEs can be used as backup synchronization sources. With the first sync ref UE using the longest CPE and remainder UEs attempt S-SSB transmission with a shorter CPE, the remainder UEs serve as backup sync ref in case S-SSB from primary sync ref source is not present (e.g. due to LBT failure).

Proposal 18: Support multiple CPE lengths to be (pre)configured for S-SSB.

## COT sharing principles

In NR-U the initiating device can share its acquired COT with its intended receiver (the responding device). For this purpose, the initiating device shall inform (e.g. via control signaling) the responding device about the duration of this COT. The responding device uses then this information to decide which type of LBT it should apply upon performing a transmission for which the intended receiver is the initiating device. In case the responding device transmission falls outside of the COT, then the responding device will have to acquire a new COT using the LBT Type 1 with the appropriate CAPC.

The NR-U COT sharing principles can be applied to SL-U. However, it is not clear in SL-U how restrictive should the relation between initiating device and responding device be. For example, an initiating device when sharing the COT is expected to do it so via control signaling, which in SL can be conveyed in the 1st stage, 2nd stage SCIs and/or MAC CE. However, any device in the proximity of the initiating device will be able to receive and decode the 1st stage, 2nd stage SCIs and MAC CE of the initiating device transmission even if it is not the intended receiver; and therefore, the COT sharing information is available to any of these devices.

Observation 3: Any SL device can receive and decode content all the way to the MAC CE level, therefore the COT sharing information would be available to any device in proximity of the COT initiating device.

Proposal 19: RAN1 to support the signaling of COT sharing in SL. FFS how the COT sharing information is transmitted (e.g. 1st and/or 2nd stage SCI and/or MAC CE).

In RAN1 Meeting #109-e, the following agreement was made (numbering added to facilitate the discussion) in agenda item 9.4.1.1 regarding COT sharing:

Agreement  (1) UE-to-UE COT sharing is supported in NR sidelink operation in a shared channel (SL-U). o (a) FFS applicable SL channels and signals (e.g., PSCCH/PSSCH, PSFCH, S-SSB) for shared COT access and any restrictions (e.g. whether the COT can be shared with a single UE or multiple UEs) o (b) FFS all other details in compliance with the regulatory requirements

Regarding RAN1#109-e FFS (1).a when having the COT sharing information available, a device can potentially take the role of responding device and attempt to access the channel in the following cases:

a.	To perform a unicast transmission towards the initiating device;

b.	To perform a groupcast transmission towards the initiating device and any other device in the responding device proximity that it is part of the groupcast group;

c.	To perform a broadcast transmission towards any device in the responding device proximity (including the initiating device);

d.	To perform unicast transmission towards another device other than the initiating device;

e.	To perform a groupcast transmission towards any device in the proximity of the responding device other than the initiating device.

From all these cases, only case a) is similar to NR-U and therefore clear. However, if the condition to decode at least partially the information about a COT is enough to become a responding device, then all the cases above would be valid. Then in RAN1 Meeting #110-e, the following agreement was made regarding COT sharing:

Agreement For UE-to-UE COT sharing, - When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT.When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator - When performing PSSCH/PSCCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when the responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to the shared COT is intended for the COT initiating UE - FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information. - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE. - FFS whether the responding UE can utilize the COT when at least the responding UE’s PSCCH transmission in the reserved resources within the shared COT or MCSt is intended for the COT initiating UE and what are the restrictions (e.g., priority, etc.) and indication to the responding UE. - FFS: UE forwarding/relaying information about a COT initiated by another UE.

For PSFCH transmission, our understanding is that the responding UE can transmit PSFCH(s) to UE(s) other than the initiator within the RB sets corresponding to the shared COT, at least when one PSFCH is intended to COT initiating UE.

Proposal 20: The responding UE can transmit PSFCH(s) to UE(s) other than the initiator, at least when one or more PSFCH(s) in the slot is intended to a COT initiating UE.

From previous agreement, PSCCH/PSSCH is not precluded as the channel type of the responding UE’s transmission. However, as in NR-U, it should be ensured that COT propagation does not take place to ensure regulatory compliance and fair coexistence, i.e., responding device should directly receive COT information from initiating devices. Therefore the support for UE forwarding/relaying information about a COT initiated by another UE does not justify.

At least PSCCH/PSSCH towards the COT initiator should be allowed with the same cast type. In our view, each PSSCH transmission of the responding device should at least have the COT initiator device as a destination.

Proposal 21: Regarding PSCCH/PSSCH transmission in a shared COT, each PSCCH/PSSCH transmission of the responding device should at least have the COT initiator device as a destination (based on cast type and Destination ID).

Proposal 22: RAN1 does not support UE forwarding/relaying information about a COT initiated by another UE.

Further regarding COT sharing, the following agreements were reached in RAN1#112:

Agreement - A responding UE over a shared COT can be: - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator - FFS Limitations on what additional IDs may be included and how they may be indicated Agreement A responding UE’s SL transmission(s) within RB set(s) corresponding to a shared COT can be transmitted when the CAPC value(s) of the SL transmission(s) have an equal or smaller CAPC value than the CAPC value indicated in the COT sharing information. Agreement A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when, - In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported) - In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported) FFS: all other details and additional restrictions

From the above agreements it is not clear what is the benefit of supporting additional IDs in the COT sharing information. In case additional IDs are added for identifying UEs which can use the COT, but do not have a transmission response towards the COT initiator, that should not be supported as it goes against the definition of “a responding UE” in regulations. And if additional IDs are only for restricting who can use a COT, such restriction can be achieved by CAPC already, i.e., only UEs with same or lower CAPC value can use the COT. Otherwise, there is no clear benefit of restricting a UE with higher priority transmission to use the COT.

Proposal 23: Additional ID(s) in the COT sharing information is not supported.

# 3	Wideband Operation for SL-U

Regarding multi-channel access for SL-U, the following agreement was made in RAN1 Meeting #110bis-e:

Agreement For dynamic channel access mode with multi-channel case in SL-U, NR-U UL channel access procedure is considered as baseline for transmission on multiple channels - FFS: whether transmission of PSFCH and/or S-SSB on a subset of RB sets is supported (using the NR-U DL channel access procedure as baseline) - FFS any necessary enhancement and modification for the SL-U operation

And in RAN1#111 it was agreed the following:

Agreement For dynamic channel access mode with multi-channel case in SL-U, use NR-U DL (Type A or Type B) multi-channel access procedure as the baseline for multiple PSFCH transmissions on multiple channels, where each PSFCH transmission is confined within one LBT channel - FFS: the case for S-SSB if agreed to transmit S-SSB (or S-SSB can be (pre-)configured) in more than one RB set - FFS: whether type A or type B or both will be supported for this case for PSFCH - FFS: whether multiple PSFCH transmissions on multiple channels after performing the multi-channel access procedure is limited to contiguous RB sets

## Wideband Operation for PSFCH

PSFCH is a control channel critical for resource efficiency of sidelink communication. If a PSFCH occupying a PRB is missed, retransmission of the corresponding PSSCH which occupies much larger resource size (usually tens of PRBs) is needed. This is clearly not resource efficient, and also increases transmission latency of PSSCHs. Different from PSSCH, preparation of a PSFCH for different outcomes of LBTs on multi-channels almost brings no increase of processing complexity. A UE just prepares the PSFCH and transmits it in the channel(s) on which the LBT(s) pass. Thus support of transmission of PSFCH on a subset of RB sets (using the NR-U DL channel access procedure as baseline) is necessary. In our view, both Type A and Type B multi-channel access can be supported, but at least Type A should be specified as it provides more reliability for PSFCH transmission since the UE should be able to provide PSFCH at least on part of the channels where LBT finishes successfully. Regarding whether to limit to contiguous RB sets, that may be up to RAN4.

Proposal 24: RAN1 to support transmission of PSFCH on a subset of RB sets (using the NR-U DL channel access procedure as baseline). Both or at least Type A multi-channel access can be supported for PSFCH.

## Wideband Operation for S-SSB

Regarding the support for S-SSB in more than one RB set, in our view the motivation for such feature is not very clear. From previous discussions, the intention is to cope with UEs which do not have capability to operate in multiple RB-sets, therefore the redundant S-SSBs could be provided in multiple RB-sets. However, NR-U design does not seem to have introduced SSB in more than one RB set provided by gNB, meaning that the has not been considered relevant. In addition, in the current SL design, the S-SSB position is related to the SL BWP configuration, which means that SL UEs configured with a certain BWP should be able to monitor S-SSBs within it. Thus, in our view, optimizations for the support of S-SSB in more than one RB-set should not be in the scope, as UEs should be able to monitor the whole configured SL BWP searching for S-SSB in one of the RB-sets.

Proposal 25: RAN1 does not support for a UE to transmit S-SSB in more than one RB set in a slot.

# 4	SL resource allocation

SL transmissions are allocated over a set of time-frequency resources defined as a Resource Pool. The resource pool is preconfigured within a SL BWP. The SL BWP configuration includes which symbols of the slot can be used for SL communication, meaning that a NR slot may contain gaps where SL transmission cannot be present. Also, the resource pool can have contiguous or non-contiguous resources in time domain. In frequency, the resources are defined in sub-channels formed by contiguous PRBs. SL design includes the following mechanisms for allocating the resources from a resource pool.

	Mode 1

o	In this mode, the gNB, triggered e.g. by a SL SR or SL BSR from a UE, provides a dynamic grant for SL transmissions through DCI format 3_0 for the UEs which are in RRC_CONNECTED state. The grant can allocate resources for up to 3 transmissions of a same transport block.

o	Alternatively, one or multiple configured grant configurations of periodic resources are provided via RRC. Both configured grant type 1 (fully RRC configured and activated) and type 2 (activated via DCI) are supported.

	Mode 2:

o	In this mode, the UE selects the resources for a preconfigured maximum number of transmissions (blind or HARQ based retransmissions) of a same transport block

o	UE can also reserve resources for future transport blocks by indicating a resource reservation period indicated in the 1st stage SCI.

o	The UE should reserve the resources within a selection window which the interval is limited by the remaining packet delay budget.

o	The UE typically follows a sensing-based approach for selecting the resource, i.e., it first identifies the candidate resources in the selection window (by excluding resources indicated in a received SCI and which the SL-RSRP are higher than a threshold) and then selects randomly from the set of candidate resources.

SL-U should be supported for in-coverage and out-of-coverage UEs, therefore both mode 1 and mode 2 should be supported. As per WI scope, Uu operation for mode 1 is limited to licensed spectrum only, therefore SL SR, DCI transmission or RRC resource configuration may not be affected by unlicensed channel access. While control and data channels (SL SSB, PSCCH, PSSCH, PSFCH, etc) when transmitted on unlicensed spectrum should of course follow unlicensed channel access procedures either in mode 1 or mode 2.

In RAN1 Meeting #109-e, the following agreement was made (numbering added to facilitate the discussion) in agenda item 9.4.1.1 regarding resource allocation:

Agreement  (1) The existing sidelink mode 1 RA including dynamic grant, Type 1 and Type 2 configured grants are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the allocated resource(s), in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. o (a) FFS whether/how mode 1 resource allocation selection procedure needs to be updated / enhanced due to shared spectrum channel access  (2) The existing sidelink mode 2 RA schemes are supported as a baseline for sidelink operation in a shared carrier, subject to applicable regional regulations. At least in dynamic channel access, SL UE performs Type 1 or one of the Type 2 LBTs before SL transmission using the selected and/or reserved resources, in compliance with transmission gap and LBT sensing idle time requirements specified in TS37.213. o (a) FFS whether/how mode 2 resource selection procedure needs to be updated / enhanced due to shared spectrum channel access  (3) FFS whether/how multi-consecutive slots transmission can be supported for NR sidelink operation in unlicensed spectrum, including the following aspects o (a) channel access, resource allocation and PHY channel design  (4) FFS whether/how enhancement is needed between the end of the LBT procedure and the start of the SL transmission to retain channel access RAN1 to strive for a common solution for channel access for Mode 1 and Mode 2

Regarding FFSes (1).a and (2).a, the LBT type of channel access procedures as adopted in NR-U are designed for collision avoidance and fair coexistence with other RATs which use asynchronous channel access. However, the Mode 2 sensing-based procedures are designed for coordinating the resource allocation in a distributed way among SL UEs transmitting in a frame-based structure. This means that these mechanisms may not fully replace each other, on the contrary, they should operate together and complement each other. However, issues may arise if existing SL resource allocation procedure does not consider the channel access mechanism in place. Figure 3 shows an example where a SL-U UE may potentially select a candidate resource prior to a previously reserved resource indicated in a received SCI from another SL-U UE. If transmitting symbols of a selected candidate resource overlap with the LBT interval for transmission in the reserved resource, it may cause LBT failure to the other UE and potentially loss of a high priority packet.

Figure 3 - Example of a case where selection of a certain candidate resource will lead to a channel access failure for transmission in a reserved resource from another UE.

Observation 4: The selection of a candidate resource preceding a reserved resource, e.g., of another UE, may cause LBT failure for the transmission on the reserved resource.

Another example is shown in Figure 4, in which a UE has a candidate resource after a previously reserved resource indicated in a received SCI from another SL-U UE. In that case, if the LBT for the selected candidate resource overlaps with transmitting symbols of the reserved resource, it risks an LBT failures.

Figure 4 - Example of a case where selection of a certain candidate resource will lead to a channel access failure as it does not take into account the LBT interval overlapping with a reserved resource of another UE.

Observation 5: In mode 1 SL resource allocation, the gNB scheduler implementation should be able to prevent the occurrence of LBT blocking between connected SL devices.

Observation 6: In mode 2 SL resource allocation, the selection of a candidate resource after a reserved resource, e.g., of another UE, may cause LBT failure for the transmission on the candidate resource.

Note that in both example the LBT failure related to the transmission of other SL-U UE can be avoided. In the case of Mode 1, if the gNB schedules a contiguous resource for SL transmission prior or after a reserved resource, it may lead to LBT failure for the transmission in reserved resource or for the scheduled resource if transmitting symbols overlap with LBT of each other. However, in mode 1 it should be feasible for the scheduler implementation to avoid such situation. In case of Mode 2, the SL-U UE when performing sensing can acquire the knowledge about the reserved resources based on received SCIs.

In case multi-consecutive slots transmissions are supported in SL-U, as discussed in FFS (3), we note that the same issue described above is present. In other words, in case the LBT of any candidate consecutive resource overlaps with a reserved resource or in case LBT of reserved resource overlap with any of the candidate consecutive resources, the LBT failure due to the presence of a reserved resource can be avoided by making the resource allocation procedures (i.e. both Mode 1 and 2) become aware to this problem.

Therefore, in our view, RAN1 should investigate the interaction between channel access procedures and resource allocation mode 1 and mode 2 in order to avoid resource allocation which may cause LBT failures. It should be discussed how to modify the candidate resource selection mechanism to prevent avoidable LBT issues between SL-U UEs, for example, by excluding candidate resources which (i) are subject to LBT failure caused by a reserved resource transmission or (ii) may cause LBT failure to a reserved resource transmission.

Proposal 26: Resource allocation procedure should avoid selection of a candidate resource before a reserved resource in case the transmitting symbols of candidate resource overlap with LBT of the reserved resource.

Proposal 27: Resource allocation procedure should avoid selection of a candidate resource after a reserved resource in case the transmitting symbols of the reserved resource overlap with LBT of candidate resource.

In RAN1#112 the feature lead listed the following options for discussion:

Proposal 6 (I): - To resolve the Type 1 LBT blocking issue, where one UE performing a Type 1 LBT procedure for using its own selected/reserved resource(s) is blocked by another UE’s SL transmission at least in a slot preceding to the selected/reserved resource and causing the LBT to fail. Further study at least the following solution options for down-selection in a future meeting. Other options are not precluded. - Option 1: - UE avoid selection of a resource before a reserved resource when the transmitting symbols of the selected resource overlap with Type 1 LBT of the reserved resource. - UE avoid selection of a resource after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. - FFS: the avoidance should be performed by L1 exclusion or L2 MAC selection - FFS: whether / how to achieve this in RA mode 1 - Option 2: - UE prioritizes/selects resource(s) for transmission in slot(s) after a reserved resource when transmission of the selected resource is able to share the initiated COT of the reserved resource (i.e., the selected resource(s) is within the COT duration of the reserved resource and the CAPC value of the selected resource(s) is equal to or higher than that of the reserved resource). - UE prioritizes/selects resource(s) for transmission in slot(s) before a reserved resource when transmission of the selected resource is able to share its initiated COT with the reserved resource (i.e., the reserved resource is within the CAT duration of the selected resource(s) and the CAPC value of the selected resource(s) is equal to or smaller than that of the reserved resource). - FFS whether / how to achieve this in RA mode 1. - Option 3: UE selects extra / more resources than required for transmitting a TB (i.e., overbooking) to accommodate potential Type 1 LBT failures. - Option 4: LBT duration is determined firstly, then resource selection takes into account of the LBT duration is performed. - Option 5: At MAC layer, selection of resource(s) among the reported set of candidate resources from L1 is up to UE implementation in mode 2 for SL-U, instead of random selection.

As was discussed in this section, Option 1 should be preferable, as well as Option 2 which can also be supported together. Option 3 should not be supported as it arises resource efficiency issues while it is not clear what benefits it has compared to existing repetition and retransmission mechanisms. Option 4 should be up to implementation if supported, and finally, Option 5 alone may not be sufficient to avoid inter-UE blocking because MAC layer does not know of other UEs reservations.

Proposal 28: Down select to option 1 and option 2 from Proposal 6 (I) of the FL summary for AI 9.4.1.1.

Also about Mode 2 resource allocation, it was discussed in RAN1#112 whether to address the potential issue of insufficient time for a UE to perform Type 1 LBT before a selected resource, as LBT sensing time can be longer than T1 of resource selection window. In our view, there is no straightforward solution to handle that issue, since the exact LBT sensing time cannot be known in advance, i.e., only an estimated duration can be determined a priori based on CW, while the resultant duration will depend on the randomly selected counter within CW and on the deferrals if a CCA slot is deemed as busy. In our view, this issue should be addressed by implementation together with a resource re-selection if a UE deem that Type 1 LBT cannot succeed before a selected resource.

Proposal 29: In case a Type 1 LBT sensing time gets longer than the time until the selected resource, a resource re-selection can be triggered.

# 5	Multi-consecutive slots transmissions (MCSt)

In NR-U, scheduling of multiple consecutive PUSCH allocations with a single UL grant were specified to improve the COT usage by the UE, as well as to reduce PDCCH overhead for allocating the UE in the shared channel. Also, a burst of consecutive allocations can be configured via RRC for CG PUSCH transmissions, which allows the UE to reduce the amount of LBT attempts needed to transmit the data over multiple slots.

In both cases, dynamic and configured grants, the UE can transmit different TBs on the consecutive PUSCH allocations, which allows to deliver more data in an available COT. If k repetitions are enabled, the UE can also attempt transmitting the same TB in the contiguous slots, which provides multiple starting occasions, i.e., if LBT failed for transmitting in the first PUSCH, the UE still has the chance to deliver the TB if LBT succeeds in the following PUSCH occasion. In NR-U design, the time domain resources are indicated with a row of a preconfigured set of multiple SLIV allocations, while in frequency domain, the same PRB allocation is used for all PUSCHs.

For SL-U, we should follow the principles of NR-U, i.e., multiple consecutive allocations for a UE should be supported.

In RAN1 Meeting #110-e, the following agreement was made in agenda item 9.4.1.1 regarding multi-consecutive slot transmissions:

Agreement Multi-consecutive slots transmission (MCSt) is supported for Mode 1 and Mode 2 resource allocation in SL-U. - FFS details

For mode 1, the gNB should be able to dynamically decide whether to provide multiple scheduling grants for a number of consecutive allocations, e.g., based on amount of data the UE needs to transmit informed on the SL BSR.

Proposal 30: For mode 1, it is up to gNB how to schedule the multiple consecutive allocations to a SL-U UE.

For mode 2, in RAN1 Meeting #110bis-e, the following agreement regarding MCSt operation was achieved:

Agreement On the support of MCSt operation in SL-U, following options are to be further studied and one or more of the following options will be selected in future meetings. - When L1 is triggered for reporting a subset of candidate resources for MCSt, - Option 1: Only one set of parameters (, remaining PDB, and ) is provided for the resource selection procedure in L1 - Note, this is applicable for transmission of a single TB and multiple TBs - FFS: whether this is the same or different than Rel-16 - Option 2: one or multiple sets of parameters (, remaining PDB, and ) are provided for the resource selection procedure in L1 - FFS: any further information needs to be provided to L1 for MCSt - When L1 reports a subset of candidate resources for MCSt, - Option A: L1 reports candidate multi-slot resources in SA where a candidate multi-slot resource consists of a set of single-slot resources that are consecutive in time - FFS whether the set of single-slot resources within a candidate multi-slot resource can have different sizes - Option B: L1 reports candidate single-slot resources in (SA) as in Rel-16 - It is up to the higher (MAC) layer to select a set of single-slot resources that are consecutive in logical slots - Option C: L1 reports consecutive single-slot candidate resources in SA - FFS whether the consecutive single-slot candidate resources can have different sizes - FFS: any further information needs to be reported to MAC layer, provided to L1 or utilized for MCSt - FFS: whether/how to consider the additional LBT time in SL resource allocation

Regarding when L1 is triggered for reporting a subset of candidate resources for MCSt, two options are listed for the higher layer to provide the parameters for enabling L1 to select the candidate resources.

In our understanding, Option 1 does not give information for the L1 to determine a number of consecutive slots. So with this option, either MAC should trigger the procedure multiple times until it can obtain a number of consecutive slots from the candidate resource provided by L1, this behavior would be compatible only with Option B for L1 reporting. Or, at least a parameter to indicate the number of MCSt slots would have to be provided to L1.Also Option 2 does not explicitly indicate the number of multi-consecutive slots. Therefore, if it is left for L1 to provide multi-slot candidate resources, the number of consecutive slots should either be provided explicitly or determined based on the number of sets provided.

Still, for both options, it is unclear whether L1 should be in charge of providing candidate resources with a number of consecutive slots. Regardless of whether one set of parameters is provided multiple times, or multiple sets of parameters are provided to L1, in case the selection window is larger than 1, the candidate resources reported by L1 may be spread over time and acquiring consecutive slots cannot be guaranteed.

Observation 7: Regarding when L1 is triggered for reporting a subset of candidate resources for MCSt, both Option 1 or Option 2 are unclear how L1 determines the number of consecutive slots.

Proposal 31: When L1 is triggered for reporting a subset of candidate resources for MCSt, in case L1 should report candidate multi-slot resources (or consecutive single-slot candidate resources), RAN1 should consider another information for L1 to know the number of consecutive slots. Otherwise, the acquisition of candidate resources in consecutive slots should be based on implementation.

Regarding when L1 reports a subset of candidate resources for MCSt, three options are listed for discussion.

Option A and Option C are similar in our view, as both imply that L1 should report candidate resources with a number of consecutive slots to MAC. The consequence of these options is potential change on the candidate resource selection procedure to ensure that consecutive slots resources are provided.

Option B follows Rel-16 procedure and leaves it up to MAC to select consecutive slots for MCSt. In this case, if it’s not L1 to guarantee consecutive slots resources are provided to MAC, then L1 resource selection procedure may have no impact. However, the MAC procedure, which is currently based on selecting randomly from the candidate resources reported by L1, should be modified to select consecutive slots resources.

As an alternatively solution, the consecutive resources could be ensured by implementation, e.g., by defining a selection window equal to one in sl-SelectionWindowList at least for traffic priorities which can benefit of MCSt, so that L1 is triggered multiple times, i.e. at each slot of the multiple slots in the MCSt during a traffic burst.

Proposal 32: Regarding when L1 reports a subset of candidate resources for MCSt, RAN1 may discuss: (i) in case Option A/C is supported, how should L1 know about the number of consecutive slots for reporting (ii) in case Option B is supported, is up to MAC to select consecutive resources based on implementation instead of random selection (iii) MCSt only supported by implementation, e.g. using a selection window equal to one.

Another aspect of MCSt which has been discussed by some companies in RAN1 are mechanisms to fill the GP symbol as well as whether GP should be disabled between the MCSt slots in order to maintain an uninterrupted channel occupancy by the UE over consecutive slots. In our understanding, RAN1 can define rules for determining whether GP should be disabled or enabled (i.e., applying a transmission gap in the GP symbol) during a MCSt of a Tx UE, e.g., depending on whether it expects other SL UE transmission allocations overlapping in time with its MCSt allocation. If there is no other transmission expected to be FDMed with the MCSt of the Tx UE the GP can be disabled, otherwise, if there is a resource reservation of another UE which overlaps in time with an allocation of its MCSt, the Tx UE should enable the GP on the MCSt slot prior to detected reserved resource. When the Tx UE enables the GP, it may enable a partial GP filling (i.e. only stop its transmission in a fraction of the GP symbol in order to achieve a certain gap duration), in order to reduce the possibility of losing the COT and allowing other SL-U UE to access the channel simultaneously, e.g. with Type 2A/2B LBT, for FDMed transmissions. It can also be defined that GP can be enabled to facilitate FDM for SL-U UEs, in case there is no other RAT using the channel, as there is less risk to lose the COT in the middle of a MCSt.

Proposal 33: RAN1 can define rules for enabling/disabling GPs during an MCSt, e.g., depending on whether different SL UE transmissions are expected to overlap in time with a MCSt allocation.

# 6	Conclusions

In this contribution, we made the following observations and proposals:

Observation 1:	Channel access mechanisms defined for NR-U in TS 37.213 can be largely re-used with SL-U, such that the role of TX UE is similar to gNB, and the role of RX UE is similar to UE.

Proposal 1: SL-U supports both dynamic (LBE) as well as semi-static (FBE) channel access mechanisms, the latter with minimum specification impact.

Observation 2: Simulation results based on the agreed evaluation scenario show that there is a consistent performance improvement for SL-U when absenceOfAnyOtherTechnology parameter is configured.

Proposal 2: The higher layer parameter absenceOfAnyOtherTechnology is supported for SL-U.

Proposal 3: The agreed UL CAPC table is adopted for SL-U without specific exceptions for mp value.

Proposal 4: SL-U adopts NR-U DL CW adjustment mechanism when SL-HARQ feedback is enabled in SCI for unicast, without any further enhancement for SL-U operation.

Proposal 5: SL-U allows using a different CW size other than the latest CWS, when a transmission is not associated with HARQ feedback. CW size can be determined based on , for example, CBR (Option 3 of CW adjustment when SL-HARQ feedback is disabled).

Proposal 6: In both groupcast option 1 (NACK-only) and groupcast option 2, the SL-U UE increases the CW size if UE retransmits (regardless of how many ACK/NACKs it has received), otherwise it shall set it to CWmin (aligned with ETSI regulations).

Proposal 7: In case of blind retransmissions without HARQ feedback across different COTs, RAN1 to study if CW size should be increased.

Proposal 8: Regarding whether to support another ending timing, the definition can be made like in NR-U specs, e.g., “the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted, or until the end of the first transmission burst by the UE that contains PSSCH with ACK/NACK HARQ-ACK enabled, whichever occurs earlier”.

Proposal 9: Same principles for EDT as NR-U can be applied for S-SSB transmissions with Type 2A LBT, i.e., based on maximum transmit power and TX bandwidth.

Proposal 10: S-SSB can be transmitted with Type2A/2B/2C (depending on gap from previous transmission) if there is a shared COT from any UE.

Proposal 11: RAN1 to confirm that additional S-SSB (if supported) is also associated to CAPC with p=1.

Proposal 12: RAN1 to support the use of SCSt with LBT Type 2A for PSFCH transmissions. The total duration of S-SSB and PSFCH transmissions by a UE using SCSt with Type 2A LBT is at most 2.5 ms within a 50 ms observation period.

Proposal 13: RAN1 to confirm that additional PSFCH(s) (if supported) are also associated with CAPC with p=1.

Proposal 14: Support CPE starting position within one symbol before the AGC for any subcarrier spacing (Option 1). Option 2 should be only considered if RAN1 assumes that SL CG transmissions or SL semi persistent transmissions from different UEs may occur on same PSSCH resources.

Proposal 15: If multiple CPE are supported for PSCCH/PSSCH, it should be up to configuration/implementation which of the CPE lengths should be used for partial or full RB set allocation.

Proposal 16: Support a single preconfigured or predefined CPE for PSFCH.

Proposal 17: The allowed CPE starting position for PSFCH should be discussed after deciding whether PSFCH can be transmitted to any UE during a shared COT and after deciding if PSFCH can use SCSt with Type 2A.

Proposal 18: Support multiple CPE lengths to be (pre)configured for S-SSB.

Observation 3: Any SL device can receive and decode content all the way to the MAC CE level, therefore the COT sharing information would be available to any device in proximity of the COT initiating device.

Proposal 19: RAN1 to support the signaling of COT sharing in SL. FFS how the COT sharing information is transmitted (e.g. 1st and/or 2nd stage SCI and/or MAC CE).

Proposal 20: The responding UE can transmit PSFCH(s) to UE(s) other than the initiator, at least when one or more PSFCH(s) in the slot is intended to a COT initiating UE.

Proposal 21: Regarding PSCCH/PSSCH transmission in a shared COT, each PSCCH/PSSCH transmission of the responding device should at least have the COT initiator device as a destination (based on cast type and Destination ID).

Proposal 22: RAN1 does not support UE forwarding/relaying information about a COT initiated by another UE.

Proposal 23: Additional ID(s) in the COT sharing information is not supported.

Proposal 24: RAN1 to support transmission of PSFCH on a subset of RB sets (using the NR-U DL channel access procedure as baseline). Both or at least Type A multi-channel access can be supported for PSFCH.

Proposal 25: RAN1 does not support for a UE to transmit S-SSB in more than one RB set in a slot.

Observation 4: The selection of a candidate resource preceding a reserved resource, e.g., of another UE, may cause LBT failure for the transmission on the reserved resource.

Observation 5: In mode 1 SL resource allocation, the gNB scheduler implementation should be able to prevent the occurrence of LBT blocking between connected SL devices.

Observation 6: In mode 2 SL resource allocation, the selection of a candidate resource after a reserved resource, e.g., of another UE, may cause LBT failure for the transmission on the candidate resource.

Proposal 26: Resource allocation procedure should avoid selection of a candidate resource before a reserved resource in case the transmitting symbols of candidate resource overlap with LBT of the reserved resource.

Proposal 27: Resource allocation procedure should avoid selection of a candidate resource after a reserved resource in case the transmitting symbols of the reserved resource overlap with LBT of candidate resource.

Proposal 28: Down select to option 1 and option 2 from Proposal 6 (I) of the FL summary for AI 9.4.1.1.

Proposal 29: In case a Type 1 LBT sensing time gets longer than the time until the selected resource, a resource re-selection can be triggered.

Proposal 30: For mode 1, it is up to gNB how to schedule the multiple consecutive allocations to a SL-U UE.

Observation 7: Regarding when L1 is triggered for reporting a subset of candidate resources for MCSt, both Option 1 or Option 2 are unclear how L1 determines the number of consecutive slots.

Proposal 31: When L1 is triggered for reporting a subset of candidate resources for MCSt, in case L1 should report candidate multi-slot resources (or consecutive single-slot candidate resources), RAN1 should consider another information for L1 to know the number of consecutive slots. Otherwise, the acquisition of candidate resources in consecutive slots should be based on implementation.

Proposal 32: Regarding when L1 reports a subset of candidate resources for MCSt, RAN1 may discuss: (i) in case Option A/C is supported, how should L1 know about the number of consecutive slots for reporting (ii) in case Option B is supported, is up to MAC to select consecutive resources based on implementation instead of random selection (iii) MCSt only supported by implementation, e.g. using a selection window equal to one.

Proposal 33: RAN1 can define rules for enabling/disabling GPs during an MCSt, e.g., depending on whether different SL UE transmissions are expected to overlap in time with a MCSt allocation.

# References

- RP-230077, WID on NR sidelink evolution, 3GPP TSG RAN#99.

- R1-2302290, On Physical Channel Design Framework for SL-U, 3GPP TSG RAN#112bis-e.

# A.1	Appendix

The parameters used in the simulations are provided in the following table:

Table 1: Evaluation assumption parameters

| Parameters | SL-U |
| --- | --- |
| Layout | Indoor 120 x 80m |
| Propagation scenario | NR InH Mixed Office |
| UE distribution | 24 SL-U UEs Uniformly distributed, 3 km/h speed |
| Carrier and Bandwidth | 5GHz, 20MHz |
| SCS | 15kHz |
| PHY | 100 RBs per subchannel (1 subchannel RP in 20MHz) |
| Receiver | MMSE-IRC |
| UE/STA antenna Array configuration | Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Max Rank | 1 |
| UE/STA TX power | 18 dBm including antenna gain (0 dBi) |
| MCS | Max Modulation supported 256QAM 4/5 |
| Traffic model | FTP3 aperiodic traffic, 500kB packet, 1500B PDU, Variable load |
| Pairing | Unicast/Unidirectional Tx UE pairs with first strongest Rx UE |
| LBT | LBT Type 1 with CAPC p = 4 COT duration: 6ms or 10ms |
| EDT | -72dBm |
| Channel model | Directly use InH office pathloss model with proper d_3D with indoor mixed office LOS probability |