3GPP TSG RAN WG1 #112bis-e	R1-2302291

e-Meeting, April 17th - April 26th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On Co-channel Coexistence for LTE Sidelink and NR Sidelink

Agenda item:	9.4.2

Document for:	Discussion and Decision

# Introduction

The following justification is given as background for the objective to support LTE SL and NR SL co-channel coexistence in V2X deployments in ITS spectrum:

Another aspect to consider is the V2X deployment scenario where both LTE V2X and NR V2X devices are to coexist in the same frequency channel. For the two different types of devices to coexist while using a common carrier frequency, it is important that there is mechanism to efficiently utilize resource allocation by the two technologies without negatively impacting the operation of each technology. This requirement was also mentioned as part of the input from 5G Automotive Association to the Rel-18 RAN Workshop.

The main motivation to consider co-channel coexistence between LTE SL and NR SL, is associated to the expectation that most (if not all) of the ITS spectrum will be allocated by regulators to LTE SL, leaving limited (or no) dedicated spectrum for NR SL. The motivation to prioritize LTE SL in the ITS spectrum, is related to the need to enable the basic safety V2X use cases (such as the ones described in TS 22.885) in a relatively short term in as many vehicles as possible to minimize the occurrence of traffic related accidents and fatalities. As new vehicles (that support both LTE SL and NR SL, and further in the future potentially NR SL only) are introduced into the market then at some point in time there will be enough market penetration to enable the use of advanced V2X use cases (such as the ones described in TS 22.886). However, for these advanced V2X use cases to be feasible it is required that enough spectrum is made available for NR SL both in non-ITS bands as well as the ITS band While the former case is being tackled by the introduction in NR SL of features such as carrier aggregation, operation in unlicensed band and beam management at FR2, the latter is to be enabled via LTE SL and NR SL co-channel coexistence.

In RAN Meeting #99, the WID on NR sidelink evolution was revised in RP-230077 [1] such that the objective on co-channel coexistence of NR sidelink and LTE sidelink operation in common spectrum is now:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

Additionally, RAN#99 provided the following guidance [2] [2]

- WGs are tasked to complete Co-Ex objective (#4) by June 2023.

With the latest changes the scope of the objective is as follows:

- Dynamic co-channel coexistence has high priority

- Support 15 and 30kHz SCS for dynamic co-channel coexistence

- Type A devices are considered with high priority

- Only Mode 2 and high priority on operating combination A (i.e. Mode 2 NR and Mode 4 LTE).

- 30 kHz SCS is to be supported with no changes to the PHY resource allocation procedure.

  - Starting symbol of the first NR SL slot is aligned with the first symbol of the LTE subframe.

  - Existing NR SL slot structure is reused.

- PSFCH support for dynamic co-channel coexistence is supported by the NR SL Tx UE avoiding to select resources where the PSFCH will be overlapping an LTE SL reservation.

In this contribution we are addressing the open issues to be resolved under the constraints of the RAN guidance above.

# On dynamic co-channel coexistence

Dynamic co-channel coexistence comes with a few challenges which RAN1 should study how to solve. In this section we will discuss how NR may support PSFCH and higher numerologies for dynamic co-channel coexistence.

## NR using PSFCH

HARQ feedback is carried over PSFCH, which is transmitted in 2 symbols within the last 4 symbols (guard, PSFCH AGC, PSFCH and guard) in an NR slot configured with PSFCH. However, while HARQ will improve NR SL resource usage (by reducing the need for blind retransmissions) the use of PSFCH is another cause of LTE Rx AGC issue when the PSFCH overlaps with an LTE transmission.

In RAN1#112 the following working assumption was made, for how to support PSFCH for dynamic co-channel coexistence:

Working assumption - For dynamic resource pool sharing, select one option between 1-1 and 1-2, and select one option between 3-1 and 4-1-a, and define how the two selected options can be used in the operation: - Option 1-1: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is performed by the UE transmitting PSFCH - FFS whether/how to define condition(s) under which the UE transmitting PSFCH drops its PSFCH transmission overlapping with the LTE SL resources reserved for LTE SL transmission - Option 1-2: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is ensured by the UE transmitting PSSCH - FFS whether/how to define condition(s) of selecting NR SL candidate resources for NR SL PSCCH/PSSCH - Option 3-1: Additional PSFCH periodicity(ies) is introduced - Alt 3-1-a: 10 logical slots - Alt 3-1-b: 8 logical slots - Alt 3-1-c: 5 logical slots - FFS: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration - Option 4-1: PSFCH resources and LTE SL resources are TDMed - Alt 4-1-a: PSFCH resources is confined within the guard symbol of the LTE SL subframe in the time domain - Note: selecting a single option is not precluded

The WA lists 4 options whereas it was intended that the combination of either Option 1-1 or Option 1-2 is combined with either Option 3-1 or Option 4-1-a, but with a note in the end allowing for selecting a single option.

The guidance from RAN#99 dictates the following:

- For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered.

Which has a very similar wording to Option 1-2 where the Tx UE is the one that must ensure that the LTE SL Rx UE will not suffer from an AGC issue caused by the NR SL Rx UE transmitting PSFCH. However, based on the discussions in the last meetings Option 1-2 can be realized in at least two different ways. In one variant the Tx UE will conditionally select candidate resources based on the information it has received from the LTE module (illustrated in Figure 1), and another where the Tx UE selects candidate resources in such a way that the NR SL PSSCH resources appear in periodically to the LTE SL which hence will refrain from selecting the candidate resources. The two variants can be seen as the two extremes in terms of which conditions are applied, and there can therefore also be variants in-between the two.

Figure 1. Illustration of Option 1-2 where the Tx UE ensures that the mapped PSFCH is not overlapping an LTE SL reservation.

The benefits of these procedures are that the Tx UE attempts to select a PSSCH resource that will minimize the risk of a collision between LTE and PSFCH, but the drawbacks are a constrained candidate resource selection procedure, as well as the inability to handle initial LTE transmissions for aperiodic traffic and the first LTE transmission for newly selected periodic resources, as well as not being able to exploit an LTE SL resource that is reserved but then not used. The extreme variant of this option where the NR SL Tx UE may relax its conditions as it relies on the LTE SL Tx UE to not select resources overlapping a specific NR SL Tx resource set have the main challenge that there is no guarantee that LTE SL will exclude the resources due to either a low NR generated S-RSSI contribution for low NR load or that RSSI ranking not performed when LTE has less than 20% candidate resources remaining.

Observation 1: Option 1-2 for supporting PSFCH for dynamic co-channel coexistence can be realized in two different variants; constraining the NR candidate resources to only be in a reduced/basic resource set, or by excluding candidate resources that will map to a PSFCH that overlaps an LTE SL reservation.

Observation 2: The variant of Option 1-2 for supporting PSFCH for dynamic co-channel coexistence, which excludes NR SL candidate resources that will map to a PSFCH that overlaps an LTE SL reservation is able to give some protection of the LTE reservations. Both variants of Option 1-2 won’t be able to detect initial LTE transmissions though.

A concern on Tx UE to exclude candidate resources based on the mapped PSFCH overlapping an LTE SL reservation is that this may lead to extensive candidate resource exclusion, and if not properly handled, resource exhaustion. For that reason, means to ensure that only the absolutely needed exclusions is done. The challenge is that the NR SL Tx UE which will be excluding on behalf of the NR SL Rx UE with the intention that the NR SL Rx UE will not cause an AGC issue to an LTE SL Rx UE, will have to exclude based on received LTE SCI from LTE SL Tx UEs, and therefore will not know how an NR SL Rx UE will interfere the transmission from the LTE SL Tx UE towards the LTE SL Rx UE. That said, it will still make sense to exclude NR candidate resources based on at least the overlapped LTE SL reserved resource priority and its associated RSRP. The LTE SL RSRP threshold for exclusion could be a function of the priority.

Proposal 1: Support Option 1-2 where NR SL Tx UE will exclude candidate resources that maps to a PSFCH overlapping an LTE subframe only if the priority of the overlapped LTE SL resource is higher than the priority of the NR candidate resource and the overlapped LTE PSSCH-RSRP above a threshold that is a function of LTE PSSCH priority and NR PSSCH priority.

The WA states that either Option 1-1 or Option 1-2 is considered together with either Option 3-1 or Option 4-1 and when combining that with the guidance from RAN to focus only on Option 1-2, that can either mean that only Option 1-2 (out of all four options) are considered, or only Option 1-2 (out of Option 1-1) is considered which would then leave Option 3-1 and Option 4-1 in scope together with Option 1-2. Option 3-1 might have some benefits when combining with the variant of Option 1-2 where no exclusion is done at the Tx UE. Option 4-1 is out of scope at least by the constraint of not using higher SCS than 30kHz SCS and no new slot formats allowed. In the end it is our proposal that both Option 3-1 and 4-1 should also be considered out of scope.

Proposal 2: Only Option 1-2 for PSFCH support with dynamic co-channel coexistence is considered based on RAN guidance (i.e. do not further discuss Option 1-1, Option 3-1 or Option 4-1 (or Option 4-1a)).

## NR SL using a numerology with higher SCS than LTE

This issue was discussed in RAN1#112, but did not result in an agreement. The latest, not agreed proposal on higher SCS is given with Proposal 1-1 (II) from the FL summary:

Draft proposal 1-1(II): - For dynamic resource pool sharing, if higher SCS is supported for NR SL: - The NR SL UE transmits PSCCH/PSSCH at least on the first of the NR SL slots overlapping with an LTE SL subframe, and can transmit on the subsequent overlapping NR SL slot(s) - Whether the NR SL UE transmits PSCCH/PSSCH only on the first of, or one or more of subsequent NR SL slot(s) overlapping with the LTE SL subframe, is up to UE implementation. - MCSt is used for the transmission on the subsequent overlapping NR SL slot(s) and no further enhancements on MCSt is considered in this agenda item.

But the updated WID [1] now includes the following description related to higher SCS for dynamic co-channel coexistence:

- RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe

Our understanding from the updated WID, is that 15 and 30 kHz SCS are to be supported for dynamic co-channel coexistence, but it needs to be done in such a way that no changes are allowed to the resource selection procedure in TS 38.214 8.1.4, no changes are allowed to the NR SL slot format and an NR SL PSSCH/PSCCH transmission with 30kHz SCS is always starting in a symbol that overlaps the first symbol of an LTE SL subframe which is ensured by the MAC layer. This can be realized in at least two different ways; a) the MAC layer will have to identify and exclude candidate resources that are not aligned with the start of an LTE SL subframe or b) the NR SL Tx resource set will need to be changed to only include resources that are in NR SL slots which start is aligned with the LTE SL subframe.

In a) it could be assumed that the MAC layer will be aware of the LTE SL resource pool configuration and can therefore exclude NR candidate resource sets that are not fulfilling the criteria, but it will also be able to match two NR SL candidate resources that are in consecutive slots and overlaps the same LTE SL subframe. In b) the MAC layer does not need to be aware of the LTE resource pool configuration (apart from when the NR SL Tx resource set is configured), to make a decision of which NR candidate resource should be considered. If the MAC layer wants to select a resource in the consecutive slot, it will need to make this selection “blindly”, i.e. autonomously extending the NR candidate resource set to the next slot.

The problem with b) is that there is a risk that as the NR SL load increase, the PHY will see reservations from other NR SL UEs in the first NR SL slots, but fewer in the second NR SL slots. This means that, as PHY targets to have 20% of all initial candidate resources in the final candidate resource set forward to MAC, there might not be any candidate resources from the first NR SL candidate resource set, but only candidate resources from the second candidate resource set, which would force the MAC layer to either reinitialize the PHY procedure or discard the TB.

Observation 3: If no changes are allowed in the PHY resource (re-)selection procedure, but the responsibility for only initiating an NR SL PSSCH/PSCCH transmission in an NR SL slot which start is aligned with the first symbol of the LTE subframe, is handled by MAC, it is a risk that PHY will only identify candidate resources that belongs to the second NR SL slot.

Proposal 3: When an NR SL slot overlaps an LTE SL subframe, a potential resource starvation when supporting 30kHz SCS is avoided by configuring the NR candidate single-slot resource set is initialized with resources in NR SL slots which are first of NR SL slots overlapping with an LTE SL subframe where the starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe.

# How Type A devices can support dynamic co-channel coexistence

The updated WID [1], states that this objective on sidelink co-channel coexistence shall focus on so-called Type A devices, which is a NR SL Type A device comprises both an LTE SL and an NR SL module and is assumed to have an inter-module interface allowing for internal device module coordination. Such modules support LTE SL to ensure day 1 traffic safety, while being ready to use NR SL when the penetration of NR SL devices increase, which could be desirable for vehicle manufacturers given the long life-span of vehicles.

There are issues to be discussed on Type A devices such as how the module will use the information received from LTE, in its resource (re-)selection procedure, how to handle non-monitored LTE subframes, how to handle the LTE modules own reservations in the NR module and how to address the exclusion needed for the PSFCH solution.

## Type A resource (re-)selection procedure

In RAN1#112 the following agreement was reached regarding the options on how the NR module in a Type A device use the information shared from the LTE module.

Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied

Despite the agreement of applying Option 1, there are still several issues that remains to be addressed on how NR will treat the information from “other” LTE devices that it has received from the LTE module:

- Can NR use the LTE RSRP associated with an LTE SCI directly or does it need to do some conversion?

- Which RSRP list should NR use for LTE reservations?

- Which variant of the Q-formula should be used (the one from LTE or the one from NR)?

Regarding the first issue on the applicability of LTE RSRP to the NR resource (re-)selection procedure. The LTE module should use PSSCH-RSRP (TS 36.213 and defined in TS 36.214) in its resource selection procedure whereas the NR can use either PSSCH-RSRP or PSCCH-RSRP (depending on sl-RS-ForSensing). The LTE PSSCH-RSRP is defined as the averaged power (in watts) over the allocated frequency resources (indicated by LTE PSCCH), measured over the DM-RS symbols (and averaged over the DM-RS symbols). This also means that the LTE PSSCH-RSRP is not covering the LTE PSCCH which is 3dB power boosted. NR PSSCH-RSRP is defined similarly, so for a similar allocated resource, with perfect time and frequency overlap, the NR PSSCH-RSRP and LTE PSSCH-RSRP should be comparable. However, when it comes to the applied thresholds, that will need to take into account this power boost of LTE PSCCH as the interference from an LTE to NR is different than interference from NR to NR, and similarly the interference from NR towards LTE is not the same as LTE to LTE.

There is obviously also a potential need for scaling due to differences in time and/or frequency domain. If NR and LTE RP are only partly overlapping (in time), for example due to NR using a higher SCS than LTE, that should not require any scaling. In the frequency domain, a partly overlap can occur due to NR and LTE are not using the same frequency resources, NR subchannels are configured to be larger or smaller than LTE. In this case, scaling might be needed to handle this partial overlap, but it should be feasible do leave that for UE implementation as it is not dissimilar than converting RSRP from a transmission using a particular number of subchannels to another number of subchannels.

Regarding partial overlap between LTE sub-channels and NR sub-channels, the easiest case is of course that these sub-channels are of the same size and aligned such that partial overlap does not occur. Since it may be the case the LTE sidelink devices have already been deployed and that their configuration cannot be updated we can investigate which LTE sub-channel size(s) is/are likely to be used. For LTE sidelink, ETSI TC ITS has published a recommended sub-channel size (sizeSubchannel) of 10 RBs (for 15 kHz SCS) [3].

For NR sidelink, the ETSI TC ITS recommendation has not been published yet, but the most recent draft version of EN 303 798 recommends a sub-channel size (sl-SubchannelSize) of 12 RBs for a 10 MHz channel and 10 RBs for a 20 MHz channel (for 30 kHz SCS). It is clear that using a combination of NR sub-channel size of 12 RBs (with 30 kHz SCS) and LTE sub-channel size of 10 RBs (with 15 kHz SCS) would lead to multiple situations of partial overlap of NR sub-channels and LTE sub-channels with varying degree of overlap.

That situation, however, can easily be avoided by (pre)configuring an NR sub-channel size of 10 RBs. For NR SCS 15 kHz, an NR sub-channel will then have the same size (in Hz) as an LTE sub-channel and partial overlap can be completely avoided by aligning the NR and LTE sub-channels. Moreover, a sub-channel size of 10 RBs allows configuring 5 sub-channels in a 10 MHz carrier, wasting only 2 RBs out of the 52 RBs of the maximum transmission bandwidth configuration for SCS 15 kHz in 10 MHz channel bandwidth.

For NR SCS 30 kHz, for the smallest supported sub-channel size of 10 RBs, an NR sub-channel will have twice the size (in Hz) as an LTE sub-channel of 10 RBs and partial overlap cannot be avoided. However, by aligning LTE sub-channels and NR sub-channels it can at least be ensured that one LTE sub-channel overlaps with only one NR sub-channel. One could consider introducing a new NR sub-channel size of 5 RBs for 30 kHz SCS to achieve the same NR sub-channel size (in Hz) as an LTE sub-channel. However, the smallest currently supported number of PRBs for PSCCH transmission is 10 PRBs (sl-FreqResourcePSCCH) and the PSCCH design assumed that PSCCH is contained within one sub-channel; therefore, introducing such a new smaller NR sub-channel size would require additional work on PSCCH design. One drawback of using an NR sub-channel size of 10 RBs in this configuration is that only 2 NR sub-channels can be configured in a 10 MHz channel, wasting 4 out of the 24 RBs of the maximum transmission bandwidth configuration for SCS 30 kHz in 10 MHz channel bandwidth. If, however, a 20 MHz channel bandwidth can be used for NR then a sub-channel size of 10 RBs is very efficient – 5 NR sub-channels can be configured, wasting only 1 RB out of the 51 RBs of the maximum transmission bandwidth configuration for SCS 30 kHz in 20 MHz channel bandwidth.

Observation 4: Based on the above discussion, assuming that LTE and NR will use sub-channels of same size and that NR and LTE sub-channels are aligned, LTE SL RSRP needs no special overlap-dependent scaling (compared to an NR SL RSRP) before being used in the NR SL resource selection procedure. The NR module will need to be able to associate LTE subchannels and LTE subframes with NR slots and LTE subchannels.

Regarding the RSRP list options, four different alternatives are captured in the agreement above. As discussed above, the RSRP list would need to take into account the different impact for interference from an LTE to NR and NR to LTE compared as an LTE to LTE transmission or NR to NR transmission. With Alt. 1 it is our understanding that NR will use the RSRP threshold list that is use when determining whether to exclude NR candidate resources. This means that NR considers an LTE SL RSRP as an NR SL RSRP, which would be problematic as the LTE SL transmission would cause more interference than an NR SL transmission.

The similar, but reversed issue is present with Alt. 2, where the NR module would use an LTE RSRP threshold list to determine whether an NR candidate resource should excluded if it overlaps an LTE or NR SCI based on the LTE RSRP threshold list. This is problematic as the NR transmission would not cause as much interference as the LTE transmission would.

In Alt. 3, a new RSRP threshold list is configured in specifically for dynamic co-channel coexistence. This alternative will allow to make a fair RSRP threshold that can take into account the difference of an LTE to NR and NR to LTE compared to LTE to LTE and NR to NR.

In Alt. 4, a new RSRP threshold list is also configured for dynamic co-channel coexistence as in Alt. 3, but with the additional option of having a specific RSRP threshold in slots that contains NR PSFCH. This alternative is heavily tied together with the constraining of NR SL Tx resource set in PSFCH resources and if not supported, we do not see any benefits of this alternative.

Proposal 4: Down-select the RSRP threshold list to Alt. 3 in the Type A resource (re-)selection procedure.

Regarding the FFS on which Q-formula should be used it has been discussed to use either the Q-formula expression from NR (TS 38.214 section 8.1.4), or the Q-formula from LTE (TS 36.213 section 14.1.1.6). The two are different, but serve the same purpose, namely to determine a future periodic occurrences of a received SCI. If the NR expression is adapted, that would that would mean that an LTE reservation will be translated to a future NR slot, which can then be directly identified whether it is overlapping an NR candidate resource. That would require:

- PrsvpRX from LTE to be in the unit of ms instead of s

- Conversion from LTE subframe (e.g. tm'LTE SL) to NR logical slot (e.g. tm'SL)

And should otherwise be straight forward. The benefits of doing so, is that all the expressions for the NR selection window for full and partial sensing would be applicable as is. The conversion from LTE subframes to NR logical slots (and reversed) could be specified in a separate section as is done to translate from ms to a number of NR logical slots in 8.1.7.

If the LTE Q-formula is applied, that would mean that translation from the subframe where the LTE SCI was received and the future subframes is done as in LTE, but then would need to be translated to a corresponding NR slots, and filtered by those that falls into the NR selection window.

It is our understanding that both options can be done, but the most straight forward option is to adapt the NR Q-formula expression, with LTE SCI parameters translated into the same unit as NR.

Proposal 5: Adapt the NR Q-formula to LTE reservations.

### How to handle non-monitored LTE subframes

The issue on how the NR module can the information of which LTE subframes the LTE module has not monitored, in the NR SL resource (re-)selection procedure is also open. It was discussed in RAN1#112, but did not reach an agreement:

Proposal 2-3(II): - In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources associated with non-monitored subframe of LTE SL module - Option 1: - The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module - For determining the LTE SL resources associated with non-monitored subframe of LTE SL module, - All the LTE SL resources in the non-monitored subframes are assumed to be repeatedly reserved Q times for each LTE SL resource reservation period (pre)configured in a LTE SL resource pool - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module according to Step 5 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL resources associated with non-monitored subframe of LTE SL module used in the above procedure is shared from LTE SL module to NR SL module - Option 2: - The PHY layer of NR SL module does not perform NR SL candidate resource exclusion based on the information regarding LTE SL resources associated with non-monitored subframe of LTE SL module

Exclusion based on non-monitored LTE subframes intends to allow the NR module to exclude NR candidate resources that might, or might not, overlap an LTE reservation that has been transmitted in the non-monitored subframe. If exclusion based on non-monitored slots are not done by the NR module, it would risk selecting a candidate resource that overlaps an “unknown” LTE reservation and since all exclusion for PSFCH and higher SCS is either done by configuration (subframe timing or by LTE SL reservations), this cannot be compensated for by the NR module detecting the presence of an ongoing LTE transmission. Therefore, it is hard to see how Option 2 will not result in LTE AGC issues, while Option 1 would fit with the solutions for higher SCS and PSFCH support and be a natural feature to include as it is also done all LTE SL and NR SL modules.

However, exclusion based on non-monitored LTE subframes in addition to exclusion based on not-monitored NR slots can lead to extensive exclusion. This is addressed in NR by allowing to “reinclude” the candidate resources that is hypothetically indicated by an SCI in the non-monitored slot if there are fewer than X remaining candidate resources in the initial candidate resource set. Something similar should be adapted for exclusion of candidate resources that hypothetically could be overlapped by an LTE SCI transmitted in a non-monitored LTE subframes. This could be done by doing exclusion and “re-inclusion” of NR and LTE caused exclusion due to non-monitored slots/sub-frames (i.e. by extending Step 5 only), or by making a separate exclusion and “re-inclusion” of candidate resources that were excluded based on non-monitored LTE subframes. This would require two additional steps in Step 5) one to exclude based on LTE non-monitored subframes and one to check if there are sufficient candidate resources. In our view, it makes sense to consider exclusion based on non-monitored NR slots and exclusion based on non-monitored LTE subframes as two different steps such that:

- Step 5 is kept as is.

- (new). Exclusion based on non-monitored LTE subframes.

- (new). Check if the candidate resources are less than a threshold, and if so, reinitialize the candidate resource set to after Step 5.

- (old NR Step 5a). Check if the candidate resources are less than a threshold, and if so, reinitialize the candidate resource set as after Step 4.

Proposal 6: Introduce a separate step for exclusion based on non-monitored LTE subframes and another step for checking of the number of remaining candidate resources is above a threshold after the LTE non-monitored subframe related exclusion.

### How to handle the resource selection information from the LTE module

The second agreement from RAN1#112, is about how to handle the LTE module resource selections:

Agreement In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module - Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

It is agreed that the choice of how to handle the information of the LTE module own resource selections is between Option 1 (exclude overlapping NR candidate resources) and Option 2 (do not exclude). The arguments for Option 2, is that if the NR module selects a candidate resource that overlaps an LTE transmission that can be handled by the in-device co-existence framework. While this is true, it also means that shortly prior to initiating the intended transmission (which can also have been reserved), one of the two modules will have to drop its transmission based on priority or implementation if the priorities are equal. This is clearly undesirable and could cause for example a waisted resource, reinitiate the resource (re-)selection procedure or a retransmission (if the dropped transmission contained PSFCH or was a retransmission).

Option 2, on the other hand, targets to prevent some of these events by allowing the NR module to exclude candidate resources that will collide with the selected resources from the LTE module. If it is assumed that the LTE module will use the same interface and format as used to inform the NR module on the “other” LTE module reservations, the LTE module will be able to indicate fields that it intends to transmit (after having selected a candidate resource) to the NR module, with the difference that no RSRP value will be associated to the information as it would if it came from an “other” LTE module. The exclusion of such LTE modules own selected resources can be done either in Step 5 (as stated in the agreement), but it can also be done in Step 6 where exclusion of reservations based on information from “other” LTE modules will take place.

Step 5 is where non-monitored slots (and possibly also sub-frames) is taken into account, and while a past subframe where the LTE module has been transmitted can be considered as a non-monitored subframe, but it is not yet a step that considers a resource selected in the future. The reason is that a non-monitored subframe excludes candidate resources in a subframe that can be indicated by an SCI transmitted in the non-monitored subframe and hence takes all possible periodicities into account. This would also bean that if the exclusion behavior for resources selected by the LTE module is kept in Step 5, the procedure would likely need to be decoupled from the existing Step 5.

If this is done in Step 6 instead, it would also need some changes compared to the normal Step 6 c), as exclusion would need to be done for all candidate resources overlapping the LTE SL subframe where the LTE module has selected a resource, further there would not be an RSRP associated to the LTE module selected resource but exclusion based on priority is already present and the lack of an RSRP can be handled by assuming a very high RSRP value.

It is also worth discussing whether the exclusion of NR candidate resources based on the information on selected resources by the LTE module should be based on a condition, such as priority. Arguments for not doing so, could be that NR will always prevent a collision by always excluding these candidate resources, and if the LTE module is not expected to transmit very frequently, it is not expected to cause much impact to the NR module. Arguments for doing so, could be that NR knows its own priority and can therefore favor either NR or LTE by evaluating whether the IDC framework would force either LTE or NR to drop its transmission if it ends up selecting a candidate resource that will collide in time with the LTE transmission. In one variant of that, the NR module can decide to only exclude if it determines that it would be forced to drop its own transmission if it ends up selecting these particular candidate resources.

Based on the above discussion, it seems most fair that NR will, unconditionally, exclude all candidate resources that overlap (in time) an LTE SL subframe that contains a selected resource by the LTE SL module. If this is done in Step 5, it would likely need to be in a separate new step. It could then be considered if the exclusion of these candidate resources should be should be “re-added” if the remaining number of candidate resources is below a threshold.

Proposal 7: The NR module should exclude candidate resources overlapping a resource selected by the LTE module.

### Resource (re-)selection impact due to PSFCH Option 1-2

Resource (re-)selection impact due to PSFCH overlap might also needs to be considered now that Option 1-2 has been dictated in the updated WID [1] but depending on which variant of Option 1-2 will be agreed in RAN1. If it is agreed that Option 1-2 will imply that the NR SL Tx UE will exclude candidate resources that it determines will cause the mapped PSFCH occasion to be overlapping (in time) with a reserved LTE resource, it should be excluded (possibly based on some criteria such as priority). This could be a relatively straight forward additional condition for exclusion in Step 6.

## Timeline for data exchange between LTE and NR module

The timeline discussion on information from LTE to the NR module has agreements from both RAN1#110e-bis and RAN1#111 and are provided below.

Agreement For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission. - T is defined using - T≤Tmax ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence. - FFS: Value of Tmax - FFS: any discussion on the earliest information, if needed Agreement Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms.

The NR module will have to take into account information from the LTE module, and therefore it is important that the information from the LTE module is not significantly delayed in the process of being processed at the LTE module, delivered over some interface to the NR module and in the end processed at the NR module. A simplified time representation of the LTE information acquisition, and then delivery to the NR module is provided in Figure 2. Upon sensing, at some point the LTE module processes the sensing results for transmission, then the information is transmitted and processed at the NR module. At the NR module the information is then available at slot n and used in the resource selection procedure. The agreement above states that “the information shared by the LTE module is known by the NR SL module at least T ms prior to slot n”, and T is bounded by Tmax which is 4 ms. In principle, the current definition of T might only capture the NR processing time, in other words the agreement imposes an upper bound only on the NR processing time TNR,proc≤Tmax, but there is currently no bound or assumption on the other components of the overall processing delay, namely LTE processing time TLTE,proc and time taken to transfer the information from LTE SL module to the NR SL module, Ttransfer.

Figure 2. Illustration of a the time delays expected for exchange of LTE information from an LTE module to the NR module.

This timeline is important, as the assumptions made on the timeline affects the performance of the NR and LTE co-channel coexistence performance. For example, NR will not be able to account for the LTE sensing information in the time gap from the LTE sensing window till the NR timeslot n (TLTE,proc+Ttransfer+TNR,proc) so the larger this is, the large risk of NR colliding with an LTE transmission due to selecting resources based on “old” information. While we cannot put a timing constraint on the information delivered from the LTE module to the NR module, companies providing evaluations with a Type A module can at least inform what assumptions they make on the timeline, in particular TLTE,proc, Ttransfer and TNR,proc.

There is also an open FFS on what is the earliest information that can be shared from the LTE module to the NR module. It is our proposal that the LTE module will exchange information from the entire LTE sensing window, hence the earliest information shared from the LTE module to the NR module is given by terliest=tn-(TLTE,sensing_window+TLTE,proc+Ttransfer+TNR,proc).

Proposal 8: Companies evaluating with a Type-A device is to report their assumptions of LTE to NR information timeline (e.g. delay of LTE processing of the sensing information (TLTE,proc,), transfer delay to the NR module (Ttransfer) and delay of NR processing the received information (TNR,proc)).

Proposal 9: The earliest time for information delivered from the LTE module to the NR module accounting for the full LTE sensing window (e.g. the full LTE sensing window (TLTE,sensing_window), the delay of LTE processing of the sensing information (TLTE,proc,), the transfer delay to the NR module (Ttransfer) and delay of NR processing the received information (TNR,proc).

# How Type B devices can support dynamic co-channel coexistence

A NR SL Type B device, compared to a Type A device, is a device which contains only an NR module (or has no interface or shared hardware to an LTE SL module). That means that it will not have an internal interface to provide it with information on LTE SL sensing results. We consider the Type-B (NR only) device to be essential for the co-channel coexistence framework as it can be expected that new (at some point in time) will not be deployed with an LTE module and that this module allow deploying a existing LTE only connected vehicle with NR SL capabilities. For a Type B device to act fairly towards NR Type A devices and LTE devices it will need to have some mechanisms to become LTE SL aware in its resource selection and re-selection procedures to not be LTE SL agnostic.

Proposal 10: For co-channel coexistence, Type B devices should be supported. A Type B device is a Release-18 NR-only device which does not have the sensing information shared by the LTE module (if present).

In general, three options can be considered, when it is assumed that these devices must work in Mode 2, which are:

Option a)	relying on IUC from a Type A device; or

Option b)	be equipped with LTE detection capabilities

Option c)	be equipped with LTE sensing capabilities

It is our view that at least option a) and at least one of b) and c) should be supported. The reason being Option a) cannot stand alone for a Type B device, but can be considered as an assisting procedure when a Type A device is nearby and can provide it with IUC support. This could be the case when a Type A device could be a RSU for example.

Option b) and c) provides the Type B device with LTE SL awareness by itself and should be considered the primary options. Option b) refers to the capability of detecting the presence of an LTE SL transmission, and then avoid initiating an NR transmission in an overlapping slot. This is certainly feasible for PSFCH with the existing NR slot format for PSFCH, as well as when NR use a higher numerology than LTE, but it might not be feasible to timely detect an LTE transmission when the start of the subframe and slot are aligned.

Option c) refers to the NR device conducting LTE based sensing, whereas at the very simplest level, the Type B device will conduct energy-based sensing for LTE transmissions which can be sufficient to detect occupied LTE resources and derive LTE reservation patterns. However, this is obviously not the most reliable nor resource efficient way to detect future LTE reservations, and does not give the device any information on what priority the LTE reservation have and therefore a more efficient capability option is that the Type B device can receive LTE PSCCH including LTE SCI and from those determine for example the periodicity and priority of the LTE reservations.

Option b) and c) provides the Type B device with LTE SL awareness by itself and should be considered the primary options.

Proposal 11: A Type B device should support at least one of the following LTE detection or sensing capabilities:  Capability A: NR SL device capability to detect ongoing LTE transmissions.  Capability B1: NR SL device capability to conduct energy-based sensing of LTE transmissions.  Capability B2: NR SL device can receive LTE PSCCH (incl SCI) signals FFS whether to include the measurement of the corresponding PSSCH RSRP.

# On semi-static co-channel coexistence

In RAN1#109e the agreement below was made regarding the study of the feasibility of both semi-static and dynamic resource sharing.

Agreement Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied.

And in RAN1#110e we reached the following conclusion on TDM based co-channel coexistence:

Conclusion For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs. - Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning. - Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe - FFS: potential enhancements for synchronization can be further investigated

The use of semi-static TDM to enable co-channel coexistence between LTE SL and NR SL has the advantage that it can be achieved via resource pool configuration (i.e. without specification impact). However, once a resource pool pre-configuration is established, then it is expected that this TDM configuration will remain static for several decades due to regulatory concerns associated with traffic safety. In other words, transition/re-farming of LTE SL spectrum resources to NR SL is very unlikely to occur if co-channel coexistence is implemented via TDM.

The use of semi-static FDM to enable co-channel coexistence, has again the advantage that it can be achieved via resource pool configuration. However, the semi-static co-channel coexistence mode has two main drawbacks. First, the same drawback as semi-static TDM, that it might be infeasible to update the resource pre-configuration once it has been established. Secondly, there is a risk of causing LTE SL receiver AGC issues, where the LTE’s AGC setting is impacted by the power variations associated with the NR’s V2X transmission in the case of NR slots configured with PSFCH symbols as also discussed for dynamic co-channel coexistence.

Observation 5: Semi-static co-channel coexistence approaches (FDM and TDM) prevent transition/re-farming of LTE SL spectrum resources to NR SL due to the inability to update a V2X pre-configuration once it has been established.

Should we have to choose between TDM and FDM (to supplement dynamic) it seems that a FDM approach is not as straightforward to support as an TDM approach, and hence a TDM approach should be the chosen.

Proposal 12: RAN1 does not further discuss an FDM semi-static approach to co-channel coexistence as a TDM approach is already agreed to be feasible.

# Synchronization between NR and LTE

Semi-static FDM-based and dynamic co-channel coexistence modes require at least slot and subframe boundary alignment between LTE and NR SL transmissions to avoid AGC issues. Strict slot and subframe boundary alignment is not necessary for semi-static TDM, if guard slots/subframes are configured between NR and LTE in the resource pool configurations and the timing offset between LTE and NR is known. However, since the slot and subframe boundaries in Rel-16 in-device coexistence framework are assumed to be aligned, it is reasonable to do the same assumption in co-channel coexistence operation to avoid unnecessary overhead. In case of device type A, the alignment may be achieved by implementation means as in Rel-16.

Observation 6: For any co-channel coexistence mode slot and subframe boundary alignment between LTE and NR SL is assumed

During RAN1#110 the following conclusion was made:

Conclusion For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs. - Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning. - Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe - FFS: potential enhancements for synchronization can be further investigated

In addition to slot and subframe alignment, configuration of non-overlapping resource pools for LTE SL and NR SL requires 10240ms DFN period alignment so that resource pool bitmaps in LTE and NR have the same starting point. Rel-16 in device framework states that UE must know frame indexes of LTE and NR. Our understanding is that this is not sufficient but frame indexes in LTE and NR should be aligned in TDM based co-channel coexistence operation.

Proposal 13: When non-overlapping resource pools are configured for semi-static TDM based co-channel coexistence, DFN/SFN and subframe/slot alignment between NR SL and LTE SL is assumed.

GNSS or eNB can be used as synchronization source for both LTE and NR SL operation. If the UE synchronizes to one of them then DFN and subframe/slot alignment between NR and LTE is guaranteed. But if NR SL unit is synchronized to gNB then according to the current specification LTE SL unit needs to find some other synchronization source because gNB is not supported as a synchronization source for LTE. Also, NR S-SSB is not supported as synchronization source for LTE SL and LTE S-SSB is not supported as synchronization source for NR SL. In case of type A device, the UE could compare priority of synchronization sources available for LTE unit and NR unit so that the device would be aware of synchronization priorities of LTE SL and NR SL. In case of type B device, the awareness of synchronization priority of LTE SL would require capability to receive LTE S-SSB. The requirement that LTE operation should not be changed limits the choice of synchronization sources for LTE SL but at least for the case when LTE S-SSB is the highest priority synchronization source, NR should support to use it as a synchronization reference source.

Proposal 14: Type A device should support transmitting NR S-SSB that is based on reception of LTE S-SSB in co-channel coexistence operation.

If slots for S-SSB transmissions in NR and synchronization signal subframes in LTE are configured independently it may happen that NR PSSCH overlaps with LTE synchronization signals and/or LTE PSSCH overlaps with NR synchronization signals. NR resource pool definition in co-channel coexistence could be modified so that in addition to NR S-SSB slots, also LTE synchronization signal subframe occasions are excluded from the NR resource pool. However, exclusion of NR S-SSB slots from the LTE resource pools is not possible without modifications to LTE specifications. This needs to be resolved.

Proposal 15: RAN1 to discuss how the overlap of LTE SLSS and NR PSSCH and the overlap of NR S-SSB and LTE PSSCH can be avoided.

# Resource allocation modes of LTE and NR SL for co-channel coexistence

The RAN#97e agreed to add a note to the objective stating to focus on Combination A (Mode 2 NR SL with Mode 4 LTE SL) and RAN#99 added a further guidance on “Mode 2 operation only”. This basically means that only Mode 2 and Mode 4 (Combination A) and Mode 2 and Mode 3 (Combination C) are now in scope of the WID. It is our view, that existing LTE deployments only support Mode 4, and that it is unlikely that a Mode 3 LTE SL mode will be deployed. It is also our view that the predominant mode for NR in co-channel coexistence will be Mode 2, meaning that both LTE and NR would operate in a UE-autonomous manner.

Proposal 16: RAN1 to limit the scope of resource allocation modes to Combination A (Mode 2 NR SL with Mode 4 LTE SL).

# Conclusions

In this contribution, we have the following observations and contributions:

Observation 1: Option 1-2 for supporting PSFCH for dynamic co-channel coexistence can be realized in two different variants; constraining the NR candidate resources to only be in a reduced/basic resource set, or by excluding candidate resources that will map to a PSFCH that overlaps an LTE SL reservation.

Observation 2: The variant of Option 1-2 for supporting PSFCH for dynamic co-channel coexistence, which excludes NR SL candidate resources that will map to a PSFCH that overlaps an LTE SL reservation is able to give some protection of the LTE reservations. Both variants of Option 1-2 won’t be able to detect initial LTE transmissions though.

Proposal 1: Support Option 1-2 where NR SL Tx UE will exclude candidate resources that maps to a PSFCH overlapping an LTE subframe only if the priority of the overlapped LTE SL resource is higher than the priority of the NR candidate resource and the overlapped LTE PSSCH-RSRP above a threshold that is a function of LTE PSSCH priority and NR PSSCH priority.

Proposal 2: Only Option 1-2 for PSFCH support with dynamic co-channel coexistence is considered based on RAN guidance (i.e. do not further discuss Option 1-1, Option 3-1 or Option 4-1 (or Option 4-1a)).

Observation 3: If no changes are allowed in the PHY resource (re-)selection procedure, but the responsibility for only initiating an NR SL PSSCH/PSCCH transmission in an NR SL slot which start is aligned with the first symbol of the LTE subframe, is handled by MAC, it is a risk that PHY will only identify candidate resources that belongs to the second NR SL slot.

Proposal 3: When an NR SL slot overlaps an LTE SL subframe, a potential resource starvation when supporting 30kHz SCS is avoided by configuring the NR candidate single-slot resource set is initialized with resources in NR SL slots which are first of NR SL slots overlapping with an LTE SL subframe where the starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe.

Observation 4: Based on the above discussion, assuming that LTE and NR will use sub-channels of same size and that NR and LTE sub-channels are aligned, LTE SL RSRP needs no special overlap-dependent scaling (compared to an NR SL RSRP) before being used in the NR SL resource selection procedure. The NR module will need to be able to associate LTE subchannels and LTE subframes with NR slots and LTE subchannels.

Proposal 4: Down-select the RSRP threshold list to Alt. 3 in the Type A resource (re-)selection procedure.

Proposal 5: Adapt the NR Q-formula to LTE reservations.

Proposal 6: Introduce a separate step for exclusion based on non-monitored LTE subframes and another step for checking of the number of remaining candidate resources is above a threshold after the LTE non-monitored subframe related exclusion.

Proposal 7: The NR module should exclude candidate resources overlapping a resource selected by the LTE module.

Proposal 8: Companies evaluating with a Type-A device is to report their assumptions of LTE to NR information timeline (e.g. delay of LTE processing of the sensing information (TLTE,proc,), transfer delay to the NR module (Ttransfer) and delay of NR processing the received information (TNR,proc)).

Proposal 9: The earliest time for information delivered from the LTE module to the NR module accounting for the full LTE sensing window (e.g. the full LTE sensing window (TLTE,sensing_window), the delay of LTE processing of the sensing information (TLTE,proc,), the transfer delay to the NR module (Ttransfer) and delay of NR processing the received information (TNR,proc).

Proposal 10: For co-channel coexistence, Type B devices should be supported. A Type B device is a Release-18 NR-only device which does not have the sensing information shared by the LTE module (if present).

Proposal 11: A Type B device should support at least one of the following LTE detection or sensing capabilities:  Capability A: NR SL device capability to detect ongoing LTE transmissions.  Capability B1: NR SL device capability to conduct energy-based sensing of LTE transmissions.  Capability B2: NR SL device can receive LTE PSCCH (incl SCI) signals FFS whether to include the measurement of the corresponding PSSCH RSRP.

Observation 5: Semi-static co-channel coexistence approaches (FDM and TDM) prevent transition/re-farming of LTE SL spectrum resources to NR SL due to the inability to update a V2X pre-configuration once it has been established.

Proposal 12: RAN1 does not further discuss an FDM semi-static approach to co-channel coexistence as a TDM approach is already agreed to be feasible.

Observation 6: For any co-channel coexistence mode slot and subframe boundary alignment between LTE and NR SL is assumed

Proposal 13: When non-overlapping resource pools are configured for semi-static TDM based co-channel coexistence, DFN/SFN and subframe/slot alignment between NR SL and LTE SL is assumed.

Proposal 14: Type A device should support transmitting NR S-SSB that is based on reception of LTE S-SSB in co-channel coexistence operation.

Proposal 15: RAN1 to discuss how the overlap of LTE SLSS and NR PSSCH and the overlap of NR S-SSB and LTE PSSCH can be avoided.

Proposal 16: RAN1 to limit the scope of resource allocation modes to Combination A (Mode 2 NR SL with Mode 4 LTE SL).

# References

- RP-230077, WID on NR sidelink evolution, 3GPP TSG RAN#99.

- RP-230769, Way forward on R18 SL-Evo WI objectives, 3GPP TSG RAN#99

- ETSI EN 303 613 V1.1.1

# Appendix - List of previous agreements

The topic was discussed in RAN1#112 wit the following agreements:

Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module FFS whether/how LTE SL RSRP is applied Agreement In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module - Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule Working assumption - For dynamic resource pool sharing, select one option between 1-1 and 1-2, and select one option between 3-1 and 4-1-a, and define how the two selected options can be used in the operation: - Option 1-1: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is performed by the UE transmitting PSFCH - FFS whether/how to define condition(s) under which the UE transmitting PSFCH drops its PSFCH transmission overlapping with the LTE SL resources reserved for LTE SL transmission - Option 1-2: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is ensured by the UE transmitting PSSCH - FFS whether/how to define condition(s) of selecting NR SL candidate resources for NR SL PSCCH/PSSCH - Option 3-1: Additional PSFCH periodicity(ies) is introduced - Alt 3-1-a: 10 logical slots - Alt 3-1-b: 8 logical slots - Alt 3-1-c: 5 logical slots - FFS: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration - Option 4-1: PSFCH resources and LTE SL resources are TDMed - Alt 4-1-a: PSFCH resources is confined within the guard symbol of the LTE SL subframe in the time domain - Note: selecting a single option is not precluded

The topic was also discussed in RAN1#111 with the following agreements:

Agreement Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms. Agreement For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where - The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer. - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information. - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214. - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set.

The topic was also discussed during RAN1#110e-bis with the following agreements:

Agreement For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected: - Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs - SL RSRP measurement results - Resource reservation periods based on decoded SCI and for own LTE SL transmissions - Priority based on decoded SCI and for own LTE SL transmissions - Time and frequency location of resources used for own LTE SL transmissions - Candidate resource set SA or SB - SL RSSI measurements - LTE logical subframe related information - Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE Agreement For dynamic resource pool sharing, the NR SL module uses the information shared by the LTE SL module to the NR SL module to determine the set of resources for its own transmission. - FFS: which layer carries out the resource determination: PHY layer or MAC layer. Agreement For dynamic resource pool sharing, where the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, continue studying the following alternatives: - Alt 1: The LTE SL module provides the NR SL module with the candidate information (excluding at least the candidate resource sets SA or SB) - The NR SL module identifies a set of resources based on information shared by the LTE SL module. - FFS: how to identify the set of resources - The NR SL module excludes these identified resources from its own candidate resource set when performing the resource (re)selection procedure. - The exclusion process is performed in the PHY layer. - Note: implementation of Alt 1 should not have specification impact to LTE - Alt 2: The LTE SL module provides the NR SL module with the candidate resource sets SA or SB shared by the LTE SL module - The LTE PHY SL module is provided information from the higher layer to generate a candidate resource set SA or SB. The resource set SA or SB is then shared to NR SL module. - The NR SL module performs an intersection operation with the candidate resource set received from the LTE SL module and the candidate resource set generated by the NR SL module. - FFS: how to handle the case where this results in an insufficient set of resources - The intersection operation is performed in the MAC layer. - FFS: How to handle NR V2X parameter settings that are not supported by LTE V2X, e.g., periodicities, sub-channel sizes, etc - Note: implementation of Alt 2 should not have specification impact to LTE - In the next meeting strive to decide between the two alternatives Agreement For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission. - T is defined using - T≤Tmax ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence. - FFS: Value of Tmax - FFS: any discussion on the earliest information, if needed

The topic was also discussed during RAN1#110e with the following agreements:

Working assumption Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module. Conclusion For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs. - Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning. - Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe - FFS: potential enhancements for synchronization can be further investigated Agreement For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints: - NR SL resource pool is configured with 15 kHz SCS. - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools - For NR PSFCH (if configured), at least the following alternatives are studied: - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions. - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH. - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots. - FFS: periodicities of the set.

The topic was also discussed during RAN1#109e with the following agreements:

Agreement For co-channel coexistence in Rel-18, no changes in the LTE SL specifications are allowed. Agreement For co-channel coexistence in Rel-18, Rel-16/17 simulation assumptions are reused for evaluation of solutions, except for the UE dropping model.  FFS: UE dropping model Agreement For the study of co-channel coexistence solutions in Rel-18, the combination of operational modes Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority.  FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C). Agreement For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models:  UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}.  UE Dropping Model B: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}. Companies should mention the UE dropping model and the distribution of each device type (single/dual module) used in their simulation assumptions. Agreement Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied. Agreement For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence,  For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module. o FFS details on how the NR SL module uses this information. o FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc.  FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL.  FFS: Whether/how device type B should be supported.