3GPP TSG RAN WG1 #112bis-e	R1-2302294

e-Meeting, April 17th - April 26th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On measurements and reporting for SL positioning

Agenda item:	9.5.1.2

Document for:	Discussion and Decision

# Introduction

At RAN#98, a new work item “Expanded and improved NR positioning” (NR_pos_enh2) was approved; the WID was most recently revised at RAN#99 [1]. The relevant WID objective is copied here for convenience:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we present our views on measurements and reporting for SL positioning.

# Discussion

From WID [1], Rel-18 NR positioning supports SL-AoA, SL-TDOA, and RTT-type solutions for SL positioning. To support these techniques based on SL PRS, we first need to discuss which measurements should be specified. In this regard, so far SL-PRS based Rx-Tx, RSTD, RSRP, RSRPP, RTOA, AoA and ZoA measurements have captured by RAN1 as contents of measurement report. But the reporting of these information might be the minimum reporting contents for SL positioning. On top of these measurements, the target UE may need to report additional information such as time stamp and the measurement quality. And we may need discussion on the reporting of the anchor UE location for absolute SL positioning. In the following we discuss more specific issues on measurement and reporting for each positioning techniques and provide our views considering vehicle UEs in particular.

## SL Positioning Measurements and Procedures

Deploying time or angle-based positioning method for SL positioning is expected to be both use-case dependent (e.g. V2X, IIoT, etc.) and UE-capability dependent (e.g. whether/to whom the SL UEs are synchronized, whether the SL UEs share a common coordinate system, number of UE antennas, whether the UE is RedCap, XR, LPHAP, etc.).

### SL-TDOA

With regards to SL-TDOA positioning method, the following was agreed in previous meetings:

Agreement (RAN1#110bis) With regards to SL-TDOA positioning method for SL-only positioning, - It corresponds to a method where SL-PRS is being transmitted from multiple anchor UEs to a target UE (i.e., DL-TDOA-like operation), and/or from a target UE to multiple anchor UEs (i.e. UL-TDOA-like operation) at least for the purpose of absolute positioning estimation of the target UE - Study the detailed procedure, necessary signalling for SL-TDOA, method(s) to mitigate impact of synchronization error between multiple anchor UEs including whether such method(s) are needed.

Agreement (RAN1#111) For SL-TDOA, DL-TDOA-like operation and UL-TDOA-like operation should be introduced. - A UE is not required to support both DL-TDOA-like operation and UL-TDOA-like operation

SL-TDOA technique is one of the efficient positioning methods as it does not need two-way signal transmissions between transmitter and receiver. According to the WID and the previous agreement, DL-TDOA-like operation and UL-TDOA-like operation will be introduced, and its concept can be illustrated as Figure 1.

Figure 1 - An illustrative example of DL-TDOA-like operation (left) and UL-TDOA-like operation (right)

As shown in Figure 1, in DL-TDOA-like operation, the target UE estimates RSTD measurement from SL PRSs transmitted by different anchor UEs. On the other hand, in UL-TDOA-like operation, the target UE transmits SL-PRS to multiple anchor UEs, and the anchor UEs measure RTOA similar to UL-TDOA.

More specifically, the target UE should report SL RSTD measurement including at least time stamp(s), and information on anchor UEs and SL PRSs to support DL-TDOA-like UE-assisted positioning, while the anchor UEs need to report SL RTOA measurement to support UL-TDOA-like operation. The time stamp information implies when the target UE measured the SL RSTD measurement for specific SL PRS transmitted from anchor nodes. The time stamp(s) would be helpful to obtain the target UE location and to assess the validity of location information of anchor UE(s).

In addition, for DL-TDOA-like operation, a reference timing is critical to the SL RSTD measurement accuracy, which should be determined and reported by the target UE. For example, a static anchor UE close to the target UE could be used as a good reference for the SL RSTD calculation.

Proposal 1: RAN1 support at least following features to support SL-TDOA positioning - For DL-TDOA-like operation, the target UE should report at least SL RSTD measurement(s) with anchor UE ID, a reference for SL RSTD, time stamp(s). - For UL-TDOA-like operation, the anchor UEs should report at least SL RTOA measurement(s) with target UE ID, time stamp(s), and the anchor UE ID.

#### Synchronization for RSTD and RTOA

In TDOA based SL positioning solutions, the positioning accuracy highly depends on the synchronization precession between the selected anchor UEs. SL positioning evaluation results in our tdoc for RAN1#111 [6] reflected the same where it is shown that the Set A requirements (error less than 1m@90%) cannot be met when synchronization error is introduced among anchor nodes, even with 100 MHz of SL PRS bandwidth for the InF-SH scenario. Therefore, well synchronized anchor UEs are required to support SL TDOA methods for meeting the accuracy requirements of SL positioning.

RAN1#112 discussed the anchor synchronization related aspects. The below captures a FL lead proposal [5] from RAN1#112 for which there was no conclusion reached due to time limitation.

#### [ROUND3][High] FL proposal 3.2.3-v1: Study whether and how to mitigate impact of synchronization errors between anchor UEs for TDoA measurement, considering the following alternatives: - Alt1: indication/reporting of e.g., synchronization source type, synchronization error or quality of synchronization information - Alt2: SL-PRS transmission between anchor UEs - Alt3: (pre-)configuration and capability report of the clock source’s accuracy requirement/threshold. If a UE does not satisfys the requirement, it does not participate in the positioning. - Others are not precluded

An awareness of time synchronization difference between the anchor UEs can enable selection of well synchronized anchor UEs or it may be used compensate for synchronization errors for a specific RSTD measurement. For example, the unit/entity computing the UE location (e.g., LMF) may know time synchronization difference between sync sources of anchor UEs such as cells/TRPs as it can provide information on RTD, so it may compensate for synchronization errors for a specific RSTD measurement to two different cells. Specifically, knowledge about the synchronization sources for each anchor UE may be used to compensate for time synchronization error included in SL RSTD measurement. This would be similar in UL-TDOA-like operation: the anchor UE needs to report synchronization sources such as GNSS, gNB, or a specific cell ID for both cases. In another example, the entity performing (e.g. LMF or target UE) that is performing the anchor selection can selected only those candidate anchors which are well synchronized with each other as its anchors to achieve high accuracy SL positioning.

In this regard, the anchor UE may need to provide sidelink synchronization related information e.g. anchor UEs synchronization source, co-synchronized other anchor UEs, synchronization level w.r.t. other anchor UEs, etc. to LMF or UE (e.g. target UE) in assisting anchor (re)selection. Here, the anchors may determine about their synchronization level to other anchor UEs using e.g. SL-PRS transmitted by the other UEs.

Proposal 2: Anchor UEs can report synchronization information incl. synchronization source, co-synchronized other anchor UEs, synchronization with respect to other anchor UEs or its own synchronization source.

#### Time difference measurement between different Rx antennas in a single UE

As shown in Figure 2Figure 2 of subsection 2.1.2, the vehicle type UEs may be equipped with at least two antennas or antenna panels, so we can consider RSTD measurement per receive antenna for specific SL PRSs transmitted from two different anchor nodes. For an anchor node which is not far from the target UE, we can additionally consider using a time difference measurement between two different receive antennas of the target UE in case they are connected to the same oscillator or time-offset is somewhat known. For example, the target UE can estimate two time of arrivals at the two receive antennas for a SL PRS transmitted by an anchor UE, and it calculates difference of them. The location variable would be the location of a receive antenna since the location of another receive antenna is fixed with respect to the first receive antenna location. Similarly, we can consider that anchor UE measures the time difference measurement between different receive antennas for a SL PRS transmitted from the target UE.

The distance between receive antennas is relatively large in the vehicles than mobile phones, so we may be able to use this measurement. It might be a new concept of RSTD measurement, but it would be beneficial as time synchronization between different anchor nodes does not affect the measurement, and it has a benefit in terms of resource efficiency as it is still RSTD measurement requiring one-way transmission only. Furthermore, since it is a time difference at the two receive antennas for a SL PRS transmitted from a transmission antenna, a TEG-related timing error of the anchor node is cancelled.

Proposal 3: For the UE which can cancel time offset between different receive antennas, support time difference measurement between different receive antennas with respect to a SL PRS.

#### SL PRS based Rx-Rx measurement

During the SI phase, RAN1 discussed an issue on the time difference of arrival of 2 SL-PRS transmitted at 2 different times from the same anchor. The below captures a FL lead proposal [4] for which there was no conclusion reached due to time limitation in RAN1#111.

##### [HIGH] Feature Lead Proposal 3.1.9-v1 - With regards to the reporting of time difference of arrival of 2 SL-PRS transmitted at 2 different times from the same anchor - Alt. 1: a new measurement should also be introduced - Alt. 2: such functionality should be introduced without the need of a new measurement - Alt. 3: study further how such functionality could be introduced without the need of a new measurement …

This measurement might correspond to a modified TDOA technique, but the benefit of this technique is unclear compared to TDOA using RSTD measurement. In the case of the RSTD measurement, the time sync between the receiver and transmitter is cancelled as the target UE measures multiple RSs at the similar time, but the consistency of the time synch error over two different time slots may not be guaranteed between the transmitter and the receiver. Also, the Rx TEG-related timing error at the target UE might not be the same at two different time slots. In addition, at the two different time slots, the UE can measure two RSTD measurements separately, and it is unclear that the Rx-Rx time difference-based positioning could be better than the TDOA technique using measurements obtained at the two different time slots. Thus, we propose to focus on the currently supported measurements for SL positioning.

Proposal 4: Deprioritize Rx-Rx time different measurement for SL positioning since its benefit over RSTD measurement is unclear.

### SL-AoA

With regards to SL-AoA positioning method, the following were agreed in previous RAN1 meetings:

Agreement (RAN1#110bis) For the study of SL-AoA positioning method, - Both SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement should be included - FFS: Definition of the measurements - Study further whether other measurements can be applicable - Study further the frame of reference (LCS or GCS) Agreement (RAN1#112) Support both GCS and LCS for SL-PRS based Azimuth of arrival (AoA) and zenith of arrival (ZoA) measurement. - FFS on the applicable scenario/service for AoA/ZoA relative to LCS without translation of the LCS to GCS

SL-AoA positioning technique will be introduced based on the WID [1]. For the measurement definition of SL Azimuth of arrival (AoA) and SL zenith of arrival (ZoA), the already defined UL AoA would be a good starting point for the discussion, but the detailed definition needs to be discussed in terms of SL PRS. For the study on other measurements for AoA positioning technique, RSRP measurement would be helpful, but it is not essential.

The vehicle UE may be able to measure both horizontal and vertical angle. However, it should be noted that the angle measurement from the target UE would depend on the location of the equipped antennas, especially if the target UE is close to the anchor UE. As illustrated in Figure 2, the angle measurement from Rx antenna panel #1 and #2 would not be the same. Also, the angle measurement at a specific antenna panel would depend on the antenna panel transmitting SL PRS. Thus, for the positioning accuracy, at least the location estimation entity (e.g., LMF) needs geographical location information on the transmission and reception antenna used to make the angle measurement(s).

The SL positioning supports unicast, groupcast, and broadcast. For the broadcast case, the measurement can be made by anchor UEs for an SL PRS transmitted from the target UE, and it would be very effective in terms of reference signal overhead. If the anchor UE is a receiver, the anchor UE should report angle measurements, ARP/panel information used to make the measurements, a target UE ID, and an SL PRS resource ID transmitted from the target UE. In this case, the target UE needs to report the used ARP/panel information to transmit the SL PRS, as the angle measurement may be different depending on the transmit antenna panel, and the location estimation entity (e.g., LMF) will consider this when it runs location estimation algorithm.

For the unicast scenario, the target UE could be a receiver. The target UE should report angle measurements, ARP/panel information used to make the measurements, anchor UE ID, and an SL PRS resource ID transmitted from the anchor UE. Also, the anchor UE needs to report the used ARP/panel information to transmit the SL PRS. Thus, we should consider both cases for SL-AoA positioning method, and we have the following proposal

Proposal 5: For SL-AoA positioning, - Case 1: positioning measurement from a target UE - For a SL PRS resource transmitted from an anchor UE, the target UE should report at least angle measurement (AoA/ZoA), ARP/panel info., anchor UE ID, and ID of the SL PRS transmitted from the anchor UE. - The anchor UE should report ARP/panel information used to transmit the SL PRS. - Case 2: positioning measurement from an anchor UE - For a SL PRS resource transmitted from a target UE, the anchor UE should report at least angle measurement (AoA/ZoA), ARP/panel info., target UE ID, and ID of the SL PRS transmitted from the target UE. - The target UE should report ARP/panel information used to transmit the SL PRS.

Figure 2 - An illustrative example of angle measurement among UEs.

### SL-RTT

With regards to SL-RTT positioning method, the following is agreed in the last RAN1 meeting:

Agreement (RAN1#112) For definition of SL-PRS based Rx-Tx measurement, downselect one of the following alternatives in RAN1# 112b to minimize the impact of UE reference timing offset and mobility - Alt1: actual SL-PRS transmission time is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt2: SL-PRS transmission time based on the sidelink PRS receiving symbol is used for the definition of SL-PRS based Rx-Tx time difference measurement - Alt3: based on the Rel-16/17 definition for gNB Rx-Tx time difference/UE Rx-Tx time difference in Uu.

Agreement (RAN1#111) With regards to the RTT-type solutions using SL, both single-sided and double-sided RTT methods should be introduced - Strive to minimize the changes needed on top of the specification support for single-sided RTT, if any, for the introduction of double-sided RTT. Note: a UE should be able to support single-sided RTT without having to support double-sided RTT

RTT-type solutions using SL, referred as SL-RTT henceforth, has been identified for possible introduction as SL positioning method. Here, both single-sided and double-sided SL RTT were agreed to be introduced.

In single-sided SL-RTT method, the target UE transmits SL PRS to anchor UE(s) and the anchor UE(s) transmits a response which include at least a response SL PRS and a Rx-Tx time difference measurement. While it requires single exchange of SL PRS and hence offers low latency performance, the Rx-Tx time difference measurement may be impacted by measurement error introduced by clock drift, synchronization reference (SyncRef) change. The severity of the error introduced due to clock drift depends on the clock stability and the response time. It is noted that, for a given UE that UE should have a maximum clock drift of 0.1 ppm according to RAN4 specifications in case the two UEs have the same synchronization source. As such it is easy to calculate the maximum error coming from clock drift in single-sided RTT.

emax=0.1×Tgap+0.1×Tgap+RTT

where Tgap is the response time i.e. time gap between the SL PRS reception from the initiating UE and the SL PRS transmission from the other UE, and RTT is the true RTT between the UEs. The 2nd term is dominated by Tgap for reasonable distances between the UEs. Therefore emax≈2×0.1×Tgap. Therefore, for reasonable values of Tgap (e.g., a few ms), the value of emax is on the order of 0.1 ns (or 3 cm) of error. However, only when clock drift is larger (in case of different synchronization sources), response time Tgap is large and/or there has been change in synchronization reference (SyncRef) within the response time, then the UE location estimation may have higher error.

Observation 1: The error for single sided SL RTT due to clock drift is on the order of 0.1 ns for short response time (e.g., a few ms) and for a maximum clock drift of 0.1 ppm. The performance may worsen only in case of larger clock drift, larger response time and change in SyncRef within the response time.

On the other hand, the double-sided (two-way) RTT method is argued to offer robust timing measurements in the presence of uncompensated clock offsets. This is achieved by introducing 2nd stage SL-RTT round (additional SL PRS exchange) i.e. taking into account Rx-Tx time difference measurements from both anchor UE and target UE perspective. However, this introduces additional signalling overhead and latency in positioning, so it should be restrictively used under strict conditions and the single-side RTT should be a default mode. In our understanding the double-sided RTT may be desired when very high accuracy is needed and in case of large response time (more than 1 second), significant clock drift and change in SyncRef within response time.

Observation 2: Double-sided RTT may be desired when very high accuracy is needed and in case of large response time (> 1s), significant clock drift and change in SyncRef within response time. However, it introduces additional signalling overhead and latency in positioning.

Since both methods have their own aforementioned advantages and limitations, one of the methods needs to be dynamical selected to minimize signalling overhead and latency while meeting accuracy requirements of the SL positioning session. To this end, the anchor UE can assist in SL RTT method since it may be aware of its own clock drift characteristics considering the response time and SyncRef change within the response time, and can may expect error in Rx-Tx measurements. In this regard, the anchor UE may indicate to the target UE or the method selection entity the need for switching from e.g. single-sided RTT to double-sided SL RTT when the measurement error is expected to be higher considering its clock drift characteristics, response time and/or change in SyncRef. Alternatively, the anchor UE can report expected clock error, SyncRef change and/or expected measurement error to the target UE or the method selection entity (e.g. server UE), so that selection of SL RTT method can be done in an informed manner. Furthermore, to minimize the impact on specification support, single-sided and double-sided RTT can be viewed as SL RTT with one RTT stage and two RTT stages, respectively. Then, based on such a report from anchor UE, an RTT stage (single-stage RTT) is complemented with an additional stage of RTT (i.e. transitioned to double-sided RTT) only when a single RTT stage cannot ensure the required accuracy performance at the target UE.

Proposal 6: In SL RTT, UE reports expected error for Rx-Tx measurement, SyncRef change status and/or expected clock error to the other UE.

Proposal 7: In SL RTT, UE indicates to the other UE the need for switching from one SL RTT method to another.

#### Report-free SL-RTT

RAN1 discussed a scenario of SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement. The below captures a FL lead proposal [5] from RAN1#112 for which there was no conclusion reached due to time limitation.

[ROUND2][Meduim] FL proposal 3.1.3-v2: For SL-PRS based Rx-Tx measurement, study the benefits and necessity of introducing functionality of SL Rx-Tx measurement not being reported but the SL-PRS transmission time being adjusted based on the measurement.

In report-free SL RTT method, anchor UE is made to respond to the target UE by applying an offset equal to a fixed Rx-Tx time so that the anchor UE does not need to report Rx-Tx time to the target UE. This reduces signalling overhead and avoids the need at anchor UE to perform additional resource selection and transmission of Rx-Tx measurement. Hence, adoption of this method can be beneficial.

However, having a fixed Rx-Tx time may be too restrictive to anchor UE to comply with since e.g. it may not be able to find suitable interference-limited radio resources to transmit as per the fixed Rx-Tx time. For example, the anchor UE may need to wait for a later slot to transmit the responding SL PRS due to resources being reserved/used by other UEs. In that case (i.e. when anchor UE is not able to strictly meet the fixed Rx-Tx time difference), positioning estimate at the target UE based on fixed Rx-Tx time may not be accurate. This performance degradation can be critical for use cases such as V2X, IIoT and public safety, etc. which require high positioning accuracy. But, having multiple fixed Rx-Tx time values may enable the anchor UE to flexibly select suitable transmission time of SL PRS and hence SL RTT response time (Rx-Tx time). Then, however, how a target UE can understand which fixed Rx-Tx time is applied at the anchor UE needs to be dealt. To this end, multiple Rx-Tx time values (a pool of Rx-Tx time values) can be configured for a report-free SL RTT session between a target UE and an anchor UE, where each Rx-Tx time value is associated with (or mapped to a) a specific SL PRS configuration (e.g. SL PRS sequence, time-frequency resource). In other words, a unique SL PRS configuration is used to indicate a given Rx-Tx time difference. Then, based on the characteristics of received SL PRS, the target UE can derive the Rx-Tx time applied at the anchor UE. This allows to exploit the benefits of report-free RTT while providing the flexibility to the anchor UE in setting the Rx-Tx time for its response.

Observation 3: Having a fixed Rx-Tx time may be too restrictive to anchor UE to comply with since e.g. it may not be able to find suitable interference-limited radio resources to transmit as per the fixed Rx-Tx time.

Proposal 8: Introduce functionality of SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement.

Proposal 9: Investigate possibility of having multiple fixed Rx-Tx time values for functionality of SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement.

## SL Positioning Measurement Report

The following agreements were reached in RAN1#112:

Agreement Study measurement report content for both the cases of sidelink positioning measurement reported to LMF and UE. Agreement Study the necessity and scenarios of including location information and quality information of the location of a UE in sidelink positioning measurement report, considering different measurements and different reporting targets (LMF and UE). Agreement Study the following candidates for identification information in sidelink positioning report, considering different measurements and different reporting targets (LMF and UE): - SL-PRS resource ID/SL-PRS resource set ID if multiple resources/resource sets are configured to a UE - FFS: whether SL-PRS resource set is supported - Source ID and/or destination ID - Other identification information not precluded Agreement LoS/NLoS indicator can be included in a sidelink positioning measurement report, considering different reporting targets (LMF and UE). - LOS/NLOS indicator specified in Rel-17 positioning is reused as much as possible. - No specification impact for how to set this indicator. - From RAN1 perspective, no performance requirements are expected to be defined for setting indicator in Rel-18. Agreement Companies are encouraged to provide expected measurement report content in the following table to facilitate discussion in RAN1 #112bis-e. Note: this does not imply a different measurement report content for reporting to LMF or to UE. Table 6.2 Collection of measurement report content reporting to LMF; reporting to UE. SL-PRS based Rx-Tx measurement. SL-PRS based RSTD measurement. SL-PRS based RSRP measurement. SL-PRS based RSRPP measurement. SL-PRS based RTOA measurement. SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement. etc.

Depending on which entity i.e. LMF or UE is responsible for positioning estimate computation, the SL positioning measurement may need to be reported to LMF or UE. It is noted that the network node LMF and UE perform the positioning estimation in network-based and UE-based positioning, respectively. Further, the server UE may perform positioning estimation for UEs in particular in out-of-coverage scenarios.

Apart from the positioning estimation computation, the SL positioning measurement may also be used for SL positioning method selection or to compute corrections (e.g. time correction for anchors to minimize the impact of synchronization errors between anchor UEs for TDoA) to applied in positioning estimation. Since both LMF and UE (e.g. server UE) may perform e.g. SL positioning method selection, the SL positioning measurement may need to be reported either LMF or UE.

Proposal 10: SL positioning report contents are as captured in Table 1. Table 1: SL positioning measurement report reporting to LMF; reporting to UE; additional comments. SL-PRS based Rx-Tx measurement; P; P. SL-PRS based RSTD measurement; P; P; can also be measured between different Rx antennas at a single UE. SL-PRS based RSRP measurement; P; P. SL-PRS based RSRPP measurement; P; P. SL-PRS based RTOA measurement; P; P. SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement; P; P. LoS/NLoS indicator; P; P. Quality of measurement; P; P; Or expected measurement error. Time stamp; P; P. Antenna panel ID; P; P; for Rx-Tx, RSTD, RSRP, RSRPP, RTOA, AoA, ZoA measurements, and LoS/NLoS indicator. Antenna location information; P; P; with respect to e.g. a reference location, incl. both distance and angle info. Anchor UE IDs; P; P; for at least DL TDOA-like SL TDOA method. Reference Anchor UE for SL RSTD; P; P; for at least DL TDOA-like SL TDOA. Target UEs; P; P; for at least UL TDOA-like SL TDOA. Synchronization source; P; P; for at least SL TDOA method. Time difference in synchronization with other anchor UEs and own synchronization source; P; P; for at least both DL-TDOA-like and UL-TDOA-like SL TDOA. SyncRef change status; P; P; for at least SL RTT method. Expected clock error; P; P; for at least SL RTT method.

One of the most promising use cases of the SL positioning would be V2X positioning, and the size of vehicle type UEs would be relatively larger than the smartphone or tablet type UEs. There may be a couple of points we should consider. The first one is that the location of Tx/Rx antenna at the vehicle. Depending on the location of the receive antenna panel at the vehicle, the estimated location would be different. For example, if the receive antenna is at the rear side of the vehicle UE, the estimated location of this UE would be there. Thus, even if the UE has a single antenna panel, the location of the antenna panel may affect the location estimation of the UE. Also, the vehicle UE may be equipped with multiple antenna panels. For example, vehicle UEs may have one antenna panel on both the front and back bumpers. Even in the same UE, LoS/NLoS conditions of wireless channel to another UE and/or positioning measurement may be different at different antenna panels. Note that RAN1#112 agreed that LoS/NLoS indicator can be included in a sidelink positioning measurement report.

Proposal 11: Introduce positioning measurement and LoS/NLoS indication per panel or per ARP for sidelink positioning.

# Conclusions

In this contribution, we have made the following observations and proposals related to measurements and reporting for SL positioning:

Proposal 1: RAN1 support at least following features to support SL-TDOA positioning - For DL-TDOA-like operation, the target UE should report at least SL RSTD measurement(s) with anchor UE ID, a reference for SL RSTD, time stamp(s). - For UL-TDOA-like operation, the anchor UEs should report at least SL RTOA measurement(s) with target UE ID, time stamp(s), and the anchor UE ID.

Proposal 2: Anchor UEs can report synchronization information incl. synchronization source, co-synchronized other anchor UEs, synchronization with respect to other anchor UEs or its own synchronization source.

Proposal 3: For the UE which can cancel time offset between different receive antennas, support time difference measurement between different receive antennas with respect to a SL PRS.

Proposal 4: Deprioritize Rx-Rx time different measurement for SL positioning since its benefit over RSTD measurement is unclear.

Proposal 5: For SL-AoA positioning, - Case 1: positioning measurement from a target UE - For a SL PRS resource transmitted from an anchor UE, the target UE should report at least angle measurement (AoA/ZoA), ARP/panel info., anchor UE ID, and ID of the SL PRS transmitted from the anchor UE. - The anchor UE should report ARP/panel information used to transmit the SL PRS. - Case 2: positioning measurement from an anchor UE - For a SL PRS resource transmitted from a target UE, the anchor UE should report at least angle measurement (AoA/ZoA), ARP/panel info., target UE ID, and ID of the SL PRS transmitted from the target UE. - The target UE should report ARP/panel information used to transmit the SL PRS.

Observation 1: The error for single sided SL RTT due to clock drift is on the order of 0.1 ns for short response time (e.g., a few ms) and for a maximum clock drift of 0.1 ppm. The performance may worsen only in case of larger clock drift, larger response time and change in SyncRef within the response time.

Observation 2: Double-sided RTT may be desired when very high accuracy is needed and in case of large response time (> 1s), significant clock drift and change in SyncRef within response time. However, it introduces additional signalling overhead and latency in positioning.

Proposal 6: In SL RTT, UE reports expected error for Rx-Tx measurement, SyncRef change status and/or expected clock error to the other UE.

Proposal 7: In SL RTT, UE indicates to the other UE the need for switching from one SL RTT method to another.

Observation 3: Having a fixed Rx-Tx time may be too restrictive to anchor UE to comply with since e.g. it may not be able to find suitable interference-limited radio resources to transmit as per the fixed Rx-Tx time.

Proposal 8: Introduce functionality of SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement.

Proposal 9: Investigate possibility of having multiple fixed Rx-Tx time values for functionality of SL Rx-Tx measurement not being reported but the transmit time of SL-PRS being adjusted based on the measurement.

Proposal 10: SL positioning report contents are as captured in Table 1. Table 1: SL positioning measurement report reporting to LMF; reporting to UE; additional comments. SL-PRS based Rx-Tx measurement; P; P. SL-PRS based RSTD measurement; P; P; can also be measured between different Rx antennas at a single UE. SL-PRS based RSRP measurement; P; P. SL-PRS based RSRPP measurement; P; P. SL-PRS based RTOA measurement; P; P. SL-PRS based Azimuth of arrival (AoA) and SL zenith of arrival (ZoA) measurement; P; P. LoS/NLoS indicator; P; P. Quality of measurement; P; P; Or expected measurement error. Time stamp; P; P. Antenna panel ID; P; P; for Rx-Tx, RSTD, RSRP, RSRPP, RTOA, AoA, ZoA measurements, and LoS/NLoS indicator. Antenna location information; P; P; with respect to e.g. a referece location, incl. both distance and angle info. Anchor UE IDs; P; P; for at least DL TDOA-like SL TDOA method. Reference Anchor UE for SL RSTD; P; P; for at least DL TDOA-like SL TDOA. Target UEs; P; P; for at least UL TDOA-like SL TDOA. Synchronization source; P; P; for at least SL TDOA method. Time difference in synchronization with other anchor UEs and own synchronization source; P; P; for at least both DL-TDOA-like and UL-TDOA-like SL TDOA. SyncRef change status; P; P; for at least SL RTT method. Expected clock error; P; P; for at least SL RTT method.

Proposal 11: Introduce positioning measurement and LoS/NLoS indication per panel or per ARP for sidelink positioning.

# References

- RP-230328, “Expanded and Improved NR Positioning”

- 3GPP TR 38.859, Study on expanded and improved NR positioning

- 3GPP TR 23.700-86, “Technical Specification Group Services and System Aspects; Study on Architecture Enhancement to support Ranging based services and sidelink positioning; (Release 18)”

- R1-2212938, “Moderator Summary #4 on potential solutions for SL positioning”

- R1-2301952, “Summary #3 on Measurements and reporting for SL positioning”

- R1-2210831, “Evaluation of SL positioning”