3GPP TSG RAN WG1 #112bis-e	R1-2302295

e-Meeting, April 17th - April 26th, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	On resource allocation for SL positioning reference signal

Agenda item:	9.5.1.3

Document for:	Discussion and Decision

# Introduction

At RAN#98, a new work item “Expanded and improved NR positioning” (NR_pos_enh2) was approved; the WID was most recently revised at RAN#99 [1]. The relevant WID objective is copied here for convenience:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre)-configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we present our initial views on resource allocation for the SL positioning reference signal (SL-PRS).

# Discussion

## Resource allocation

### Resource pools

In the previous RAN1 meetings, the following agreement has been reached regarding SL resource pool usage for SL positioning, in particular for SL PRS transmissions.

Agreement (RAN1#111) With regards to the SL Positioning resource allocation, support - Alt. 2: either dedicated resource pool(s) and/or a shared resource pool(s) with sidelink communication can be (pre)-configured for SL-PRS. - Note: this does not imply that the design is the same for both types of resources pools - Note: shared resources pool(s) should be supported with backward compatibility

From the formulation of Alt. 2 with ‘either.. and/or..’ in the aforementioned agreement, it is not clear if both dedicated and shared resource pools can be configured at a given time. Using dedicated resource pool(s) may result in waste of the scarce SL resources if dedicated resource pools are utilized on demand, only when a request for positioning arises. Instead, using shared pools offers a more efficient alternative in terms of resource utilization. Nevertheless, a dedicated pool would allow transmission of SL PRS without any collisions with legacy transmissions, in case of sufficient number of dedicated resources. We therefore prefer to have the possibility of (pre)-configuring both dedicated and shared pools (also including either one of them) for SL PRS transmissions.

Proposal 1: For SL PRS transmission, support (pre-)configuration of either both dedicated and shared resource pools with (Rel-16/17/18) SL communication, or only one of them.

In addition, the bandwidth of SL PRS is discussed in the previous RAN1 meeting and the following is agreed:

Agreement (RAN1#110bis) At least for a dedicated resource pool for positioning, - With regards to the bandwidth of SL-PRS transmission, downselect from the following alternatives: - Alt. 1: The bandwidth of SL-PRS can be same or smaller than that of the resource pool - Alt. 2: The bandwidth of SL-PRS shall be the same as that of the resource pool - Note: Companies are encouraged to provide their analysis and views on the above alternatives - FFS: Bandwidth of SL-PRS transmission for shared resource pool (if supported)

Accuracy of positioning methods, especially the time-based ones, depend highly on the bandwidth of the transmitted reference signals. Given that different positioning requests may have different QoS requirements in terms of accuracy, the bandwidth of SL-PRS should be (pre)-configurable based on the accuracy requirements, irrespective of whether the resource pool for positioning is dedicated or shared. This means, the SL-PRS bandwidth is not necessarily equal to the size of the resource pool utilized.

Proposal 2: For both dedicated and shared resource pools for SL positioning, the bandwidth of SL-PRS shall be (pre)-configurable, e.g., depending on positioning accuracy requirements, which can be same or smaller than that of the resource pool.

Further, in order to accommodate SL PRS transmissions that occupy wide bandwidth, UEs could make use of multiple SL resource pools that share the same set of time resources. This could also include dedicated resource pools (pre)-configured for SL PRS, which may overlap with other SL resource pools. Network can (pre)-configure such resource pools, which UEs do not need to constantly monitor for their SL data transmissions, but only utilize when a need arises for transmitting/receiving a SL PRS with large bandwidth for a given positioning session by aggregating them. To enable such aggregation, additional cooperation among UEs that utilize these pools regularly is needed, namely to monitor and reserve SL resources in respective pools for the transmission of the SL PRS by aggregating the pools.

Proposal 3: A SL PRS transmission may occupy a wide bandwidth spanning across multiple SL resource pools. For monitoring and reserving SL resources across multiple resource pools, study methods that enable coordination between UEs that are configured with different resource pools.

RAN1 has made the following agreements with regards to channel(s) to be included in dedicated pool in addition to SL-PRS.

Agreement (RAN1#110bis) For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included - FFS: Details - FFS: definition of PSSCH associated with SL-PRS transmission(s) - Note: Companies are encouraged to provide their analysis and views on the above

Agreement (RAN1#112) For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

In dedicated resource pools for SL positioning, having other channels beyond SL-PRS transmission(s) would defragment the resources, which would result in reduced bandwidth or number of resources available for SL-PRS transmissions, even if the resources are dedicated for SL PRS.

If PSCCH is also included in the dedicated pools, it can be used to carry SCI associated with SL PRS transmissions, as in legacy SL transmissions. However, the size of SCI would be quite limited, and may not enable forward compatibility (allowing for introduction of new fields in next releases). For this, a second stage SCI can be transmitted in PSSCH, as in legacy SL. While PSSCH enables transmission of larger control data size, and flexibility in terms of forward compatibility, this would consume up the remaining resources available for SL PRS in a higher extend.

On the other hand, introducing PSFCH might be beneficial since it can also carry control information associated with SL PRS transmissions, such as to be used for transmitting a feedback indication from the intended receiver of SL PRS. Further, PSFCH in dedicated resource pools can be designed to occupy limited number of time/frequency resources as in legacy SL resource pools.

Therefore, among the Option 2 and 3, Option 2 is preferable and any further control signaling associated with SL-PRS that does not fit into PSCCH (or PSFCH) in the dedicated pool, as well as other positioning-related data can be transmitted in the shared resource pool(s). Such signaling may also contain, e.g., any assistance data that might be part of SLPP signaling. To further increase amount of available resources for SL PRS, in a dedicated pool, periodic PSCCH resources can be configured, i.e., a SL PRS slot with PSCCHs occurs once in every K slots and the other SL PRS slots only contain SL PRSs. The parameter K is configurable and can be configured as 1.

Proposal 4: Dedicated resource pool for SL positioning should not include any other channel or transmission beyond SL-PRS and PSCCH that carries associated SCI. FFS whether PSFCH is included.

Proposal 5: Any signaling associated with a SL PRS transmission in a dedicated resource pool that does not fit into associated control channels (e.g., PSCCH or PSFCH) can be transmitted in an associated resource pool (which could be a shared resource pool or SL-communications-only resource pool).

Proposal 6: Consider supporting SL PRS slots with/without PSCCHs in a dedicated resource pool: a SL PRS slot with PSCCHs can be configured occurring once in every K slots; the parameter K is (pre-)configurable and can be configured as 1.

### Multiplexing of SL PRS with other SL transmissions

In the case of congested SL resource pools, it may not be possible for the UEs to find SL resources that are sufficiently wide enough in frequency domain so as to transmit a SL PRS occupying a large bandwidth. Under such cases, UEs can transmit a SL PRS that is segmented over different time resources, with (partial) overlap in frequency domain or connected with a contiguous set of resources in time domain that would enable successful reconstruction of the reference signal, as indicated in Figure 1.

Figure 1 - Transmission of SL PRS across segmented parts

Further, for a resource pool with only Rel-18 UEs, gNB can (pre)-configure different frequency offsets to PSFCH resources in PSFCH slots in a period (e.g. two or three PSFCH slots in a period) such that PSFCH resources in the slots occupy different frequency locations. The unused frequency resources are available for SL PRS and the SL PRS resources in the PSFCH slots can be stitched together to occupy the whole bandwidth of sidelink resource pool.

Proposal 7: SL PRS can be transmitted in multiple segments whereby different SL PRS segments - may be transmitted in different time slots - partially overlap in frequency domain - jointly cover wider bandwidth whereby the following transmission options are considered: - SL PRS segments are FDM’ed with data (PSSCH). - SL PRS segments are FDM’ed with PSFCH.

### RE-level multiplexing of SL PRS and associated Scheme 2 impacts

With regards to multiplexing of SL PRS from different UEs, RAN1 has reached the following agreement:

Agreement (RAN1#112) - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs

Considering the possibility of multiplexing SL PRS transmissions of different UEs at RE-level, the Scheme 2 resource allocation requires enhancements over the legacy NR SL Mode 2 resource allocation since it was based on subchannel-level granularity. In order to enable RE-level multiplexing of users, modifications to the UE behavior, sensing procedure, and control signaling mechanisms would be necessary.

Observation 1: RE-level multiplexing of SL PRS transmissions of different UEs necessitates enhancements over the legacy NR SL resource allocation Mode 2 to allow for Scheme 2 resource allocation for SL positioning.

For example, in the case of shared resource pools, the legacy Mode 2 resource preemption mechanism would result in UEs to unnecessarily preempt their selected resources for SL transmissions upon sensing that these resources are reserved for SL positioning. However, it would be in fact possible for them to reuse these reserved time/frequency resources via RE-level multiplexing. In order to allow this, the UE transmitting SL PRS can indicate such multiplexing possibility to the other transmitters (SL communication UEs), so as to enable better resource utilization efficiency. UEs may also perform sensing using particular comb size and comb offsets in order to determine if the resulting power measurement (e.g. RSSI) allows for RE-level multiplexing.

Proposal 8: For Scheme 2 resource allocation in shared pools, UE transmitting SL PRS may provide feedback to other transmitting UEs to avoid their resource preemption and reuse the same time/frequency resources via RE-level multiplexing.

Proposal 9: For Scheme 2 resource allocation in at least dedicated resource pools, UE performing sensing SL PRS may use comb size and comb offset as parameters during sensing to determine if reuse of the same time resources via RE-level multiplexing is possible. This sensing operation may be (pre)-configurable.

In order to efficiently utilize SL resources and reduce channel congestion, SL PRS transmitted by multiple SL positioning anchors in vicinity can be also multiplexed using same time frequency resources, however occupying sufficiently orthogonal sequences in code domain. To determine whether such resource re-use would be acceptable, additional mechanisms are necessary that would involve feedback from respective target UEs. In this regard, upon monitoring a SL PRS resource reservation by another anchor UE (UE-A2), an anchor UE (UE-A1) may use same time frequency resources but with sufficiently orthogonal sequences (for target UEs) to improve resource utilization. However, given that SL PRS sequences may not be completely orthogonal, whether or not the intended target UEs of both anchor UEs (UE-T1 and UE-T2) can successfully receive their respective SL PRS may not be known at the anchor UE. Therefore, the anchor UE can announce its intention (e.g. via SCI) to reuse one or more reserved SL PRS (reserved by another anchor UE) with chosen SL PRS sequence. Then the target UEs of both the anchor UEs (another anchor UE that has already reserved the resource and the anchor UE that intends to reuse it) can respond to such announcement with whether or not the multiplexing of the said SL PRS is acceptable to them. Here, HARQ-like feedback (in e.g. PSFCH resource associated with the intent announcement SCI) can be used by target UEs for their response. For example, if such multiplexing is not acceptable by any of the target UE, it sends NACK in the associated PSFCH resource. In this case, the anchor UE will not perform sequence level multiplexing. In case no NACK is received, the anchor UE will transmit SL PRS with the indicated sequence in the same time frequency resource reserved by another anchor UE as illustrated in Figure 2.

Figure 2: A scheme for code-domain multiplexing of SL PRS

Proposal 10: For code-domain multiplexing (CDM) of SL PRS transmissions, study IUC like mechanism to determine if SL PRS sequence multiplexing is acceptable at the receiver UE.

The same mechanism can be also employed for comb-based multiplexing of SL PRS transmitted by multiple UEs. Namely, UE-A1 may reuse the same time/frequency resources reserved by UE-A2, by using a separate comb offset. The comb-level multiplexing, however, is prone to the IBE problem. Depending on the mobility conditions among the UEs, the multiplexed SL PRS transmissions may lose orthogonality if they are using adjacent comb resources, e.g., due to high relative speed yielding Doppler effect. Therefore, whether such comb-level multiplexing would be acceptable or not, as well as the selection of the comb offset/size can be feedback to UE-A1 by the intended receivers UE-T1 and UE-T2. The same ACK/NACK mechanism described above for code-domain multiplexing is therefore applicable to comb-based multiplexing as well.

Proposal 11: For frequency-domain multiplexing of SL PRS transmissions, study IUC like mechanism to determine if comb-based multiplexing of SL PRS is acceptable at the receiver UE.

### Scheme 2: UE scheduling other UEs and IUC

In the previous RAN1 meetings, the following agreements on resource allocation for SL positioning were reached:

Agreement (RAN1#110) Regarding SL-PRS resource allocation, both Scheme 1 and Scheme 2 should be introduced for supporting SL positioning/ranging: - Scheme 1: Network-centric operation SL-PRS resource allocation (e.g. similar to a legacy Mode 1 solution) - The network (e.g. gNB, LMF, gNB & LMF) allocates resources for SL-PRS. - Scheme 2: UE autonomous SL-PRS resource allocation (e.g. similar to legacy Mode 2 solution) - At least one of the UE(s) participating in the sidelink positioning operation allocates resources for SL-PRS Agreement (RAN1#110) Regarding Scheme 2 SL-PRS resource allocation, study at least the following aspects: - Resource selection mechanism for SL-PRS - Inter-UE coordination - Aspects for congestion control mechanisms for SL-PRS

Agreement (RAN1#110bis) With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication

With regards to Scheme 2, the below captures a FL lead proposal [4] from RAN1#111 for which there was no conclusion reached due to time limitation.

##### [LOW] Feature Lead Proposal 4.4.3-v0 For Scheme 2, with regards to Inter-UE coordination for SL-PRS: - Option 1: study Rel-17 Scheme 1 IUC procedure as a starting point - Option 2: study Rel-17 Scheme 2 IUC procedure as a starting point - Option 3: study Rel-17 Scheme 1 or and 2 IUC procedure as a starting point - Option 4: Inter-UE coordination procedures can be deprioritized

In legacy Uu positioning, the central role of gNBs can be conveniently used to implement the underlying resource allocation tasks efficiently. However, this is not possible in distributed resource allocation typical for out-of-coverage or partial-coverage scenarios when SL UEs conduct resource allocation by themselves without the involvement of the gNB. Further, several positioning methods such as multi-RTT or OTDOA require coordinated PRS transmission by multiple UEs within pre-defined times or time intervals. It is therefore of interest to study collective resource allocation scheme where one UE (for example the target UE) allocates the resource for multiple PRS transmissions by itself and/or other UEs (such as its anchor UEs).

Proposal 12: Study the resource allocation mechanisms that allow one UE to allocate multiple SL PRS transmission opportunities for itself and/or other UE(s) in a SL positioning session.

Naturally, the priority of the resource allocation requests on behalf of multiple TX UEs (e.g., DL-like SL PRS transmission by multiple anchor UEs to target UE), or intended for multiple RX UEs (e.g., UL-like reception of target UE SL PRS by multiple anchor UEs), or with specific timing constraints (e.g., for TDOA or RTT methods) should be raised appropriately to ensure competitiveness with respect to standard single-UE resource requests. For example, the SCI signaling can have priority increased proportionally to the number of TX / RX UEs involved or with more stringent timing constraints.

Proposal 13: In the case of a SL PRS allocation for multiple TX and/or RX UEs or under strict timing constraints, study how the priority of resource allocation can be raised as a function of the number of involved UEs and/or timing constraints.

Further, when multiple target UEs are involved with an anchor UE and are allocating or indicating (non-)preferred SL PRS resources to the anchor UE, there may be conflicts in allocated or (non-)preferred SL-PRS resources (i.e., SL-PRS resources indicated by the target UEs may not be preferred for each other). Hence, avoidance or handling of any potential conflicts of SL-PRS resources needs to be studied.

Proposal 14: Study SL PRS resource conflict avoidance/resolution for IUC scenarios involving a UE transmitting (multicast) SL PRS to multiple UEs based on IUC assistance received from them.

For example, a target UE may determine that a resource conflict is occurring as there are overlapping resources used from different anchor UEs as per their respective SL PRS configuration. In this case the target UE may request one or more anchor UEs to adapt or change their PRS configurations. As such a request may impact other target UEs that are also using the same SL PRS from anchor UEs, and hence it may be best to modify the SL PRS configurations as little as possible. Another option is to have the SL PRS be FDM by use of comb offsets similar to in Uu positioning which are coordinated among the anchor UEs. For example, the SL PRS of one UE and another UE can be FDM using different comb offsets by coordination via IUC (i.e., one UE suggests a preferred comb offset for another UE). Another option is to rely on CDM for overlapping resources by using appropriate SL PRS sequences. Again using IUC it may be possible for UEs to coordinate on sequence selection and/or indicate if CDM is possible for particular time-frequency resources.

One option is to introduce SL PRS muting similar to DL PRS which allows certain occasions of a periodic SL PRS to be muted. SL PRS muting may be useful in the case where an anchor UE needs to select a SL PRS configuration (e.g., comb offset or sequence) which is not preferred by another UE. This can happen if one anchor is serving multiple UEs or if anchor UE has another reason to select a non-preferred parameter for a particular UE. In the case of periodic SL PRS transmission then the anchor UE can apply a muting pattern to give the UE (which did not prefer that configuration) to still receive other signals during some occasions.

Proposal 15: Study at least the following methods for SL PRS conflict avoidance: comb offset coordination, SL PRS sequence selection, and CDM to share the same time-frequency resource (e.g., via IUC) and SL PRS muting patterns.

## Configuration/activation/deactivation/triggering of SL-PRS

In the previous RAN1 meetings, the following agreements have been reached:

Agreement (RAN1#110) - With regards to the configuration/activation/deactivation/triggering of SL-PRS, Option 3 from the previous corresponding RAN1 #109 agreement will not be considered further. - With regards to reservation of SL-PRS, it can be considered based on the Option 1 or Option 2 from the previous corresponding RAN1 #109 agreement. Agreement (RAN1#109) With regards to the configuration/activation/deactivation/triggering of SL-PRS, study the following options: - Option 1: High-layer-only signaling involvement in the SL-PRS configuration - No Lower layer involvement, e.g. SL-MAC-CE or SCI or DCI, for the activation or the triggering of a SL-PRS. - Based on the study, this option may correspond to - A SL-PRS configuration that is a single-shot or multiple shots - A high-layer configuration that may be received from an LMF, a gNB, or a UE - Option 2: High-layer and lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - For example, high layer signaling can may be used for SL-PRS configuration and lower layer signaling can may be used for initiating SL positioning and/or configuration/triggering/activating/deactivating/indicating and potential resource indication/reservation transmission of SL-PRS. - Option 3: Only lower-layer signaling involvement in the SL-PRS configuration - Lower-layer may correspond to SL-MAC-CE, or SCI, or DCI - Note 1: Include aspects in the study related to flexibility, overhead, latency, and reliability as/if needed.

Agreement (RAN1#110bis) With regards to the SL-PRS time domain behavior, at least study the following behaviors from Tx UE perspective: - Periodic SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity - FFS: any additional details, including whether or not higher layers can start/stop transmission. - Semi-persistent SL-PRS - SL-PRS is transmitted periodically with a transmission periodicity after activation and until deactivation - FFS: any additional details - Aperiodic SL-PRS - SL-PRS is transmitted at least once after [triggering/request] - Note: the brackets in the above means that companies are encouraged to study further whether “triggering” and/or “request” should be used and provide their definitions. - FFS: any additional details - FFS: Applicability of the above time behaviors for scheme 1 & scheme 2 - FFS: Rx UE behavior is separately discussed. - FFS: What mechanism(s) are used for activation/deactivation/triggering is part of the study

Since there is no central entity like LMF in out of coverage SL positioning/ranging scenarios, there is a need for a way to configure SL-PRS transmissions. In this regard, a straightforward way is to utilize coordination among peer UEs for SL positioning, i.e., enable, e.g., a target UE to configure or assist in configuration of SL-PRS transmission of, e.g., an anchor UE, by means of providing SL-PRS configuration assistance information to the anchor UE as per its preference. Here, the assistance information can include details related to, e.g., types of positioning measurements, SL-PRS characteristics (e.g., its bandwidth determined based on the accuracy requirements).

Proposal 16: A UE (e.g., target UE) may assist configuring parameters of SL PRS (e.g., bandwidth, periodicity, etc.) for other UEs (e.g., anchor UEs) such as by providing (non-)preferred set of SL PRS parameters.

Given a SL PRS (pre-)configuration, its continuous transmission, e.g., by anchor UEs, as in legacy DL PRS transmissions, would highly occupy the SL resources, and may not be needed at all times since the UEs are expected to be mobile in most of the scenarios. Instead, SL PRS, which can be periodic or aperiodic, can be (pre-)configured by the network and can be activated based on presence of the intended receiver(s), coarse location (e.g., area) of the anchor UEs, as well as over a certain time duration, which can be also (pre-)configured by the network.

Proposal 17: Periodic or aperiodic (pre-)configured SL PRS can be activated or deactivated based on indications coming from higher layers, e.g., based on parameters such as distance and/or time thresholds that are (pre-)configurable.

The configuration of resources for SL-PRS transmission must ensure the desired SL-PRS reception quality. But how a UE selects an appropriate SL-PRS configuration, in particular when the UE is not aware of channel towards the other UE (e.g. in the initial stages of SL positioning session) remains as a key open question. While the UE may always choose the most conservative SL-PRS configuration to ensure high SL-PRS reception quality, this may be resource inefficient and therefore may not scale well e.g. in denser UE deployment scenarios. To this end, SL-PRS configuration needs to be tuned as per changing propagation and interference conditions between peer UEs of SL positioning. In this regard, a closed-loop type of approach for tuning the SL-PRS configuration to the propagation and interference conditions particular to each target-anchor link can be introduced. A reference signal e.g., high density SL-PRS which, when applied different masks on it, emulates different SL PRS sent from one UE (anchor UE) to the other UE (target UE). Since the SL-PRS masking emulates the effect of sending differently configured SL-PRS, i.e., with different time/frequency/space patterns and densities, it can help the other UE (target UE) to anticipate how a SL-PRS configuration will impact the accuracy of the subsequent SL positioning measurements/estimates. Here, by mask, we mean a binary (boolean) matrix, which when applied on top of a reference signal matrix, mutes some of the entries of the later matrix. Then, the other UE (target UE) can compute and report to the UE (anchor UE) channel information (e.g., RSRP, RSSI) per mask and this information can then be used at the UE to determine the SL-PRS configuration.

Proposal 18: SL-PRS transmission can be re-configured in response to changing target-anchor link conditions as per channel information report from receiver UE.

## Congestion control for SL-PRS

As per WID [1], solutions for congestion control for SL PRS is to be studied and specified. In RAN1#111, congestion control was discussed. The below captures a FL lead proposal [4] from RAN1#111 for which there was no conclusion reached due to time limitation.

##### [LOW] Feature Lead Proposal 4.4.4-v0 For Scheme 2, with regards to congestion control mechanisms for SL-PRS: - Study congestion metrics definition for SL-PRS (e.g. CBR and CR) - Study which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism

SL positioning is expected to suffer from SL congestion especially in the case of shared resource pools, such as due to high density of users generating frequent SL message traffic, which is typical in V2X use cases.

To avoid degradation of positioning QoS, efficient mechanisms to cope with congestion are required. Different from SL communications, SL positioning involves multiple UEs (e.g., anchor UEs) transmitting to a single UE (e.g., target UE), whereby each of the UEs might experience totally different SL congestion conditions. Therefore, under such varying conditions, it should be possible to “distribute” the congestion in the spatial domain, by “offloading” SL PRS transmissions from the anchors suffering from high congestion to the anchors experiencing lower congestion, even in distributed settings (without a central coordinator such as gNB or LMF).

One way is to enable coordination among UEs to configure their SL PRS transmissions by sharing their channel congestion information with each other. For example, two anchor UEs transmitting SL PRS with a certain configuration, may inform each other about SL CBR they observe, and adjust their configuration based on that, such that the UE observing higher congestion may utilize less resources to transmit SL PRS (e.g., with reduced periodicity), and the other UE observing lower congestion may increase its resource usage for transmitting SL PRS. In this way, the target UE would be able to continue SL positioning measurements with a certain QoS, without being impacted by the congestion at a specific anchor UE as it can be offloaded to other anchor UEs.

Proposal 19: UEs involved in SL positioning may exchange information on the channel congestion they observe, so as to adapt their SL PRS configuration, e.g., SL PRS bandwidth depending on their relative congestion status.

## SL-PRS cast types

The following agreement was reached in the last RAN1 meeting:

Agreement (RAN1#110bis) From RAN1 perspective, the following cast types of SL-PRS transmission can be introduced for SL positioning: Unicast, Groupcast (not including many to one) - Broadcast (as a working assumption). - FFS: Applicability of the above cast types

The below captures a FL lead proposal [4] from RAN1#111 for which there was no conclusion reached due to time limitation.

##### [LOW] Feature Lead Proposal 6.2-v0 After the transmission of a SL-PRS, study whether a feedback mechanism is needed to be introduced from the Rx UE to the Tx UE for the purpose of SL-PRS re-transmission. - Study whether such feedback should be lower layer or higher layer signaling

Utilization of different cast types for transmitting SL-PRS would depend on several factors, such as the selected positioning method, number of anchors, as well as number of target UEs that may benefit from a single anchor UE. Unicast SL-PRS transmissions would be useful when SL-RTT method is utilized between a single target UE and a single anchor UE. On the other hand, groupcast SL-PRS transmission would benefit positioning a target UE, when UL-TDOA-like SL-TDOA method is utilized. Broadcast SL-PRS transmissions would be useful, e.g., in a scenario where any target vehicular UE can benefit from SL-PRS transmitted by UE-type RSUs deployed along the road, using the DL-like SL-TDOA positioning method.

Further, unicast and groupcast SL PRS transmissions would open a possibility for the intended receivers to provide feedback on the transmissions. A lower layer signaling of an ACK/NACK indication as in current SL HARQ mechanism might enable retransmissions (or remove the necessity of a retransmission) of a SL PRS. Whereas a higher layer signaling, containing more information, might help adapting the configuration of the upcoming SL PRS transmissions depending on the changing link conditions and positioning QoS requirements.

Since SL-RTT method doesn’t require stringent synchronization among UEs, it is a critical method for SL positioning. For SL-RTT, PRSs are exchanged between a pair of UEs to estimate RTT. Different PRSs (e.g., two different cyclic shifts of a PRS sequence) from a UE can be used to convey HARQ feedback to trigger the retransmission of PRS from the corresponding UE or not. It triggers prompt retransmission of sidelink PRS without extra resource consumption at very slightly more processing complexity. It doesn’t require design of new PSFCH channel and resource mapping as Rel-16 SL PSFCH.

Proposal 20: Support ACK/NACK feedback for unicast and groupcast SL PRS transmissions to allow or prevent retransmission of a SL PRS transmission.

Proposal 21: Support employing different SL PRSs (e.g., two different cyclic shifts of a SL PRS sequence) from a UE to convey ACK/NACK feedback to trigger retransmission of SL PRS from the corresponding UE or not.

Proposal 22: SL PRS transmissions can be re-configured based on higher layer feedback from the intended receiver that accounts for changing link conditions and positioning QoS requirements.

# Conclusions

In this contribution, we have made the following observations and proposals related to resource allocation for SL-PRS:

Proposal 1: For SL PRS transmission, support (pre-)configuration of either both dedicated and shared resource pools with (Rel-16/17/18) SL communication, or only one of them.

Proposal 2: For both dedicated and shared resource pools for SL positioning, the bandwidth of SL-PRS shall be (pre)-configurable, e.g., depending on positioning accuracy requirements, which can be same or smaller than that of the resource pool.

Proposal 3: A SL PRS transmission may occupy a wide bandwidth spanning across multiple SL resource pools. For monitoring and reserving SL resources across multiple resource pools, study methods that enable coordination between UEs that are configured with different resource pools.

Proposal 4: Dedicated resource pool for SL positioning should not include any other channel or transmission beyond SL-PRS and PSCCH that carries associated SCI. FFS whether PSFCH is included.

Proposal 5: Any signaling associated with a SL PRS transmission in a dedicated resource pool that does not fit into associated control channels (e.g., PSCCH or PSFCH) can be transmitted in an associated resource pool (which could be a shared resource pool or SL-communications-only resource pool).

Proposal 6: Consider supporting SL PRS slots with/without PSCCHs in a dedicated resource pool: a SL PRS slot with PSCCHs can be configured occurring once in every K slots; the parameter K is (pre-)configurable and can be configured as 1.

Proposal 7: SL PRS can be transmitted in multiple segments whereby different SL PRS segments - may be transmitted in different time slots - partially overlap in frequency domain - jointly cover wider bandwidth whereby the following transmission options are considered: - SL PRS segments are FDM’ed with data (PSSCH). - SL PRS segments are FDM’ed with PSFCH.

Observation 1: RE-level multiplexing of SL PRS transmissions of different UEs necessitates enhancements over the legacy NR SL resource allocation Mode 2 to allow for Scheme 2 resource allocation for SL positioning.

Proposal 8: For Scheme 2 resource allocation in shared pools, UE transmitting SL PRS may provide feedback to other transmitting UEs to avoid their resource preemption and reuse the same time/frequency resources via RE-level multiplexing.

Proposal 9: For Scheme 2 resource allocation in at least dedicated resource pools, UE performing sensing SL PRS may use comb size and comb offset as parameters during sensing to determine if reuse of the same time resources via RE-level multiplexing is possible. This sensing operation may be (pre)-configurable.

Proposal 10: For code-domain multiplexing (CDM) of SL PRS transmissions, study IUC like mechanism to determine if SL PRS sequence multiplexing is acceptable at the receiver UE.

Proposal 11: For frequency-domain multiplexing of SL PRS transmissions, study IUC like mechanism to determine if comb-based multiplexing of SL PRS is acceptable at the receiver UE.

Proposal 12: Study the resource allocation mechanisms that allow one UE to allocate multiple SL PRS transmission opportunities for itself and/or other UE(s) in a SL positioning session.

Proposal 13: In the case of a SL PRS allocation for multiple TX and/or RX UEs or under strict timing constraints, study how the priority of resource allocation can be raised as a function of the number of involved UEs and/or timing constraints.

Proposal 14: Study SL PRS resource conflict avoidance/resolution for IUC scenarios involving a UE transmitting (multicast) SL PRS to multiple UEs based on IUC assistance received from them.

Proposal 15: Study at least the following methods for SL PRS conflict avoidance: comb offset coordination, SL PRS sequence selection, and CDM to share the same time-frequency resource (e.g., via IUC) and SL PRS muting patterns.

Proposal 16: A UE (e.g., target UE) may assist configuring parameters of SL PRS (e.g., bandwidth, periodicity, etc.) for other UEs (e.g., anchor UEs) such as by providing (non-)preferred set of SL PRS parameters.

Proposal 17: Periodic or aperiodic (pre-)configured SL PRS can be activated or deactivated based on indications coming from higher layers, e.g., based on parameters such as distance and/or time thresholds that are (pre-)configurable.

Proposal 18: SL-PRS transmission can be re-configured in response to changing target-anchor link conditions as per channel information report from receiver UE.

Proposal 19: UEs involved in SL positioning may exchange information on the channel congestion they observe, so as to adapt their SL PRS configuration, e.g., SL PRS bandwidth depending on their relative congestion status.

Proposal 20: Support ACK/NACK feedback for unicast and groupcast SL PRS transmissions to allow or prevent retransmission of a SL PRS transmission.

Proposal 21: Support employing different SL PRSs (e.g., two different cyclic shifts of a SL PRS sequence) from a UE to convey ACK/NACK feedback to trigger retransmission of SL PRS from the corresponding UE or not.

Proposal 22: SL PRS transmissions can be re-configured based on higher layer feedback from the intended receiver that accounts for changing link conditions and positioning QoS requirements.

# References

- RP-230328, “Expanded and Improved NR Positioning”

- 3GPP TR 38.859, Study on expanded and improved NR positioning

- 3GPP TR 23.700-86, “Technical Specification Group Services and System Aspects; Study on Architecture Enhancement to support Ranging based services and sidelink positioning; (Release 18)”

- R1-2212938, “Moderator Summary #4 on potential solutions for SL positioning”