3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302298

e-Meeting, 17th – 26th April 2023

Agenda Item:	9.6.1

Source:	Ericsson

Title:	Further RedCap UE complexity reduction

Document for:	Discussion, Decision

# 1	Introduction

One of the objectives of the Rel-18 work item on “Enhanced support of reduced capability NR devices” is to specify support for the following further UE complexity reduction features [1]:

Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

For the above objective, a RAN1 agreement summary can be found in [2], and a feature lead summary for the last RAN1 meeting can be found in [3].

RAN#99 discussed whether UE peak data rate reduction (“PR1”) should be supported as a standalone feature or only in combination with UE BB bandwidth reduction (“BW3/PR3”) and endorsed the following proposal [4]:

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of “Rel-18 eRedCap: UE capable of 20MHz + PR1” and “Rel-18 eRedCap: UE capable of BW3/PR3 + PR1” is same including unicast and broadcast respectively. Note 2: PRB processing capability of “Rel-18 eRedCap: UE capable of 20MHz + PR1” is not limited to “25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS” and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between “Rel-18 eRedCap: UE capable of 20MHz + PR1” and “Rel-18 eRedCap: UE capable of BW3/PR3 + PR1” is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

The nicknames for the UE complexity reduction features (“PR1” and “BW3/PR3”) originate from TR 38.865 [5].

In this contribution, we present our views on the Rel-18 UE complexity reduction features, i.e., UE peak data rate reduction (a.k.a. “PR1”) and UE BB bandwidth reduction (a.k.a. “BW3/PR3”).

# 2	UE peak data rate reduction (a.k.a. “PR1”)

The peak data rate supported by an NR UE is calculated using the following expression from TS 38.306 [6]:

### 4.1.2 Supported max data rate for DL/UL For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows. data rate (in Mbps)=10-6⋅j=1JvLayers(j)⋅Qm(j)⋅f(j)⋅RmaxNPRBBW(j),μ⋅12Tsμ1-OH(j) wherein J is the number of aggregated component carriers in a band or band combination Rmax = 948/1024 For the j-th CC, is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink. Qm(j) is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink. f(j) is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4. μ is the numerology (as defined in TS 38.211 [6]) Tsμ is the average OFDM symbol duration in a subframe for numerology μ, i.e. Tsμ=10-314⋅2μ. Note that normal cyclic prefix is assumed. NPRBBWj,μ is the maximum RB allocation in bandwidth BWj with numerology μ, as defined in 5.3 TS 38.101-1 [2] and 5.3 TS 38.101-2 [3], where BWj is the UE supported maximum bandwidth in the given band or band combination. OH(j)is the overhead and takes the following values 0.14, for frequency range FR1 for DL 0.18, for frequency range FR2 for DL 0.08, for frequency range FR1 for UL 0.10, for frequency range FR2 for UL The approximate maximum data rate can be computed as the maximum of the approximate data rates computed using the above formula for each of the supported band or band combinations. For single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅f(j) is no smaller than 4. NOTE: As an example, the value 4 in the component above can correspond to vLayers(j)=1, Qmj= 4 and f(j)=1.

The WI objective for UE peak data rate reduction concerns relaxing the constraint vLayers·Qm·f ≥ 4 for Rel-18 eRedCap UEs, i.e., to relax the constraint that the product of the maximum number of supported MIMO layers, the ‘maximum supported modulation order’, and the peak rate scaling factor should be no smaller than 4 for NR SA single carrier operation. Note that the so called ‘maximum supported modulation order’ in this context only concerns the modulation order assumed in the peak data rate calculations. The network may still use a higher modulation order than the value indicated by this field when scheduling the UE.

With the current constraint (≥ 4), based on the expression above, the peak data rate supported by a baseline Rel-17 RedCap UE should be at least 55 Mbps in DL and UL.

For UE peak data rate reduction, the following agreements have been made in RAN1 [2]:

Agreement: - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X - If UE peak data rate reduction is supported as a standalone feature, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ Y. - FFS: the value of Y - Note: Whether this option is supported will be decided in RAN plenary. Agreement: - The minimum DL peak rate target (for FD-FDD) is 10 Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL Agreement: For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X.

In the following sections, we discuss our views on the values of X and Y, which remain to be decided.

## 2.1	With UE BB bandwidth reduction (“PR1 + BW3/PR3”)

Table 1 shows supported peak data rates for different relaxations of the constraint when UE peak data rate reduction is used in combination with UE BB bandwidth reduction.

Table 1: Peak data rate for a Rel-18 eRedCap UE for different relaxed constraints when UE peak data rate reduction is used in combination with UE BB bandwidth reduction.

| vLayers·Qm·f (X) | 15 kHz SCS (25 PRBs) | 30 kHz SCS (12 PRBs) |
| --- | --- | --- |
| 4 (current value) | DL: 13.4 Mbps UL: 14.3 Mbps | DL: 12.8 Mbps UL: 13.7 Mbps |
| 3.2 | DL: 10.7 Mbps UL: 11.4 Mbps | DL: 10.3 Mbps UL: 11.0 Mbps |
| 3 | DL: 10.0 Mbps UL: 10.7 Mbps | DL: 9.6 Mbps UL: 10.3 Mbps |

From Table 1, it can be observed that X=3.2 satisfies the 10-Mbps peak rate target for both 15 and 30 kHz SCS.

For UE peak data rate reduction in combination with UE BB bandwidth reduction, the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 3.2.

## 2.2	Without UE BB bandwidth reduction (“PR1 only”)

Table 2 shows supported peak data rates for different relaxations of the constraint when UE peak data rate reduction is used without UE BB bandwidth reduction.

Table 2: Peak data rate for a Rel-18 eRedCap UE for different relaxed constraints when UE peak data rate reduction is used without UE BB bandwidth reduction.

| vLayers·Qm·f (Y) | 15 kHz SCS (106 PRBs) | 30 kHz SCS (51 PRBs) |
| --- | --- | --- |
| 4 (current value) | DL: 56.7 Mbps UL: 60.7 Mbps | DL: 54.6 Mbps UL: 58.4 Mbps |
| 1 | DL: 14.2 Mbps UL: 15.2 Mbps | DL: 13.6 Mbps UL: 14.6 Mbps |
| 0.8 | DL: 11.3 Mbps UL: 12.1 Mbps | DL: 10.9 Mbps UL: 11.7 Mbps |
| 0.75 | DL: 10.6 Mbps UL: 11.4 Mbps | DL: 10.2 Mbps UL: 10.9 Mbps |
| 0.7 | DL: 9.9 Mbps UL: 10.6 Mbps | DL: 9.6 Mbps UL: 10.2 Mbps |
| 0.4 | DL: 5.7 Mbps UL: 6.1 Mbps | DL: 5.5 Mbps UL: 5.8 Mbps |

From Table 2, it can be observed that Y=0.75 satisfies the 10-Mbps peak rate target when the UE supported maximum bandwidth is 20 MHz, corresponding to 106 PRBs with 15 kHz SCS and 51 PRBs with 30 kHz SCS.

It is imperative that the UE-supported maximum bandwidth of 20 MHz is always applied when calculating the maximum peak rate, even when the channel/carrier bandwidth is less than 20 MHz. Otherwise, if Y=0.75 is applied when the channel/carrier bandwidth is less than 20 MHz, this may be problematic due to the following reasons:

- Firstly, the data rate supported by the PR1-only UEs would become too low. For example, a bandwidth of 10 MHz would correspond to 5.2/5.6 Mbps and 4.8/5.2 Mbps in DL/UL with 15 and 30 kHz SCSs, respectively. And a bandwidth of 5 MHz would correspond to 2.5/2.7 Mbps and 2.2/2.4 Mbps in DL/UL with 15 and 30 kHz SCSs, respectively.

- Secondly, the maximum TBS values corresponding to the above peak rate values would be too small to receive initial access messages. For example, based on TS 38.213/38.331, the maximum TBS for SIB is 2976 bits. That is, a peak rate of at least 3 Mbps with 15 kHz SCS and 6 Mbps with 30 kHz are needed to properly receive the SIB.

Therefore, we have the following proposal:

For UE peak data rate reduction without UE BB bandwidth reduction, the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 0.75. The UE supported maximum bandwidth of 20 MHz is always applied, irrespective of the carrier bandwidth, in the max data rate calculation as defined in TS 38.306 clause 4.1.2.

# 3	UE BB bandwidth reduction (a.k.a. “BW3/PR3”)

In the following subsections, we provide our views on open issues for UE baseband (BB) bandwidth reduction.

## 3.1	RAR PDSCH bandwidth and Msg3 transmission timeline

The NR specification allows PDSCH carrying RAR to be scheduled with a bandwidth larger than 5 MHz – up to 17.2 MHz. The 17.2 MHz corresponds to the maximum CORESET#0 bandwidth in FR1. Note that the SIB1-configured initial downlink BWP, which can be larger than CORESET#0 bandwidth, is not applied until after reception of Msg4. Also, as per the specification, higher order modulation than QPSK cannot be used for RAR, but the code rates associated with QPSK may be used, i.e., MCS0 to MCS9 in Table 5.1.3.1-1 of TS 38.214. However, typically MCS0 would be used to achieve sufficient coverage.

If the PDSCH contains a single RAR message for just one UE, the required bandwidth can be expected to be smaller than 5 MHz as the TBS would be rather small (e.g., 72 bits). However, if RAR messages for many UEs are multiplexed in the same PDSCH, the TBS would be larger, and as a result, the required bandwidth would be larger than 5 MHz. Furthermore, as identified in the Rel-18 RedCap study item [5], RAR PDSCH is one of the coverage limiting channels for 1-Rx UEs in some scenarios. Applying TBS scaling to recover coverage would require transmitting PDSCH over a larger number of PRBs, exceeding 5 MHz even when it contains only a single RAR message.

If RARs for many UEs are multiplexed in the same PDSCH, the TBS would be larger, and as a result, the required RAR bandwidth would be larger than 5 MHz.

Applying TBS scaling to recover coverage of 1-Rx RedCap UEs would require transmitting PDSCH over a larger number of PRBs, exceeding 5 MHz even when it contains only a single RAR message.

As mentioned, the maximum RAR bandwidth is 17.2 MHz. This corresponds to 96 PRBs for 15 kHz SCS and 48 PRBs for 30 kHz SCS. If the maximum number of PRBs that a Rel-18 eRedCap UE can process per slot is 25 for 15 kHz SCS and 12 for 30 kHz SCS, it can take multiple slots for the UE to complete processing of RAR.

RAN1#111 and RAN1#112 made the following agreements regarding the RAR PDSCH bandwidth and the Msg3 transmission timeline [2]:

Agreement: For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS Agreement: For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH Agreement: For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to be selected for SCSs

That is, the RAR bandwidth is allowed to be larger than 5 MHz. However, for such cases, the time between last symbol of RAR PDSCH and first symbol of corresponding Msg3 PUSCH transmission may need to be larger than the legacy minimum time of NT,1 + NT,2 + 0.5 ms. Here, NT,1 and + NT,2 are the time durations of N1 and N2 symbols corresponding to PDSCH processing and PUSCH preparation, respectively. The values for N1 and N2 for different SCSs are given in Table 3 and Table 4.

Table 3: PDSCH processing time for PDSCH processing capability 1 (from Table 5.3-1 of TS 38.214)

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB if either higher layer parameter is configured, and in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if either higher layer parameter is configured | dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2, or if none of the higher layer parameters is configured |
| 0 | 8 | 14 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |
| 5 | 80 | 96 |
| 6 | 160 | 192 |

Table 4: PUSCH preparation time for PUSCH timing capability 1 (from Table 6.4-1 of TS 38.214)

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |
| 5 | 144 |
| 6 | 288 |

Based on Table 3 and Table 4, the legacy minimum time NT,1 + NT,2 + 0.5 ms (for dmrs-AdditionalPosition ≠ 'pos0') would in symbols correspond to:

- 31 symbols for 15 kHz SCS

- 39 symbols for 30 kHz SCS

The new minimum time is given by NT,1 + NT,2 + 0.5 + X ms, where the value of X is to be selected from [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30 kHz SCS, according to the RAN1 agreements listed above. Table 5 lists the possible minimum time in symbols.

Table 5: Possible minimum time in symbols between RAR and Msg3 for Rel-18 eRedCap UEs

| μ | NT1 | NT2 | Legacy minimum time [symbols] | New minimum time [symbols] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | RAR BW ≤ 5 MHz with X = 0 | RAR BW > 5 MHz with X = 0.5 / 0.25 ms for 15 / 30 kHz SCS | RAR BW > 5 MHz with X = 1 / 0.5 ms for 15 / 30 kHz SCS | RAR BW > 5 MHz with X = 2 / 1 ms for 15 / 30 kHz SCS |
| 0 | 14 | 10 | 31 | 38 | 45 | 59 |
| 1 | 13 | 12 | 39 | 46 | 53 | 67 |

Beside the minimum time discussed above, it is important to also discuss the scheduling time offset between RAR PDSCH and Msg3 PUSCH, as depicted in Figure 1. If a UE receives a PDSCH with a RAR message ending in slot n, the UE transmits Msg3 PUSCH in slot n + K2 + Δ, where the value of Δ depends on SCS of PUSCH (µPUSCH) and is given in Table 6, and the value of K2 (as well as the start and length of the PUSCH allocation) is indicated by the four-bit PUSCH time-domain resource allocation (TDRA) field in the RAR grant given in Table 7. The TDRA field points to one of the 16 rows in Table 8, the default TDRA table, which provides the value of K2. Note that, in Table 8, K2 is given in slots, whereas start and length of PUSCH allocation (S and L) are given in symbols. The value of j used to determine K2 depends on µPUSCH and is given in Table 9.

Figure 1: Illustration of scheduling time offset between RAR PDSCH and Msg3 PUSCH

Table 6: Definition of value Δ (from Table 6.1.2.1.1-5 of TS 38.214)

| µPUSCH | Δ |
| --- | --- |
| 0 | 2 |
| 1 | 3 |
| 2 | 4 |
| 3 | 6 |
| 5 | 24 |
| 6 | 48 |

Table 7: Random Access Response Grant Content field size (from Table 8.2-1 of TS 38.213)

| RAR grant field | Number of bits |
| --- | --- |
| Frequency hopping flag | 1 |
| PUSCH frequency resource allocation | 12, for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided 14, otherwise |
| PUSCH time resource allocation | 4 |
| MCS | 4 |
| TPC command for PUSCH | 3 |
| CSI request | 1 |
| ChannelAccess-Cpext | 2, for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided 0, otherwise |

Table 8: Default PUSCH time domain resource allocation A for normal CP (from Table 6.1.2.1.1-2 of TS 38.214)

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 10 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 14 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 14 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 6 |
| 15 | Type A | j+3 | 0 | 14 |
| 16 | Type A | j+3 | 0 | 10 |

Table 9: Definition of value j (from Table 6.1.2.1.1-4 of TS 38.214)

| µPUSCH | j |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |
| 5 | 11 |
| 6 | 21 |

Based on Table 6, Table 8, and Table 9, the minimum and the maximum values for the scheduling time offset (i.e., K2 + Δ in slots). Table 10 shows that the range of the time gap (i.e., K2 + Δ – 1 in symbols) between RAR PDSCH and Msg3 PUSCH is 28 to 70 symbols for 15 kHz SCS and 42 to 84 symbols for 30 kHz SCS.

Table 10: Scheduling time offset between RAR PDSCH and Msg3 PUSCH.

| μ | Δ | K2 + Δ – 1 (symbols) (j) | K2 + Δ – 1 (symbols) (j+1) | K2 + Δ – 1 (symbols) (j+2) | K2 + Δ – 1 (symbols) (j+3) |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 28 | 42 | 56 | 70 |
| 1 | 3 | 42 | 56 | 70 | 84 |

This time gap must be larger than the minimum time NT,1 + NT,2 + 0.5 + X (in symbols) for the entries in the default TDRA table (Table 8) to be valid. Now, comparing the columns in Table 10 and the minimum time in Table 5, the following can be observed for the candidate values for X:

- For X = 0.5/0.25 ms for 15/30 kHz SCS:

  - Entries with j in the default TDRA table (Table 8) are invalid for both 15 and 30 kHz.

  - The other entries (with j+1, j+2 or j+3) are valid for both 15 and 30 kHz.

- For X = 1/0.5 ms for 15/30 kHz SCS:

  - Entries with j or j+1 for 15 kHz and entries with j for 30 kHz are invalid.

  - The other entries (with j+1 or j+2 for 15 kHz, and j+1, j+2 or j+3 for 30 kHz) are valid.

- For X = 2/1 ms for 15/30 kHz SCS:

  - Entries with j, j+1 or j+2 for 15 kHz and entries with j or j+1 for 30 kHz are invalid.

  - The other entries (with j+3 for 15 kHz, and j+2 or j+3 for 30 kHz) are valid.

Based on this analysis, we propose to adopt the smallest value, i.e., 0.5/0.25 ms for 15/30 kHz SCS. Otherwise, i.e., if larger values are adopted, fewer valid entries in the default TDRA table would be available for the network to schedule Rel-18 eRedCap UEs. This impacts the scheduling flexibility of not only the Rel-18 eRedCap UEs but also other types of UEs in the cell.

Furthermore, specifying the smallest value for X would also help to minimize the impact (e.g., in terms of random-access delay) to Rel-17 RedCap UEs and “PR1-only UEs” in cases where “BW3/PR3 UEs” share the same early indication as Rel-17 RedCap UEs or in cases where the additional separate early indication (as discussed in Section 3.2) is common to both “PR1-only UEs” and “BW3/PR3 UEs”.

For UE BB bandwidth reduction, for the relaxation of the minimum time between the last symbol of RAR PDSCH and the first symbol corresponding Msg3 PUSCH transmission, select X = 0.5/0.25 ms for 15/30 kHz SCS.

Note that the network also has the possibility to configure specific values of K2 (as well as start and length of the PUSCH allocation) by configuring pusch-TimeDomainAllocationList in pusch-ConfigCommon in SIB1. If pusch-TimeDomainAllocationList is configured, the network applies the values in this list and not those in default the TDRA table. The K2 value in the PUSCH-TimeDomainResourceAllocationList is common to all UEs or to all RedCap UEs in the cell. It might be beneficial to have a PUSCH-TimeDomainResourceAllocationList that is specific to Rel-18 eRedCap UEs. This would enable the network to configure a larger K2 when scheduling Msg3 PUSCH, if needed.

Support PUSCH TDRA in pusch-ConfigCommon that is specific to Rel-18 eRedCap UEs.

- Detailed signaling solution is up to RAN2.

## 3.2	Additional separate early indication(s)

Rel-17 RedCap UEs always provide an indication in Msg3 to let the gNB know at an early stage that the UE is indeed a RedCap UE. If gNB configures separate, RedCap-specific PRACH resources, then the gNB receives an even earlier, implicit indication already in Msg1 whenever a RedCap UE accesses these PRACH resources.

There is a WI objective on support of additional separate early indication(s) for UE BB bandwidth reduction [1]:

- UE BB bandwidth reduction - […] - Support additional separate early indication(s) [RAN1, RAN2]

RAN2#121 made the following agreements regarding support of additional separate early indication(s) [7]:

Introduce Msg3/MsgA PUSCH based early indication for Rel-18 eRedCap. FFS how to implement this in the spec (e.g., new LCIDs or not). We will wait for RAN1 progress to see if there is a need for a Msg1 early indication for eRedCap.

So, it is already clear that there will be support for additional Msg3 based early indication, but it remains to decide whether there will also be support for additional Msg1 based early indication.

For Rel-18 eRedCap UEs (with UE BB bandwidth reduction) the need for separate early indication in Msg1 can be motivated by the following reasons:

- If the PDSCH conveying RAR messages to Rel-17 RedCap and Rel-18 eRedCap UEs is wider than 5 MHz, the Rel-18 eRedCap UEs might be unable to handle the legacy minimum time between RAR PDSCH and Msg3 PUSCH (as discussed in previous section). If it is desired to apply timing relaxation for Rel-18 eRedCap UEs alone, i.e., if a common timing relaxation for both Rel-17 RedCap and Rel-18 eRedCap UEs is not desired, then a separate Msg1 indication for Rel-18 eRedCap UEs is needed.

- If it is desired to use different bandwidths for RAR PDSCH, i.e., larger than 5 MHz for Rel-17 RedCap UEs and equal to or smaller than 5 MHz for Rel-18 eRedCap UEs (so that the legacy minimum time is followed), then a separate Msg1 indication is needed. This might happen if there are many Rel-17 RedCap UEs attempting random access (and hence, larger RAR bandwidth due to RAR multiplexing of RARs) and TBS scaling is used to recover coverage of (1-Rx) Rel-17 RedCap UEs.

- If Msg3 PUSCH is to be scheduled with a wider bandwidth than 5 MHz for Rel-17 RedCap UEs (e.g., for RA-SDT) but with a smaller bandwidth than 5 MHz for Rel-18 eRedCap UEs, then a separate Msg1 indication is needed.

A potential separate Msg1 indication would be configurable by the network. Therefore, support of a separate Msg3 indication might also be useful for the cases when separate Msg1 indication is not configured (e.g., to minimize PRACH fragmentation), and Msg4 or Msg5 is to be scheduled with larger bandwidth than 5 MHz for Rel-17 RedCap UEs coming from RRC idle state. Note that it is not possible to schedule Msg3, Msg4, and Msg5 with larger than 5 MHz for Rel-18 eRedCap UEs.

Based on above considerations, in our view, additional separate early indication in Msg1 should be supported.

For UE BB bandwidth reduction, support additional separate early indication in Msg1 for 4-step RACH.

Due to similar reasons, separate early indication in MsgA PRACH of 2-step RACH should be supported.

For UE BB bandwidth reduction, support additional separate early indication in MsgA PRACH for 2-step RACH.

As mentioned in the introduction of this contribution, RAN#99 endorsed the following proposal [4]:

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of “Rel-18 eRedCap: UE capable of 20MHz + PR1” and “Rel-18 eRedCap: UE capable of BW3/PR3 + PR1” is same including unicast and broadcast respectively. Note 2: PRB processing capability of “Rel-18 eRedCap: UE capable of 20MHz + PR1” is not limited to “25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS” and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between “Rel-18 eRedCap: UE capable of 20MHz + PR1” and “Rel-18 eRedCap: UE capable of BW3/PR3 + PR1” is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

According to Note 4 above, UEs that support both UE peak data rate reduction and UE BB bandwidth reduction should use the same initial access procedure as UEs that only support UE peak data rate reduction. According to one interpretation of Note 4, this means that if a UE that supports both UE peak data rate reduction and UE BB bandwidth reduction provides separate early indication(s), then a UE that only supports UE peak data rate reduction also needs to provide the same separate early indication(s).

This is a change compared to the WI objective in the WID [1] which so far only concerned additional separate early indication(s) for UEs supporting UE BB bandwidth reduction. The drawback with this recent change is that it may increase the implementation and testing burden on the network side. For a network to implement support for UEs supporting UE peak data rate reduction, the network implementation of the initial access procedure may now need to be updated to consider that the accessing UE might be a UE that supports not only UE peak data rate reduction but also UE BB bandwidth reduction.

Some simple means should be considered that will enable a phased approach in the network implementation. If the network can distinguish early on whether the UE is a UE that supports both UE BB bandwidth reduction and UE peak data rate reduction, or a UE that only supports UE peak data rate reduction, the initial burden for network implementation and testing will be significantly smaller – this can be achieved by specifying that the additional separate early indication in Msg1 only concerns those UEs that support UE BB bandwidth reduction. However, if this would not be desired, an alternative approach is to specify that access control/barring is separate for UEs that support UE BB bandwidth reduction and UEs that only support UE peak data rate reduction – this will ensure that the network implementation only allows access to UEs for which full support has been implemented on the network side. Adopting one of these options also helps from IODT point of view.

Support at least one of the following options to ensure that gNB knows whether to expect access by UEs supporting UE BB bandwidth reduction:

- Option 1: Additional separate early indication in Msg1 only concerns UEs that support UE BB bandwidth reduction.

- Option 2: Access control/barring is separate for UEs that support UE BB bandwidth reduction and UEs that only support UE peak data rate reduction.

The discussion and decision about the above proposal can potentially also be left up to RAN2.

## 3.3	Simultaneous reception of unicast and broadcast

RAN1#112 made the following conclusion about simultaneous reception of two broadcast transmissions [2]:

Conclusion: For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR.

This means that for simultaneous reception of two broadcast transmissions, the following applies [8]:

The UE in RRC_IDLE and RRC_INACTIVE modes shall be able to decode two PDSCHs each scheduled with SI-RNTI, P-RNTI, RA-RNTI or TC-RNTI, with the two PDSCHs partially or fully overlapping in time in non-overlapping PRBs.

RAN1#112 also discussed simultaneous reception of unicast and broadcast transmissions without reaching a conclusion [3]. Currently, the following is specified for this case [8]:

On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI. The UE is expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI during a process of autonomous SI acquisition.

For UE BB bandwidth reduction, unicast PDSCH (including those scheduled with TC-RNTI) cannot exceed 5 MHz. Therefore, for handling simultaneous reception of unicast and broadcast channels for such UEs, one possibility would be to reuse similar procedure as currently specified for FR2, as given below [8]:

On a frequency range 2 cell, the UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time.

Note that a gNB implementation can choose to avoid scheduling of a unicast channel and a broadcast channel on overlapping time resources.

For UE BB bandwidth reduction, discuss whether a UE can be expected to decode a unicast PDSCH (including those scheduled with TC-RNTI) and another PDSCH scheduled with SI-RNTI when they overlap in time and their combined bandwidth exceeds 5 MHz.

## 3.4	Msg4 PDSCH bandwidth

RAN1#112 made the following working assumption regarding the Msg4 PDSCH bandwidth [2]:

Working assumption: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

We propose to confirm the above working assumption.

Confirm the following working assumption from RAN1#112:

- For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

  - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

In one of the RAN1#112 sessions, contention resolution was discussed. In case of contention between random access attempts from different UEs, the Msg4 PDSCH transmission that a UE tries to receive may in fact be intended for another UE. So, if the network makes a Msg4 PDSCH transmission to a Rel-17 RedCap UE or a non-RedCap UE, there is a risk that a Rel-18 eRedCap UE tries to receive it. If the scheduled Msg4 PDSCH bandwidth is too wide for the Rel-18 eRedCap UE to receive or process, it will detect this already when it receives the DCI scheduling the Msg4 PDSCH, but it may not be obvious how the Rel-18 eRedCap UE ought to react in this situation, assuming that this situation is allowed to happen. A few different approaches were discussed in the RAN1#112 session, and RAN1 could select one of these approaches. We may also be fine with other (simple, robust) approaches.

For UE BB complexity reduction, for the case when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process, RAN1 selects one of the following options:

- Option 1: The UE considers the contention resolution as not successful.

- Option 2: The UE discards the DCI (as if the DCI was never received).

- Option 3: The UE behaviour is undefined (as if the event is not expected).

RAN2 involvement may be needed before a final decision is made, since the contention resolution procedure is partly specified in 38.213 [9] and partly in 38.321 [10].

## 3.5	MsgB PDSCH bandwidth

RAN1#112 discussed the following proposal on Msg4 PDSCH bandwidth without reaching a conclusion [3]:

Medium Priority Proposal 2.9-1b: - The MsgB PDSCH bandwidth and number of PRBs are limited in the same way as for Msg4 PDSCH.

Once an agreement has been reached for the Msg4 PDSCH bandwidth (discussed in previous subsection), probably a corresponding agreement can be made for the MsgB PDSCH bandwidth.

The MsgB PDSCH bandwidth and number of PRBs are limited in the same way as for Msg4 PDSCH.

## 3.6	Support for 60 kHz SCS

RAN1#112 made the following agreement regarding the maximum number of PRBs for PUSCH/PDSCH for 15 and 30 kHz SCS [2]:

Agreement: For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number

The WID [1] stipulates that “both 15 kHz SCS and 30 kHz SCS are supported”, but it does not say anything about 60 kHz SCS. To also cover 60 kHz SCS, the above agreement can be extended in the following way, where the maximum number of PRBs for PUSCH/PDSCH is proposed to be 6 PRBs for 60 kHz SCS.

Make a similar agreement (or working assumption) for 60 kHz as for 15/30 kHz SCS, i.e.:

- For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable:

  - 6 PRBs for 60 kHz SCS

- For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot:

  - 6 PRBs for 60 kHz SCS

- Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number

Perhaps the above proposal can be considered at least as a potential working assumption at this point, since companies may want to check further before it can be agreed.

The selection of the value for the relaxed constraint for UE peak data rate reduction (discussed in the beginning of this contribution) may need to be revisited if 60 kHz SCS is to be supported beside 15 and 30 kHz SCS.

# 4	Conclusion

In the previous sections, we made the following observations:

Observation 1	If RARs for many UEs are multiplexed in the same PDSCH, the TBS would be larger, and as a result, the required RAR bandwidth would be larger than 5 MHz.

Observation 2	Applying TBS scaling to recover coverage of 1-Rx RedCap UEs would require transmitting PDSCH over a larger number of PRBs, exceeding 5 MHz even when it contains only a single RAR message.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UE peak data rate reduction in combination with UE BB bandwidth reduction, the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 3.2.

Proposal 2	For UE peak data rate reduction without UE BB bandwidth reduction, the constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ 0.75. The UE supported maximum bandwidth of 20 MHz is always applied, irrespective of the carrier bandwidth, in the max data rate calculation as defined in TS 38.306 clause 4.1.2.

Proposal 3	For UE BB bandwidth reduction, for the relaxation of the minimum time between the last symbol of RAR PDSCH and the first symbol corresponding Msg3 PUSCH transmission, select X = 0.5/0.25 ms for 15/30 kHz SCS.

Proposal 4	Support PUSCH TDRA in pusch-ConfigCommon that is specific to Rel-18 eRedCap UEs.

	Detailed signaling solution is up to RAN2.

Proposal 5	For UE BB bandwidth reduction, support additional separate early indication in Msg1 for 4-step RACH.

Proposal 6	For UE BB bandwidth reduction, support additional separate early indication in MsgA PRACH for 2-step RACH.

Proposal 7	Support at least one of the following options to ensure that gNB knows whether to expect access by UEs supporting UE BB bandwidth reduction:

	Option 1: Additional separate early indication in Msg1 only concerns UEs that support UE BB bandwidth reduction.

	Option 2: Access control/barring is separate for UEs that support UE BB bandwidth reduction and UEs that only support UE peak data rate reduction.

Proposal 8	For UE BB bandwidth reduction, discuss whether a UE can be expected to decode a unicast PDSCH (including those scheduled with TC-RNTI) and another PDSCH scheduled with SI-RNTI when they overlap in time and their combined bandwidth exceeds 5 MHz.

Proposal 9	Confirm the following working assumption from RAN1#112:

	For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot.

o	The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Proposal 10	For UE BB complexity reduction, for the case when the UE detects a DCI scheduling a Msg4 PDSCH transmission with a larger bandwidth than it can receive or process, RAN1 selects one of the following options:

	Option 1: The UE considers the contention resolution as not successful.

	Option 2: The UE discards the DCI (as if the DCI was never received).

	Option 3: The UE behaviour is undefined (as if the event is not expected).

Proposal 11	The MsgB PDSCH bandwidth and number of PRBs are limited in the same way as for Msg4 PDSCH.

Proposal 12	Make a similar agreement (or working assumption) for 60 kHz as for 15/30 kHz SCS, i.e.:

	For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable:

o	6 PRBs for 60 kHz SCS

	For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot:

o	6 PRBs for 60 kHz SCS

	Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number

# References

RP-223544, “Revised WID on Enhanced support of reduced capability NR devices”, Ericsson, RAN#98-e, December 2022.

R1-2301885, “RAN1 agreements for Rel-18 NR RedCap”, Rapporteur (Ericsson), RAN1#112, March 2023.

R1-2301889, “FL summary #4 on Rel-18 RedCap UE complexity reduction”, Moderator (Ericsson), RAN#112, March 2023.

RP-230778, “Proposal for PR1 in eRedCap”, Moderator (CMCC), RAN#99, March 2023.

TR 38.865 V18.0.0, “Study on further NR RedCap UE complexity reduction”, September 2022.

TS 38.306 V17.4.0, “NR; User Equipment (UE) radio access capabilities (Release 17)”, March 2023.

R2-2301910, “Report from eRedCap breakout session”, Session chair (Ericsson), RAN2#121, March 2023.

TS 38.214 V17.5.0, “NR; Physical layer procedures for data (Release 17)”, March 2023.

TS 38.213 V17.5.0, “NR; Physical layer procedures for control (Release 17)”, March 2023.

TS 38.321 V17.4.0, “NR; Medium Access Control (MAC) protocol specification (Release 17)”, March 2023.