3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302317

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.8.1

Source:	Futurewei

Title:	XR-specific capacity enhancements

Document for:	Discussion and decision

# Introduction

In RAN1 #112 meeting, there were a lot of progresses or agreements achieved [1], such as, for multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration and dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE. However, some critical details of the agreements are still not clear and further down selections are also needed for different alternatives or options in future. In this paper, we will discuss and compare pros and cons of different alternatives or options for the relevant important issues of the agreements or conclusions, and then give our opinions and proposals for different relevant important issues based on the agreements or conclusions, which could be helpful for the group to conduct selections of the different alternatives or options for the important issues.

# Discussion

## Multiple transmission occasions in a period of a single CG PUSCH configuration

### Variable frame size for uplink AR traffic

  - As mentioned in uplink AR traffic model 1 in the appendix, the size of frames is variable and subject to a certain probability distribution. The maximum frame size for uplink AR traffic model 1 is 150% mean packet size. To cater the large and variable frame size for uplink AR traffic or other XR traffic, capacity enhancements for legacy configured grant (CG) are needed. A straightforward approach to enhance the capacity of CG is to configure multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration. The multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration may have same periodicity and different starting points, as shown in Figure 1. If the frame size of XR traffic being equal to (or less than) the size of CG resources in a single CG PUSCH configuration, UE may allocate the data of the frame to all the CG PUSCH transmission occasions (or to a subset of the CG PUSCH transmission occasions). Additionally, to recycle the unused CG resources to other UE(s) by gNB, the unused CG PUSCH occasion(s) need to be indicated to gNB by the UE based on UCI, which will be discussed in detail later.

Figure 1: Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

### Configuration of multiple CG PUSCH transmission occasions

Regarding the configurations of multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration, the below agreements and conclusions were achieved in RAN1 #112 meeting:

Conclusion For convenience in discussion, the term "multi-PUSCHs CG" refers to " a CG PUSCH configuration with multiple CG PUSCH transmission occasions within a period of the CG PUSCH configuration". Agreement - Multi-PUSCHs CG is supported for Type-1 configured grant. - Multi-PUSCHs CG is supported for Type-2 configured grant. Agreement For determination of the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG, the following alternatives for further study: - Alt-A: TDRA determination based on repetition framework. - Alt-A1: Follow the time domain resource mapping of Type A repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The same SLIV in N PUSCH in consecutive slots per CG period - FFS for non-consecutive slots - FFS details, including related RRC parameters - Alt-A2: Follow the time domain resource mapping of Type B repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The SLIV used for 1st PUSCH per CG period. - N consecutive nominal PUSCHs with same duration per CG period - Note: N is not necessarily the repetition factor. FFS details, including related RRC parameters - Alt-B: TDRA determination based on NR-U framework - N and M configured by higher layers - Single SLIV is determined from TDRA. - The SLIV used for 1st PUSCH per CG period. - M consecutive PUSCH TOs with same duration in slot. The M PUSCH TOs are used in N consecutive slots per CG period - Note: N and M are configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. M and N configuration is independent from cgRetransmissionTimer configuration. - FFS details, including related RRC parameters - Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs - Alt-C1: Follow Rel-16 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with k2-r16 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs should be consecutive PUSCH TOs in consecutive slots. - FFS details, including related RRC parameters - Alt-C2: Follow Rel-17 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with extendedK2-r17 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs can be non-consecutive PUSCHs and/or in non-consecutive slots. - FFS details, including related RRC parameters

To configure multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration, three mechanisms, developed in previous releases from Rel-15 to Rel-17, can be considered, which include:

1). Alt-A: TDRA determination based on repetition framework (Alt-A1 and Alt-A2);

2). Alt-B: TDRA determination based on NR-U framework; and

3). Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs (Alt-C1 and Alt-C2).

In the legacy CG design as mentioned in the appendix, Alt-A (i.e., TDRA determination based on repetition framework) was adopted to increase uplink reliability for URLLC services. More specifically, RRC signaling indicates the full-time domain resource allocation for type 1 CG including periodicity, time offset, start symbol and length of PUSCH and K-repetition of the configured grant resources. For type 2 CG, only periodicity and K-repetition in time domain are given through RRC signaling. The other time domain related parameters are given through DCI activation scrambled with CS-RNTI. For a URLLC service, the parameter repK indicates one of {1, 2, 4, 8} repetitions of the same TB in a period of a single CG PUSCH configuration as shown in below ConfiguredGrantConfig IE [2]. Additionally, both PUSCH repetition Type A (i.e., Alt-A1) and Type B (i.e., Alt-A2) are supported in the repetition framework mechanism. Based on the repetition configuration, UE can repeat the TB across the K consecutive slots applying the same symbol allocation in each slot for PUSCH repetition Type A (i.e., Alt-A1) or repeat the TB across the K nominal repetitions applying the same symbol allocation in each repetition for PUSCH repetition Type B (i.e., Alt-A2). To simplify the configurations of multiple CG PUSCH transmission occasions in a period of a single CG PUSCH in Rel-18 XR Enhancements, the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration may have the same symbol allocation in each CG PUSCH transmission occasion as the legacy CG repetition design. If considering some typical TDD frame structures (e.g., DDDSU), the restriction of the K consecutive uplink slots in legacy CG repetition design may be invalid and the K consecutive slots can be reinterpreted as K (consecutive) available uplink slots in a period of a single CG PUSCH configuration. To further reduce configuration signaling overhead, the K (consecutive) available uplink slots can be defined as the earliest K (consecutive) available uplink slots in a period of a single CG PUSCH configuration. Therefore, the legacy TDRA determination based on repetition framework can be reused with slight enhancements and/or reinterpreted as the configurations of multiple CG PUSCH transmission occasions in a period of a single CG PUSCH for Rel-18 XR Enhancements.

For Alt-B (i.e., TDRA determination based on NR-U framework in Rel-16), there can be a number of PUSCH transmission occasions with the same symbols in a period of a single CG PUSCH configuration and the number is determined by high-layer configured parameters cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16. More specifically, the higher layer parameter cg-nrofSlots-r16 provides the number of consecutive slots allocated within a configured grant period. The higher layer parameter cg-nrofPUSCH-InSlot-r16 provides the number of consecutive PUSCH allocations within a slot, where the first PUSCH allocation follows the higher layer parameter timeDomainAllocation for Type 1 CG PUSCH transmission or time domain resource assignment field in the activation DCI for Type 2 CG PUSCH transmission, and the remaining PUSCH allocations have the same length and PUSCH mapping type and are appended following the previous allocations without any gaps. The same combination of start symbol and length and PUSCH mapping type repeats over the consecutively allocated slots. To simplify the configurations of multiple CG PUSCH transmission occasions in a period of a single CG PUSCH in Rel-18 XR Enhancements, the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration may have the same symbol allocation in each CG PUSCH transmission occasion as the legacy NR-U framework. However, to reduce unnecessary listen before talk (LBT) channel access, only consecutive PUSCH allocations is supported for NR-U in Rel-16. Similarly, if considering some typical TDD frame structures (e.g., DDDSU), the limitation of the consecutive PUSCH allocations may be invalid in licensed carriers, and the consecutive PUSCH allocations can be reinterpreted as multiple (consecutive) available uplink PUSCH allocations in a period of a single CG PUSCH configuration. To further reduce configuration signaling overhead, the multiple (consecutive) available uplink PUSCH allocations can be defined as the earliest multiple (consecutive) available uplink PUSCH allocations in a period of a single CG PUSCH configuration. Therefore, the design of TDRA determination based on NR-U framework in Rel-16 can also be slightly enhanced to meet the requirement of the configuration of multiple CG PUSCH transmission occasions in a period of a single CG PUSCH for Rel-18 XR Enhancements.

Regarding Alt-C (i.e., TDRA determination based on single DCI scheduling multiple PUSCHs), the uplink grants fully configured by RRC signaling (i.e., type 1 CG) or given through DCI activation scrambled with CS-RNTI (i.e., type 2 CG) can indicate a TDRA row index indicating (or associated with) multiple SLIV entries and each SLIV entry is associated with one or more PUSCH transmission occasions of the multiple CG PUSCH transmission occasions. This mechanism has the best flexibility of resource allocation to support the multiple CG PUSCH transmission occasions in Rel-18 XR Enhancements compared with the other two mechanisms. However, if more flexible resource allocation needs to be supported for the configured multiple CG PUSCH transmission occasions in Rel-18 XR Enhancements, the size of TDRA entry list would be significantly extended and at same time the signaling overhead, especially for type 2 CG with the activation DCI, needs to be dramatically increased as well, since UE determines the bit width of the DCI field based on the number of entries in the PUSCH-TimeDomainResourceAllocationList. Additionally, in Type 1 CG, TDRA is directly configured by RRC signaling with a row index of a predefined TDRA row table. Similarly, if more flexible resource allocation needs to be supported for the configured multiple CG PUSCH transmission occasions in Rel-18 XR Enhancements, the corresponding predefined TDRA row table needs to be extended to include all the possible resource allocation rows, which inevitably makes the TDRA row table too large size and certainly is not preferred.  From our point of view, to simplify the complexity of configuration of multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration in Rel-18 XR Enhancements, at least the same symbol allocation for the multiple CG PUSCH transmission occasions should be supported. To that end, the TDRA determination based on single DCI scheduling multiple PUSCHs is not necessary and is not preferred, because of its high signaling overhead in physical layer and/or too large table size for the corresponding predefined TDRA row table if type 1 CG also supported by the adopted alternative.

Observation 1: Both the repetition framework (i.e., Alt-A) and NR-U framework in Rel-16 (i.e., Alt-B) can support the configuration of multiple CG PUSCH transmission occasions in a period of a single CG PUSCH in Rel-18 XR Enhancements.

Proposal 1: Support at least the same symbol allocation for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration in Rel-18 XR Enhancements.

Proposal 2: The configured multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration in Rel-18 XR Enhancements can take repetition framework (i.e., Alt-A) or NR-U framework in Rel-16 (i.e., Alt-B) as the baseline.

## Transmission and retransmission of the multiple CG PUSCH transmission occasions

### HARQ process determination for the multiple CG PUSCH transmission occasions

In terms of the HARQ process ID(s) determination for multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration, the below agreements and conclusions were achieved in RAN1 #112 meeting:

Conclusion RAN1 discusses to decide how to determine the HARQ process ID of CG PUSCHs of a multi-PUSCHs CG. Agreement For determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH, consider the following alternatives: - Alt. 1: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period. - Alt 1-1; X = 1 - Alt 1-2: X is the number of configured PUSCHs in a period - Alt 1-3: X is provided by RRC configuration. - FFS details - Alt. 2: Support that UE can decide, as in NR-U, the HARQ IDs for the multiple CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - FFS details - Alt. 3: The HARQ process ID for the configured PUSCHs in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - FFS on potential enhancements different from previous alternatives - Alt 3-1: Note: Same HP ID would be used for all PUSCHs within a period. - FFS details - Alt 3-2: Note: Different HP ID could be used for all PUSCHs within a period. - FFS details - Alt. 4: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS on potential enhancements different from previous alternatives - Alt 5: Support that UE can decide, as in NR-U, the HARQ IDs for the first CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS details - Alt 6: FFS other solutions Note: The case of one TB map to multiple PUSCHs is not considered here.

How to determine the HARQ process identifier(ID)(s), for multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration, was intensively discussed in last RAN1 meeting and five alternatives were figured out and agreed for further down selection. For the down selection from the agreed candidate alternatives and/or FFS other solutions, we think the finally adopted solution should be much flexible enough to support all possible traffic models/patterns of XR even in future, since XR is still an infantile field and new applications would continuously come out in the years ahead, meanwhile, new traffic model/pattern of XR more likely would also be introduced in future. For example, based on the latest RAN2 agreement in RAN2#121, RAN2 thinks uplink jitter may be present for XR (e.g., for tethering use cases). Therefore, the new introduced traffic model/pattern of XR in future should have as less standardization impacts on the finally adopted solution as possible. To that end, the finally adopted solution should be flexible enough to support future different frame rates, different jitter ranges, different frame packet sizes, different packet delay budgets, etc. Overall, the current five alternatives listed in the above agreement can be classified into two types, in which the first type is formula based HARQ process ID determination, and the second type is UE based HARQ process ID determination. The formula based HARQ determination includes Alt. 1, Alt. 3, and Alt. 4. The UE based HARQ determination includes Alt. 2 and Alt. 5. Additionally, based on the conclusion achieved in last RAN1 meeting, RAN1 will decide how to determine the HARQ process ID(s) of CG PUSCHs of a multi-PUSCHs CG. However, some critical details of all the alternatives in the agreement are still not clear, especially for the formula based HARQ ID determinations. In legacy CG configured with single CG PUSCH occasion in a period of a single CG PUSCH configuration, the HARQ process ID associated with the first symbol of the single CG PUSCH occasion is derived based on a regular periodicity of CG resources in the legacy Type 1 CG and Type 2 CG as below,

TS 38.321V17.2.0 5.4.1 UL Grant reception … For configured uplink grants neither configured with harq-ProcID-Offset2 nor with cg-RetransmissionTimer, the HARQ Process ID associated with the first symbol of a UL transmission is derived from the following equation: HARQ Process ID = [floor(CURRENT_symbol/periodicity)] modulo nrofHARQ-Processes where CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211 [8].

Given that the mismatch between non-integer periodicity of XR traffic and the legacy integer DRX cycles is a well-known problem, it was also agreed to adopt the periodicity mismatch issue as one requirement to be specified (i.e., DRX support of XR frame rates corresponding to non-integer periodicities) in the scope of the WID on XR Enhancements for NR as in the appendix. Hence, we think the periodicity mismatch between non-integer periodicity of XR traffic and the legacy integer CG periodicity has the same problems and also needs to be solved, regardless of single or multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration. To solve the periodicity mismatch between non-integer periodicity of XR traffic and the legacy integer CG periodicity, a straightforward approach is to configure a nominal periodicity, for the CG, which is the same as the non-integer periodicity of XR traffic. Other potential methods may include further rounding operations (e.g., ceiling operation or flooring operation) based on the nominal periodicity, nested configuration with multiple periodicities, etc. Therefore, based on the above potential methods, the finally configured CG resources, for multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration, may have a nominal periodicity which is the same as non-integer periodicity of XR traffic, or may have no regular periodicity which cannot be used directly in the legacy formula of HARQ process ID determination. From our point of view, to simplify or reuse the formula based HARQ process ID determination as in legacy CG, the periodicity of CG resources in legacy HARQ process ID determination formula needs to be replaced with non-integer periodicity of XR traffic or a nominal periodicity of CG resources, which is the same as non-integer periodicity of XR traffic. Therefore, we have the following observation and proposal:

Observation 2: Due to the regular periodicity of the configured CG resources for CG PUSCH configuration being not guaranteed, the legacy HARQ process ID determination mechanism based on regular periodic resource allocation (i.e., Alt. 1, Alt. 3, and Alt. 4) can’t be applied directly to the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration for uplink XR traffic.

Proposal 3: Support that the periodicity of CG resources in legacy HARQ process ID determination formula (i.e., Alt. 1, Alt. 3, and Alt. 4) can be replaced with non-integer periodicity of XR traffic or a nominal periodicity of CG resources which is the same as non-integer periodicity of XR traffic if formula based HARQ process ID determination supported.

As mentioned in uplink XR traffic model in the appendix, model 1 of uplink AR traffic model may have some statistical parameters including different jitter range values. More specifically, the range of jitters for uplink XR traffic could be [-4, 4] ms, [-5, 5] ms, or may be even larger in some scenarios in future. To fully (or partially) avoid the jitter’s impacts on resources allocation and improve transmission performance for the CG configuration, a time offset value needs to be introduced between the regular arrival time of XR packet and starting time of allocations for CG PUSCH transmission. Therefore, the legacy HARQ process ID determination mechanism based on regular periodic resource allocation of CG can’t be applied directly to the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration for the uplink XR traffic in this case, unless a time offset is also introduced to the HARQ process ID determination and the CG resource periodicity is replaced with the periodicity of uplink XR traffic as in the below formula:

HARQ Process ID = [floor((CURRENT_symbol – time_offset)/(periodicity/X))] modulo nrofHARQ-Processes

where, the time_offset is semi-static configured by RRC messages (or dynamically indicated by MAC CE or DCI), the periodicity is the periodicity of uplink XR traffic or a nominal periodicity of CG resources (X may be the number (X) of the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration; or X may be 1) in terms of OFDM symbols, and CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot. Furthermore, to compare the results of the formula based HARQ process ID determinations (e.g., Alt.1, Alt.3 and Alt.4) with the enhancement based on the time offset value, another alternative is also defined as Alt.1 with time offset. All the results of the formula based HARQ process ID determinations, including Alt.1, Alt.3, Alt.4, and Alt.1with time offset, are shown in Figure 2, in which the XR traffic frame rate is 60 fps, jitter range is [-5, 5] ms, the number of multiple CG PUSCH occasion in a period is 8, and the total number of HARQ processes is 16. In Alt.1 with time offset, the HARQ process ID for the first configured/valid PUSCH in a period is determined based on the above HARQ process ID determination formula with the time offset, and the HARQ process ID of the remaining PUSCHs in the same period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period.

Figure 2: Example results of HARQ process ID determination using Alt. 1, Alt. 1 with time offset, Alt. 3, and Alt. 4 based on XR traffic periodicity, 8 PUSCH transmission occasions per period and the total number of HARQ processes 16

From the results, we can see, if considering jitter impacts on CG PUSCH resource allocation and a time offset introduced between the regular arrival time of XR packet and starting time of allocations for CG PUSCH transmission, Alt.1 may have some problems because HARQ process ID (HPI) exceeds the maximum number of the total number of HARQ processes based on the current rule or definition (e.g., HPI=16 emerges based on the current rules if the total number of HARQ processes is 16 in this case). After the time offset value is introduced in the HARQ process ID determination formula as mentioned above, Alt.1 with time offset can obtain the best results of all the alternatives (including Alt.1, Alt.3, Alt.4, and Alt.1with time offset) (e.g., the time offset = 1ms or 2 ms) as shown in Figure 2. Therefore, based on the example results of different formula based HARQ process ID determinations with jitter impacts and impacts of irregular periodicity of CG PUSCH resources, we have the following observation and proposal:

Observation 3: To overcome (or reduce) jitter impacts on CG PUSCH resource allocation, a time offset value, between the regular arrival time of XR traffic and the CG PUSCH resource allocation, needs to be introduced, which may impact the HARQ process ID determination results for formula based HARQ process ID determinations (i.e., Alt 1-2 and Alt 1-3).

Proposal 4: Support that a time offset value is introduced for the formula based HARQ process ID determination (i.e., Alt. 1-2 and Alt. 1-3) to avoid jitter impacts on CG PUSCH resource allocation and HARQ process ID determination if formula based HARQ process ID determination supported (i.e., Alt. 1).

Even though formula based HARQ process ID determinations (such as Alt. 1, Alt. 1 with time offset, Alt. 3, and Alt. 4) have many benefits, including high reliability and much less signaling overhead, the flexibility of HARQ process ID allocation for the formula based HARQ process ID determinations are much lower because of the fixed association relationship between HARQ process ID(s) and the allocation of the CG PUSCH resources. For instance, if a CG PUSCH transmission occasion, of the multiple CG PUSCH transmission occasions, is not used by UE, the HARQ process ID associated with the CG PUSCH transmission occasion may be wasted. One potential approach is that UE can indicate the unused HARQ process ID(s) to gNB, and then gNB can further reallocate the unused HARQ process ID(s) to dynamic grant-based scheduling for the UE. However, the uplink signaling would be inevitably increased because of the indications of unused HARQ process ID(s), and the system complexity is increased as well if considering XR traffic allocated for the same unused HARQ process ID(s) in future periodicity periods. In that regard, UE can decide the HARQ process IDs for the multiple CG PUSCH transmission occasions and indicate the determined HARQ process IDs to gNB, similarly as in Rel-16 NR-U, which has the best flexibility for HARQ process ID determinations (e.g., Alt.2 and Alt.5). If a CG PUSCH transmission occasion, of the multiple CG PUSCH transmission occasions, is not used by UE, UE need not allocate HARQ process ID for the unused CG PUSCH transmission occasion. Furthermore, periodicity mismatch issue and jitter range issue have much less impacts on UE based HARQ process ID determination. Even the uplink signaling overhead is higher than formula based HARQ process ID determinations, the standardization efforts are much low because that part was already specified in Rel-16 NR-U and can be reused directly for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration.

Proposal 5: Support that UE can decide, as in NR-U, the HARQ process ID(s) (i.e., Alt.2 and Alt.5) for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration for uplink XR traffic and indicate the decided HARQ process ID(s) to gNB.

### Retransmission of the multiple CG PUSCH transmission occasions

Regarding retransmission for CG PUSCH transmission, three retransmission mechanisms were specified in previous releases from Rel-15 to Rel-17, which includes:

1) Based on dynamic uplink grant via dynamic grant resources;

2) CG-DFI based retransmission;

3) Timer-based retransmission.

The first retransmission mechanism is based on dynamic uplink grant via dynamic grant resources and UE may assume ACK in absence of reception of feedback after a timer expires. If CBG based retransmission is supported to increase retransmission efficiency in the first retransmission mechanism, the dynamic uplink grant needs to include a CBGTI field, which indicates NACK/ACK for each corresponding CBG. And then, UE may retransmit the corresponding CBGs if NACKs are received in the CBGTI field for the corresponding CBGs. However, for NR-U in Rel-16, it was identified that it is problematic for the UE to assume ACK in absence of reception of feedback because of LBT impacts or hidden terminal issues, which results in introducing explicit feedback in the form of uplink grants (i.e., CG-DFI) for PUSCH transmission. That is, the second retransmission mechanism is CG-DFI based retransmission. More specifically, if the corresponding bit in CG-DFI indicating a NACK for a TB, UE may perform retransmission for the corresponding TB via configured grant resources. Meanwhile, due to high signaling overhead, for example, CBGTI field needs to be added in CG-DFI and CG-UCI, CBG based retransmission via configured grant resources is not supported for NR-U in Rel-16. Furthermore, it was additionally identified in NR-U Rel-16 that UE assuming NACK upon a timer expiration can be helpful to avoid LBT impact on reception of the feedback. The third retransmission mechanism is a timer-based retransmission, in which a timer starts when a TB is transmitted, and if no ACK is received before the timer expires, UE assumes NACK and perform non-adaptive retransmission via configured grant resources. The second and the third retransmission mechanisms are specifically designed for shared spectrum access to reduce the LBT impacts or hidden terminal issues on retransmission. Therefore, the first retransmission mechanism can be reused for retransmission of the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration.

Proposal 6: Retransmissions of the multiple CG PUSCH transmission occasions, in a period of a single CG PUSCH configuration, are based on dynamic uplink grant(s) via dynamic grant resources and UE assumes ACK(s) in absence of reception of feedback after a timer expires if retransmissions for the multiple CG PUSCH transmission occasions supported.

## Dynamic indication of unused CG PUSCH occasion(s) based on UCI

### UCI used for indicating unused CG PUSCH occasion(s)

Regarding indications of unused CG PUSCH transmission occasion(s) in a period of a single CG PUSCH configuration, the below agreements were achieved in RAN1 #112 meeting:

Agreement For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, the following options for further down-scoping with possible revision, are considered for the transmission occasion of the UCI: - Option 1: A transmitted CG PUSCH, includes the UCI. - FFS details - Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC. - FFS details - Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion. - FFS details - Example of a pre-determined occasion: 1st configured PUSCH TO in a CG period or 1st configured PUSCH TO in a multiple CG periods - Option 4: A transmitted CG PUSCH includes the UCI, if it is transmitted in a transmission occasion determined satisfying given condition(s). - FFS details - Examples of a condition: A first transmitted PUSCH in a CG period, or a first PUSCH transmission within a multiple of CG periods. Other options are not precluded. Proponent companies to provide details. Agreement For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the following options for further down-scoping, are considered for the information provided by the UCI: - Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused” - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain. - Applicable numbers can be determined from information obtained from configuration. - FFS details - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain) - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - FFS whether/how the unused TO(s) can be associated to multiple CG configuration. - Other options are not precluded. Proponent companies to provide details.

As mentioned in the scope of the WID in the appendix, unused CG PUSCH occasion(s) need to be indicated to gNB based on UCI as shown in Figure 3. For the transmission occasion of the UCI, four options were figured out and agreed as candidate alternatives for further down selection. When performing the down selection, we think some basic rules should be followed to make sure the unused CG PUSCH transmission occasion(s) are efficiently reallocated to other UEs by the gNB:

1) Firstly, the adopted option should have as little uplink signaling overhead as possible to reduce complexity and power consumption for both UE and gNB;

2) Secondly, the adopted option should be flexible and robust enough to overcome (or reduce) the impacts of jitters or other interferences for XR traffic;

3) Additionally, the adopted option should make sure the transmission occasion of the UCI is relatively fixed and transmitted as early as possible, during the multiple CG PUSCH transmission occasions, to reduce detection complexity and waiting time in gNB.

Even if no option of the above four candidate options can fully meet all the requirements of the three basic rules, a reasonable tradeoff can be made, for the down selection based on the three basic rules, to avoid only considering some rules but ignoring others. Obviously, Option 1 has highest flexibility and robust, but the signaling overhead is too high compared with other options because all the transmitted CG PUSCH occasions include the UCI. Another problem for Option 1 is that the UCI indication could not be used by gNB if the transmission occasion of the UCI is too close to the unused transmission occasion indicated by UCI. Hence, Option 1 is not preferred from our point of view. In our understanding, Option 3 and Option 4 are almost similar, and have the lowest flexibility and robust compared with other options, especially in dynamic jitter impact situation for XR traffic, for instance, if considering jitter impacts on the 1st configured PUSCCH transmission occasion, the fixed 1st configured PUSCCH transmission occasion may be not a good choice to carry UCI. In Option 2, the transmitted CG PUSCH including the UCI can be indicated by RRC, which can fully meet the last two basic rules, except for some extra downlink indication signaling overhead. However, in our understanding, Option 2 is not necessary to be limited to only RRC signaling, and dynamic signaling including MAC CE and DCI (e.g., CG activation DCI) can also be considered as well. Therefore, from our point of view, Option 2 is preferred compared with other three options. Based on the indications of RRC messages (or MAC CEs/DCIs), UE can transmit the UCI indicating the unused CG PUSCH occasion(s) via the candidate positions indicated by gNB. The candidate positions may be the first CG PUSCH transmission occasion of the multiple CG PUSCH transmission occasions, the first CG PUSCH transmission occasion with an actual uplink transmission, or one or more other CG PUSCH transmission occasions of the multiple CG PUSCH transmission occasions, which may or may not have PUSCH data.

Proposal 7: Support that gNB can transmit, to UE, RRC message(s) (or MAC CE/DCI) indicating the candidate positions for UCI transmission (i.e., Option 2).

  - Figure 3: Dynamic indication of unused CG PUSCH occasion(s) based on UCI

Regarding the design of the UCI indicating unused CG PUSCH occasion(s), two options (i.e., Option 1 and Option 2) were figured out and agreed for further down selection. The first option (i.e., Option 1) means that the bitfield of the UCI can jointly indicate a quantity of consecutive unused CG PUSCH occasions, which includes Option 1-1 and Option 1-2. Option 1-1 is shown in the below Table 1, for example, in which the number of the multiple CG PUSCH transmission occasions is 6 and the bitwidth of the bitfield is 3 bits. To reduce signaling overhead in physical layer, for instance, the starting point of the quantity of unused CG PUSCH occasions can be from the last CG PUSCH transmission occasion of the multiple CG PUSCH transmission occasions. If all the CG PUSCH occasions (e.g., 6 CG PUSCH occasions) are used by UE, the bitfield of UCI can be set to 000, which just indicates the occupancy status, of the multiple CG PUSCH occasions in UE side, to base station. If the bitfield of UCI is set to 101, the number of unused CG PUSCH occasions is 5 (i.e., the last five CG PUSCH transmission occasions) and the number of used CG PUSCH occasion is 1 (i.e., the first CG PUSCH transmission occasion). Similarly, if the bitfield of UCI is set to 100, the number of unused CG PUSCH occasions is 4 (i.e., the last four CG PUSCH transmission occasions) and the number of used CG PUSCH occasion is 2 (i.e., the first two CG PUSCH transmission occasions). Option 1-2 is almost the same as Option 1-1, except for the additional concept of duration/range. The second option (i.e., Option 2) is based on bitmap indication of UCI, which includes Option 2-1 and Option 2-2. In Option 2-1, each bit of the bitmap in UCI can be associated with one of the multiple CG PUSCH transmission occasions, for example, the number of the multiple CG PUSCH transmission occasions also is 6. For instance, if a bit is set to 1, the CG PUSCH occasion associated with the bit is unused. Instead, if the bit is set to 0, the CG PUSCH occasion associated with the bit is used. The bitwidth of the bitmap, for the UCI, indicating unused CG PUSCH occasion(s) may be equal to the number of the multiple CG PUSCH transmission occasions. For example, if the bitmap is set to 000111 (e.g., bitwidth of the bitmap is 6 bits), the last three CG PUSCH occasions are unused and the first three CG PUSCH occasions are used. Similarly, Option 2-2 is almost the same as Option 2-1, except for the additional concept of duration/range. Obviously, the first option (i.e., Option 1) can reduce signaling overhead in DCI with less flexibility compared to the second option (i.e., Option 2). In our view, the additional concept of duration/range introduced for Option 1-2 and Option 2-2 is not necessary, which could introduce more configurations signaling overhead, especially in RRC layer, and make the simple problems become more complicated. The comparisons of pros and cons for different options in terms of flexibility, signaling overhead and whether RRC signaling needed are shown in the below Table 2.

Table 1: Bitfield indication for UCI

| - Bitfield [b2 b1 b0] for Option 1-1 | - Unused CG PUSCH occasions |
| --- | --- |
| - 000 | - 0 |
| - 001 | - 1 |
| - 010 | - 2 |
| - 011 | - 3 |
| - 100 | - 4 |
| - 101 | - 5 |
| - 110 | - 6 |
| - 111 | - Reserved |

Table 2: Comparison among candidate options

|  | Flexibility | Signaling overhead | RRC signaling needed |
| --- | --- | --- | --- |
| Option 1-1: The UCI provides the number of consecutive TO(s) in time domain | medium | low | no |
| Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain | low | medium | yes |
| Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range | high | high | no |
| Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range | medium | medium | yes |

Proposal 8: Support that bitfield of the UCI jointly indicates a quantity of unused CG PUSCH occasions starting from the last CG PUSCH transmission occasion of the multiple CG PUSCH transmission occasions (i.e., Option 1-1).

### Timing relationship for indication of the unused CG PUSCH occasion(s)

The motivation for UCI indicating unused CG PUSCH occasion(s) to gNB is to recycle the unused CG PUSCH occasion(s) to other UEs. When gNB schedules an unused CG PUSCH occasion to other UEs, gNB needs to determine a time offset K2 based on UE capability of PUSCH preparing time, and further indicates the time offset K2. More specifically, the time offset K2 is a time difference between the DL slot where the PDCCH(DCI) for uplink scheduling is received and the UL Slot where the UL data needs to be sent on PUSCH, as shown in below Figure 4. Then, the other UEs can perform uplink transmission via the unused CG PUSCH occasion to the gNB based on the time offset K2. To guarantee the indicated unused CG PUSCH occasion(s) to be really recycled to other UEs, time offset between UCI and the indicated unused CG PUSCH occasion(s) should be equal to or greater than the PUSCH preparing time for at least one of the other UEs. Otherwise, the indicated unused CG PUSCH occasion(s) can’t be recycled by gNB to any of the other UEs, even though UCI is detected by the gNB. In that sense, some potential approach needs to be considered to make sure the unused CG PUSCH occasion(s) can be really scheduled to the other UEs, for example, indicating unused CG PUSCH occasion(s) to gNB can be based on a time offset threshold, indicated by gNB, between UCI and the unused CG PUSCH occasion(s). If time offset between UCI and the unused CG PUSCH occasion(s) is less than the time offset threshold, the UCI indications for the unused CG PUSCH occasion(s) may be wasted with high cost of uplink signaling overhead and power consumption for UE. The time offset threshold may be determined by gNB based on UE capability of PUSCH preparing of the other UEs and/or capacity of gNB. Additionally, if there is no downlink slot (or resources) between UCI and the unused CG PUSCH occasion(s), the unused CG PUSCH occasion(s) also can’t be recycled/scheduled by gNB to other UEs, in which indicating unused CG PUSCH occasion(s) to gNB is not necessary as well.

  - Figure 4: Timing relationship of dynamic indication of unused CG PUSCH occasion(s)

Observation 4: To guarantee the indicated unused CG PUSCH occasion(s) to be really recycled to other UEs, time offset between UCI and the indicated unused CG PUSCH occasion(s) should be equal to or greater than the PUSCH preparing time for at least one of the other UEs.

Proposal 9: Indicating unused CG PUSCH occasion(s) to gNB can be determined based on a time offset threshold, indicated by gNB, between UCI and the unused CG PUSCH occasion(s).

# Conclusions

Based on above discussions, we have the following observations and proposals:

Observation 1: Both the repetition framework (i.e., Alt-A) and NR-U framework in Rel-16 (i.e., Alt-B) can support the configuration of multiple CG PUSCH transmission occasions in a period of a single CG PUSCH in Rel-18 XR Enhancements.

Proposal 1: Support at least the same symbol allocation for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration in Rel-18 XR Enhancements.

Proposal 2: The configured multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration in Rel-18 XR Enhancements can take repetition framework (i.e., Alt-A) or NR-U framework in Rel-16 (i.e., Alt-B) as the baseline.

Observation 2: Due to the regular periodicity of the configured CG resources for CG PUSCH configuration being not guaranteed, the legacy HARQ process ID determination mechanism based on regular periodic resource allocation (i.e., Alt. 1, Alt. 3, and Alt. 4) can’t be applied directly to the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration for uplink XR traffic.

Proposal 3: Support that the periodicity of CG resources in legacy HARQ process ID determination formula (i.e., Alt. 1, Alt. 3, and Alt. 4) can be replaced with non-integer periodicity of XR traffic or a nominal periodicity of CG resources which is the same as non-integer periodicity of XR traffic if formula based HARQ process ID determination supported.

Observation 3: To overcome (or reduce) jitter impacts on CG PUSCH resource allocation, a time offset value, between the regular arrival time of XR traffic and the CG PUSCH resource allocation, needs to be introduced, which may impact the HARQ process ID determination results for formula based HARQ process ID determinations (i.e., Alt 1-2 and Alt 1-3).

Proposal 4: Support that a time offset value is introduced for the formula based HARQ process ID determination (i.e., Alt. 1-2 and Alt. 1-3) to avoid jitter impacts on CG PUSCH resource allocation and HARQ process ID determination if formula based HARQ process ID determination supported (i.e., Alt. 1).

Proposal 5: Support that UE can decide, as in NR-U, the HARQ process ID(s) (i.e., Alt.2 and Alt.5) for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration for uplink XR traffic and indicate the decided HARQ process ID(s) to gNB.

Proposal 6: Retransmissions of the multiple CG PUSCH transmission occasions, in a period of a single CG PUSCH configuration, are based on dynamic uplink grant(s) via dynamic grant resources and UE assumes ACK(s) in absence of reception of feedback after a timer expires if retransmissions for the multiple CG PUSCH transmission occasions supported.

Proposal 7: Support that gNB can transmit, to UE, RRC message(s) (or MAC CE/DCI) indicating the candidate positions for UCI transmission (i.e., Option 2).

Proposal 8: Support that bitfield of the UCI jointly indicates a quantity of unused CG PUSCH occasions starting from the last CG PUSCH transmission occasion of the multiple CG PUSCH transmission occasions (i.e., Option 1-1).

Observation 4: To guarantee the indicated unused CG PUSCH occasion(s) to be really recycled to other UEs, time offset between UCI and the indicated unused CG PUSCH occasion(s) should be equal to or greater than the PUSCH preparing time for at least one of the other UEs.

Proposal 9: Indicating unused CG PUSCH occasion(s) to gNB can be determined based on a time offset threshold, indicated by gNB, between UCI and the unused CG PUSCH occasion(s).

# References

Draft Report of 3GPP TSG RAN WG1 #112 v0.2.0 (Athens, Greece, 27th February – 3rd March 2023).

TS38.331 V17.2.0, NR; Radio Resource Control (RRC) protocol specification.

TR38.835 V1.0.0, NR; Study on XR enhancements for NR.

TR38.838 V17.0.0, NR; Study on XR (Extended Reality) Evaluations for NR.

# Appendix

## A. Uplink XR traffic model and CG in current specification

### A.1 Uplink XR traffic model

Extended reality (XR) refers to all real-and-virtual combined environments and human-machine interactions generated by computer technology and wearables [3]. XR is also an umbrella term for different types of realities including Virtual reality (VR), Augmented reality (AR), and Mixed reality (MR). VR is a rendered version of a delivered visual and audio scene. Here, the rendering is designed to mimic the visual and audio sensory stimuli of the real world as naturally as possible to an observer or user as they move within the limits defined by the application. AR is when a user is provided with additional information or artificially generated items, or content overlaid upon their current environment. MR is an advanced form of AR where some virtual elements are inserted into the physical scene with the intent to provide the illusion that these elements are part of the real scene.

According to the conclusions achieved in study phase of XR enhancements for NR, different types of realities may have different traffic models, which can be applied to both downlink XR traffic and uplink XR traffic. For uplink XR traffic, there are three traffic models, defined in [4], including uplink VR traffic model, uplink cloud gaming traffic model, and uplink AR traffic model. In model 1 of uplink AR traffic model, all AR uplink flows are modelled as a single stream with following parameters as shown in table 5.5.2.1-1 and table 5.1.1.2-1 [4].

Table 5.5.2.1-1: Statistical parameters for AR UL Model 1 (one stream model)

| Parameters | unit | value |
| --- | --- | --- |
| Packet size | byte | Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=10.5/50/150%) |
| packet generation rate: F | Hz | 60 |
| Jitter | ms | Optional, follows the description in clause 5.1.1.2 |
| Data rate: R | Mbps | 10 (baseline), 20 (optional) |
| PDB | ms | 30 (baseline), 10 or 15 or 60 (optional) |

Table 5.1.1.2-1: Statistical parameters for jitter

| Parameter | unit | Baseline value for evaluation | Optional value for evaluation |
| --- | --- | --- | --- |
| Mean | ms | 0 |  |
| STD | ms | 2 |  |
| Truncation range | ms | [-4, 4] | [-5, 5] |

Based on the statistical parameters for uplink AR traffic model 1, the XR uplink traffic can be modelled as a sequence of video frames arriving at UE according to the considered video frame rates and random jitter. Additionally, the size of each frame is also random and subject to a certain distribution, as shown in the below figure.

Uplink AR traffic model 1

To simplify problem analysis, the following discussions are mainly based on the uplink AR traffic model with single stream (i.e., uplink AR traffic model 1).

### A.2 CG in current specification

Two types of configured grant transmissions are supported in current NR specification, in which one is type 1 CG and another one is type 2 CG. For type1 CG, all parameters of configured grant are configured by RRC messages. However, for type 2 CG, only part of parameters of configured grant are configured by RRC messages and other parameters of configured grant are indicated by downlink control information (DCI), which not only indicates MCS and available resources, but also activates the type 2 CG, as shown in the below figure. In Type 1 CG, RRC messages provide all CG parameters including periodicity, time offset, time-frequency allocation, UE-specific DMRS configuration, MCS/TBS, #repetitions (K), power control, etc. In Type 2 CG, RRC messages provide periodicity, power control, #repetitions (K), while activation DCI provides time offset, time-frequency allocation, MCS/TBS, UE-specific DMRS configuration, etc. If the RRC messages include "rrc-ConfiguredUplinkGrant", UE performs transmission with fully RRC-configured UL grant (i.e., Type 1 CG). If this field is absent, UE uses UL grant indicated by DCI addressed to CS-RNTI (i.e., Type 2 CG). Regarding retransmission of transport block (TB) for CG in licensed carriers, hybrid automatic repeat request (HARQ) process supports two schemes, in which one scheme is dynamic grant-based retransmission after the first grant free transmission via corresponding CG resources and the other scheme is grant free transmission with multiple repetitions. If UE receives an explicit ACK from gNB for the TB before transmission of any repetition of the TB, UE can terminate the transmission of the TB. For shared spectrum scenario, UE can perform autonomous uplink retransmission of the TB via CG resources if UE receives CG downlink feedback information (CG-DFI) indicating NACK for the TB. If not receiving the CG-DFI (e.g., listen before talk (LBT) failure at gNB) before cg-RetransmissionTimer expires, UE may perform autonomous retransmission for the TB after the cg-RetransmissionTimer expires. For both type 1 CG and type 2 CG, the HARQ ID is implicitly determined based on the selected CG resource in licensed carriers. Instead, for shared spectrum scenario, UE determines the HARQ ID for uplink autonomous transmission and indicates the determined HARQ ID to gNB via CG uplink control information (CG-UCI) piggybacked on PUSCH. For CG with multiple repetitions, the HARQ redundancy version (RV) for each repetition of the TB is implicitly determined based on the index of the repetition transmission occasion and an RV sequence configured by RRC messages in licensed carriers. But for shared spectrum scenario, UE determines the RV for uplink autonomous transmission/retransmission and indicates the determined RV to gNB via CG-UCI piggybacked on PUSCH.

Type 2 CG

## B. WI scope for New WID on XR Enhancements for NR

### B.1 Objective of SI or Core part WI or Testing part WI

Specify the enhancements related to power saving:

-	DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2).

Specify the enhancements related to capacity:

-	Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2);

-	Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2);

-	Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2);

-	Delay reporting of buffered data in uplink (RAN2);

-	Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

Specify the enhancements for XR Awareness:

-	Signalling by CN of semi-static information per QoS flow (e.g. PDU set QoS parameters), dynamic information per PDU set (PDU Set information and Identification) and End of Data Burst indication (RAN3, RAN2);

-	Impact of identifying by UE of PDU Sets, Data bursts and PSI, as needed (RAN2);

-	Provisioning by UE of XR traffic assistance information e.g. periodicity, UL traffic arrival information (RAN2, RAN3);

-	Support signalling the congestion information from RAN to the CN in alignment with SA2 (RAN3);