3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302323

e-Meeting, April 17 - 26, 2023

Agenda Item:	9.6.1

Source:	Futurewei

Title:	Discussion on R18 RedCap complexity

Document for:	Discussion/Decision

# Introduction

In RAN#98, a work item [1] was approved and later revised [2] following the completion of the study item. The objectives of this revised work item include:

Power saving/energy efficiency enhancements - Enhanced eDRX in RRC_INACTIVE (>10.24s) [RAN2, RAN3, RAN4] - Note that this objective requires SA2, CT1 and CT4 involvement Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified. Notes: - The work defined as part of this WI is not to overlap with LPWA use cases. - Coexistence with non-RedCap UEs and Rel-17 RedCap UEs should be ensured. - This WI considers all applicable duplex modes unless otherwise specified. Check in RAN#99 regarding: - Whether UE peak data rate reduction for UE is limited only with UE BB bandwidth reduction or standalone

In RAN#99, discussions regarding the checkpoint led to a subsequent agreement [3] (presented in Appendix B for convenience). This contribution includes proposals resulting from the agreement.

In RAN1#112, there were some agreements requiring down-selection and one working assumption needing further discussion. In addition, some topics remain open. This contribution addresses those various topics.

# Discussion

The decisions from RAN1#110b to RAN1#112 [4] are captured in Appendix A. In this section, there are proposals based on the agreements in RAN1#112 and remaining questions / down-selections from the FL summary [5] are presented.

## RAN plenary discussion

RAN#99 had a checkpoint to decide whether standalone PR1 (10 Mbps data rate without a 5MHz baseband limit) is included within the Rel-18 RedCap work item. In the discussions for the checkpoint, an agreement [3] was that a Rel-18 RedCap (eRedCap) device is designed/targeted to the same peak data rate, i.e., 10 Mbps. With this agreement, standalone PR1 devices and baseband reduction with add-on PR1 devices are both considered as eRedCap devices. The agreement has several notes with one being the PRB processing capability for standalone PR1 corresponds to the PRB size for 20 MHz. A related note states that the values from the formula vlayers×Qm×f may be different for both devices. Another note (Note 4) is initial access procedure is the same for both devices.

Note 4 is significant in that despite both standalone PR1 UEs and baseband reduced complexity UEs having some differences which affect initial access, both UEs follow the same Rel-18 initial access procedure. In particular, accessing as a Rel-17 RedCap was not agreed to be included within Note 4 (see RP-230777). The implication is that neither access control nor early indication will be used to distinguish between the standalone PR1 and baseband reduced complexity UE variants.

Observation 1: Access control and early indication do not distinguish standalone PR1 and baseband reduced complexity UE variants.

## UE BB bandwidth reduction.

### Msg3 timeline

In RAN1#112, an agreement for the value of X requires a down-selection between 3 options [4].

For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs

Among the factors to consider for selecting a value of X are the times for PDSCH RB processing (e.g., FFT, receiver processing block [channel estimation, equalization, LLR generation]) and for TB processing (e.g., decoding). Regardless of the maximum number of RBs can process per slot, a UE still must perform a 20 MHz FFT and estimate a 20 MHz channel. While the processing for other tasks of equalization and LLR generation typically scales linearly with the number of RBs, the amount of processing is generally not too large. For the decoding, the UE must already support decoding TBS of 10,000/5,000 bits each slot for 15/30 kHz SCS, respectively.  With these observations, we favor a down-selection between 0.5/0.25 and 1/0.5 ms, with no preference to either pair.

Proposal 1. Select X for the scheduling of RAR PDSCH as either 0.5/0.25 or 1/0.5 ms for 15/30 kHz SCS, respectively

With Note 4 regarding the initial access procedure for standalone PR1, a standalone PR1 device must support the same value of X as above even though it can process all the RBs in 20 MHz. We propose

Observation 2. For the scheduling of RAR PDSCH, the network applies the same value of X for a Rel-18 eRedCap UE capable of 20MHz + PR1 and a Rel-18 eRedCap UE capable of BW3/PR3 + PR1

### Additional separate early indication(s)

With the note regarding the initial access procedure for standalone PR1, a standalone PR1 device must support the additional separate early indication (EI) as a BW3/PR3 + PR1 device. We propose

Proposal 2. A Rel-18 eRedCap UE capable of 20MHz + PR1 supports the same additional separate early indication(s) as a Rel-18 eRedCap UE capable of BW3/PR3 + PR1

In RAN2#121, RAN2 agreed to support Msg3 EI for Rel-18 RedCap UEs but left Msg1 EI up to RAN1. Unfortunately, RAN1 could not reach any agreements on whether to support a Msg1 early indication (EI) in RAN1#112.

For Rel-18 UEs capable of BW3/PR3, Msg1 EI informs the network about the presence of these UEs to ensure that the timing for Msg3 is correct and that allocation for Msg3 is supported. The timing for Msg3 is based on the size of the PDSCH carrying Msg2 while the size of Msg3 is based on the service supported.

The size of the PDSCH carrying Msg2 depends on two factors: payload size (number of UEs scheduled) and channel coding, and can easily exceed the eRedCap UE processing limit as shown in the following. The payload size in bits can be expressed as overhead (8) + number of UEs * 64 while the channel coding maps each payload bit into 4.266 REs (1/0.2344 from Table 5.1.3.1-1 of [7]) assuming MCS0. The number of RBs is then #REs / 156 REs/RB / S where S  {1, 0.5, 0.25} is the scaling factor for TB scaling. In one example [6], two MAC RARs are scheduled in one PDSCH. With the expressions above, the number of RBs without TB scaling is at least 4 RBs. With a TB scaling factor S=0.25, there are 16 RBs. If 30 kHz SCS were used, the 12 RB processing limit would apply. In another example, eight MAC RARs are scheduled. Without any TB scaling, there are at least 14 RBs, which exceeds the 12 RB processing limit for 30 kHz SCS. While one can argue about the MCS used or the amount of TB scaling, these are allowed values that may depend on the scenario, and these examples show the eRedCap UE processing limit can be easily exceeded.

The size of Msg3 was also questioned. It is important to note that the size of Msg3 is independent of the processing delay. Without Rel-18 Msg1 EI, the network must assume the smallest possible size for Msg3 for each UE based on the potential Msg3 payload and the amount of coding needed. In one example, assuming MCS0 and 12 RBs (the limit for 30 kHz SCS), the number of information bits is about 440 bits (=12 RBs * 156 REs/RB * 0.2344 bits/RE). While it is true 440 bits is more than sufficient for the 64-bit MAC CE CCCH1 in Msg3, the UE may be required to transmit other control elements in Msg3. For instance, a UE may support small data transmission (SDT) using RACH. Because contention free random access (CFRA) is not supported over RACH (clause 18.0 of [8]), a UE can transmit a Msg3 larger than 440 bits for SDT. This example shows Rel-18 Msg1 EI is needed to ensure the size of Msg3 is not limited due to Rel-18 RedCap UEs.

Observation 3. There are many situations where the size of PDSCH carrying Msg2 and where the size of Msg3 exceed the maximum RB limit for baseband complexity reduction.

In all the discussions about Msg1 early indication, the important point is the network must be able to choose how it manages initial access for Rel-18 RedCap UEs. If Msg1 EI is not supported at all, there could be significant network restrictions or impacts.

Without Rel-18 separate EI in Msg1, the network is unaware of the presence of eRedCap UEs, and the network may have to implement approaches to accommodate eRedCap UEs for Msg3. These approaches can waste of network resources and processing. In one possible approach, the network can delay the scheduling of Msg3 for all UEs by at least one or more uplink slots (depending on the value of X). The delay may complicate network scheduling, especially for TDD systems where the next available uplink slot may be several time slots later.

In another approach, when the network fails to receive Msg3 from the Rel-18 RedCap UE (due to the processing delay), the network may attempt to receive the Msg3 by transmitting another Msg2. While this sequence of actions can occur for non-RedCap UEs, the failure to receive Msg3 is due to channel conditions or collisions, not processing times. There is a huge impact on network resources especially when the number of eRedCap UEs is large. Before using such an approach, the network may attempt to receive an eRedCap Msg3 in a later slot assuming that an eRedCap UE transmits based on its TDRA field. This extra decoding may possibly avoid transmitting another Msg2 for that eRedCap UE.

Observation 4. There is network impact to accommodate the additional delay for processing Rel-18 RedCap Msg2 without Msg1 EI.

With support of Rel-18 separate EI in Msg1, the network can choose to partition its RACH preambles or to have separate RACH occasions. But it is the network’s choice to decide how to manage initial access.

An example where a Network may elect to use separate Rel-18 Msg1 EI was the 20MHz deployment scenario, where a Rel-18 RedCap UE would need a Rel-18 Msg1 EI but a Rel-17 RedCap UE does not need a Rel-17 Msg1 EI (as discussed in our contribution [9]). A feature lead proposal was discussed in RAN1#112 [5] where companies were OK with the scenario and need for a Msg1 EI only for the Rel-18 RedCap UEs. However, the proposal could not be agreed because some companies wanted the separate Rel-18 Msg1 EI to be used ONLY when the Rel-17 Msg 1 EI was not being used, to limit possible Msg1 preamble fragmentation.

Observation 5. A Rel-18 Msg1 EI is required (at least) in the case where Rel-17 does not use Msg1 EI.

At the UE side, however, there is no difference in implementation of the Msg1 EI – Msg1 EI is anyway already implemented in the UE, and it does not care who else is sharing the preambles/occasions.

Observation 6. There is no impact to the UE for supporting a separate Msg1 EI.

In our view, it should be up to the network whether to configure (using the same Rel-17 early indication framework) a dedicated Rel-18 RedCap Msg1 EI or to “take the hit” in increased processing / larger delays. In any case it should be obvious to all that a network should be able to implement a separate EI for Rel-18 RedCap UEs in a 20MHz deployment scenario, where the EI does not apply to the Rel-17 RedCap UEs that are treated similarly to the non-RedCap UEs.

Based on our analysis regarding Msg1 EI, we propose:

Proposal 3. Support an additional separate Msg1 early indication for Rel-18.

### Separate Initial BWP

In RAN1#110bis, the following agreement was reached regarding the relationship between the separate initial DL/UP BWP for Rel-17 and Rel-18 RedCap UEs (eRedCap UEs).

Agreement For a cell supporting both Rel-17 and Rel-18 RedCap UEs, - The Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs. - FFS: whether to support an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs

There were discussions in RAN1#112 attempting to resolve the FFS with the last proposal in the FL summary [5] and the outcome from the online session below.

High Priority Proposal 2.4-1d: Down-select between the following alternatives: - Alt. 1: - No additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is configured by the SIB1 in the cell. - Whether it should be supported that Rel-18 RedCap UEs use a separate initial DL/UL BWP while Rel-17 RedCap UEs and non-RedCap UEs use the normal initial DL/UL BWP is up to RAN2. - Alt. 2: - RAN1 concludes that there is no significant RAN1 impact from supporting additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs, and that the decision can be left up to RAN2/RAN4. The following conclusion was made in the Wednesday online session: Conclusion: There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell”.

In Rel-17, there are several properties of the separate initial DL/UL BWP. (a) When configured by the base station, it is transitory as it is only active during initial access (or SDT). (b) It is up to the network whether to configure the separate initial DL/UL BWP when the bandwidth of the initial DL/UL BWP for non-RedCap UEs is not larger than the maximum RedCap UE BW. (c) For TDD systems, because the center frequencies of the initial DL/UL BWP used by RedCap UEs must be aligned, a separate initial DL/UL BWP may be needed. (d) When the bandwidth of the initial DL/UL BWP for non-RedCap UEs is larger than the maximum RedCap UE BW, a separate initial DL/UL BWP is needed.

There is one possible network benefit of the separate initial DL BWP during initial access. If RedCap UEs and non-RedCap UEs share CORESET#0 in the same slot, the network can only use the PDSCH resources within the span of CORESET#0 for both UEs. From the network perspective, when a different CORESET (not CORESET#0) is configured, the number of resources available for PDSCH during initial access increases because the network can schedule PDSCH within the span of CORESET#0 for non-RedCap UEs and within the CORESET for RedCap UEs separately. This network benefit is also possible if RedCap and non-RedCap UEs use CORESET#0 in different slots.

When considering an additional separate initial DL/UL BWP for Rel-18, the perspectives of the UE and network should be considered. For a Rel-18 RedCap UE, it is possible that the network broadcasts a Rel-17 separate initial DL/UL BWP and a Rel-18 additional separate initial DL/UL BWP configurations in SIB1. While a Rel-18 RedCap UE uses only one DL/UL BWP configuration, it must be sure to use the correct one. From the network perspective, the network must broadcast the configurations for additional separate initial DL/UL BWP (increasing the overhead of SIB1). In addition, the network would need to manage up to 3 initial DL/UL BWP configurations (non-RedCap, Rel-17 RedCap, Rel-18 RedCap) and may complicate scheduling.

The proponents of the additional separate initial DL/UL BWP cite one benefit is power savings due to using a smaller DL BWP. While a using smaller DL BWP can provide power savings, the amount of power savings during initial access is unclear because the UE does not use the additional separate initial DL BWP for a long period of time (only for Msg2 – Msg5). It is possible that the UE uses this BWP for a longer duration. For example, when four Rel-18 RedCap UEs perform initial access at the same time, the network can service one UE at a time instead of four UEs if the BWP were 5 MHz. With round-robin scheduling, the UE is “active” four times longer with the 5 MHz BWP.

Unless a clear scenario showing the benefits of the additional separate initial DL/UL BWP, we do not see a need to support an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs.

Observation 7. No clear scenario is shown for an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs.

### Multiple PDSCH

In RAN1#112, there were some discussions regarding receiving multiple PDSCH in the same slot. There are two scenarios: a UE receiving two broadcasts and a UE receiving a unicast PDSCH and a broadcast. For the former, there was a conclusion stating

Conclusion: For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR.

With this conclusion, no further discussion is needed for the former. For the latter, a proposal was discussed online but no decision was reached [4].

High Priority Proposal 2.5-2c For UE BB complexity reduction, it is FFS whether some specification changes (e.g., regarding prioritization for processing and decoding) are needed for simultaneous reception of unicast and broadcast PDSCH transmissions.

A related 38.214 clause 5.1 [6] regarding the number of PDSCH a UE can decode when the UE is in the active mode is shown below. That clause can be considered for unicast.

On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI.

For a UE supporting baseband complexity reduction, the base station can retransmit the unicast PDSCH if the UE is unable to process the unicast PDSCH when it simultaneously receives unicast and broadcast PDSCH transmissions. A behavior similar to the existing standard text can be adopted.

Proposal 4: For UE BB complexity reduction, the UE may skip decoding the unicast PDSCH for simultaneous reception of unicast and broadcast PDSCH transmissions

For standalone PR1, since a UE is not limited by the processing per slot agreement, the existing behavior / standards for simultaneous reception of unicast and broadcast PDSCH transmissions apply.

### Msg4 and MsgB PDSCH

In RAN1#112, a working assumption for Msg4 was

Working assumption: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

The discussion of MsgB PDSCH appeared to be contingent on how Msg4 PDSCH is resolved.

Medium Priority Proposal 2.9-1b: The MsgB PDSCH bandwidth and number of PRBs are limited in the same way as for Msg4 PDSCH.

One company was concerned about contention between a non-RedCap UE and a Rel-18 RedCap UE during initial access. The gNB may be sending Msg4 to a non-RedCap UE while eRedCap UE believes it is the recipient of Msg4. In this case, the eRedCap UE could infer that its Msg3 was not received correctly because the gNB scheduled an improperly sized Msg4. Note this issue may not arise if the eRedCap UE and non-RedCap UE use different CORESETs/search spaces during initial access.

One observation regarding a UE supporting standalone PR1. While it can support a larger number of PRBs, the RAN agreement of

“the initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1”

implies that UE supporting standalone PR1 follows any agreement for Msg4/MsgB.

If it is common understanding that “not required to process” implies no HARQ-ACK feedback is transmitted, the working assumption would capture a UE behavior during RACH.

Proposal 5. confirm the working assumption “The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.”

## PR1

For peak data rate reduction, there are two areas for discussion: the values of vLayers·Qm·f for add-on and standalone. There is a down-selection for the value of vLayers·Qm·f for add-on.

### Add-on

In RAN1#112, the agreements about the number of RBs for 30 kHz SCS and the minimum data rate resulted in the following down-selection agreement for PR1 as an add-on.

Agreement For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X.

In our contribution [3], we showed the downlink data rate in Mbps as a function of X can be expressed as

|  | DL data rate≤0.133757∙X∙NPRBBW,μ, μ=00.267514∙X∙NPRBBW,μ, μ=1 | (1) |
| --- | --- | --- |

An analogous expression for the uplink data rate in Mbps is

|  | UL data rate≤0.143089∙X∙NPRBBW,μ, μ=00.286178∙X∙NPRBBW,μ, μ=1 | (2) |
| --- | --- | --- |

The scalar for the UL data rate is obtained from the scalar for the DL data rate after multiplying with a factor 1.07= (1.0-0.08) / (1.0-0.14), where the terms 0.08 and 0.14 are the overhead factors for the UL and DL, respectively.

The following table lists the values of X as a function of the number of RBs per SCS as well as the Dl/UL data rates. The last column shows the data rate for the current minimum value of X=4.

Table 1. Values of X as a function of numerology and bandwidth.

|  |  | Minimum DL data rate, Mbps |  |  | Minimum UL data rate, Mbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| µ | NPRBBW,μ | X=3 | X=3.2 | X=4 | X=3 | X=3.2 | X=4 |
| 0 | 25 | 10 | 10.7 | 13.4 | 10.7 | 11.4 | 14.3 |
| 1 | 12 | 9.6 | 10.3 | 12.8 | 10.3 | 11.0 | 13.7 |

Examining Table 1, both X=3 and X=3.2 seem reasonable for the 10 Mbps target. Before deciding on the value of X, we also need to consider standalone PR1, which is presented in the next section.

### Standalone

Standalone PR1 is now supported based on the agreement in RAN#99 [3]. Following the agreements for add-on PR1, the agreement should be updated as

- If UE peak data rate reduction is supported as a standalone feature, - The constraint vLayers×Qm×f ≥ 4 is relaxed to vLayers×Qm×f ≥ Y. - FFS: the value of Y - Note: Whether this option is supported will be decided in RAN plenary.

The agreement states

“The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate.”

Since the PxSCH bandwidth for standalone PR1 is four times the PxSCH bandwidth for baseband complexity reduction, the value of Y should be one-fourth the value of X.

Observation 8. Based on the PxSCH bandwidths, the value of Y should be one fourth the value of X

Because the number of RBs is different for 20 MHz and 5 MHz and because the data rate is based on the number of RBs, it is important to verify that the value of Y meets the data rate requirements if it is one-fourth the value of X. The follow table shows that the ratio of RBs is less than one-fourth, meaning that if Y is one-fourth of X, the data rate for standalone PR1 will slightly higher.

Table 2. Number of RBs in PxSCH

| SCS, kHz | RBs for Add-on PR1 | RBs for Standalone PR1 | Ratio of RBs |
| --- | --- | --- | --- |
| 15 | 25 | 106 | 0.2358 (=1/4.24) |
| 30 | 12 | 51 | 0.2353 (=1/4.25) |

The DL and UL data rates, obtained by replacing “X” by “Y” in equations (1) and (2) with Y=X/4, is shown in Table 3. The two possible values of Y are 0.75 and 0.8, corresponding to X=3 and 3.2.

Table 3. Values of Y as a function of numerology and bandwidth

|  |  | Minimum DL data rate, Mbps |  |  | Minimum UL data rate, Mbps |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| µ | NPRBBW,μ | Y=0.75 | Y=0.8 | Y=4 | Y=0.75 | Y=0.8 | Y=4 |
| 0 | 106 | 10.6 | 11.3 | 60.7 | 11.4 | 12.1 | 56.7 |
| 1 | 51 | 10.2 | 10.9 | 58.4 | 10.9 | 11.7 | 54.6 |

From Table 3, either Y=0.75 or Y=0.8 provide at least 10 Mbps for both numerologies while being one-fourth the value of X. The results also indicate that Y=0.75 is much closer to 10 Mbps than Y=0.8

### Peak data rate summary

Since the ratio of the number of RBs for standalone PR1 and add-on PR1 is not exactly four, the data rates with peak data rate reduction will not be equal. However, when considering the two options for X and the relationship between X and Y, we first propose

Proposal 6. For peak data rate reduction where X and Y are the relaxed constraints for add-on to UE BB bandwidth reduction and standalone, respectively, down-select between

- X=3.0 and Y=0.75 and

- X=3.2 and Y=0.8

Note for the either sets of values, no new scale factor f is needed for vLayers·Qm·f. Based on the resulting data rates for X and Y in Table 1 and Table 3, we propose

Proposal 7. For peak data rate reduction where X and Y are the relaxed constraints for add-on to UE BB bandwidth reduction and standalone, respectively, select X=3.0 and Y=0.75.

# Conclusion

Our observations and proposals on various topics are summarized below.

Observation 1: Access control and early indication do not distinguish standalone PR1 and baseband reduced complexity UE variants.

Msg3 Timeline

Proposal 1. Select X for the scheduling of RAR PDSCH as either 0.5/0.25 or 1/0.5 ms for 15/30 kHz SCS, respectively

Observation 2. For the scheduling of RAR PDSCH, the network applies the same value of X for a Rel-18 eRedCap UE capable of 20MHz + PR1 and a Rel-18 eRedCap UE capable of BW3/PR3 + PR1

Early indication

Proposal 2. A Rel-18 eRedCap UE capable of 20MHz + PR1 supports the same additional separate early indication(s) as a Rel-18 eRedCap UE capable of BW3/PR3 + PR1

Observation 3. There are many situations where the size of PDSCH carrying Msg2 and where the size of Msg3 exceed the maximum RB limit for baseband complexity reduction.

Observation 4. There is network impact to accommodate the additional delay for processing Rel-18 RedCap Msg2 without Msg1 EI.

Observation 5. A Rel-18 Msg1 EI is required (at least) in the case where Rel-17 does not use Msg1 EI.

Observation 6. There is no impact to the UE for supporting a separate Msg1 EI.

Proposal 3. Support an additional separate Msg1 early indication for Rel-18.

Separate initial BWP

Observation 7. No clear scenario is shown for an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs.

Multiple PDSCH

Proposal 4. For UE BB complexity reduction, the UE may skip decoding the unicast PDSCH for simultaneous reception of unicast and broadcast PDSCH transmissions

Msg4 and MsgB PDSCH

Proposal 5. confirm the working assumption “The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.”

Peak data rate reduction

Observation 8. Based on the PxSCH bandwidths, the value of Y should be one fourth the value of X

Proposal 6. For peak data rate reduction where X and Y are the relaxed constraints for add-on to UE BB bandwidth reduction and standalone, respectively, down-select between

•	X=3.0 and Y=0.75 and

•	X=3.2 and Y=0.8

Proposal 7. For peak data rate reduction where X and Y are the relaxed constraints for add-on to UE BB bandwidth reduction and standalone, respectively, select X=3.0 and Y=0.75.

# References

RP-222675, “New WID on enhanced support of reduced capability NR devices”, Ericsson, RAN#97, Sep. 12-16, 2022

RP-223544, “Revised WID on Enhanced support of reduced capability NR devices”, Ericsson, RAN#98, Dec. 12-16, 2022

RP-230778, “Proposal for PR1 in eRedCap”, CMCC, RAN#99, Mar. 20-23, 2023

R1-2301885, “RAN1 agreements for Rel-18 NR RedCap”, Ericsson, RAN1#112, Feb. 27 – Mar. 3, 2023

R1-2301889, “FL summary #4 on Rel-18 RedCap UE complexity reduction”, RAN1#112, Feb. 27 – Mar. 3, 2023

RP-230176, “Views on work item scope for Rel-18 RedCap UEs”, FUTUREWEI, RAN#99, Mar. 20-23, 2023

38.214

38.300

R1-2300058, “Discussion on R18 RedCap complexity techniques”, FUTUREWEI, RAN1#112, Feb. 27 – Mar. 3, 2023

# Appendix

## Appendix A: Past agreements

#### Initial BWP

Agreement: For a cell supporting both Rel-17 and Rel-18 RedCap UEs, - The Rel-18 RedCap UEs can share the same separate initial DL/UL BWP as the Rel-17 RedCap UEs. - FFS: whether to support an additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs Conclusion: There is no consensus to continue discussion on “whether additional separate initial DL/UL BWP specific to Rel-18 RedCap UEs is allowed to be configured by the SIB in the cell”.

#### Number of PRBs

Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for PUSCH, down-select between the following options for the maximum number of PRBs that the UE can transmit: - Option 1: 28 PRBs for 15 kHz SCS and 14 PRBs for 30 kHz SCS - Option 2: 27 PRBs for 15 kHz SCS and 13 PRBs for 30 kHz SCS - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (at least for unicast), down-select between the following options for the maximum number of PRBs that the UE can [receive/process]: - Option 1: 28 PRBs for 15 kHz SCS and 14 PRBs for 30 kHz SCS - Option 2: 27 PRBs for 15 kHz SCS and 13 PRBs for 30 kHz SCS - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS Same option will be selected for both PDSCH (at least for unicast) and PUSCH Agreement: (replaced by later agreement) Replace the agreement on the maximum number of PRBs supported by UE with the following: For UE BB bandwidth reduction, for PUSCH, down-select between the following options for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 1: 28 PRBs for 15 kHz SCS and 14 PRBs for 30 kHz SCS - Option 2: 27 PRBs for 15 kHz SCS and 13 PRBs for 30 kHz SCS - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (at least for unicast), down-select between the following options for the maximum number of PRBs that the UE can process per slot: - Option 1: 28 PRBs for 15 kHz SCS and 14 PRBs for 30 kHz SCS - Option 2: 27 PRBs for 15 kHz SCS and 13 PRBs for 30 kHz SCS - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS Same option will be selected for both PDSCH (at least for unicast) and PUSCH. Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for PUSCH, down-select between the following options for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), down-select between the following options for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Option 4: 25 PRBs for 15 kHz SCS and 11 PRBs for 30 kHz SCS Same option will be selected for both PDSCH and PUSCH. Agreement: For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS Note: No intention to change the RAN4 RF specifications about maximum transmission PRB number

#### SIB1 bandwidth

Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for SIB1 (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of SIB1 to be within 5 MHz - Option 2: Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous Agreement: Replace the agreement on SIB1(PDSCH) for UE BB bandwidth reduction with the following: For UE BB bandwidth reduction, for SIB1 (PDSCH), - Allow the scheduling of SIB1 to be larger than 5 MHz (as in legacy operation) - FFS: UE post-FFT buffering “assumption” (replaced by later agreement) Conclusion: For UE BB complexity reduction, broadcast of separate SIB1/OSI (PDSCH) to Rel-18 RedCap UEs is not supported. Conclusion: For UE BB complexity reduction, there is no need to relax the requirements on simultaneous reception of two broadcast PDSCH transmissions for SIB1/OSI/paging/RAR.

#### OSI bandwidth

Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for broadcast OSI (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of OSI PDSCH to be within 5 MHz - Option 2: Allow the scheduling of OSI PDSCH to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous Agreement: Replace the agreement on broadcast OSI (PDSCH) for UE BB bandwidth reduction with the following: For UE BB bandwidth reduction, for broadcast OSI (PDSCH), - Allow the scheduling of broadcast OSI (PDSCH) to be larger than 5 MHz (as in legacy operation)

#### Paging bandwidth

Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of paging channel to be within 5 MHz - Option 2: Allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous Agreement: (replaced by later agreement) From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). Agreement: Update the agreement for PDSCH paging with the clarification as follows: - From RAN1 perspective, for UE BB complexity reduction, for paging channel (PDSCH) to Rel-18 RedCap UEs, allow the scheduling of paging channel to be larger than 5 MHz (as in legacy operation). The scheduling of paging PDSCH is allowed to be larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

#### RAR bandwidth and timeline

Agreement: (replaced by later agreement) For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, down-select between the following options: - Option 1: Restrict the scheduling of RAR PDSCH to be within 5 MHz - Option 2: Allow the scheduling of RAR PDSCH to be larger than 5 MHz (as in legacy operation) - FFS: whether 5MHz is assumed to be physically contiguous Agreement: For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH Agreement: For the earlier RAN1 agreement achieved in RAN1#111 as following, For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed - Note: it will not be used as example for unicast PDSCH. For the “FFS: value(s) of X” - X = [0.5/0.25 or 1/0.5 or 2/1] ms for 15/30kHz SCS - Note: Single Value pair for X is to selected for SCSs

#### PUSCH bandwidth / Msg3 / MsgA

Agreement: For UE BB bandwidth reduction, a UE is not expected to receive an UL grant in a DCI with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement: - For UE BB bandwidth reduction, a UE is not expected to be configured with a CG grant with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement: (replaced by later agreement) - For UE BB bandwidth reduction, it is FFS whether a UE can be expected to receive an UL grant in a RAR with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement: - For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement: For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable.

#### UE post-FFT buffer size

Conclusion: For UE BB complexity reduction, for broadcast and unicast PDSCH, RAN1 does not assume that the UE post-FFT buffer size per slot is smaller than 20 MHz.

#### Unicast PDSCH bandwidth / Msg4

Agreement: - For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e Working assumption: - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

#### Peak data rate reduction

Agreement: - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, (replaced by later agreement) - The constraint vLayers×Qm×f ≥ 4 is relaxed to vLayers×Qm×f ≥ X. - FFS: the value of X - If UE peak data rate reduction is supported as a standalone feature, - The constraint vLayers×Qm×f ≥ 4 is relaxed to vLayers×Qm×f ≥ Y. - FFS: the value of Y - Note: Whether this option is supported will be decided in RAN plenary. Agreement: (replaced by later agreement) - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL Agreement: Revise the earlier agreement by removing the square brackets like this: - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL Agreement: For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X.

## Appendix B [3]

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

A typical TDD configuration is DDDSU and consists of three downlink slots (D), one flexible slot (S) with mostly DL symbols, and one UL slot. If the scheduling for Msg3 is delayed until the next uplink slot, the next uplink slot is 5 slots later.