3GPP TSG RAN WG1 #112bis-e											R1-2302324

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.4.1.1

Source:	FUTUREWEI

Title:	Discussion on channel access mechanism for sidelink on unlicensed spectrum

Document for:	Discussion and decision

# Introduction

At RAN#98, the WI on sidelink evolution was updated for Rel-18 (RP-222806) [1]. In this WI, the objective on sidelink in unlicensed spectrum is specified as:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- Focus on FR1 unlicensed bands (n46 and n96/n102).

- Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

In this contribution, we present our views on the channel access mechanisms for sidelink transmission in unlicensed spectrum.

# RAN1 # 112 Agreements

Agreement

The CAPC level that should be used for S-SSB transmissions:

- Option 1: CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for S-SSB transmission.

Agreement

The CAPC level that should be used for PSFCH transmission, CAPC value (p) should be set to 1 when UE performs Type 1 channel access procedure for PSFCH transmission.

Agreement

The end timing for the definition of reference duration in the contention window adjustment procedure for SL-U is defined as follows:

- Option 1a

  - the end of the first slot where at least one PSSCH with ACK/NACK HARQ-ACK enabled is transmitted

  - Note, SL reference duration is not used if PSSCH with ACK/NACK HARQ-ACK enabled cannot be found in the latest COT

  - FFS: Whether to support another ending timing is FFS, e.g. for MCSt if needed

  - Whether/how to adjust CWS for groupcast option 1 NACK-only case and whether/how to define reference duration for groupcast option 1 NACK-only case can still be discussed

Agreement

A CPE can be transmitted from a CPE starting position before SL transmission for the following two options:

- Option 1: within the symbol just before the next AGC symbol

- Option 2:

  - within the symbol just before the next AGC symbol for 15 kHz SCS

  - within at most 2 symbols just before the next AGC symbol for 30 or 60 kHz SCS

- FFS applicable scenario(s), condition(s) and channel type(s) to apply Option 1 or Option 2

Agreement

- A responding UE over a shared COT can be:

  - a receiving UE, which is the target of a PSCCH/PSSCH transmission of a COT initiator

    - In the case of unicast from the COT initiator, within the same COT when the source and destination IDs contained in the COT initiator’s SCI match to the corresponding destination and source IDs relating to the same unicast at the receiving UE

    - In the case of groupcast and broadcast, when the destination ID contained in the COT initiator’s SCI match to a destination ID known at the receiving UE

  - a UE identified by ID(s), if additional IDs are supported in the COT sharing information (in addition to the source and destination IDs of the PSCCH/PSSCH transmission), when additional IDs are included in the COT sharing information from the COT initiator

    - FFS Limitations on what additional IDs may be included and how they may be indicated

Agreement

A responding UE’s SL transmission(s) within RB set(s) corresponding to a shared COT can be transmitted when the CAPC value(s) of the SL transmission(s) have an equal or smaller CAPC value than the CAPC value indicated in the COT sharing information.

Agreement

A responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to a shared COT is intended for the COT initiating UE when,

- In the case of unicast from the responding UE, when the source and destination IDs contained in the responding UE’s PSCCH/PSSCH match to the destination and source IDs from a COT initiator’s unicast transmission that included COT sharing information, or match to the additional ID(s) included in the COT sharing information (if supported)

- In the case of groupcast or broadcast from the responding UE, when the destination ID contained in the responding UE’s PSCCH/PSSCH matches to the destination ID from a COT initiator’s groupcast or broadcast transmission that included COT sharing information, or matches to the additional ID(s) included in the COT sharing information (if supported)

- FFS: all other details and additional restrictions

# Open Issues

•	Specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only.

o	Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

	Remaining details of Type 1 channel access procedure for SL

	Remaining details of Type 2 channel access procedures for SL

	Remaining details of contention window adjustment

	Remaining details of cyclic prefix extension

	Remaining details of shared channel occupancy

	Remaining details of multi-channel access procedures

	Remaining details of multi consecutive slots transmission (MCSt)

	Remaining details of SCI for SL-U channel access and resource allocation

	Whether any update necessary on SL mode 1 and mode 2 RA

# Discussions

## S-SSB

S-SSB transmission may use Type2A outside of COT sharing, under the duty cycle constraints. If the duty cycle constraints cannot be satisfied Type 1 channel access can be used. When Type 1 channel access is used CAPC value is set to 1. Type 1 channel access may be used to initiate a COT.

A UE is a responding UE for COT sharing in either of these two cases:

- When is a receiving UE (of PSCCH/PSSCH)\

- When is identified (via its ID) in the COT sharing information

A prior agreement from RAN1#111 allows a responding UE to transmit S-SSB in a shared COT.

Agreement

For UE-to-UE COT sharing,

When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. [text omitted]

To enable the S-SSB transmissions in a shared COT, a COT initiator should either keep track of the possible S-SSB transmitters in its neighborhood, and either know their schedule or use them as destination. This solution increases the UE complexity.  Limiting transmission of S-SSB only to responding UEs may affect the synchronization and the discovery procedures as other potential S-SSB transmitters are not allowed to transmit during a shared COT.

Proposal 1: Define conditions for S-SSB transmissions in shared COT.

One simple solution for S-SSB transmission in shared COT may be to allow always S-SSB transmissions in a shared COT when the S-SSB transmitter can decode the COT sharing information. For this solution the channel access for S-SSB transmission should respect the COT sharing channel access rules. In this case the S-SSB transmission should have CAPC set to 1, which means that the S-SSB transmission can be done in any COT regardless of the CAPC value used by the COT initiator.

Proposal 2: S-SSB transmission should be always allowed in COT sharing provided that S-SSB transmitter can decode the COT sharing information.

Other alternatives may be for the COT initiator to explicitly allow or forbid S-SSB transmissions in its COT, for instance having a bit indication in the COT sharing information, or to configure per resource pool whether the S-SSB transmission is allowed in the shared COT or not.

RAN1 group did decide that may be at least one CPE starting position prior to S-SSB transmission, but it did not decide how they are provided (configured, pre-defined, or indicated, etc).

Proposal 3: Define the number of CPE starting positions for S-SSB transmission and how they are provided.

## PSFCH

PSFCH transmission uses CAPC set to 1 when Type 1 channel access is used. Only a single CPE starting position is supported for PSFCH. RAN1 has not decided yet how the CPE starting position is provided (configured, pre-defined, or indicated, etc).

The CPE location may be for instance such that the transmission gap prior to the PSFCH AGC symbol is 16us or less to avoid the need for clear channel assessment or 25us to have a short deterministic clear channel assessment.

The configuration or the indication allows a more flexible approach than having a pre-defined CPE starting position. However, this flexibility may not be necessary given that PSFCH is a high priority control message. Therefore, the CPE may be pre-defined such that at most 16us gap between prior transmission and AGC PSFCH symbol is guaranteed.

Proposal 4: A single CPE starting position for PSFCH is pre-defined to 16us from the starting guard symbol prior to PSFCH AGC symbol.

An open question is whether Type 2A channel access can be supported for the PSFCH transmission, and whether the constraints for duty cycle should apply.

It was agreed in RAN1#111 that when performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE.

We note that the agreement allows (but not explicitly states) that a responding UE transmits PSFCH to an SL UE (which may not be a responding UE) other than the COT initiator if this responding UEs transmits at the same time PSFCH to the COT initiator.

Proposal 5: In shared COT a responding UE’s PSFCH transmission to other than COT initiator is supported.

Regarding channel access for PSFCH transmission it was proposed in the last meeting by several companies that Type 2A should be supported, however RAN1 could not reach an agreement.

The companies against the proposal argued that combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure may impact S-SSB transmission or may lead to the need of additional rules for the transmission decision when the exempt rules are not met. Another argument against the proposal was that this approach is not supported in NR-U where PUCCH and PDCCH (when used for ACK/NACK of CG-PUSCH) do not use Type 2A.

The companies supporting Type 2A for PSFCH argued that when the exempt rules are not met Type 2A should not be applied to PSFCH and allow gNB possibility to configure whether Type 2A may be used for PSFCH. Another argument in support of it was that those UEs that do not transmit S-SSB should be able to use Type 2A for PSFCH.

We first note that having some UEs that use Type 2A and some UE that do not use Type 2A for PSFCH may create a fairness issue. For the argument that when the duty cycle is met the PSFCH should not use Type 2A channel access we observe that a UE needs to predict when the duty cycle limit will be reached and stop using the Type 2A for PSFCH prior to this limit being reach. If one avoids impacting the S-SSB transmission this approach leads to a conservative decision to stop Type 2A usage for PSFCH. In any case, situations when PSFCH cannot use Type 2A may occur.

Therefore, the practical solution for PSFCH transmission is to use Type 1 for channel access with CAPC value of 1.

Proposal 6: For PSFCH channel transmission without COT sharing use Type 1 channel access.

Sidelink supports subcarrier spacings of 15, 30, 60 and 120 kHz. Type 2A channel access consists of sensing the channel idle for at least 25us. For higher SCS (60 and 120 kHz), 25us is longer than a single OFDM symbol duration, which is the duration of the guard symbol. Therefore, even using Type 2A channel access, the transmission of PSFCH can be blocked by the transmission of PSCCH/PSSCH (Figure 1).

Figure 1

In Type 1 channel access, the channel must be sensed to be idle during all the sensing slot durations of a defer duration Td immediately before transmission. For CAPC=1, the defer duration is 25us. Therefore, the blockage may occur for high SCS as in Type 2A channel access.

Observation 1: For higher SCS (60kHz, 120kHz) a PSFCH transmission may be blocked by the PSSCH transmissions in the same slot.

Transmission of PSFCH is relatively short as it occupies only two symbols, which means that it can take between 18us at SCS 120 kHz to 143 us at SCS 15kHz. For short control signal transmission, the regulation allows Type 2C channel access if the transmission takes less than 584us. In order to avoid the blockage of PSFCH due to PSSCH transmission Type 2C channel access can be used. In this way, the blockage issue is solved for the PSSCH transmissions, which is expected during shared COT, while for other types of transmissions present in the channel, such as WiFi, which is expected for transmissions without COT sharing, the blockage is avoided via Type 1 channel access.

Proposal 7: Support Type 2C channel access for PSFCH transmissions with COT sharing.

For sidelink unlicensed transmissions it was agreed that two possible starting point symbols are supported. If the LBT fails prior to the first starting symbol, a second LBT can be executed before the second starting symbol, and if successful the sidelink transmission may start.  This transmission may overlap with the PSFCH transmission occasion, which is not desired.

Proposal 8:  During a COT a sidelink UE should avoid transmissions (PSCCH/PSSCH) that may overlap with PSFCH transmission occasions.

There are few possible ways to avoid collision between PSCCH/PSSCH and PSFCH, such as dropping the PSCCH/PSSCH transmission or shortening it. Selecting one or the other it would depend on the location of second starting symbol and the TBS.

## Resource Allocation

One objective of the WID is to

- “Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation.”

In Rel-16/Rel-17 congestion control is supported [TS 38.214]. In order to support the congestion, control a SL UE must evaluate the channel busy ratio (CBR), which is defined in TS38.215:

SL Channel Busy Ratio (SL CBR) measured in slot n is defined as the portion of sub-channels in the resource pool whose SL RSSI measured by the UE exceed a (pre-)configured threshold sensed over a CBR measurement window [n-a, n-1], wherein a is equal to 100 or 100·2µ slots, according to higher layer parameter sl-TimeWindowSizeCBR.

We note that the definition of CBR is based on the SL RSSI measured by the UE, which is defined as

Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol.

This definition considers only the slots configured for PSCCH and PSSCH and does not take into consideration other RAT transmission in the CBR measurement window. If a slot is configured for PSCCH and PSSCH transmission but the transmission does not take place because another RAT transmission is in place, the UE may count the symbols as SL occupied symbols, which may impact adversely the future sidelink transmissions.  For instance, if most of the slots in sensing window are occupied by Wi-Fi transmissions and there are no SL-U transmissions, the CBR value is zero because SL RSSI measured is zero.

One way to adjust the CBR to consider other transmissions is to measure the RSSI rather than SL RSSI in the sensing window and differentiate between the slots occupied by SL-U transmissions and slots occupied by other transmissions.

Proposal 9: Update the CBR definition for SL-U to consider the non-SL transmissions.

In the last RAN1 meeting (#112) few enhancement proposals for Mode 2 resource selection were made (R1-231797). One of them suggested that consistent LBT failures should be considered when the resources are selected.  However, there is a drawback to this solution. LBT is done only prior to a transmission, which implies that SL-U may not have a record of the occupied channels in the past, or the record may not be any longer valid for the next resource selection. A more preferable solution to identify the busiest resources is to monitor the RSSI during the sensing window and identify which resources are occupied by SL-U transmissions and which resources are occupied by other transmissions.  Reporting these resources to the upper layer would allow to avoid selection of those resources with poor history.

Proposal 10: A SL-U should identify in the selected set of resources those resources occupied by another RAT.

Because RAN1 decided that two starting symbols in a slot are supported for the SL-U transmissions, the RSSI monitoring may be reduced to a subset of symbols in a slot. Moreover, the set of sub-channels affected by other RAT transmissions may be used during IUC to provide the non-preferred set of resources for transmission/reception.

Proposal 11: Support IUC Scheme 1, investigate how the non-preferred set of resources may include resources affected by other RAT interference.

## COT sharing

Channel Occupancy Time (COT) is defined in ETSI EN 301 893, and in TS 37.213, and consists of one or more transmissions separated by gaps less or equal of 25us, where the first transmission is executed by the initiating device after Type 1 channel access. COT should not exceed the maximum COT duration (MCOT) corresponding to the channel access priority (CAPC) used by the initiating UE. Therefore, the initiating device must provide COT sharing information to the responding devices that participate in the COT sharing such as the COT sharing indication, remaining COT duration, the energy detection threshold (EDT) that needs to be checked before sharing COT, the ID(s) of the potential responding devices. Some of this information may be (pre-) configured such the EDT.

Proposal 12: When initiating a COT SL-U must provide the COT sharing information.

As corollaries of this proposal several observations can be noted:

Observation 2: Only the UEs that provide COT sharing information may initiate a COT.

Observation 3: A S-SSB transmission cannot initiate a COT because the S-SSB slot format soes not support COT sharing information.

Observation 4: A PSFCH transmission alone cannot initiate a COT. To be part of the COT transmission initiation the PSFCH transmission should follow a PSCCH/PSSCH transmission in the same slot which carries COT sharing information.

## Multi-channel access

Regarding multi-channel transmissions the following agreement was reached in RAN1#110b:

Agreement

For dynamic channel access mode with multi-channel case in SL-U, NR-U UL channel access procedure is considered as baseline for transmission on multiple channels.

- FFS: whether transmission of PSFCH and/or S-SSB on a subset of RB sets is supported (using the NR-U DL channel access procedure as baseline)

- FFS any necessary enhancement and modification for the SL-U operation.

We note that NR-U UL multi-channel access procedure requires as necessary condition for multi-channel transmission that UE can access all the channels, of the carrier bandwidth, on which the UE is scheduled or configured with UL resources.

We also note that TS 37.213 does not define a multi-channel COT, but rather defines the conditions for a multi-channel access, which does not necessarily initiate a multi-channel COT.

Following the NR-U UL multi-channel access as the baseline [TS 37.213, Clause 4.2.1.0.4] implies that a SL UE cannot transmit in any channel (RB set) (ci∈C ) on which the UE is scheduled or configured with SL resources if it fails to access any channel on which the UE is scheduled or configured with SL resources. For multi-channel access, the transmission should be preceded by successful LBT Type 1 on a channel randomly selected and successful LBT Type 2 on each of the other BPW channels. In other words, to transmit on channel cj (where cj∈C is uniformly randomly selected), the SL UE uses Type 1 channel access and to transmit on channel ci≠cj, ci∈C, SL shall sense the channel ci for at least a sensing interval Tmc=25μs.  For the multi-channel transmission to proceed the sensing in each channel (RB set) should indicate that the channel is available.

There are a few issues that need to be clarified in relation to the multi-channel access.

The first issue is how long the multi-channel transmission can take place?

One possibility is to limit any multi-channel transmission to a single transmission (i.e., no transmission gaps) allowed and not multi-channel COT is initiated. In this case, the SL UE at most initiates a COT only in the channel cj where a Type 1 channel accessed was performed. In this case, if the multi-channel transmission stops before the COT in channel cj ends, the COT in channel cj continues and may be shared, i.e., transmissions from other SL UEs takes place in channel cj in a shared COT manner. The PSFCH feedback on channel ci≠cj is provided following the channel access rules without COT sharing.

Another possibility is to allow a burst of multiple multi-channel transmissions (not necessarily from the same SL UE) that can be separated by short gaps (<16us) and not exceeding a maximum total time limit. For this end, a natural choice would be to limit the total duration of multi-channel transmissions to the MCOT defined by the successful Type 1 channel (cj∈C) access.   This approach may allow PSFCH feedback transmission in the same burst with data transmission, for instance using a CPE to maintain channel control.

Proposal 13: The maximum duration of multi-channel transmission on any channel ci≠cj, ci∈C, shall not exceed Tmcot,p, where the value of Tmcot,p is determined using the Type 1 channel access parameters used to access channel cj.

In this case, the remaining time duration of the transmission burst must be provided by the SL UE to the surrounding SL UEs.

Another issue to clarify is whether a multi-channel transmission may a multi-channel COT, and if so, how the multi-channel COT is shared?

Like in a single channel COT, one may define a multi-channel COT, which consists of consecutive transmission in the same multi-channels, where the transmissions are separated by gaps no larger than 25us. To initiate a multi-channel COT, Type 1 channel access should be performed in each of the channels prior to the initial transmission. Then a COT sharing may be considered in each of the channels (RB sets)

Proposal 14: Define conditions for SL-U multi-channel COT initiating and sharing.

# Conclusion

Proposal 1: Define conditions for S-SSB transmissions in shared COT.

Proposal 2: S-SSB transmission should be always allowed in COT sharing provided that S-SSB transmitter can decode the COT sharing information.

Proposal 3: Define the number of CPE starting positions for S-SSB transmission and how they are provided.

Proposal 4: A single CPE starting position for PSFCH is pre-defined to 16us from the starting guard symbol prior to PSFCH AGC symbol.

Proposal 5: In shared COT a responding UE’s PSFCH transmission to other than COT initiator is supported.

Proposal 6: For PSFCH channel transmission without COT sharing use Type 1 channel access.

Observation 1: For higher SCS (60kHz, 120kHz) a PSFCH transmission may be blocked by the PSSCH transmissions in the same slot.

Proposal 7: Support Type 2C channel access for PSFCH transmissions with COT sharing.

Proposal 8:  During a COT a sidelink UE should avoid transmissions (PSCCH/PSSCH) that may overlap with PSFCH transmission occasions.

Proposal 9: Update the CBR definition for SL-U to consider the non-SL transmissions.

Proposal 10: A SL-U should identify in the selected set of resources those resources occupied by another RAT.

Proposal 11: Support IUC Scheme 1, investigate how the non-preferred set of resources may include resources affected by other RAT interference.

Proposal 12: When initiating a COT SL-U must provide the COT sharing information.

Observation 2: Only the UEs that provide COT sharing information may initiate a COT.

Observation 3: A S-SSB transmission cannot initiate a COT because the S-SSB slot format soes not support COT sharing information.

Observation 4: A PSFCH transmission alone cannot initiate a COT. To be part of the COT transmission initiation the PSFCH transmission should follow a PSCCH/PSSCH transmission in the same slot which carries COT sharing information.

Proposal 13: The maximum duration of multi-channel transmission on any channel ci≠cj, ci∈C, shall not exceed Tmcot,p, where the value of Tmcot,p is determined using the Type 1 channel access parameters used to access channel cj.

Proposal 14: Define conditions for SL-U multi-channel COT initiating and sharing.

# References

RP-213678, “NR sidelink evolution”, RAN#94-e, Dec. 2021.

ETSI EN 301 893 5 GHz RLAN; Harmonized Standard covering the essential requirements

TS 37.213, Physical layer procedures for shared spectrum channel access

RAN1#109-e, Chair’s Notes

TR 21.916 	Release 16 Description; Summary of Rel-16 Work Items

TS 38.331 NR; Radio Resource Control (RRC); Protocol specification

TS 38.212 NR; Multiplexing and channel coding

TS 38.214 NR; Physical layer procedures for data

RAN1#110, Chair Notes

RAN1#111, Chair Notes

RAN1#112, Chair Notes