3GPP TSG RAN WG1 #112bis-e	R1-2302326

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.1.1

Source:	Futurewei

Title:	Discussion on SL positioning reference signal

Document for:	Discussion/Decision

# Introduction

In RAN#98-e the new WID on Expanded and Improved NR Positioning in RP-223549 was approved [1]. The first objective of the WID is the support of positioning for sidelink:

- “Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]:

  - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1].

    - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum.

    - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified.

  - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

  - Specify support of resource allocation for SL PRS:

    - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1].

      - For resource allocation mechanism for SL PRS in Scheme 2:

        - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1].

        - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1].

    - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1].

      - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

  - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1].

  - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

  - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]:

    - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)).

    - Specify the protocol and procedures for SL positioning between UEs and LMF.

  - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2]

  - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].”

In this contribution, we focus on the solutions for SL positioning reference signal including procedures related to transmit power control.

# Agreements from RAN1#112

Agreement

SL PRS sequence is generated based on Gold sequence:

rn=121-2⋅c2n+j121-2⋅c2n+1,

where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211.

Agreement

- For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS.

- The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS

Agreement

For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option:

  - Option 1: nID,seqSL-PRS is a higher layer configured parameter

  - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission

  - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission

  - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID

  - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits

  - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined.

Agreement

Range of the parameter nID,seqSL-PRS is:  nID,seqSL-PRS∈0,1,…,4095

Agreement

A SL PFL is not defined. SL positioning RS are defined directly with respect to and contained within a single SL BWP and carrier.

Agreement

Support SCS values for SL PRS include:

- 15 kHz, 30 kHz, 60 kHz for FR1, and 60 kHz, 120 kHz for FR2

  - Which SCS values are required, and which ones are optional follow Rel-16 UE capabilities.

R1-2301876	FL summary #2 on SL positioning reference signal	Moderator (Intel Corporation)

Agreement

For RE-offset sequence for SL PRS, the RE-offset sequences specified for DL PRS are considered as a starting point.

- FFS: Exact RE-offset sequences

Agreement

- Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools.

  - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools.

- For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible .

  - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values.

- FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs

Agreement

For SL PRS in shared or dedicated resource pools,

- at least comb sizes (N) 2, 4 are supported.

- Comb size 6 is supported at least in dedicated resource pool

  - FFS: comb size 6 in shared resource pool

- Comb size 1 is supported at least in shared resource pool

  - FFS: comb size 1 in dedicated resource pool

- comb sizes (N) > 12 are not supported.

- FFS: support of comb sizes (N) of 8, 12.

Agreement

For SL PRS in shared and dedicated resource pools,

- SL PRS patterns with full staggering are supported.

  - FFS: whether (M,N)=(6,6) is supported

- SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs:

  - (M, 2) with M = {1}

  - (M, 4) with M = {2}

  - FFS: constraints on maximum effective comb size

  - FFS: support of partial staggering for other comb sizes

- FFS: Support of SL PRS patterns with M > N at least with full staggering.

Agreement

TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools.

- FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools.

- FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols.

- FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any

- FFS: which resource allocation schemes are applicable

- FFS: whether or not this is a separate UE capability

Agreement

The OLPC framework defined for PSSCH/PSCCH is considered as a starting point for OLPC for SL PRS.

# Discussions

## Sequence Generation

In RAN1 #112, it was decided that for the SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID, seqSL-PRS. Companies agreed that nID, seqSL-PRS∈0,1,…,4095. For the definition of nID, seqSL-PRS seven options where considered. For nID, seqSL-PRS, the first option, which also attracted the most support, was that this parameter is to be provided by the upper layers. However, the first option was not clear whether the parameter is specific to the UE or to the resource pool. Note that the sequence ID for DL PRS is configured per resource. Some of the other options defined the sequence ID based on the CRC of PSCCH, while some of the options defined the sequence ID based on PSSCH DMRS initialization, source ID, destination ID, or CRC of 2nd SCI. We first observe that the 2nd SCI support for was not yet decided per the following RAN1#112 agreement:

Agreement

For a dedicated resource pool for SL positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options:

  - Opt. 1: No other channel can be included beyond SL-PRS

  - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included

  - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included

    - FFS: Details

    - FFS: definition of PSSCH associated with SL-PRS transmission(s)

- Note: Companies are encouraged to provide their analysis and views on the above

To reduce implementation complexity, we prefer that the sequence ID definition is the same for dedicated and shared resource pools. Thus, either a decision of the supported channels in the dedicated resource pools is taken first, or the options based on the 2nd SCI are excluded. Note that if the options based on the 2nd SCI are excluded from the definition of the sequence ID, the decision to support 2nd SCI in the dedicated channels is not impacted.

Proposal 1: For the definition of nID, seqSL-PRS, down select Options 1-3 for further discussion. Clarify the intent of Option 1.

It was agreed in RAN1#112 that at least comb sizes (N) 2 and 4 are supported, comb size 6 is supported in dedicated resource pools and comb size 1 is supported in shared pool. Comb sizes 8 and 12 were left for further discussions. For SL PRS in shared and dedicated resource pools, SL-PRS patterns with full staggering and partial staggering are supported. For the number of symbols (M), a majority of companies supported values 2, 4 and 6, which allows full staggering (M=N).

Supporting larger N and M values, (comb values 8 and 12, and number of symbols 8, 10 and 12) would allow an increased precision of SL-PRS measurements in conditions of low SNR and higher interference.

In the recent RAN1 meetings, the possibility of having the first symbol SL-PRS duplicated in the last SL-PRS symbol was discussed. The overlap in frequency is beneficial to estimate phase variations between SL-PRS consecutive symbols transmissions, which may occur for in higher frequency and for larger number of SL-PRS symbols transmissions. Such overlap may be achieved for full staggering transmissions by adding one more symbol to the transmission, and for partial staggering transmissions by duplicating the first SL-PRS symbol into the last SL-PRS symbol.

Proposal 2: Consider increasing the range of M and N values and supporting the duplication of first SL-PRS symbol into the last SL-PRS symbol for large M values.

## Slot Structure

### Bandwidth

Positioning accuracy is dependent on the BW of the PRS signal. Therefore, is preferable that SL-PRS occupies the maximum BW available in that resource pool. If in the dedicated resource pools SL-PRS occupies less than maximum available BW, the resource frequency is not efficiently used, and performance measurement are affected. For applications with different requirements for accuracy, different dedicated resource pools may be configured in the same BWP.

Proposal 3: Support multiple dedicated positioning resource pools within one SL BWP. In each of the dedicated RP SL-PRS occupies the entire BW.

Because of the legacy traffic in shared resource pools the maximum BW of the resource pool may not be always available, and for this reason SL-PRS transmission may occupy less than maximum BW of the shared RP.

Observation 1: In the shared resource pools SL-PRS BW may be less than the BW of the RP.

### Slot structure

Part of the discussion regarding mapping of SL-PRS to physical resources is defining the values for comb size (N) and number of symbols (M). This discussion is directly related to the slot structure discussion. For instance, if the number of maximum number of SL-PRS symbols is 12, then transmission of SL-PRS without PSCCH associated must be supported. The support of 12 symbol SL-PRS slots allows more UEs to multiplex their transmission in the same slot as SRS transmission in Uu link.

Because SL-PRS TDM from multiple UEs is supported in the same slot, the above SL-PRS configurations may require that in a single slot more than 6 symbols are occupied with SL-PRS from different UEs.  For instance, if an AGC symbol is associated with SL-PRS symbols from the same UE, 12 symbols are required for 4 SL-PRS TDM transmissions, where each transmission occupies 1 AGC symbol and 2 SL-PRS symbols.  For 3 SL-PRS TDM transmissions where each of two transmission occupies 5 symbols (1 AGC + 4 SL-PRS repetitions) and one transmission occupies 3 symbols (1 AGC+ 2 SL-PRS repetitions) 13 symbols are necessary, last one being left as guard symbol (as in the Figure 1).

Proposal 4: In dedicated resource pools support slot formats that carry only SL-PRS (no PSCCH).

Figure 1

Note that our proposal does not exclude other slot formats for the dedicated resource pools.

It was agreed in AI 9.5.1.3 that SL-PRS transmissions with periodic reservation and SL-PRS transmissions without periodic reservation will be supported. Periodic reservations may be required in semi-persistent (if supported) or periodic SL-PRS (pre-)configured transmissions. The activation/deactivation for semi-persistent SL-PRS transmission may be done via MAC CE like for semi-persistent configurations for SRS for positioning. Supporting SL-PRS-only slot format, as proposed above, increases the number of UEs that can select resources and transmit in the same slot, thus decreasing the latency of the positioning procedures.

The support of semi-persistent SL-PRS transmission requires activation and deactivation. When MAC CE is used, as in the SRS semi-persistent configurations, the activation/deactivation may be done by the gNB for Scheme 1 and by the UE that triggers positioning procedure in Scheme 2. For instance, when a RSU tracks a mobile SL UE on highway, it may activate a semi-persistent SL-PRS transmission from that SL UE to achieve tracking for a finite time duration.

In the last meeting some confusion was created by the semi-persistent terminology. The companies pointed out that two interpretations are possible. In one interpretation, semi-persistent SL-PRS transmission is like semi-persistent SRS transmission, which requires a MAC CE activation from gNB. In another interpretation semi-persistent may be viewed like legacy SL communication.

In our understanding, a perfect SL-PRS periodicity cannot be guaranteed due to the hidden node, race conditions, and collisions in resource selection. However, based on the applications, a SL UE should be able to be (pre-)configured with a periodic or semi-persistent transmission. Such configuration may be implemented via periodic resource reservation at the time of configuration, or at the time when the activation command is received. Thus, the mechanisms associated with semi-persistent and periodic transmissions such as (pre-)configuration and activation/deactivation via MAC CE can be reused on top of the periodic resource selection and reservation mechanism in SL communication.

Proposal 5: Support aperiodic, semi-persistent and periodic SL-PRS transmissions, where semi-persistent transmission relies on MAC CE activation (like in SRS) and SL periodic resource reservation.

It is preferable if the SL Positioning reuses as much as possible the SL legacy solutions. To this end, for aperiodic SL-PRS transmission up to 3 resources should be indicated in the associated SCI (2nd SCI). In the legacy sidelink resource allocation a maximum number of 32 retransmissions is possible when the first transmission fails. We note that the concept of re-transmission is not yet defined for SL-PRS. The reliability can be solved by repeating the SL-PRS transmission multiple times.

RAN1 defined IUC scheme 1 to assist other UEs for SL resource selection. In this IUC scheme a UE uses a received non-preferred resource set when performing resource selection to exclude non-preferred resources. Similarly, for SL positioning to avoid SL-PRS collisions, due to the hidden node problem, the IUC scheme may be used in SL-PRS resource selection.

For instance, a target UE that requires SL-PRS transmissions from one or multiple anchor UEs, it could provide the non-preferred resource sets for the SL-PRS receptions. An IUC method may be used for the reception of measurements reports from the sidelink that receives SL-PRS.

Proposal 6: Use the IUC Scheme 1 to provide non-preferred resource sets for SL-PRS transmission.

We note that this scheme is suitable for both dedicated and shared resource pool transmissions.

### FDM multiplexing

In RAN1#112 was agreed that comb-based multiplexing of SL-PRS from different UEs will be supported, with at least the configuration where all UEs have same (M, N, where N represents the comb-size, and M represents the number of SL-PRS symbols in a slot. However, the exact details of multiplexing and slot structure are not yet defined.

It expected that the SL-PRS transmission in shared resource pools to differ from the transmission in dedicated resource pools. The main difference is that in the shared resource pools the SL-PRS transmissions should not impact the existing traffic from legacy Rel-16 and Rel-17 SL devices.

For dedicated resource pools, RAN1 narrowed down to two options on which channels can be included.

  - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included;

  - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included.

We note that both options support PSCCH associated with SL-PRS transmission, where PSCCH should contain the information necessary to identify and decode SL-PRS.

If the multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools, a solution to multiplex in the same slot the associated PSCCH is necessary.

Because each legacy PSCCH can occupy just contiguous sub-channels, multiple PSCCH in the same slot should occupy different sub-channels, where each first symbol of PSCCH field is duplicated in the first symbol of the slot for AGC purposes. In addition, to PSCCH each transmission should contain a DMRS.

A second AGC symbol, which consists of duplication multiplexed SL-PRS might be necessary if the SL-PRS transmission power differs from the PSCCH transmission power. A possible slot structure is presented in the following figure:

Figure 2

We note that the current specs allow both FDM and TDM multiplexing of SRS signals. A similar approach may be considered for SL-PRS multiplexing among different UEs.

The SL-PRS in different symbols may be affected by a phase shift due to mobility (Doppler) or due to oscillator frequency shifts. In order to estimate the phase shift between symbols and compensate for it is preferable that the RE-offset of the first symbol SL-PRS and the RE-offset of the last SL-PRS symbol are the same from the same UE. The RE-offset indicates the subcarrier offset between two consecutive SL-PRS symbols and may be provided by the upper layers or in the absence of upper layer configuration may have a default value.

The orthogonality of the SL-PRS coming from different UEs may be related to the associated PSCCH orthogonality. For this purpose, location of PSCCH may implicitly determine the parameters (such RE-offset, cyclic shift) of the associated SL-PRS, which can guarantee the orthogonality of the SL-PRS transmission.

Proposal 7: Support an implicit one-to-one mapping between the location of PSCCH and the comb parameters of the SL-PRS multiplexed in the same slot.

If the 2nd SCI associated with SL-PRS is supported, additional slot formats should be considered that includes the DMRS associated with PSSCH. An example is showed in the Figure 3.

Figure 3

### TDM Multiplexing

TDM SL-PRS will be supported in the same slot at least in the dedicated resource pools. As in the FDM case, multiple PSCCH should be multiplexed in the same slot. Because SL-PRS received from different UEs have different powers, each set of consecutive SL-PRS symbols from the same UE must be preceded by an AGC symbol.  A possible slot structure is presented in the following figure

Figure 4

Like in the FDM approach, a direct mapping between the PSCCH location and SL-PRS starting symbol may be considered.

Proposal 8: Support implicit one-to-one mapping between the frequency location of PSCCH and the symbol location of the SL-PRS TDM multiplexed in the same slot.

In addition, an AGC symbol should be introduced before each sequence of SL-PRS symbols.

Proposal 9: For TDM each sequence of SL-PRS symbols from the same UE is preceded by an AGC symbol.

### Further discussion on multiplexing

When multiplexing SL-PRS from multiple UEs in the same slots, interference between SL-PRS may occur. Inter-symbol interference may be caused due to differences in synchronization and propagation time. If such differences are shorter than the OFDM symbol cyclic prefix, the impact is negligible. For larger time differences with respect to the OFDM symbol duration, which may be the case for higher SCS, additional restrictions on resource reservation and selection should be considered. For instance, leaving unoccupied one symbol between consecutive transmissions from different UEs.

Similarly, for FDM, frequency differences between SL-PRS from different UEs caused by the carrier frequency offsets (CFOs), and by the Doppler spread due to channel time-variation may lead to ICI. To avoid ICI an additional frequency offset between different SL-PRS transmissions may be introduced.

Finally, we note that for short distance between UEs and lower frequency, it is expected that the ISI and ICI interference is relatively low due to the short propagation time with respect to the OFDM symbol duration.

## Power Control

The WID specifies that the open loop power control for SL-PRS will be supported, which requires an estimate of the pathloss between transmitter and receiver.

For in-coverage scenarios (Scheme 1) the pathloss between gNB and SL UE may be estimated based on SSB received from gNB.

For out-of-coverage scenarios (Scheme 2), the pathloss between SL UEs can be estimated based on S-SSB received power from the intended receiver, if S-SSB is available.

In RAN1#112 was agreed that in dedicated resource pools no other channels can be included beyond SL-PRS. For these cases when S-SSB is not available, the only available signal for pathloss estimation remains SL-PRS. Note that the SL-PRS received power might also be necessary for resource reservation and congestion control in the dedicated resource pools.  The SL-PRS reference signal received power (SL PRS-RSRP) was defined in RAN1#112, AI 9.5.1.2, as the linear average over the power contributions (in W) of the resource elements that carry SL PRS reference signals. Thus, the SL-PRS received power may be used for the estimation of pathloss provided that the transmission power is known to the receiver.

Proposal 10: Support the received power measurements SL-PRS RSRP for the estimation of the UE-to-UE pathloss.

The transmit power value may be provided to the SL UE receiver in various ways. This may be accomplished via the SCI associated with SL-PRS, or via the triggering command for the SL-PRS transmission, which may come from the receiver of SL-PRS. In the absence of such information a (pre-)configured value for SL-PRS transmission may be assumed.

We note that the PSCCH associated with SL-PRS may, in principle, be transmitted with a different power. If this is the case, additional AGC symbols are necessary prior to the SL-PRS.

# Conclusions

Proposal 1: For the definition of nID, seqSL-PRS, down select Options 1-3 for further discussion. Clarify the intent of Option 1.

Proposal 2: Consider increasing the range of M and N values and supporting the duplication of first SL-PRS symbol into the last SL-PRS symbol for large M values.

Proposal 3: Support multiple dedicated positioning resource pools within one SL BWP. In each of the dedicated RP SL-PRS occupies the entire BW.

Observation 1: In the shared resource pools SL-PRS BW may be less than the BW of the RP.

Proposal 4: In dedicated resource pools support slot formats that carry only SL-PRS (no PSCCH).

Proposal 5: Support aperiodic, semi-persistent and periodic SL-PRS transmissions, where semi-persistent transmission relies on MAC CE activation (like in SRS) and SL periodic resource reservation.

Proposal 6: Use the IUC Scheme 1 to provide non-preferred resource sets for SL-PRS transmission.

Proposal 7: Support an implicit one-to-one mapping between the location of PSCCH and the comb parameters of the SL-PRS multiplexed in the same slot.

Proposal 8: Support implicit one-to-one mapping between the frequency location of PSCCH and the symbol location of the SL-PRS TDM multiplexed in the same slot.

Proposal 9: For TDM each sequence of SL-PRS symbols from the same UE is preceded by an AGC symbol.

Proposal 10: Support the received power measurements SL-PRS RSRP for the estimation of the UE-to-UE pathloss.

# References

RP-223549, New WID on Expanded and Improved NR Positioning, RAN#98-e, December 2022.

TR 38.859, Study on expanded and improved NR positioning; (Release 18), December 2022.

TS 38.214, Physical layer procedures for data (Release 17), March 2022

TS 38.331, Radio Resource Control (RRC) protocol specification, (Release 17), June 2022

TR 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 16), December 2021

TS 38.211, Physical channels and modulation (Release 17), March 2022

RAN1#110, Chair notes

RAN1#111, Chair notes

RAN1#112, Chair notes