3GPP TSG RAN WG1 #112bis-e	R1-2302328

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.5.1.3

Source:	Futurewei

Title:	Discussion on resource allocation for SL PRS

Document for:	Discussion/Decision

# Introduction

In RAN#98-e the New WID on Expanded and Improved NR Positioning was approved [RP-223549]. The first objective of the WID is the support of positioning for sidelink:

- “Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]:

  - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1].

    - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum.

    - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified.

  - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

  - Specify support of resource allocation for SL PRS:

    - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1].

      - For resource allocation mechanism for SL PRS in Scheme 2:

        - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1].

        - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1].

    - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1].

      - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

  - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1].

  - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

  - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]:

    - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)).

    - Specify the protocol and procedures for SL positioning between UEs and LMF.

  - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2]

  - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].”

In this contribution, we focus on potential solutions for SL positioning resource allocation including details related to the support of unicast/groupcast/broadcast of SL PRS transmissions.

# Agreements reached in RAN1#112

Agreement

- A UE can be configured to perform either resource allocation Scheme 1 or Scheme 2, applicable to all resource pools (dedicated or shared resource pools).

- SL PRS unicast/groupcast/broadcast can occur in either a shared or a dedicated resource pool.

Agreement

For a dedicated resource pool for positioning:

- The set of slots that belong to a resource pool is determined in the same way as for legacy SL communication pool (i.e. see section 8 of 38.214).

  - FFS: additional slots that can be used for SL PRS is not precluded

- Study what the dedicated resource pools for positioning (pre-)configuration should include, and consider at least the following: The start PRB position, the number of contiguous PRBs, SL-PRS configuration, synchronization configuration, resource allocation scheme 2 related configuration, power control configuration, sub-channel size and sub-channel count, time-domain bitmap, reporting configuration

Agreement

For a dedicated resource pool for Positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further.

- Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH).

Agreement

Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further).

Agreement

Sensing based or random selection in Scheme 2 is allowed by (pre-)configured per resource pool (similar to Rel-17 NR sidelink communication).

  - Working assumption: Sensing-based and random selection can be allowed in the same resource pool

  - FFS: whether any enhancements are needed for coexistence of random selection and sensing-based resource selection in a resource pool

- FFS: Details on the sensing-based resource selection and random selection, whether it will be similar to NR Rel-16 or NR Rel-17.

Agreement

With regards to random resource selection, reuse existing Rel-17 random selection mechanism from sidelink communications.

  - Study if any changes are needed

Agreement

In Scheme 2, with regards to the triggering of SL-PRS, support one or both of the following options:

- Option 1: Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers.

  - Note: this also includes higher layer triggering from another UE

- Option 2: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A.

  - FFS: Whether lower-layer signaling is SCI or SL MAC-CE

Agreement

For SL-PRS transmission, at least support the following

- SL-PRS transmissions with periodic reservation: SL-PRS transmissions which are being reserved with a similar mechanism as the SL periodic resource reservation for another TB in legacy SL communication

  - FFS: whether/what changes are needed

- SL-PRS transmissions without periodic reservation: SL-PRS transmissions in which the SL-PRS is transmitted at least once without periodic reservation, with a similar mechanism as in legacy SL communication with SL resource without periodic reservation.

  - FFS: Maximum number of reservations and transmissions after triggering

Agreement

For the scheme 2 sensing-based resource allocation,

- Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool.

  - Study if/what changes are needed

- Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool.

  - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc)

- From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

# Discussions

## General aspects

In RAN1#112 it was agreed as a working assumption that sensing-based and random selection can be allowed in the same resource pool. We note that these types of resource selection are associated only with Scheme 2 (Mode 2) resource allocation.

In Mode 2, as defined in 8.1.4 of TS 38.214, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. The indication of resource selection mechanism(s) is provided as sl-AllowedResourceSelectionConfig, which may comprise of full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof. For a minimum impact on the existing specs we propose to confirm the working assumption.

Proposal 1: Confirm the working assumption.

The exact multiplexing in time and frequency of the SL-PRS signals in the same slot is not yet finalized, however an agreement in AI 9.5.1.1 supports comb-based multiplexing from different UEs.

Agreement

- Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools.

  - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools.

- For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible .

  - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values.

- FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs

If multiple SL-PRS from different SL UEs are multiplexed in the same slot, the SL resource reservation procedures in Scheme 2 may need to be revised. In sensing-based resource allocation [8.1.4, TS 38.214], a UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full/partial sensing. This implies that the selected resources cannot overlap in frequency with existing reserved resources having the same priority.

If a similar approach were used for SL-PRS, multiplexing in frequency becomes inefficient because each SL-PRS would be required to occupy different sets of contiguous sub-channels. This would substantially limit the BW of each SL-PRS, and thus, the precision of positioning measurements. However, if comb-based multiplexing is allowed in the same symbol, the resource SL allocation needs to be changed to allow selection of frequency resources that overlap.

We observe that if several UE can multiplex their SL-PRS transmission in frequency (for instance comb-based), a multiplexing solution for the associated PSCCH is necessary.

Proposal 2: Investigate the necessary changes of the resource selection and allocation procedures to support comb-based SL-PRS multiplexing.

A possible change to the resource selection procedure is the addition of the comb and offset to the resources to avoid selection of the same resources.

After the decision to support sensing-based and random selection in the same resource pool, an open question remains: how to avoid resource selection collisions and resource allocation unfairness when, for instance, sensing-based and random selection are applied in the same slot?

A remaining FFS point is whether any enhancements are needed for the coexistence of random selection and sensing-based resource selection in a resource pool. Companies observed that a possible fairness issue can occur as well as collisions with higher priority transmissions by random allocations from lower priority requests.

A straightforward solution is to have sensing-based resource selection and random based resource selection in separate resources of the same RP, for instance separate slots or in the same slot in separate symbols. For instance, in the same slot have selected resources that can FDM/TDM based on sensing and other resources where random selection can be applied. Alternatively, to limit the impact of random based selection, sensing-based resource selection may be used on all resources and just a subset of all resources to be allocated using a random based selection.

Proposal 3: For the coexistence between random and sensing-based resource selection in the same resource pool, the random-based selection resources are separate or a subset of the sensing-based resources.

## Dedicated resource pool

For the dedicated resource pools RAN1 has yet to decide which channels can be included.

Agreement

For a dedicated resource pool for SL positioning,

- With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options:

  - Opt. 1: No other channel can be included beyond SL-PRS

  - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included

  - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included

    - FFS: Details

    - FFS: definition of PSSCH associated with SL-PRS transmission(s)

- Note: Companies are encouraged to provide their analysis and views on the above

Option 2 offers a simple solution by supporting only the PSCCH channel. However, in this solution. A second stage SCI is not included. SCI carried on PSSCH is a 2nd-stage SCI, which transports sidelink scheduling information, source and destinations ID, zone indicator and inter-UE coordination indicator. Therefore, Option 2 limits considerably the signaling in the dedicated resource pool, which requires additional signaling in the shared resource pool. For instance, a 2nd stage SCI may carry the trigger for SL-PRS for RTT procedure in the same slot as the initial SL-PRS transmission. Another 2nd stage SCI usage may be signaling the IUC transmission that can be used to assist SL-PRS multiplexing.

Proposal 4: Support Option 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included

- FFS: Details

- FFS: definition of PSSCH associated with SL-PRS transmission(s)

## Shared resource pool

For the shared resource pool, three basic options were considered in the last meeting:

- Opt. 1: SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in a slot

    - The PSSCH and SL-PRS bandwidth is the same

    - With regards to PSSCH and SL-PRS multiplexing

      - Alt. 1: Only TDMing is supported

      - Alt. 2: TDMing and RE-level FDMing (similar to SL-CSI-RS) is supported

  - Discuss further whether PSFCH could also exist in this slot

- Opt. 2: SL-PRS and associated PSCCH are included (no PSSCH) in a slot

    - The presence of SL-PRS is indicated in the 1st SCI in the PSCCH

- Opt. 3: SL-PRS without PSCCH are included (no PSSCH)

    - SL-PRS may be reserved by a SCI siganling in the previous slot

- In either option, study multiplexing option between PSCCH (if present)  and SL-PRS are TDMed

As a design principle, we would prefer that the designs of the shared and the dedicated resource pools to be as similar as possible. To this end Option 2 should be supported. We also note that the above options are not mutually exclusive. In other words, SL-PRS may be supported in slots that carry PSCCH and PSSCH. For instance, SL-PRS may be provided via PSFCH either stand alone or multiplexed with HARQ feedback. Option 3 may be considered for CG transmissions; however, the lack of PSCCH increases decoding complexity, therefore it is not preferable. However, such configurations (SL-PRS transmissions without PSCCH) may supported in the dedicated resource pools.

Proposal 5: For shared resource pools SL-PRS transmission, support Option 1 and Option 2.

In the last meeting most companies supported SCI-1 and SCI-2 (possibly a new format) for shared resource pools [R1-230162], with the majority preferring Option 1 of the following FL proposal:

In a shared resource pool, in addition to SL PRS transmission, the UE transmits

- Opt. 1: SCI1-A & a new 2nd stage SCI format used for SL-PRS indication

    - FFS: Details for the new 2nd stage SCI format

    - Note: data can also be indicated using the 2nd stage SCI in addition to SL-PRS

- Opt. 2: SCI1-A & existing 2nd stage SCI format used for SL-PRS indication

    - The presence of SL-PRS is indicated in the 1st SCI

The SCI-2 format used in shared resource pool may be used for L1 related signaling for shared as well as dedicated resource pools. Because there is no legacy SCI-2 to support L1 SL-PRS related signaling, a new SCI-2 format to support SL-PRS  is necessary.

Proposal 6: In shared resource pools support Option 1, i.e., SCI-1A and a new SCI-2 for SL-PRS related L1 signaling.

## Scheme 1 (Mode 1 of resource allocation)

In Scheme 1, a SL UE is in gNB coverage, and the SL-PRS resources are provided by the gNB. It was agreed in RAN1#112 that Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e., Option 1 is not pursued further).

Thus, a transmitting UE receives resource allocation only via Opt. 2: through Dynamic grant, or through configured grant type 1/type 2 from gNB.

For Scheme1, the specs [7.3.1, TS38.212] define DL signalling (DCI Format 3_0) for scheduling of NR PSCCH and NR PSSCH in one cell. Given that in shared resource pools, the PSSCH and SL-PRS may be transmitted (multiplexed) in the same slot, it is convenient to further extended DCI Format 3_0 to provide the necessary frequency, time and power resources for SL-PRS transmission. For dedicated resource pools, if RAN1 decides that no PSSCH is supported in addition to SL PRS, the DCI 3_0 may be further simplified.

Proposal 7: For Scheme 1 (Mode 1) SL PRS resource allocation use DCI Format 3_0 signalling resource allocation as a starting point.

We note that gNB uses DCI Format 0_1 to inform a SL UE (Sidelink assignment index field) DCI Format 3_0

A new DCI format (these fields shall be defined in AI 9.5.1.1) should contain (among others):

  - Resource pool id (shared or dedicated)

  - Frequency resource (including the comb, offset, etc)

  - Time resource (including transmission period)

  - Power transmission

  - Destination ID

## Scheme 2 (Mode 2 of resource allocation)

In Scheme 2 the SL-PRS resource allocation is done autonomously by at least one of the UE(s) participating in the sidelink positioning operation. As specified in the WID, both sensing-based and random resource selection should be supported.

In sensing-based resource selection, as defined in TS38.214, the UE shall monitor slots which belong to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the selection based on PSCCH decoded and RSRP measured in these slots.

The RSRP measure associated with PSCCH is based on the average linear power measured over the DMRS resources associated with that PSCCH.

When multiple SL-PRS signals are multiplexed in frequency, their associated PSCCH and potentially PSSCH may be multiplexed in the same slot as well. Therefore, the granularity of resource selection should be extended to consider the type of multiplexing supported. For instance, if the SL-PRS is multiplexed in frequency using different comb index and offset, the comb index and the offset shall be considered as a resource that can be selected and reported to upper layer when available. For TDM the transmission symbols in a slot can be a resource to select as well.

Proposal 8: For multiple SL-PRS multiplexed in a single slot, extend the granularity of the resource selection procedure.

For RTT-based positioning methods, it is important that the duration between received and transmit positioning RS is minimized. A reduced Rx-Tx duration will limit the impact of the clock drift between transmitter and receiver, which leads to better accuracy. To achieve a lower latency, it would be preferable if a UE initiator of the SL positioning would allocate resources for both transmit and receive of SL-PRS. To this end, the UE may use the preferred or non-preferred SL-PRS resources from the other UE(s) participating in the SL positioning. For instance, a SL UE may reserve resources two slots for SL-PRS transmission, where the second slot is dedicated for the responder SL UE transmission. The resources may be reserved for a single slot or periodic transmissions. It is necessary that the reserved resources be apart in time to allow the SL UE receiver (responder) enough processing time.

Proposal 9: For sensing-based resource allocation a SL UE may reserve SL-PRS resources for other SL UEs participants in the positioning process.

### IUC

RAN1 defined IUC scheme 1 to assist other UEs for SL resource selection. In this IUC scheme, a UE uses a received non-preferred resource set when performing resource selection to exclude non-preferred resources. Similarly, for SL positioning to avoid SL-PRS collisions, due to the hidden node problem, the IUC scheme may be used in SL-PRS resource selection.

For instance, a target UE that requires SL-PRS transmissions from one or multiple anchor UEs, it could provide the non-preferred resource sets for the SL-PRS receptions. An IUC method may be used for the reception of measurements reports from the sidelink UE that receives SL-PRS.

Proposal 10: Use the IUC Scheme 1 to provide non-preferred resource sets for SL-PRS transmission.

When a SL UE requests or triggers SL-PRS transmissions from multiple SL UEs, the latency is substantially reduced if multiple SL-PRS can be multiplexed in the same slot. To this end, the SL UE requesting SL-PRS transmission should be able to reserve resources for multiple UEs SL-PRS transmissions, for instance indicating the slot when the UEs should transmit SL-PRS, where each SL-PRS may have different comb index and frequency offset. This can be achieved using IUC Scheme 2.

Proposal 11: Use the IUC Scheme 2 to provide the preferred resources for multiple SL-PRS same slot multiplexing.

### Congestion control

Congestion control is based on the measurement of channel occupancy ratio (CR) and channel busy ratio (CBR), defined in TS 38.215, Clause 5.1.26/27. The condition to transmit PSSCH is that the sum of all CRs for all priority values of the transmissions occurring in the measuring window is less than a threshold associated with the priority of the PSSCH transmission and CBR range (as defined in TS 38.214, Clause 8.1.6).  This condition limits the CR for each priority traffic category.

The approach is suitable for shared resource pool transmission of SL-PRS, where the SL-PRS transmission may be postponed when there are higher priority transmissions.  To implement such solution for congestion control, the SL-PRS transmissions should be associated with the sidelink priority.

Proposal 12: For SL-PRS resource allocation in shared resource pools support the congestion control in sidelink resource allocation Scheme 2.

RAN1 has not yet decided whether in the dedicated resource pools other than SL-PRS related traffic is supported.

In RAN1#112 it was agreed that the SL-PRS priority is provided by the upper layers. Discussions did not identify any motive to support different priorities for different SL-PRS transmissions. Especially in a dedicated resource pool we do not see any argument to support different priorities for different SL-PRS transmissions. If some scenarios, for instance such as SL positioning for first responders require lower latency, a separate dedicated resource pool can be configured to avoid any possible interference or blocking issues. If the same dedicated resource pool is used for SL positioning applications with different requirements, just the simple usage of different priorities for different SL-PRS transmissions may not guarantee that the transmissions not impact each other either due to the hidden node problem or due the random resource allocation.

Proposal 13: For a dedicated resource pool support only SL-PRS with the same priority.

If only SL-PRS traffic is supported in the dedicated resource pools, congestion control becomes less important as all traffic has some priority.

Observation 1: If only SL-PRS traffic with the same priority is supported in a dedicated resource pool, congestion control may not be necessary.

## Cast types

Based on WID, RAN1 should specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions.

Methods such multi-RTT requires multiple measurements of the Rx-Tx duration between SL-PRS received and transmitted signals. Such methods may be implemented as successive RTT procedures where the originator UE unicasts SL-PRS successively to multiple UEs, which are unicasting back their SL-PRS transmissions. An alternative, which reduces overhead and latency, is for the UE to multicast a SL-PRS transmission to multiple UEs. Each UE successively responds with its unicast SL-PRS transmissions. In this case, a group of anchor UEs (responder UEs) may be formed by the upper layers for instance, using anchor UEs signal strength, synchronization source or the LOS/NLOS propagation between the target and anchor UEs. For some scenarios, such as the highway, the RSU may periodically broadcast the SL-PRS to all UEs in its coverage.

Proposal 14: For multi-RTT support SL-PRS multicast transmission, where multicast groups may be defined by some specific criteria (e.g., location-zone, RSRP, LOS/NLOS, requirement on its sync source/location information).

As pointed out in our contribution R1-2302327, out-of-order SL-PRS transmission for multi-RTT is supported. In these scenarios, the Rx-Tx time difference may become quite large, and therefore may be affected by clock drifts, or other sources of errors. It is preferable, if the Rx-Tx time difference is limited to a maximum value. If due to the resource selection conditions the Rx-Tx time difference may become too large, a retransmission of SL-PRS can be requested, or just triggered at the transmitter by a timer expiration.

Proposal 15: For RTT-based positioning a maximum time gap (delay) of the SL-PRS transmission from the responding SL UE should be supported.

# Conclusions

Proposal 1: Confirm the working assumption.

Proposal 2: Investigate the necessary changes of the resource selection and allocation procedures to support comb-based SL-PRS multiplexing.

Proposal 3: For the coexistence between random and sensing-based resource selection in the same resource pool, the random-based selection resources are separate or a subset of the sensing-based resources.

Proposal 4: Support Option 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included

- FFS: Details

- FFS: definition of PSSCH associated with SL-PRS transmission(s)

Proposal 5: For shared resource pools SL-PRS transmission, support Option 1 and Option 2.

Proposal 6: In shared resource pools support Option 1, i.e., SCI-1A and a new SCI-2 for SL-PRS related L1 signaling.

Proposal 7: For Scheme 1 (Mode 1) SL PRS resource allocation use DCI Format 3_0 signalling resource allocation as a starting point.

Proposal 8: For multiple SL-PRS multiplexed in a single slot, extend the granularity of the resource selection procedure.

Proposal 9: For sensing-based resource allocation a SL UE may reserve SL-PRS resources for other SL UEs participants in the positioning process.

Proposal 10: Use the IUC Scheme 1 to provide non-preferred resource sets for SL-PRS transmission.

Proposal 11: Use the IUC Scheme 2 to provide the preferred resources for multiple SL-PRS same slot multiplexing.

Proposal 12: For SL-PRS resource allocation in shared resource pools support the congestion control in sidelink resource allocation Scheme 2.

Proposal 13: For a dedicated resource pool support only SL-PRS with the same priority.

Observation 1: If only SL-PRS traffic with the same priority is supported in a dedicated resource pool, congestion control may not be necessary.

Proposal 14: For multi-RTT support SL-PRS multicast transmission, where multicast groups may be defined by some specific criteria (e.g., location-zone, RSRP, LOS/NLOS, requirement on its sync source/location information).

Proposal 15: For RTT-based positioning a maximum time gap (delay) of the SL-PRS transmission from the responding SL UE should be supported.

# References

RP-223549, New WID on Expanded and Improved NR Positioning, RAN#98-e, December 2022.

TS 38.855, Study on NR positioning support (Release 16), March 2019

TR 38.305, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN (Release 16), December 2021

TS 38.214, Physical layer procedures for data (Release 17), March 2022

TS 38.211, Physical channels and modulation (Release 17), March 2022

RAN1#110, Chair notes

RAN1#111, Chair notes

TR 38.857

RAN1#112, Chain notes