3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302329

Electronic Meeting, April 17 – 26, 2023

Agenda Item:	9.5.5

Source:	Futurewei

Title:	On positioning for RedCap UE in Rel-18

Document for:	Discussion and Decision

# Introduction

In the approved new WID on Expanded and Improved NR Positioning [1], the objectives of positioning for UEs with Reduced Capabilities are as follows:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4].

This contribution discusses and addresses the open issues associated with the agreements reached at the RAN1#112 meeting.

# Discussion

The majority of the evaluation results captured in TR 38.859 [3] indicate legacy RedCap UE cannot meet the horizontal positioning requirement for IIOT use cases for FR1 with 5 MHz or 20 MHz bandwidth. Consequently, frequency hopping is a recommended solution, which can improve positioning accuracy by extending the maximum narrow transmission bandwidth of SRS and PRS to beyond 20 MHz and 100 MHz for FR1 and FR2, respectively. The frequency hopping solution enables RedCap UE to transmit/receive narrowband reference signals (e.g., 20 MHz or 5 MHz) for positioning at each hopping occasion. At each hop, the RedCap UE selects a different chunk of the wide bandwidth. Frequency hopping will continue until a sufficiently wide bandwidth for positioning (e.g., 100 MHz for FR1 or 400 MHz for FR2) is covered. At the receiver side, the received narrowband reference signal at each hop is coherently combined to obtain wideband positioning measurements.

## PRS resource configuration for UE Rx frequency hopping

At the RAN1#112 meeting, RAN1 concluded that only Rx frequency hopping of the DL PRS is supported for positioning enhancements for RedCap UEs [2].

Conclusion For positioning enhancements for RedCap UEs, only Rx frequency hopping of the DL PRS is supported.

With regard to DL PRS Rx frequency hopping, RAN1 made the following agreements, but several issues remain unresolved [2]:

Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW Agreement For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details

Referring to the above agreement, gNB configures and transmits one DL PRS resource over a full carrier bandwidth (e.g., 100 MHz for FR1) as shown in Fig. 1. The full carrier bandwidth would be the total frequency-hopping bandwidth of the RedCap UE.

t

f

Wideband PRS resource i

Full carrier bandwidth (e.g., 100 MHz) for FR1

gNB Tx

RedCap UE Rx

PRS resource i

PRS resource i

PRS resource i

PRS resource i

Instantaneous Bandwidth IBW

Fig. 1. RedCap UE Rx hops within one DL PRS resource

Switching time tswitch

Switching time tswitch

Measurement Gap Length

Dwell time tdwell

Fig. 2. Wideband frequency hopping measurement gap

The RedCap receiver hops within the configured DL PRS resource defined by a measurement gap as illustrated in Figure 2. The frequency-hopping measurement gap (FHMG) can include all the hops needed by the RedCap receiver in a single instance, which depends on the length of the FHMG. If it cannot, then multiple instances are required. The measurement gap length can be expressed as

TFHMG=N×tswitch+N×tdwell

where N = the number of frequency hops, tswitch is the hop switching time defined as the RF retuning delay for the RedCap receiver to change from one carrier frequency to another, and tdwell is instantaneous dwell time corresponding to the duration (OFDM symbols) of the configured PRS resource in the time domain. The hop switching time tswitch may need to be expressed in terms of an integer multiple of OFDM symbols so that the instantaneous dwell time tdwell is aligned with the OFDM symbol boundary.

An example of the frequency-hopping measurement gap length based on the number of hops, channel bandwidth BWchannel, and total bandwidth BWtotal is presented in Table 1, assuming tswitch=100 μs and tdwell=2 OFDM symbols which depend on the subcarrier spacing.

Table 1: Example of frequency-hopping measurement gaps

| SCS | BWchannel | BWtotal | N=Nmin | TFHMG |
| --- | --- | --- | --- | --- |
| 30 kHz | 5 MHz | 100 MHz | 20 | 2.7 ms |
|  | 10 MHz |  | 10 | 1.4 ms |
|  | 15 MHz |  | 7 | 0.7 ms |
|  | 20 MHz |  | 5 | 0.7 ms |
| 60 kHz | 10 MHz | 100 MHz | 10 | 1.2 ms |
|  | 15 MHz |  | 7 | 0.8 ms |
|  | 20 MHz |  | 5 | 0.6 ms |
| 120 kHz | 50 MHz | 400 MHz | 8 | 0.9 ms |
|  | 100 MHz |  | 4 | 0.4 ms |

Observation 1: For RedCap UEs with DL PRS Rx Hopping, the measurement gap depends on the following:

- the number of hops,

- the hop switching time,

- instantaneous dwell time (i.e., the number of OFDM symbols) of PRS

Proposal 1: For RedCap UE with DL PRS Rx hopping, RAN1 to discuss measurement gap length.

## SRS resource configuration for UE Tx frequency hopping

Frequency hopping is recommended as one enhancement to improve positioning accuracy in RedCap UEs. Frequency hopping enables legacy RedCap UEs to transmit SRS signals for positioning over a wide bandwidth which is greater than the maximum supported 20 MHz and 100 MHz for FR1 and FR2, respectively. During the RAN1#112 meeting, RAN1 discussed frequency hopping for which an agreement for SRS resources configured with frequency hopping for positioning was reached [2], but how to configure such SRS resources for RedCap UE remains an open issue.

Agreement For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources

Referring to the above agreement, two options, which can be used to configure SRS resources for positioning to support frequency hopping, are possible:

Option 1: RedCap UE Tx frequency hopping is configured within one SRS resource;

Option 2: RedCap UE Tx frequency hopping is configured across SRS resources

### Option 1: UE Tx frequency hopping is configured within one SRS resource

With this option, the RedCap UE is configured with one SRS resource for positioning (SRSp resource) only. Two alternative configurations for the SRSp resource are feasible:

Alt 1: One SRS resource spanning the instantaneous bandwidth

Alt 2: One SRS resource spanning the total frequency hopping bandwidth

Alt 1 configuration: One SRS resource for the instantaneous bandwidth

Fig. 3 illustrates an example in which the RedCap UE is configured with one SRS positioning resource SRSp Resource i. The bandwidth of SRSp resource i is equal to IBW (instantaneous bandwidth). For this alternative, the same SRSp resource configuration is applied to the SRS transmission in each hop. As shown in the figure, SRSp Resource i is the SRS resource configuration used by the RedCap UE over the five consecutive hops assuming the total frequency hopping bandwidth BWtotal equals 100 MHz and the IBW equals 20 MHz. The SRS instantaneous hopping bandwidth is fixed at each hopping occasion. A smaller instantaneous bandwidth can be used but this increases the number of hops required in a hopping cycle; thus, the positioning measurement delay over the overall bandwidth also increases. At the receiver end, the gNB is expected to coherently combine the SRSp resource received in each hop to form a wideband positioning measurement.

It is worth noting that Option 1 shares a similar concept to the Rel-15 NR MIMO SRS with frequency hopping. This option assumes the parameter configuration of the SRSp resource is the same at each frequency hop.

t

f

SRSp Resource i

Total frequency hopping bandwidth

(e,g., 100 MHz) for FR1

RedCap UE Tx

SRSp Resource i

SRSp Resource i

SRSp Resource i

SRSp Resource i

Instantaneous Bandwidth IBW

Fig. 3. UE Tx frequency hopping is configured within one SRS resource spanning the instantaneous bandwidth – Alt 1

Alt 2 configuration: One SRS resource for the total frequency hopping bandwidth

For this alternative, one SRS resource, which is SRSp Resource i, is configured over the total frequency hopping bandwidth BWtotal (e.g., 100 MHz for FR1) as shown in Fig. 4. At each hop, the RedCap UE selects a chunk of the total bandwidth configured with SRSp Resource i for SRS transmissions, where the bandwidth of each chunk corresponds to the instantaneous bandwidth IBW. As shown in Fig. 3, if IBW=20 MHz for FR1, then five hops are needed to sound the full 100 MHz bandwidth. The instantaneous bandwidth is fixed at each hop. Similar to Alt 1, a smaller instantaneous bandwidth can be used but this increases the number of frequency hops required; thus, the positioning measurement delay over the overall bandwidth also increases. At the receiver end, the gNB coherently combines the chunk received in each hop to form a wideband positioning measurement.

t

f

Total frequency hopping bandwidth (e,g., 100 MHz)

for FR1

RedCap UE Tx

Instantaneous Bandwidth IBW

SRS resource i

Fig. 4. UE Tx frequency hopping is configured within one SRS resource spanning the total bandwidth – Alt 2

Observation 2: For UE Tx frequency hopping configured with one SRS positioning resource, a small instantaneous bandwidth increases the positioning measurement delay over the total frequency hopping bandwidth.

Proposal 2: For RedCap UE Tx frequency hopping is configured within one SRS resource, RAN1 should clarify which one or both of the following possible alternatives:

Alt 1: One SRS resource spanning the instantaneous bandwidth

Alt 2: One SRS resource spanning the total frequency hopping bandwidth

### Option 2: UE Tx frequency hopping is configured across SRS resources

With this option, the RedCap UE is configured with a set of SRSp resources for positioning. The RedCap UE may select a different SRSp resource from the set for SRS transmissions at each hopping occasion. Fig. 5 illustrates an example in which the RedCap UE is configured with five different SR-p resources, which include SRS-pos Resource i, j, k, l, and m. As such, the SRS transmission at each hopping occasion can have a different SRSp resource. This option allows each SRSp resource to be configured with different instantaneous bandwidth IBWi. In comparison with Option 1, this option provides more flexibility. The SRSp resource received by the gNB at each hopping occasion is coherently combined to form a wideband positioning measurement.

Observation 3:  RedCap UE Tx frequency hopping configured across SRS resources for positioning provides more flexibility (e.g., different instantaneous hopping bandwidths, etc.) than the one with one SRS resource.

t

f

SRSpos Resource i

Overall frequency hopping bandwidth (e,g., 100 MHz)

for FR1

RedCap UE Tx

SRSpos Resource j

SRSpos Resource k

SRSpos Resource l

SRSpos Resource m

Instantaneous Bandwidth IBWj

Instantaneous Bandwidth IBWl

Fig. 5. UE Tx frequency hopping is configured across SRS resources spanning the instantaneous bandwidth

### Options 1 vs 2

Table 2 provides a comparison between Options 1 and 2.

Table 2: Option 1 vs Option 2

| Option 1 |  | Option 2 |
| --- | --- | --- |
| Alt 1 | Alt 2 |  |
| Fixed instantaneous bandwidth | Flexible instantaneous bandwidth | Flexible instantaneous bandwidth |
| The same SRSp resource configuration for each hop | One SRSp resource configuration over the total frequency hopping bandwidth for positioning | Different SRSp resource configuration for each hop |
| Good PAPR | Degraded PAPR | Good PAPR |
| No support for adaptive frequency hopping | No support adaptive frequency hopping | Support for adaptive frequency hopping |

## Instantaneous bandwidth configurations for frequency hopping RedCap UE

During the RAN1#112 meeting, RAN1 concluded that the scope for RedCap UE positioning includes FR1 and FR2.

Conclusion The scope for RedCap positioning includes FR1 and FR2.

For Rel-17 RedCap UEs, Tables 3 and 4 present the transmission bandwidth configuration corresponding to each of the specified channel bandwidths up to 20 MHz and 100 MHz for FR1 and FR2, respectively. The number of Tx chains supported by RedCap UE is capped at one and Rx chains is two. Refer to Appendix A for further details about legacy RedCap UE capabilities.

Table 3:  Maximum transmission bandwidth configuration NRB for FR1 RedCap UE (from TS 38.101-1)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 |
| 30 | 11 | 24 | 38 | 51 |
| 60 | N/A | 11 | 18 | 24 |

Table 4:  Maximum transmission bandwidth configuration NRB for FR2 RedCap UE (from TS 38.101-2)

| SCS (kHz) | 50 MHz | 100 MHz |
| --- | --- | --- |
|  | NRB | NRB |
| 60 | 66 | 132 |
| 120 | 32 | 66 |

The instantaneous bandwidth IBW determines how many hops are needed to sound bandwidth which is wider than the above channel bandwidth. If the instantaneous bandwidth IBW is equal to a given channel bandwidth, then the required number of hops is the minimum, which is

Nmin=BWtotalBWchannel

where BWtotal= total frequency hopping bandwidth (e.g., 100 MHz)

BWchannel= channel bandwidth (e.g., 20 MHz)

It is not possible to configure the Rel-16 SRS resource for positioning, i.e., SRS-posResource-r16 (see Appendix B for the definition) with frequency hopping since the only parameter in the definition is c-SRS-r16. The parameter c-SRS-r16 simply makes a reference to a table specified in TS 38.211, providing the bandwidth allocated to the SRS-posResource. A subset of the table is presented in Table 5, which is reused from the Rel-15 NR MIMO SRS with frequency hopping. As frequency hopping is not supported, the only SRS bandwidth configuration in Table 5, which is applicable to configuring SRS-posResource-r16 is the BSRS=0 column. The value of mSRS,0 in this column determines the maximum bandwidth which can be sounded by the configured SRS-posResource. All values of mSRS,0 are an integer multiple of 4.

Table 5: SRS bandwidth configuration (from TS 38.211, Table 6.4.1.4.3-1)

| 0 | 4 | 1 | 4 | 1 | 4 | 1 | 4 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 | 1 | 4 | 2 | 4 | 1 | 4 | 1 |
| 2 | 12 | 1 | 4 | 3 | 4 | 1 | 4 | 1 |
| The rest is omitted |  |  |  |  |  |  |  |  |

One possibility is to extend the Rel-16 SRS-posResource definition with frequency hopping along with the SRS bandwidth configuration to support RedCap UE positioning. Table 6 presents an example of instantaneous SRS bandwidth configuration.

Table 6: Example of instantaneous SRS bandwidth configuration for frequency-hopping RedCap UE

| CSRSp | SCS | BWchannel | BSRSp=0 |  |
| --- | --- | --- | --- | --- |
|  |  |  | mSRSp,0 | N0 |
| a | 15 kHz | 5 MHz | 25 | N0=BWtotalBWchannel |
| b |  | 10 MHz | 52 |  |
| c |  | 15 MHz | 79 |  |
| d |  | 20 MHz | 106 |  |
| e | 30 kHz | 5 MHz | 11 |  |
| f |  | 10 MHz | 24 |  |
| g |  | 15 MHz | 38 |  |
| h |  | 20 MHz | 51 |  |

mSRSp,0 is the instantaneous bandwidth IBW, which corresponds to the maximum number of PRBs for given BWchannel and SCS. N0 is equal to Nmin defined previously.

Proposal 3: For RedCap UE Tx frequency hopping, RAN1 to discuss instantaneous SRS bandwidth configurations per hop.

## DL and UL frequency hopping

In both the DL Rx and UL Tx frequency hopping, two options are listed in [3]:

Option 1: Non-overlapping frequency hopping

Option 2: Partial overlapping frequency hopping

Each option is described and analyzed below.

Option 1: Non-overlapping frequency hopping

With this option, each hopping occasion of the SRS transmission within a hopping cycle covers a different and disjoint portion of the whole UE sounding bandwidth, i.e., there is no overlapping between the frequency resources used by different hops within a hopping cycle. Fig. 6 illustrates an example of SRS transmission with frequency hopping. There are four OFDM symbols within a slot allocated for the UE for SRS transmissions (shown as the shaded blocks). The bandwidth per hop is e.g., 20 MHz and the complete or aggregate sounding bandwidth is 100 MHz. The hopping cycle in this example is one slot.

With non-overlapping frequency hopping, it is possible that some unknown or random phase rotation is introduced to each hop of the SRS due to RF retuning at either the transmitter and receiver. Further, a phase shift discontinuity can occur when time and/or frequency change. In that case, the performance of positioning estimation based on each hop of the SRS is impacted. The reason is that the measurement over each hop of the SRS cannot be combined directly to form a wideband positioning measurement due to the unknown or random phase rotation of each hop. Compared to the case where SRS frequency hopping is not required and wideband positioning measurement is available, there will be performance loss on positioning estimation when the non-overlapping frequency hopping is utilized. Fig. 7 shows the performance comparison of channel delay estimation performance between SRS frequency hopping and wideband SRS transmissions (without frequency hopping). The simulation assumptions can be found in Appendix C. It can be observed in Fig. 7 that a performance loss of around 10 dB is possible for the frequency hopping case compared to the wideband case.

t (OFDM Symbols)

f

Hopping Cycle

Fig. 6. Non-overlapping frequency hopping for positioning with SRS

Fig. 7. Channel delay estimation error rate vs SINR

Observation 4: Non-overlapping frequency hopping for SRS positioning causes an unknown or random phase rotation at every hop resulting in the positioning accuracy degradation.

Option 2: Partially overlapping frequency hopping

As compared with the non-overlapping frequency hopping, this option supports partial overlapping of frequency resources between two successive hops. Fig. 8 illustrates an example of SRS frequency hopping transmission with partial overlapping of frequency resources. The example is similar to the one in Option 1 except there is an overlap of frequency resources, e.g., one PRB between successive hops in adjacent frequency resources. The hopping pattern may then repeat in the next hopping cycle if the SRS transmission is configured as periodic, or semi-persistent and is activated. The size of the overlapping frequency resources may be indicated from the base station to the RedCap UE in an RRC message, a MAC CE, or a DCI message. The size of the overlapping frequency resources can be expressed as an integer multiple of PRBs. After receiving the indication of the size of the overlapping frequency resources, the UE may then decide the frequency domain starting position of each hop over time accordingly. The UE may then transmit the SRSs according to the frequency domain starting position of each hop.

t (OFDM Symbols)

f

Hopping Cycle

Fig. 8. Partial overlapping frequency hopping for positioning with SRS

The partial overlapping of frequency resources enables the base station to estimate the phase difference of the random phase rotation over each hop. This is achieved by the base station performing measurement on the SRS over the overlapping frequency resources, assuming that the phase of the measured channel response should be equal over the overlapping portion of two different hops. The measured phase difference over the overlapping portion of two different hops may then be counted as the phase difference of the random phase rotation over the two different hops. The base station may then compensate for or rotate back the random phase difference among hops and combine the positioning measurement over each hop to obtain a wideband positioning measurement. The base station may use the wideband positioning measurement to obtain a robust estimation of the position of the RedCap UE.

Observation 5: Partial overlapping frequency hopping for SRS positioning can resolve the random phase rotation, resulting in improved positioning accuracy.

One main drawback of partial overlapping frequency hopping is that some of the frequency resources allocated for SRS transmissions are left unused to avoid collisions among different users. Fig. 9 shows one example consisting of 3 RedCap UEs allocated frequency resources for SRS transmissions. The frequency resources with different colors represent the different UEs. Due to the adjustment of the frequency domain starting position of a new hop according to the size of the overlapping frequency resources, some frequency resources reserved for SRS transmissions are not used. Compared to the wideband case, there will be a performance loss attributed to the never sounding some of the reserved resources.

Observation 6: With partial overlapping frequency hopping, some frequency resources allocated for SRS transmissions are not fully utilized.

t (OFDM Symbols)

f

Hopping Cycle

Fig. 9. Unused SRS frequency resources in partial overlapping frequency hopping

Based on the above observations, the partial overlapping frequency hopping for positioning with SRS is preferred over the non-overlapping option because it can deal with the random phase rotation without adding implementation complexity while improving the positioning accuracy.

For positioning with PRS in the downlink, either transmit or receive frequency hopping can be used [2]. The basic concept of the transmit frequency hopping for PRS is similar to the SRS positioning as described in Section 2. Here, the base station selects and transmits a PRS signal using a different portion of the wide bandwidth allocated for PRS transmissions at each hopping occasion. The receive frequency hopping, on the other hand, does not require the base station to perform frequency hopping. In this case, the base station transmits the PRS signal over a wideband channel while the receiver (i.e., RedCap UE) performs a narrowband positioning measurement at each hopping occasion until the entire wideband is covered.

Proposal 4: For improving the positioning accuracy of RedCap UEs, support transmit/receive frequency hopping for SRS and PRS with partial overlapping frequency resources for both FR1 and FR2.

# Conclusion

The contribution has discussed wideband frequency hopping RedCap UE for positioning with uplink SRS and downlink PRS. The following observations and proposals have been made:

Observation 1: For RedCap UEs with DL PRS Rx Hopping, the measurement gap depends on the following:

- the number of hops,

- the hop switching time,

- instantaneous dwell time (i.e., the number of OFDM symbols) of PRS

Observation 2: For UE Tx frequency hopping configured with one SRS positioning resource, a small instantaneous bandwidth increases the positioning measurement delay over the total frequency hopping bandwidth.

Observation 3:  RedCap UE Tx frequency hopping configured across SRS resources for positioning provides more flexibility (e.g., different instantaneous hopping bandwidths, etc.) than the one with one SRS resource.

Observation 4: Non-overlapping frequency hopping for SRS positioning causes an unknown or random phase rotation at every hop resulting in the positioning accuracy degradation.

Observation 5: Partial overlapping frequency hopping for SRS positioning can resolve the random phase rotation, resulting in improved positioning accuracy.

Observation 6: With partial overlapping frequency hopping, some frequency resources allocated for SRS transmissions are not fully utilized.

Proposal 1: For RedCap UE with DL PRS Rx hopping, RAN1 to discuss measurement gap length.

Proposal 2: For RedCap UE Tx frequency hopping is configured within one SRS resource, RAN1 should clarify which one or both of the following possible alternatives:

Alt 1: One SRS resource spanning the instantaneous bandwidth

Alt 2: One SRS resource spanning the total frequency hopping bandwidth

Proposal 3: For RedCap UE Tx frequency hopping, RAN1 to discuss instantaneous SRS bandwidth configurations per hop.

Proposal 4: For improving the positioning accuracy of RedCap UEs, support transmit/receive frequency hopping for SRS and PRS with partial overlapping frequency resources for both FR1 and FR2.

# References

RP-223549, “New WID on Expanded and Improved NR Positioning”, Intel Corporation, CATT, Ericsson

R1-2302065, Session notes for 9.5 Expanded and Improved NR Positioning, Ad-Hoc Chair (Huawei)

TR 38.859, Study on Expanded and Improved NR Positioning, Release 18

TS 38.211, Physical channels and modulation, Release 17 (v17.4.0)

## Appendix A: Legacy RedCap UE capabilities

From TS 38.306, Section 4.2.21.1, RedCap UE is the UE with reduced capability:

-	The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs;

-	The maximum mandatory supported DRB number is 8;

-	The mandatory supported PDCP SN length is 12 bits while 18 bits being optional;

-	The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional;

-	For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; for FR2, either 1 or 2 DL MIMO layers can be supported, while 2 Rx branches are always supported. For FR1 and FR2, UE features and corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branch or more than 1 UL MIMO layer are not supported by RedCap UEs;

-	CA, MR-DC, DAPS, CPAC and IAB (i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as non-RedCap UEs, unless indicated otherwise.

## Appendix B: SRS positioning resource for legacy non-RedCap UEs

From TS 38.331, the SRS resource definition for positioning for legacy non-RedCap UEs is defined as follows:

## Appendix C: Simulation assumptions

Table A - 1: Simulation assumptions for channel delay estimation

| Parameters | Values |
| --- | --- |
| SRS total sounding bandwidth | 48 PRBs |
| SRS comb spacing | 2 |
| SRS frequency hopping | {Yes, No} |
| SRS bandwidth per hop frequency hopping | 8 PRBs |
| Subcarrier spacing | 15 kHz |
| SRS error modeling | Follow Table A.1-2 in TR 36.897, Delta = 9 dB |
| Channel model | CDL based simplified model, TR 36.873, UMa, NLOS |
| Receiver diversity | No |
| Estimation method | MMSE |
| Power control | No |