3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302338

E-meeting, 17-26 April, 2023

Agenda item:	9.7.2

Source:	Huawei, HiSilicon

Title:	Cell DTX/DRX mechanism for network energy saving

Document for:	Discussion and decision

- Introduction

A new work item on network energy saving was approved in RAN plenary #98e and revised as in [1]. More details [2-3] can be found in Appendix. In this paper, we discuss the details of the cell DTX/DRX mechanism including framework of cell DTX/DRX, applied signals/channels of cell DTX/DRX, and interaction between UE C-DRX and cell DTX/DRX.

- Framework of cell DTX/DRX

Application granularity of cell DTX/DRX

Figure 1 Cell DTX/DRX configuration of different cells in the gNB

UE C-DRX is configured for a group of serving cells. That is, the active/inactive periods of all configured cells are aligned. From the point of view of UE power saving, this is very beneficial. However, from the perspective of gNB, for example, the PCell can be kept active or configured a DTX/DRX pattern with a shorter periodicity to maintain basic data transmission, while the SCell(s) can be configured a DTX/DRX pattern with a longer periodicity for energy saving, as shown in Figure 1. Therefore, the active/inactive period of each cell should be flexibly configured/(de)activated by the gNB.

Proposal 1: Cell DTX/DRX should be applied in a serving cell level.

Signalling for (de)activation

According to TR 38.864 and the discussion in RAN2#120, cell DTX/DRX could be configured by UE-specific RRC. However, the load of gNB may vary in a dynamic way, and the configuration of cell DTX/DRX needs to be adjusted accordingly. In this case, (de)activation only by RRC signalling is not flexible. Therefore, cell DTX/DRX (de)activation based on L1 signalling can be considered.

Further, since from gNB perspective the expected behaviour for sleeping/active would affect all UEs in a same manner in terms of active/inactive, period etc., a group/cell-level signalling should be introduced.

Furthermore, the design of L1 signalling should be further discussed from RAN1. There are two possible options to achieve cell DTX/DRX L1 signalling. One option is to introduce a new type of DCI format for cell DTX/DRX (de)activation, through a new DCI format or a new RNTI. The other option is to reuse a current DCI format for cell DTX/DRX (de)activation. Then new indication, by reusing existing field(s) or introducing new fields for differentiation would be needed.

Proposal 2: Consider to support dynamic cell DTX/DRX (de)activation by group-common L1 signaling

- RAN1 to discuss whether the L1 signaling is based on a new DCI format or an existing DCI format.

- Applied signals/channels for cell DTX/DRX

In this subsection, we will discuss which kind of signals/channels can be affected by cell DTX/DRX:

- PDCCH: Currently, UE C-DRX is designed for UE not to monitor PDCCH. However, when a UE is configured with C-DRX, it might be specific to that UE considering the UE traffic. Even if in practice that UE C-DRX can be configured in a cell level manner, for some cases the gNB may further seek for energy saving in different time slots from the configured UE C-DRX on duration, if the gNB decides to do so due to changed traffic or urgent energy saving demand. Then for sure gNB can just go to saving mode and leave the UE to monitor potential PDCCH. On the other hand, if the gNB can further indicate UEs that there won’t be PDCCH for a time that is over their on duration, the UEs can benefit from this indication with additional power saving. Therefore, cell DTX can be applied to at least UE specific PDCCH.

- PDSCH/PUSCH:

  - DG PDSCH/PUSCH: assuming the related PDCCHs can be applied with cell DTX, there is no need to restrict the transmission of DG PUSCH/PUSCH. That’s to say, UE can receive/transmit DG PDSCH/PUSCH during the inactive time of cell DTX/DRX if the corresponding PDCCH is scheduled in this way;

  - SPS PDSCH/CG PUSCH: for greater network energy saving gain, cell DTX/DRX should be applied to these channels. It may be ensured by gNB implementation that there are sufficient transmission occasions for SPS PDSCH/CG PUSCH in the active period of cell DTX/DRX;

- UCI:

  - HARQ-ACK: this information follows UE specific PDSCH, thus, if a corresponding PDSCH is received, the corresponding feedback is expected by gNB as well;

  - CSI:

    - Periodic/Semi-persistent CSI: similar to semi-static PDSCH/PUSCH, UE shall not transmit these CSIs in the inactive time of cell DRX;

    - Aperiodic CSI: similar to dynamic PDCCH and other scheduled channels, a UE can transmit A-CSI during the inactive time of cell DRX if the corresponding PDCCH is scheduled in this way;

  - SR: In general, following the same handling of other semi-static UL transmissions, UE may not transmit SR in the inactive time of cell DRX in most cases. Since gNB and UE knows where cell DTX enters the inactive time, if a corresponding UL grant is not received while cell DTX turns to inactive period, UE shall cancel the SR, or keep the SR pending till the next active period of cell DRX without triggering RACH;

    - However, a special case is SR for SCell BFR. UE may fail to complete SCell BFR procedure in cell DRX inactive time without this type of SR;

- Reference signals:

  - SRS: same handling as A/P/SP-CSI;

  - CSI-RS: in general, also similar to other channels that dynamic A-CSI-RS should follow the scheduling indication from gNB and semi-static P/SP-CSI-RS can be applied with cell DTX, since a UE can perform measurement (CSI-RS based RRM/RLM) based on SSB when necessary. Some further considerations may be needed for the following

      - CSI-RS for tracking (TRS): potential TRS/CSI-RS occasion(s) available in connected mode may be reused for idle/inactive-mode UEs. From this aspect, it is recommended that CSI-RS for tracking is not applied with Cell DTX;

      - CSI-RS for beam management for SCell BFR: Since SSB may not be configured for SCell, a UE may fail to complete SCell BFR procedure in cell DTX inactive time without this type of CSI-RS.

Figure 2 SCell BFR procedure when cell DTX/DRX is configured (or activated)

Proposal 3: Support the following signals/channels to be applied with Cell DTX/DRX:

- UE-specific PDCCH

- SPS PDSCH and CG PUSCH

- Periodic/Semi-persistent CSI

- Periodic/Semi-persistent SRS

- Periodic/Semi-persistent CSI-RS

- FFS: SR for SCell BFR, CSI-RS for tracking and CSI-RS for SCell BFR.

- Interaction between UE C-DRX and cell DTX

Figure 3 C-DRX is in active time while cell DTX is in inactive time

As shown in Figure 3, even if cell DTX/DRX is aligned with UE C-DRX, there are still some occasions that UE C-DRX is in active time and cell DTX/DRX is in inactive time. When cell DTX and UE C-DRX are applied to the same signal/channel for a same UE, whether the configuration/indication from cell DTX and UE C-DRX should be aligned or not, whether one can be higher priority or not and the corresponding UE behaviours in these cases need to be further studied.

Table I Possible cases of active/inactive time of C-DRX and cell DTX

| Possible cases | Cell DTX active time | Cell DTX inactive time |
| --- | --- | --- |
| UE C-DRX active time | UE normally monitors a DL channel/signal, etc., PDCCH. | UE behaviour is unclear if the occasion of a DL channel/signal that UE C-DRX and cell DTX can be both applied to is during the colliding period, regardless whether the configurations of UE C-DRX and cell DTX are aligned or not. |
| UE C-DRX inactive time | UE behaviour is unclear if the occasion of a DL channel/signal that UE C-DRX and cell DTX can be both applied to is during the colliding period, regardless whether the configurations of UE C-DRX and cell DTX are aligned or not. | UE does not monitor a DL channel/signal, etc., PDCCH. |

Proposal 4: Further discuss the case and UE behavior that cell DTX and UE C-DRX are applied to a same UE simultaneously.

- Conclusions

The following proposals are provided:

Proposal 1: Cell DTX/DRX should be applied in a serving cell level.

Proposal 2: Consider to support dynamic cell DTX/DRX (de)activation by group-common L1 signaling

- RAN1 to discuss whether the L1 signaling is based on a new DCI format or an existing DCI format.

Proposal 3: Support the following signals/channels to be applied with Cell DTX/DRX:

- UE-specific PDCCH

- SPS PDSCH and CG PUSCH

- Periodic/Semi-persistent CSI

- Periodic/Semi-persistent SRS

- Periodic/Semi-persistent CSI-RS

- FFS: SR for SCell BFR, CSI-RS for tracking and CSI-RS for SCell BFR.

Proposal 4: Further discuss the case and UE behavior that cell DTX and UE C-DRX are applied to a same UE simultaneously.

Appendix

After the RAN#98-e meeting, the objectives of the work item on cell DTX/DRX are the following [1]:

2. Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] • Note: No change for SSB transmission due to cell DTX/DRX. • Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

In RAN1#112 meeting, the cell DTX/DRX was discussed and the following agreement was reached [2]:

Agreement - RAN1 continues discussion on the at least following physical layer related aspects of cell DTX/DRX aspects - physical layer signals/channels and procedures expected to be impacted during non-active periods of cell DTX/DRX - consider impact to at least KPIs from the SI when physical layers/signals/channels are impacted by cell DTX/DRX - Further discussions on other aspects are not precluded Agreement At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS. - DL - Periodic/Semi-persistent CSI-RS (including TRS) - PRS - PDCCH scrambled with UE specific RNTI - PDCCH in Type-3 CSS - SPS-PDSCH - UL - SR - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - CG-PUSCH Other signals/channels are not precluded

In RAN2#121 meeting, the cell DTX/DRX was discussed with the following agreements [3]:

Agreements

- There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs

- Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX).  No further enhancements for CBRA and CFRA will be pursued.

- Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.   FFS whether we have DTX UE specific inactivity timer .  FFS on configuration signaling and stage 3.

- Confirm study item agreement that we can have separate DTX and DRX configuration. We will focus on designing DTX/DRX for at least single configuration.  FFS whether multiple configuration of cell DTX or DRX will be supported.

Confirm study item agreement that we can have separate DTX and DRX configuration.   We will focus on designing DTX/DRX for at least single configuration.  FFS whether multiple configuration of cell DTX or DRX will be supported. In the approved TR 38.864, the higher layer procedures of cell DTX/DRX have the following description [4]:

Cell DTX/DRX is applied to at least UEs in RRC_CONNECTED state. A periodic Cell DTX/DRX (i.e., active and non-active periods) can be configured by gNB via UE-specific RRC signalling per serving cell. Below examples on Cell DTX/DRX behaviour during non-active periods are assumed to be possible options, and the UE behaviour/impact will be studied: - Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods. - Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals) - Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS). - Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement). The study focus on UE behavior when at any point in time the cell activates a single DTX/DRX configuration. It is up to NW whether legacy UEs can access cells with Cell DTX/DRX. The Cell DTX/DRX mode can be activated/de-activated via dynamic L1/L2 signalling and UE-specific RRC signaling. Both UE specific and common L1/L2 signalling can be considered for activating/deactivating the Cell DTX/DRX mode. Cell DTX and Cell DRX modes can be configured and operated separately (e.g., one RRC configuration set for DL and another for UL). Cell DTX/DRX can also be configured and operated together. At least the following parameters can be configured per Cell DTX/DRX configuration: periodicity, start slot/offset, on duration. Details related to UE behaviour can be discussed during WI phase. Whether to support multiple Cell DTX/DRX configurations can be discussed later in the WI phase. It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs. The alignment mechanism can be discussed during the WI phase. From RAN2 perspective, Cell DTX/DRX is feasible.

References

- RP-230566, Huawei, revised WID: Network energy savings for NR, November 14 –November 18, 2022.

- R1-2302131, Moderator (Intel Corporation), Discussion Summary #2 for enhancements on cell DTX/DRX mechanism, February 14 – March 3, 2022.

- 3GPP TR 38.864, NR; Study on network energy savings for NR.